| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0142-v1 |
| --- | --- |

| Title: | AHG8: MI-RPR for machine consumption |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ayoung Kim Eun-vin An Kwang-deok Seo Soon-heung Jung Sangwoon Kwak Won-Sik Cheong Jin Young Lee Hyon-Gon Choo | Email: | {aykim90, eunbin.an, kdseo}@yonsei.ac.kr {zeroone, s.kwak, wscheong, jinlee, hyongonchoo}@etri.re.kr |
| Source: | Yonsei University, ETRI |  |  |

_____________________________

- Abstract

This contribution describes a machine inference-based RPR mechanism employed in VCM to preserve accuracy of machine vision tasks and enhance coding efficiency. The proposed method consists of two stages: 1) a pre-analysis stage to determine the optimal scale factor using the Object Occupancy Distribution, 2) a VVC encoding stage incorporated with MI-RPR to generate a bitstream suitable for the machine vision tasks.

Introduction

To achieve the goal of maximizing both the performance of machine vision tasks and the compression efficiency of videos, this contribution presents the adaptive spatial resampling based on MI-RPR, which is adopted in Video Coding for Machines (VCM) [1]. The MI-RPR consists of two stages. In the first stage, a pre-analysis is conducted to determine the optimal scale factor for encoding using Object Occupancy Distribution (OOD) information, which is obtained from the analysis on the area occupied by objects within a single frame. The derived adaptive scale factor enables the application of a downscaling ratio to reduce the spatial resolution of the input frame before encoding the video, while not causing performance degradation in the machine vision tasks. And then, the second stage involves a compression process to generate a bitstream that is suitable for the machine vision tasks by using the VVC incorporated with the MI-RPR.

MI-RPR for machine consumption

Pre-analysis process for determining the optimal scale factor

In order to determine the optimal scale factor which is used in the MI-RPR, the object detection is performed in advance. To accomplish this, yolo-v7 is utilized [2], and the result is referred to as Detected Object Information (DOI). In this context, the collection of all DOI for each given scale factor becomes the DOIs. The pre-analysis stage utilizes DOI information to carry out the Valid Object Filtering, Optimal Scale Decision, and Scale Adjustment processes sequentially, leading to the decision of the optimal scale factor. Figure 1 depicts the flowchart of the pre-analysis composed of the three modules. Each role of the modules is described as follows:

- Valid Object Filtering: Identifying valid objects from all the detected objects.

- Optimal Scale Decision: Analyzing the area occupied by the identified valid objects in the frame and deriving the optimal scale factor.

- Scale Adjustment: Adjusting the derived scale factor to compensate for video quality degradation caused by applying quantization with the given QP values.

Figure 1.Pre-analysis flowchart composed of the three modules for determining the optimal scale factors.

For the valid objects obtained from the Valid Object Filtering module, OOD is generated for every candidate scale factor x (ex. x={100%, 90%, 80%, …, 10%}) based on the analysis on the area occupied by the valid objects in each frame. Afterward, the cross-correlation between the OODx of candidate scale factors x and the OOD100  (OOD of original resolution 100%) is computed.

Figure 2 shows the example of OOD for various scale factors for the 84th frame of the BasketballDrive test sequence. As observed in the Figure 2, it can be noted that there is a significant decrease in the correlation below a specific scale factor (50% in this example). According to these results, the scale factor 50% is determined as the optimal scale factor.

Figure 2. Experiment results related to OOD generation for the 84th frame of the BasketballDrive_sequence.

Compression process using the MI-RPR

VVC introduces a RPR feature to support changing spatial resolution of a coded video without inserting an IDR or IRAP frame. This feature enables improved coding efficiency for adaptive video streaming since it allows seamless frame-based bit-rate adaptation. The document JVET-AB0080 [2] presents how to decide when it is likely better to encode frames in reduced resolution with RPR and when it is likely better to encode them in the source resolution. Figure 3 illustrates the operation process of the conventional RPR adopted in VVC. RPR is applied when the PSNR calculated between the original source video and the upscaled reconstructed video (from the downscaled video) exceeds the given threshold condition.

Figure 3. Conceptual architecture of the conventional RPR

However, it is necessary to modify the conventional RPR tool due to the fact that the codec for machine consumption is performed for the machine vision tasks rather than human vision applications.

Figure 4. Conceptual architecture of the MI-RPR

The operation process of the MI-RPR is illustrated in Figure 4. The derived optimal scale factor list, which contains the optimal scale factor for each input frame, is obtained through the pre-analysis and the adaptive scale factor decision as illustrated in the subsection 2.1. This list is then passed onto the VVC incorporated with MI-RPR for further encoding by adaptively changing the spatial resolution using the determined optimal scale factors.

Conclusion

This contribution elaborates the MI-RPR tool for the codec considering the machine consumption and the performance of the MI-RPR could be found in [3]. It is recommended to consider using the MI-RPR concept for optimization of encoders and receiving systems for machine analysis of coded video content.

References

[1]  ISO/IEC JTC 1/SC 29/WG 04, “[VCM] Adaptive spatial resampling based on MI-RPR for VCM”, m64124, Geneva, July 2023.

[2]  K. Andersson, J. Ström, R. Yu, P. Wennersten, W. Ahmad, "AHG10: GOP-based RPR encoder control", JVET-AH2030, October 2022, Mainz, DE.

[3]	ISO/IEC JTC 1/SC 29/WG 04, “[VCM] Summary report of CE3: spatial resampling”, m66623, Online, January 2024.

Patent rights declaration(s)

Yonsei University and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).