| Joint Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0155 |
| --- | --- |

| Title: | AhG14: Report of HOP3 training using additional dataset from JVET-AH0120 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Franck Galpin, Tangi Poirier | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of the training of HOP3 using an additional dataset presented in JVET-AH0120. The stage 3 of HOP3 model was performed under common conditions, where the dataset was added to already existing dataset. The additional data represents 0.9% of the new dataset. First results show a large error in both validation loss during the training and the inference using CTC. More tests are needed to identify the root of these error increase.

# Overview

In JVET-AH0120, new video sequences, including some CG content were proposed to enrich the current dataset used for training NN based tools in NNVC. We performed the training of HOP3 in NNVC 9.1 using this new added content.

# Content

The dataset is described in the attached json file. It is composed of the following sequences:

- Barn_1920x1080_30fps_10bits_420

- Birthday_2048x1088_30fps_8bits_420

- Breakfast_1920x1080_30fps_10bits_420

- Dispute_1920x1080_25fps_10bits_420

- Joker_1920x1080_25fps_10bits_420

- Kart_1920x1080_30fps_10bits_420

- Mannequin_1920x1080_25fps_10bits_420

- Mask_1920x1080_25fps_10bits_420

- Painter_2048x1088_30fps_8bits_420

For each sequence, 65 frames are used, sometimes not at the beginning of the sequence. For each chunk of 65 frames, 11 frames are used to extract the patches. The final dataset contains 60280 patches of size 144x144.

# Results

## Validation loss

The validation loss is compared to the one of a HOP3 training using official dataset versus the new dataset in JVET-AH0120 added. It is shown that the loss is higher and more unstable in the second case.

## Results

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 4.48% | 34.33% | 31.25% | 5.35% | 38.32% | 37.27% |  |  |
| Class C | 4.04% | 31.53% | 29.26% | 5.21% | 34.40% | 34.00% |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D | 4.69% | 29.84% | 28.02% | 4.81% | 32.37% | 31.81% |  |  |
| Class F | 1.42% | 17.87% | 16.32% | 2.74% | 23.87% | 22.15% |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 3.25% | 27.98% | 28.83% | 3.92% | 32.46% | 34.30% |  |  |
| Class C | 3.39% | 25.59% | 25.23% | 4.34% | 33.19% | 33.40% |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D | 3.52% | 22.99% | 24.01% | 3.92% | 31.52% | 33.42% |  |  |
| Class F | 1.98% | 17.63% | 16.42% | 2.56% | 23.84% | 23.36% |  |  |

Results show large loss over legacy HOP3 model.

# Conclusion

First results show a large error in both validation loss during the training and the inference using CTC. More tests are needed to identify the cause of these error increase.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).