| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0180-v1 |
| --- | --- |

| Title: | AHG9: Adding original source picture dimensions to EOI SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is proposed to add optional signalling of the original source picture dimensions in the encoder optimization information (EOI) SEI message. The exisiting eoi_spatial_resampling_type_flag is proposed to be made conditionally present when the original source picture dimensions are not signalled.

# Proposal

The EOI SEI message contains an eoi_spatial_resampling_type_flag, signalled whenever the eoi_type syntax element indicates that spatial resampling had taken place by the encoding system, to indicate whether the resampling operation was upsampling or downsampling.

It is asserted to be useful for the receiver to be provided the original source picture dimensions in order to enable reconstruction of decoded pictures at the original source resolution. It is noted that when the EOI SEI message indicates temporal resampling, the number of pictures the encoding system excluded is signalled, as eoi_num_int_pics.

It is proposed to add optional signalling of the original source picture dimensions in the encoder optimization information (EOI) SEI message. A flag is signalled to indicate the presence of the original source picture dimensions. The eoi_spatial_resampling_type_flag is proposed to be made conditionally present when the original source picture dimensions are not signalled, as it is unnecessary when the original source picture dimensions are available.

## Syntax

Modifications to the EOI SEI message syntax are highlighted in yellow.

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| if(EoiSpatialResamplingFlag) { |  |
| eoi_orig_pic_dimensions_flag | u(1) |
| if ( eoi_orig_pic_dimensions_flag ) { |  |
| eoi_orig_pic_width | u(16) |
| eoi_orig_pic_height | u(16) |
| } |  |
| else |  |
| eoi_spatial_resampling_type_flag | u(1) |
| } |  |
| if( EoiPrivacyProtectionFlag ) { |  |
| eoi_privacy_protection_type_idc | u(4) |
| eoi_privacy_protected_info_type | u(8) |
| } |  |
| } |  |
| } |  |

## Semantics

    - Encoder optimization information SEI message semantics

The encoder optimization information SEI message is used to indicate if the video has been optimized for human viewing or machine analysis and which types of optimization have been applied in pre-processing or encoding.

eoi_cancel_flag equal to 1 specifies that the persistence of the encoder optimization information SEI message included in any previous PU in output order is cancelled. eoi_cancel_flag equal to 0 indicates that information on optimization that has been applied in pre-processing or encoding follows.

eoi_persistence_flag specifies the persistence of the optimization information provided in this SEI message. eoi_persistence_flag equal to 0 specifies that the optimization information applies for the current picture only. eoi_persistence_flag equal to 1 specifies that the optimization information applies for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with an encoder optimization information SEI message is output that follows the current picture in output order.

eoi_for_human_viewing_idc equal to 3 specifies that purposes for the applied optimization include human viewing. eoi_for_human_viewing_idc equal to 2 specifies that the video is suitable but not specifically optimized for human viewing. eoi_for_huma_viewing_idc equal to 1 specifies that the video is unsuitable for human viewing. eoi_for_human_viewing_idc equal to 0 specifies that it is unknown if the video is suitable for human viewing.

eoi_for_machine_analysis_idc equal to 3 specifies that purposes for the applied optimization include machine analysis. eoi_for_machine_analysis_idc equal to 2 specifies that the video is suitable but not specifically optimized for machine analysis. eoi_for_machine_analysis_idc equal to 1 specifies that the video is unsuitable for machine analysis. eoi_for_machine_analysis_idc equal to 0 specifies that it is unknown if the video is suitable for machine analysis.

It is a requirement of bitstream conformance that the value of eoi_for_human_viewing_idc and eoi_for_machine_analysis_idc shall not be both equal to 1.

eoi_type indicates the types of optimization method as specified in Table x1 where ( eoi_type & bitMask ) not equal to 0 indicates that the optimization type with the bitMask value in Table x1 has been applied. When eoi_type is greater than 0 and ( eoi_type & bitMask ) is equal to 0, the optimization type with the bitMask value has not been applied. When eoi_type is equal to 0, optimization as determined by the application has been used.

Table x1 – Definition of eoi_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| 0x02 | Temporal resampling optimization |
| 0x04 | Spatial resampling optimization |
| 0x08 | Temporal quality optimization in a manner that quality fluctuates temporally |
| 0x10 | Spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |
| 0x20 | Privacy protection optimization; the pictures for which this SEI message persists have been pre-processed or encoded to protect personal information. (e.g. removal or replacing of personal identifiable information, pseudonymization, anonymization) |

The variables EoiObjectBasedFlag, EoiTemporalResamplingFlag, EoiSpatialResamplingFlag, EoiTemporalQualityFlag, EoiSpatialQualityFlag, and EoiPrivacyProtectionFlag, specifying whether eoi_type indicates the type of the optimization to include object-based optimization, temporal resampling optimization, spatial resampling optimization, temporal quality optimization, spatial quality optimization, and privacy protection optimization, respectively, are derived as follows:

EoiObjectBasedFlag = ( ( eoi_type & 0x01 ) > 0 ) ? 1 : 0
EoiTemporalResamplingFlag = ( ( eoi_type & 0x02 ) > 0 ) ? 1 : 0
EoiSpatialResamplingFlag = ( ( eoi_type & 0x04 ) > 0 ) ? 1 : 0	(xx)
EoiTemporalQualityFlag = ( ( eoi_type & 0x08 ) > 0 ) ? 1 : 0
EoiSpatialQualityFlag = ( ( eoi_type & 0x10 ) > 0 ) ? 1 : 0
EoiPrivacyProtectionFlag = ( ( eoi_type & 0x20 ) > 0 ) ? 1 : 0

NOTE – For example, when certain highest temporal sublayers have been encoded with such coarse quantization that human viewers perceive the quality fluctuation annoying, but machine task performance is not compromised, eoi_for_human_viewing_flag and eoi_for_machine_analaysis_flag can be set equal to 0 and 1, respectively, and eoi_type can be set equal to a value that causes EoiTemporalQualityFlag to be equal to 1.

When eoi_persistence_flag is equal to 0, it is a requirement of bitstream conformance that EoiTemporalResamplingFlag shall be equal to 0 and EoiTemporalQualityFlag shall be equal to 0.

eoi_object_based_idc, when present, indicates the type of object-based optimization as specified in Table x2, where ( eoi_object_based_idc & bitMask ) not equal to 0 indicates that the object-based optimization type associated with the bitMask value in Table x2 has been applied. When eoi_object_based_idc is greater than 0 and ( eoi_object_based_idc & bitMask ) is equal to 0, the object-based optimization type associated with the bitMask value has not been applied. When eoi_object_based_idc is equal to 0, an application-defined type of object-based optimization has been applied. The value of eoi_object_based_idc shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 65 535, inclusive, for eoi_object_based_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_object_based_idc is in the range of 8 to 65 535, inclusive, decoders conforming to this version of this Specification shall ignore eoi_object_based_idc.

Table x2 – Definition of eoi_object_based_idc

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Areas outside the detected objects have been blurred prior to encoding. |
| 0x02 | Areas outside the detected objects have been encoded with coarser transform-domain quantization than the quantization used for the detected objects. |
| 0x04 | Areas outside the detected objects have been overwritten. For example, an encoding system can overwrite areas outside the detected objects with a constant sample value. |

eoi_temporal_resampling_type_flag equal to 0 specifies that the temporal resampling optimization is a subsampling operation. eoi_temporal_resampling_type_flag equal to 1 specifies that the temporal resampling optimization is an upsampling operation.

eoi_num_int_pics greater than 0 indicates that the count of pictures that the encoding system excluded between each pair of coded pictures in output order (when eoi_temporal_resampling_type_flag is equal to 0) or added between each pair of source pictures for encoding (when eoi_temporal_resampling_type_flag is equal to 1) within the persistence of this SEI message is constant. When eoi_temporal_resampling_type_flag is equal to 0 and eoi_num_int_pics is greater than 0, eoi_num_int_pics specifies the count of pictures that the encoding system excluded between each pair of coded pictures in output order. When eoi_temporal_resampling_type_flag is equal to 1 and eoi_num_int_pics is greater than 0, eoi_num_int_pics specifies the count of pictures that the encoding system added between each pair of source pictures for encoding.

eoi_num_int_pics equal to 0 indicates that the count of pictures that the encoding system excluded between each pair of coded pictures in output order (when eoi_temporal_resampling_type_flag is equal to 0) or added between each pair of source pictures for encoding (when eoi_temporal_resampling_type_flag is equal to 1) within the persistence of this SEI message is unknown or varying.

The value of eoi_num_int_pics shall be in the range of 0 to 63, inclusive.

eoi_orig_pic_dimensions_flag equal to 1 specifies that the eoi_orig_pic_width and eoi_orig_pic_height syntax elements are present. eoi_orig_pic_dimensions_flag equal to 0 specifies that the eoi_orig_pic_width and eoi_orig_pic_height are not present.

eoi_orig_pic_width and eoi_orig_pic_height, when present, indicate the width and height, respectively, of the original source picture in units of luma samples.

eoi_spatial_resampling_type_flag equal to 0 specifies that the spatial resampling optimization is a subsampling operation. eoi_spatial_resampling_type_flag equal to 1 specifies that the spatial resampling optimization is an up-sampling operation.

eoi_privacy_protection_type_idc, when present, indicates the type of privacy protection optimization as specified in Table x3.

Table x3 – Definition of eoi_privacy_protection_type_idc

| eoi_privacy_protection_type_idc | Interpretation |
| --- | --- |
| 0 | Unknown or determined by the application. |
| 1 | Blurring; personal information is blurred to make it unidentifiable. |
| 2 | Replacing; personal information is replaced with something different from the original to make it unidentifiable. |
| 3 | Masking; personal information is masked so that it cannot be identified |
| 4…15 | Reserved for future use. |

eoi_privacy_protected_info_type, when present, indicates the types of protected information as specified in Table x4 where eoi_privacy_protected_info_type is greater than 0 and ( eoi_privacy_protected_info_type & bitMask ) not equal to 0 indicates that the information type with the bitMask value in Table x4 has been protected. When eoi_privacy_protected_info_type is equal to 0, an application-defined type of information has been protected. The value of eoi_privacy_protection_info_type shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 255, inclusive, for eoi_privacy_protected_info_type are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_privacy_protected_info_type is in the range of 8 to 255, inclusive, decoders conforming to this version of this Specification shall ignore eoi_privacy_protected_info_type.

Table x4 – Definition of eoi_privacy_protection_info_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Information that identifies a person is protected. For example, the face of the person. |
| 0x02 | Information that can identify vehicles is protected. For example, the license plate of the vehicle. |
| 0x04 | Information that can infer locations is protected. For example text or images on signs. |

…

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).