| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0181-v4 |
| --- | --- |

| Title: | AHG9: Display overlays information SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a display overlays information SEI message. The information conveyed in the SEI message can be used to form a target display picture, which is formed by overlaying multiple display overlays in a specified order. A display overlay has a texture component and an optional alpha component, which are rectangular sample regions. Each component can be coded as a picture, a subpicture, or a constituent rectangle in a single layer or multi-layer CVS. Use cases for the proposed SEI message include score overlays, title overlays, logo insertion, product placement, and background replacement.

In the -v3 and -v4 versions, an alternative syntax is proposed that enables composition use cases, which could be applicable to functionalities raised in  JVET-AI0256.

# Proposal

Displayed video content frequently contains a composition of camera-captured content and overlay graphics. Typically overlay composition is performed before video is encoded for broadcast, streaming, or storage, causing the overlaid portions of the original camera-capture content to be lost.

The proposed display overlays information (DOI) SEI message enables multiple display overlays to be coded within the same video bitstream and composed at the decoder/receiver. A “display overlay” is a rectangular region of texture samples and optional rectangular region of corresponding alpha channel samples, referred to as the texture component and alpha component, respectively. The term “display overlay” is used instead of “layer”, in an attempt to minimize confusion with layers in a video bitstream which have a particular nuh_layer_id value. For the purposes of the SEI message, even a background or “base” display layer is considered to be a display overlay.

Each display overlay component can be coded as a picture, a subpicture, or a constituent rectangle in a single layer or multi-layer CVS. SEI message syntax elements are used to identify the location of the display overlay component in the coded video and its intended display order and display location in a target display picture.

## Examples

Figures 1 and 2 illustrate an example usage of the proposed SEI message. For this example, there are 3 display overlays. Figure 1 shows the target display picture, which includes the 3 display overlays:  a sports scene, a scoreboard overlay, and a broadcast station logo overlay. Figure 2 shows what is coded in the bitstream. The 0-th display overlay is shown in Figure 2(a), and contains the “background” video content, e.g. the sports scene, which is coded as a picture in layer 0, and does not have an alpha channel. Figure 2(b) shows a coded picture containing both the second and third display overlays, the scoreboard and logo, with both display overlays having an alpha channel, for a total of 4 components. The 4 components in layer 1 are coded as subpictures or constituent rectangles of a picture in this example, as shown in Figure 2(d), representing display overlay 1 texture (DO1 tex), display overlay 1 alpha (DO1 alp), display overlay 2 texture (DO2 tex) and display overlay 2 alpha (DO1 alp).

.

Figure 1. Example target display picture

| - Coded layer 0 picture | - Coded layer 1 picture |
| --- | --- |
| - Labeled coded layer 0 picture | - Labeled coded layer 1 picture |

Figure 2. Example using 2 coded pictures

Figure 3 shows an alternate way to code the same content using a single coded picture which contains all 5 components of the 3 display overlays.

Figure 3. Alternative example using one coded picture

Video players or Media Aware Network Elements (MANEs) would have the ability to choose to display or not display individual display overlays.

Figures 3 and 4 illustrate an example usage for background replacement, using two display overlays, representing the background and foreground. Figure 3 shows the target display picture, in which a person is shown in front of a virtual background, which may be a static or dynamic background, e.g. a background image or a background video. Figure 4(a) shows the background image/video, coded in layer 0, which is the display overlay 0 texture component. Figure 4(b) shows foreground (display overlay 1) texture component, which is coded in layer 1. Figure 4(c) shows the foreground (display overlay 1) alpha component, which is coded in layer 2.

Figure 3. Example target display picture

| - Coded layer 0 pic: display overlay 0 texture component | (b) Coded layer 1 pic: display overlay 1 texture component |
| --- | --- |
| (c) Coded layer 2 pic: display overlay 1 alpha component |  |

Figure 4. Example coded pictures using display overlays SEI

## Summary of Features

Features of the proposed SEI message are summarized below:

- Multiple display overlays are described by the SEI message and are used to form a target display picture

- Each display overlay has a texture component and optionally may have an alpha component

- A display overlay component may be coded as a picture, subpicture, or constituent rectangle, in a single-layer or multiple-layer CVS

- Target display picture display resolution is set to the resolution of the 0-th display overlay

- Display overlays are applied in index order when forming the target display picture

- Display overlays may differ in resolution. The position and size of each display overlay in the target display picture are optionally signalled

- Display overlay components may be resampled when forming the target display picture, when signalled size differs from coded size

- Display overlays coded in separate layers may have different frame rates

## Motivation and Design Discussion

The proposed SEI message design provides flexibility, allowing encoders to decide how best to address specific applications.

Use cases include overlaying graphics such as logos, scores, or onscreen text such as titles. Another use case is product placement, where a particular product can be replaced without re-encoding the main video. Background replacement is another supported use case.

A display overlay component may be coded as a complete picture, a subpicture, or a constituent rectangle in a single layer or multi-layer CVS.

An encoder for a specific application may prefer to code multiple display overlay components within a single layer using subpictures or constituent rectangles, for simpler operation by avoiding the need to synchronize decoding of multiple layers.

An encoder for another application may prefer using multiple layers for coding of the display overlays, including the alpha component for a particular display overlay in the same layer as its corresponding texture component. The multiple layers would likely be independently coded layers, but the proposed semantics do not impose that restriction.

Use of multiple layers could be used, for example, to separate the 0-th display overlay, e.g. the background display overlay, from the foreground display overlay(s). Coding of the foreground and background as display overlays coded in separate layers enables a Media Aware Network Element (MANE) to replace the background without requiring re-encoding of the primary content layer.

Placing display overlay components in separate layers enables different frame rates of overlays, which may help subjective quality by avoiding visible pulsing of static graphic overlays. It may also save bitrate, especially for semi-transparent overlays with dynamic background video.

Using a separate layer for display overlay components also allows encoders the option of using a different chroma format or color space for graphic overlays, for example using RGB 4:4:4 for the graphic overlay while the background video is coded in YUV 4:2:0.

Using separate layers also allows encoders to enable different sets of tools for the different display overlays, e.g. using screen content coding tools for a graphics overlay.

Placing display overlay components in a separate layer enables easier replacement of an individual layer. For example, text overlays such as displayed onscreen titles may be contained in a display overlay in a separate layer than the main video content, allowing replacement of titles for different languages while avoiding the need to re-encode the main video content.

Having display overlays separately available to a video decoder/player enables a variety of use cases, such as reprojection, special effects such as transitions, or enabling/disabling display of individual overlays.

Individual overlay components may be coded with a different effective resolution and resampled, whether the components are encoded in a single layer or multiple layers.

# Syntax and semantics

## VSEI Syntax

| display_overlays_info( payloadSize ) { | Descriptor |
| --- | --- |
| doi_id | u(6) |
| doi_cancel_flag | u(1) |
| if( !doi_cancel_flag ) { |  |
| doi_persistence_flag | u(1) |
| doi_num_display_overlays_minus2 | ue(v) |
| doi_nuh_layer_id_present_flag | u(1) |
| doi_pic_partition_flag | u(1) |
| if( doi_pic_partition_flag ) { |  |
| doi_partition_type_flag | u(1) |
| doi_partition_id_len_minus1 | u(4) |
| } |  |
| doi_offset_params_present_flag | u(1) |
| if( doi_offset_params_present_flag ) |  |
| doi_offset_param_length_minus1 | u(4) |
| doi_resampling_enabled_flag | u(1) |
| if( doi_resampling_enabled_flag ) |  |
| doi_size_param_length_minus1 | u(4) |
| for( i = 0; i < doi_num_display_layers_minus2 + 2; i++ ) { |  |
| if ( doi_nuh_layer_id_present_flag ) |  |
| doi_nuh_layer_id[ i ] | u(6) |
| if ( doi_pic_partition_flag ) |  |
| doi_partition_id[ i ] | u(v) |
| doi_alpha_present_flag[ i ] | u(1) |
| if ( doi_alpha_present_flag[ i ] ) { |  |
| if (doi_nuh_layer_id_present_flag) |  |
| doi_alpha_nuh_layer_id[ i ] | u(6) |
| if (doi_pic_partition_flag) |  |
| doi_alpha_partition_id[ i ] | u(v) |
| } |  |
| if( i > 0 ) { |  |
| if (doi_offset_params_present_flag ) |  |
| doi_top_left_x[ i ] | u(v) |
| doi_top_left_y[ i ] | u(v) |
| } |  |
| if( doi_resampling_enabling_flag ) { |  |
| doi_width_minus1[ i ] | u(v) |
| doi_height_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## VSEI Semantics

The display overlays information (DOI) SEI message provides metadata to enable formation of a target display picture formed by overlaying multiple ordered display overlays in a specified order. A display overlay contains texture and optionally an alpha channel, each contained within a cropped decoded picture, a subpicture, and/or a constituent rectangle.

Use of this SEI message requires the definition of the following variables, where i is the layer identifier of a layer that may be present in the current CVS:

–	An array of picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples[ i ] and PicHeightInLumaSamples[ i ], respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc[ i ], as described in subclause ‎7.3.

–	An array of subpicture counts, denoted by NumSubpics[ i ].

–	Arrays of the width and height of the subpictures, denoted herein by SubPicWidth[ i ][ j ] and SubPicHeight[ i ][ j ] respectively, where j is the subpicture index in 0 .. NumSubpics[ i ] – 1.

doi_id specifies an identifier of the DOI SEI message.

doi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous DOI SEI message with the same doi_id in output order. doi_cancel_flag equal to 0 indicates that display overlays information follows.

doi_persistence_flag specifies the persistence of the DOI SEI message for the CVS.

doi_persistence_flag equal to 0 specifies that the DOI SEI message applies to the current AU only.

doi_persistence_idc equal to 1 specifies that the DOI SEI message applies to the current AU and persists for all subsequent AUs in output order until one or more of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A picture in the current AU with a DOI SEI message with the same value of doi_id is output that follows the current picture in output order.

doi_num_display_overlays_minus2 plus 2 specifies the number of display overlays for which information is signalled in the SEI message.  The value of doi_num_display_overlays_minus2 shall be in the range of 0 to 30, inclusive.

doi_nuh_layer_id_present_flag equal to 1 specifies that the doi_nuh_layer_id[ i ] syntax element is present in the SEI message. doi_nuh_layer_id_present_flag equal to 0 specifies that the doi_nuh_layer_id[ i ] syntax element is not present in the SEI message.

doi_pic_partition_flag equal to 1 specifies that display overlay components are coded as constituent rectangles or subpictures. doi_pic_partition_flag equal to 0 specifies that display overlay components are coded as pictures.

It is a requirement of bitstream conformance that at least one of doi_nuh_layer_id_present_flag or doi_pic_partition_flag shall be equal to 1.

doi_partition_type_flag equal to 1 specifies that a display overlay component is coded as a constituent rectangle. doi_partition_type_flag equal to 0 specifies that a display overlay component is coded as a subpicture.

When doi_partition_type_flag equal to 1, it is a requirement of bitstream conformance that there is a constituent rectangles SEI message preceding the DOI SEI message in decoding order in the current PU.

doi_partition_id_len_minus1 + 1 specifies the length of the doi_partition_id[ i ] syntax elements.

doi_offset_params_present_flag[ i ] equal to 1 specifies that offset parameters are present for the i-th display overlay. doi_offset_params_present_flag[ i ] equal to 0 specifies that offset parameters are not present for the i-th display overlay.

doi_offset_param_length_minus1 plus 1 specifies the length of the doi_top_left_x[ i ] and doi_top_left_y[ i ], syntax elements in bits.

doi_resampling_enabled_flag equal to 1 specifies that display overlay components may be resampled in the target display picture.   doi_resampling_enabled_flag equal to 0 specifies that display overlay components are not resampled in the target display picture.

doi_size_param_length_minus1 plus 1 specifies the length of the doi_width_minus1[ i ] and doi_height_minus1[ i ] syntax elements in bits.

doi_nuh_layer_id[ i ], when present, specifies the layer identifier of the texture component of the i-th display overlay. When not present, the value of doi_nuh_layer_id[ i ] is inferred to be equal to the layer identifier of the PU containing the DOI SEI message.

If this SEI message is present in any layer in the current AU, it is a requirement of bitstream conformance that a DOI SEI message with the same value of doi_id and the same payload is present in the layer with layer identifier doi_nuh_layer_id[ 0 ].

doi_partition_id[ i ], when present and doi_partition_type_flag equal to 1, specifies  that the texture component of the i-th display overlay is represented by the j-th constituent rectangle when cr_rect_id[ j ] equal doi_partition_id[ i ]. doi_partition_id[ i ], when present and doi_partition_type_flag equal to 0, specifies the subpicture index of the texture component of the i-th display overlay.  When not present, the value of doi_partition_id[ i ] is inferred to be equal to 0.

When doi_partition_type_flag equal to 1, doi_partition_id[ i ] shall be in the range of 0 ..  cr_num_rects_minus1[ doi_nuh_layer_id[ i ] ] – 1.  When doi_partition_type_flag equal to 0, doi_partition_id[ i ] shall be in the range of 0 .. NumSubpics[ doi_nuh_layer_id[ i ] ] – 1.

doi_alpha_present_flag[ i ] equal to 1 specifies that an alpha component is provided for the i-th display overlay. doi_alpha_present_flag[ i ] equal to 0 specifies that an alpha component is not provided for the i-th display overlay.

doi_alpha_nuh_layer_id[ i ], when present, specifies the layer identifier value of the alpha component of the i-th display overlay. When not present, the value of doi_alpha_nuh_layer_id[ i ] is inferred to be equal to the layer identifier of the PU containing the DOI SEI message.

doi_alpha_partition_id[ i ], when present and doi_partition_type_flag equal to 1, specifies the cr_rect_id[ j ] of the alpha component of the i-th display overlay. doi_partition_id[ i ], when present and doi_partition_type_flag equal to 0, specifies the subpicture index of the alpha component of the i-th display overlay.  When not present, the value of doi_alpha_partition_id[ i ] is inferred to be equal to 0.

When doi_partition_type_flag equal to 1, doi_alpha_partition_id[ i ] shall be in the range of 0 ..  cr_num_rects_minus1[ doi_nuh_layer_id[ i ] ] – 1.  When doi_partition_type_flag equal to 0, doi_alpha_partition_id[ i ] shall be in the range of 0 .. NumSubpics[ doi_nuh_layer_id[ i ] ] – 1.

doi_top_left_x[ i ] and doi_top_left_y[ i ] specify the horizontal and vertical positions, respectively, of the top left corner of the i-th display overlay in the target display picture, in luma samples. When not present, the values of doi_top_left_x[ i ] and doi_top_left_y[ i ] are inferred to be equal to 0. The length of the syntax elements is doi_offset_param_length_minus1 + 1 bits.

doi_width_minus1[ i ] plus 1 and doi_height_minus1[ i ] plus 1, when present, specify the width and height, respectively, in luma samples arrays of the i-th display overlay in the target display picture. The length of the syntax elements is doi_size_param_length_minus1 + 1 bits.

The variables CodedOverlayTexture[ i ] and CodedOverlayAlpha[ i ] are picture sample arrays with luma resolution CodedOverlayWidth[ i ] × CodedOverlayHeight[ i ], derived as follows:

If doi_pic_partition_flag is equal to 0, the following applies:

–	CodedOverlayWidth[ i ] is set equal to PicWidthInLumaSamples[ doi_nuh_layer_id[ i ] ] ].

–	CodedOverlayHeight[ i ] is set equal to PicWidthInLumaSamples[ doi_nuh_layer_id[ i ] ].

–	If there is a picture in the AU for the layer with layer identifier doi_nuh_layer_id[ i ], CodedOverlayTexture[ i ] is set equal to the cropped decoded picture from the layer with layer identifier doi_nuh_layer_id[ i ] in the AU. Otherwise (there is no picture in the AU for the layer with layer identifier doi_nuh_layer_id[ i ]), CodedOverlayTexture[ i ] is set equal to the previous cropped decoded picture in output order in the layer with layer identifier doi_nuh_layer_id[ i ].

–	If doi_alpha_present_flag[ i ] equal to 1, the following applies:

- If there is a picture in the AU for the layer with layer identifier doi_alpha_nuh_layer_id[ i ], CodedOverlayAlpha[ i ]  is set equal to the cropped decoded picture from the layer with layer identifier doi_alpha_nuh_layer_id[ i ] in the AU.

- Otherwise (there is no picture in the AU for the layer with layer identifier doi_nuh_layer_id[ i ]), CodedOverlayAlpha[ i ] is set equal to the previous cropped decoded picture in output order in the layer with layer identifier doi_alpha_nuh_layer_id[ i ].

Otherwise, if doi_partition_type_flag is equal to 0, the following applies:

–	CodedOverlayTexture[ i ] is set equal to the subpicture with subpicture index doi_partition_id[ i ] from the layer with layer identifier doi_nuh_layer_id[ i ].

–	CodedOverlayWidth[ i ] is set equal to SubPicWidth[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayHeight[ i ] is set equal to SubPicHeight[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	If doi_alpha_present_flag[ i ] equal to 0, the following applies:

–	CodedOverlayAlphaWidth[ i ]  is set equal to SubPicWidth[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

– 	CodedOverlayAlphaHeight[ i ]  is set equal to SubPicHeight[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlpha[ i ]  is set equal to the subpicture with subpicture index doi_alpha_partition_id[ i ] from the layer with layer identifier doi_alpha_nuh_layer_id[ i ].

Otherwise (doi_partition_type_flag is equal to 1), the following applies:

–	CodedOverlayWidth[ i ] is set equal to CrRectWidth[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayHeight[ i ] is set equal to CrRectHeight[ doi_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayTexture[ i ] is set equal to the constituent rectangle with cr_rect_id[ j ] equal to doi_partition_id[ i ] from the layer with layer identifier doi_nuh_layer_id[ i ].

–	If doi_alpha_present_flag[ i ] equal to 0

–	CodedOverlayAlphaWidth[ i ] is set equal to CrWRectidth[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlphaHeight[ i ] is set equal to CrRectHeight[ doi_alpha_nuh_layer_id[ i ] ][ doi_partition_id[ i ] ].

–	CodedOverlayAlpha is set equal to the constituent rectangle with cr_rect_id[ j ] equal to doi_alpha_partition_id[ i ] from the layer with layer identifier doi_alpha_nuh_layer_id[ i ].

The variables DisplayOverlayTexture[ i ] and DisplayOverlayAlpha[ i ] are sample arrays with luma resolution DisplayOverlayWidth[ i ] × DisplayOverlayHeight[ i ], derived as follows:

–	If doi_resampling_enabled_flag is equal to 1, the following applies:

–	DisplayOverlayWidth[ i ]  is set equal to doi_width_minus1[ i ] + 1.

–	DisplayOverlayHeight[ i ] is set equal to doi_height_minus1[ i ] + 1.

–	OverlayTexture[ i ] is derived by resampling CodedOverlayTexture[ i ] from a luma resolution of ( CodedOverlayWidth[ i ] × CodedOverlayHeight[ i ] ) to a luma resolution of ( DisplayOverlayWidth[ i ] × DisplayOverlayHeight[ i ] ).

–	OverlayAlpha[ i ] is derived by resampling CodedOverlayAlpha from a luma resolution of ( CodedOverlayAlphaWidth[ i ] × CodedOverlayAlphaHeight[ i ] ) to a luma resolution of ( DisplayOverlayWidth[ i ]  × DisplayOverlayHeight[ i ] )

–	Otherwise (doi_resampling_enabled_flag is equal to 0), the following applies:

–	DisplayOverlayWidth[ i ] is set equal to CodedOverlayWidth[ i ].

–	DisplayOverlayHeight[ i ] is set equal to CodedOverlayHeight[ i ].

– 	OverlayTexture[ i ] is set equal to CodedOverlayTexture[ i ].

– 	OverlayAlpha[ i ] is set equal to CodedOverlayAlpha[ i ].

The variables TargetPicWidth and TargetPicHeight, for the target display picture width and height, respectively, are derived as follows:

–	TargetPicWidth is set equal to DisplayOverlayWidth[ 0 ].

–	TargetPicHeight is set equal to DisplayOverlayHeight[ 0 ].

Let OverlayWithAlpha( tgtPic[ x ][ y ], ovlTex[ w ][ h ], ovlAlp[ w ][ h ] ) be specified as a function that returns the sample values derived by applying the alpha channel using tgtPic[ x ][ y ] as the background sample, ovlTex[ w ][ h ] as the foreground sample, and ovlAlp[ w ][ h ] as the alpha channel sample, using the process specified in subclause 8.23 if an alpha channel information SEI message is present in the CVS, or using a process determined via external means.

A target display picture should be formed as follows, with picture array, TargetPicture[ cIdx ][ x ][ y ] , with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? TargetPicWidth : TargetPicWidth / SubWidthC − 1, y = 0..( cIdx = = 0 ) ? TargetPicHeight : TargetPicHeight / SubHeightC − 1.

for( i = 0; i < doi_num_display_layers_minus2 + 2; i++ )  {
	for( h = 0, y = doi_top_left_y[ i ]; y < DisplayOverlayHeight[ i ]; h++, y++ ) 
		for( w = 0, x = doi_top_left_x[ i ]; x < DisplayOverlayWidth[ i ]; w++, x++ )
			if( !doi_alpha_present_flag[ i ] )
				TargetPicture[ 0 ][ x ][ y ] = OverlayTexture[ i ][ 0 ][ w ][ h ]	
			else
				TargetPicture[ c ][ x ][ y ] = OverlayWithAlpha( TargetPicture[ 0 ][ x ][ y ], 
						OverlayTexture[ 0 ][ i ][ w ][ h ], OverlayAlpha[ i ][ w ][ h ] )

for( ( cIdx = 1; cIdx < ChromaFormatIdc = = 0 ) ? 1 : 3; cIdx++ ++ )  {
		for( h = 0, y = doi_top_left_y[ i ]/SubHeightC; y < DisplayOverlayHeight[ i ]/ SubHeightC; h++, y++ ) 
			for( w = 0, x = doi_top_left_x[ i ]/SubWidthC; x < DisplayOverlayWidth[ i ]/ SubWidthC; w++, x++ )
				if( !doi_alpha_present_flag[ i ] )
					TargetPicture[ cIdx ][ x ][ y ] = OverlayTexture[ i ][ cIdx ][ w ][ h ]					
				else
					TargetPicture[ cIdx ][ x ][ y ] = OverlayWithAlpha( TargetPicture[ cIdx ][ x ][ y ], 
							OverlayTexture[ cIdx ][ i ][ w ][ h ], OverlayAlpha[ i ][ w ][ h ] )

## VVC Semantics

For purposes of interpretation of the constituent rectangles SEI message, the following variables are specified:

–	PicWidthInLumaSamples[ i ] and PicHeightInLumaSamples[ i ] are set equal to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, respectively, of the picture with nuh_layer_id equal to i.

–	ChromaFormatIdc[ i ] is set equal to sps_chroma_format_idc, of the picture with nuh_layer_id equal to i.

–	NumSubpics[ i ] is set equal to sps_num_subpics_minus1 + 1 of the picture with nuh_layer_id equal to i.

–	SubPicWidth[ i ][ j ] is set equal to (sps_subpic_width_minus1[ j ] + 1 ) * CtbSizeY – 1 of the picture with nuh_layer_id equal to i.

–	SubPicHeight[ i ][ j ] is set equal to (sps_subpic_height_minus1[ j ] + 1 ) * CtbSizeY – 1 of the picture with nuh_layer_id equal to i.

It is a requirement of bitstream conformance that there shall be at least one OLS with index olsIdx that has values of OutputLayerIdInOls[ olsIdx ][ j ] equal to doi_nuh_layer_id[ i ] and doi_alpha_nuh_layer_id[ i ], when present, for each value of i in the range of 0 to doi_num_display_overlays_minus2 + 1, inclusive, and for any values of j in the range of 0 to NumOutputLayersInOls[ olsIdx ] − 1, inclusive.

# Comparison with related SEI messages

## HEVC overlay info SEI

HEVC has an overlay info SEI message. This message is not included in VSEI. The HEVC message differs from the proposed SEI message is several respects.

In the HEVC overlay info SEI message, in addition to the primary layer, two or three additional auxiliary layers are coded. The first auxiliary layer contains all overlay elements in the position where they will appear in the final pictures. The second auxiliary layer contains a label map, in which sample values are used to identify particular overlay elements. The third auxiliary layer is an optional alpha channel.

The proposed SEI message uses SEI syntax elements to indicate the position of individual display overlays vs. the use of a label map in the HEVC overlay info SEI. The proposed SEI message provides encoder flexibility to use a single layer or multiple layers, and the option to reposition and/or resample individual display overlay components.

## MPII SEI

The multiplane image information (MPII) SEI message specifies multiplane image (MPI) scene representation information that may be used for view synthesis.

The MPII SEI message enables signaling texture and opacity for multiple layers, with depth signalled per layer. All layers are the same size and are interleaved within a single layer.

The proposed display overlays SEI message does not provide depth information per display overlay, and allows display overlays to vary in size and effective resolution.

# Alternative design to address composition

An alternative design is added to address a composition use case, similar to the functionality proposed in JVET-AI0256. In the syntax and semantic in section 2, the target picture resolution is set equal to the resolution of the 0-th display overlay. In this alternative, the target picture resolution may be explicitly signalled, conditioned on a presence flag. This enables the target picture to be larger than any of the individual display overlays.

The updated syntax table and modification to the semantics is provided below.

## Syntax

Differences from the syntax in section 2.1 are highlighted in yellow.

| display_overlays_info( payloadSize ) { | Descriptor |
| --- | --- |
| doi_id | u(6) |
| doi_cancel_flag | u(1) |
| if( !doi_cancel_flag ) { |  |
| doi_persistence_flag | u(1) |
| doi_num_display_overlays_minus2 | ue(v) |
| doi_target_pic_size_present_flag | u(1) |
| if( doi_target_pic_size_present_flag ) |  |
| doi_target_pic_width_minus1 | u(16) |
| doi_target_pic_height_minus1 | u(16) |
| } |  |
| doi_pic_partition_flag | u(1) |
| if( doi_pic_partition_flag ) { |  |
| doi_partition_type_flag | u(1) |
| doi_partition_id_len_minus1 | u(4) |
| } |  |
| doi_offset_params_present_flag | u(1) |
| if( doi_offset_params_present_flag ) |  |
| doi_offset_param_length_minus1 | u(4) |
| doi_resampling_enabled_flag | u(1) |
| if( doi_resampling_enabled_flag ) |  |
| doi_size_param_length_minus1 | u(4) |
| for( i = 0; i < doi_num_display_layers_minus2 + 2; i++ ) { |  |
| if ( doi_nuh_layer_id_present_flag ) |  |
| doi_nuh_layer_id[ i ] | u(6) |
| if ( doi_pic_partition_flag ) |  |
| doi_partition_id[ i ] | u(v) |
| doi_alpha_present_flag[ i ] | u(1) |
| if ( doi_alpha_present_flag[ i ] ) { |  |
| if (doi_nuh_layer_id_present_flag) |  |
| doi_alpha_nuh_layer_id[ i ] | u(6) |
| if (doi_pic_partition_flag) |  |
| doi_alpha_partition_id[ i ] | u(v) |
| } |  |
| if( i > 0 ) { |  |
| if (doi_offset_params_present_flag ) |  |
| doi_top_left_x[ i ] | u(v) |
| doi_top_left_y[ i ] | u(v) |
| } |  |
| if( doi_resampling_enabling_flag ) { |  |
| doi_width_minus1[ i ] | u(v) |
| doi_height_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Modified syntax

doi_target_pic_size_present_flag equal to 1 specifies that the doi_target_pic_width_minus1 and doi_target_pic_width_minus1 syntax elements are present. doi_target_pic_size_present_flag equal to 0 specifies that the doi_target_pic_width_minus1 and doi_target_pic_width_minus1 syntax elements are present.

doi_target_pic_width_minus1 + 1, when present,  indicates the width of the target picture.

doi_target_pic_height_minus1+ 1, when present,  indicates the height of the target picture.

The variables TargetPicWidth and TargetPicHeight, for the target display picture width and height, respectively, are derived as follows:

–	If doi_target_pic_size_present_flag)

–	TargetPicWidth is set equal to doi_target_pic_width_minus1 + 1

–	TargetPicHeight is set equal to doi_target_pic_height_minus1 + 1

–	Otherwise

–	TargetPicWidth is set equal to DisplayOverlayWidth[ 0 ].

–	TargetPicHeight is set equal to DisplayOverlayHeight[ 0 ].

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).