| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0183-v2 |
| --- | --- |

| Title: | EE2-3.4: MVP extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Jian-Liang Lin, Han Huang, Vadim Seregin, Marta Karczewicz | Email: | zhizhang@qti.qualcomm.com jilin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of EE2-3.4. It proposes to add scaled merge candidates, TMVP candidates with alternative reference indices, and TMVP candidates based on motion trajectory crossing the current block.  The proposed method was implemented on top of ECM-13.0 and the test results are as follows:

Test 3.4a, Additional scaled merge candidates with alternative reference indices:

RA: -0.01% (Y), 0.01% (U), -0.07% (V), 99.5% (EncT), 99.3% (DecT), 99.9% (EncVmPeak), 99.1% (DecVmPeak)

Test 3.4b, Additional TMVP and SbTMVP candidates with alternative reference indices:

RA: -0.06% (Y), -0.10% (U), -0.07% (V), 100.4% (EncT), 100.1% (DecT), 100.1% (EncVmPeak), 102.1% (DecVmPeak)

Test 3.4c, Additional bi-TMVP candidates:

RA: -0.04% (Y), -0.07% (U), -0.05% (V), 99.6% (EncT), 100.3% (DecT), 100.0% (EncVmPeak), 100.2% (DecVmPeak)

Test 3.4d, combination of Test 3.4a, Test 3.4b and Test 3.4c:

RA: -0.11% (Y), -0.08% (U), -0.08% (V), 100.5% (EncT), 100.6% (DecT), 100.2% (EncVmPeak), 103.1% (DecVmPeak)

# Introduction

In ECM-13.0, to derive the reference picture for TMVP and SbTMVP, it determines a reference picture where the POC distance comparing to the current picture is the same or closest compared to the POC distance between the collocated block’s reference picture and the collocated picture. The MV is then scaled accordingly.

# Proposal

This contribution proposes to add additional candidates using a selected reference picture with a scaled MV. For merge candidates, it adds a candidate before the default zero MV candidates with the reference index 1 when the existing candidates in the merge list has reference index 0, otherwise it adds a candidate with the reference index 0.

For TMVP and SbTMVP, it adds a candidate with a reference picture corresponding to the collocated block’s reference picture and the collocated MV is scaled accordingly, if the picture is not in the reference picture list of the current block, it selects a reference picture between the collocated picture and the collocatd reference picture with the largest POC distance to the current picture.

Finally, it adds a bi-TMVP candidate (MV0, MV1 shown in the next figure) after HMVP if a motion trajectory between a block in a reference picture and its reference block crosses the current block. For a given MV shown in red, which is a reference block MV, a pair of MV0 and MV1 is constructed and if it crosses the current block then those MVs are used as bi-TMVP.

The proposed methods are applied to random access pictures when the low delay condition is false.

# Simulation results

Test 3.4a: Additional scaled merge candidates with alternative reference indices

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.06% | -0.13% | 99.4% | 99.6% | 100.1% | 99.6% |
| Class A2 | -0.02% | -0.08% | 0.01% | 100.1% | 99.7% | 99.5% | 99.8% |
| Class B | 0.00% | 0.08% | -0.07% | 99.6% | 99.5% | 99.9% | 99.1% |
| Class C | 0.00% | 0.06% | -0.09% | 99.1% | 98.6% | 100.0% | 98.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | 0.01% | -0.07% | 99.5% | 99.3% | 99.9% | 99.1% |
| Class D | -0.04% | -0.04% | 0.11% | 100.3% | 99.2% | 99.6% | 98.1% |
| Class F | 0.02% | 0.00% | -0.13% | 99.8% | 99.4% | 99.4% | 98.6% |
| Class TGM | -0.07% | -0.06% | -0.06% | 100.0% | 98.9% | 99.8% | 98.8% |

Test 3.4b: Additional TMVP and SbTMVP candidates with alternative reference indices

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.29% | -0.03% | 100.1% | 100.1% | 100.0% | 103.5% |
| Class A2 | -0.07% | -0.12% | -0.02% | 101.0% | 99.9% | 100.1% | 101.7% |
| Class B | -0.07% | -0.03% | -0.18% | 100.8% | 100.0% | 99.9% | 102.8% |
| Class C | -0.07% | -0.02% | -0.02% | 99.8% | 100.3% | 100.2% | 100.7% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.10% | -0.07% | 100.4% | 100.1% | 100.1% | 102.1% |
| Class D | -0.06% | 0.08% | -0.07% | 100.6% | 100.0% | 100.2% | 99.4% |
| Class F | -0.06% | -0.03% | -0.14% | 99.7% | 100.3% | 98.6% | 101.9% |
| Class TGM | -0.05% | -0.01% | -0.02% | 100.5% | 98.8% | 99.5% | 104.1% |

Test 3.4c: Additional bi-TMVP candidates based on motion trajectory crossing the current block

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.05% | -0.06% | 99.4% | 100.9% | 100.3% | 101.3% |
| Class A2 | -0.03% | -0.10% | -0.01% | 99.3% | 100.8% | 99.8% | 101.0% |
| Class B | -0.02% | -0.06% | -0.12% | 100.1% | 100.3% | 100.4% | 100.3% |
| Class C | -0.06% | -0.05% | 0.02% | 99.4% | 99.5% | 99.5% | 98.7% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.07% | -0.05% | 99.6% | 100.3% | 100.0% | 100.2% |
| Class D | -0.08% | -0.15% | 0.19% | 100.7% | 100.1% | 99.8% | 98.2% |
| Class F | 0.01% | 0.04% | -0.05% | 100.0% | 101.1% | 100.1% | 99.2% |
| Class TGM | -0.02% | 0.02% | 0.06% | 100.4% | 101.0% | 99.8% | 100.1% |

Test 3.4d: Combination of Test 3.4a, Test 3.4b and Test 3.4c

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | -0.27% | -0.06% | 100.5% | 100.8% | 100.4% | 105.0% |
| Class A2 | -0.09% | -0.14% | -0.03% | 100.7% | 100.9% | 100.8% | 103.4% |
| Class B | -0.09% | 0.00% | -0.15% | 100.6% | 100.6% | 99.8% | 103.7% |
| Class C | -0.15% | 0.01% | -0.04% | 100.3% | 100.1% | 100.3% | 100.7% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.11% | -0.08% | -0.08% | 100.5% | 100.6% | 100.2% | 103.1% |
| Class D | -0.16% | 0.13% | 0.02% | 99.9% | 100.6% | 100.5% | 99.5% |
| Class F | -0.04% | -0.05% | -0.07% | 99.9% | 101.7% | 99.9% | 102.4% |
| Class TGM | -0.07% | -0.01% | -0.02% | 99.8% | 100.7% | 100.9% | 104.9% |

# References

[1] ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).