| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH2032-v2 |
| --- | --- |

| Title: | Technologies under consideration for future extensions of VSEI (version 4) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Sean McCarthy Jill Boyce Jie Chen Sachin Deshpande Miska M. Hannuksela Hendry | Email: | sean.mccarthy@dolby.com jill.boyce@nokia.com jiechen.cj@alibaba-inc.com sdeshpande@sharplabs.com miska.hannuksela@nokia.com dr.hendry@lge.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains draft text for changes under consideration for future extensions to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7) to modify existing SEI messages, specify additional SEI messages, or specify other functionalities. This document also contains draft text for changes under consideration for future extensions to the VVC, HEVC, and AVC standards  for specifying the use of SEI messages specified in VSEI in VVC, HEVC, and AVC bitstreams, respectively.

This version of the document adds draft text changes for adding the packed regions, constituent rectangles, and quality metrics SEI messages. This version of the document removes the copyright SEI and AI marking SEI messages. Support for the features provided in those SEI messages are now included in the text descriptions SEI message in JVET-AH2006, Additional SEI message for VSEI version 4 (Draft 2).

This version of the document also removes the neural-network post-filter gain characteristics SEI message. Support for the features provided in that SEI message are now included in the Quality metrics SEI message.

This version of the document includes the following two alternatives for handling of processing chains specified by SEI processing order SEI messages:

- Breadth-first: All the pictures (in the entire bitstream, for ease of text specification writing purposes) are processed by a processing stage before moving on to the next processing stage.

- Depth-first: A picture is processed by all the processing stages, before moving on the next picture in output order.

The base text for this version of the document is JVET-AG2032.

Version 2 of this document corrects a missing bracket in the constituent rectangles SEI message.

To be integrated: None

# Changes that have been integrated:

Changes noted below correspond to aspects of JVET-AH0049, JVET-AH0161, JVET-AH0162, JVET-AH0164, JVET-AH0325, and JVET-AH0350.

- Add interface text for AVC and HEVC to support modaility information SEI as proposed in JVET-AH0049

- Add packed regions SEI as proposed in JVET-AH0161

- Add constituent rectangles SEI as proposed in JVET-AH0162

- Add quality metric SEI as proposed in JVET-AH0164

- Remove post-processing filter gain SEI message (features include in quality metric SEI)

- Remove Copyright SEI and AI marking SEI (features included in text description SEI in JVET-AH2006)

- Add alternative persistence signalling as proposed in JVET-AH0325

- Add modification to generative face video SEI in JVET-AH0053

- Document both the breadth-first and depth-first SEI processing order approach of JVET-AH0350 as TuC text specification to determine whether depth-first has some advantage.

- Semantics fixes for GFV SEI message form JVET-AH0349

- Add generative face video enhancement SEI message from JVET-AH0127

- Added gfv_low_confidence_face_parameter_flag to generative face video enhancement SEI message from JVET-AH0054

Summary of JVET-AG0232: changes noted below corresponding to aspects of JVET-AG0044, JVET-AG0045, JVET-AG0051, JVET-AG0086, JVET-AG0087, JVET-AG0203, and JVET-AG0328

- Remove encoder optimization information SEI message (move to JVET-AG2034)

- Add the capability for indicating temporal sublayers where optimization has been applied into the the encoder optimization information SEI message (JVET-AG0086, item 2 option A).

- Remove source picture timing information SEI message (move to JVET-AG2034)

- Remove object mask information SEI message (move to JVET-AG2034)

- Remove modality information in VUI (move to JVET-AG2034 as modality information SEI message (JVET-AG0322))

- Remove NNPFC SEI message application information signaling (move to JVET-AG2034)

- Include copyright SEI message as proposed in JVET-AG0044 proposal 1

- Include AI marking SEI message as proposed in JVET-AG0045 proposal 1

- Include generative face video SEI message

  - as proposed in JVET-AG0203

    - Aspect 1 (gating flag for signalling the translator) (also agreed in context of JVET-AG0087)

    - Aspect 2 (predictive signalling of matrix elements)

    - Aspect 3 (more flexible interface between translator and model in terms of numbers of keypoints and matrix elements)

  - with modifications agreed from JVET-AG0087 as follows.

    - enable the capability of sending parameter translator description only at an IRAP

    - allow multiple GFV SEI messages in a picture unit, e.g., carried within suffix SEI NAL units. Additional ingredients of this item include the following:

      - Each new instance of a GFV SEI message will invoke the generative neural network (NN) inference.

      - To differentiate between repetitions and new instances of GFV SEI messages, add a counter syntax element (gfv_cnt) in the syntax.

- Include film grain regions characteristics SEI message as proposed in JVET-AG0328

- Include versatile SEI RBSP and versatile SEI message as proposed in JVET-AG0051

  - move to a separate section together with LSEI

  - Proponents of both approaches are asked to work out possible combination

# Changes to the specification text:

# Additions to VSEI subclause 2.3 for support of quality metrics SEI

–	ISO/IEC 23001-10 Carriage of timed metadata metrics of media in ISO base media file format

–	ISO/IEC 23001-11 Energy-Efficient Media Consumption (Green Metadata)

# Changes to VSEI subclause 8.28 for adding the support of grouping of NNPFs

8.28 Neural-network post-filter SEI messages

### 8.28.1	General post-processing filtering process using NNPFs

### 8.28.1.1	General

Input to this process is a bitstream BitstreamToFilter. Output of this process is a list of NNPF output pictures ListNnpfOutputPics.

First, BitstreamToFilter is decoded, and the list CroppedDecodedPictures is set to be the list of the cropped decoded pictures in output order resulted from decoding BitstreamToFilter.

Second, NnpfCand is set to contain any single NNPF or any single NNPF group. When NnpfCand contains an NNPF group with nnpfgc_grouping_type equal to 3, the subseequnt specifications of this subclause apply when NnpfCand is set to contain individually each member NNPF, if any, and each member NNPF group, if any, of the NNPF group with nnpfgc_grouping_type equal to 3.

Third, the filtering process for one picture, as specified in subclause 8.28.1.2, is repeatedly invoked, in output order, for each cropped decoded picture that is in CroppedDecodedPictures and for which the single NNPF contained in NnpfCand or the single NNPF group contained in NnpfCand, or one or more NNPFs or NNPF groups defined as alternatives or alternating in the NNPF group contained in NnpfCand are activated.

The order of the pictures in ListNnpfOutputPics is in output order.

Within ListNnpfOutputPics there shall be no more than one picture pertaining to any particular output time instance. [Ed. Check phrasing of this. “Pertains” is not used in a similar way anywhere in the standard.] When for any particular picture in CroppedDecodedPictures there are multiple NNPFs activated and only one of the NNPFs is allowed to be chosen to be applied although any of the NNPFs may be chosen, the above constraint shall apply regardless of which NNPF is chosen to be applied to the particular picture.

BitstreamToFilter may be processed multiple times to generate multiple different ListNnpfOutputPics through the second and third steps above.

### 8.28.1.2	Filtering process for one picture

The filtering process specified in this subclause applies to each cropped decoded picture, referred to as the current picture, that is in CroppedDecodedPictures and for which one or more NNPFs or NNPF groups in NnpfCand are activated.

An NNPF or an NNPF group to be applied to the current picture is selected as follows:

–	If NnpfCand contains a single NNPF and that NNPF is activated for the current picture according to an NNPFA SEI message, that NNPF is selected to be applied to the current picture.

–	Otherwise, if NnpfCand contains an NNPF group with nnpfgc_grouping_type equal to 2 and any NNPF of the NNPF group is activated for the current picture according to NNPFA SEI message, that NNPF is selected to be applied to the current picture.

–	Otherwise, if NnpfCand contains an NNPF group with nnpfgc_grouping_type equal to 0 and that NNPF group is activated for the current picture according to an NNPFGA SEI message, that NNPF group is selected to be applied to the current picture.

–	Otherwise (NnpfCand contains an NNPF group with nnpfgc_grouping_type equal to 1), the following applies:

–	A set of candidate NNPFs or NNPF groups candSet is initially empty and then set to contain the following:

–	The NNPFs that are activated for the current picture according to NNPFA SEI messages and are included in the NNPF group contained in NnpfCand.

–	The NNPF groups that are activated for the current picture according to NNPFGA SEI messages and are included in the NNPF group contained in NnpfCand.

–	For each candidate NNPF or NNPF group candFilter in candSet, the following applies:

–	When one or more of the input pictures of candFilter are input pictures to the NNPF or NNPF group prevFilter that was used in any previous invocation of the filtering process specified in this subclause for the same NnpfCand, candFilter is excluded from candSet.

–	Any NNPF or NNPF group remaining in candSet is selected to be applied to the current picture.

When applying an NNPF to the current picture, the following applies:

–	The filtered and/or interpolated pictures are generated by the NNPF by applying the NNPF process specified in the semantics of the NNPFC SEI message, in a patch-wise manner, to the current picture.

–	The order of the pictures generated by the NNPF by applying the NNPF process being stored into the output tensor of the NNPF is in output order.

–	The pictures generated by the NNPF and output by the NNPF process are included into ListNnpfOutputPics, in the same order as when the pictures are stored into the output tensor of the NNPF.

When applying an NNPF group to the current picture, the following applies:

–	The filtered and/or interpolated pictures are generated by applying the NNPF process specified in the semantics of the NNPFC SEI message, in a patch-wise manner, as specified in the semantics of the NNPFGA SEI message activating the NNPF group.

–	The pictures in FinalNnpfgaOutputPicList are included into ListNnpfOutputPics, in the same order as the pictures are stored in FinalNnpfgaOutputPicList.

    - Neural-network post-filter group characteristics SEI message

      - Neural-network post-filter group characteristics SEI message syntax

| nn_post_filter_group_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfgc_id | ue(v) |
| nnpfgc_grouping_type | ue(v) |
| if( nnpfgc_grouping_type = = 0 ¦ ¦ nnpfgc_grouping_type = = 2 ) |  |
| nnpfgc_purpose | u(16) |
| nnpfgc_num_members_minus2 | ue(v) |
| for( i = 0; i <= nnpfgc_num_members_minus2 + 1; i++ ) { |  |
| nnpfgc_member_id[ i ] | ue(v) |
| if( nnpfgc_grouping_type = = 1 ¦ ¦ nnpfgc_grouping_type = = 3 ) |  |
| nnpfgc_member_purpose[ i ] | u(16) |
| } |  |
| nnpfgc_complexity_info_present_flag | u(1) |
| if( nnpfgc_complexity_info_present_flag ) { |  |
| nnpfgc_parameter_type_idc | u(2) |
| if( nnpfgc_parameter_type_idc != 2 ) |  |
| nnpfgc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfgc_num_parameters_idc | u(6) |
| nnpfgc_num_kmac_operations_idc | ue(v) |
| nnpfgc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |

      - Neural-network post-filter group characteristics SEI message semantics

The neural-network post-filter group characteristics (NNPFGC) SEI message specifies a neural network post-filter (NNPF) group. It is indicated by the SEI message if the NNPF group defines an NNPF cascade or defines NNPFs or NNPF groups of NNPF cascades that are alternatives to each other. The use of NNPF groups of NNPF cascades for specific pictures is indicated with neural-network post-filter group activation (NNPFGA) SEI messages. An NNPF group may have members that are NNPF or NNPF group. An NNPF is a direct member of an NNPF group when it is included in the list of members in the NNPFGC SEI message of the NNPF group. An NNPF is an indirect member of an NNPF group nnpfg_1 when it is a member of another NNPF group nnpfg_2 and nnpfg_2 is a member of nnpfg_1.

nnpfgc_id contains an identifying number that may be used to identify an NNPF group. The value of nnpfgc_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfgc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFGC SEI message with nnpfgc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message. The value of nnpfgc_id shall not be equal to any nnpfc_id value of any NNPFC SEI message present in the same CLVS. When the value of nnpfgc_id of an NNPFGC SEI message nnpfgcSeiA is equal to the value of nnpfgc_id of another NNPFGC SEI message nnpfgcSeiB present in the same CLVS, nnpfgcSeiA and nnpfgcSeiB shall be identical.

nnpfgc_grouping_type equal to 0 indicates that this SEI message specifies a group of cascaded neural-network post-filters.

nnpfgc_grouping_type equal to 1 indicates that the NNPFs or NNPF groups identified by the nnpfgc_member_id[ i ] are alternatives to each other out of which the post-processor should select only one to be applied.

nnpfgc_grouping_type equal to 2 indicates that this SEI message specifies a group of NNPFs that are intended to be used jointly and are activated in an alternating manner so that at most one of these NNPFs is activate for any picture.

nnpfgc_grouping_type equal to 3 indicates that the NNPFs or NNPF groups identified by the nnpfgc_member_id[ i ] are intended to be used in parallel.

nnpfgc_grouping_type equal to 4 indicates that the NNPFs or NNPF groups identified by the nnpfgc_member_id[ i ] are optional, i.e., may or may not be applied by the post-processor.

The value of nnpfgc_grouping_type shall be in the range of 0 to 255, inclusive. Values of nnpfgc_grouping_type in the range of 5 to 255, inclusive, are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFGC SEI messages with nnpfgc_grouping_type in the range of 5 to 255, inclusive.

nnpfgc_purpose has the semantics of nnpfc_purpose but with the exception that the semantics are specified for the NNPF group defined by this SEI message rather than the NNPF defined by an NNPFC SEI message. When present, the value of nnpfgc_purpose shall be the union of the values of nnpfc_purpose of direct and indirect members of the NNPF group.

nnpfgc_num_members_minus2 plus 2 indicates the number of NNPFs or NNPF groups in the NNPF group that this SEI message defines.

nnpfgc_member_id[ i ] indicates the i-th member in the NNPF group defined by this SEI message as follows:

–	If there is an NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] defined in the CLVS, the i-th member in the NNPF group defined by this SEI message is an NNPF that has nnpfc_id equal to nnpfgc_member_id[ i ].

–	Otherwise (there is no NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] defined in the CLVS), the i-th member in the NNPF group defined by this SEI message is an NNPF group with nnpfgc_id equal to nnpfgc_member_id[ i ].

When an nnpfgc_member_id[ i ] value references an nnpfgc_id value of an NNPFGC SEI message nnpfgcSei, it is a requirement of bitstream conformance that the NNPFGC SEI message nnpfgcSei shall have nnpfgc_grouping_type equal to 0.

NOTE 1 – In other words, when a second NNPF group is a member of a first NNPF group, the type of the second NNPF group cannot be any other than a cascade of NNPFs.

When nnpfgc_grouping_type is equal to 0 or 2, it is a requirement of bitstream conformance that there is an NNPF with nnpfc_id value equal to nnpfgc_member_id[ i ] defined in the CLVS.

NOTE 2 – In other words, the members of a cascade or alternating NNPF group are individual NNPFs and cannot be NNPF groups.

When nnpfgc_grouping_type is equal to 1, 3, or 4, it is a requirement of bitstream conformance that there is an NNPF with nnpfc_id value equal to nnpfgc_member_id[ i ] or an NNPF group with nnpfgc_id value equal to nnpfgc_member_id[ i ] defined in the CLVS.

When nnpfgc_grouping_type is equal to 0, the NNPFs with nnpfc_id equal to nnpfgc_member_id[ i ] are performed in cascade in increasing order of i, as activated by an NNPFGA SEI message with nnpfga_target_id equal to nnpfgc_id.

nnpfgc_member_purpose[ i ] has the semantics of nnpfc_purpose but with the exception that the semantics are specified for the NNPFC or NNPFGC associated with nnpfgc_member[ i ]. When present, the value of nnpfgc_member_purpose[ i ] shall be as follows:

–	If nnpfgc_member_id[ i ] is equal to the value of nnpfc_id of an NNPF, the value of nnpfgc_member_purpose[ i ] shall be equal to the value of nnpfc_purpose of the NNPF.

–	Otherwise (nnpfgc_member_id[ i ] is equal to the value of nnpfgc_id of an NNPF group ), the value of nnpfgc_member_purpose[ i ] shall be equal to nnpfgc_purpose of the associatied NNPF group.

nnpfgc_complexity_info_present_flag, nnpfgc_parameter_type_idc, nnpfgc_log2_parameter_bit_length_minus3, nnpfgc_num_parameters_idc, nnpfgc_num_kmac_operations_idc, and nnpfgc_total_kilobyte_size have the semantics of nnpfc_complexity_info_present_flag, nnpfc_parameter_type_idc, nnpfc_log2_parameter_bit_length_minus3, nnpfc_num_parameters_idc, nnpfc_num_kmac_operations_idc, and nnpfc_total_kilobyte_size, respectively, but with the exception that the semantics are specified for the NNPF group defined by this SEI message rather than the NNPF defined by an NNPFC SEI message. When nnpfgc_grouping_type is equal to 1, nnpfgc_complexity_info_present_flag shall be equal to 0.

    - Neural-network post-filter group activation SEI message

      - Neural-network post-filter group activation SEI message syntax

| nn_post_filter_group_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfga_target_id | ue(v) |
| nnpfga_cancel_flag | u(1) |
| if( !nnpfga_cancel_flag ) { |  |
| nnpfga_persistence_flag | u(1) |
| nnpfga_no_prev_clvs_flag | u(1) |
| if( nnpfga_persistence_flag ) |  |
| nnpfga_no_foll_clvs_flag | u(1) |
| nnpfga_num_filters_minus2 | ue(v) |
| for( i = 0; i <= nnpfga_num_filters_minus2 + 1; i++ ) { |  |
| nnpfga_target_base_flag[ i ] | u(1) |
| nnpfga_input_all_pics_flag[ i ] | u(1) |
| if( !nnpfga_input_all_pics_flag[ i ] ) { |  |
| nnpfga_num_input_pics_minus1[ i ] | ue(v) |
| for( j = 0; j <= nnpfga_num_input_pics_minus1[ i ]; j++ ) |  |
| nnpfga_input_pic_skip_count[ i ][ j ] | ue(v) |
| } |  |
| nnpfga_num_output_entries[ i ] | ue(v) |
| for( j = 0; j < nnpfga_num_output_entries[ i ]; j++ ) |  |
| nnpfga_output_flag[ i ][ j ] | u(1) |
| } |  |
| nnpfga_num_output_pic_update | ue(v) |
| for( i = 0; i < nnpfga_num_output_pic_update; i++ ) |  |
| nnpfga_output_pic_update_flag[ i ] | u(1) |
| } |  |
| } |  |

      - Neural-network post-filter group activation SEI message semantics

The neural-network post-filter group activation (NNPFGA) SEI message activates or de-activates the possible use of the target neural-network post-processing filter group (NNPFG) of NNPF groups, identified by nnpfga_target_id, for post-processing filtering of a set of pictures.

nnpfgc_grouping_type for the identfied NNPF group, which has nnpfgc_id equal to nnpfga_target_id, shall be equal to 0 (cascade) or 1 (alternatives). When nnpfgc_grouping_type of the identified NNPF group is equal to 1, each member of the group shall have the same number of input pictures and NNPF output pictures.

For a particular picture for which the NNPFG is activated, the associated NNPFGC SEI message is the last NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id, that precedes the first VCL NAL unit of the current picture in decoding order. If the nnpfgc_grouping_type in the associated NNPGC SEI message is equal to 0, the target NNPFG is the NNPFG specified by the associated NNPFGC SEI message. Otherwise (nnpfgc_grouping_type in the associated NNPFGC SEI message is equal to 1), the target NNPFG is any NNPFG, if any, that is a member of the NNPFG specified by the associated NNPFGC SEI message. The NNPFs of the target NNPFG are defined by the NNPFC SEI messages that have nnpfc_id equal to any nnpfgc_member_id[ i ] value of the target NNPFG and are present in the current picture unit or precede the current picture in decoding order.

NOTE 1 – The members of the target NNPFG are individual NNPFs and cannot be NNPF groups.

Use of this SEI message requires the definition of the following variables:

–	Input picture width and height in units of luma samples, denoted herein by InitCroppedWidth[ idx ] and InitCroppedHeight[ idx ], respectively, of the candidate input pictures with index idx in the range of 0 to numCandInputPics − 1, inclusive, that may be used as input for the NNPFG.

–	Luma sample array InitCroppedYPic[ idx ] and chroma sample arrays InitCroppedCbPic[ idx ] and InitCroppedCrPic[ idx ], when present, of the candidate input pictures with index idx in the range of 0 to numCandInputPics − 1, inclusive, that may be used as input for the NNPFG.

–	Bit depth BitDepthY for the luma sample array of the candidate input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the candidate input pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value array StrengthControlVal[ idx ] that shall contain real numbers in the range of 0 to 1, inclusive, of the candidate input pictures with index idx in the range of 0 to numCandInputPics − 1, inclusive.

Candidate input picture with index 0 corresponds to the picture for which the NNPFG is activated by this NNPFGA SEI message. Candidate input picture with index i in the range of 1 to numCandInputPics − 1, inclusive, precedes the candidate input picture with index i − 1 in output order. Let candInputPicList[ 0 ] be the list of candidate input pictures in inverse output order.

nnpfga_target_id indicates the target NNPFG, which is specified by the NNPFGC SEI message that pertains to the current picture and have nnpfgc_id equal to nnpfga_target_id. [Ed. Check phrasing of this. “Pertains” is not used in a similar way anywhere in the standard.]

The value of nnpfga_target_id shall be in the range of 0 to 232 − 2, inclusive.

An NNPFGA SEI message with a particular value of nnpfga_target_id shall not be present in a current PU unless there is an NNPFGC SEI message with nnpfgc_id equal to the particular value of nnpfga_target_id and nnpfgc_grouping_type equal to 0 present in the current PU or in a PU that precedes the current PU in decoding order within the current CLVS.

When a PU contains both an NNPFGC SEI message with a particular value of nnpfgc_id and an NNPFGA SEI message with nnpfga_target_id equal to the particular value of nnpfgc_id, the NNPFGC SEI message shall precede the NNPFGA SEI message in decoding order.

nnpfga_cancel_flag equal to 1 indicates that the persistence of the target NNPFG established by any previous NNPFGA SEI message with the same nnpfga_target_id as the current SEI message is cancelled, i.e., the target NNPFG is no longer used unless it is activated by another NNPFGA SEI message with the same nnpfga_target_id as the current SEI message and nnpfga_cancel_flag equal to 0. nnpfga_cancel_flag equal to 0 indicates that the target NNPFG is activated for use.

nnpfga_persistence_flag specifies the persistence of the target NNPFG for the current layer.

nnpfga_persistence_flag equal to 0 specifies that the target NNPFG may be used for post-processing filtering for the current picture only.

nnpfga_persistence_flag equal to 1 specifies that the target NNPFG may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFGA SEI message with the same nnpfga_target_id as the current SEI message that follows the current picture in output order.

NOTE 2 – The target NNPFG is not applied for this subsequent picture in the current layer associated with a NNPFGA SEI message with the same nnpfga_target_id as the current SEI message.

Let the nnpfgcTargetPictures be the set of pictures to which the last NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id that precedes the current NNPFGA SEI message in decoding order pertains. [Ed. Check phrasing of this. “Pertains” is not used in a similar way anywhere in the standard.] Let nnpfgaTargetPictures be the set of pictures for which the target NNPFG is activated by the current NNPFGA SEI message. It is a requirement of bitstream conformance that any picture included in nnpfgaTargetPictures shall also be included in nnpfgcTargetPictures.

nnpfga_no_prev_clvs_flag equal to 1 specifies that the input pictures for the NNPF group do not originate from a previous CLVS. nnpfga_no_prev_clvs_flag equal to 0 specifies that the input pictures for the NNPF group may or may not originate from a previous CLVS.

NOTE 3 – The value of nnpfga_no_prev_clvs_flag can be changed from 0 to 1, when the current CLVS is spliced from another bitstream next to the previous CLVS and this NNPFGA SEI message would cause one or more input pictures to be selected from one or more previous CLVSs and therefore is likely to impact the output of the target NNPF group negatively.

nnpfga_no_foll_clvs_flag equal to 1 specifies that when this NNPFGA SEI message persists for the last PU of a CLVS in output order, the NNPFGA SEI message is treated like it persisted for the last PU, in output order, of the current layer within the bitstream. When this NNPFGA SEI message does not persist for the last PU, in output order, of a CLVS in output order or nnpfga_no_foll_clvs_flag is equal to 0, the value of nnpfga_no_foll_clvs_flag causes no specific impact.

NOTE 4 – The value of nnpfga_no_foll_clvs_flag can be  changed from 0 to 1 for an NNPF group including a picture-rate-upsampling NNPF, when the following CLVS is spliced from a different bitstream next to the current CLVS. Consequently, the NNPF process of the picture-rate-upsampling NNPF interpolates pictures up to the end of the current CLVS using input pictures originating from the current CLVS only.

nnpfga_num_filters_minus2 plus 2 indicates the number of NNPFs in the NNPFG that this SEI message activates. The value of nnpfga_num_filters_minus2 shall be equal to the value of nnpfgc_num_members_minus2 in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id.

nnpfga_target_base_flag[ i ] equal to 1 specifies that the i-th NNPF in the target NNPFG is the base NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id. nnpfga_target_base_flag[ i ] equal to 0 specifies that the i-th NNPF in the target NNPFG is the NNPF specified by the last NNPFC SEI message that has nnpfc_id equal to nnpfgc_member_id[ i ] in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id, precedes the first VCL NAL unit of the current picture in decoding order, and is not a repetition of the NNPFC SEI message that contains the base NNPF.

nnpfga_input_all_pics_flag[ i ] equal to 1 specifies that the input pictures to the i-th NNPF are selected from the list of candidate input pictures candInputPicList[ i ] without skipping. nnpfga_input_all_pics_flag[ i ] equal to 0 specifies that the input pictures to the i-th NNPF are selected from the list of candidate input pictures candInputPicList[ i ] in a manner that some candidate input pictures are skipped.

nnpfga_num_input_pics_minus1[ i ] specifies the number of input pictures for the i-th NNPF in the target NNPFG. When present, nnpfga_num_input_pics_minus1[ i ] shall be equal to nnpfc_num_input_pics_minus1 for an NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] of an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id. When not present, nnpfga_num_input_pics_minus1[ i ] is inferred to be equal to nnpfc_num_input_pics_minus1 for an NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id.

nnpfga_input_pic_skip_count[ i ][ j ] specifies a j-th picture count that is skipped in the list of candidate input pictures candInputPicList[ i ] when selecting input pictures for the NNPF activated by the i-th loop entry. When nnpfga_input_pic_skip_count[ i ][ j ] is not present, it is inferred to be equal to 0 for all values of j in the range of 0 to nnpfga_num_input_pics_minus1[ i ], inclusive. The variable numCandInputPics, which indicates the number of candidate input pictures to the NNPFG, is derived as follows:

numCandInputPics = 0
for( j = 0; j  <=  nnpfga_num_input_pics_minus1[ 0 ]; j++ )
	numCandInputPics += 1 + nnpfga_input_pic_skip_count[ 0 ][ j ]	(xx)

Let candInputPicList[ m ] for m in the range of 1 to nnpfga_num_filters_minus2 + 1, inclusive, be a list of pictures in inverse output order that is initially empty and formed in decreasing order of n in the range of 0 to m − 1, inclusive, by including each picture that is output by the NNPF process of the n-th loop entry that has no corresponding picture already present in candInputPicList[ m ], and lastly including each picture present in candInputPicList[ 0 ] that has no corresponding picture already present in candInputPicList[ m ].

When a candidate input picture candInputPicList[ m ][ idx ] for any value of m in the range of 1 to nnpfga_num_filters_minus2 + 1, inclusive, is an NNPF output picture of the n-th NNPF process with the value of n being less than the value of m, the width and height of the candidate input picture are respectively equal to nnpfcOutputPicWidth and nnpfcOutputPicHeight of the NNPF output picture.

The list of input pictures inputPicList[ m ] to the NNPF of the m-th loop entry is derived as follows:

candIdx = 0
for( k = 0; k  <=  nnpfga_num_input_pics_minus1[ m ]; k++ ) {
	candIdx += nnpfga_input_pic_skip_count[ m ][ k ]
	inputPicList[ m ][ k ] = candInputPicList[ m ][ candIdx ]	(xx)
	candIdx++
}

It is a requirement of bitstream conformance that candIdx shall not exceed the number of pictures in candInputPicList[ m ].

It is a requirement of bitstream conformance that the pictures present in inputPicList[ m ], for any value of m in the range of 1 to nnpfga_num_filters_minus2 + 1, inclusive, shall have the same width, height, bit depth, and chroma format.

For purposes of interpretation of the NNPFC SEI message with nnpfc_id equal to nnpfgc_member_id[ i ] in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id, the following variables are specified for the i-th loop entry:

–	The variables BitDepthY, BitDepthC, and ChromaFormatIdc are used as provided for the interpretation of this SEI message.

–	CroppedWidth and CroppedHeight are set equal to the width and height of the pictures in inputPicList[ i ], respectively, in units of luma samples.

–	For each input picture k in the range of 0 to nnpfga_num_input_pics_minus1[ i ], inclusive, the following applies:

–		CroppedYPic[ k ], CroppedCbPic[ k ], and CroppedCrPic[ k ], when present, are set equal to respective sample array of inputPicList[ i ][ k ]

–	When nnpfc_auxiliary_inp_idc is equal to 1 for the NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] in an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id, the following applies:

–	It is a requirement of bitstream conformance that inputPicList[ i ][ k ] is the same as candInputPicList[ 0 ][ idx ] for any value of idx in the range of 0 to numCandInputPics − 1, inclusive.

–	StrengthControlVal[ k ] is set equal to InitStrengthControlVal[ idx ].

nnpfga_num_output_entries[ i ] specifies the number of nnpfga_output_flag[ i ][ j ] syntax elements present in the NNPFGA SEI message. The value of nnpfga_num_output_entries[ i ] shall be in the range of 0 to NumInpPicsInOutputTensor, inclusive, for an NNPF with nnpfc_id equal to nnpfgc_member_id[ i ] of an NNPFGC SEI message with nnpfgc_id equal to nnpfga_target_id..

nnpfga_output_flag[ i ][ j ] equal to 1 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ j ] derived for the i-th NNPF of the target NNPFG is output by the NNPF process activated by this loop entry, where the NNPF process is specified in the semantics of the NNPFC SEI message. nnpfga_output_flag[ i ][ j ] equal to 0 specifies that the NNPF-generated picture that corresponds to the input picture having index InpIdx[ j ] derived for the i-th NNPF of the target NNPFG is not output by the NNPF process activated by this loop entry. When nnpfga_num_output_entries[ i ] is less than NumInpPicsInOutputTensor derived for the i-th NNPF of the target NNPFG, nnpfga_output_flag[ i ][ j ] is inferred to be equal to 1 for each value of i in the range of nnpfga_num_output_entries[ i ] to NumInpPicsInOutputTensor − 1, inclusive.

Let NnpfgaOutputPicList, which is the list of pictures output by NNPF process of the NNPFG in output order, be initially empty and formed in decreasing order of n in the range of 0 to nnpfga_num_filters_minus2 + 1, inclusive, by including each picture that is output by the NNPF process of the n-th loop entry that has no corresponding picture already present in NnpfgaOutputPicList.

nnpfga_num_output_pic_update specifies the number of output picture update flag that is present. When nnpfga_num_output_pic_update is greater than 0, nnpfga_output_pic_update_flag[ i ] for i in the range from 0 to nnpfga_num_output_pic_update − 1, inclusive, are present. The value of nnpfga_num_output_pic_update shall be in the range from 0 to number of pictures in NnpfgaOutputPicList, inclusive.

nnpfga_output_pic_update_flag[ i ] equal to 1 specifies that the i-th picture NnpfgaOutputPicList is included in the final output picture list. nnpfga_output_pic_update_flag[ i ] equal to 0 specifies that the i-th picture NnpfgaOutputPicList is not included in the final output picture list. When the value of nnpfga_num_output_pic_update is less than the number of pictures in NnpfgaOutputPicList, the value of nnpfga_output_pic_update_flag [ i ] for i in the range from nnpfga_num_output_pic_update to the number of pictures in NnpfgaOutputPicList − 1, inclusive, is inferred to be equal to 1.

Let numCandidateOutputPics be the number of pictures in NnpfgaOutputPicList. The list of output picture flag candOutputPicFlag[ i ] for i in the range from 0 to numCandidateOutputPics − 1, inclusive is derived as follows:

for( i = 0; i < numCandidateOutputPics; i++ )
	candOutputPicFlag[ i ] = 1
for( i = 0; i < nnpfga_num_output_pic_update; i++ )	(xx)
	candOutputPicFlag[ i ] = nnpfga_output_pic_update_flag[ i ]

Let FinalNnpfgaOutputPicList, which is the list of pictures output by NNPF process of the NNPFG in output order, be derived as follows:

j = 0
for( i = 0; i < numCandidateOutputPics; i++ )
	if( candOutputPicFlag[ i ] )					(xx)
		FinalNnpfgaOutputPicList[ j++ ] = NnpfgaOutputPicList[ i ]

# Alternative changes to VSEI subclause 8.28 and VVC subclause D.12.11 for adding the support of grouping of PFs (both NNPFs and non-NN PFs)

## 8.28 General post-processing filtering process and neural-network post-filter SEI messages

### 8.28.1	General post-processing filtering process using PPFs

### 8.28.1.1	General

Input to this process is a bitstream BitstreamToFilter. Output of this process is a list of output pictures ListOutputPics.

A post-processing filter (PPF) may be indicated by an SEI message for which the payloadType value is in SeiProcessingOrderSeiList, specified in the semantics of the SEI processing order (SPO) SEI message.

For each picture, there can be multiple PPFs activated and belonging to one or more PPF groups. PPF groups are alternative to each other, i.e., at most one group can be chosen to be applied.

A special PPF cascading case is defined as the case when such two PPFs are both activated for a picture: the two PPFs are both NNPFs (i.e., the payloadType value for the PPFs indicates the NNPFC SEI message), one of the two NNPFs has nnpfc_purpose equal to 4 and the other has multiple input pictures, and neither of the two NNPFs is associated with an SEI processing order SEI message. In this case, the two NNPFs are implicitly considered as belonging to one PPF group, and the NNPF with nnpfc_purpose equal to 4 is applied first.

Except for the special PPF cascading case, each PPF group containing multiple PPFs is associated with an SPO SEI message with a particular value of po_id.

Except for the special PPF cascading case, any PPF not associated with an SPO SEI message is in its own PPF group.

One or more PPFs in the chosen PPF group can be applied. When multiple PPFs (in the chosen PPF group) are applied, they are applied in the cascading manner, meaning that they are applied in the order indicated by the SEI processing order SEI message associated with the chosen PPF group, and for each applied PPF that is not the last applied PPF, the output is used as the input of the next applied PPF.

First, BitstreamToFilter is decoded, the list CroppedDecodedPictures is set to be the list of the cropped decoded pictures in output order resulted from decoding BitstreamToFilter, and the list ListOutputPics is initialized to be the same as CroppedDecodedPictures.

Second, the filtering process for one picture, as specified in subclause 8.28.1.2, is repeatedly invoked, in output order, for each cropped decoded picture that is in CroppedDecodedPictures and for which one or more PPFs of one or more PPF groups are activated and only one of the groups is chosen to be applied.

The order of the pictures in ListOutputPics is in output order.

Within ListOutputPics there shall be no more than one picture pertaining to any particular output time instance. [Ed. Check phrasing of this. “Pertains” is not used in a similar way anywhere in the standard.] When for any particular picture in CroppedDecodedPictures there are PPFs of multiple PPF groups activated, the above constraint shall apply regardless of which group of PPFs is chosen to be applied when the particular picture is the current picture.

For any particular pair of pictures inputPicA and inputPicB consecutive in output order in CroppedDecodedPictures, when there are one or more pictures interpolatedPicSetA in ListOutputPics between inputPicA and inputPicB in output order, the pictures in interpolatedPicSetA shall be among the pictures that were output by applying a particular PPF ppfA when a particular picture currPicA in CroppedDecodedPictures was the current picture. The application of any other PPF that was used in the filtering process for one picture when currPicA was the current picture or the application of any PPF (including ppfA) that was used in the filtering process for one picture when any other picture currPicB in CroppedDecodedPictures was the current picture shall not output any picture between the inputPicA and inputPicB in output order.

NOTE – The intent of the constraints expressed in the above paragraph is to disallow generating PPF output pictures between any particular pair of consecutive input pictures more than once.

### 8.28.1.2	Filtering process for one picture

The filtering process specified in this subclause applies to each cropped decoded picture, referred to as the current picture, that is in CroppedDecodedPictures and for which one or more groups of PPFs are activated, only one of the PPF groups is chosen to be applied, and the number of PPFs (in the chosen PPF group) to be applied is greater than 0.

The filtering process for one picture using one PPF, as specified in subclause 8.28.1.3, is repeatedly invoked for each of the PPFs to be applied. When the number of PPFs to be applied is greater than 1, the following applies:

–	If the special PPF cascading case applies for the chosen PPF group, the NNPF with nnpfc_purpose equal to 4 is applied first, followed by the NNPF with multiple input pictures.

–	Otherwise (the special PPF cascading case does not apply for the chosen PPF group), the PPFs are applied in the preferred order indicated by the SEI processing order SEI message associated with the chosen group of PPFs.

### 8.28.1.3	Filtering process for one picture using one PPF

The filtering process specified in this subclause applies when a particular PPF is applied when a particular picture is the current picture.

Before the PPF is applied, when the PPF is the first PPF to be applied, the list CandInputPicList is set to be identical to CroppedDecodedPictures.

When applying a PPF to the current picture, the input pictures for the PPF are selected from the list CandInputPicList, and the order of the pictures generated and output by the PPF are in output order.

When applying a PPF that is an NNPF to the current picture, the following applies:

–	The filtered and/or interpolated pictures are generated by the NNPF by applying the NNPF process specified in the semantics of the NNPFC SEI message, in a patch-wise manner, to the current picture.

–	The order of the pictures generated by the NNPF by applying the NNPF process being stored into the output tensor of the NNPF is in output order.

After the PPF is applied, the lists CandInputPicList and ListOutputPics are both updated, in the same manner, by 1) replacing each of those pictures in the list having a corresponding PPF output picture of the PPF with the corresponding PPF output picture, and 2) inserting those interpolated pictures, if any, into the list and placing them such that all pictures in the updated list are in output order.

### 8.28.2	Neural-network post-filter characteristics SEI message

### 8.28.2.1	Neural-network post-filter characteristics SEI message syntax

(no change)

### 8.28.2.2	Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified neural-network post-processing filters (NNPFs) for specific pictures is indicated with neural-network post-filter activation (NNPFA) SEI messages.

Use of this SEI message requires the definition of the following variables:

–	The list CandInputPicList that contains a list of pictures in output order from which the input pictures for the NNPF are selected.

NOTE 1 – This list is updated by the PPF filtering process each time when a PPF is applied for the current picture unless the PPF is the last PPF that is applied for the current picture.

–	Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ] and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], when present, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the NNPF.

–	Bit depth BitDepthY for the luma sample array of the input pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

–	Chroma format indicator ChromaFormatIdc, as described in subclause 7.3, of the input pictures.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value array StrengthControlVal[ idx ] that shall contain real numbers in the range of 0 to 1, inclusive, of the input pictures with index idx in the range of 0 to numInputPics − 1, inclusive.

Input picture with index 0 is the picture in CandInputPicList corresponding to the picture for which the NNPF defined by this NNPFC SEI message is activated by an NNPFA SEI message. Input picture with index i in the range of 1 to numInputPics − 1, inclusive, precedes the input picture with index i − 1 in output order.

NOTE 2 – The picture in CandInputPicList corresponding to a picture is either the cropped decoded output picture of that picture or a filtered version of the cropped decoded output picture that was an output picture of a previously applied PPF.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

…

nnpfc_absent_input_pic_zero_flag equal to 1 indicates that the NNPF expects an input picture corresponding to a picture that is not present in the bitstream to be represented by sample arrays with sample values equal to 0. nnpfc_absent_input_pic_zero_flag equal to 0 indicates that the NNPF expects an input picture inputPicA corresponding to a picture that is not present in the bitstream to be represented by the input picture inputPicB that is the closest to inputPicA in output order and the picture corresponding to inputPicB is present in the bitstream.

…

nnpfa_no_prev_clvs_flag equal to 1 specifies that the pictures corresponding to the input pictures for the NNPF do not originate from a previous CLVS. nnpfa_no_prev_clvs_flag equal to 0 specifies that the pictures corresponding to the input pictures for the NNPF may or may not originate from a previous CLVS.

NOTE 4 – The value of nnpfa_no_prev_clvs_flag can be changed from 0 to 1, when the current CLVS is spliced from another bitstream next to the previous CLVS and this NNPFA SEI message would cause one or more input pictures to be selected that have corresponding pictures from one or more previous CLVSs and therefore is likely to impact the output of the target NNPF negatively.

nnpfa_no_foll_clvs_flag equal to 1 specifies that when this NNPFA SEI message persists for the last PU of a CLVS in output order, the NNPFA SEI message is treated like it persisted for the last PU, in output order, of the current layer within the bitstream. When this NNPFA SEI message does not persist for the last PU, in output order, of a CLVS in output order or nnpfa_no_foll_clvs_flag is equal to 0, the value of nnpfa_no_foll_clvs_flag causes no specific impact.

NOTE 5 – The value of nnpfa_no_foll_clvs_flag can be  changed from 0 to 1 for a picture-rate-upsampling NNPF, when the following CLVS is spliced from a different bitstream next to the current CLVS. Consequently, the NNPF process interpolates pictures up to the end of the current CLVS using input pictures corresponding to pictures originating from the current CLVS only.

…

### 8.28.3	Neural-network post-filter activation SEI message

(no change)

### D.12.11	Use of the post-processing filter SEI messages, including the neural network post-filter characteristics SEI message and the neural network post-filter activation SEI message

A post-processing filter (PPF) may be indicated by an SEI message for which the payloadType value is in SeiProcessingOrderSeiList, which is specified in the semantics of the SEI processing order SEI message.

Let currPic be the cropped decoded output picture for which the post-processing filter (PPF), e.g., neural-network post-processing filter (NNPF) defined by the neural-network post-filter characteristics (NNPFC) SEI message, is activated, e.g., by a neural-network post-filter activation (NNPFA) SEI message, and currLayerId be the nuh_layer_id value of currPic.

The list candInputPicList contains a list of pictures in output order from which the input pictures for the PPF are selected.

NOTE 1 – This list is updated by the PPF filtering process each time when a PPF is applied for the current picture unless the PPF is the last PPF that is applied for the current picture.

When the PPF is not an NNPF, the PPF is considered to have only one input picture that is the picture in candInputPicList corresponding to the current picture.

NOTE 2 – The picture in candInputPicList corresponding to a cropped decoded output picture is either the cropped decoded output picture itself or a filtered version of the cropped decoded output picture that was an output picture of a previously applied PPF.

If the PPF is the first PPF that is applied for the current picture, the following applies:

–	CroppedWidth is set equal to the value of pps_pic_width_in_luma_samples − ‌SubWidthC * ‌ ( pps_conf_win_left_offset + pps_conf_win_right_offset ) for currPic.

–	CroppedHeight is set equal to the value of pps_pic_height_in_luma_samples − ‌SubHeightC * ‌ ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) for currPic.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

Otherwise (the PPF is not the first PPF that is applied for the current picture), the following applies:

–	CroppedWidth is set equal to the picture width in units of luma samples of the picture in candInputPicList corresponding to the current picture.

–	CroppedHeight is set equal to the picture height in units of luma samples of the picture in candInputPicList corresponding to the current picture.

–	BitDepthY is set equal to the bit depth BitDepthY for the luma sample array of the picture in candInputPicList corresponding to the current picture.

–	BitDepthC is set equal to the bit depth BitDepthC for the chroma sample arrays, if any, of the picture in candInputPicList corresponding to the current picture.

–	ChromaFormatIdc is set equal to the chroma format indicator ChromaFormatIdc of the the picture in candInputPicList corresponding to the current picture.

The remainder of this subclause applies when the PPF is an NNPF.

It is a requirement of bitstream conformance that when a picture unit contains an NNPFA SEI message, the value of ph_pic_output_flag in the picture header contained in that picture unit shall be equal to 1.

NOTE 3 – Since when the NNPF is the first PPF that is applied for currPic only cropped decoded output pictures are used as input pictures of the NNPF, the value of ph_pic_output_flag in the picture header of the coded picture corresponding to each input picture of the NNPF is equal to 1.

The variable pictureRateUpsamplingFlag is set equal to ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0.

The variable numInputPics is set equal to nnpfc_num_input_pics_minus1 + 1.

The variable numInferences is derived as follows:

–	If all of the following conditions are true, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0 and the variable numInferences is set equal to 1 + numPostRoll:

–	nnpfc_purpose is equal to 8 (i.e., the only purpose for the NNPF is picture rate upsampling).

–	nnpfa_persistence_flag is equal to 1.

–	nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0.

–	Either of the following conditions is true:

–	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

–	currPic is the last picture in the CLVS in output order and nnpfa_no_foll_clvs_flag is equal to 1.

–	Otherwise, if all of the following conditions are true, the variable numPostRoll is set equal to InpIdx[ i ] for the value of i such that nnpfa_output_flag[ i ] is equal to 1, and the variable numInferences is set equal to 1 + numPostRoll:

–	pictureRateUpsamplingFlag is equal to 0.

–	numInputPics is greater than 1.

–	nnpfa_persistence_flag is equal to 1.

–	nnpfa_output_flag[ idx ] is equal to 1 for a single value of idx in the range of 0 to NumInpPicsInOutputTensor − 1, inclusive, and for that single value of idx, InpIdx[ idx ] is greater than 0.

–	Either of the following conditions is true:

–	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

–	currPic is the last picture in the CLVS in output order and nnpfa_no_foll_clvs_flag is equal to 1.

–	Otherwise, the variable numInferences is set equal to 1.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies for the derivation of the input pictures for the NNPF such that each input picture is a picture in candInputPicList:

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] is set to be the picture in candInputPicList corresponding to currPic and inputPresentFlag[ k ] is set equal to 0.

–	The j-th input picture, inputPic[ j ], is set to be the picture in candInputPicList corresponding to currPic and inputPresentFlag[ j ] is set equal to 1.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − 1, inclusive, in increasing order of i:

–	If both of the following conditions are true, inputPic[ i ] is set to be the picture in candInputPicList corresponding to prevPic and inputPresentFlag[ i ] is set equal to 1:

–	Either of the following conditions is true:

–	pictureRateUpsamplingFlag is equal to 1 and currPic is associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId, precede inputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

–	pictureRateUpsamplingFlag is equal to 0 or currPic is not associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order.

–	nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same CLVS.

–	Otherwise, the following applies:

–	inputPic[ i ] is set to be the same picture as inputPic[ i − 1 ] and inputPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that, when pictureRateUpsamplingFlag is equal to 1, nnpfc_interpolated_pics[ i − 1 ] shall be equal to 0.

–	It is a requirement of bitstream conformance that when inputPresentFlag[ i ] is equal to 0 and nnpfc_input_pic_output_flag[ i ] is equal to 1, the value of nnpfa_output_flag[ idx ] shall be equal to 0 for the value of idx such that InpIdx[ idx ] is equal to i.

–	For purposes of interpretation of the NNPFC SEI message, the following variables are specified:

–	The luma sample arrays CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are derived as follows for each value of i in the range of 0 to numInputPics − 1, inclusive:

–	The variable sourcePic is derived as follows:

–	If inputPresentFlag[ i ] is equal to 1 or nnpfc_absent_input_pic_zero_flag is equal to 0, sourcePic is set to be inputPic[ i ].

–	Otherwise (inputPresentFlag[ i ] is equal to 0 and nnpfc_absent_input_pic_zero_flag is equal to 1), sourcePic is set to be a picture with a luma sample array of CroppedWidth × CroppedHeight samples equal to 0 and Cb and Cr sample arrays of ( CroppedWidth / SubWidthC ) × ( CroppedHeight / SubHeightC ) samples equal to 0.

–	The luma sample array CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are set to be the 2-dimensional arrays of decoded sample values of the Y, Cb and Cr components, respectively, of sourcePic.

–	The array StrengthControlVal[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, specifying the filtering strength control value for the input pictures for the NNPF, is derived as follows:

–	StrengthControlVal[ i ] is set equal to the value of ( firstSliceQpY + QpBdOffset ) ÷ ( 63 + QpBdOffset ), where firstSliceQpY is equal to SliceQpY of the first slice of the cropped decoded output picture corresponding to inputPic[ i ].

There shall not be more than two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id. When there are two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id, these SEI messages shall have different content. When two NNPFC SEI messages with the same nnpfc_id and different content are present in the same picture unit, both of these NNPFC SEI messages shall be in the same SEI NAL unit.

# Modify clause 8.30 as indicated with the highlighting:

  - Encoder optimization information SEI message

    - Encoder optimization information SEI message syntax

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_idc | u(2) |
| eoi_for_machine_analysis_idc | u(2) |
| eoi_type | u(16) |
| if( eoi_persistence_flag ) { |  |
| eoi_num_sublayers | u(3) |
| if( eoi_num_sublayers < 7 ) |  |
| eoi_min_tid | u(3) |
| } |  |
| if( EoiObjectBasedFlag ) |  |
| … |  |

    - Encoder optimization information SEI message semantics

…

eoi_persistence_flag specifies the persistence of the optimization information provided in this SEI message. eoi_persistence_flag equal to 0 specifies that the optimization information applies for the current picture only. eoi_persistence_flag equal to 1 specifies that the optimization information applies for the pictures in the temporal sublayers indicated by eoi_num_sublayers and eoi_min_tid, when present, among the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

…

eoi_num_sublayers equal to 7 specifies that this SEI message applies to all temporal sublayers. eoi_num_sublayers less than 7 specifies the number of temporal sublayers to which this SEI message applies. eoi_num_sublayers equal to 0 is reserved.

eoi_min_tid specifies the smallest temporal sublayer identifier value to which this SEI message applies. The value of eoi_min_tid shall be in the range of 0 to 6 − eoi_num_sublayers, inclusive. When eoi_persistence_flag is equal to 1 and eoi_min_tid is not present, eoi_min_tid is inferred to be equal to 0.

When eoi_persistence_flag is equal to 1, the variable eoiMaxSublayer, which is the greatest sublayer identifier value to which this SEI message applies, is derived as follows:

eoiMaxSublayer = eoi_min_tid + eoi_num_sublayers − 1

When eoi_persistence_flag is equal to 1, the semantics of eoi_for_human_viewing_flag and eoi_for_machine_analysis_flag apply to the sublayer representation of eoiMaxSubLayer and the semantics of eoi_type, eoi_object_based_idc (when present), eoi_temporal_resampling_type_flag (when present), and eoi_num_int_pics (when present) apply to the sublayers with a sublayer identifier in the range of eoi_min_tid to eoiMaxSublayer, inclusive.

…

When multiple encoder optimization information SEI messages are present in a particular PU, the following constraints apply for these SEI messages:

–	All of the SEI messages shall have the same value of eoi_cancel_flag.

–	When eoi_persistence_flag is present in the SEI messages, all the SEI messages shall have the same value of eoi_persistence_flag.

–	When any of the SEI messages has eoi_num_sublayers equal to 7, all the SEI messages shall have the same SEI payload content.

–	When an SEI message among the SEI messages has eoi_num_sublayers less than 7, the following constraints apply:

–	All the SEI messages with the same values of eoi_num_sublayers and eoi_min_tid, which are assigned to numSubLayersA and minTidA, respectively, shall have the same SEI payload content.

–	There shall be no such SEI message among the SEI messages for which both of the following are true:

–	eoi_num_sublayers is not equal to numSubLayersA or eoi_min_tid is not equal to minTidA.

–	eoi_min_tid is greater than minTidA and less than minTidA + numSubLayersA.

# Add VSEI subclauses 8.34 to 8.40

8.34 Image format metadata SEI messages

8.34.1 Exif metadata SEI message

8.34.1.1 Exif metadata SEI message syntax

| exif_metadata( payloadSize ) { | Descriptor |
| --- | --- |
| exif_cancel_flag | u(1) |
| if( !exif_cancel_flag ) { |  |
| exif_persistence_flag | u(1) |
| exif_mode_id | u(8) |
| if( exif_mode_id = = 0 ) |  |
| for( i = 1; i < payloadSize; i++ ) |  |
| exif_data_payload_byte | b(8) |
| else if( exif_mode_id = = 1 ) |  |
| exif_data_uri | st(v) |
| } |  |
| } |  |

8.34.1.2 Exif metadata SEI message semantics

The Exchangeable Image File (Exif) Format for digital still cameras includes a set of metadata that captures information regarding the digital photography process that was used to record an image. Such metadata includes: the acceleration vector of the camera at the time the image was captured, camera lens information, GPS data, the color space used, spectral sensitivity, maximum lens aperture, and more.

Exif metadata is specified by any of the Exchangeable image file (Exif) format for digital still cameras standards developed jointly by the Camera & Imaging Products Association (CIPA) and the Japan Electronics and Information Technology Industries Association (JEITA), for the associated video source pictures prior to encoding, e.g., for camera-captured content. To date, there are multple versions of Exif deployed, each with its own indicator to signal the version number in use.

[Ed. Note: Neither CIPA nor JEITA are listed in the organizations for which normative references may be made to documents published by CIPA or JEITA. The list of such organizations is available at: https://www.itu.int/net4/ITU-T/lists/sdo.aspx. To add either CIPA or JEITA to the list, the A.4 process needs to be executed for one or both organizations, and their status as A.5 organizations needs to be approved by SG16. There would probably also need to be a specific list of documents referenced rather than the concept of “any of the” standards produced by those organizations.]

The Exif metadata SEI message specifies an SEI message for which the payload of Exif metadata can be carried in the video bitstream.

exif_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous Exif metadata SEI message in output order. exif_cancel_flag equal to 0 indicates that Exif metadata information follows.

exif_persistence_flag specifies the persistence of the Exif metadata SEI message for the current layer.

exif_persistence_flag equal to 0 specifies that the Exif metadata SEI message applies to the current decoded picture only.

exif_persistence_flag equal to 1 specifies that the Exif metadata SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an Exif metadata SEI message is output that follows the current picture in output order.

exif_mode_id specifies the mode by which the Exif metadata is obtained.

–	If exif_mode_id is equal to 0, the Exif metadata is obtained directly from the payload of the SEI message.

–	Otherwise if exif_mode_id is equal to 1, the Exif metadata is obtained from a URI with syntax and semantics as specified in IETF Internet Standard 66.

All other values of exif_mode_id are reserved.

exif_data_payload_byte shall be a byte containing data having syntax and semantics as specified by any of the Exchangeable image file format for digital still cameras standards developed jointly by the Camera & Imaging Products Association (CIPA) and the Japan Electronics and Information Technology Industries Association (JEITA).

exif_data_uri shall contain a URI with syntax and semantics as specified in IETF Internet Standard 66 identifying the Exif metadata.

8.34.2 JFIF metadata SEI message

8.34.2.1 JFIF metadata SEI message syntax

| jfif_metadata( payloadSize ) { | Descriptor |
| --- | --- |
| jfif_cancel_flag | u(1) |
| jfif_type_id | u(8) |
| if( !jfif_cancel_flag ) { |  |
| jfif_persistence_flag | u(1) |
| if( jfif_type_id = = 0 ) |  |
| for( i = 1; i < payloadSize; i++ ) |  |
| jfif_data_payload_byte | b(8) |
| else if( jfif_type_id = = 1 ) |  |
| for( i = 1; i < payloadSize; i++ ) |  |
| jfif_extension_payload_byte | b(8) |
| else if( jfif_type_id = = 2 ) |  |
| for( i = 1; i < payloadSize; i++ ) |  |
| jfif_header_payload_byte | b(8) |
| } |  |
| } |  |

[Ed. (GJS): Are some of those grid lines thicker than others?]

8.34.2.2 JFIF metadata SEI message semantics

The JFIF metadata SEI message contains JFIF metadata, as specified by ITU-T Recommendation T.871 | ISO/IEC International Standard 10918-5.

JFIF metadata and its semantics are specified by ITU-T Recommendation T.871 | ISO/IEC International Standard 10918-5 (hereafter, the JFIF standard). Of particular importance is the distinction by JFIF between an APP0 marker that contains information describing the organization of the image data, and an APP0 marker that contains “extension” information. [Ed. Note: Needs formal reference (in clause 2). Does that document define what an “APP0 marker” is?]

A single APP0 marker that carries the information to describe the organization of the image is signalled by the zero-terminated string “JFIF” (‘0x4A46494600’) within that APP0 marker. One or more subsequent APP0 markers may follow the first APP0 marker in which the subsequent APP0 marker(s) carries “extension” information. In particular, the JFIF extension mechanism can be used to carry image thumbnails, although there is no specification by the JFIF standard for the precise type of data carried in JFIF extensions.

Approaches for carriage of JFIF within an SEI message include:

–	The JFIF payload is carried entirely as a single “container” within an SEI message. In this approach, the SEI payload includes the payloads from the first and all, if any, subsequent JFIF APP0 markers for an image.

–	The JFIF payload from only extension APP0 markers, i.e., all JFIF APP0 markers that appear subsequent to the first APP0 marker, is carried as a single container. This approach facilitates the carriage of thumbnail images without the overhead of the carriage of the first JFIF APP0 marker.

–	The JFIF payload from only the first JFIF APP0 marker is carried as a single container.

jfif_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous JFIF metadata SEI message in output order. jfif_cancel_flag equal to 0 indicates that JFIF metadata information follows.

jfif_persistence_flag specifies the persistence of the JFIF metadata SEI message for the current layer.

jfif_persistence_flag equal to 0 specifies that the JFIF metadata SEI message applies to the current decoded picture only.

jfif_persistence_flag equal to 1 specifies that the JFIF metadata SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an JFIF metadata SEI message is output that follows the current picture in output order.

jfif_type_id specifies the type of JFIF metadata.

–	If jfif_type_id is equal to 0, the remaining SEI payload bytes shall contain JFIF metadata from one or more concatenated APP0 marker segments without the APP0 marker segment identifiers themselves, i.e., an APP0 marker segment with  bytes containing 0x4A46494600, i.e, the zero-terminated string "JFIF", according to Rec. ITU-T T.50 or ISO 646 coding, and one or more APP0 marker segments containing 0x4A46585800, i.e,  the zero-terminated string "JFXX", according to Rec. ITU-T T.50 or ISO 646 coding.

–	Otherwise if jfif_type_id is equal to 1, the remaining SEI payload bytes shall contain bytes from one or more JFIF extension markers, i.e., one or more APP0 markers that include 0x4A46585800, i.e,  the zero-terminated string "JFXX", according to Rec. ITU-T T.50 or ISO 646 coding.

–	Otherwise if jfif_type_id is equal to 2, the remaining SEI payload bytes shall contain bytes obtained from the first APP0 marker following the 0xFFD8 Start of Image marker within the image. The first APP0 marker includes 0x4A46494600, i.e,  the zero-terminated string "JFIF", according to Rec. ITU-T T.50 or ISO 646 coding.

All other values of jfif_type_id are reserved.

jfif_data_payload_byte shall be a byte containing data from one or more concatenated APP0 JPEG marker segments without the bytes 0xFFE0 from each APP0 marker segment identifier, each such marker segment having syntax and semantics specified by ITU-T Recommendation T.871 | ISO/IEC International Standard 10918-5.

jfif_extension_payload_byte shall be a byte containing data from one or more concatenated APP0 JPEG marker segments, without the bytes 0xFFE0 from each APP0 marker segment identifier, each such marker segment having syntax and semantics of a JFIF extension APP0 marker segment specified by ITU-T Recommendation T.871 | ISO/IEC International Standard 10918-5.

jfif_header_payload_byte shall be a byte containing data from the first APP0 JPEG marker segment  following the 0xFFD8 Start of Image marker segment, without the bytes 0xFFE0 from the APP0 marker segment identifier, such marker segment having syntax and semantics of the first APP0 marker segment immediately following the 0xFFD8 Start of Image marker segment specified by ITU-T Recommendation T.871 | ISO/IEC International Standard 10918-5.

8.34.3 XMP metadata SEI message

8.34.3.1 XMP metadata SEI message syntax

| xmp_metadata( payloadSize ) { | Descriptor |
| --- | --- |
| xmp_cancel_flag | u(1) |
| if( !xmp_cancel_flag ) { |  |
| xmp_persistence_flag | u(1) |
| for( i = 1; i < payloadSize; i++ ) |  |
| xmp_data_payload_byte | b(8) |
| } |  |
| } |  |

8.34.3.1 XMP metadata SEI message semantics

The XMP metadata SEI message specifies an SEI message for XMP metadata.

Extensible Metadata Platform (XMP) metadata is specified in ISO 16684-1: Graphic Technology – Extensible metadata platform (XMP) specification. [Ed. Note (GJS): Formal reference needed in clause 2.] XMP is widely deployed by digital cameras and digital image editing packages to record provenance and editing history with an image. The most common set of metadata included in XMP refers to a vocabulary defined by the Dublin Core Metadata Initiative (DCMI), e.g., to store information such as digital rights ownership and names of software packages used to modify the image.

The XMP metadata SEI message indicates the XMP metadata, as specified by [1] in XML or [3] in JSON. [Ed. Note: Fix or remove unresolved references and undefined abbreviations.]

xmp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous XMP metadata SEI message in output order. xmp_cancel_flag equal to 0 indicates that XMP metadata information follows.

xmp_persistence_flag specifies the persistence of the XMP metadata SEI message for the current layer.

xmp_persistence_flag equal to 0 specifies that the XMP metadata SEI message applies to the current decoded picture only.

xmp_persistence_flag equal to 1 specifies that the XMP metadata SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an XMP metadata SEI message is output that follows the current picture in output order.

xmp_data_payload_byte shall be a byte containing data having syntax and semantics as specified by ISO 16684-1: Graphic Technology – Extensible metadata platform (XMP) specification – Part 1 Data model, serialization and core properties or ISO 16684-3: Graphic Technology – Extensible metadata platform (XMP) specification – Part 3 JSON-LD serialization of XMP.

8.35 Multiplane image information SEI message

8.35.1 Multiplane image information SEI message syntax

| multiplane_image_information( payloadSize ) { | Descriptor |
| --- | --- |
| mpii_num_layers_minus1 | ue(v) |
| mpii_layer_depth_equal_distance_flag | u(1) |
| if( mpii_layer_depth_equal_distance_flag ) { |  |
| depth_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| depth_rep_info_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| } else |  |
| for( i = 0; i <= mpii_num_layer_minus1; i++ ) |  |
| depth_rep_info_element( ZSign[ i ], ZExp[ i ], ZMantissa[ i ], ZManLen[ i ] ) |  |
| mpii_texture_opacity_interleave_flag | u(1) |
| if( mpii_texture_opacity_interleave_flag = = 0 ) |  |
| mpii_texture_opacity_arrangement_flag /* 0: Top-and-Bottom, 1: Side-by-Side */ | u(1) |
| mpii_picture_num_layers_in_height_minus1 | ue(v) |
| } |  |

8.35.2 Muliplane image information SEI message semantics

The multiplane image information (MPII) SEI message specifies multiplane image (MPI) scene representation information that may be used for view synthesis.

When an MPII SEI message is present in any AU of a CLVS, an MPII SEI message shall be present in the first AU of the CLVS and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an MPII SEI message is output that follows the current picture in output order.

NOTE 1 – This SEI message can work together with the multiview acquisition information SEI message for view synthesis. The multiview acquisition information SEI message specifies the intrinsic and extrinsic parameters for current camera view. When multiple views are available, the reconstructed novel views can be rendered from multiplane images of nearby views.

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	A cropped decoded picture array decPicCurr0[ cIdx ][ x ][ y ], with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? CroppedWidth : CroppedWidth / SubWidthC − 1, y = 0..( cIdx = = 0 ) ? CroppedHeight : CroppedHeight / SubHeightC − 1.

–	In output order a temporally following cropped decoded picture array decPicCurr1[ cIdx ][ x ][ y ], with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? CroppedWidth : CroppedWidth / SubWidthC − 1, y = 0..( cIdx = = 0 ) ? CroppedHeight : CroppedHeight / SubHeightC − 1.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

mpii_num_layers_minus1 plus 1 specifies the number of texture and opacity layers for the MPI representation.

mpii_layer_depth_equal_distance_flag equal to 1 indicates that equal distances are used to generate MPI layers and depth parameters for each layer. In this case, Z[ i ] can be derived using the nearest depth value ZNear and the farthest depth value ZFar.

The depth value for i-th MPI layer, Z[ i ], is derived as follows:

Z[ i ] = i * ( ZFar − Znear ) ÷ (mpi_num_layers_minus1 ) + ZNear	(xx)

mpii_layer_depth_equal_distance_flag equal to 0 indicates that the depth information for each layer follows next in the SEI message.

The variables in the x column of Table xx are derived from the respective variables in the s, e, n and v columns of Table xx as follows:

–	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( − 1)s * 2( e − 31 ) * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x is set equal to ( − 1)s * 2−( 30 + v ) * n.

NOTE 2 – The above specification is similar to that found in IEC 60559:1989.

Table xx – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| Z[ i ] | ZSign[ i ] | ZExp | ZMantissa | ZManLen |

NOTE 3 – In some applications, disparity is used instead of depth (the disparity value D and depth value Z relationship is D = 1 ÷ Z). Correspoding to Equation (xx), the disparity value for the i-th MPI layer is D[ i ] = i * ( DFar − Dnear ) ÷ (mpi_num_layers_minus1 ) + DNear.

mpii_texture_opacity_interleave_flag equal to 1 indicates decoded output pictures correspond to temporally interleaved texture and opacity constituent pictures in output order as illustrated in Figure XX. mpii_texture_opacity_interleave_flag equal to 0 indicates decoded output pictures correspond to spatially packed texture and opacity constituent pictures as illustrated in Figures XX and XX.

mpii_texture_opacity_arrangement_flag equal to 0 indicates decoded output pictures represent texture and opacity constituent pictures in a top-bottom packing arrangement as illustrated in Figure XX. mpii_texture_opacity_arrangement_flag equal to 1 indicates decoded output pictures represent texture and opacity constituent pictures in a side-by-side packing arrangement as illustrated in Figure XX.

For each specified picture packing arrangement scheme, there are two constituent pictures that are referred to as picture 0 and picture 1. When mpii_texture_opacity_interleave_flag is equal to 0, the constituent picture associated with the upper-left sample of the decoded picture is considered to be constituent picture 0 and the other constituent picture is considered to be constituent picture 1. When mpii_texture_opacity_interleave_flag is equal to 1, the first decoded picture in the current CLVS is constituent picture 0 and the next decoded picture in output order is constituent picture 1 and the display time of the constituent picture 0 should be delayed to coincide with the display time of constituent picture 1. The two constituent pictures form the spatially packed texture and opacity picture of a MPI, with picture 0 being associated with the spatially packed texture picture and picture 1 being associated with the spatially packed opacity picture.

mpii_picture_num_layers_in_height_minus1 plus 1 specifies the number of spatially packed layers in height for picture 0 and picture 1. The variable hLayers is set equal to mpii_picture_num_layers_in_height_minus1 + 1 and the variable wLayers is set equal to (mpii_num_layers_minus1 + 1) / hLayers.

Let variable fWidth and fHeight specify the width and height of picture 0 and picture 1, respectively, and are derived as follows:

–	If mpii_texture_opacity_interleave_flag is equal to 1, the following applies:

fWidth = CroppedWidth

fHeight = CroppedHeight

–	Otherwise (mpii_texture_opacity_interleave_flag is equal to 0)

–	If mpii_texture_opacity_arrangement_flag is equal to 0, the following applies:

fWidth = CroppedWidth , fHeight = CroppedHeight / 2

–	Otherwise (mpii_texture_opacity_arrangement_flag is equal to 1), the following applies:

fWidth = CroppedWidth / 2 , fHeight = CroppedHeight

Let variable cWidth = fWidth / subWidthC and variable cHeight = fHeight / subHeightC.

Let array picture0[ cIdx ][ x ][ y ] specify samples in picture 0 and array picture1[ cIdx ][ x ][ y ] specify samples in picture 1, with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..( cIdx = = 0 ) ? fWidth: cWidth − 1, y = 0..( cIdx = = 0 ) ? fHeight : cHeight − 1 and are derived as follows:

–	If mpii_texture_opacity_interleave_flag is equal to 1, the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr1[ cIdx ][ x ][ y ]

–	Otherwise (mpii_texture_opacity_interleave_flag is equal to 0)

–	Let variable cW = ( cIdx = = 0 )? fWidth : cWidth

–	Let variable cH = ( cIdx = = 0 )? fHeight : cHeight

–	If mpii_texture_opacity_arrangement_flag is equal to 0, the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y + cH ]

–	Otherwise (mpii_texture_opacity_arrangement_flag is equal to 1), the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x + cW ][ y ]

Let variable layerWidth and layerHeight specify the width and height for decoded MPI layer, respectively. The variables are derived as follows:

layerWidth = fWidth / wLayers

layerHeight = fHeight / hLayers

The reconstruction of MPI process is described as follows:

The outputs of this process are:

–	a 4D MPI texture layer array recTextureLayer[ i ][ cIdx ][ w ][ h ] with i = 0..mpii_num_layers_minus1, cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, w = 0..( cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC − 1, and h = 0..( cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC − 1.

–	a 3D MPI opacity layer array recOpacityLayer[ i ][ w ][ h ] with i = 0..mpii_num_layers_minus1, x = 0..layerWidth − 1, and y = 0..layerHeight − 1.

The array recTextureLayer and array recOpacityLayer are derived as follows:

for( i = 0; i  <=  mpii_num_layers_minus1; i++ ) {
	k = i % wLayers
	m = ( i − k ) / hLayers
	for( cIdx = 0; cIdx < ChromaFormatIdc = = 0 ) ? 1 : 3; cIdx++ )
		for( h = 0; h < ( cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC ; h++ )
			for( w = 0; w < ( cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC ; w++ ) {
				u = k * ( cIdx  = =  0 ) ? layerWidth : layerWidth / SubWidthC + w
				v = m * ( cIdx  = =  0 ) ? layerHeight : layerHeight / SubHeightC + h
				recTextureLayer[ i ][ cIdx ][ w ][ h ] = picture0[ cIdx ][ u ][ v ]
			}
	for( h = 0; h < layerHeight; h++ )
		for( w = 0; w < layerWidth; w++ )
			recOpacityLayer[ i ][ w ][ h ] = picture1[ 0 ][ k * layerWidth + w ][ m * layerHeight + h ]
}

8.36 Generative face video SEI message

8.36.1 Generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| if( gfv_cnt = = 0 ) |  |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| gfv_nn_present_flag | u(1) |
| if( gfv_nn_present_flag ) { |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to GenerativeNN( )*/ | ue(v) |
| gfv_low_confidence_face_parameter_flag | u(1) |
| gfv_coordinate_present_flag | u(1) |
| if( gfv_coordinate_present_flag ) { |  |
| gfv_coordinate_precision_factor_minus1 | ue(v) |
| gfv_num_kps_minus1 | ue(v) |
| gfv_kps_pred_flag | u(1) |
| gfv_coordinate_z_present_flag | u(1) |
| if(gfv_coordinate_z_present_flag ) |  |
| gfv_coordinate_z_max_value_minus1 | ue(v) |
| for( i = 0; i <= gfv_num_kps_minus1; i++ ) { |  |
| if(!gfv_kps_pred_flag) { |  |
| gfv_coordinate_x_abs[ i ] | u(v) |
| if( gfv_coordinate_x_abs[ i ] ) |  |
| gfv_coordinate_x_sign_flag[ i ] | u(1) |
| gfv_coordinate_y_abs[ i ] | u(v) |
| if( gfv_coordinate_y_abs[ i ] ) |  |
| gfv_coordinate_y_sign_flag[ i ] | u(1) |
| if( gfv_coordinate_z_present_flag ) { |  |
| gfv_coordinate_z_abs[ i ] | u(v) |
| if( gfv_coordinate_z_abs[ i ] ) |  |
| gfv_coordinate_z_sign_flag[ i ] | u(1) |
| } |  |
| } else { |  |
| gfv_coordinate_dx_abs[ i ] | u(v) |
| if(gfv_coordinate_dx_abs[ i ] ) |  |
| gfv_coordinate_dx_sign_flag[ i ] | u(1) |
| gfv_coordinate_dy_abs[ i ] | u(v) |
| if( gfv_coordinate_dy_abs[ i ] ) |  |
| gfv_coordinate_dy_sign_flag[ i ] | u(1) |
| if( gfv_coordinate_z_present_flag ) { |  |
| gfv_coordinate_dz_abs[ i ] | u(v) |
| if( gfv_coordinate_dz_abs[ i ] ) |  |
| gfv_coordinate_dz_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| gfv_matrix_present_flag | u(1) |
| if(gfv_matrix_present_flag ) { |  |
| gfv_matrix_element_precision_factor_minus1 | ue(v) |
| gfv_num_matrix_types_minus1 | ue(v) |
| if( !gfv_base_pic_flag ) |  |
| gfv_matrix_pred_flag | u(1) |
| for( i = 0; i <= num_matrix_types_minus1; i++ ) { |  |
| gfv_matrix_type_idx[ i ] | u(6) |
| if( gfv_matrix_type_idx[ i ] = = 0 ¦ ¦ gfv_matrix_type_idx[ i ] = = 1 ) { |  |
| if( gfv_coordinate_present_flag ) |  |
| gfv_num_matrices_equal_to_num_kps_flag[ i ] | u(1) |
| if(!gfv_num_matrices_equal_to_num_kps_flag[ i ] ) |  |
| gfv_num_matrices_info[ i ] | ue(v) |
| }else if( gfv_matrix_type_idx[ i ] = = 2 ¦ ¦ gfv_matrix_type_idx[ i ] = = 3 ¦ ¦ gfv_matrix_type_idx[ i ] >= 7 ){ |  |
| if( gfv_matrix_type_idx[ i ] >= 7 ) |  |
| gfv_num_matrices_minus1[ i ] | ue(v) |
| gfv_matrix_width_minus1[ i ] | ue(v) |
| gfv_matrix_height_minus1[ i ] | ue(v) |
| }else if( gfv_matrix_type_idx[ i ] >= 4 && gfv_matrix_type_idx[ i ] <= 6 && !gfv_coordinate_present_flag ){ |  |
| gfv_matrix_for_3D_space_flag[ i ] | u(1) |
| for( j = 0; j < numMatrices[ i ]; j++ ) |  |
| for( k = 0; k < matrixHeight[ i ]; k++ ) |  |
| for( m = 0; m <matrixWidth[ i ]; m++ ) { |  |
| if( !gfv_matrix_pred_flag ) { |  |
| gfv_matrix_element_int[ i ][ j ][ k ][ m ] | ue(v) |
| gfv_matrix_element_dec[ i ][ j ][ k ][ m ] | u (v) |
| if( gfv_matrix_element_int[ i][ j ][ k ][ m ] ¦ ¦ gfv_matrix_element_dec[ i ][ j ][ k ][ m ] ) |  |
| gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ] | u(1) |
| } |  |
| else { |  |
| gfv_matrix_delta_element_int[ i ][ j ][ k ][ m ] | ue(v) |
| gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ] | u (v) |
| if( gfv_matrix_delta_element_int[ i][ j ][ k ][ m ] ¦ ¦ gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ] ) |  |
| gfv_matrix_delta_element_sign_flag[ i ][ j ][ k ][ m ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( gfv_nn_present_flag ) |  |
| if( gfv_nn_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| gfv_nn_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

8.36.2 Generative face video SEI message semantics

The generative face video (GFV) SEI message indicates facial parameters and specifies a facial parameter translator network, denoted as TranslatorNN( ), that may be used to convert various formats of facial parameters signaled in the SEI message into a fixed format of parameters, and a face picture generator neural network, denoted as GenerativeNN( ), that may be used to generate output pictures using the fixed format of facial parameters and previously decoded output pictures.

NOTE 1 – Facial parameters could be determined from source pictures prior to encoding. Such source pictures may be referred to as driving pictures.

NOTE 2 – Previously decoded output pictures input to GenerativeNN( ) may be a base picture (a decoded output picture that provides the reference texture from which the face pictures may be generated) and, optionally, a picture that can be fused by GenerativeNN( ) to improve background texture and facial details. When the current picture is not a base picture, the GFV SEI message may be used to generate a face picture based on the previously decoded base picture, the facial parameters conveyed by the GFV SEI message, and, optionally, the current decoded picture for fusion purpose.

Use of this SEI message requires the definition of the following variables:

- Input and output picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

- Luma sample array baseCroppedYPic and chroma sample arrays baseCroppedCbPic and baseCroppedCrPic for a decoded output picture, denoted as BasePicture, corresponding to a source base picture.

- Luma sample array driveCroppedYPic and chroma sample arrays driveCroppedCbPic and driveCroppedCrPic for a decoded output picture, denoted as DrivePicture, corresponding to a source driving picture.

- Bit depth BitDepthY for the luma sample array of the input and output pictures.

- Bit depth BitDepthC for the chroma sample arrays, if any, of the input and output pictures.

- A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

gfv_id contains an identifying number that may be used to identify face feature information and specify a neural network that may be used as TranslatorNN( ). The value of gfv_id shall be in the range of 0 to 232 − 2, inclusive. Values of gfv_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering a GFV SEI message with gfv_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

gfv_cnt specifies a GFV SEI message instance count value for this gfv_id value within a picture unit.

The gfv_cnt of the first GFV SEI message, in decoding order, with a particular value of gfv_id within a picture unit shall be equal to 0. When gfv_cnt assigned to currGfvCnt is greater than 0, a GFV SEI message with the same gfv_id value and gfv_cnt equal to currGfvCnt − 1 shall be present in the same picture unit and precede the current GFV SEI message in decoding order.

The value of gfv_cnt shall be in the range of 0 to 65 535, inclusive.

gfv_base_pic_flag equal to 1 indicates that the current decoded output picture corresponds to a base picture. gfv_base_pic_flag equal to 0 indicates that the current decoded output picture does not correspond to a base picture or this SEI message does not specify syntax elements for a base picture. When gfv_base_pic_flag is not present, it is inferred to be equal to 0.

The following constraints apply to the value of gfv_base_pic_flag:

–	When a GFV SEI message is the first GFV SEI message, in decoding order, that has a particular gfv_id value within the current CLVS, the value of gfv_base_pic_flag shall be equal to 1.

–	When a GFV SEI message that has a particular gfv_id value has gfv_base_pic_flag being equal to 0, this SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent GFV SEI message, in decoding order, having gfv_base_pic_flag equal to 0 and that particular gfv_id value within the current CLVS, whichever is earlier.

gfv_nn_present_flag equal to 1 indicates that a neural network that may be used as a TranslatorNN( ) is contained or indicated by the SEI message. gfv_nn_present_flag equal to 0 indicates that a neural network that may be used as a TranslatorNN( ) is not contained or indicated by the SEI message. When gfv_nn_present_flag is not present, it is inferred to be 0.

When a GFV SEI message with a particular value of gfv_id is present in an IRAP picture unit or in a picture unit that follows an IRAP picture unit in output order and is not preceded in output order by any picture unit that follows the IRAP picture unit in output order and has a GFV SEI message with that particular value of gfv_id, gfv_nn_present_flag shall be present and equal to 1.

When gfv_nn_present_flag is equal to 0 and TranslatorNN is referenced in the semantics of this SEI message, the applicable TranslatorNN is defined by the GFV SEI message that is present in the last preceding picture unit in output order that has the same value of gfv_id as that in the current GFV SEI message and gfv_nn_present_flag equal to 1.

gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] specify a neural network that may be used as a TranslatorNN( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] have the same syntax and semantics as nnpfc_base_flag, nnpfc_mode_idc, nnpfc_reserved_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_payload_byte[ i ], respectively.

When either of the following conditions is true, GFV SEI messages shall have the same SEI payload content:

–	The GFV SEI messages are present in the same picture unit, have gfv_cnt equal to 0, have gfv_nn_base_flag present, and have the same value of gfv_id and gfv_nn_base_flag,

–	The GFV SEI messages are present in the same picture unit, have the same value of gfv_cnt that is greater than 0, and have the same value of gfv_id.

gfv_drive_pic_fusion_flag, when present, equal to 1 indicates the current decoded picture, which corresponds to a driving picture that may be used for fusion, may be input to GenerativeNN( ). gfv_drive_pic_fusion_flag equal to 0 indicates the current decoded picture should not be input to GenerativeNN( ).

NOTE 3 – A gfv_drive_pic_fusion_flag value of 1 can be used, for example, to indicate that the current decoded picture can be used to improve face details or handle background changes.

NOTE 4 – Fusion takes the three inputs: the base picture, features from keypoints and/or matrices carried in the GFV SEI message, and the current decoded picture, and outputs a picture.

NOTE 5 – When the current decoded picture corresponds to a driving picture, it should be indicated as not for output purpose.

gfv_low_confidence_face_parameter_flag equal to 1 indicates the facial parameters have been derived with low confidence. gfv_low_confidence_face_parameter_flag equal to 0 indicates the confidence information of the facial parameters is not specified.

gfv_coordinate_present_flag equal to 1 indicates that coordinate information of keypoints is present. gfv_coordinate_present_flag equal to 0 indicates that coordinate information of keypoints is not present.

It is a requirement of bitstream conformance that when gfv_matrix_type_idx[ i ] for any i from 0 to gfv_num_matrix_types_minus1 is equal to 0 or 1, the value of gfv_coordinate_present_flag shall be equal to 1.

gfv_coordinate_precision_factor_minus1 plus 1 indicates the length, in bits, of syntax elements gfv_coordinate_x_abs[i], gfv_coordinate_y_abs[i], gfv_coordinate_z_abs[i], gfv_coordinate_dx_abs[ i ], gfv_coordinate_dy_abs[ i ] and gfv_coordinate_dz_abs[ i ].

gfv_num_kps_minus1 plus 1 indicates the number of keypoints. The value of gfv_num_kps_minus1 shall be in the range of 0 to 210 – 1, inclusive.

gfv_kps_pred_flag equal to 1 indicates syntax elements gfv_coordinate_dx_abs[ i ] ,gfv_coordinate_dy_abs[ i ], and gfv_coordinate_dz_abs[ i ] are present and syntax elements gfv_coordinate_dx_sign_flag[ i ], gfv_coordinate_dy_sign_flag[ i ] and gfv_coordinate_dz_sign_flag[ i ] may be present. gfv_kps_pred_flag equal to 0 indicates gfv_coordinate_x_abs[ i ] ,gfv_coordinate_y_abs[ i ], and gfv_coordinate_z_abs[ i ] are present and syntax elements gfv_coordinate_x_sign_flag[ i ], gfv_coordinate_y_sign_flag[ i ] and gfv_coordinate_z_sign_flag[ i ] may be present.

gfv_coordinate_z_present_flag equal to 1 indicates that z-axis coordinate information of the keypoints is present. coordinate_z_present_flag equal to 0 indicates that the z-axis coordinate information of the keypoints is not present.

gfv_coordinate_z_max_value_minus1 plus 1 indicates the maximum absolute value of z-axis coordinates of keypoints.

gfv_coordinate_x_abs[ i ] is used to derive the x-axis coordinate of the i-th keypoint.

gfv_coordinate_x_sign_flag[ i ] specifies the sign of the x-axis coordinate of the i-th keypoint. When gfv_coordinate_x_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_y_abs[ i ] is used to derive y-axis coordinate of i-th keypoint.

gfv_coordinate_y_sign_flag[ i ] specifies the sign of the y-axis coordinate of the i-th keypoint. When gfv_coordinate_y_sign_flag[i] is not present, it is inferred to be equal to 0.

gfv_coordinate_z_abs[ i ] is used to derive z-axis coordinate of the i-th keypoint.

gfv_coordinate_z_sign_flag[ i ] specifies the sign of the z-axis coordinate of the i-th key point. When gfv_coordinate_z_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_dx_abs[ i ] specifies a difference value that is used to derive x-axis coordinate of the i-th keypoint.

gfv_coordinate_dx_sign_flag[ i ] specifies the sign of the difference value of the x-axis coordinate of the i-th keypoint. When gfv_coordinate_dx_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_dy_abs[ i ] specifies a difference value that is used to derive y-axis coordinate of the i-th keypoint.

gfv_coordinate_dy_sign_flag[ i ] specifies the sign of the difference value of the y-axis coordinate of the i-th keypoint. When gfv_coordinate_yd_sign_flag[i] is not present, it is inferred to be equal to 0.

gfv_coordinate_dz_abs[ i ] specifies a difference value that is used to derive z-axis coordinate of the i-th keypoint.

gfv_coordinate_dz_sign_flag[ i ] specifies the sign of the difference value of the z-axis coordinate of the i-th key point. When gfv_coordinate_dz_sign_flag[ i ] is not present, it is inferred to be equal to 0.

The variables coordinateDeltaX[ i ], coordinateDeltaY[ i ] and coordinateDeltaZ[ i ] indicating the delta x-axis coordinate, delta y-axis coordinate and delta z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

coordinateDeltaX[ i ] = (1-2 * gfv_coordinate_dx_sign_flag[ i ]) * gfv_coordinate_dx_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateDeltaY[i] = (1-2 * gfv_coordinate_dy_sign_flag[ i ]) * gfv_coordinate_dy_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

if (gfv_coordinate_z_present_flag ) 
	  coordinateDeltaZ[i] = (1-2 * coordinate_dz_sign_flag[ i ]) * gfv_coordinate_dz_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

The variables coordinateX[ i ], coordinateY[ i ] and when gfv_coordinate_z_present_flag  is equal to 1, coordinateZ[ i ] indicating the x-axis coordinate, y-axis coordinate and z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

When gfv_kp_pred_flag is equal to 0,

coordinateX[ i ] = (1-2 * gfv_coordinate_x_sign_flag[ i ]) * gfv_coordinate_x_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateY[i] = (1-2 * gfv_coordinate_y_sign_flag[ i ]) * gfv_coordinate_y_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

if (gfv_coordinate_z_present_flag ) 
	    coordinateZ[i] = (1-2 * gfv_coordinate_z_sign_flag[ i ]) * gfv_coordinate_z_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

When gfv_kps_pred_flag is equal to 1,

if( gfv_base_pic_flag ) {
	coordinateX[ i ] = (( i > 0 ) ? coordinateX[ i - 1 ] : 0 ) + coordinateDeltaX[ i ]
	coordinateY[ i ] = (( i > 0 ) ? coordinateY[ i - 1 ] : 0 ) + coordinateDeltaY[ i ]
	if (gfv_coordinate_z_present_flag ) 
	    coordinateZ[ i ] = (( i > 0 ) ? coordinateZ[ i - 1 ] : 0 ) + coordinateDeltaZ[ i ]
}
else {
	coordinateX[ i ] = BaseKpCoordinateX[ i ] + coordinateDeltaX[ i ]
	coordinateY[ i ] = BaseKpCoordinateY[ i ] + coordinateDeltaY[ i ]
	if (gfv_coordinate_z_present_flag ) 
	    coordinateZ[ i ] = BaseKpCoordinateZ[ i ] + coordinateDeltaZ[ i ]
}

where BaseKpCoordinateX[ i ], BaseKpCoordinateY[ i ], BaseKpCoordinateZ[ i ] indicating the x-axis, y-axis and z-axis coordinates, respectively, of the i-th keypoint for the base picture are derived as follows:

if( gfv_base_pic_flag ) {
	BaseKpCoordinateX[ i ] = coordinateX[ i ]
	BaseKpCoordinateY[ i ] = coordinateY[ i ]
	if (gfv_coordinate_z_present_flag ) 
	    BaseKpCoordinateZ[ i ] = coordinateZ[ i ]
}

gfv_matrix_present_flag equal to 1 indicates that matrix parameters are present. gfv_matrix_present_flag equal to 0 indicates that matrix parameters are not present.

gfv_matrix_element_precision_factor_minus1 plus 1 indicates the length, in bits, of syntax elements gfv_matrix_element_dec[ i ][ j ][ k ][ m ] and gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ].

gfv_num_matrix_types_minus1 plus 1 indicates the number of matrix types signalled in the SEI message. The value of gfv_num_matrix_types_minus1 shall be in the range of 0 to 26 – 1, inclusive. It is a requirement of bitstream conformance that when gfv_matrix_pred_flag is equal to 1 and gfv_base_pic_flag is equal to 0, the value of gfv_num_matrix_types_minus1 shall be equal to the value of gfv_num_matrix_types_minus1 in each of the preceding GFV SEI message in decoding order in the current CLVS which has the same gfv_id value as the gfv_id value in the current SEI and has gfv_base_pic_flag equal to 1.

gfv_matrix_pred_flag equal to 1 indicates syntax elements gfv_matrix_element_int [ i ][ j ][ k ][ m ] , gfv_matrix_element_dec [ i ][ j ][ k ][ m ] are present and gfv_matrix_element_sign_flag [ i ][ j ][ k ][ m ] may be present. gfv_matrix_pred_flag equal to 0 indicates gfv_matrix_delta_element_int [ i ][ j ][ k ][ m ] , gfv_matrix_delta_element_dec [ i ][ j ][ k ][ m ] are present and syntax element gfv_matrix_delta_element_sign_flag [ i ][ j ][ k ][ m ] may be present. When gfv_matrix_pred_flag is not present, it is inferred to be 0.

gfv_matrix_type_idx[ i ] indicates the index of the i-th matrix type as specified in Table 3.

Table 3 - Specification of gfv_matrix_type_idx[ i ]

| Value | Specification |
| --- | --- |
| 0 | Affine translation matrix with the size of 2*2 or 3*3. |
| 1 | Covariance matrix with size of 2*2 or 3*3. |
| 2 | Mouth matrix representing mouth motion. |
| 3 | Eye matrix representing the open-close status and level of eyes. |
| 4 | Head rotation paramters with the size of 2*2 or 3*3 representing the head rotation in 2D space or 3D space. |
| 5 | Head translation matrix with the size of 1*2 or 1*3 representing head translationin 2D space or 3D space. |
| 6 | Head location matrix with size of 1*2 or 1*3 representing the head location in 2D space or 3D space. |
| 7 | Compact feature matrix with the size being specified by gfv_matrix_width_minus1[i] and gfv_matrix_height_minus1[i]. |
| 8…31 | Other matrix that may be used as determined by the application with the size being specified by gfv_matrix_width_minus1[i] and gfv_matrix_height_minus1[i]. |
| 32…63 | Reserved |

NOTE 6 . The undefined matrxi type is used to represent the matrxi type rather than affine translation matrix, covariance matrix, rotation matrix, translation matrix and compact feature matrix. It can be used by the user to extend the matrix type.

gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 1 indicates that the number of matrices of the i-th matrix type is equal to gfv_num_kps_minus1 + 1. gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 0 indicates the number of matrices of the i-th matrix type is not equal to gfv_num_kps_minus1 + 1. When not present gfv_num_matrices_equal_to_num_kps_flag[ i ] is inferred to be equal to 0.

gfv_num_matrices_info[ i ] provides information to derive the number of the matrices of the i-th matrix type.

gfv_matrix_width_minus1[ i ] plus 1 indicates the width of the matrix of the i-th matrix type.

gfv_matrix_height_minus1[ i ] plus 1 indicates the height of the matrix of the i-th matrix type.

gfv_matrix_for_3D_space_flag[ i ] equal to 1 indicates the matrix of the i-th matrix type is a matrix defined in three-dimensional space. gfv_matrix_for_3D_space_flag[ i ] equal to 0 indicates the matrix of the i-th matrix type is a matrix defined in two-dimensional space.

When gfv_matrix_width_minus1[ i ] is not present, it is inferred as follows:

––	If gfv_matrix_type_idx[ i ] is equal to 0, 1 or 4, and one of coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 1, gfv_matrix_width_minus1[i] is inferred to be equal to 2

––	otherwise, if matrix_type_idx[ i ] is equal to 0, 1 or 4, and one of coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 0, gfv_matrix_width_minus1[ i ] is inferred to be equal to 1

––	otherwise (matrix_type_idx[ i ] is equal to 5 or 6), gfv_matrix_width_minus1[ i ] is inferred to be equal to 0

When gfv_matrix_height_minus1[ i ] is not present, it is inferred as follows:

––	If matrix_type_idx is equal to 0, 1, 4, 5 or 6, and one of gfv_coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 1, gfv_matrix_height_minus1[ i ] is inferred to be equal to 2.

––	otherwise (gfv_matrix_type_idx is equal to 0, 1, 4, 5 or 6, and one of gfv_coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is 0), gfv_matrix_height _minus1[ i ] is inferred to be equal to 1.

The variables matrixWidth[ i ] and matrixHeight[ i ] indicating the width and height of the matrix of the i-th matrix type are derived as follows

matrixWidth[ i ] = gfv_matrix_width_minus1[ i ] + 1

matrixHeight[ i ] = gfv_matrix_height_minus1[ i ] + 1

gfv_num_matrices_minus1[ i ] plus 1 indicates the number of matrices of the i-th matrix type.

The variable numMatrices[ i ] indicating the number of the matrices of the i-th matrix type is derived as follows:

if( gfv_matrix_type_idx[ i ] == 0 || gfv_matrix_type_idx[ i ] == 1 ) {

if( gfv_coordinate_present_flag )

numMatrices[ i ] = gfv_num_matrices_equal_to_num_kps_flag[ i ] ? gfv_num_kps_minus1 + 1 : ( gfv_num_matrices_info[ i ] < gfv_num_kps_minus1 ? gfv_num_matrices_info [ i ] + 1 : gfv_num_matrices_info [ i ] + 2 )

else

numMatrices[ i ] = gfv_num_matrices_info[ i ] + 1

}

else if( gfv_matrix_type_idx[ i ] >= 2 &&  gfv_matrix_type_idx[ i ] < 7)

numMatrices[ i ] = 1

else

numMatrices[ i ] = gfv_num_matrices_minus1[ i ] + 1

It is a requirement of bitstream conformance that when gfv_matrix_pred_flag is equal to 1 and gfv_base_pic_flag is equal to 0, the values of numMatrices[ i ] , matrixWidth[ i ], and matrixHeight[ i ] for i in the range of 0 to gfv_num_matrix_types_minus1, inclusive shall be respectively equal to the values of numMatrices[ i ], matrixWidth[ i ], and matrixHeight[ i ] for i in the range of 0 to gfv_num_matrix_types_minus1, inclusive in each of the preceding GFV SEI message in decoding order in the current CLVS which has the same gfv_id value as the gfv_id value in the current SEI and has gfv_base_pic_flag equal to 1.

gfv_matrix_element_int[ i ][ j ][ k ][ m ] indicates the integer part of the value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type.

gfv_matrix_element_dec[ i ][ j ][ k ][ m ] indicates the decimal part of the value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type.

gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ] indicates the sign of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type. When gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ]is not present, it is inferred to be equal to 0.

gfv_matrix_delta_element_int[ i ][ j ][ k ][ m ] indicates the integer part of the difference value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type.

gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ] indicates the decimal part of the difference value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type.

gfv_matrix_delta_element_sign_flag[ i ][ j ][ k ][ m ] indicates the sign of the difference value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type. When gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ]is not present, it is inferred to be equal to 0.

The variable matrixElementDeltaVal[ i ][ j][ k ][ m ] representing the difference value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type is derived as follows:

matrixElementDeltaVal[ i][ j ][ k ][ m ] = 1-2*gfv_matrix_delta_element_sign_flag[ i ][ j ][ k ][ m ]*(gfv_matrix_delta_element_int i  j  k  m  +gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ]1<< (gfv_matrix_element_precision_factor_minus1 +1))

The variable matrixElementVal[ i ][ j][ k ][ m ] representing the value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type is derived as follows:

When gfv_matrix_pred_flag is equal to 0,

matrixElementVal[ i][ j ][ k ][ m ] = 1-2*gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ]*(gfv_matrix_element_int i  j  k  m  +gfv_matrix_element_dec[ i ][ j ][ k ][ m ]1<< (gfv_matrix_element_precision_factor_minus1 +1)) 
if( gfv_base_pic_flag ) 
	BaseMatrixElementVal[ i][ j ][ k ][ m ] = matrixElementVal[ i][ j ][ k ][ m ]

When gfv_matrix_pred_flag is equal to 1,

matrixElementVal[ i][ j ][ k ][ m ] = BaseMatrixElementVal[ i][ j ][ k ][ m ] + matrixElementDeltaVal[ i][ j ][ k ][ m ]

For a particular gfv_id value, the following process is used in increasing order of gfv_cnt to generate a video picture per each GFV SEI message that has gfv_base_pic_flag equal to 0 and a unique value of gfv_cnt within a picture unit:

DeriveSigParam( ) 
TranslatorNN (sigKeyPoint , sigMatrix) 
DeriveInputTensors( ) 
if( gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 0) {
  if(ChromaFormatIdc == 0 ) 
    GenerativeNN( inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
  else 
    GenerativeNN( inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
}
else if(gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 1) {
  if(ChromaFormatIdc == 0 ) 
    GenerativeNN( inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
  else 
    GenerativeNN( inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix) 
}

StoreOutputTensors( )

The process DeriveSigParam () for deriving the inputs of TranslatorNN () is specified as follows:

The keypoint coordinate array sigKeyPoint and the matrix sigMatrix are derived as follows:

if( gfv_coordinate_present_flag ) {
  for ( i = 0; i<= gfv_num_kps_minus1; i++ ) {
    sigKeyPoint[ i ][ 0 ] = coordinateX[ i ]
    sigKeyPoint[ i ][ 1 ] = coordinateY[ i ]
    if ( gfv_coordinate_z_present_flag ) 
      sigKeyPoint[ i ][ 2 ] = coordinateZ[ i ]
  }
}
else {
  for ( i = 0; i <= gfv_num_kps_minus1; i++ ) {
    sigKeyPoint [ i ][ 0 ] = 0
    sigKeyPoint [ i ][ 1 ] = 0
    if ( gfv_coordinate_z_present_flag ) 
      sigKeyPoint [ i ][ 2 ] = 0
  }
}
if( gfv_matrix_present_flag ) {
  for ( i = 0; i <= gfv_num_matrix_types_minus1; i++ ) {
    for ( j = 0; j < numMatrices[ i ]; j++ ) {
      for( k = 0; k < matrixHeight [ i ]; k++ ) {
        for ( l = 0;l < matrixWidth [ i ]; l++) {
          sigMatrix[ i ][ j ][ k ][ l ] = matrixElementVal[ i ][ j][ k][ l ] 
        }
      }
    }
  }
}
else {
  for ( i = 0; i <= gfv_num_matrix_types_minus1; i++ ) {
    for ( j = 0; j < numMatrices[ i ]; j++ ) {
      for( k = 0; k < matrixHeight [ i ]; k++ ) {
        for ( l = 0;l < matrixWidth [ i ]; l++) {
          sigMatrix [ i ][ j ][ k ][ l ] = 0
        }
      }
    }
  }
}

TranslatorNN( ) is a process to translate the various formats of the facial parameters carried in the SEI message to the fixed format of the facial parameters to be input to the generative network to generate the output picture

Inputs to TranslatorNN() are:

- sigKeyPoint and sigMatrix

Outputs of TranslatorNN() are:

- convKeyPoint and convNumKeyPoint

- convMatrix and convNumMatrix, convMatrixWidth, convMatrixHeight

The process DeriveInputTensors( ) for deriving the inputs of GenerativeNN ( ) is specified as follows:

When gfv_base_pic_flag is equal to 1, the BasePicture input tensor inputBaseY, inputBaseCb and inputBaseCr are derived as follows:

for( x = 0; x < CroppedWidth; x++ ) {
  for ( y = 0; y < CroppedHeight; y++ ) {
    inputBaseY[ x ][ y ] = InpY( baseCroppedYPic[ x ][ y ] ) 
  }
}
if (ChromaFormatIdc !=0) {
  for( x = 0; x < CroppedWidth/ SubWidthC; x++ ) {
    for ( y = 0; y < CroppedHeight/ SubHeightC; y++ ) {
      inputBaseCb[ x][ y ] = InpC( baseCroppedCbPic[ x][ y ] )
      inputBaseCr[ x][ y ] = InpC( baseCroppedCrPic[ x ][ y ] )
    }
  }
}

When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1, the DrivePicture luma sample array inputDriveY, inputDriveCb and input DriveCr are derived as follows:

for( x = 0; x< CroppedWidth; x++ ) {
  for ( y = 0; y< CroppedHeight; y++ ) {
    inputDriveY[ x ][ y ] = InpY( driveCroppedYPic[ x ][ y ] )
  }
}
if (ChromaFormatIdc !=0) {
  for( x = 0; x< CroppedWidth/ SubWidthC; x++ ) {
    for ( y = 0; y < CroppedHeight/ SubHeightC; y++ ) {
      InputDriveCb[ x][ y ] = InpC( driveCroppedCbPic[ x][ y ] )
      InputDriveCr[ x][ y ] = InpC( driveCroppedCrPic[ x ][ y ] ) 
    }
  }
}

When gfv_base_pic_flag is equal to 0, the keypoint coordinate array inputDriveKeyPoint and the matrix inputDriveMatrix for the current picture are derived as follows:

for ( i = 0; i < = convNumKeyPoint; i++ ) {
    inputDriveKeyPoint[ i ][ 0 ] = convKeyPoint[ i ][ 0 ]
    inputDriveKeyPoint [ i ][ 1 ] = convKeyPoint[ i ][ 1 ]
    inputDriveKeyPoint [ i ][ 2 ] = convKeyPoint[ i ][ 2 ]
  }

for( j = 0; j < convNumMatrix; j++ ) {
  for( k=0; k< convMatrixHeight; k++ ) {
    for ( m=0;m< convMatrixWidth; m++) {
      inputDriveMatrix[ j ][ k ][ m ] = convMatrix [ j ][ k ][ m ] 
    }
  }
}

When gfv_base_pic_flag is equal to 1, the keypoint coordinate array inputBaseKeyPoint and the matrix inputBaseMatrix for the base picture are derived as follows:

for ( i = 0; i < = convNumKeyPoint; i++ ) {
    inputBaseKeyPoint[ i ][ 0 ] = convKeyPoint[ i ][ 0 ]
    inputBaseKeyPoint [ i ][ 1 ] = convKeyPoint[ i ][ 1 ]
    inputBaseKeyPoint [ i ][ 2 ] = convKeyPoint[ i ][ 2 ]
}
for( j = 0; j < convNumMatrix; j++) {
  for( k=0; k< convMatrixHeight; k++ ) {
    for ( l=0;l<  convMatrixWidth; l++) {
      inputBaseMatrix[ j ][ k ][ l ] = convMatrix [ j ][ k ][ l ] 
    }
  }
}

where the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) – 1 )

InpC( x ) = x ÷ ( ( 1  <<  BitDepthC ) – 1 )

GenerativeNN ( ) is a process to generate the sample values of an output picture corresponding to a driving picture. It is only invoked when gfc_base_pic_flag is equal to 0. Input values to GenerativeNN( ) and output values from GenerativeNN( ) are real numbers.

Inputs to GenerativeNN() are:

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaFormatIdc is equal to 0: inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaFormatIdc is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaFormatIdc is equal to 0: inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaFormatIdc is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix

Outputs of GenerativeNN( ) are:

- A luma sample array genY

- When ChromaFormatIdc is not equal to 0, two chroma sample arrays genCb and genCr.

The process StoreOutputTensors( ) for deriving the output is specified as follows:

when gfv_base_pic_flag is equal to 0, the output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = OutY( genY[ x ][ y ] )
  }
}

if(ChromaFormatIdc != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = OutC( genCb[ x ][ y ] )
      outputCrPic[ x][ y ] = OutC( genCr[ x ][ y ] )
    }
  }
}

when gfv_base_pic_flag is equal to 1, the output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = baseCroppedYPic [ x ][ y ] 
  }
}

if(ChromaFormatIdc != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = baseCroppedCbPic [ x ][ y ] 
      outputCrPic[ x][ y ] = baseCroppedCbPic [ x ][ y ] 
    }
  }
}

where the functions OutY( ) and OutC( ) are specified as follows:

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) – 1 , x * ( ( 1  <<  BitDepthY ) – 1 ) 
OutC( x ) = Clip3( 0, ( 1  <<  BitDepthC ) – 1 , x * ( ( 1  <<  BitDepthC ) – 1 )

8.37 Generative face video enhancement SEI message

8.37.1 Generative face video enhancement SEI message syntax

| generative_face_video_enhancement ( payloadSize ) { | Descriptor |
| --- | --- |
| gfve_id | ue(v) |
| gfve_gfv_id | ue(v) |
| gfve_gfv_cnt | ue(v) |
| gfve_nn_present_flag | u(1) |
| if( gfve_nn_present_flag ) { |  |
| gfve_nn_base_flag | u(1) |
| gfve_nn_mode_idc | ue(v) |
| if( gfve_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfve_nn_reserved_zero_bit_a | u(1) |
| gfve_nn_tag_uri | st(v) |
| gfve_nn_uri | st(v) |
| } |  |
| } |  |
| gfve_matrix_element_precision_factor | ue(v) |
| gfve_num_matrices_minus1 | ue(v) |
| for(i=0; i <= gfve_num_matrices_minus1; i++){ |  |
| gfve_matrix_height_minus1[ i ] | ue(v) |
| gfve_matrix_width_minus1[ i ] | ue(v) |
| for( j = 0; j <= gfve_matrix_height_minus1[ i ]; j++ ) |  |
| for( k = 0; k <= gfve_matrix_width_minus1[ i ]; k++ ) { |  |
| gfve_matrix_element[ i ][ j ][ k ] | ue(v) |
| if( !gfve_matrix_element[ i ][ j ][ k ]) |  |
| gfve_matrix_element_sign_flag[ i ][ j ][ k ] | u(1) |
| } |  |
| } |  |
| if( gfve_nn_present_flag ) |  |
| if( gfve_nn_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfve_nn_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| gfve_nn_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

8.37.2 Generative face video enhancement SEI message semantics

The generative face video enhancement (GFVE) SEI message indicates enhancement facial parameters and specifies an enhancement network, denoted as EnhancerNN( ), that may be used to enhance the visual quality of the face pictures generated with GFV SEI message.

NOTE 1 – Enhancement facial parameters could be determined from source pictures prior to encoding.

NOTE 2 – When the current picture is not a base picture, the GFV SEI message may be used to generate a face picture based the facial parameters conveyed by the GFV SEI message, and the GFVE SEI message may be further used to enhance the generated face picture to improve the visual quality.

Use of this SEI message requires the definition of the following variables:

- Input and output picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

- Luma sample array baseCroppedYPic and chroma sample arrays baseCroppedCbPic and baseCroppedCrPic for a decoded output picture, denoted as BasePicture, corresponding to a base picture.

- Luma sample array genCroppedYPic and chroma sample arrays genCroppedCbPic and genCroppedCrPic for a generated picture with associated GFV SEI message, denoted as GenPicture, corresponding to a driving picture.

- Bit depth BitDepthY for the luma sample array of the input and output pictures.

- Bit depth BitDepthC for the chroma sample arrays, if any, of the input and output pictures.

- A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

gfve_id contains an identifying number that may be used to identify GFVE SEI message and specify a neural network that may be used as EnhancerNN( ). The value of gfve_id shall be in the range of 0 to 232 − 2, inclusive. Values of gfve_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering a GFVE SEI message with gfve_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

gfve_gfv_id and gfve_gfv_cnt specifies gfv_id and gfv_cnt of the associated GFV SEI message. The associated GFV SEI message is a GFV SEI message in the same picture unit with the GFVE SEI message having gfv_id equal to gfve_gfv_id, gfv_cnt equal to gfve_gfv_cnt. The GFVE message is used to enhance the picture generated with the associated GFV SEI message.

For a GFVE SEI message, the following applies:

- The GFVE SEI message shall be present in the same picture unit with the associated GFV SEI message. And the associated GFV SEI message shall be precede the GFVE SEI message in the decoding order. When the associated GFV SEI message is not present in the picture unit containing the GFVE SEI message, the GFVE SEI message shall be ignored.

- When a GFVE SEI message is the first GFVE SEI message with a particular value of gfve_id is present in an IRAP picture unit or in a picture unit that follows IRAP picture unit in output order and is not preceded in output order by any picture unit that follows the IRAP picture unit in output order and has a GFVE SEI message with that particular value of gfve_id, gfve_nn_present_flag shall be present and equal to 1.

- If a GFV SEI message A is associated with a GFVE SEI message A, and a GFV SEI message B is associated with a GFVE SEI message B, and GFV SEI message A precede the GFV SEI message B in decoding order, the GFVE SEI A shall also precede GFVE SEI message B in decoding order.

- If a GFVE SEI message A with gfve_gfv_id equal to gfveGfvIdA, gfve_gfv_cnt value equal to gfveGfvCntA, and a GFVE SEI message B with gfve_gfv_id equal to gfveGfvIdB, gfve_gfv_cnt value equal to gfveGfvCntB, are present in the same picture unit, and gfveGfvIdA is equal to gfveGfvIdB and gfveGfvCntA is less than gfveGfvCntB, the GFVE SEI message A shall precede the GFVE SEI message B in decoding order.

- If a GFVE SEI message A with gfve_gfv_id equal to gfveGfvIdA, gfve_gfv_cnt value equal to gfveGfvCntA, and a GFV SEI message B with gfv_id equal to gfvIdB, gfv_cnt value equal to gfvCntB, are present in the same picture unit, if gfveGfvIdA is equal to gfvIdB, gfveGfvCntA is less than gfvCntB, the GFVE SEI message A shall precede the GFV SEI message B in decoding order.

- When either of the following conditions is true, GFVE SEI messages shall have the same SEI payload content:

  - The GFVE SEI messages are present in the same picture unit, have gfve_nn_base_flag present and equal to 1 and have the same value of gfve_id, gfve_gfv_id, gfve_gfv_cnt.

  - The GFVE SEI messages are present in the same picture unit, have gfve_nn_base_flag not present and have the same value of gfve_id, gfve_gfv_id, gfve_gfv_cnt.

gfve_nn_present_flag equal to 1 indicates a neural network that may be used as a EnhancerNN( ) is contained or indicated by the SEI message. gfve_nn_present_flag equal to 0 indicates a neural network that may be used as a EnhancerNN( ) is not contained or indicated by the SEI message.

gfve_nn_base_flag, gfve_nn_mode_idc, gfve_nn_reserved_zero_bit_a, gfve_nn_tag_uri, gfve_nn_uri, gfve_nn_payload_byte[ i ] specify a neural network that may be used as a EnhancerNN( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] have the same syntax and semantics as nnpfc_base_flag, nnpfc_mode_idc, nnpfc_reserved_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_payload_byte[ i ], respectively.

gfve_matrix_element_precision_factor indicates quantization factor of matrix elements signalled in the SEI message.

gfve_num_matrices_minus1 plus 1 specifies the number of matrices signalled in the SEI message. The value of gfv_num_matrices_minus1 shall be in the range of 0 to 216 − 1, inclusive.

gfve_matrix_height_minus1[ i ] plus 1 indicates the height of the i-th matrix.

gfve_matrix_width_minus1[ i ] plus 1 indicates the width of the i-th matrix.

gfve_matrix_element[ i ][ j ][ k ] is used to derive the value of the element at position (k, j) of the i-th matrix.

gfve_matrix_element_sign_flag[ i ][ j ][ k ] indicates the sign of the matrix element at position (k, j) of the i-th matrix.

The variable matrixElementVal[ i ][ j][ k ] representing the value of the matrix element at position (k, j) of the i-th matrix is derived as follows:

matrixElementVal[ i][ j ][ k ] = 1-2*gfve_matrix_element_sign_flag[ i ][ j ][ k ]*gfve_matrix_element[ i ][ j ][ k ]1<< gfve_matrix_element_precision_factor

The following process is invoked for each GFVE SEI message to enhance the picture generated with the associated GFV SEI message:

DeriveInputTensors( ) 
if(ChromaFormatIdc == 0 ) 
  EnhancerNN( inputBaseY, inputGenY, inputMatrix) 
else 
  EnhancerNN( inputBaseY, inputBaseCb, inputBaseCr, inputGenY, inputGenCb, inputGenCr, inputMatrix) 
}
StoreOutputTensors( )

The process DeriveInputTensors( ) for deriving the inputs of EnhancerNN ( ) is specified as follows:

The BasePicture input tensor inputBaseY, inputBaseCb and inputBaseCr are derived as follows:

for( x = 0; x < CroppedWidth; x++ ) {
  for ( y = 0; y < CroppedHeight; y++ ) {
    inputBaseY[ x ][ y ] = InpY( baseCroppedYPic[ x ][ y ] ) 
  }
}
if (ChromaFormatIdc !=0) {
  for( x = 0; x < CroppedWidth/ SubWidthC; x++ ) {
    for ( y = 0; y < CroppedHeight/ SubHeightC; y++ ) {
      inputBaseCb[ x][ y ] = InpC( baseCroppedCbPic[ x][ y ] )
      inputBaseCr[ x][ y ] = InpC( baseCroppedCrPic[ x ][ y ] )
    }
  }
}

The GenPicture input tensor inputGenY, inputGenCb and inputGenCr are derived as follows:

for( x = 0; x < CroppedWidth; x++ ) {
  for ( y = 0; y < CroppedHeight; y++ ) {
    inputGenY[ x ][ y ] = InpY( genCroppedYPic[ x ][ y ] ) 
  }
}
if (ChromaFormatIdc !=0) {
  for( x = 0; x < CroppedWidth/ SubWidthC; x++ ) {
    for ( y = 0; y < CroppedHeight/ SubHeightC; y++ ) {
      inputGenCb[ x][ y ] = InpC( genCroppedCbPic[ x][ y ] )
      inputGenCr[ x][ y ] = InpC( genCroppedCrPic[ x ][ y ] )
    }
  }
}

The matrix input tensor inputMatrix is derived as follows:

for ( i = 0; i <= gfve_num_matrices_minus1; i++ ) {
  for ( j = 0; j <= gfve_matrix_height_minus1[ i ]; j++) {
    for( k = 0; k <= gfve_matrix_width_minus1[ i ]; k++ ) {
      inputMatrix[ i ][ j ][ k ] = matrixElementVal[ i ][ j][ k ] 
    }
  }
}

where the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) – 1 )

InpC( x ) = x ÷ ( ( 1  <<  BitDepthC ) – 1 )

EnhancerNN ( ) is a process to enhance the sample values of a generated picture that is generated with the associated GFV SEI message. Input values to EnhancerNN ( ) and output values from EnhancerNN ( ) are real numbers.

Inputs to EnhancerNN () are:

- When ChromaFormatIdc is equal to 0: inputBaseY, inputGenY, inputMatrix

- When ChromaFormatIdc is not equal to 1: inputBaseY, inputBaseCb, inputBaseCr, inputGenY, inputGenCb, inputGenCr, inputMatrix

Outputs of EnhancerNN ( ) are:

- A luma sample array enhanceY

- When ChromaFormatIdc is not equal to 0, two chroma sample arrays enhanceCb and enhanceCr.

The process StoreOutputTensors( ) for deriving the output is specified as follows:

The output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = OutY(enhanceY[ x ][ y ]) 
  }
}

if(ChromaFormatIdc != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = OutC(enhanceCb[ x ][ y ]) 
      outputCrPic[ x][ y ] = OutC(enhanceCr[ x ][ y ]) 
    }
  }
}

where the functions OutY( ) and OutC( ) are specified as follows:

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) – 1 , x * ( ( 1  <<  BitDepthY ) – 1 ) 
OutC( x ) = Clip3( 0, ( 1  <<  BitDepthC ) – 1 , x * ( ( 1  <<  BitDepthC ) – 1 )

8.38 Film grain regions characteristics SEI message

8.38.1 Film grain regions characteristics SEI message syntax

| film_grain_regions_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| fgr_cancel_flag | u(1) |
| if( !fgr_cancel_flag ) { |  |
| fgr_model_id | u(2) |
| fgr_separate_colour_description_present_flag | u(1) |
| if( fgr_separate_colour_description_present_flag ) { |  |
| fgr_bit_depth_luma_minus8 | u(3) |
| fgr_bit_depth_chroma_minus8 | u(3) |
| fgr_full_range_flag | u(1) |
| fgr_colour_primaries | u(8) |
| fgr_transfer_characteristics | u(8) |
| fgr_matrix_coeffs | u(8) |
| } |  |
| fgr_blending_mode_id | u(2) |
| fgr_log2_scale_factor | u(4) |
| for( c = 0; c < 3; c++ ) |  |
| fgr_comp_model_present_flag[ c ] | u(1) |
| for( c = 0; c < 3; c++ ) |  |
| if( fgr_comp_model_present_flag[ c ] ) { |  |
| fgr_num_intensity_intervals_minus1[ c ] | u(8) |
| fgr_num_model_values_minus1[ c ] | u(3) |
| for( i = 0; i <= fgr_num_intensity_intervals_minus1[ c ]; i++ ) { |  |
| fgr_intensity_interval_lower_bound[ c ][ i ] | u(8) |
| fgr_intensity_interval_upper_bound[ c ][ i ] | u(8) |
| for( j = 0; j <= fgr_num_model_values_minus1[ c ]; j++ ) |  |
| fgr_comp_model_value[ c ][ i ][ j ] | se(v) |
| } |  |
| } |  |
| fgr_region_information_present_flag | u(1) |
| if( fgr_region_information_present_flag ) { |  |
| fgr_region_based_adaptation_flag | u(1) |
| if( fgr_region_based_adaptation_flag ) { |  |
| fgr_active_regions_number_minus1 | u(8) |
| for( i = 0; i <= fgr_active_regions_number_minus1; i++ ) { |  |
| fgr_region_top[ i ] | u(16) |
| fgr_region_left[ i ] | u(16) |
| fgr_region_width[ i ] | u(16) |
| fgr_region_height[ i ] | u(16) |
| fgr_film_grain_enabled_flag[ i ] | u(1) |
| if ( fgr_film_grain_enabled_flag[ i ] ) { |  |
| for( c = 0; c < 3; c++ ) |  |
| if ( fgr_comp_model_present_flag[ c ] ) { |  |
| fgr_region_interval_min[ i ][ c ] | u(8) |
| fgr_region_interval_max[ i ][ c ] | u(8) |
| } |  |
| } |  |
| } |  |
| } |  |
| else |  |
| fgr_alpha_channel_adaptation_flag | u(1) |
| } |  |
| fgr _persistence_flag | u(1) |
| } |  |
| } |  |

8.38.2 Film grain regions characteristics SEI message semantics

This SEI message provides the decoder with a parameterized model for a film grain synthesis process. The film grain synthesis process should be applied to the decoded pictures prior to their display.

Compared to the FGC SEI message defined in 8.5, this SEI message also provides the decoder with parameters defining how to apply locally the film grain synthesis.

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	When the syntax element rdf_separate_colour_description_present_flag of the film grain regions characteristics SEI message is equal to 0, the following additional variables:

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

The film grain models specified in the film grain regions characteristics SEI message are expressed for application to decoded pictures that have 4:4:4 colour format with luma and chroma bit depths corresponding to the luma and chroma bit depths of the film grain model and use the same colour representation domain as the identified film grain model. When the colour format of the decoded video is not 4:4:4 or the decoded video uses a different luma or chroma bit depth from that of the film grain model or uses a different colour representation domain from that of the identified film grain model, an unspecified conversion process is expected to be applied to convert the decoded pictures to the form that is expressed for application of the film grain model.

NOTE 4 – Because the use of a specific method is not required for performing the film grain generation function used by the display process, a decoder could, if desired, down-convert the model information for chroma in order to simulate film grain for other chroma formats (4:2:0 or 4:2:2) rather than up-converting the decoded video (using a method not specified in this Specification) before performing film grain generation.

fgr_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous film grain regions characteristics SEI message in output order that applies to the current layer. fgr_cancel_flag equal to 0 indicates that film grain regions characteristics follows.

fgr_model_id identifies the film grain simulation model as specified in Table 5. The value of fgr_model_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for fgr_model_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain regions characteristics SEI messages with fgr_model_id equal to 2 or 3.

Table 5 – fgr_model_id values

| Value | Description |
| --- | --- |
| 0 | Frequency filtering |
| 1 | Auto-regression |

fgr_separate_colour_description_present_flag equal to 1 indicates that a distinct combination of luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients for the film grain characteristics specified in the SEI message is present in the film grain regions characteristics SEI message syntax. fgr_separate_colour_description_present_flag equal to 0 indicates that the combination of luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients for the film grain characteristics specified in the SEI message are the same as indicated in VUI parameters for the CLVS.

NOTE 5 – When fgr_separate_colour_description_present_flag is equal to 1, any of the luma bit depth, chroma bit depth, video full range flag, colour primaries, transfer characteristics, and matrix coefficients specified for the film grain characteristics specified in the SEI message could differ from that for the pictures in the CLVS.

When VUI parameters are not present for the CLVS or the value of vui_colour_description_present_flag is equal to 0, and equivalent information to that conveyed when vui_colour_description_present_flag is equal to 1 is not conveyed by external means, fgr_separate_colour_description_present_flag shall be equal to 1.

The input image Î, which may be the decoded picture or converted from the decoded picture, used in the equations in this clause is in the same colour representation domain as the simulated film grain signal. Therefore, when any of these parameters does differ from that for the pictures in CLVS, the input image Î used in the equations in this clause would be in a different colour representation domain than that for the pictures in the CLVS. For example, when the value of fgr_bit_depth_luma_minus8 + 8 is greater than BitDepthY (i.e., the bit depth of the luma component of the pictures in the CLVS), the bit depth of the input image Î used in the equations in this clause is also greater than BitDepthY. In such a case, the input image Î would be generated by converting the actual decoded picture to be in the same colour representation domain as the simulated film grain signal. The process for converting an actual decoded picture to the 4:4:4 colour format with same colour representation domain as the simulated film grain signal is not specified in this Specification.

fgr_bit_depth_luma_minus8 plus 8 specifies the bit depth used for the luma component of the film grain characteristics specified in the SEI message. When fgr_bit_depth_luma_minus8 is not present in the film grain regions characteristics SEI message, the value of fgr_bit_depth_luma_minus8 is inferred to be equal to BitDepthY − 8.

The value of fgBitDepth[ 0 ] is derived as follows:

fgBitDepth[ 0 ] = fgr_bit_depth_luma_minus8 + 8		(19)

fgr_bit_depth_chroma_minus8 plus 8 specifies the bit depth used for the Cb and Cr components of the film grain characteristics specified in the SEI message. When fgr_bit_depth_chroma_minus8 is not present in the film grain regions characteristics SEI message, the value of fgr_bit_depth_chroma_minus8 is inferred to be equal to BitDepthC − 8.

The value of fgBitDepth[ c ] for c = 1 and 2 is derived as follows:

fgBitDepth[ c ] = fgr_bit_depth_chroma_minus8 + 8, with c = 1, 2	(20)

fgr_full_range_flag has the same semantics as specified in clause 7.3 for the vui_full_range_flag syntax element, except as follows:

–	fgr_full_range_flag specifies the video full range flag of the film grain characteristics specified in the SEI message, rather than the video full range flag used for the CLVS.

–	When fgr_full_range_flag is not present in the film grain regions characteristics SEI message, the value of fgr_full_range_flag is inferred to be equal to vui_full_range_flag.

fgr_colour_primaries has the same semantics as specified in clause 7.3 for the vui_colour_primaries syntax element, except as follows:

–	fgr_colour_primaries specifies the colour primaries of the film grain characteristics specified in the SEI message, rather than the colour primaries used for the CLVS.

–	When fgr_colour_primaries is not present in the film grain regions characteristics SEI message, the value of fgr_colour_primaries is inferred to be equal to vui_colour_primaries.

fgr_transfer_characteristics has the same semantics as specified in clause 7.3 for the vui_transfer_characteristics syntax element, except as follows:

–	fgr_transfer_characteristics specifies the transfer characteristics of the film grain characteristics specified in the SEI message, rather than the transfer characteristics used for the CLVS.

–	When fgr_transfer_characteristics is not present in the film grain regions characteristics SEI message, the value of fgr_transfer_characteristics is inferred to be equal to vui_transfer_characteristics.

fgr_matrix_coeffs has the same semantics as specified in clause 7.3 for the vui_matrix_coeffs syntax element, except as follows:

–	fgr_matrix_coeffs specifies the matrix coefficients of the film grain characteristics specified in the SEI message, rather than the matrix coefficients used for the CLVS.

–	When fgr_matrix_coeffs is not present in the film grain regions characteristics SEI message, the value of fgr_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

–	The values allowed for fgr_matrix_coeffs are not constrained by the chroma format of the decoded video pictures that is indicated by the value of ChromaFormatIdc for the semantics of the VUI parameters.

fgr_blending_mode_id identifies the blending mode used to blend the simulated film grain with the input images as specified in Table 6. fgr_blending_mode_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for fgr_blending_mode_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with fgr_blending_mode_id equal to 2 or 3.

Table 6 – fgr_blending_mode_id values

| Value | Description |
| --- | --- |
| 0 | Additive |
| 1 | Multiplicative |

Depending on the value of fgr_blending_mode_id, the blending mode is specified as follows:

–	If fgr_blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] + G[ c ][ x ][ y ] )	(21)

–	Otherwise (fgr_blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] +	(22)
			Round( ( Î[ c ][ x ][ y ] * G[ c ][ x ][ y ] ) ÷ ( ( 1  <<  fgBitDepth[ c ] ) − 1 ) ) )

where Î[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the input image Î, G[ c ][ x ][ y ] is the simulated film grain value at the same position and colour component, and fgBitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays Igrain[ c ][ x ][ y ], Î[ c ][ x ][ y ], and G[ c ][ x ][ y ], where c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples − 1.

fgr_log2_scale_factor specifies a scale factor used in the film grain characterization equations.

fgr_comp_model_present_flag[ c ] equal to 0 indicates that film grain is not modelled on the c-th colour component, where c equal to 0 refers to the luma component, c equal to 1 refers to the Cb component, and c equal to 2 refers to the Cr component. fgr_comp_model_present_flag[ c ] equal to 1 indicates that syntax elements specifying modelling of film grain on colour component c are present in the SEI message.

When fgr_separate_colour_description_present_flag is equal to 0 and ChromaFormatIdc is equal to 0, the value of fgr_comp_model_present_flag[ 1 ] and fgr_comp_model_present_flag[ 2 ] shall be equal to 0.

fgr_num_intensity_intervals_minus1[ c ] plus 1 specifies the number of intensity intervals for which a specific set of model values has been estimated.

NOTE 6 – The intensity intervals could overlap in order to simulate multi-generational film grain.

fgr_num_model_values_minus1[ c ] plus 1 specifies the number of model values present for each intensity interval in which the film grain has been modelled. The value of fgr_num_model_values_minus1[ c ] shall be in the range of 0 to 5, inclusive.

fgr_intensity_interval_lower_bound[ c ][ i ] specifies the lower bound of the i-th intensity interval for which the set of model values applies.

fgr_intensity_interval_upper_bound[ c ][ i ] specifies the upper bound of the i-th intensity interval for which the set of model values applies.

The variable intensityIntervalIdx[ c ][ x ][ y ][ j ] represents the j-th index to the list of intensity intervals selected for the sample value Î[ c ][ x ][ y ] for c = 0..2, x = 0..PicWidthInLumaSamples − 1, y = 0..PicHeightInLumaSamples − 1, and j = 0..numApplicableIntensityIntervals[ c ][ x ][ y ] − 1, where numApplicableIntensityIntervals[ c ][ x ][ y ] is derived below.

The minimum and maximum interval index for the sample of colour component c at coordinate x, y is derived as follows:

- If fgr_region_based_adaptation_flag is equal to 0, intensityIntervalMin[ c ][ x ][ y ] is set equal to 0 and intensityIntervalMax[ c ][ x ][ y ] is set equal to fgr_num_intensity_intervals_minus1[ c ]

- Otherwise, intensityIntervalMax[ c ][ x ][ y ] and intensityIntervalMin[ c ][ x ][ y ] are derived as follows:

intensityIntervalMin[ c ][ x ][ y ] = 1
intensityIntervalMax[ c ][ x ][ y ] = 0
for( i = 0; i <= fgr_active_regions_number_minus1; i++ )
	if ( x >= fgr_region_left[ i ] && x < fgr_region_left[ i ] + fgr_region_width[ i ] &&
			y >= fgr_region_top[ i ] && y < fgr_region_top[ i ] + fgr_region_height[ i ] )
		if ( fgr_film_grain_enabled_flag[ i ] ) {	(23)
			intensityIntervalMin[ c ][ x ][ y ] = fgr_region_interval_min[ i ][ c ]
			intensityIntervalMax[ c ][ x ][ y ] = fgr_region_interval_max[ i ][ c ]
		} else {
			intensityIntervalMin[ c ][ x ][ y ] = 1
			intensityIntervalMax[ c ][ x ][ y ] = 0
	}

Depending on the value of fgr_model_id, the selection of the one or more intensity intervals for the sample value Î[ c ][ x ][ y ] is specified as follows:

–	The variable numApplicableIntensityIntervals[ c ][ x ][ y ] is initially set equal to 0.

–	If fgr_model_id is equal to 0, the following applies:

- The top-left sample location ( xB, yB ) of the current 8x8 block b that contains the sample value Î[ c ][ x ][ y ] is derived as ( xB, yB ) = ( x / 8, y / 8 ).

- The average value bavg of the current 8x8 block b is derived as follows:

sum8x8 = 0
for( i = 0; i < 8; i++ )
	for( j = 0; j < 8; j++ )
		sum8x8  +=  Î[ c ][ xB * 8 + i ][yB * 8 + j ]	(23)
bavg = Clip3( 0, 255, ( sum8x8 + ( 1  <<  ( fgBitDepth[ c ] − 3 ) ) )  >>  ( fgBitDepth[ c ] − 2 ) )

- The value of intensityIntervalIdx[ c ][ x ][ y ][ j ] is derived as follows:

for( i = intensityIntervalMin[ c ][ x ][ y ], j = 0; i <= intensityIntervalMax[ c ][ x ][ y ]; i++ )
	if( bavg  >=  fgr_intensity_interval_lower_bound[ c ][ i ]  &&
			bavg  <=  fgr_intensity_interval_upper_bound[ c ][ i ] ) {
		intensityIntervalIdx[ c ][ x ][ y ][ j ] = i		(24)
		j++
	}
numApplicableIntensityIntervals[ c ][ x ][ y ] = j

–	Otherwise (fgr_model_id is equal to 1), the value of intensityIntervalIdx[ c ][ x ][ y ][ j ] is derived as follows:

I8[ c ][ x ][ y ] = ( fgBitDepth[ c ]  = =  8 ) ? ( Î[ c ][ x ][ y ] :
		Clip3( 0, 255, ( Î[ c ][ x ][ y ] + ( 1  <<  ( fgBitDepth[ c ] − 9 ) ) )  >>  ( fgBitDepth[ c ] − 8 ) )
for( i = intensityIntervalMin[ c ][ x ][ y ], j = 0; i <= intensityIntervalMax[ c ][ x ][ y ]; i++ )
	if( I8[ c ][ x ][ y ]  >=  fgr_intensity_interval_lower_bound[ c ][ i ]  &&	(25)
			I8[ c ][ x ][ y ]  <=  fgr_intensity_interval_upper_bound[ c ][ i ] ) {
		intensityIntervalIdx[ c ][ x ][ y ][ j ] = i
		j++
	}
numApplicableIntensityIntervals[ c ][ x ][ y ] = j

Samples that do not fall into any of the defined intervals (i.e., those samples for which the value of numApplicableIntensityIntervals[ c ][ x ][ y ] is equal to 0) are not modified by the grain generation function. Samples that fall into more than one interval (i.e., those samples for which the value of numApplicableIntensityIntervals[ c ][ x ][ y ] is greater than 1) will originate multi-generation grain. Multi-generation grain results from adding the grain computed independently for each of the applicable intensity intervals.

In the equations in the remainder of this clause, the variable sj in each instance of the list fgr_comp_model_value[ c ][ sj ] is the value of intensityIntervalIdx[ c ][ x ][ y ][ j ] derived for the sample value Î[ c ][ x ][ y ].

fgr_comp_model_value[ c ][ i ][ j ] specifies the j-th model value for the colour component c and the i-th intensity interval. The set of model values has different meaning depending on the value of fgr_model_id.

The value of fgr_comp_model_value[ c ][ i ][ j ] is constrained as follows, and could be additionally constrained as specified elsewhere in this clause:

–	If fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ j ] shall be in the range of 0 to 2fgBitDepth[ c ] − 1, inclusive.

–	Otherwise (fgr_model_id is equal to 1), fgr_comp_model_value[ c ][ i ][ j ] shall be in the range of −2( fgBitDepth[ c ] − 1 ) to 2( fgBitDepth[ c ] − 1 ) − 1, inclusive.

Depending on the value of fgr_model_id, the synthesis of the film grain is modelled as follows:

–	If fgr_model_id is equal to 0, a frequency filtering model enables simulating the original film grain for c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples − 1 as specified by:

G[ c ][ x ][ y ] = ( fgr_comp_model_value[ c ][ sj ][ 0 ] * Q[ c ][ x ][ y ] + fgr_comp_model_value[ c ][ sj ][ 5 ] *
		G[ c − 1 ][ x ][ y ] )  >>  fgr_log2_scale_factor	(26)

where Q[ c ] is a two-dimensional random process generated by filtering 16x16 blocks gaussRv with random-valued elements gaussRvij generated with a normalized Gaussian distribution (independent and identically distributed Gaussian random variable samples with zero mean and unity variance) and where the value of an element G[ c − 1 ][ x ][ y ] used in the right-hand side of the equation is inferred to be equal to 0 when c − 1 is less than 0.

NOTE 7 – A normalized Gaussian random variable can be generated from two independent, uniformly distributed random values over the interval from 0 to 1 (and not equal to 0), denoted as uRv0 and uRv1, using the Box-Muller transformation specified by:

gaussRvi,j = Sqrt( −2 * Ln( uRv0 ) ) * Cos( 2 * π * uRv1 )	(27)

where  is Archimedes' constant 3.141 592 653 589 793....

The band-pass filtering of blocks gaussRv can be performed in the discrete cosine transform (DCT) domain as follows:

for( y = 0; y < 16; y++ )
	for( x = 0; x < 16; x++ )
		if( ( x < fgr_comp_model_value[ c ][ sj ][ 3 ]  &&  y < fgr_comp_model_value[ c ][ sj ][ 4 ] )  | |	(28)
				x > fgr_comp_model_value[ c ][ sj ][ 1 ]  | |  y > fgr_comp_model_value[ c ][ sj ][ 2 ] )
			gaussRv[ x ][ y ] = 0
filteredRv = IDCT16x16( gaussRv )

where IDCT16x16( z ) refers to a unitary inverse discrete cosine transformation (IDCT) operating on a 16x16 matrix argument z as specified by:

IDCT16x16( z ) = r * z * rT		(29)

where the superscript T indicates a matrix transposition and r is the 16x16 matrix with elements rij specified by:

ri,j = (  i  = =  0  ? 1  : Sqrt( 2 )4* Cos i *  2 * j + 1   * π32 		(30)

where  is Archimedes' constant 3.141 592 653 589 793....

Q[ c ] is formed by the frequency-filtered blocks filteredRv.

NOTE 8 – Coded model values are based on blocks of size 16x16, but a decoder implementation could use other block sizes. For example, decoders implementing the IDCT on 8x8 blocks could down-convert by a factor of two the set of coded model values fgr_comp_model_value[ c ][ sj ][ i ] for i equal to 1..4.

NOTE 9 – To reduce the degree of visible blocks that result from mosaicking the frequency-filtered blocks filteredRv, decoders could apply a low-pass filter to the boundaries between frequency-filtered blocks.

–	Otherwise (fgr_model_id is equal to 1), an auto-regression model enables simulating the original film grain for c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples − 1 as specified by:

G[ c ][ x ][ y ] = ( fgr_comp_model_value[ c ][ sj ][ 0 ] * n[ c ][ x][ y ] +
		fgr_comp_model_value[ c ][ sj ][ 1 ] * ( G[ c ][ x − 1 ][ y ] +
		( ( fgr_comp_model_value[ c ][ sj ][ 4 ] * G[ c ][ x ][ y − 1 ] )  >>
		fgr_log2_scale_factor ) ) + fgr_comp_model_value[ c ][ sj ][ 3 ] *
		( ( fgr_comp_model_value[ c ][ sj ][ 4 ] * ( G[ c ][ x − 1 ][ y − 1 ] + G[ c ][ x + 1 ][ y − 1 ] ) )  >>	(31)
		fgr_log2_scale_factor ) +
		fgr_comp_model_value[ c ][ sj ][ 5 ] * ( G[ c ][ x − 2 ][ y ] +
		( ( fgr_comp_model_value[ c ][ sj ][ 4 ] * fgr_comp_model_value[ c ][ sj ][ 4 ] * G[ c ][ x ][ y − 2 ] )  >>
		( 2 * fgr_log2_scale_factor ) ) ) +
		fgr_	comp_model_value[ c ][ sj ][ 2 ] * G[ c − 1 ][ x ][ y ] )  >>  fgr_log2_scale_factor

where n[ c ][ x ][ y ] is a random value with normalized Gaussian distribution (independent and identically distributed Gaussian random variable samples with zero mean and unity variance for each value of c, x, and y) and where the value of an element G[ c ][ x ][ y ] used in the right-hand side of the equation is inferred to be equal to 0 when any of the following conditions are true:

–	c is less than 0,

–	x is less than 0,

–	y is less than 0.

fgr_comp_model_value[ c ][ i ][ 0 ] provides the first model value for the model as specified by fgr_model_id. fgr_comp_model_value[ c ][ i ][ 0 ] corresponds to the standard deviation of the Gaussian noise term in the generation functions specified in Equations 26 through 31.

fgr_comp_model_value[ c ][ i ][ 1 ] provides the second model value for the model as specified by fgr_model_id. When fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 1 ] shall be greater than or equal to 0 and less than 16.

When not present in the film grain regions characteristics SEI message, fgr_comp_model_value[ c ][ i ][ 1 ] is inferred as follows:

–	If fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 1 ] is inferred to be equal to 8.

–	Otherwise (fgr_model_id is equal to 1), fgr_comp_model_value[ c ][ i ][ 1 ] is inferred to be equal to 0.

fgr_comp_model_value[ c ][ i ][ 1 ] is interpreted as follows:

–	If fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 1 ] indicates the horizontal high cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (fgr_model_id is equal to 1), fgr_comp_model_value[ c ][ i ][ 1 ] indicates the first order spatial correlation for neighbouring samples ( x − 1, y ) and ( x, y − 1 ).

fgr_comp_model_value[ c ][ i ][ 2 ] provides the third model value for the model as specified by fgr_model_id. When fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 2 ] shall be greater than or equal to 0 and less than 16.

When not present in the film grain regions characteristics SEI message, fgr_comp_model_value[ c ][ i ][ 2 ] is inferred as follows:

–	If fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 2 ] is inferred to be equal to fgr_comp_model_value[ c ][ i ][ 1 ]

–	Otherwise (fgr_model_id is equal to 1), fgr_comp_model_value[ c ][ i ][ 2 ] is inferred to be equal to 0.

fgr_comp_model_value[ c ][ i ][ 2 ] is interpreted as follows:

–	If fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 2 ] indicates the vertical high cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (fgr_model_id is equal to 1), fgr_comp_model_value[ c ][ i ][ 2 ] indicates the colour correlation between consecutive colour components.

fgr_comp_model_value[ c ][ i ][ 3 ] provides the fourth model value for the model as specified by fgr_model_id. When fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 3 ] shall be greater than or equal to 0 and less than or equal to fgr_comp_model_value[ c ][ i ][ 1 ].

When not present in the film grain regions characteristics SEI message, fgr_comp_model_value[ c ][ i ][ 3 ] is inferred to be equal to 0.

fgr_comp_model_value[ c ][ i ][ 3 ] is interpreted as follows:

–	If fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 3 ] indicates the horizontal low cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (fgr_model_id is equal to 1), fgr_comp_model_value[ c ][ i ][ 3 ] indicates the first order spatial correlation for neighbouring samples ( x − 1, y − 1 ) and ( x + 1, y − 1 ).

fgr_comp_model_value[ c ][ i ][ 4 ] provides the fifth model value for the model as specified by fgr_model_id. When fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 4 ] shall be greater than or equal to 0 and less than or equal to fgr_comp_model_value[ c ][ i ][ 2 ].

When not present in the film grain regions characteristics SEI message, fgr_comp_model_value[ c ][ i ][ 4 ] is inferred to be equal to fgr_model_id.

fgr_comp_model_value[ c ][ i ][ 4 ] is interpreted as follows:

–	If fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 4 ] indicates the vertical low cut frequency to be used to filter the DCT of a block of 16x16 random values.

–	Otherwise (fgr_model_id is equal to 1), fgr_comp_model_value[ c ][ i ][ 4 ] indicates the aspect ratio of the modelled grain.

fgr_comp_model_value[ c ][ i ][ 5 ] provides the sixth model value for the model as specified by fge_model_id.

When not present in the film grain regions characteristics SEI message, fge_comp_model_value[ c ][ i ][ 5 ] is inferred to be equal to 0.

fgr_comp_model_value[ c ][ i ][ 5 ] is interpreted as follows:

–	If fgr_model_id is equal to 0, fgr_comp_model_value[ c ][ i ][ 5 ] indicates the colour correlation between consecutive colour components.

–	Otherwise (fgr_model_id is equal to 1), fgr_comp_model_value[ c ][ i ][ 5 ] indicates the second order spatial correlation for neighbouring samples ( x, y − 2 ) and ( x − 2, y ).

fgr_region_based_adaptation_flag equal to 1 indicates that the spatial adaption of the film grain is defined per each region of the present SEI message and described by a rectangle. When not present in the film grain regions characteristics SEI message, fgr_region_based_adaptation_flag is inferred to be equal to 0.

fgr_active_regions_number_minus1 plus 1 indicates the number of regions for which the film grain adaptation is specified. fgr_active_regions_number_minus1 shall be in the range of 0 to 255, inclusive.

fgr_region_top[ i ], fgr_region_left[ i ], fgr_region_width[ i ], and fgr_region_height[ i ] specify the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the i-th region in the picture.

The value of fgr_region_left[ i ] shall be in the range of 0 to PicWidthInLumaSamples − 1, inclusive.

The value of fgr_region_top[ i ] shall be in the range of 0 to PicHeightInLumaSamples  − 1, inclusive.

The value of fgr_region_width[ i ] shall be in the range of 0 to PicWidthInLumaSamples  − fgr_region_left [ i ], inclusive.

The value of fgr_region_height[ i ] shall be in the range of 0 to PicHeightInLumaSamples  − fgr_region_top[ i ], inclusive.

fgr_film_grain_enabled_flag[ i ] equal to 1 indicates that the film grain characteristics using region intervals are applied on the i-th region. fge_film_grain_enabled_flag[ i ] equals to 0 indicates that no film grain synthesis is applied to the i-th region of the current picture.

fgr_region_interval_min[ i ][ c ] and fgr_region_interval_max[ i ][ c ] specify the range of interval indexes for color component c that are applicable to the i-th region. fgr_region_interval_min[ i ][ c ] shall be in the range of 0 to fg_num_intensity_intervals_minus1[ c ], inclusive, and fgr_region_interval_max[ i ][ c ] shall be in the range of fgr_region_interval_min[ i ][ c ] to fg_num_intensity_intervals_minus1[ c ], inclusive.

NOTE – Overlapping regions are processed in the increasing order of region index i, which means that a sample belonging to the region with index i=N with min and max film grain intervals indexes equal to x and y, respectively, and also belonging to region with index i=M (with M>N) with min and max film grain intervals indexes equal to x’ and y’, respectively, is processed with film grain characteristics defined with min and max film grain intervals indexes equal to x’ and y’.

fgr_alpha_channel_adaptation_flag equal to 1 indicates that the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA are applied to the simulated film grain values defined in the present SEI message prior to be applied are associated primary picture.

When an AU contains an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, the SDI SEI message shall precede the FRGC SEI message in decoding order.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an alpha auxiliary layer by an SDI SEI message, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

If the current PU contains an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, the simulated film grain values are weighted by the auxiliary data as follow:

–	If fgr_blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] + G[ c ][ x ][ y ]* Iaux[ c ][ x ][ y ])

–	Otherwise (fgr_blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] +
			Round( ( Î[ c ][ x ][ y ] * G[ c ][ x ][ y ]* Iaux[ c ][ x ][ y ]) ) ÷ ( ( 1  <<  fgBitDepth[ c ] ) − 1 ) ) )

where Î[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the input image Î, G[ c ][ x ][ y ] is the simulated film grain value at the same position and colour component, and fgBitDepth[ c ], is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays Igrain[ c ][ x ][ y ], Î[ c ][ x ][ y ], and G[ c ][ x ][ y ], where c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples – 1, and where Iaux[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the decoded auxiliary picture.

fgr_persistence_flag specifies the persistence of the film grain regions characteristics SEI message for the current layer.

fgr_persistence_flag equal to 0 specifies that the film grain regions characteristics SEI message applies to the current decoded picture only.

fgr_persistence_flag equal to 1 specifies that the film grain regions characteristics SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a film grain regions characteristics SEI message is output that follows the current picture in output order.

8.39 Packed regions information SEI message

8.39.1 Packed regions information SEI message syntax

| packed_regions_info( payloadSize) | Descriptor |
| --- | --- |
| pri_cancel_flag | u(1) |
| if ( !pri_cancel_flag ) { |  |
| pri_persistence_flag | u(1) |
| pri_num_regions_minus1 | ue(v) |
| pri_use_max_dimensions_flag | u(1) |
| pri_log2_unit_size | u(4) |
| pri_region_size_len_minus1 | u(4) |
| pri_region_id_present_flag | u(1) |
| pri_target_pic_params_present_flag | u(1) |
| if( pri_target_pic_params_present_flag ) { |  |
| pri_target_pic_width_minus1 | u(16) |
| pri_target_pic_height_minus1 | u(16) |
| } |  |
| pri_num_resampling_ratios_minus1 | ue(v) |
| for( i = 1; i <= pri_num_resampling_ratios_minus1; i++ ) { |  |
| pri_resampling_width_num_minus1[ i ] | ue(v) |
| pri_resampling_width_denom_minus1[ i ] | ue(v) |
| pri_fixed_aspect_ratio_flag[ i ] | u(1) |
| if( !pri_fixed_aspect_ratio_flag[ i ] ) { |  |
| pri_resampling_height_num_minus1[ i ] | ue(v) |
| pri_resampling_height_denom_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| for( i = 0; i <= pri_num_regions_minus1; i++ ) { |  |
| if( pri_region_id_present_flag ) |  |
| pri_region_id[ i ] | ue(v) |
| pri_region_top_left_in_units_x[ i ] | u(v) |
| pri_region_top_left_in_units_y[ i ] | u(v) |
| pri_region_width_in_units_minus1[ i ] | u(v) |
| pri_region_height_in_units_minus1[ i ] | u(v) |
| if( pri_num_resampling_ratios_minus1 > 0 ) |  |
| pri_resampling_ratio_idx[ i ] | u(v) |
| if( pri_target_pic_params_present_flag ) |  |
| pri_target_region_top_left_x[ i ] | u(v) |
| pri_target_region_top_left_y[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

8.39.2 Packed regions information SEI message semantics

The packed regions info SEI message provides information regarding rectangular regions packed with the coded picture. This information may optionally be used to reconstruct a target picture from the samples of the cropped decoded picture corresponding to the regions described in this SEI message.

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	A maximum picture width and maximum picture height in units of luma samples, denoted herein by MaxPicWidth and MaxPicHeight, respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

pri_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous packed regions information SEI message in output order that applies to the current layer. pri_cancel_flag equal to 0 indicates that packed regions information follows.

pri_persistence_flag specifies the persistence of the packed regions information SEI message for the current layer.

pri_persistence_flag equal to 0 specifies that the packed regions information applies to the current decoded picture only.

pri_persistence_flag equal to 1 specifies that the packed regions information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a packed regions information SEI message is output that follows the current picture in output order.

pri_num_regions_minus1 plus 1 specifies the number of regions for which information is signalled.

pri_use_max_dimensions_flag equal to 1 specifies that MaxPicWidth, MaxPicHeight, PicWidthInLumaSamples and PicHeightInLumaSamples will be used in variable calculations. pri_use_max_dimensions_flag equal to 0 specifies that MaxPicWidth, MaxPicHeight, PicWidthInLumaSamples and PicHeightInLumaSamples will not be used in variable calculations for the region parameters.

pri_log2_unit_size specifies a unit size used in variable calculations for the region parameters.

The variable priUnitSize is set equal to 1 << pri_log2_unit_size.

pri_region_size_len_minus1 plus 1 specifies the number of bits used to signal pri_region_top_left_in_units_x[ i ], pri_region_top_left_in_units_y[ i ], pri_region_width_in_units_minus1[ i ], pri_region_height_in_units_minus1[ i ],  pri_target_region_top_left_x[ i ], and pri_target_region_top_left_y[ i ].

pri_region_id_present_flag equal to 1 indicates the pri_region_id[ i ] syntax element is present. pri_region_id_present_flag equal to 0 indicates the pri_region_id[ i ] syntax element is not present.

pri_target_pic_params_present_flag equal to 1 indicates the pri_target_region_top_left_x[ i ], pri_target_region_top_left_y[ i ], pri_target_pic_width_minus1, and pri_target_pic_height_minus1 syntax elements are present. pri_target_pic_params_present_flag equal to 0 indicates the pri_target_region_top_left_x[ i ], pri_target_region_top_left_y[ i ], pri_target_pic_width_minus1, and pri_target_pic_height_minus1 syntax elements are not present.

pri_target_pic_width_minus1 plus 1 and pri_target_pic_height_minus1 plus 1, when present, indicate the width and height, respectively, in luma samples of the target picture that may be reconstructed from the samples of the cropped decoded picture corresponding to the regions described in this SEI message.

pri_num_resampling_ratios_minus1 specifies the number of resampling ratios that are signalled.

pri_resampling_width_num_minus1[ i ] plus 1 and pri_resampling_width_denom_minus1[ i ] plus 1 specify the numerator and denominator, respectively, for the width resampling of the i-th resampling ratio. Both pri_resampling_width_num_minus1[ i ] and pri_resampling_width_denom_minus1[ i ] shall be in the range of 0 to 65 535, inclusive.

The values of pri_resampling_ratio_width_num_minus1[ 0 ] and pri_resampling_ratio_width_denom_minus1[ 0 ] are inferred to be equal to 0.

pri_fixed_aspect_ratio_flag[ i ] equal to 1 specifies that the pri_resampling_height_num_minus1[ i ] and pri_resampling_height_denom_minus1[ i ] syntax elements are not present. pri_fixed_aspect_ratio_flag[ i ] equal to 0 specifies that the pri_resampling_height_num_minus1[ i ] and pri_resampling_height_denom_minus1[ i ] syntax elements are present.

pri_resampling_height_num_minus1[ i ] plus 1 and pri_resampling_height_denom_minus1[ i ] plus 1 specify the numerator and denominator, respectively, for the height resampling of the i-th resampling ratio. Both pri_resampling_height_num_minus1[ i ] and pri_resampling_height_denom_minus1[ i ] shall be in the range of 0 to 65 535, inclusive. When not present, the values of pri_resampling_height_num_minus1[ i ] and pri_resampling_height_denom_minus1[ i ] are inferred to be equal to the pri_resampling_width_num_minus1[ i ] and pri_resampling_width_denom_minus1[ i ], respectively.

pri_region_id[ i ] indicates the ID of the i-th region. When not present, the value of pri_region_id[ i ] is inferred to be equal to i.

pri_region_top_left_in_units_x[ i ] and pri_region_top_left_in_units_y[ i ] specify the horizontal and vertical positions, respectively, of the top left sample of the i-th region in units. The length of the syntax elements are pri_region_size_len_minus1 + 1.

The variables priRegionTopLeftX[ i ] and priRegionTopLeftY, representing the horizontal and vertical positions, respectively, in luma samples of the region in the cropped decoded picture, are derived as follows: [Ed. (JB): Add equation numbers when integrated.]

if( !pri_use_max_dimensions_flag ) {
	priRegionTopLeftX[ i ] =	 pri_region_top_left_in_units_x[ i ] * priUnitSize
	priRegionTopLeftY[ i ] =	 pri_region_top_left_in_units_y[ i ] * priUnitSize
} else {
	priRegionTopLeftX[ i ] =	( pri_region_top_left_in_units_x[ i ] * priUnitSize * 
			PicWidthInLumaSamples + MaxPicWidth /2 ) / MaxWidth
	priRegionTopLeftY[ i ] =	( pri_region_top_left_in_units_y[ i ] * priUnitSize* 
			PicHeightInLumaSamples + MaxPicHeight /2 ) / MaxHeight
}

pri_region_width_in_units_minus1[ i ] plus 1 and pri_region_height_in_units_minsu1[ i ] plus 1 specify the horizontal and vertical positions, respectively, of the width and height of the i-th region in units. The length of the syntax elements are pri_region_size_len_minus1 + 1.

The variables priRegionWidth[ i ] and priRegionHeight[ i ], representing the width and height, respectively, in luma samples of the i-th region in the cropped decoded picture are derived as follows:

if( !pri_use_max_dimensions_flag ) {
	priRegionWidth[ i ] =	 ( pri_region_width_in_units_minus1 [ i ] + 1) * priUnitSize 
	priRegionHeight[ i ] =	 ( pri_region_height_in_units_minus1 [ i ] + 1) * priUnitSize 
} else {
	priRegionWidth[ i ] =  ( ( pri_region_width_in_units_minus1 [ i ] + 1) * priUnitSize *
			PicWidthInLumaSamples + MaxPicWidth/2 ) / MaxWidth
	priRegionHeight[ i ] = ( ( pri_region_height_in_units_minus1 [ i ] + 1) * priUnitSize * 
			PicHeightInLumaSamples + MaxPicHeight/2 ) / MaxHeight
}

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

It is a requirement of bitstream conformance that priRegionWidth[ i ] % SubWidthC shall be equal to 0 and priRegionHeight[ i ] % SubHeightC shall be equal to 0.

pri_resampling_ratio_idx[ i ] specifies the index of the resampling ratio used for the i-th region. The length of the syntax element is Ceil( Log2( pri_num_resampling_ratios_minus1 + 1 ) ).

The variables priResampleWidthNum[ i ], priResampleWidthDenom[ i ], priResampleHeightNum[ i ]  and  priResampleHeightDenom[ i ]  are derived as follows.

priResampleWidthNum[ i ] = pri_resampling_width_num_minus1[ pri_rsampling_ratio_idx[ i ] ] + 1

priResampleWidthDenom[ i ] =

pri_resampling_width_denom_minus1[ pri_rsampling_ratio_idx[ i ] ] + 1

priResampleHeightNum[ i ] =

pri_resampling_height_num_minus1[ pri_rsampling_ratio_idx[ i ] ] + 1

priResampleHeightDenom[ i ] =

pri_resampling_height_denom_minus1[ pri_rsampling_ratio_idx[ i ] ] + 1

pri_target_region_top_left_x[ i ] and pri_target_region_top_left_y[ i ], when present, indicate the horizontal and vertical positions, respectively, of the top left sample position in luma samples of the i-th region in the reconstructed target picture.

The variables priTargetRegionWidth and priTargetHeight, representing the width and height, respectively, in luma samples of the resampled region in the reconstructed target picture, are derived as follows:

priTargetRegionWidth = Round( ( priRegionWidth[ i ] * priResampleWidthNum[ i ] ) ÷

( priResampleWidthDenom[ i ]  * SubWidthC ) ) * SubWidthC

priTargetRegionHeight = Round( ( priRegionHeight[ i ] * priResampleHeightNum[ i ] ) ÷

( priResampleHeightDenom[ i ] * SubHeightC ) ) * SubHeightC

When reconstructing a target picture with luma sample array of size (pri_target_pic_width_minus1 + 1) × (pri_target_pic_height_minus1 + 1), all luma sample values are initialized to value 1  << ( BitDepthY – 1) and chroma samples, if present, to 1  << ( BitDepthC – 1).

If for any sample position (x,y) and regions j and k the following conditions are all met:

–	pri_region_id[ j ] > pri_region_id[ k ]
–	x is in (priRegionTopLeftX[ j ] .. priRegionTopLeftX[ j ] + priRegionWidth[ j ]) 
–	y is in (priRegionTopLeftY[ j ] .. priRegionTopLeftY[ j ] + priRegionHeight[ j ] 
–	x is in (priRegionTopLeftX[ k ] .. priRegionTopLeftX[ k ] + priRegionWidth[ k]) 
–	y is in (priRegionTopLeftY[ k ] .. priRegionTopLeftY[ k ] + priRegionHeight[ k ]

Then the reconstructed target picture sample at position (x, y) should be determined by the parameters signalled for the j-th region.

8.40 Constituent rectangles SEI message

8.40.1 Constituent rectangles SEI message syntax

| constituent_rectangles( payloadSize ) { | Descriptor |
| --- | --- |
| cr_num_rects_minus1 | u(12) |
| cr_rect_id_present_flag | u(1) |
| if ( cr_rect id_present_flag ) |  |
| cr_rect_id_len | u(4) |
| cr_rect_type_enabled_flag | u(1) |
| cr_rect_type_descriptions_enabled_flag | u(1) |
| cr_subpics_partitioning_flag | u(1) |
| if( !cr_subpics_partitioning_flag ) { | u(1) |
| cr_rect_same_size_flag | u(1) |
| if ( cr_rect_same_size_flag ) { |  |
| cr_num_cols_minus1 | ue(v) |
| cr_num_rows_minus1 | ue(v) |
| } else { |  |
| cr_log2_unit_size | u(4) |
| cr_rect_size_len_minus1 | u(4) |
| } |  |
| } |  |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) { |  |
| if ( cr_rect type_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if ( cr_rect_type_present_flag[ i ] ) |  |
| cr_rect_type_idc[ i ] | u(8) |
| } |  |
| if ( cr_rect type_idc[ i ] != 255 ) { |  |
| if ( cr_rect id_present_flag ) |  |
| cr_rect_id[ i ] | u(v) |
| if ( cr_rect type_description_enabled_flag ) |  |
| cr_rect_type_description_present_flag[ i ] | u(1) |
| } |  |
| if( !cr_subpics_partitioning_flag && !cr_rects_same_size_flag ) { |  |
| cr_rect_top_left_in_units_x[ i ] | u(v) |
| cr_rect_top_left_in_units_y[ i ] | u(v) |
| cr_rect_width_in_units_minus1[ i ] | u(v) |
| cr_rect_height_in_units minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( cr_rect_type_descriptions_enabled_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| cr_bit_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) |  |
| if( rect_type_description_present_flag[ i ] ) |  |
| cr_rect_type_description[ i ] | st(v) |
| } |  |
| } |  |

8.40.2 Constituent rectangles SEI message semantics

The constituent rectangles SEI message enables composition of multiple rectangles within a coded picture and provides information about the rectangles, including ID, type, text description, location, and size.

Use of this SEI message requires the definition of the following variables:

–	A picture width and picture height in units of luma samples, denoted herein by PicWidthInLumaSamples and PicHeightInLumaSamples, respectively.

–	A maximum picture width and maximum picture height in units of luma samples, denoted herein by MaxPicWidth and MaxPicHeight, respectively.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY, and when ChromaFormatIdc is not equal to 0, a bit depth for the samples of the two associated chroma components, denoted herein by BitDepthC.

–	A subpicture count, denoted by NumSubpics.

–	Arrays of the top left X and Y positions of the subpictures, denoted herein by SubPicTopLeftX[ i ] and SubPicTopLeftY[ i ], respectively.

–	Arrays of the width and height of the subpictures, denoted herein by SubPicTopLeftX[ i ] and SubPicTopLeftY[ i ], respectively.

If this SEI message is present in any picture unit that is not the first picture unit of a CLVS in decoding order, a constituent rectangles SEI message with the same payload content shall be present in the first picture unit of the CLVS in decoding order.

cr_num_rects_minus1 plus 1 specifies the number of constituent rectangles for which information is signalled in the SEI message.

cr_rect_id_present_flag equal to 1 specifies that the cr_rect_id[ i ] syntax element is present in the SEI message. cr_rect_id_present_flag equal to 0 specifies that the cr_rect_id[ i ] syntax element is not present in the SEI message.

cr_rect_id_len specifies the length of the cr_rect_id[ i ] syntax element.

cr_rect_type_enabled_flag equal to 1 specifies that the cr_rect_type_present_flag[ i ] syntax element is present in the SEI message. cr_rect_type_enabled_flag equal to 0 specifies that the cr_rect_type_ present_flag[ i ] syntax element is not present in the SEI message.

cr_rect_type_descriptions_enabled_flag equal to 1 specifies that the cr_rect_type_description_present_flag[ i ] syntax element is present in the SEI message. cr_rect_type_descriptions_enabled_flag equal to 0 specifies that the cr_rect_type_description_present_flag[ i ] syntax element is not present in the SEI message.

cr_subpics_partitioning_flag equal to 1 indicates that the subpic partitioning parameters in the SPS are used to determine the of constituent rectangle sizes and positions. cr_subpics_partitioning_flag equal to 0 indicates that determination of the constituent rectangle sizes and positions is not based on subpic partitioning parameters in the SPS.

When cr_subpics_partitioning_flag is equal to 1, it is a requirement of bitstream conformance that NumSubpics be greater than or equal to cr_num_rects_minus1 + 1.

cr_rect_same_size_flag equal to 1 indicates that all constituent rectangles have the same size and are arranged in a grid pattern. cr_rect_same_size_flag equal to 0 indicates that the sizes of the constituent rectangles may differ.

cr_num_cols_minus1 plus 1 and cr_num_rows_minus1 plus 1 specify the number of columns and rows, respectively, of the constituent rectangle grid when cr_rect_same_size_flag equal to 1.

The variable crNumCols is set equal to cr_num_cols_minus1 + 1.

The variable crNumRows is set equal to cr_num_rows_minus1 + 1.

cr_log2_unit_size specifies a unit size used in variable calculations for the constituent rectangle parameters.

The variable crUnitSize is set equal to 1 << cr_log2_unit_size.

cr_rect_size_len_minus1 plus 1 specifies the length of syntax elements cr_rect_top_left_in_units_x[ i ], cr_rect_top_left_in_units_y[ i ], cr_rect_width_in_units_minus1[ i ], and cr_rect_height_in_units minus1[ i ].

cr_rect_type_present_flag[ i ] equal to 1 specifies that the cr_rect_type_idc[ i ] syntax element is present in the SEI message. cr_rect_type_present_flag[ i ] equal to 0 specifies that the cr_rect_type_idc[ i ] syntax element is not present in the SEI message.

cr_rect_type_idc[ i ] indicates the constituent picture type of the i-th rectangle from Table X. When not present and i equal to 0, the value of cr_rect_type_idc[ i ] is inferred to be equal to 0. When not present and i greater than 0, cr_rect_type_idc[ i ] is inferred to be equal to cr_rect_type_idc[ i − 1 ].

Table X – Mapping of cr_rect_type_idc[ i ] to the type of constituent rectangle

| cr_rect_type_idc[ i ] | Name | Type of constituent rectangle |
| --- | --- | --- |
| 0 | CR_TEXTURE | Texture |
| 1 | CR_ALPHA | Alpha plane |
| 2 | CR_DEPTH | Depth picture |
| 3 | CR_OBJECT_MASK | Object mask |
| 4 .. 127 |  | Reserved |
| 128..254 |  | Unspecified |
| 255 | CR_EMPTY | Empty |

cr_rect_id[ i ] indicates the ID of the i-th rectangle. The length of the syntax element is cr_rect_id_len bits. When not present, the value of cr_rect_id[ i ] is inferred to be equal to i.

It is a requirement of bitstream conformance that when j not equal to k, cr_rect_id[ j ] shall not be equal to cr_rect_id[ k ].

cr_rect_type_description_present_flag[ i ] equal to 1 specifies that the cr_rect_type_description[ i ] syntax element is present in the SEI message. cr_rect_type_description_present_flag[ i ] equal to 0 specifies that the cr_rect_type_description[ i ] syntax element is not present in the SEI message. When not present, the value of cr_rect_type_description_present_flag[ i ] is inferred to be equal to 0.

cr_rect_top_left_in_units_x[ i ] and cr_rect_top_left_in_units_y[ i ], when present,  indicate the horizontal and vertical positions, respectively, of the top left position of the i-th constituent picture rectangle in units.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

cr_rect_width_in_units_minus1[ i ] plus 1 and cr_rect_height_in_units_minus1[ i ] plus 1, when present, indicate the width and height, respectively, of the i-th constituent rectangle in units.  The length of the syntax elements are cr_rect_size_len_minus1 + 1.

The variables crRectTopLeftX[ i ] and crRectTopLeftY[ i ], representing the x and y location, respectively, and variables crRectWidth[ i ] and crRectHeight[ i ], representing the width and height, respectively, of the i-th constituent rectangle are derived as follows.

If cr_subpics_partitioning_flag is equal to 0 and cr_rect_same_size_flag is equal to 0, the following applies:

–	The variable crRectTopLeftX[ i ] is set equal to cr_rect_top_left_in_units_x[ i ] * crUnitSize

–	The variable crRectTopLeftY[ i ] is set equal to cr_rect_top_left_in_units_y[ i ] * crUnitSize

–	The variable crRectWidth[ i ] is set equal to ( cr_rect_width_in_units minus1 + 1 ) * crUnitSize

–	The variable crRectHeight[ i ] is set equal to ( cr_rect_height_in_units minus1 + 1 ) * crUnitSize

Otherwise, if cr_subpics_partitioning_flag is equal to 1, the following applies:

–	The variable crRectTopLeftX[ i ] is set equal to SubPicTopLeftX[ i ]

–	The variable crRectTopLeftY[ i ] is set equal to SubPicTopLeftY[ i ]

–	The variable crRectWidth[ i ] is set equal to SubPicWidth[ i ]

–	The variable crRectHeight[ i ] is set to equal to SubPicHeight[ i ]

Otherwise (cr_rect_same_size_flag is equal to 1), the following applies:

–	The variable crRectTopLeftX[ i ] is set equal to (i % crNumCols ) * MaxPicWidth / crNumCols

–	The variable crRectTopLeftY[ i ] is set equal to (i / crNumCols ) * MaxPicHeight / crNumRows

–	The variable crRectWidth [ i ] is set equal to MaxPicWidth / crNumCols

–	The variable crRectHeight [ i ] is set to equal to MaxPicHeight / crNumRows

When PicWidthInLumaSamples is not equal to MaxPicWidth, the following applies:

–	crRectTopLeftX[ i ] is set equal to (crRectTopLeftX[ i ] * PicWidthInLumaSamples + MaxPicWidth / 2 ) / MaxPicWidth

–	crRectWidth[ i ] is set equal to (crRectWidth[ i ] * PicWidthInLumaSamples + MaxPicWidth / 2 ) / MaxPicWidth

When PicHeightInLumaSamples is not equal to MaxPicHeight, the following applies:

–	crRectTopLeftY[ i ] is set equal to (crRectTopLeftY[ i ] * PicWidthInLumaSamples + MaxPicHeight / 2 ) / MaxPicHeight

–	crRectHeight[ i ] is set equal to (crRectHeight[ i ] * PicWidthInLumaSamples + MaxPicHeight / 2 ) / MaxPicHeight

It is a requirement of bitstream conformance that for each sample position (x, y) in the coded picture there shall be at most one rectangle, j, for which both of the following conditions apply:

–	x is in (crRectTopLeftX[ j ] .. crRectTopLeftX[ j ] + crRectWidth[ j ] - 1)

–	y is in (crRectTopLeftY[ j ] .. crRectTopLeftY[ j ] + crRectHeight[ i ] - 1)

It is a requirement of bitstream conformance that crRectTopLeftX[ i ] % SubWidthC shall be equal to 0, crRectTopLeftX [ i ] % SubHeightC shall be equal to 0, crRectWidth[ i ] % SubWidthC shall be equal to 0, and crRectHeight[ i ] % SubHeightC shall be equal to 0.

cr_bit_equal_to_zero shall be equal to 0.

cr_rect_type_description[ i ] specifies a text description of the constituent rectangle. The length of the syntax element shall be less than or equal to 4097 bytes, not including the null termination byte.

8.41 Quality metrics SEI message

8.41.1 Quality metrics SEI message syntax

| quality_metric( payloadSize ) { | Descriptor |
| --- | --- |
| qm_metric_definitions_present_flag | u(1) |
| qm_clvs_values_present_flag | u(1) |
| qm_pic_values_present_flag | u(1) |
| qm_num_metrics_minus1 | u(4) |
| if ( qm_metric_definitions_present_flag ) { |  |
| qm_gain_enabled_flag | u(1) |
| for( i = 0; i <= qm_num_metrics_minus1; i++ ) { |  |
| if ( qm_gain_enabled_flag ) |  |
| qm_gain_flag[ i ] | u(1) |
| if ( qm_gain_flag[ i ] ) |  |
| qm_gain_reference_flag[ i ] | u(1) |
| qm_metric_type[ i ] | u(8) |
| qm_three_component_flag[ i ] | u(1) |
| if ( qm_metric_type[ i ] = = 0 ¦ ¦ qm_metric_type[ i ] >= 128) { |  |
| qm_metric_increasing_flag[ i ] | u(1) |
| qm_full_reference_flag[ i ] | u(1) |
| qm_value_len_minus1_in_bytes[ i ] | u(2) |
| qm_metric_description_present_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| qm_bit_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i <= qm_num_metrics_minus1; i++ ) { |  |
| if ( qm_metric_description_present_flag[ i ] ) |  |
| qm_metric_description[ i ] | st(v) |
| if( qm_clvs_values_present_flag ) |  |
| for( i = 0; i <= qm_num_metrics_minus1; i++ ) |  |
| for( c = 0; c < ( qm_three_component_flag ? 3 : 1 ); c++ ) |  |
| qm_clvs_metric_value[ i ][ c ] | u(v) |
| if( qm_pic_values_present_flag ) |  |
| for( i = 0; i <= qm_num_metrics_minus1; i++ ) |  |
| for( c = 0; c < ( qm_three_component_flag ? 3 : 1 ); c++ ) |  |
| qm_pic_metric_value[ i ][ c ] | u(v) |
| } |  |

8.41.2 Quality metrics SEI message semantics

The quality metrics SEI message signals quality metric values indicating any of the following:

–	The quality of a single picture.

–	The mean quality of all the pictures corresponding to a CLVS.

–	The quality gain of a single picture, which is difference between the quality of a single picture relative to the quality of a gain reference picture.

–	The mean quality gain of all the pictures corresponding to a CLVS.

Use of this SEI message requires the definition of the following variables:

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	A count of pictures NumPics.

–	Lists of picture widths and heights in units of luma samples, denoted herein by PicWidth[ i ] and PicHeight[ i ], respectively, where i is in the range of 0 to NumPics − 1, inclusive.

–	A list of pictures TestPicList[ i ] where i is in the range of 0 to NumPics − 1, inclusive.

–	When any qm_gain_flag[ i ] is equal to 1, a list of gain reference pictures GainRefPicList[ i ] for i is in the range of 0 to NumPics − 1, inclusive.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

TestPicList[ i ][ cIdx ] and ReferencePicList[ i ][ cIdx ] denote the cIdx-th sample array of the i-th picture among TestPicList and ReferencePicList, respectively.

TestPicList[ i ][ cIdx ][ x ][ y ] and ReferencePicList[ i ][ cIdx ][ x ][ y ] denote the sample at location ( x, y ) within the cIdx-th sample array of the i-th picture among TestPicList and ReferencePicList, respectively, where x is in the range of 0 to ( ( cIdx = = 0 ) ? PicWidth[ i ] : PicWidth[ i ] / SubWidthC ) − 1, inclusive and y is in the range of 0 to ( ( cIdx = = 0 ) ? PicHeight[ i ] : PicHeight[ i ] / SubHeightC ) − 1, inclusive.

Let currPicIdx have a value such that the output time of TestPicList[ currPicIdx ] is equal to the output time of the current picture.

qm_metric_definitions_present_flag equal to 1 specifies that information defining the quality metrics is present. qm_metric_definitions_present_flag equal to 0 specifies that information defining the quality metrics is not present.

When this SEI message is the first quality metrics SEI message in a CLVS, in decoding order,  qm_metric_definitions_present_flag shall be equal to 1.

Otherwise, (this SEI message is not the first quality metrics SEI message in a CLVS, in decoding order,)  it is a requirement of bitstream conformance that at least one of the two following conditions shall be met:

–	qm_metric_definitions_present_flag is equal to 0

–	the values of the qm_metric_type[ i ], qm_three_component_flag[ i ],  qm_gain_flag[ i ], qm_gain_reference_flag[ i ], qm_metric_increasing_flag[ i ], qm_full_reference_flag[ i ], qm_value_len_minus1_in_bytes[ i ], qm_metric_description_present_flag[ i ], and qm_metric_description[ i ] syntax elements, when present,  shall be equal to the respective syntax elements in the first quality metric SEI message in the CLVS

qm_clvs_values_present_flag equal to 1 specifies that qm_clvs_metric_value[ i ][ c ] syntax elements are present. qm_clvs_values_present_flag equal to 0 specifies qm_clvs_metric_value[ i ][ c ] syntax elements are not present.

qm_pic_values_present_flag equal to 1 specifies that qm_pic_metric_value[ i ][ c ] syntax elements are present. qm_pic_values_present_flag equal to 0 specifies that qm_pic_metric_value[ i ][ c ] syntax elements are not present.

qm_num_metrics_minus1 plus 1 specifies the number of quality metric entries signaled.

qm_gain_enabled_flag equal to 1 specifies that the qm_gain_flag[ i ] syntax element is present. qm_gain_enabled_flag equal to 0 specifies that the qm_gain_flag[ i ] syntax element is not present.

qm_gain_flag[ i ] and qm_gain_reference_flag[ i ], when present, indicate the interpretation of the values of the qm_clvs_metric_value[ i ][ c ] and qm_pic_metric_value[ i ][ c ] syntax elements in clause 8.41.2.1. When qm_gain_flag[ i ] and qm_gain_reference_flag[ i ] are not present, they are inferred to be equal to 0.

qm_metric_type[ i ] specifies the type of the quality metric of the i-th entry, as specified in Table X. The value of qm_metric_type[ i ] shall be in the range of 0..8, inclusive, or in the range of 128..255, inclusive, in bitstreams conforming to this version of the Specification. Values in the range of of 9..127, inclusive, for qm_metric_type[ i ] are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

When the value of qm_metric_type [ i ] is in the range of 9 .. 127, decoders conforming to this version of this Specification shall ignore all the syntax elements for the i-th entry in this syntax structure.

When the value of qm_metric_type[ i ] is in the range of 128 .. 255, the quality metric type is unspecified or specified by other means not specified in this Specification.

Table X. Interpretation of qm_metric_type[ i ]

| t | Description | IncreasingFlag[t] | FullReferenceFlag[t] | NumBytes[t] |
| --- | --- | --- | --- | --- |
| 0 | User-defined |  |  |  |
| 1 | PSNR [2] | 1 | 1 | 2 |
| 2 | PSNR-YUV [2] | 1 | 1 | 2 |
| 3 | SSIM [2] | 1 | 1 | 2 |
| 4 | MS-SSIM [1] | 1 | 1 | 2 |
| 5 | MOS [1] | 1 | 0 | 1 |
| 6 | wPSNR [2] | 1 | 1 | 2 |
| 7 | WS-PSNR [2] | 1 | 1 | 2 |
| 8 | MSE | 0 | 1 | 2 |
| 9..127 | Reserved |  |  |  |
| 128..255 | Unspecified |  |  |  |

qm_three_component_flag[ i ] equal to 1 indicates that 3 component values will be present for the i-th metric. qm_three_component_flag[ i ] equal to 0 indicates that a single value will be present for the i-th metric. It is a requirement of bitstream conformance that when ChromaFormatIdc is equal to 0, qm_three_component_flag[ i ] shall be equal to 0.

qm_metric_increasing_flag[ i ] equal to 1 indicates that a higher value of the i-th metric value represents an improvement in quality. qm_metric_increasing_flag[ i ] equal to 0 indicates that a lower value of the i-th metric value represents an improvement in quality. When not present, the value of qm_metric_increasing_flag[ i ] is inferred to be equal to IncreasingFlag[qm_metric_type[ i ] ] in Table X.

qm_full_reference_flag[ i ] equal to 1 indicates that the quality metric is a full reference quality metric, calculated by comparison of the pictures in TestPicList with the respective quality reference pictures. qm_full_reference_flag[ i ] equal to 0 indicates that the quality metric may or may not include comparison of the pictures in TestPicList with the respective quality reference picture. When not present, the value of qm_full_reference_flag[ i ] is inferred to be equal to FullReferenceFlag[qm_metric_type[ i ] ]  in Table X.

qm_value_len_minus1_in_bytes[ i ] plus 1 specifies the length in bytes of the qm_pic_metric_value[ i ][ c ] syntax element. When not present, the qm_value_len_minus1_in_bytes[ i ] is inferred to be equal to NumBytes[ [qm_metric_type[ i ] ] – 1.

qm_metric_description_present_flag[ i ] equal to 1 specifies that qm_metric_description[ i ] is present. qm_metric_description_present_flag[ i ]  equal to 0 specifies that qm_metric_description[ i ] is not present.

qm_bit_equal_to_zero shall be equal to 0.

qm_metric_description[ i ] specifies a text description of the i-th quality metric. The length of the syntax element shall be less than or equal to 4097 bytes, not including the null termination byte.

qm_clvs_metric_value[ i ][ c ] specifies the mean value of the i-th quality metric for the c-th component of the CLVS. The length of the syntax element is 8 * (qm_value_len_minus1_in_bytes[ i ] + 1) bits.

qm_pic_metric_value[ i ][ c ] specifies the value of the i-th quality metric for the c-th component of the current picture. The length of the syntax element is 8 * (qm_value_len_minus1_in_bytes[ i ] + 1) bits.

The semantics of the quality metric value are determined by the values of qm_metric_type[ i ], qm_gain_flag[ i ], and qm_gain_reference_flag[ i ].

When qm_pic_values_present_flag is equal to 1, qm_pic_metric_value[ i ][ c ] indicates the picture metric value, picMetricValue[ i ][ c ], of type qm_metric_type[ i ] as described in Table X, and the following applies:

–	If qm_gain_flag[ i ] is equal to 0, qm_pic_metric_value[ i ][ c ] has value derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be TestPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	Otherwise (qm_gain_flag[ i ] is equal to 1), the following applies:

–	picMetricValueTest[ i ][ c ] is set equal to picMetricValue[ i ][ c ] derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be TestPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	picMetricValueGainRef[ i ][ c ] is set equal to picMetricValue[ i ][ c ] derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be GainRefPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	qm_pic_metric_value[ i ][ c ] has the value picMetricValueTest[ i ][ c ] − picMetricValueGainRef[ i ][ c ].

When qm_clvs_values_present_flag is equal to 1, qm_clvs_metric_value[ i ][ c ] indicates the mean value of the picture metric values, listPicMetricValue[ j ][ i ][ c ], calculated over all pictures in TestPicList, of type qm_metric_type[ i ] as described in Table X, where each listPicMetricValue[ j ][ i ][ c ] is derived as follows for each value of j in the range of 0 to NumPics − 1, inclusive:

–	If qm_gain_flag[ i ] is equal to 0, listPicMetricValue[ j ][ i ][ c ] is equal to the picMetrictValue[ i ][ c ] derived by the process specified in clause X.X is performed with testPic, picWidth, and picHeight assigned to be TestPicList[ j ], PicWidth[ j ], and PicHeight[ j ], respectively.

–	Otherwise (qm_gain_flag[ i ] is equal to 1), the following applies:

–	listPicMetricValueTest[ j ][ i ][ c ] is the set equal to picMetricValue[ i ][ c ] derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be TestPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	picMetricValueGainRef[ j ][ i ][ c ] is the set equal to picMetricValue[ i ][ c ] derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be GainRefPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	qm_pic_metric_value[ j ][ i ][ c ] has the value picMetricValueTest[ j ][ i ][ c ] − picMetricValueGainRef[ j ][ i ][ c ].

8.41.2.1 Process for derivation of picMetricValue[ i ][ c ]

Inputs to this process are a tested picture testPic, a picture width picWidth in luma samples, and a picture height picHeight in luma samples.

Let a quality reference picture referencePic be the original picture that was given as input to the encoding system and has the output time equal to the output time of testPic.

testPic[ cIdx ] and referencePic[ cIdx ] denote the cIdx-th sample array of the testPic and referencePic, respectively.

testPic[ cIdx ][ x ][ y ] and referencePic[ cIdx ][ x ][ y ] denote the sample at location ( x, y ) within the cIdx-th sample array of testPic and referencePic, respectively.

The pictures quality metric, picMetricValue[ i ][ c ] is derived as follows:

–	When qm_metric_type[ i ] is equal to 0,

–	listPicMetricValueTest[ j ][ i ][ c ] is the set equal to picMetricValue[ i ][ c ] derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be TestPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	picMetricValueGainRef[ j ][ i ][ c ] is the set qual to picMetricValue[ i ][ c ] derived by the process specified in clause X.X with testPic, picWidth, and picHeight assigned to be GainRefPicList[ currPicIdx ], PicWidth[ currPicIdx ], and PicHeight[ currPicIdx ], respectively.

–	When qm_metric_increasing_flag[ i ] is equal 1, a higher value of picMetricValue[ i ][ c ] indicates testPic is of better quality than for a picture with a lower value of picMetricValue[ i ][ c ].

–	When qm_full_reference_flag[ i ] is equal to 1, of picMetricValue[ i ][ c ] indicates a quality metric value calculated from a comparison of testPic with referencePic.

–	qm_metric_description[ i ] provides a text description of the quality metric indicated by picMetricValue[ i ][ c ].

–	Additional interpretation of picMetricValue[ i ][ c ] is determined by external means not specified in this Specification.

–	When qm_metric_type[ i ] is equal to 1,

–	picMetricValue[ i ][ 0 ] is set equal to the PSNR value calculated using clauses 9.4.2 and D.2 of ISO/IEC 23001-11 [2] for the luma components of testPic and referencePic, with bit depth OrigBitDepth, width picWidth, and height picHeight.

–	When qm_three_component_flag[ i ] is equal to 1,

–	picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ] are set equal to the PSNR values calculated using clauses 9.4.2 and D.2 of ISO/IEC 23001-11 [2] for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth width picWidth/SubWidthC, and height picHeight/SubHeightC.

–	When qm_metric_type[ i ] is equal to 2,

–	picMetricValue[ i ][ 0 ] is set equal to the value of the variable psnrYUV calculated as follows:

–	The variable psnrY is set equal to the PSNR value calculated using clauses 9.4.2 and D.2 of ISO/IEC 23001-11 [2]  for the luma components of testPic and referencePic, with bit depth OrigBitDepth width picWidth, and height picHeight.

–	The variables psnrU and psnrV are set equal to the PSNR values calculated using clauses 9.4.2 and D.2 of ISO/IEC 23001-11 [2] for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth, width picWidth/SubWidthC, and height picHeight/SubHeightC.

psnrYUV =  (10*psnrY + psnrU + psnrV ) ÷ 12

–	When qm_metric_type[ i ] is equal to 3,

–	picMetricValue[ i ][ 0 ] is set equal to the value of SSIM calculated from clauses 9.4.2 and D.5 of ISO/IEC 23001-11 [2] for the luma components of testPic and referencePic, with bit depth OrigBitDepth width picWidth, and height picHeight.

–	When qm_three_component_flag[ i ] is equal to 1,

–	picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the SSIM values calculated from clauses 9.4.2 and D.5 of ISO/IEC 23001-11 [2] for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth, width picWidth/SubWidthC, and height picHeight/SubHeightC.

–	When qm_metric_type[ i ] is equal to 4,

–	picMetricValue[ i ][ 0 ] is set equal to the value of MS-SSIM calculated from clause 4.3.3 of ISO/IEC 23001-10 [1]  for the luma components of testPic and referencePic, with bit depth OrigBitDepth.

–	When qm_three_component_flag[ i ] is equal to 1,

–	picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the MS-SSIM values calculated from clause 4.3.3 of ISO/IEC 23001-10 [1]  for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth.

–	When qm_metric_type[ i ] is equal to 5,

–	picMetricValue[ i ][ 0 ] is set equal to the value of MOS specified in clause 4.3.6 of ISO/IEC 23001-10 [1].

–	When qm_metric_type[ i ] is equal to 6,

–	picMetricValue[ i ][ 0 ] is set equal to the value of wPSNR calculated from clauses 9.4.2 and D.3 of ISO/IEC 23001-11 [2]  for the luma components of testPic and referencePic, with bit depth OrigBitDepth width picWidth, and height picHeight.

–	When qm_three_component_flag[ i ] is equal to 1,

–	picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the wPSNR values calculated from clauses 9.4.2 and D.3 of ISO/IEC 23001-11 [2] for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth, width picWidth/SubWidthC, and height picHeight/SubHeightC.

–	When qm_metric_type[ i ] is equal to 7,

–	picMetricValue[ i ][ 0 ] is set equal to the value of WS-PSNR calculated from clauses 9.4.2 and D.4 of ISO/IEC 23001-11 [2] for the luma components of testPic and referencePic, with bit depth OrigBitDepth, width picWidth, and height picHeight.

–	When qm_three_component_flag[ i ] is equal to 1,

–	picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the WS-PSNR values calculated from clauses 9.4.2 and D.4 of ISO/IEC 23001-11 [2]for the Cb and Cr components, respectively, of testPic and referencePic, with bit depth OrigBitDepth, width picWidth/SubWidthC, and height picHeight/SubHeightC.

–	When qm_metric_type[ i ] is equal to 8,

–	picMetricValue[ i ][ 0 ] is set equal to the value of lumaMse, interpreted as a floating-point value, derived as follows:

lumaSse = 0

for( y = 0; y < picHeight; y++)
	for( x = 0; x < picWidth; x++ ) 
		lumaSse += (testPic[ 0 ][ x ][ y ] – referencePic[ 0 ][ x ][ y ])2

lumaMse = ( lumaSse ÷ (CroppedHeight * CroppedWidth) )/ 100

–	When qm_three_component_flag[ i ] is equal to 1, picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ]  are set equal to the values of CbMse and CrMse, respectively, interpreted as floating-point values, derived as follows:

CbSse = 0

CrSse = 0

for( y = 0; y < picHeight / SubWidthC; y++)
	for( x = 0; x < picWidth / SubWidthC; x++ ) {
		CbSse += (testPic[1][y][x] – referencePic[1][y][x])2

CbSse += (testPic[1][y][x] – referencePic[1][y][x])2

}

CbMse = ( CbSse ÷ ( picHeight * CroppedWidth / (SubWidthC * SubWidthC) ) / 100

CrMse = ( CrSse ÷ ( picHeight * CroppedWidth / (SubWidthC * SubWidthC) ) / 100

# Changes to VVC subclause D.11.1.2 and addition of subclause D.11.3 for specifying handling of processing chains specified by an SEI processing order SEI message

#### D.11.1.2 SEI processing order SEI message semantics

…

SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 137, 142, 144, 147, 148, 149, 150, 153, 155, 165, 177, 210, and 211. The value of po_sei_payload_type[ i ] for each i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive, shall be equal to a value in SeiProcessingOrderSeiList.

SpoProcessingList is set to consist of the payloadType values 19, 142, 155, 210, and 211.

...

### D.11.3 Handling of a processing chain

Processing chains are alternative to each other, i.e., at most one processing chain can be chosen to be applied by a decoding system at one time.

A decoding system may choose and apply a processing chain as follows:

–	First, the bitstream is decoded, and the list PoPicList is set to be the list of the cropped decoded pictures in output order that resulted from decoding the bitstream, and a processing chain is chosen.

–	(Option 1: picture-by-picture for one filter, zig zag, breadth first) For each of the SEI message types of the chosen processing chain, the following applies in a non-decreasing order of the corresponding po_sei_processing_order[ i ] values:

–	The following applies for each picture picA in PoPicList in output order, when an SEI message associated with the i-th SEI message type persists for picA or a picture for which an NNPF that generated picA was activated by a preceding process in the processing chain:

–	When picA is not a cropped decoded picture, the following exceptions apply for the interpretation of the SEI message:

–	The interface variables for purposes of interpretation of the SEI message are derived from picA instead of the syntax elements indicating properties for the respective cropped decoded picture.

–	The semantics of the SEI message, or the semantics of the SEI message and, when the SEI message is an NNPFA SEI message, the associated NNPFC SEI message, apply to pictures in PoPicList instead of cropped decoded pictures.

–	When the i-th SEI message type is present in SpoProcessingList, the process implied by the SEI message is performed and PoPicList is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the process and inserting the other pictures, if any, resulting from the process into PoPicList so that the output order is obeyed.

–	(Option 2: filter-by-filter for one picture, zag zip, depth first) The following is repeatedly applied, in output order, for each picture picA in PoPicList for which a set of SEI messages associated with SEI message types in SpoProcessingList of the chosen processing chain persist for picA, the following applies:

–	The following applies for each of the set of SEI messages in a non-decreasing order of the corresponding po_sei_processing_order[ i ] values:

–	When the current SEI message is not the first in the set of the SEI messages, the following exceptions apply for the interpretation of the SEI message:

–	The interface variables for purposes of interpretation of the SEI message are derived from the pictures in the updated PoPicList instead of the syntax elements indicating properties for the respective cropped decoded pictures.

–	The semantics of the SEI message, or of the SEI message and, when the SEI message is an NNPFA SEI message, the associated NNPFC SEI message, apply to pictures in PoPicList instead of cropped decoded pictures.

–	The process implied by the SEI message is invoked repeatedly, in output order, for picA and each of the pictures in PoPicList that are, or correspond to, interpolated or extrapolated pictures generated by the application of the process implied by any preceding SEI message, if any, to picA. After each invocation of the process, PoPicList is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the process and inserting the other pictures, if any, resulting from the process into PoPicList so that the output order is obeyed.

# Changes to VVC subclause D.12 for adding the support of packed regions, constitiuent rectangles, and quality metrics SEI messages

### D.12.11	Use of the packed regions information SEI message in VVC

For purposes of interpretation of the packed regions information SEI message, the following variables are specified:

–	PicWidthInLumaSamples is set equal to the value of pps_pic_width_in_luma_samples − ‌SubWidthC * ‌ ( pps_conf_win_left_offset + pps_conf_win_right_offset ).

–	PicHeightInLumaSamples is set equal to the value of pps_pic_height_in_luma_samples − ‌SubHeightC * ‌ ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ).

–	MaxPicWidth is set equal to the value of sps_pic_width_in_luma_samples − ‌SubWidthC * ‌ ( sps_conf_win_left_offset + sps_conf_win_right_offset ).

–	MaxPicHeight is set equal to the value of sps_pic_height_in_luma_samples − ‌SubHeightC * ‌ ( sps_conf_win_top_offset + sps_conf_win_bottom_offset ).

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

### D.12.13 Use of the constituent rectangles SEI message in VVC

For purposes of interpretation of the constituent rectangles SEI message, the following variables are specified:

–	PicWidthInLumaSamples and PicHeightInLumaSamples are set equal to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, respectively.

–	MaxPicWidth and MaxPicHeight are set equal to sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples, respectively.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	NumSubpics is set equal to sps_num_subpics_minus1 + 1

–	SubPicTopLeftX[ i ] is set equal to sps_subpic_ctu_top_left_x[ i ] * CtbSizeY

–	SubPicTopLeftY[ i ] is set equal to sps_subpic_ctu_top_left_y[ i ] * CtbSizeY

–	SubPicWidth[ i ] is set equal to (sps_subpic_width_minus1[ i ] + 1 ) * CtbSizeY − 1

–	SubPicHeight[ i ] is set equal to (sps_subpic_height_minus1[ i ] + 1 ) * CtbSizeY – 1

D.12.14 Use of the quality metric SEI message in VVC

For purposes of interpretation of the quality metric SEI message, the derivation of the variables ChromaFormatIdc, NumPics, TestPicList, PicWidth, PicHeight, and GainRefPicList is specified below.

Let TestPicList initially consist of the cropped decoded pictures of the current CLVS in output order.

When the quality metric SEI message is included as a type of an SEI message in an SEI processing order SEI message, any quality metric SEI message that is associated with the SEI processing order SEI message shall be included in a processing order nesting SEI message.

When the quality metric SEI message is included as the i-th type of an SEI message in an SEI processing order SEI message, the following applies:

–	It is a requirement of bitstream conformance that an SEI message seiB that implies post-processing to be performed shall be present as the j-th type of an SEI message in the same SEI processing order SEI message and po_sei_processing_order[ j ] shall be equal to po_sei_processing_order[ i ].

–	The quality metric SEI message indicates the picture quality resulting from the post-processing implied by seiB.

–	TestPicList is updated as follows for each post-processing stage with po_sei_processing_order[ j ] less than or equal to po_sei_processing_order[ i ] in a non-decreasing order of j.

–	When the post-processing stage results into a picture picA with an output time that is equal to the output time of a picture picB in TestPicList, picB in TestPicList is replaced by picA.

–	When the post-processing stage results into a picture picA with an output time that is not equal to the output time of any picture in TestPicList, picA is inserted in TestPicList in a manner that pictures in TestPicList remain in output order.

NumPics is set equal to the count of pictures in TestPicList.

PicWidth[ i ] and PicHeight[ i ] are set equal to the width and height of TestPicList[ i ], respectively, in luma samples.

When a quality metric SEI message is included as the k-th SEI message in a processing order nesting SEI message and qm_gain_flag[ i ] is equal to 1 for any value of i, the following applies:

–	If qm_gain_reference_flag[ i ] is equal to 0, the i-th metric value in the quality metric SEI message represents a gain of the post-processing stage with po_sei_processing_order[ j ] equal to pon_processing_order[ k ] relative to the picture or pictures used as input to that post-processing stage and GainRefPicList is set equal to TestPicList derived for the processing stages up to but not including the processing stage with po_sei_processing_order[ j ].

–	Otherwise (qm_gain_reference_flag[ i ] is equal to 1), the i-th metric value in the quality metric SEI message represents a cumulative gain of all the post-processing stages with po_sei_processing_order[ j ] less than or equal to pon_processing_order[ k ] relative to the cropped decoded picture or pictures and GainRefPicList consists of the cropped decoded pictures of the current CLVS in output order.

–	It is a requirement of bitstream conformance that the count of pictures in GainRefPicList shall be equal to NumPics and the width and height of GainRefPicList[ i ] in luma samples shall be equal to PicWidth[ i ] and PicHeight[ i ], respectively.

The value of ChromaFormatIdc is derived as follows:

–	If the quality metric SEI message is not included in a processing order nesting SEI message, ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	Otherwise (the quality metric SEI message is included in a processing order nesting SEI message), ChromaFormatIdc is set to a value that matches the chroma format of the pictures in TestPicList and it is a requirement of bitstream conformance that the chroma format of all the pictures in TestPicList and GainRefPicList, when present, shall be identical.

When a quality metric SEI message qmSeiA is present in a picture unit that is not the first picture unit of a CLVS in decoding order and at least one value of qm_clvs_metric_value[ i ][ c ] is present, the following applies:

–	If qmSeiA is not included in a processing order nesting SEI message, each value of qm_clvs_metric_value[ i ][ c ] shall be equal to the value of qm_clvs_metric_value[ i ][ c ] in the quality metric SEI message that is present in the first picture unit of the CLVS and is not is included in a processing order nesting SEI message.

–	Otherwise (qmSeiA is included in a processing order nesting SEI message with a certain set of pon_target_po_id[ i ] values), the following applies:

–	It is a requirement of bitstream conformance that there shall be quality metric SEI message qmSeiB that is present in the first picture unit of the CLVS and is included in a processing order nesting SEI message with the same set of pon_target_po_id[ i ] values.

–	Each value of qm_clvs_metric_value[ i ][ c ] in qmSeiA shall be equal to the value of qm_clvs_metric_value[ i ][ c ] in qmSeiB.

# Changes to HEVC for adding support for modality information SEI message

## D.2 SEI payload syntax

### D.2.1 General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| else if( payloadType = = 210 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_characteristics( payloadSize ) |  |
| else if( payloadType = = 211 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_activation( payloadSize ) |  |
| else if( payloadType = = 212 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| phase_indication( payloadSize ) |  |
| … |  |
| else if( payloadType = = 218) |  |
| modality_info( payloadSize ) |  |
| … |  |
| } |  |

## D.3 SEI payload semantics

### D3.1 General SEI payload semantics

The list SingleLayerSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, 205, 210 to 212, inclusive, and 218.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, 205, 210 to 212, inclusive, and 218.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, 205, 210 to 212, inclusive, and 218.

# Changes to AVC for adding support for modality information SEI message

### D.1 SEI payload syntax

### D.1.1 General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| else if( payloadType = = 210 ) |  |  |
| nn_post_filter_characteristics( payloadSize ) | 5 |  |
| else if( payloadType = = 211 ) |  |  |
| nn_post_filter_activation( payloadSize ) | 5 |  |
| else if( payloadType = = 212 ) |  |  |
| phase_indication( payloadSize ) | 5 |  |
| … |  |  |
| else if( payloadType = = 218 ) |  |  |
| modality_info( payloadSize ) | 5 |  |
| else |  |  |
| reserved_sei_message( payloadSize ) | 5 |  |
| if( !byte_aligned( ) ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 5 | f(1) |
| while( !byte_aligned( ) ) |  |  |
| bit_equal_to_zero /* equal to 0 */ | 5 | f(1) |
| } |  |  |
| } |  |  |

## G.13 Supplemental enhancement information

The specifications in Annex ‎D together with the extensions and modifications specified in this clause apply.

### G.13.2 SEI payload semantics

The semantics of the SEI messages with payloadType in the range of 0 to 23, inclusive, or equal to 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 218, which are specified in clause ‎D.2, are extended as follows:

–	If payloadType is equal to 3, 8, 19, 20, or 22, the following applies:

–	If the SEI message is not included in a scalable nesting SEI message, it applies to the layer representations of the current access unit that have dependency_id equal to 0 and quality_id equal to 0.

The semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to 0 and qIdTarget equal to 0. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for layer representations with dependency_id equal to 0 and quality_id equal to 0. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to layer representations with dependency_id equal to 0 and quality_id equal to 0.

–	Otherwise (the SEI message is included in a scalable nesting SEI message), the SEI message applies to all layer representations of the current access unit for which DQId is equal to any value of ( ( sei_dependency_id[ i ] << 4 ) + sei_quality_id[ i ] ) with i in the range of 0 to num_layer_representations_minus1, inclusive.

For each value of i in the range of 0 to num_layer_representations_minus1, inclusive, the semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to sei_dependency_id[ i ] and qIdTarget equal to sei_quality_id[ i ]. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for layer representations with dependency_id equal to sei_dependency_id[ i ] and quality_id equal to sei_quality_id[ i ]. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to layer representations with dependency_id equal to sei_dependency_id[ i ] and quality_id equal to sei_quality_id[ i ].

–	Otherwise, if payloadType is equal to 2, 6, 7, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 21, 23, 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 218, the following applies:

–	If the SEI message is not included in a scalable nesting SEI message, it applies to the dependency representations of the current access unit that have dependency_id equal to 0.

The semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to 0. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for dependency representations with dependency_id equal to 0. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to dependency representations with dependency_id equal to 0.

–	Otherwise (the SEI message is included in a scalable nesting SEI message), the scalable nesting SEI message containing the SEI message shall have all_layer_representations_in_au_flag equal to 1 or, when all_layer_representations_in_au_flag is equal to 0, all values of sei_quality_id[ i ] present in the scalable nesting SEI message shall be equal to 0. The SEI message that is included in the scalable nesting SEI message applies to all dependency representations of the current access unit for which dependency_id is equal to any value of sei_dependency_id[ i ] with i in the range of 0 to num_layer_representations_minus1, inclusive.

For each value of i in the range of 0 to num_layer_representations_minus1, inclusive, the semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to sei_dependency_id[ i ]. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for dependency representations with dependency_id equal to sei_dependency_id[ i ]. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to dependency representations with dependency_id equal to sei_dependency_id[ i ].

When payloadType is equal to 10 for the SEI message that is included in a scalable nesting SEI message, the semantics for sub_seq_layer_num of the sub-sequence information SEI message is modified as follows:

sub_seq_layer_num specifies the sub-sequence layer number of the current picture. When the current picture resides in a sub-sequence for which the first picture in decoding order is an IDR picture, the value of sub_seq_layer_num shall be equal to 0. For a non-paired reference field, the value of sub_seq_layer_num shall be equal to 0. sub_seq_layer_num shall be in the range of 0 to 255, inclusive.

–	Otherwise, if payloadType is equal to 0 or 1, the following applies:

–	If the SEI message is not included in a scalable nesting SEI message, the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 not included in a scalable nesting SEI message are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex ‎C and the decoding process specified in clauses ‎2 to ‎9 is used, the bitstream shall be conforming to this Recommendation | International Standard.

The value of seq_parameter_set_id in a buffering period SEI message not included in a scalable nesting SEI message shall be equal to the value of seq_parameter_set_id in the picture parameter set that is referenced by the layer representation with DQId equal to 0 of the primary coded picture in the same access unit.

–	Otherwise (the SEI message is included in a scalable nesting SEI message), the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 included in a scalable nesting SEI message with identical values of sei_temporal_id, sei_dependency_id[ i ], and sei_quality_id[ i ] are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex ‎C, the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with tIdTarget equal to sei_temporal_id, dIdTarget equal to sei_dependency_id[ i ], and qIdTarget equal to sei_quality_id[ i ] shall be conforming to this Recommendation | International Standard.

In the semantics of clauses ‎D.2.1 and ‎D.2.3, the syntax elements num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag and the derived variables NalHrdBpPresentFlag, VclHrdBpPresentFlag, and CpbDpbDelaysPresentFlag are substituted with the syntax elements vui_ext_num_units_in_tick[ i ], vui_ext_time_scale[ i ], vui_ext_fixed_frame_rate_flag[ i ], vui_ext_nal_hrd_parameters_present_flag[ i ], vui_ext_vcl_hrd_parameters_present_flag[ i ], vui_ext_low_delay_hrd_flag[ i ], and vui_ext_pic_struct_present_flag[ i ] and the derived variables VuiExtNalHrdBpPresentFlag[ i ], VuiExtVclHrdBpPresentFlag[ i ], and VuiExtCpbDpbDelaysPresentFlag[ i ].

The value of seq_parameter_set_id in a buffering period SEI message included in a scalable nesting SEI message with the values of sei_dependency_id[ i ] and sei_quality_id[ i ] shall be equal to the value of seq_parameter_set_id in the picture parameter set that is referenced by the layer representation with DQId equal to (( sei_dependency_id[ i ] << 4 ) + sei_quality_id[ i ]) of the primary coded picture in the same access unit.

–	Otherwise (payloadType is equal to 4 or 5), the corresponding SEI message semantics are not extended.

When an SEI message having a particular value of payloadType equal to 137 or 144, contained in a scalable nesting SEI message, and applying to a particular combination of dependency_id, quality_id, and temporal_id is present in an access unit, the SEI message with the particular value of payloadType applying to the particular combination of dependency_id, quality_id, and temporal_id shall be present a scalable nesting SEI message in the IDR access unit that is the first access unit of the coded video sequence.

All SEI messages having a particular value of payloadType equal to 137 or 144, contained in scalable nesting SEI messages, and applying to a particular combination of dependency_id, quality_id, and temporal_id present in a coded video sequence shall have the same content.

For the semantics of SEI messages with payloadType in the range of 0 to 23, inclusive, or equal to 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 218, which are specified in clause ‎D.2, SVC sequence parameter set is substituted for sequence parameter set; the parameters of the picture parameter set RBSP and SVC sequence parameter set RBSP that are in effect are specified in clause ‎G.7.4.1.2.1.

Coded video sequences conforming to one or more of the profiles specified in Annex ‎G shall not include SEI NAL units that contain SEI messages with payloadType in the range of 36 to 44, inclusive, or equal to 46, which are specified in clause ‎H.13, or with payloadType in the range of 48 to 53, inclusive, which are specified in clause ‎I.13.

When an SEI NAL unit contains an SEI message with payloadType in the range of 24 to 35, inclusive, which are specified in clause ‎G.13, it shall not contain any SEI message that has payloadType less than 24 or equal to 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 218 that is not included in a scalable nesting SEI message, and the first SEI message in the SEI NAL unit shall have payloadType in the range of 24 to 35, inclusive.

When an SEI NAL unit contains an SEI message with payloadType equal to 24, 28, or 29, it shall not contain any SEI message with payloadType not equal to 24, 28, or 29.

When a scalable nesting SEI message (payloadType is equal to 30) is present in an SEI NAL unit, it shall be the only SEI message in the SEI NAL unit.

The semantics for SEI messages with payloadType in the range of 24 to 35, inclusive, are specified in the following.

# Addition(s) related to large SEI message and versatile SEI RBSP / versatile SEI message

[Ed: Proponents of both approaches are asked to work out possible combination]

X.1 Image format metadata SEI messages

X.1.1	Large supplemental enhancement information message syntax

| lsei_message( ) { | Descriptor |
| --- | --- |
| lsei_position | u(2) |
| lsei_relevance | u(2) |
| lsei_reserved | u(4) |
| lsei_payload_type_byte | u(8) |
| lsei_payload_size_16bits | u(16) |
| lsei_payload( lseiPayloadType, lseiPayloadSize ) |  |
| } |  |

X.1.2	Large supplemental enhancement information message semantics

Each large SEI message consists of the variables specifying the type payloadType and size payloadSize of the large SEI message payload. Large SEI message payloads are specified in Annex D. The derived large SEI message payload size payloadSize is specified in bytes and shall be equal to the number of RBSP bytes in the large SEI message payload.

NOTE – The NAL unit byte sequence containing the large SEI message might include one or more emulation prevention bytes (represented by emulation_prevention_three_byte syntax elements). Since the payload size of a large SEI message is specified in RBSP bytes, the quantity of emulation prevention bytes is not included in the size payloadSize of a large SEI payload.

lsei_position indicates if the SEI message corresponds to the PREFIX_SEI_NUT and SUFFIX_SEI_NUT.  lsei_position equal 0 indicates that the SEI message is treated as PREFIX_SEI_NUT. lsei_position equal 1 indicates that the SEI message is treated as SUFFIX_SEI_NUT. Values 3 and 4 of lsei_position are reserved for future use and shall be ignored.

lsei_relevance indicates the relevance of the SEI message for the target application. lsei_relevance ranges from 0 to 3, with 0 being the least relevant and 3 being the most relevant.

NOTE – The relevance of an SEI message is an arbitrary decision and its use is to be specified by the target application.

lsei_reserved is reserved for future use and shall be ignored.

lsei_payload_type_byte is a byte of the payload type of a large SEI message. payloadType = lsei_payload_type_byte.

payload_size_16bits is the payload size in bits of a large SEI message. payloadSize = payload_size_16bits.

X.2 Versatile SEI RBSP

X.2.1	Versatile SEI RBSP syntax

| vsei_rbsp( ) { | Descriptor |
| --- | --- |
| vsei_importance | u(1) |
| vsei_reserved | u(7) |
| versatile_sei_message() |  |
| rbsp_trailing_bits( ) |  |
| } |  |

X.2.2	Versatile SEI RBSP semantics

Versatile Supplemental enhancement information RBSP contains information that is not necessary to decode the samples of coded pictures from VCL NAL units. A VSEI RBSP contains one VSEI message.

vsei_importance equal 1 indicates that the versatile SEI message may be an important or required. Vsei_importance equal 0 indicates that the versatile SEI message does not have particular importance.

NOTE – Entity that is aware of this flag may use this information when it needs to make decision whether or not to deliver / drop the verstatile SEI message.

vsei_reserved is reserved for future use and shall be ignored.

X.2.3	Versatile SEI message syntax

| versatile_sei_message( ) { | Descriptor |
| --- | --- |
| vsei_payload_type_byte | u(8) |
| vsei_payload( vseiPayloadType ) |  |
| } |  |

X.2.4	Versatile SEI message semantics

Versatile Supplemental enhancement information RBSP contains information that is not necessary to decode the samples of coded pictures from VCL NAL units. A VSEI RBSP contains one VSEI message.

Each versatile SEI message consists of the variables specifying the importance and type payloadType of the SEI message payload. Versatile SEI message payloads are specified in XXX.

NOTE – The NAL unit byte sequence containing the SEI message might include one or more emulation prevention bytes (represented by emulation_prevention_three_byte syntax elements).

vsei_payload_type_byte is a byte of the payload type of a versatile SEI message.

# Addition(s) related to alternative persistence signalling in SEI messages

X.1 Example SEI message to describe alternative persistence signalling for SEI messages

X.1.1	Example SEI message without an ID syntax

| example( payloadSize ) { | Descriptor |
| --- | --- |
| ex_persistence_idc | u(2) |
| if( ex_persistence_idc > 0 ) { |  |
| /* additional syntax elements */ |  |
| } |  |
| } |  |

X.1.2	Example SEI message without an ID semantics

ex_persistence_idc specifies the persistence of the example SEI message for the current layer.

ex_persistence_idc equal to 0 indicates that the example SEI message cancels the persistence of any previous example SEI message in output order that applies to the current layer of the current example SEI message.

ex_persistence_idc equal to 1 specifies that the example SEI message applies to the current decoded picture only.

ex_persistence_idc equal to 2 specifies that the example SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an example SEI message that follows the current picture in output order.

ex_persistence_idc equal to 3 specifies that the example SEI message applies for the entire CLVS.

It is a requirement of bitstream conformance that when ex_persistence_idc is equal to 3 and this SEI message is not the first example SEI message in a CLVS, in decoding order, an example SEI message shall be present in the first PU of the CLVS in decoding order.

When ex_persistence_idc is equal to 3 in the first PU in the CLVS in decoding order, all example SEI messages in the CLVS shall have identical SEI payload content.

X.1.1.3	Example SEI message with an ID syntax

| example( payloadSize ) { | Descriptor |
| --- | --- |
| ex_id | u(6) |
| ex_persistence_idc | u(2) |
| if( ex_persistence_idc > 0 ) { |  |
| /* additional syntax elements */ |  |
| } |  |
| } |  |

X.1.4	Example SEI message with an ID semantics

ex_id specifies an identifier of the example SEI message.

ex_persistence_idc specifies the persistence of the example SEI message for the current layer.

ex_persistence_idc equal to 0 indicates that the example SEI message cancels the persistence of any previous example SEI message in output order that applies to the current layer and has the same ex_id as the current example SEI message.

ex_persistence_idc equal to 1 specifies that the example SEI message applies to the current decoded picture only.

ex_persistence_idc equal to 2 specifies that the example SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an example SEI message with the same ex_id is output that follows the current picture in output order.

ex_persistence_idc equal to 3 specifies that the example SEI message applies for the entire CLVS.

It is a requirement of bitstream conformance that when ex_persistence_idc is equal to 3 and this SEI message is not the first example SEI message in a CLVS, in decoding order, an example SEI message with the same value of ex_id shall be present in the first PU of the CLVS in decoding order.

All example SEI messages in a CLVS that have a particular ex_id value and ex_persistence_idc equal to 3 shall have identical SEI payload content.

When ex_persistence_idc is equal to 3 in the first PU in the CLVS in decoding order, all example SEI messages in the CLVS that have a particular ex_id value shall have identical SEI payload content.

https://www.dublincore.org