| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0186-v1 |
| --- | --- |

| Title: | AHG9: On picture order count and timing information for gfv generated pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen Olena Chubach Yu-Wen Huang Shawmin Lei | Email: | {lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

This contribution as a follow-up of JVET-AH0137 proposes to add picture order count information in addition to the timing information for the GFV generated pictures and proposes to unify the DPB management for the decoded pictures and the GFV generated pictures in terms of picture order count.

# Introduction

In the recent JVET meeting, some improvements on the GFV SEI message are include in JVET-AH2032. The updated text of the GFV SEI message in JVET-AH2032 is as below.

8.36 Generative face video SEI message

8.36.1 Generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| if( gfv_cnt = = 0 ) |  |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| gfv_nn_present_flag | u(1) |
| if( gfv_nn_present_flag ) { |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to GenerativeNN( )*/ | ue(v) |
| … |  |
| } |  |

8.36.2 Generative face video SEI message semantics

The generative face video (GFV) SEI message indicates facial parameters and specifies a facial parameter translator network, denoted as TranslatorNN( ), that may be used to convert various formats of facial parameters signaled in the SEI message into a fixed format of parameters, and a face picture generator neural network, denoted as GenerativeNN( ), that may be used to generate output pictures using the fixed format of facial parameters and previously decoded output pictures.

…

gfv_id contains an identifying number that may be used to identify face feature information and specify a neural network that may be used as TranslatorNN( ). The value of gfv_id shall be in the range of 0 to 232–− 2, inclusive. Values of gfv_id from 256 to 511, inclusive, and from 231 to 232–− 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering a GFV SEI message with gfv_id in the range of 256 to 511, inclusive, or in the range of 231 to 232–− 2, inclusive, shall ignore the SEI message.

gfv_cnt specifies a GFV SEI message instance count value for this gfv_id value within a picture unit.

The gfv_cnt of the first GFV SEI message, in decoding order, with a particular value of gfv_id within picture unit shall be equal to 0. When gfv_cnt assigned to currGfvCnt is greater than 0, a GFV SEI message with the same gfv_id value and gfv_cnt equal to currGfvCnt − 1 shall precede the current GFV SEI message in decoding order in the same picture unit.

The value of gfv_cnt shall be in the range of 0 to 65 535, inclusive.

…

The previous contribution JVET-AH0137 proposes, when gfv_cnt > 0, to indicate, timing information for the pictures generated from the GFV SEI messages. One gating flag and two timing-related syntax elements are signalled. The comment on JVET-AH0137 in the AH meeting is: “The proposed syntax requires 65 additional bits, which is significant. Is there a more bitrate efficient solution? Further study encouraged to find a more bitrate efficient solution.”

In JVET-AH0137, the 1-bit gating flag is used to indicate whether the timing is extrapolated from the timing of the decoded picture within the PU that contains the current GFV SEI message, or the timing with two syntax elements follows. In most cases, the flag is equal to 1. Only when the timing gets changed by encoding process, the full length (32 bit) signalling is needed for the two syntax elements. Therefore, it only consumes one bit in most cases.

# Proposal

This contribution as a follow-up of JVET-AH0137 proposes to add picture order count information in addition to the timing information for the GFV generated pictures and proposes to unify the DPB management for the decoded pictures and the GFV generated pictures in terms of picture order count as in the decoding process, such as VVC that the picture order count is used for output picture order.

The proposed text relative to JVET-AH2032 with the highlighted is as follows.

8.36 Generative face video SEI message

8.36.1 Generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| if( gfv_cnt = = 0 ) |  |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| else { /* gfv_cnt > 0 ) add timing info */ |  |
| gfv_timing_extrapolates_pic_timing_flag | u(1) |
| if( !gfv_timing_extrapolates_pic_timing_flag ) { |  |
| gfv_time_scale | u(32) |
| gfv_num_units_in_pic_interval | u(32) |
| } |  |
| } |  |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| … |  |
| } |  |

8.36.2 Generative face video SEI message semantics

The generative face video (GFV) SEI message indicates facial parameters and specifies a facial parameter translator network, denoted as TranslatorNN( ), that may be used to convert various formats of facial parameters signaled in the SEI message into a fixed format of parameters, and a face picture generator neural network, denoted as GenerativeNN( ), that may be used to generate output pictures using the fixed format of facial parameters and previously decoded output pictures.

…

gfv_cnt specifies a GFV SEI message instance count value for this gfv_id value within a picture unit. A value of gfv_cnt > 0 also specifies the difference between the picture order count of the picture in a PU containing this GFV SEI message and the picture indicated by this GFV SEI message.

The gfv_cnt of the first GFV SEI message, in decoding order, with a particular value of gfv_id within picture unit shall be equal to 0. When gfv_cnt assigned to currGfvCnt is greater than 0, a GFV SEI message with the same gfv_id value and gfv_cnt equal to currGfvCnt − 1i, i > 0,[PP: the picture order count values may not be consecutive] shall precede the current GFV SEI message in decoding order in the same picture unit.

The value of gfv_cnt shall be in the range of 0 to 65 535, inclusive.

When the picture is a coded picture picA in a PU containing this GFV SEI message with gfv_cnt > 0, the variable gfvGeneratedPocVal is derived as follows:

gfvGeneratedPocVal = PicOrderCntVal + gfv_cnt

The picture indicated by the GFV SEI message that follows picA is associated with variable PicOrderCntVal derived as:

PicOrderCntVal = gfvGeneratedPocVal

In decoding process, the decoded pictures and the GFV generated pictures are marked as “needed for output” and stored in the DPB with the variable values of PicOrderCntVal for each of the pictures. The “bumping” process is applied as follows:

the picture or pictures that are first for output are selected as the one having the smallest value of PicOrderCntVal of all pictures in the DPB marked as “needed for output”.

gfv_timing_extrapolates_pic_timing_flag equal to 1, for a particular value of gfv_id, indicates that the timing of generated GFV output picture is extrapolated based on the timing of the decoded picture within the PU that contains the current GFV SEI message. gfv_timing_extrapolates_pic_timing_flag equal to 0 indicates the timing of generated GFV output picture may not be extrapolated based on the timing of the decoded picture within the PU that contains the current GFV SEI message.

gfv_time_scale is the number of time units that pass in one second. For example, a time coordinate system that measures time using a 27 MHz clock has a time_scale of 27 000 000. The value of time_scale shall be greater than 0.

gfv_num_units_in_pic_interval specifies the number of time units of a clock operating at the frequency gfv_time_scale Hz that corresponds to one increment of a picture interval of consecutive pictures. gfv_num_units_in_pic_interval shall be greater than 0. The indicated increment, denoted by the variable GfvPictureIntervalIncrement, in units of seconds, is equal to the quotient of gfv_num_units_in_pic_interval divided by gfv_time_scale. For example, for a 25 Hz gfv video sequence, gfv_time_scale and gfv_num_units_in_pic_interval could be equal to 27 000 000 and 1 080 000, respectively, and consequently a GFV picture interval would be equal to 0.04 seconds.

The indicated picture interval of a GFV generated picture, relative to the decoded  picture within the PU that contains the GFV SEI message with a particular value of gfv_id and a particular value of gfv_cnt greater than 0, denoted by the variable GfvGeneratedPictureInterval[ i ][ j ], in units of seconds, for i equal to gfv_id, j equal to gfv_cnt and gfv_cnt is greater than 0, is derived as follows:

GfvGeneratedPictureInterval[ gfv_id ][ gfv_cnt ] = GfvPictureIntervalIncrement * gfv_cnt

# Summary

This contribution proposes to add picture order count information in addition to the timing information for the GFV generated pictures and proposes to unify the DPB management for the decoded pictures and the GFV generated pictures. The calculations of variable PicOrderCntVal and indicated picture interval for a GFV generated picture are also provided. It is proposed to add it to the TuC for further study.

# References

- “Technologies under consideration for future extensions of VSEI (version 4)”, JVET-AH2032, JVET 34th (AH) meeting, April 2024, Rennes, France.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).