| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0193-v1 |
| --- | --- |

| Title: | AHG9/AHG16: On no display flag in generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Shiv Gehlot, Gary J. Sullivan, Peng Yin | Email: | sean.mccarthy@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes syntax and semantics for the generative face video SEI message to control the display of decoded output pictures flagged as fusion pictures.

# Introduction

During discussion of JVET-AH0198 at the previous meeting of JVET, it was recommended to study the signalling that a decoded output picture used for fusion in generative face video shall not be displayed.

Draft text of the generative face video SEI message is in Technologies under consideration for future extensions of VSEI (version 4) (JVET-AH2032). The syntax element gfv_drive_pic_fusion_flag, when present, equal to 1 indicates the current decoded picture, denoted as a driving picture, may be input to a face picture generator neural network, denoted as GenerativeNN( ), to improve background texture and facial details, for example.

In some cases, the driving picture is intended only as input to GenerativeNN( ) and is not intended to be displayed. For example, a driving picture representing only background texture might be input to GenerativeNN( ) for fusion with a generated face. In this example, it would be beneficial to indicate that the unfused background texture should not be displayed directly. In other cases, such as described in [1], the driving picture could be input to GenerativeNN( ) and also be suitable for display directly.

It is asserted that it would be beneficial to include a syntax element to indicate that a current decoded picture should not be displayed.

# Proposal

It is proposed to add syntax and semantics to control the display of decoded output pictures flagged as fusion pictures.

8.36 Generative face video SEI message

8.36.1 Generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| … | ue(v) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| … | u(1) |
| } else { /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to GenerativeNN( )*/ | ue(v) |
| gfv_ drive_pic_no_display_flag | u(1) |
| } |  |
| … | u(1) |
| } |  |

8.36.2 Generative face video SEI message semantics

…

gfv_drive_pic_fusion_flag, when present, equal to 1 indicates the current decoded picture, which corresponds to a driving picture that may be used for fusion, may be input to GenerativeNN( ). gfv_drive_pic_fusion_flag equal to 0 indicates the current decoded picture should not be input to GenerativeNN( ).

NOTE 3 – A gfv_drive_pic_fusion_flag value of 1 can be used, for example, to indicate that the current decoded picture can be used to improve face details or handle background changes.

NOTE 4 – Fusion takes the three inputs: the base picture, features from keypoints and/or matrices carried in the GFV SEI message, and the current decoded picture, and outputs a picture.

NOTE 5 – When current decoded picture corresponds to a driving picture, it should be marked as not for output purpose.

gfv_drive_pic_no_display_flag, when present, equal to 1 indicates the current decoded picture, which corresponds to a driving picture, should not be displayed. gfv_drive_pic_no_display_flag equal to 0 indicates the current decoded picture could be displayed.

NOTE 5 – A gfv_drive_pic_no_display_flag value of 1 can be used, for example, to indicate that the current decoded picture should be used only as input to GenerativeNN( ).

# Conclusion

It is requested to adopt the proposed syntax and semantics for the generative face video SEI message to control the display of decoded output pictures flagged as fusion pictures.

# References

[1] G. Konuko, S. Lathuiliere, and G. Valenzise, “A Hybrid Deep Animation CODEC for Low-Bitrate Video Conferencing”. arXiv:2207.13530 [v1] July 2022

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).