| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0195-v2 |
| --- | --- |

| Title: | AhG9/16: Updates on GFVC common software tools and GFV SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bolin Chen, Jie Chen, Yan Ye, Ru-Ling Liao (Alibaba) Shanzhi Yin, Shiqi Wang (CityU) | Email: | chenbolin.chenboli@alibaba-inc.com |
| Source: | Alibaba and City University of Hong Kong |  |  |

- Abstract

This contribution updates the AhG16 GFVC common software tools by extending the unified python scripts included in the common software package to link it with the GFV SEI message software (i.e., VTM software with GFV SEI message implementation) so that the entire GFVC process (via GFV SEI message approach) can be automatically executed with all the facial representation models included in GFVC common software supported.

Further, the GFV SEI message is also updated in this contribution to reduce the signalling overhead. The proposed changes to the GFV SEI message are as follows:

- change binary code of key point coordinates from u(n) to ue(v)

- change binary code of decimal part of matrix elements from u(n) to ue(v) in case of predictive signaling

- change the key point coordinates that are input to the generative model from the actual coordinates to the normalized coordinates.

The experiments are conducted by invoking the updated GFVC common software and GFV SEI message software in this contribution. It is reported that proposed GFV SEI message changes give a better overall BD-rate coding performance, and with the proposed changes the coding performance benefit over VVC is observed in high bitrate range for 256px resolution and for 512px resolution.

Introduction

In the previous meetings, GFVC common software [2] [4] and test conditions [1] were established as part of AHG16 activities, and the GFV SEI message was included into the draft 3 of TuC for future extensions of VSEI [3]. In the last meeting, JVET-AH0239 [6] implemented the GFV SEI message on top of VTM software and conducted the experiments using the GFV SEI message to signal facial parameters.

However, due to the limited interface between GFVC common software and VTM GFV SEI message software, only CFTE model can be invoked. Thus, in this contribution, the interface is extended so that all the face generative models included in the GFVC common software can be invoked with GFV SEI message approach.

Further, according the experimental results reported in JVET-AH0239 [6], generative face video coding with facial parameters being signalled in GFV SEI message can only achieve coding performance benefits over VVC on high resolution sequences at high bit-rate due to the high signalling overhead. Thus, the parameters signalling method of GFV SEI message is updated in this contribution to reduce the signalling overhead.

Updates on AhG16 GFVC common software

First, the two unified python scripts which are used to link the GFVC common software with the GFV SEI message software are extended so that all the facial generative models included in the GFVC common software can be invoked. Second, the higher-resolution model is updated to improve the reconstructed picture quality for high resolution sequences based on JVET-AI0047 [7] submitted in this meeting.

After linking the GFVC common software with the GFV SEI message software, the whole processing flow is as follows

- First, the facial parameters are extracted by the analysis model in the GFVC common software and then stored in the GFV SEI message configuration file

- The encoder of GFV SEI message software is invoked to read the parameters from the configuration file and encode them into the GFV SEI message bitstream. At the same time, the base picture is also encoded. Herein, the command is as follows,

- ./vtm/bin/EncoderAppStatic_VTM22_2_SEI -c ./vtm/cfg/encoder_lowdelay_vtm.cfg -c ./vtm/cfg/per-sequence/inputyuv420.cfg -c generative_face_video.cfg  -i input.yuv -o rec.yuv -b rec.bin >>rec.log

- After encoding, the decoder of GFV SEI message is invoked to decode the bitstream to get the decoded facial parameters and the reconstructed base picture. Then the decoded facial parameters are stored in a parameter file. Herein, the command is as follows,

- ./vtm/bin/DecoderAppStatic_VTM22_2_SEI -b rec.bin -o dec.yuv --OutputDecodedSEIMessagesFilename=Out_SEI.log > dec.log

- Finally, the decoded facial parameters are read from the parameter file and fed to generative model in the GFVC common software together with the reconstructed base picture to generate the following face pictures.

Updates on GFV SEI message

Signaling modification

In the current GFV SEI message, there are two types of feature parameters: key points and matrices. When signalling key points, the coordinates of the key points are normalized to the range from -1 to 1 with the origin at the center of the picture. The fixed length code (i.e, u(n)) is used for the coordinates of the key points. When signalling the matrices, the element values of the matrices are divided into integer part and decimal part. The fixed length code is used for decimal part of matrix elements.

However, usually the key points are not uniformly distributed within the picture. There are more key points in the center area of the picture than in the boundary area. Thus, there are more coordinates close to 0 than close to 1 to be signalled. To reduce the signalling overhead, it is proposed to use 0-th Exp-Golomb code (ue(v)) to code the key point coordinates.

Further, although the decimal parts of the matrix elements are almost uniformly distributed within [0,1], when the predictive signalling is enabled, the difference of the matrix elements which usually is a small value is signalled. In this case, even the decimal part to be signalled is not uniformly distributed. Thus, similar with key point coordinate signaling, it is proposed to use 0-th Exp-Golomb code (ue(v)) to code the differences of the matrix elements when predictive signalling is enabled to reduce the signalling overhead.

The proposed text changes are as follows:

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| …. | u(1) |
| gfv_coordinate_present_flag | u(1) |
| if( gfv_coordinate_present_flag ) { |  |
| gfv_coordinate_precision_factor_minus1 | ue(v) |
| gfv_num_kp_minus1 | ue(v) |
| gfv_kp_pred_flag | u(1) |
| gfv_coordinate_z_present_flag | u(1) |
| if(gfv_coordinate_z_present_flag ) |  |
| gfv_coordinate_z_max_value_minus1 | ue(v) |
| for( i = 0; i <= num_kp_minus1; i++ ) { |  |
| if(!gfv_kp_pred_flag) { |  |
| gfv_coordinate_x_abs[ i ] | u(n)ue(v) |
| if( gfv_coordinate_x_abs[ i ] ) |  |
| gfv_coordinate_x_sign_flag[ i ] | u(1) |
| gfv_coordinate_y_abs[ i ] | u(n)ue(v) |
| if( gfv_coordinate_y_abs[ i ] ) |  |
| gfv_coordinate_y_sign_flag[ i ] | u(1) |
| if( gfv_coordinate_z_present_flag ) { |  |
| gfv_coordinate_z_abs[ i ] | u(n)ue(v) |
| if( gfv_coordinate_z_abs[ i ] ) |  |
| gfv_coordinate_z_sign_flag[ i ] | u(1) |
| } |  |
| } else { |  |
| gfv_coordinate_dx_abs[ i ] | u(n)ue(v) |
| if(gfv_coordinate_dx_abs[ i ] ) |  |
| gfv_coordinate_dx_sign_flag[ i ] | u(1) |
| gfv_coordinate_dy_abs[ i ] | u(n)ue(v) |
| if( gfv_coordinate_dy_abs[ i ] ) |  |
| gfv_coordinate_dy_sign_flag[ i ] | u(1) |
| if( gfv_coordinate_z_present_flag ) { |  |
| gfv_coordinate_dz_abs[ i ] | u(n)ue(v) |
| if( gfv_coordinate_dz_abs[ i ] ) |  |
| gfv_coordinate_dz_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| gfv_matrix_present_flag | u(1) |
| if(gfv_matrix_present_flag ) { |  |
| gfv_matrix_element_precision_factor_minus1 | ue(v) |
| …. |  |
| for( j = 0; j < numMatrices[ i ]; j++ ) |  |
| for( k = 0; k < matrixHeight[ i ]; k++ ) |  |
| for( m = 0; m <matrixWidth[ i ]; m++ ) { |  |
| if( !gfv_matrix_pred_flag ) { |  |
| gfv_matrix_element_int[ i ][ j ][ k ][ m ] | ue(v) |
| gfv_matrix_element_dec[ i ][ j ][ k ][ m ] | u(n) |
| if( gfv_matrix_element_int[ i][ j ][ k ][ m ] ¦ ¦ gfv_matrix_element_dec[ i ][ j ][ k ][ m ] ) |  |
| gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ] | u(1) |
| } |  |
| else { |  |
| gfv_matrix_delta_element_int[ i ][ j ][ k ][ m ] | ue(v) |
| gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ] | u(n)ue(v) |
| if( gfv_matrix_delta_element_int[ i][ j ][ k ][ m ] ¦ ¦ gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ] ) |  |
| gfv_matrix_delta_element_sign_flag[ i ][ j ][ k ][ m ] | u(1) |
| … |  |

gfv_coordinate_precision_factor_minus1 plus 1 indicates the precision of key point coordinates signgaled in the SEI message. the length, in bits, of syntax elements gfv_coordinate_x_abs[i], gfv_coordinate_y_abs[i], gfv_coordinate_z_abs[i], gfv_coordinate_dx_abs[ i ], gfv_coordinate_dy_abs[ i ] and gfv_coordinate_dz_abs[ i ].

gfv_matrix_element_precision_factor_minus1 plus 1 indicates the precision of matrix elements signgaled. the length, in bits, of syntax elements gfv_matrix_element_dec[ i ][ j ][ k ][ m ] and gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ].

gfv_matrix_element_dec[ i ][ j ][ k ][ m ] indicates the decimal part of the value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type. The length of gfv_matrix_element_dec[ i ][ j ][ k ][ m ] is (gfv_matrix_element_precision_factor_minus1 +1) bits.

Interface modification

The GFV SEI message defines the interface between the SEI message decoder and the generative model. The key point coordinates input to the generative model is defined in the range from -PicWidth to PicWidth, while a normalized value with range from -1 to 1 of key point coordinates is signaled. So the decoder needs to scale the decoded key point coordinates after decoding and after the key point coordinates input to the generative model, the model will re-scale them to a normalized value.

Thus, to remove these unnecessary operations and give a clean interface between the decoder and the generative model, it is proposed to directly input the normalized coordinate to the generative model. Meanwhile, as the picture width, height and maximum z-axis value are needed in the face picture generation, it is also proposed to add these parameters as the input of the generative model.

The proposed text changes are as follows:

gfv_coordinate_z_present_flag equal to 1 indicates that z-axis coordinate information of the keypoints is present. coordinate_z_present_flag equal to 0 indicates that the z-axis coordinate information of the keypoints is not present.

gfv_coordinate_z_max_value_minus1 plus 1 indicates the maximum absolute value of z-axis coordinates of keypoints.

When gfv_coordinate_z_max_value_minus1 is present, the variable CroppedDepth is equal to gfv_coordinate_z_max_value_minus1 + 1; otherwise CroppedDepth is equal to 0.

The variables coordinateDeltaX[ i ], coordinateDeltaY[ i ] and coordinateDeltaZ[ i ] indicating the delta x-axis coordinate, delta y-axis coordinate and delta z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

coordinateDeltaX[ i ] = (1-2 * gfv_coordinate_dx_sign_flag[ i ]) * gfv_coordinate_dx_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

= (1-2 * gfv_coordinate_dx_sign_flag[ i ]) * gfv_coordinate_dx_abs[ i ] ÷(1<< (gfv_coordinate_precision_factor_minus1 +1))

coordinateDeltaY[i] = (1-2 * gfv_coordinate_dy_sign_flag[ i ]) * gfv_coordinate_dy_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

= (1-2 * gfv_coordinate_dy_sign_flag[ i ]) * gfv_coordinate_dy_abs[ i ]÷(1<< (gfv_coordinate_precision_factor_minus1 +1))

if (gfv_coordinate_z_present_flag ) 
	  coordinateDeltaZ[i] = (1-2 * coordinate_dz_sign_flag[ i ]) * gfv_coordinate_dz_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

= (1-2 * coordinate_dz_sign_flag[ i ]) * gfv_coordinate_dz_abs[ i ]÷(1<< (gfv_coordinate_precision_factor_minus1 +1))

The variables coordinateX[ i ], coordinateY[ i ] and when gfv_coordinate_z_present_flag  is equal to 1, coordinateZ[ i ] indicating the x-axis coordinate, y-axis coordinate and z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

When gfv_kp_pred_flag is equal to 0,

coordinateX[ i ] = (1-2 * gfv_coordinate_x_sign_flag[ i ]) * gfv_coordinate_x_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

= (1-2 * gfv_coordinate_x_sign_flag[ i ]) * gfv_coordinate_x_abs[ i ] ÷(1<< (gfv_coordinate_precision_factor_minus1 +1))

coordinateY[i] = (1-2 * gfv_coordinate_y_sign_flag[ i ]) * gfv_coordinate_y_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

= (1-2 * gfv_coordinate_y_sign_flag[ i ]) * gfv_coordinate_y_abs[ i ]÷(1<< (gfv_coordinate_precision_factor_minus1 +1))

if (gfv_coordinate_z_present_flag ) 
	    coordinateZ[i] = (1-2 * gfv_coordinate_z_sign_flag[ i ]) * gfv_coordinate_z_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

= (1-2 * coordinate_z_sign_flag[ i ]) * gfv_coordinate_z_abs[ i ]÷(1<< (gfv_coordinate_precision_factor_minus1 +1))

For a particular gfv_id value, the following process is used in increasing order of gfv_cnt to generate a video picture per each GFV SEI message that has gfv_base_pic_flag equal to 0 and a unique value of gfv_cnt within a picture unit:

DeriveSigParam( ) 
TranslatorNN (sigKeyPoint , sigMatrix) 
DeriveInputTensors( ) 
if( gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 0) {
  if(ChromaFormatIdc == 0 ) 
    GenerativeNN( inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix, CroppedWidth, CroppedHeight, , CroppedDepth) 
  else 
    GenerativeNN( inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix, CroppedWidth, CroppedHeight, , CroppedDepth) 
}
else if(gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 1) {
  if(ChromaFormatIdc == 0 ) 
    GenerativeNN( inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix, CroppedWidth, CroppedHeight, , CroppedDepth) 
  else 
    GenerativeNN( inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix, CroppedWidth, CroppedHeight, , CroppedDepth) 
}

StoreOutputTensors( )

Inputs to GenerativeNN() are:

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaFormatIdc is equal to 0: inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix, CroppedWidth, CroppedHeight, , CroppedDepth

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaFormatIdc is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix, CroppedWidth, CroppedHeight, , CroppedDepth

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaFormatIdc is equal to 0: inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix, CroppedWidth, CroppedHeight, , CroppedDepth

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaFormatIdc is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix, CroppedWidth, CroppedHeight, , CroppedDepth

Experimental Results

Testing Conditions & Anchors

The experiments of GFVC via SEI message approach are conducted under GFVC test condition [1] in this contribution. The experiment flowchart is shown in Figure 1. The detailed settings are as follows:

- For anchor, VVC (VTM-22.2) is used and the test is conducted under low-delay B (LDB) configuration with quantization parameters (QP) setting to 37, 42, 47 and 52.

- In addition, we also provide a GFVC-AE version test, i.e., the base picture is compressed as an I picture using the VTM reference software 22.2, and other pictures are further represented with a series of facial parameters and compressed via a context-adaptive arithmetic coder. The QPs of the base picture are set to 22, 32, 42 and 52.

- For test, the base picture is coded as an I picture using the VTM-22.2 with QP value of 22, 32, 42 and 52. The other pictures are coded with GFVC method and the feature parameters are signalled via GFV SEI message. It should be mentioned that all experimental tests include different GFV SEI implementation, i.e., one is the original version (i.e., GFVC-SEI-Ori) included in JVET-AH2039  [3] and another is the optimized GFV SEI signaling (i.e., GFVC-SEI-Pro) in this contribution.

Figure 1 – Illustration of encoding/decoding processes for GFV SEI message.

Rate-Distortion Performance

Figure 2 & Figure 3 show the rate distortion performance (i.e., Rate-DISTS and Rate-LPIPS) of GFVC with the optimized GFV SEI signaling compared to other testing anchors on two testing class sequences. We can see that the proposed GFV SEI message changes give a better overall RD performance except for CFTE algorithm, illustrating the GFV SEI signaling optimization is generally beneficial for overhead reduction and can support most of GFVC algorithms.

In addition, with the resolution increase from 256px to 512px, GFV SEI-based implementation can obviously improve the coding performance benefit over VVC.

- FOMM Results

- CFTE Results

- FV2V Results

- DAC Results

- HDAC Results

Figure 2 – Rate distortion performance of VVC and AhG16 software with different GFVC algorithms measured DISTS and LPIPS in terms of average results for Class A and Class B at the 256px resolution.

Figure 3 – Rate distortion performance of VVC and AhG16 software with higher-resolution CFTE model measured DISTS and LPIPS in terms of average results for Class A and Class B at the 512px resolution.

Rate-Distortion Performance according to JVET-AI0191 syntax

According to the proposal JVET-AI0191 [8], the related syntax is refined in the updated experiments, illustrating that it can facilitate the GFV SEI overhead reduction.

- 256px Results                                                (b) 512px Results

Figure 4 – Updated Rate distortion performance of VVC and AhG16 software with higher-resolution CFTE model measured DISTS in terms of average results for Class A and Class B at the 256px and 512px resolutions.

Conclusion

This contribution updates the AhG16 GFVC common software tools by extending the unified python scripts included in the common software package to link it with the GFV SEI message software. Further, the GFV SEI message is also updated to reduce the signalling overhead. It is suggested to adopt these updates in AhG16 software tools and GFV SEI message signaling, and a further study in GFV SEI overhead reduction should be investigated.

References

- McCarthy S, Chen B. Test conditions and evaluation procedures for generative face video coding, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG2035, January 2024.

- Chen B, Ye Y, et al. AHG 16: Updated Common Software Tools for Generative Face Video Compression, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AH0114, April 2024.

- McCarthy S, Chen J et al, Technologies under consideration for future extensions of VSEI (version 3), JVET-AG2032, online meeting, January 2024.

- Chen B, Chen J, et al. AHG16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression, The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AG0042, January 2024.

- Zou R, Liao R-L, et al. “AHG16: Lightweight CFTE with Multi-resolution Support,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AH0113, April 2024.

- Chen B, Chen J, et al. “AHG9/AHG16: Software Implementation of Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AH0239, April 2024.

- Chen B, Yin S, et al. “AhG16: Higher-resolution Test Sequences and Test Results for Generative Face Video Compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AI0047, July 2024.

- Wang Y.-K., Yang K, et al. “AHG9: On GFV facial parameters signalling,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET- AI0191, July 2024.

Patent rights declaration(s)

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).