| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12-19 July 2024 | Document: JVET-AI0205-v5 |
| --- | --- |

| Title: | AHG9: Combined SEI messages for common image metadata formats |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arianne Hinds Stephan Wenger Gilles Teniou | Tel: Email: | ahinds@global.tencent.com swenger@global.tencent.com teniou@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

At the 34th meeting, JVET-AH0343 combined, into a single SEI message, multiple SEI message proposals that share a common syntax based on a text string. The rationale for doing so was “that a common SEI message which provides text information for various purposes avoids the need to define multiple SEI messages which provide specific type of text information, while providing future extensibility.” This proposal makes the observation that the same rationale may be applied to the image metadata formats proposed in JVET-AF0141 and subsequently adopted into the TuC starting in JVET-AF2032. Hence a single SEI message to carry the binary metadata of the JVET-AF0141 image metadata format SEIs is proposed herein to achieve similar benefits of having to avoid the need to define multiple SEI messages that provide specific image metadata information.

# Problem statement

The current technologies under consideration document published as JVET-AH2032 [1] contains three separate SEI messages to carry the binary information associated with the metadata for EXIF[2], JFIF[3], and XMP[4,5,6]. These SEI messages are collectively labelled as “image format metadata SEI messages” within the TuC.  The primary syntax element across the payloads for these SEI messages is a payload byte with the descriptor of b(8), which is read from the video bitstream to collect the binary payloads for each of the SEIs.

A single SEI message to carry the binary metadata associated with the image format metadata SEI messages may benefit from the same rationale used to create a single SEI that employs a syntax based on a text string, as described in JVET-AH0343. Such an SEI message would leverage the common syntax of binary payload bytes, with an option to carry the payload via a URI.

# Proposed approach

## General

A single SEI message is proposed to carry each of the image format metadata SEI messages. Similar to the purpose syntax element in JVET-AH0343, a “type” syntax element is defined to signal which of the metadata formats is being carried in the SEI message payload. As well, the option to reference the image metadata at a location determined by a URI is preserved for such a single SEI so that the image metadata payload may be accessed from the location provided by the URI.

v4: during the discussion of this proposal at the JVET meeting, several suggestions for improvement were accepted including:

- Fix minor bugs in syntax (juse “j” index instead of “i” in inner loop. Add indexing using [i] and [j] on syntax elements that are within the loop.

- Change the name of the SEI to “image format information”

- Change the name of the “uri_available” flag to “uri_present” to be consistent with naming convention used in other SEIs.

- Change “length” and “number of payloads” to ue(v)

v5: other minor fixes (add spaces between brackets between use of indexes, change the title of table xx to refer to the index “i”, do not use uppercase letters for “URI” abbreviation in the syntax table, provide additional name changes from “im” to “ifi” that were missing from v4):

# Proposal

## New normative references

- T. 871 : Information technology - Digital compression and coding of continuous-tone still images: JPEG File Interchange Format (JFIF)

- ISO/IEC 10918 – 5 Information technology — Digital compression and coding of continuoustone still images: JPEG File Interchange Format (JFIF)

- Exchangeable image file format for digital still cameras: Exif Version 3.0, Camera & Imaging Products Association, May 2023. [Ed Note: awaiting conclusion of ITU A.5 and A.4 processes]

- ISO 16684-1: Graphic Technology – Extensible metadata platform (XMP) specification– Part 1 Data model, serialization and core properties.

# ISO 16684-3: Graphic Technology – Extensible metadata platform (XMP) specification – Part 3 JSON-LD serialization of XMP.

## Image format information SEI message syntax

| image_format_information( payloadSize ) { | Descriptor |
| --- | --- |
| ifi_cancel_flag | u(1) |
| if (!ifi_cancel_flag) { |  |
| ifi_persistence_flag | u(1) |
| ifi_num_metadata_payloads | ue(v) |
| for (i=0; i<ifi_num_metadata_payloads; i++) { |  |
| ifi_type_id[ i ] | ue(v) |
| ifi_uri_present_flag[ i ] | u(1) |
| if(!ifi_uri_present_flag[ i ]) { |  |
| ifi_payload_len_minus1[ i ] | ue(v) |
| for( j = 0; j<= ifi_payload_len[ i ]; j++ ) |  |
| ifi_data_payload_byte[ i ][ j ] | b(8) |
| } |  |
| else { |  |
| while( !byte_aligned( ) ) |  |
| ifi_bit_equal_to_zero | f(1) |
| ifi_data_uri[ i ] | st(v) |
| } |  |
| } |  |
| } |  |

## Image format information SEI message semantics

The image format information SEI message specifies an SEI message in which the payload of EXIF, JFIF, XMP image format metadata formats can be carried in the video bitstream.

ifi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous image format information SEI message in output order. ifi_cancel_flag equal to 0 indicates that image format  information follows.

ifi_persistence_flag specifies the persistence of the image format information SEI message for the current layer.

ifi_persistence_flag equal to 0 specifies that the image format information SEI message applies to the current decoded picture only.

ifi_persistence_flag equal to 1 specifies that the image format information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an Exif metadata SEI message is output that follows the current picture in output order.

ifi_num_metadata_payloads indicates the number of metadata payloads that follow.

ifi_bit_equal_to_zero shall be equal to zero.

ifi_type_id[ i ] indicates the type of the metadata payload as defined in Table xx.

Table xx – Definition of ifi_type_id[ i ]

| Value | Interpretation |
| --- | --- |
| 0 | Exif metadata as specified by (Ed Note: awaiting conclusion of ITU A.5 and A.4 processes) |
| 1 | JFIF metadata from one or more concatenated JFIF segments as specified in ITU-T Recommendation T.871 ¦ International Standard ISO/IEC 10918-5 |
| 2 | one or more JFIF extension markers as specified in ITU-T Recommendation T.871 ¦ International Standard ISO/IEC 10918-5 |
| 3 | first JFIF marker found following the start of the image as specified in ITU-T Recommendation T.871 ¦ International Standard ISO/IEC 10918-5 |
| 4 | XMP metadata as specified in ISO 16684-1: Graphic Technology – Extensible metadata platform (XMP) specification– Part 1 Data model, serialization and core properties. Or ISO 16684-3: Graphic Technology – Extensible metadata platform (XMP) specification – Part 3 JSON-LD serialization of XMP. |
| 5-255 | Reserved |

When ifi_type_id[ i ] is equal to 0, the following image metadata payload is Exchangeable Image File (Exif)  metadata. The Exif Format for digital still cameras includes a set of metadata that captures information regarding the digital photography process that was used to record an image. Such metadata includes: the acceleration vector of the camera at the time the image was captured, camera lens information, GPS data, the color space used, spectral sensitivity, maximum lens aperture, and more.

Note: Exif metadata is specified by any of the Exchangeable image file (Exif) format for digital still cameras standards developed jointly by the Camera & Imaging Products Association (CIPA) and the Japan Electronics and Information Technology Industries Association (JEITA), for the associated video source pictures prior to encoding, e.g., for camera-captured content. To date, there are multple versions of Exif deployed, each with its own indicator to signal the version number in use.

[Ed. Note: Neither CIPA nor JEITA are listed in the organizations for which normative references may be made to documents published by CIPA or JEITA. The list of such organizations is available at: https://www.itu.int/net4/ITU-T/lists/sdo.aspx. To add either CIPA or JEITA to the list, the A.4 process needs to be executed for one or both organizations, and their status as A.5 organizations needs to be approved by SG16. There would probably also need to be a specific list of documents referenced rather than the concept of “any of the” standards produced by those organizations.]

When ifi_type_id[ i ] is equal to 1, 2, or 3, the following metadata payload is JFIF metadata, as specified by ITU-T Recommendation T.871 | ISO/IEC International Standard 10918-5.

Note: JFIF metadata and its semantics are specified by ITU-T Recommendation T.871 | ISO/IEC International Standard 10918-5 (hereafter, the JFIF standard). Of particular importance is the distinction by JFIF between a JFIF marker that contains information describing the organization of the image data, and a JFIF marker that contains “extension” information.

A single APP0 marker that carries the information to describe the organization of the image is signalled by the zero-terminated string “JFIF” (‘0x4A46494600’) within that APP0 marker. One or more subsequent APP0 markers may follow the first APP0 marker in which the subsequent APP0 marker(s) carries “extension” information. In particular, the JFIF extension mechanism can be used to carry image thumbnails, although there is no specification by the JFIF standard for the precise type of data carried in JFIF extensions.

Approaches for carriage of JFIF within an SEI message include:

–	The JFIF payload is carried entirely as a single “container” within an SEI message. In this approach, the JFIF payload includes the payloads from the first and all, if any, subsequent JFIF APP0 markers for an image.

–	The JFIF payload from only extension APP0 markers, i.e., all JFIF APP0 markers that appear subsequent to the first APP0 marker, is carried as a single container. This approach facilitates the carriage of thumbnail images without the overhead of the carriage of the first JFIF APP0 marker.

–	The JFIF payload from only the first JFIF APP0 marker is carried as a single container.

When ifi_type_id[ i ] is equal to 1, the following image format information payload is JFIF metadata from one or more concatenated JFIF marker segments, without the APP0 marker segment identifiers themselves, as specified in ITU-T Recommendation T.871 | International Standard ISO/IEC 10918-5.

When ifi_type_id[ i ] is equal to 2, the following image format information payload contains bytes from one or more JFIF extension markers, without the APP0 marker segment identifiers themselves, as specified in ITU-T Recommendation T.871 | International Standard ISO/IEC 10918-5.

When ifi_type_id[ i ] is equal to 3, the following image format information payload contains bytes obtained from the first JFIF marker found following the start of the image as specified in ITU-T Recommendation T.871 | International Standard ISO/IEC 10918-5.

When ifi_type_id[ i ] is equal to 4, the following image information payload contains Extensible Metadata Platform (XMP) metadata as specified by ISO 16684-1 in XML or ISO 16684-3  in JSON. [Ed. Note: Fix or remove unresolved references and undefined abbreviations.]

Note: Extensible Metadata Platform (XMP) metadata is specified in ISO 16684-1: Graphic Technology – Extensible metadata platform (XMP) specification or ISO 16684-3: Graphic Technology – Extensible metadata platform (XMP) specification – Part 3 JSON-LD serialization of XMP [Ed. Note (GJS): Formal reference needed in clause 2.] XMP is widely deployed by digital cameras and digital image editing packages to record provenance and editing history with an image. The most common set of metadata included in XMP refers to a vocabulary defined by the Dublin Core Metadata Initiative (DCMI), e.g., to store information such as digital rights ownership and names of software packages used to modify the image.

ifi_uri_present_flag[ i ] signals if the metadata is obtained via a URI with syntax and semantics as specified in IETF Internet Standard 66.

–	If ifi_uri_present_flag [ i ] is equal to 0, the image information payload is obtained directly from the payload of the SEI message.

–	Otherwise if ifi_uri_present_flag [ i ] is equal to 1, the image information payload is obtained from a URI.

ifi_payload_len_minus1[ i ] plus 1 indicates the length of the image information payload according to the ifi_type_id[ i ] as described in Table xx.

ifi_data_payload_byte[ i ][ j ] shall be a byte containing data having syntax and semantics according to ifi_type_id[ i ] as described in Table xx.

ifi_data_uri[ i ] shall contain a URI with syntax and semantics as specified in IETF Internet Standard 66 identifying the image information payload according to the ifi_type_id [ i ] as described in Table xx.

# References

[1]  Technologies under consideration for future extensions of VSEI (version 4), Joint Video Experts Team       (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 JVET-AH2032.

[2]   Exchangeable image file format for digital still cameras: Exif Version 3.0, Camera & Imaging Products Association, May 2023.

[3] Recommendation ITU-T T.871, Information technology - Digital compression and coding of continuous-tone still images: JPEG File Interchange Format (JFIF).

[4] ISO 16684-1: Graphic Technology – Extensible metadata platform (XMP) specification – Part 1 Data model, serialization and core properties.

[5] ISO 16684-2: Graphic Technology – Extensible metadata platform (XMP) specification – Part 2 Description of XMP schemas using RELAX NG.

[6] ISO 16684-3: Graphic Technology – Extensible metadata platform (XMP) specification – Part 3 JSON-LD serialization of XMP.

[7] https://www.cipa.jp/e/guide/aboutcipa.html

[8] https://www.jeita.or.jp/english/

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

https://www.dublincore.org