| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI00213-v4 |
| --- | --- |

| Title: | EE2-4.4c, EE2-4.4d and EE2-4.4g: Multiple Transform Set Selection for LFNST/NSPT using implicit kernel derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Charles Bonnineau, Karam Naser, Saurabh Puri, Fabrice Le Léannec (InterDigital) Muhammed Coban Marta Karczewicz (Qualcomm) Lei Zhao, Kai Zhang, Li Zhang (Bytedance) | Email: |  | charles.bonnineau@interdigital.com mcoban@qti.qualcomm.com zhaolei.0324@bytedance.com |  |
| Source: | InterDigital, Qualcomm Inc., Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2-4.4c, EE2-4.4d and EE2-4.4g.

In Test 4.4c, the multiple transform set selection (MTSS) method for intra blocks coded with LFNST/NSPT from Test 4.4a is investigated with implicit kernel selection.

In Test 4.4b, MTSS with CU size restriction for intra blocks coded with LFNST/NSPT from Test 4.4b is investigated with implicit kernel selection.

Test 4.4g, MTSS with implicit kernel derivation (Test 4.4b) and explicit kernel selection (Test 4.4a) applied different CU sizes is investigated.

On top of ECM-13.0, simulation results are reported as follows:

Test 4.4c

AI {-0.09% Y, 0.01% U, 0.00% V, 102.4% EncT, 100.7% DecT}

RA { xx% Y, xx% U, xx% V, xx% EncT, xx% DecT}

Test 4.4d

AI {-0.07% Y, 0.01% U, 0.0 % V, 100.6% EncT, 99.9% DecT}

RA { xx% Y, xx% U, xx% V, xx% EncT, xx% DecT}

Test 4.4g

AI {-0.11% Y, 0.01% U, -0.02% V, 101.9% EncT, 100.2% DecT}

RA {-0.06% Y, -0.03% U, -0.05% V, 100.8% EncT, 99.9% DecT}

# Introduction

Low-Frequency non-separable transforms (LFNST) was first introduced in VVC and has been further enhanced in ECM. In addition to LFNST, a non-separable primary transform (NSPT) set is included in ECM that replace the DCT2 plus LFNST transform combination with a single primary transform. The combination of LFNST/NSPT outperforms using LFNST alone.

In JVET-AG0062, Multiple Transform Set Selection (MTSS) [1] have been proposed, where an additional transform set is tested for modes that use a virtual intra prediction mode (VIPM) to select an LFNST/NSPT transform set in the lookup-table.

The mode used to derive the alternative transform set for each intra modes are summarized below:

|  | IPM to derive the 1st transform set | IPM to derive the 2nd transform set |
| --- | --- | --- |
| DIMD, OBIC, TIMD | The 1st IPM used in current mode | The 2nd IPM used in current mode |
| SGPM, MIP, EIP, ITMP | The 1st IPM derived by HoG of prediction | The 2nd IPM derived by HoG of prediction |

The 1st and 2nd candidate should have different transform sets or different transpose type. If the 2nd candidate is not yet decided by the above process, it is derived by the HoG of neighboring reconstructed pixels.

# Proposed method

In EE2-4.4c, the LFNST/NSPT kernel in the alternative transform set is implicitly derived using information about the intra mode, denoted as transform index derivation mode (TIDM). The TIDM for each intra mode is described in the table below:

| Intra mode | Transform index derivation mode (TIDM) |
| --- | --- |
| MIP | MIP index |
| EIP | EIP index |
| DIMD | DIMD derived intra mode |
| TIMD | TIMD derived intra mode |
| SGPM | SGPM index |
| IntraTMP | IntraTMP index |

A lookup-table is used to derive the final LFNST/NSPT index in the alternative transform set. The coefficients of the LUT are the following:

LUT={0, 1, 0, 1, 0, 2}

The final LFNST/NSPT index is computed as follows:

lfnstIdx=LUT[TIDM % 6]

In Test 4.4d, MTSS with implicit kernel derivation from Test 4.4c is applied to CUs with number of pixels equal or larger than 128.

In Test 4.4g, MTSS with implicit kernel derivation from Test 4.4c is applied to CUs with number of pixels equal or larger than 128 and lower than 256, and MTSS from Test 4.4a is applied to CUs with number of pixels equal or larger than 256.

# Experimental results

The proposed method is implemented on top of ECM-13.0 [2]. The simulations were performed following the CTCs [3] for the all-intra (AI) and random access (RA) configurations.

Table 1: Experimental results of Test 4.4c over ECM-13.0 (AI)

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1 | -0.10% | 0.02% |  | 0.04% |  | 102.4% |  | 99.4% |  | 99.5% |  | 99.2% |
| Class A2 | -0.06% | -0.05% |  | 0.06% |  | 102.1% |  | 100.5% |  | 99.9% |  | 100.4% |
| Class B | -0.07% | 0.01% |  | -0.11% |  | 101.5% |  | 100.4% |  | 101.7% |  | 100.1% |
| Class C | -0.10% | 0.01% |  | 0.08% |  | 102.6% |  | 101.7% |  | 99.6% |  | 100.1% |
| Class E | -0.12% | 0.06% |  | 0.01% |  | 104.2% |  | 101.6% |  | 101.9% |  | 100.1% |
| Overall | -0.09% | 0.01% |  | 0.00% |  | 102.4% |  | 100.7% |  | 100.6% |  | 100.0% |
| Class D | -0.07% | 0.23% |  | -0.11% |  | 103.4% |  | 101.5% |  | 100.0% |  | 100.0% |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2: Experimental results of Test 4.4c over ECM-13.0 (RA)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.04% | 0.09% |  | -0.04% |  | 100.5% |  | 101.2% |  | 100.1% |  | 100.1% |
| Class C | -0.06% | 0.10% |  | 0.17% |  | 99.6% |  | 99.3% |  | 100.1% |  | 100.1% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.04% | -0.31% |  | -0.05% |  | 100.6% |  | 99.8% |  | 99.9% |  | 100.0% |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3: Experimental results of Test 4.4d over ECM-13.0 (AI)\

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | 0.12% | 0.00% | 100.6% | 98.5% | 99.9% | 100.4% |
| Class A2 | -0.06% | -0.02% | 0.03% | 100.8% | 101.0% | 98.4% | 99.4% |
| Class B | -0.06% | -0.05% | 0.02% | 100.7% | 100.5% | 100.8% | 100.2% |
| Class C | -0.05% | 0.02% | 0.15% | 100.1% | 99.3% | 100.1% | 100.0% |
| Class E | -0.09% | 0.03% | -0.03% | 101.2% | 100.2% | 99.9% | 100.1% |
| Overall | -0.07% | 0.01% | 0.04% | 100.6% | 99.9% | 100.0% | 100.1% |
| Class D | -0.04% | 0.10% | 0.00% | 100.4% | 99.6% | 100.0% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 4: Experimental results of Test 4.4d over ECM-13.0 (RA)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.03% |  | -0.28% |  | 101.2% |  | 102.1% |  | 100.1% |  | 100.2% |
| Class C | -0.02% | 0.02% |  | 0.18% |  | 100.3% |  | 99.0% |  | 100.1% |  | 100.1% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.11% |  | -0.02% |  | 100.7% |  | 99.6% |  | 100.0% |  | 100.0% |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5: Experimental results of Test 4.4g over ECM-13.0 (AI)

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.20% | 0.03% | -0.14% | 102.3% | 98.7% | 100.2% | 99.3% |
| Class A2 | -0.10% | -0.01% | 0.06% | 101.9% | 99.6% | 99.4% | 99.5% |
| Class B | -0.09% | -0.03% | -0.06% | 103.5% | 102.1% | 99.0% | 98.9% |
| Class C | -0.06% | -0.01% | 0.09% | 100.7% | 100.1% | 99.7% | 99.9% |
| Class E | -0.12% | 0.10% | -0.05% | 100.7% | 99.1% | 98.8% | 99.6% |
| Overall | -0.11% | 0.01% | -0.02% | 101.9% | 100.2% | 99.4% | 99.4% |
| Class D | -0.06% | 0.12% | -0.07% | 101.0% | 99.7% | 100.2% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 6: Experimental results of Test 4.4g over ECM-13.0 (AI)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.12% | -0.05% | -0.25% | 100.8% | 102.2% | 100.0% | 99.7% |
| Class A2 | -0.04% | -0.06% | -0.04% | 100.4% | 98.2% | 100.4% | 99.9% |
| Class B | -0.06% | -0.05% | -0.08% | 101.6% | 100.7% | 100.1% | 100.3% |
| Class C | -0.04% | 0.04% | 0.12% | 100.2% | 98.5% | 100.1% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.03% | -0.05% | 100.8% | 99.9% | 100.1% | 100.1% |
| Class D | 0.00% | 0.01% | 0.11% | 101.3% | 101.0% | 100.2% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

|  | Random Access Main 10. Over ECM-13.0. Y; U; V; EncT; DecT; EncVmPeak; DecVmPeak. Class A1; -0.12%; -0.05%; -0.25%; 100.8%; 102.2%; 100.0%; 99.7%. Class A2; -0.04%; -0.06%; -0.04%; 100.4%; 98.2%; 100.4%; 99.9%. Class B; -0.06%; -0.05%; -0.08%; 101.6%; 100.7%; 100.1%; 100.3%. Class C; -0.04%; 0.04%; 0.12%; 100.2%; 98.5%; 100.1%; 100.1%. Class E. Overall; -0.06%; -0.03%; -0.05%; 100.8%; 99.9%; 100.1%; 100.1%. Class D; 0.00%; 0.01%; 0.11%; 101.3%; 101.0%; 100.2%; 100.0%. Class F; #VALUE!; #VALUE!; #VALUE!; #DIV/0!; #DIV/0!; #DIV/0!; #DIV/0!. Class TGM; #VALUE!; #VALUE!; #VALUE!; #DIV/0!; #DIV/0!; #DIV/0!; #DIV/0!. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

# References

- F. Wang, et. al., “Non-EE2: Multiple Transform Set Selection for LFNST/NSPT” JVET-AG0062, January 2024

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AH2017, October 2023

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).