| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0219-v1 |
| --- | --- |

| Title: | EE2-4.4e: Test 4.4a with reduced number of transform candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz Vadim Seregin | Tel: Email: | mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In EE2-4.4 test, multiple LFNST/NSPT transform sets are used for DIMD, TIMD, SGPM, MIP, EIP, and IntraTMP modes. In this contribution the number of transforms in secondary set is configurable and reduced to 1 from 3 with respect to EE2-4.4a.

On top of ECM-13.0, simulation results are reported as follows:

AI {-0.07% Y, 0.04% U, 0.07% V, 101.7% EncT, 99.8% DecT}

RA { xx % Y, xx % U, xx % V, xx %EncT, xx% DecT}

LB { -0.03% Y, -0.49% U, 0.03% V, xx %EncT, xx %DecT}

# Proposal

Secondary IPM for selection of LFNST/NSPT transform kernels for MIP, EIP, SGPM and IntraTMP modes is derived from second highest HoG on the prediction block. The number of transform kernels in the second set is restricted to 1 for AI configuration. For RA, LB configurations full set of secondary transforms are used as in Test 4.4a.

# Experimental results

The performance of proposed scheme was evaluated using the common test conditions defined in [1].

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.08% | 0.02% | 0.10% | 101.6% | 99.6% | 100.2% |
| Class A2 | -0.06% | 0.02% | 0.10% | 101.5% | 99.7% | 100.2% |
| Class B | -0.06% | 0.01% | -0.02% | 101.6% | 99.8% | 99.4% |
| Class C | -0.08% | 0.04% | 0.13% | 101.8% | 100.4% | 100.1% |
| Class E | -0.09% | 0.13% | 0.06% | 102.0% | 99.6% | 100.1% |
| Overall | -0.07% | 0.04% | 0.07% | 101.7% | 99.8% | 99.9% |
| Class D | -0.08% | 0.03% | 0.01% | 101.7% | 99.9% | 100.1% |
| Class F | -0.10% | 0.05% | 0.10% | 100.9% | 100.4% | 100.1% |
| Class TGM | -0.03% | -0.05% | -0.04% | 101.1% | 99.8% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.18% | 0.03% | -0.22% | #NUM! | #NUM! | 100.1% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | -0.10% | -0.01% | 0.01% | #NUM! | #NUM! | 99.8% |
| Class C | -0.09% | -0.01% | 0.20% | #NUM! | #NUM! | 99.4% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.05% | 0.02% | 0.00% | #NUM! | #NUM! | 100.1% |
| Class F | -0.04% | -0.01% | -0.11% | #NUM! | #NUM! | 99.7% |
| Class TGM | -0.07% | 0.02% | 0.05% | #NUM! | #NUM! | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.06% | -0.29% | 0.08% | #NUM! | #NUM! | 99.7% |
| Class C | -0.06% | -0.36% | -0.56% | #NUM! | #NUM! | 100.2% |
| Class E | 0.06% | -0.99% | 0.71% | #NUM! | #NUM! | 99.1% |
| Overall | -0.03% | -0.49% | 0.03% | #NUM! | #NUM! | 99.7% |
| Class D | -0.09% | -0.71% | -0.47% | #NUM! | #NUM! | 101.0% |
| Class F | -0.20% | 0.21% | 0.41% | #NUM! | #NUM! | 99.2% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

# References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, 25th Meeting, by teleconference, 12–21 January 2022.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).