| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0220-v1 |
| --- | --- |

| Title: | EE2-4.4b related: Encoder optimization for Test EE2-4.4b |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Muhammed Coban Hongtao Wang | Tel: Email: | martak@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In EE2-4.4b test, multiple LFNST/NSPT transform sets are used for DIMD, TIMD, SGPM, MIP, EIP, and IntraTMP modes. In this test, the multiple transform set selection (MTSS) method for intra blocks coded with LFNST/NSPT with CU size restriction is investigated where the minimum CU size for MTSS application is reduced to 64 from 256 and additional encoder speedups are introduced.

On top of ECM-13.0, simulation results are reported as follows:

AI {-0.10% Y, 0.09% U, 0.02% V, 101.8% EncT, 100.2% DecT}

# Proposal

In EE2-4.4b, a CU size restriction over EE2-4.4a is tested. MTSS can be applied to CUs with number of pixels equal or larger than 256. In this proposal MTSS can be applied to CUs with number of pixels equal or larger than 64. Additional early termination of transform RD checks are introduced to reduce the runtime. The proposed encoder changes are applied to AI configuration only. RA, LB configurations use the same configuration as in Test 4.4a.

# Experimental results

The performance of proposed scheme was evaluated using the common test conditions defined in [1].

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.18% | 0.08% | 0.05% | 102.8% | 100.2% | 100.2% |
| Class A2 | -0.09% | 0.07% | 0.09% | 102.4% | 99.8% | 100.2% |
| Class B | -0.09% | 0.06% | -0.03% | 102.2% | 100.2% | 100.1% |
| Class C | -0.05% | 0.07% | 0.11% | 100.8% | 100.4% | 100.1% |
| Class E | -0.10% | 0.19% | 0.02% | 101.1% | 100.4% | 100.1% |
| Overall | -0.10% | 0.09% | 0.04% | 101.8% | 100.2% | 100.1% |
| Class D | -0.02% | 0.26% | -0.05% | 100.8% | 100.4% | 100.1% |
| Class F | -0.07% | 0.20% | 0.18% | 100.4% | 100.8% | 100.1% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

# References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, 25th Meeting, by teleconference, 12–21 January 2022.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).