| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Sapporo, JP, 10-19 July 2024 | Document: JVET-AI0221-v5 |
| --- | --- |

| Title: | EE1-1.3: On Low Complexity Operational Point for In-Loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thomas Ryder, Samuel Eadie, Yun Li, Dmytro Rusanovskyy, Marta Karczewicz (Qualcomm) | Email: tryder@qti.qualcomm.com seadie@qti.qualcomm.com yli30@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Unified Filter Architecture for NN In-Loop Filtering was adopted to NNVC targeting the HOP (JVET-AF0041) at 450kMAC/pixel and LOP (JVET-AF0043) at 17kMAC/pixel complexities. Several additional contributions indicated that further complexity reduction is feasible, including the configuration for Very Low Operational Point (5kMAC/pixel), e.g. JVET-AG0051.

This contribution reports EE1-1.3 test results. Proposed NN filter was trained with LOP2 Stage 3 dataset, and tested in float32 with NNVC9.1rc SW following NNVC CTC and EE1 guidelines specified in JVET-AH2023.

At the complexity 15.6kMAC/pixel, proposed method with nnIntra ON provides {-0.8%, 1.2%, 0.1%} BD-Rate change in RA configuration and {-0.7%, 1.4%, 1.3%} in AI configuration, comparing to corresponding NNVC9.1 LOP anchor. Comparing to VTM, reported BD-Rate change is { -8.1%, -12.4%, -11.5%} and { -9.0%, -13.1%, -12.8%} in RA and AI configurations, respectively.

At the 4.66kMAC/pixel complexity, proposed method with nnIntra ON provides {-0.7%,-1.5%, -1.2%%} BD-Rate change in RA configuration and {-0.6%, -1.4%, -1.3%} in AI configuration, comparing to the corresponding NNVC9.1 VLOP anchor. Comparing to VTM, reported BD-Rate change is { -6.1%, -7.2%, -6.6%} and { -7.6%, -9.2%, -9.0%} in RA and AI configurations, respectively.

# Introduction

Unified Filter Architecture for NN In-Loop Filtering was adopted to NNVC targeting the HOP (JVET-AF0041) at 450kMAC/pixel and LOP (JVET-AF0043) at 17kMAC/pixel complexities. Several additional contributions indicated that further complexity reduction is feasible, including the configuration for Very Low Operational Point (5kMAC/pixel), e.g. JVET-AH0051.

A Unified Filter Architecture for the LOP was proposed in JVET-AF0043 and adopted to NNVC as LOP2, its architecture is shown in Figure 1. Its configuration for Very Low Operational Points (5kMAC) is being tested in EE1-5 (JVET-AH0051).

Figure 1 LOP2 network architecture.

Furthermore, the test EE1-2.3  investigates usage of attention block (AB) to improve the performance HOP, tested architecture is shown in Figure 2.

Figure 2 HOP + AB network architecture tested in EE1-2.3.

# Proposed architecture

This contribution proposes a refined neural network filter architecture that introduces a spatial attention block (AB) and the use of depth wise separable Backbone Blocks (BB). The architecture aims to improve performance of the Unifier Filter Architecture at LOP and VLOP.

The proposed architecture employs a Residual Group structure that integrates BB and AB modules. The Residual Group consists of a sequence of BBs, each followed by an AB. BBs utilize depth-wise separable convolutions for efficient feature processing, while AB modules refine features through a lightweight spatial attention mechanism. The group implements skip connections between blocks to facilitate information flow across the network.

The AB module processes input features through two parallel branches:

- A main branch that preserves spatial information through convolutions.

- A pooling branch that captures broader context using max pooling and BBs. These branches are combined to generate an attention map, which is then applied to the input features via element-wise multiplication and a hard sigmoid activation map.

In the proposed NN filter architecture, shown in Figure 3, use of the BB is extended to the Headblock (input feature extraction of pixel data), Fusion/Transition block and to the Tailblock. Additionally, use of BB and AB are harmonized by introduction of the Residual Group, shown in Figure 4. A block diagram of the proposed BB is shown in Figure 5; a block diagram of the proposed AB is shown in Figure 6.

The attention mechanism introduced here differs from that of the HOP AB model. The attention is performed with an element-wise multiplication of output features to the attention map, which is produced from the same output features.

Target complexity level (LOP or VLOP) is achieved by adjusting the number of Residual Groups, the number of BBs in each group, and the number of processing channels.

We further employ an Exponential Moving Average (EMA) of model weights to enhance generalization. This technique stabilizes training and often yields superior performance on validation sets. Importantly, we use the EMA weights for inference, which consistently outperforms the directly trained model without additional computational overhead. This approach provides an efficient alternative to model ensembling, particularly beneficial for our Unifier Filter Architecture.

Figure 3 Unified Filter Architecture with proposed changes.

Figure 4 Proposed Residual Group.

Figure 5 Proposed Backbone Block

Figure 6 Proposed Attention Block

# Training process

Proposed method in LOP and VLOP configurations were trained with LOP2 Stage 3 dataset, generally following the LOP training procedure.

Table 1 Training information.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia GeForce RTX4090 / A100 |
|  | Framework: | Pytorch v1.12.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage3: 60 epochs |
|  | Batch size: | Stage3: 64 |
|  | Loss function: | L1, L2 |
|  | Training time (for 1 model): | ~7 to 10 days per stage |
|  | Training data information: | LOP2 Stage 3 dataset: DIV2K, BVI-DVC, TVD |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | Stage3: 8e-4 |
|  | Learning rate update strategy | Stage3: 37, 39, 40. mse-epoch:40 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: |  |
|  | Other information: | EMA model, decay=0.995, updated every iteration |

# Inference process

Inference testing was conducted using NNVC9.0 software in float32 configuration. The inference information for LOP and VLOP configurations are shown in Table 2 and Table 3, respectively. The models’ parameter counts and computational complexity are reported from SADL.

Table 2 Inference information for LOP configuration

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Pytorch float |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters | 54726 |
|  | Total Parameter Number |  |
|  | Parameter Precision (Bits) | Floating (32 bits) |
|  | Memory Parameter (MB) |  |
|  | MAC/Pixel (Kilo) | 15.66 |
| Optional |  |  |
|  | Total Conv. Layers | 127 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 128x128 / 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Other information: |  |

Table 3 Inference information for VLOP configuration

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Pytorch float |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters | 18863 |
|  | Total Parameter Number |  |
|  | Parameter Precision (Bits) | Floating (32 bits) |
|  | Memory Parameter (MB) |  |
|  | MAC/Pixel (Kilo) | 4.66 |
| Optional |  |  |
|  | Total Conv. Layers | 109 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 128x128 / 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Other information: |  |

Proposed method has been tested in float32 configuration, following the NNVC CTC, with nnIntra enabled.

Summary of comparative results of proposed LOP and VLOP configurations compared to NNVC VTM are shown in Tables 4 and 5, respectively and compared to NNVC9 and NNVC9 VLOP in Tables 6 and 7, respectively [6].

Table 4 Inference testing results, LOP BD-rate change comparing to NNVC VTM

|  | LOP_float32 vs NNVC VTM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -9.38% | -11.20% | -12.51% | -9.55% | -13.44% | -13.37% |
| Class A2 | -7.84% | -11.38% | -7.74% | -8.02% | -13.44% | -10.26% |
| Class B | -7.88% | -12.75% | -12.02% | -8.13% | -12.69% | -13.10% |
| Class C | -7.57% | -13.51% | -12.73% | -8.37% | -12.87% | -12.94% |
| Class E |  |  |  | -11.94% | -13.20% | -14.25% |
| Overall | -8.09% | -12.37% | -11.45% | -9.03% | -13.07% | -12.83% |
| Class D | -8.41% | -13.35% | -13.08% | -8.36% | -11.27% | -13.00% |
| Class F |  |  |  |  |  |  |

Table 5 Inference testing results, VLOP BD-rate change comparing to NNVC VTM

|  | VLOP_float32 vs NNVC VTM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -7.44% | -6.45% | -7.22% | -8.40% | -9.58% | -8.90% |
| Class A2 | -5.94% | -6.24% | -4.12% | -6.86% | -9.48% | -6.80% |
| Class B | -5.91% | -7.55% | -7.15% | -6.98% | -8.79% | -9.69% |
| Class C | -5.26% | -8.06% | -7.21% | -6.76% | -8.77% | -8.91% |
| Class E |  |  |  | -9.88% | -9.51% | -10.01% |
| Overall | -6.05% | -7.20% | -6.57% | -7.63% | -9.15% | -8.96% |
| Class D | -5.58% | -7.37% | -7.53% | -6.78% | -7.88% | -9.33% |
| Class F | -3.02% | -4.74% | -5.15% | #VALUE! | #VALUE! | #VALUE! |

Table 6 Inference testing results, LOP BD-rate change comparing to NNVC9

|  | LOP_float32 vs NNVC9 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.56% | 1.15% | 0.23% | -0.34% | 0.87% | 1.84% |
| Class A2 | -0.48% | 0.96% | 0.44% | -0.53% | 0.11% | 0.42% |
| Class B | -0.78% | 2.19% | 0.65% | -0.56% | 1.66% | 1.50% |
| Class C | -1.11% | 0.29% | -0.90% | -1.00% | 0.75% | 0.25% |
| Class E |  |  |  | -1.11% | 3.45% | 2.46% |
| Overall | -0.76% | 1.23% | 0.11% | -0.71% | 1.36% | 1.26% |
| Class D | -1.94% | -1.22% | -1.93% | -1.19% | 0.44% | -0.84% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 10 Inference testing results, VLOP BD-rate change comparing to NNVC9_VLOP

|  | VLOP_float32 vs NNVC9_VLOP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.36% | -1.39% | -1.57% | -0.98% | -1.77% | -0.70% |
| Class A2 | -0.67% | -1.64% | -0.38% | -0.64% | -2.39% | -0.69% |
| Class B | -0.63% | -1.25% | -0.97% | -0.54% | -1.25% | -1.60% |
| Class C | -0.19% | -1.91% | -1.80% | -0.41% | -0.99% | -1.11% |
| Class E |  |  |  | -0.67% | -1.17% | -2.40% |
| Overall | -0.67% | -1.53% | -1.19% | -0.62% | -1.45% | -1.32% |
| Class D | -0.31% | -1.00% | -1.17% | -0.40% | -0.31% | -1.19% |
| Class F | -0.18% | -0.55% | -1.75% | #VALUE! | #VALUE! | #VALUE! |

# Conclusion:

This contribution proposes to improve performance of the Unifier Filter Architecture at LOP and VLOP operations points by introducing spatial attention mechanism and harmonization of the backbone block utilization. Proposed NN filter was trained with LOP2 Stage 3 dataset and tested with NNVC9.0 following NNVC CTC.

At the complexity 15.6kMAC/pixel, proposed method with nnIntra ON provides {-0.8%, 1.2%, 0.1%} BD-Rate change in RA configuration and {-0.7%, 1.4%, 1.3%} in AI configuration, comparing to corresponding NNVC9.1 LOP anchor. Comparing to VTM, reported BD-Rate change is { -8.1%, -12.4%, -11.5%} and { -9.0%, -13.1%, -12.8%} in RA and AI configurations, respectively.

At the complexity of 4.664kMAC/pixel, proposed method with nnIntra ON provides {-0.7%,-1.5%, -1.2%%} BD-Rate change in RA configuration and {-0.6%, -1.4%, -1.3%} in AI configuration, comparing to the corresponding NNVC9.1 VLOP anchor. Comparing to VTM, reported BD-Rate change is { -6.1%, -7.2%, -6.6%} and { -7.6%, -9.2%, -9.0%} in RA and AI configurations, respectively.

# References

- D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), J. N. Shingala, A. Shyam (Ittiam), T. Shao, P. Yin (Dolby), “AHG11: Status of the joint EE1-0 (LOP.2) unified filter training”, JVET-AF0043, October 2023.

- D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), “EE1-Related: On LOP2 training process”, JVET-AG0156, January 2024.

- Z. Xie, Y. Yu, H. Yu, D. Wang(OPPO), “[AHG11] Study on lower-complexity NNLF”, JVET-AG0057, January 2024.

- D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), “EE1-Related: On Low Complexity Operational Point for In-Loop Filtering”, JVET-AG0155, January 2024.

- J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), “AHG11: Complexity reduction of NN-based loop-filters”, JVET-AF0206, October 2023.

- F. Galpin, R. Chang, Y. Li, Y. Li, J. N. Shingala, Z. Xie, “J VET AHG report: NNVC software development (AHG14)”, JVET-AI0014, July 2024.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).