| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0223-v3 |
| --- | --- |

| Title: | AHG12: Improved implicit MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Charles Bonnineau Saurabh Puri Ismail Marzuki Renan Utida Karam Naser Tangi Poirier Fabrice Le Léannec | Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to replace the implicit MTS method of ECM-13.0 by an intra mode and TU size dependent method based on a lookup-table (LUT),

On top of the default MTS implicit method of ECM-13.0, simulation results are reported as follows:

AI {-0.14% Y, 0.04% U, 0.01% V, 100.4% EncT, 102.1% DecT}

# Introduction

Implicit MTS has been introduced in VVC as an alternative tool to explicit MTS. This tool reduces the encoder complexity by inferring the optimal transform type from decoder-side information instead of performing an intensive rate-distortion search in the encoder.

However, implicit MTS remained unchanged in the enhanced compression model (ECM), which introduce new primary transforms to explicit MTS. The horizontal and vertical transform are inferred as follows:

    - trHor = DST7,   if 4≤W≤16 DCT2,            otherwise

trVer = DST7,  if 4≤H≤16 DCT2,           otherwise

It is asserted that the implicit MTS method can benefit from the introduction of the new primary transforms introduced in the ECM.

# Proposed method

In this proposal, the implicit MTS method of the ECM-13.0 is improved. The implicit MTS method is made intra-mode and TU-size dependent using a LUT as in intra explicit MTS. In addition, other transforms than DST7 and DCT2 are considered.

The proposed LUT infers a single primary transform pair to the current block based on its intra prediction mode and TU. A total of 36 different transform pairs are considered in the LUT based on the following separable primary transforms:

{DCT2, DCT5, DCT8, DST1, DST4, DST7}

First, the intra-mode of the current block is obtained the same way as in intra explicit MTS. Then, both the intra mode and the size of the current block are used as input to the LUT to infer the transform pair of the current block. If the block is larger than 32x32, the size used as input to the LUT is set to 32x32.

The ECM-13.0 implicit MTS is kept unchanged for ISP blocks.

# Experimental results

The proposed method is implemented on top of the ECM-13.0 [1] and compared with the default implicit MTS method using all intra (AI) configuration.

Table 1: Experimental results of the proposed method over the of ECM-13.0 with MTS=0 and MTSImplicit=1 (AI).

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.14% | 0.05% | -0.22% | 101.1% | 102.8% | 100.2% | 99.5% |
| Class A2 | -0.15% | 0.04% | 0.05% | 100.4% | 102.2% | 100.3% | 99.5% |
| Class B | -0.17% | 0.09% | 0.09% | 100.0% | 102.7% | 100.8% | 99.7% |
| Class C | -0.12% | -0.01% | 0.07% | 100.1% | 101.7% | 99.7% | 100.2% |
| Class E | -0.13% | 0.04% | -0.05% | 100.5% | 100.8% | 100.1% | 100.1% |
| Overall | -0.14% | 0.04% | 0.01% | 100.4% | 102.1% | 100.2% | 99.8% |
| Class D | -0.15% | -0.16% | -0.03% | 100.0% | 101.0% | 100.1% | 100.0% |
| Class F | -0.08% | -0.05% | 0.05% | 100.3% | 102.4% | 100.6% | 100.1% |

Table 2: Experimental results of the proposed method over ECM-13.0 with MTS=0 and MTSImplicit=1 (RA).

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 (MTS=0, MTSImplicit=1) |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.06% | 0.03% | 100.8% | 100.6% | 100.1% | 99.9% |
| Class C | -0.04% | 0.17% | 0.28% | 100.2% | 100.4% | 100.5% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.06% | 0.10% | -0.03% | 99.8% | 99.3% | 100.2% | 100.1% |
| Class F | -0.03% | -0.15% | -0.09% | 99.7% | 101.8% | 99.9% | 99.7% |

Table 3: Experimental results of the proposed method over ECM-13.0 with MTS=0 and MTSImplicit=1 (LDB).

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 (MTS=0, MTSImplicit=1) |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.00% | -0.37% | 0.08% | 99.8% | 98.5% | 100.1% | 100.1% |
| Class E | -0.09% | 0.36% | 0.11% | 101.1% | 100.5% | 99.1% | 100.2% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.06% | 0.29% | 0.57% | 97.6% | 97.6% | 100.1% | 100.1% |
| Class F |  |  |  |  |  |  |  |

# Conclusion

It is proposed to include the proposed implicit MTS method in the ECM software.

# References

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).