| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0262-v1 |
| --- | --- |

| Title: | AHG 12: OBIC mode from reconstruction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ramin G. Youvalari, Mohsen Abdoli, Alexandre Tissier | Email: | raminyouvalari@xiaomi.com |
| Source: | Xiaomi Technology |  |  |

_____________________________

# Abstract

This contribution proposes a new occurrence-based intra coding (OBIC) mode in order to further improve the performance of ECM. The proposed method uses recalculated intra prediction modes (IPMs) from the reconstructed neighborhood blocks rather than IPMs that are used for prediction of those blocks. For this, intra prediction search process is applied to reconstructed blocks in the decoder side in order to find more accurate intra prediction modes. The recalculated intra prediction modes are then stored and used for OBIC derivation of future blocks.

The impact on coding efficiency and runtimes of proposed method over ECM-13.0 are reportedly {for Y, U, V, EncT, DecT}:

AI { -0.05%, 0.01%, 0.04%, 10x.x%, 10x.x%}

# Introduction

The occurrence-based intra coding (OBIC) [1][2] in ECM-13.0 derives the intra prediction modes of the current block based on the sample-wise occurrence of the intra modes in the spatial neighborhood of the block. For this, adjacent and non-adjacent spatial neighboring blocks are checked and the intra prediction modes of the blocks are collected into an occurrence histogram. Instead of Histogram of Gradient (HoG) as in DIMD, the OBIC method uses the Histogram of Occurrences, which consists of the intra modes and their sample-wise occurrences. The occurrence values are calculated based on the number of samples that are coded in a certain intra prediction mode in that neighborhood. For example, if a uiWidth × uiHeight block is coded with an IPM mode, the occurrence of the mode in that particular block is calculated as:

Histogram[IPM] += uiWidth * uiHeight;

Where uiWidth and uiHeight are the width and height of a spatial neighboring block. The occurrences of the existing modes from the spatial neighborhood blocks are accumulated into the histogram.

Up to five angular modes with the highest occurrence along with the planar mode or block vector-based prediction (same as in DIMD) are selected from the histogram and used for final prediction by blending the prediction of the selected modes.

The blending weights are calculated similar to the DIMD mode, but instead of using gradient values from the template, the occurrence values are used for OBIC. Moreover, the planar mode’s weight is also decided similarly to DIMD mode.

# Proposed OBIC mode

The new OBIC mode uses recalculated intra prediction modes of the neighborhood blocks in order to build the histogram of occurrences. Up to three IPMs are calculated for a block from its reconstruction and then used in the new OBIC mode.

## IPM recalculation from reconstructed block

After a block, coded in any mode, is reconstructed, its intra prediction modes are recalculated based on the reconstructed samples in the decoder side. For this, DIMD process is applied to the reconstructed samples and up-to six best IPMs with highest amplitudes from the DIMD process along with Planar and DC modes are selected to be used as initial IPM list. Figure 1 shows an example of this process.

Figure 1. DIMD-like process for IPM derivation from reconstruction

Once a list of IPMs is generated, they are used for predicting the block. The IPM list is then sorted based on the SATD cost compared to the reconstructed samples. Best three IPMs with the lowest SATD costs are selected and stored.

Figure 2. Sorting IPMs based on distortion cost

## Further details

When a new OBIC mode is applied, the histogram of occurrences is generated based on the recalculated IPMs of the neighboring blocks.

When generating the histogram, only the adjacent blocks from the neighborhood are considered.

Up to 6 intra prediction modes with the highest occurrence values in the histogram are selected and used for final prediction by blending the prediction of the selected modes. The blending weights are calculated similar to the DIMD mode.

The intra prediction mode recalculation is done for all intra coded blocks.

# Bitstream Signalling

Usage of the mode is signaled with a CABAC coded PU level flag. The new OBIC mode is used as a sub-mode of DIMD, and its flag is signaled after DIMD flag and before existing OBIC mode of ECM-13.0.

# Results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM-13.0 [2] using the JVET common test conditions [3].

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07 % | -0.01 % | 0.07 % |  |  |  |  |
| Class A2 | -0.05 % | 0.03 % | 0.06 % |  |  |  |  |
| Class B | -0.06 % | 0.00 % | -0.06 % |  |  |  |  |
| Class C | -0.04 % | -0.01 % | 0.12 % |  |  |  |  |
| Class E | -0.03 % | 0.03 % | 0.03 % |  |  |  |  |
| Overall | -0.05 % | 0.01 % | 0.04 % |  |  |  |  |
| Class D | -0.03 % | -0.03 % | -0.19 % |  |  |  |  |
| Class F | -0.04 % | 0.17 % | -0.02 % |  |  |  |  |
| Class TGM | -0.04 % | -0.01 % | -0.06 % |  |  |  |  |

# Conclusion

This contribution proposes a new occurrence-based intra coding (OBIC) mode. The proposed OBIC method uses recalculated intra prediction modes from the reconstructed blocks for generating the prediction modes for the new OBIC tool. The experimental results showed that the new OBIC mode provides 0.05% average luma BD-Rate gain in AI configuration over ECM-13.0. It is suggested to further study the proposed method in the next exploration experiment 2 (EE2) round.

# References

- Coban et.al, “Algorithm description of Enhanced Compression Model 13 (ECM 13)”, JVET-AH2025, April. 2024.

- R. G. Youvalari, M. Abdoli, “AHG 12: Occurrence-based intra coding (OBIC)”, JVET-AG0141, Jan. 2024.

- ECM-13.0 reference software, https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).