| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024 | Document: JVET-AI0264-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AI0074 (EE1-4.1: Multiple scaling ratios coding for NNVC with NNSR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chaoyi Lin | Tel: Email: | linchaoyi.cy@bytedance.com |
| Source: | Bytedance |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for EE1-4.1, which uses multiple scaling ratios coding for NNVC super resolution. For training crosscheck, the model trained by crosschecker shows maximum 0.01% luma coding performance difference compared to the results provided by JVET-AI0074. The training crosscheck is passed. For inference crosscheck, the results exactly match with JVET-AI0074.

# Introduction

This document reports the crosscheck results of EE1-4.1 (JVET-AI0074), which proposed to use single super resolution (SR) model to handle different scaling factors. The scaling factor for each frame is determined by the encoder before encoding.

# Training crosscheck

## EE-4.1.2

For EE1-4.1.2, crosschecker trained the model from scratch with the training scripts provided by proponents. The training data is generated with LOP2 and NNintra enabled. Both 2x and 1.5x data are generated and used for training. The model is trained with 60 epochs. After training, the model is quantized to integer model for inference.

The training crosscheck results are presented in the following tables.

Table 1 Results of RA configuration over NNVC9 anchor

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class A1 | -3.02% | 1.41% | 4.43% |
| Class A2 | -0.99% | 6.69% | 6.77% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |
| Overall | -0.80% | 1.62% | 2.24% |

Table 2 Results of AI configuration over NNVC9 anchor

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class A1 | -1.67% | 4.89% | 5.62% |
| Class A2 | -3.18% | 8.91% | 10.02% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |
| Overall | -0.81% | 2.30% | 2.61% |

# Inference crosscheck

## EE-4.1.1

Table 3 Results of RA configuration over NNVC9 anchor

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class A1 | -2.64% | 1.88% | 4.10% |
| Class A2 | -0.01% | 9.80% | 9.50% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |
| Overall | -0.53% | 2.34% | 2.72% |

Table 4 Results of AI configuration over NNVC9 anchor

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class A1 | -1.42% | 4.92% | 5.32% |
| Class A2 | -1.29% | 14.00% | 13.90% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |
| Overall | -0.45% | 3.15% | 3.20% |

## EE-4.1.2

Table 5 Results of RA configuration over NNVC9 anchor

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class A1 | -2.97% | 7.68% | 6.23% |
| Class A2 | -1.03% | 7.61% | 6.65% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |
| Overall | -0.80% | 3.06% | 2.58% |

Table 6 Results of AI configuration over NNVC9 anchor

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class A1 | -1.59% | 6.97% | 7.77% |
| Class A2 | -3.22% | 10.10% | 10.49% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |
| Overall | -0.80% | 2.85% | 3.04% |
| Class D | 0.00% | 0.00% | 0.00% |

# Conclusion

Training and inference crosscheck were performed. Training crosscheck is passed where minor difference is observed during training crosscheck. Besides, the results of inference crosschecks are totally matched.