| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0278-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AI0221 (EE1-1.3: On Low Complexity Operational Point for In-Loop Filtering) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Jacob Ström | Tel: Email: | jacob.strom (at) ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document is a training crosscheck of JVET-AI0221, which proposes two filters, one based on LOP2 and one on VLOP. We have crosschecked the one based on LOP2. Due to a git history problem, an older version of the training software was used. The training was done three times in order to avoid training noise. For the best training, the (partial) inference results over NNVC9.1 (LOP3+NNIntra) are:

RA (average class B+C):	-0.42% (Y) 2.19% (U) 1.01% (V)
AI (full results): 			-0.25% (Y)  2.68% (U) 2.33% (V)

The proponents inference results for these two classes results over NNVC9.1 (LOP3+NNIntra) are:

RA (average class B+C):	-0.94% (Y)	1.24% (U) -0.13% (V)
AI (full results): 			-0.71% (Y) 1.36% (U) 1.26% (V)

It is the crosschecker’s assumption that the discrepancy in training performance is due to having used the older training software.

# Crosscheck carried out

Summary of crosscheck

- Inference of the float32 model from proponents was (partially) tested with matching results. No int16 model or results are available at the time of writing.

- The training was carried out three times on the crosschecker’s side to account for training noise. After running inference on classC and classD, the third training was seleced for full inference since it produced the best results.

- The model tested is built on top of LOP2 (not on top of LOP3)

- Manually evaluating kMACs complexity is difficult due to lack of excel sheet – it may be difficult to construct an excel sheet for this type of model.

## Git history problem

At June 17, the crosschecker downloaded the code from the JVET EE1 git repository. At June 18, the day of the deadline, the proponents uploaded new code. However, since ‘git amend’ was used instead of ‘git commit’, the date of the latest commit didn’t change (it was still ’ Fri Jun 14 21:15:13 2024’), only the content. Hence the crosschecker did not notice that there was an updated training code. Hence the crosscheck was carried out using the old training code. This can explain the difference in performance between the crosschecker’s training and that of the proponents.

## SADL conversion problem

Several problems were encountered for the crosschecker when trying to convert the trained models to SADL. The first issue was that ‘Identity’-layers were introduced. When this was fixed, another issue occurred with constant layers. Fixing these issues took about a week, and this explains why only partial crosscheck results are provided.

# Results

## Random Access

Results for the crosschecker’s trained model for RA. EncT and DecT are not reliable:

|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.18% | 3.01% | 1.68% | 97% | 149% |
| Class C | -0.66% | 1.37% | 0.35% | 98% | 139% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1.19% | 1.01% | -0.12% | 105% | 150% |
| Class F | -0.31% | -0.98% | -1.80% | 108% | 147% |

## All-intra

Results for the crosschecker’s trained model for AI. EncT and DecT are not reliable:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.14% | 2.29% | 2.30% | 110% | 185% |
| Class A2 | -0.09% | 2.21% | 1.84% | 101% | 184% |
| Class B | -0.19% | 2.76% | 2.53% | 106% | 175% |
| Class C | -0.61% | 1.90% | 1.42% | 102% | 180% |
| Class E | -0.43% | 4.44% | 3.73% | 97% | 182% |
| Overall | -0.25% | 2.68% | 2.33% | 104% | 180% |
| Class D | -0.74% | 2.07% | 0.63% | 102% | 174% |
| Class F | -0.47% | -0.35% | -0.89% | 104% | 176% |

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).