| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0341-v2 |
| --- | --- |

| Title: | AHG9: Semantics of po_sei_importance_flag[ i ] and po_sei_processing_degree_flag[ i ] |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela (Nokia) Hendry Tan (LG Electronics) Ping Wu, Ying Gao (ZTE) |  |  |
| Source: | Nokia, LG Electronics, ZTE |  |  |

_____________________________

# Abstract

This document results from an offline discussion related to JVET-AI0098. It proposes semantics for po_sei_importance_flag[ i ] and po_sei_processing_degree_flag[ i ].

v2 of the contribution changes "may" to "shall" in the semantics of po_sei_importance_flag[ i ] equal to 0 as requested by the JVET review.

# Proposed text

po_sei_importance_flag[i] equal to 1 affects the derivation of PoSeiList, which is the list of SEI messages that a decoding system should process for a particular picture picA, as specified below.

po_sei_importance_flag[i] equal to 0 specifies that when the decoding system cannot interpret or does not support the functionality indicated by the i-th SEI message type, it shall ignore all data associated with the loop variable value of i and exclude the i-th SEI message type from the processing chain performed by the decoding system.

po_sei_processing_degree_flag[i] affects the derivation of PoSeiList as specified below.

PoSeiList is derived as follows:

- PoSeiList is initially empty.

- The following applies in a non-decreasing order of po_sei_processing_order[ i ] values for all values of i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive, unless terminated earlier as specified below:

  - When an SEI message seiA associated with the i-th SEI message type persists for picA, the following applies:

    - If the decoding system can interpret and supports the functionality indicated by seiA, seiA is added at the end of PoSeiList.

    - Otherwise, if po_sei_importance_flag[i] is equal to 1 and po_sei_processing_degree_flag[i] is equal to 0, the derivation of PoSeiList is terminated.

    - Otherwise, if po_sei_importance_flag[i] is equal to 1 and po_sei_processing_degree_flag[i] is equal to 1, the processing chain specified by this SPO SEI message should not be performed for picA, PoSeiList is set to be empty, and the derivation of PoSeiList is terminated.

# Patent rights declaration(s)

Nokia, LG Electronics, and ZTE may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).