| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0344-v1 |
| --- | --- |

| Title: | BoG report: Gaming content compression |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Saurabh Puri, InterDigital Johannes Sauer, Huawei Roman Chernyak, Tencent Alberto Duenas, WBD Limin Wang, Nokia | Email: | Saurabh.Puri@InterDigital.com johannes.sauer@huawei.com chernyak@global.tencent.com alberto.duenas@wbd.com limin.2.wang@nokia.com |
| Source: | BoG on gaming content compression |  |  |

_____________________________

# Abstract

This is the report on the BoG on gaming content compression. The group met in the Small Hall on 18.07.24 at 9:00 AM.

# Pruning of Class G2

| Sequence | Pro | Contra | Camera |
| --- | --- | --- | --- |
| DOTA2 | only one that not first/3rd person moving camera | inproper language, already AoV in classF | 3rd person, fixed |
| GTAV | should have one racing game | similar content as Project CARS | 3rd person, free |
| Minecraft | very original “look” |  | 1st person, free |
| Witcher3 |  | very dark | 3rd person, free |
| Baolei-Man |  | similar content as Baolei-Yard | 3rd person, free |
| Jianling-Temple | very challening, many details, text |  | 3rd person, free |
| Project CARS | should have one racing game | has strong artifacts in original, similar content as GTAV, low frame rate, 30fps | 3rd person, free |
| Baolei-Yard 4K | 4K | similar content as Baolei-Man | 3rd person, free |

- Has GTAV been used for training?

- Have AoV in classF, may request restructuring of classes later. (align classF, TGM, gaming content classes)

- Not making AoV optional

  - Can always update CTC again if necessary

  - Prooponents can always bring additional results on non-CTC sequences (which is basically same as optional)

- Consider Heroes of the storm for CTC

  - Need some results for it

# New classes structure

Suggested new classes:

Basic modification (pending confirmation of updates to ARPG/SunTemple/DesertTown):

- G1 (2 sequences SDR, auxiliary data)

- G2 (4 sequences SDR)

- G3 (3 sequences, HDR, auxiliary data)

  - Check HDR CTC

Further improvements can be done, but which require new renderings in either SDR or HDR of sequences:

Variant 1

This variant reduces the number of classes.

- G1 (5 sequences HDR, auxiliary data)

  - Can Level1 and Darktree be rendered in HDR using static meta data solution by interdigital

- G2 (4 sequences SDR)

Variant 2

This variant has the same sequences in SDR and HDR versions. It can be used to see if tools specific to SDR/HDR are useful.

- G1 (5 sequences SDR, auxiliary data)

- G2 (4 sequences SDR)

- G3 (3 or 5 sequences, HDR, auxiliary data)

  - Check HDR CTC

# Suggesting improvements to draft CTC

- Only do either LDB or LDP to reduce simulation load

  - LDB mandatory

    - Better alignment with JVET CTC

  - LDP optional

- Run time difference between of LDP/LDB. LDP about 70% time of LDB.

- Motion vector map has different coordinate origin than used in ETM/VTM

  - Y-axis is reversed twice (thus fine)

  - X-axis is reversed once (x component of mv needs to be reversed)

  - Should specify origin and direction of the axis for motion vector map

  - Compare data stored from both unity and unreal engines

# Discussion on transportation of camera parameters

- Seems out of scope

  - Too early, haven’t seen any proposal using auxiliary data yet

- For information:

  - There is existing SEI message (from MIV)  (multiview acquisition information SEI message in H274)

    - Camera intrinsics, extrinsics, projection

    - May not be sufficient to have SEI if parameters should be used at low level

    - Unclear whether there will be low level tools using those parameters

- Further study required

# Timeline

Intention to have a telco 4 weeks after meeting.

Todo:

- Upload of auxiliary data

- Conversion of sequences to SDR/HDR

- Editing of CTC accordingly

# Others

Reading auxiliary data:

- It was requested to have auxiliary data available in both VTM and ECM, not only in VTM as currently decided

  - Can there be two branches?

It was suggested to distinguish

- Auxiliary meta data

- Auxiliary video data

  - Change while editing CTC for next round

Software to use for next experiments:

- Use ECM-14.0 for next round. VTM11.0ecm14.0

Occlusions in sequences

- What is value of motion vectors in motion maps

  - Unclear, may be unreliable

Does accuracy of auxiliary data vary?

- Due to occlusions

- Due to computational load

Using MTT splitting parameter modification for QP22 also for QP27

- It was observed that QP27 takes longer QP22

- Previous contribution AH0220

- Suggested to have input on next meeting