| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI2028-v1 |
| --- | --- |

| Title: | Additional conformance bitstreams for VVC multilayer configurations (Draft 4) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Philippe de Lagrange Iole Moccagatta | Email: | iwamura.s-gc@nhk.or.jp philippe.delagrange@interdigital.com iole.moccagatta@intel.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is draft 4 of the additional conformance testing specification for VVC multilayer configurations, for testing the multilayer coding features supported in the first and second edition of VVC (Rec. ITU-T H.266 | ISO/IEC 23090-3).

Due to the size of the electronic attachments, the associated conformance test set data files are provided separately, at the following location:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCMultilayer/under_test/VTM-23.0 (user id: avguest, password: Avguest201007)

For ftp access, it is suggested to use FileZilla, for which the site manager feature should be used. See Annex A of this document for FileZilla suggested configuration.

Alternatively, the same files are available by http at:

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCMultilayer/under_test/VTM-23.0

Annex A of this document contains JVET guidelines for contributing to the conformance bitstream test set. It is not expected to be published as part of the text approved by the parent bodies.

Conformance bitstreams preparation (editors’ notes – not part of the specification text)

This section is to be removed after work on the conformance bitstream production and verification is completed.

reference software decoder: The software decoder provided in Rec. ITU-T H.266.2 | ISO/IEC 23090-16; the current implementation is VTM-23.0.

reference software encoder: The software encoder provided in Rec. ITU-T H.266.2 | ISO/IEC 23090-16; the current implementation is VTM-23.0.

See also Annex A of this document.

Table 1. List of additional bitstream, volunteering proponents and cross-checkers.

| Bitstream name | Bitstream proponent | Bitstream cross-checker |
| --- | --- | --- |
| OLSMultiProfile_A | NHK | InterDigital |
| OLSMultiProfile_B | NHK | InterDigital |
| OLSMultiProfile_C | NHK | InterDigital |
| PPSScalWin_A | NHK | Ericsson |
| PPSScalWin_B | NHK | Ericsson |
| PPSScalWin_C | NHK | Ericsson |
| ILRPL_B | InterDigital | NHK |

Draft changes to the specification text

# 6.6

# In subclause 6.6, add subclauses as follows:

    - Test bitstreams – Multilayer Main 10 profile

      - Layered coding with OLS (OLS)

        - Test bitstream OLSMultiProfile_A

Specification: The bitstream is coded with two layers, where OLS0 including the lower layer specifies Main 10 profile whereas OLS1 including both layers specifies Multilayer Main 10

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting Main 10 can properly decode OLS0 and the decoder supporting Multilayer Main 10 can properly decode both OLS0 and OLS1.

        - Test bitstream OLSMultiProfile_B

Specification: The bitstream is coded with two layers, where each layer is coded as independent layer and both OLSs specifies Main 10 profile.

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting Main 10 can properly decode both OLSs.

        - Test bitstream OLSMultiProfile_C

Specification: The bitstream is coded with three layers, where lowest independent layer is referenced from the two higher layers, where OLSs for higher layer specifies Multilayer Main 10 profile whereas OLS0 specifies Main 10 profile.

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting Main 10 can properly decode OLS0 and the decoder supporting Multilayer Main 10 can properly decode all the OLSs.

        - Test bitstream PPSScalWin_A

Specification: The bitstream is coded with two layers, where base layer is coded with scaling window disabled and enhancement layer is coded with scaling window enabled.

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting scaling window.

        - Test bitstream PPSScalWin_B

Specification: The bitstream is coded with two layers, where base layer is coded with scaling window enabled and enhancement layer is coded with scaling window disabled.

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting scaling window.

        - Test bitstream PPSScalWin_C

Specification: The bitstream is coded with two layers, where both base and enhancement layers are coded with scaling window enabled. The sizes of scaling windows are different.

Functional stage: High-level syntax.

Purpose: Check that the decoder supporting scaling window.

        - Test bitstream ILRPL_B

Specification: The bitstream is coded in GOP 16 configuration with two layers, where base layer is a downscaled version of layer 1 source video, with a ratio (0.5, 0.5). Layer 1 has no intra picture and has both inter-layer and intra-layer reference pictures.

Functional stage: High-level syntax, reference picture lists.

Purpose: in this bitstream, two features are tested:

1.	Avoid intra pictures in dependent layer.

2.	Exercise multiple reference pictures with mixed inter and intra layer dependencies.

# 6.7

# In subclause 6.7 (Conformance test suites for Rec. ITU-T H.266 | ISO/IEC 23090-3), add subclauses as follows:

    - Bitstreams for Multilayer Main 10 profile

Table 7: Bitstreams for Multilayer Main 10 profile

| Feature | Feature Name | Bitstream features |
| --- | --- | --- |
| Layered coding with OLS | OLS | Two layered coding where each OLS specifies different profile. |
| Layered coding with OLS | OLS | Two layered coding where each layer is coded as independent layer and both OLSs specifies Main 10 profile. |
| Layered coding with OLS | OLS | Three layered coding where lowest independent layer is referenced from two higher layers |
| Layered coding with Scaling Window | PPS | Two layered coding where enhancement layer is coded with scaling window enabled. |
| Layered coding With Scaling Window | PPS | Two layered coding where base layer is coded with scaling window enabled. |
| Layered coding with Scaling Window | PPS | Two layered coding where both layers are coded with different scaling window. |
| Layered coding with inter and intra layer dependencies | ILRPL | No intra pictures in dependent layer, and multiple reference pictures with mixed inter and intra layer dependencies. |

# Annex A to JVET-AA2026
Conformance bitstream generation guidelines

# (not part of the draft specification text)

  - General

This annex provides conformance bitstream generation guidelines.

  - Procedure

    - FTP site

An FTP site at ITU-T is used to exchange bitstreams. The site for FTP download of bitstreams is:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCMultilayer/under_test/VTM-21.2

(user id: avguest, password: Avguest201007)

If using FileZilla, the following configuration is suggested:

In the Filezilla Edit  Settings  Connection menu, it  was previously necessary to set the minimum TLS level to 1.0. Since around January 2024, this configuration issue went away, and ITU disabled the use of TLS 1.0 and 1.1 on its servers.

The site is also accessible by HTTP (without a password) at:

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCMultilayer/under_test/VTM-21.2/

The FTP site for uploading bitstream files is as follows:

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, password: Avguest201007)

Once a bitstream is uploaded to the dropbox, please send email to jvet-conformance@lists.rwth-aachen.de to notify the coordinators of its availability.

    - Bitstreams and associated files

Volunteers should upload the bitstream and associated files in a zip archive. All files inside the zip archive should have the same base name. Only the extension is changed in the following way:

*.bit – bitstream as described in section A.2.3 (mandatory)

*.txt – description (mandatory)

*.yuv.md5 – MD5 check sum of the complete decoded file (mandatory)

*.md5 – MD5sum of the bitstream file (mandatory)

*.opl – output picture log as described in section A.2.4 (mandatory)

*.cfg – config file used to generate bitstream with VTM encoder SW (optional, not applicable if VTM encoder release version not used)

All bitstreams are required to use the decoded picture hash SEI message.

All files in the zip archive should be in the top level, without a subfolder.

The .md5 file should contain only the MD5sum value and no additional characters.

The following naming convention is to be used.

Feature_BistreamID_Source_Version

- Feature is summarized in Table 1

- BitstreamID is a single capital letter, to distinguish among multiple bitstreams provided for a particular feature.

- Source is the name of the organization who provided the bitstream.

- Version is the version number of the file. The version number should be 1 for the first version. The version number should be incremented when a bitstream is replaced because of an identified problem or updated for compatibility with a new version of the VTM.

For example, for the feature “feature”, bitstream “A”, from source “organization”, in its version “1”, the archive is named "feature_A_organizationA_1.zip", and the files would be named

feature_A_organization_1.bit

feature_A_organization_1.txt

feature_A_organization_1.yuv.md5

feature_A_organization_1.md5

feature_A_organization_1.opl

feature_A_organization_1.cfg

    - Description of the bitstream (name.txt)

In a short text file which describes the bitstream, the following information is to be provided:

- Bitstream file name

- Explanation of bitstream features

- Profile, tier, and level for the bitstream

- Max picture width and height, and any additional picture width and height values used (i.e., for reference picture resample or scalability)

- Picture rate

- VTM decoder release version number able to correctly decode the bitstream with identical output, or description of VTM decoder incompatibility (preferably including a corresponding error tracking issue number.)

- VTM release version number used to generate the bitstream, if applicable

- Contact name and email of bitstream submitter

    - Output picture log file for each output layer set (name.opl)

Each output picture log file contains one row for each output picture in the bitstream, in output order.

Each row contains the following information, as Comma Separated Values (CSV).

- layer ID

- PicOrderCntVal

- Width of cropped output picture in luma samples

- Height of cropped output picture in luma samples

- MD5 checksum for the first coded component

- MD5 checksum for the second coded component

- MD5 checksum for the third coded component

The reference software provides support in the decoder to output an .opl file using the -opl command line argument. For example: DecoderApp -b feature_A_organization_1.bit -opl feature_A_organization_1.opl

    - Recommendations

Test bitstreams should exercise the full range of parameters of modes of the tool tested in the category, including corner cases. Multiple bitstreams should be provided for each feature.

As new releases of the VTM software are made available, it is recommended to provide updated versions of the bitstreams compatible with the latest release.

For testing bitstreams targeting Main 12 4:4:4, Main 12 4:4:4 Intra, Main 12 4:4:4 Still Picture, Main 16 4:4:4, Main 16 4:4:4 Intra and Main 16 4:4:4 Still Picture profiles in VTM software, the high bit-depth operation range extension supports should be enabled by compiling VTM with “-DENABLE_HIGH_BITDEPTH=1”.

The examples of the configurations files to enable high-bit depth processing can be found in “cfg/hbd” directory of VTM software.

The test bitstreams shall not violate the profile, tier, and level indicated in the bitstream. The minimum level that the bitstream conforms to should be used.

A VTM config file should be included in the .zip file if an unmodified VTM version is used, and the command line used should be included in the .txt file. If a modified version of the VTM is used, the VTM config file should not be included.

____________