This document contains the decided ASN.1 for H.225.0 as extracted from p225ncm11.doc.  This should be considered the single final master version for future work.  It is NOT THE SAME as any previous version; dispose of any previous version.  It should be included in the  H.323/H.225.0 Implementors guide, and as an Annex of H.225.0 version 2.  Note that when it is converted to ASCII for compilation as few problems may remain; most have been fixed.





Dale Skran


Editor, H.225.0 Version 1


June 18, 1996





	H323-MESSAGES DEFINITIONS AUTOMATIC TAGS ::=


	BEGIN


�	H323-UserInformation ::= SEQUENCE		-- root for all Q.931 related ASN.1�	{�		h323-uu-pdu	H323-UU-PDU,�		user-data	SEQUENCE�		{�			protocol-discriminator	INTEGER (0..255),�			user-information		OCTET STRING (SIZE(1..131)),


		...     �		} OPTIONAL,�		...�	  }





	H323-UU-PDU ::= 	SEQUENCE


	{


		h323-message-body   CHOICE


		{�		setup		Setup-UUIE,�		callProceeding	CallProceeding-UUIE,�		connect		Connect-UUIE,�		alerting		Alerting-UUIE,


			userInformation	UI-UUIE,


			releaseComplete	ReleaseComplete-UUIE,


			facility		Facility-UUIE,�		...


		},


		nonStandardData		NonStandardParameter OPTIONAL,


		...�}	





Alerting-UUIE			::=SEQUENCE


{


	protocolIdentifier	ProtocolIdentifier,		


	destinationInfo		EndpointType,			


	h245Address		TransportAddress OPTIONAL,		


	...


}











CallProceeding-UUIE			::=SEQUENCE


{


	protocolIdentifier	ProtocolIdentifier,				


	destinationInfo		EndpointType,					


	h245Address		TransportAddress OPTIONAL,		


	...


}











Connect-UUIE			::=SEQUENCE


{


	protocolIdentifier	ProtocolIdentifier,				


	h245Address		TransportAddress OPTIONAL,				


	destinationInfo		EndpointType,					


	conferenceID		ConferenceIdentifier,				


	...


}








UI-UUIE	::=SEQUENCE


{


	protocolIdentifier	ProtocolIdentifier,			


	...


}





ReleaseComplete-UUIE ::= SEQUENCE


{


	protocolIdentifier	ProtocolIdentifier,


	reason			ReleaseCompleteReason OPTIONAL,			


	...


}





ReleaseCompleteReason		::=CHOICE


{


	noBandwidth		NULL,		-- bandwidth taken away or ARQ denied


	gatekeeperResources	NULL,		-- exhausted		


	unreachableDestination	NULL,     	-- no transport path to the destination	


	destinationRejection	NULL,		-- rejected at destination


	invalidRevision		NULL,


	noPermission		NULL,		-- called party’s gatekeeper rejects


	unreachableGatekeeper	NULL,		-- terminal cannot reach gatekeeper for 	ARQ		gatewayResources	NULL,


	badFormatAddress	NULL,


	adaptiveBusy		NULL,		--  call is dropping due to LAN crowding


	inConf			NULL,		-- no address in AlternativeAddress


	undefinedReason	NULL,


	...


}	


 








Setup-UUIE			::=SEQUENCE


{


	protocolIdentifier	ProtocolIdentifier,				


	h245Address		TransportAddress OPTIONAL,				


	sourceAddress		SEQUENCE OF AliasAddress OPTIONAL,


	sourceInfo		EndpointType,						destinationAddress	SEQUENCE OF AliasAddress OPTIONAL,	


	destCallSignalAddress	TransportAddress OPTIONAL,			


	destExtraCallInfo	SEQUENCE OF AliasAddress OPTIONAL,	-- Note(1) 


	destExtraCRV		SEQUENCE OF CallReferenceValue OPTIONAL,-- Note(1)


	activeMC		BOOLEAN,					


	conferenceID		ConferenceIdentifier,				


	conferenceGoal		CHOICE					


	{


		create		NULL,


		join		NULL,


		invite		NULL,


		...


	},


	callServices		QseriesOptions  OPTIONAL,


	callType		CallType,


	...


}











Facility-UUIE ::= SEQUENCE


{


	protocolIdentifier	ProtocolIdentifier,


	alternativeAddress	TransportAddress OPTIONAL,


	alternativeAliasAddress	SEQUENCE OF AliasAddress OPTIONAL,


		conferenceID		ConferenceIdentifier OPTIONAL,			


	reason			FacilityReason,			


	...


}





FacilityReason		::=CHOICE


{


		routeCallToGatekeeper	NULL,		-- call must use gatekeeper model


							-- gatekeeper is alternativeAddress


		callForwarded		NULL,


		routeCallToMC		NULL,


		undefinedReason	NULL,


		...


	}





TransportAddress		::=CHOICE


{


	ipAddress		SEQUENCE


	{


		ip		OCTET STRING (SIZE(4)),


		port		INTEGER(0..65535)


	},


	ipSourceRoute	SEQUENCE	{


		ip		OCTET STRING (SIZE(4)),


		port		INTEGER(0..65535),


		route		SEQUENCE OF OCTET STRING(SIZE(4)),


		routing 		CHOICE


		{


			strict	NULL,


			loose	NULL,


			...


		},


		...


	},


	ipxAddress		SEQUENCE


	{


		node		OCTET STRING (SIZE(6)),


		netnum		OCTET STRING (SIZE(4)),


		port		OCTET STRING (SIZE(2))


	},


	ip6Address		SEQUENCE


	{


		ip	OCTET STRING (SIZE(16)),


		port		INTEGER(0..65535),


		...


	},


	netBios		OCTET STRING (SIZE(16)),


	nsap		OCTET STRING (SIZE(1..20)),


	nonStandardAddress	NonStandardParameter,	


	...	


}





EndpointType		::=SEQUENCE


{


	


nonStandardData	NonStandardParameter OPTIONAL,


vendor			VendorIdentifier OPTIONAL,


gatekeeper		GatekeeperInfo OPTIONAL,


	gateway		GatewayInfo OPTIONAL,


	mcu			McuInfo OPTIONAL,  	-- mc must be set as well


	terminal		TerminalInfo OPTIONAL,


	mc			BOOLEAN,		-- shall not be set by itself


	undefinedNode		BOOLEAN,	


	...


}





GatewayInfo		::=SEQUENCE{


	protocol		SEQUENCE OF SupportedProtocols OPTIONAL,


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}








SupportedProtocols 	::= CHOICE


{


nonStandardData	NonStandardParameter,


	h310		H310Caps,


	h320		H320Caps,


	h321		H321Caps,


	h322		H322Caps,


	h323		H323Caps,


	h324		H324Caps,


	voice		VoiceCaps,


	t120-only	T120OnlyCaps,


	...


}





H310Caps 	::= SEQUENCE


{


nonStandardData	NonStandardParameter OPTIONAL,


	...


}





H320Caps 	::= SEQUENCE


{


nonStandardData	NonStandardParameter OPTIONAL,


	...


}





H321Caps 	::= SEQUENCE


{


nonStandardData	NonStandardParameter OPTIONAL,


	...


}





H322Caps ::= SEQUENCE


{


nonStandardData	NonStandardParameter OPTIONAL,


	...


}





H323Caps ::= SEQUENCE


{


nonStandardData	NonStandardParameter OPTIONAL,


	...


}





H324Caps ::= SEQUENCE


{


nonStandardData	NonStandardParameter OPTIONAL,


	...


}





VoiceCaps ::= SEQUENCE


{


nonStandardData	NonStandardParameter OPTIONAL,


	...


}





T120OnlyCaps ::= SEQUENCE


{


nonStandardData	NonStandardParameter OPTIONAL,


	...


}








McuInfo		::=SEQUENCE


{


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}





TerminalInfo		::=SEQUENCE 


{


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}





GatekeeperInfo		::=SEQUENCE 


{


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}





VendorIdentifier		::=SEQUENCE


{


	vendor			H221NonStandard,


	productId		OCTET STRING (SIZE(1..256)) OPTIONAL,	-- per vendor


	versionId		OCTET STRING (SIZE(1..256)) OPTIONAL,	-- per product


	...


}











AliasAddress		::=CHOICE


{


	e164			IA5String (SIZE (1..128)) (FROM (“0123456789#*,")),


	h323-ID		BMPString (SIZE (1..256)),	-- Basic ISO/IEC 10646-1 (Unicode)


	...


}	











QseriesOptions			::=SEQUENCE


{


	q932Full		BOOLEAN,	-- if true, indicates full support for Q.932


	q951Full		BOOLEAN, 	-- if true, indicates full support for Q.951


	q952Full		BOOLEAN, 	-- if true, indicates full support for Q.952


	q953Full		BOOLEAN, 	-- if true, indicates full support for Q.953


 	q955Full		BOOLEAN, 	-- if true, indicates full support for Q.955


	q956Full		BOOLEAN, 	-- if true, indicates full support for Q.956


	q957Full		BOOLEAN, 	-- if true, indicates full support for Q.957


	q954Info		Q954Details,	


	...


}





Q954Details			::=SEQUENCE


{


	conferenceCalling	BOOLEAN,


	threePartyService	BOOLEAN,


	...


}











ConferenceIdentifier	::=	OCTET STRING (SIZE (16))


RequestSeqNum	::=	INTEGER (1..65535)


GatekeeperIdentifier	::=	BMPString (SIZE(128))


BandWidth		::=	INTEGER (1.. 4294967295)	-- in 100s of bits


CallReferenceValue	::=	INTEGER (1..65535)


EndpointIdentifier	::=	BMPString (SIZE(128))


ProtocolIdentifier	::=	OBJECT IDENTIFIER	


					-- shall be set to  


					-- {itu-t (0) recommendation (0) h (8) 2250 version (0) 1}








NonStandardParameter	::=SEQUENCE


{


	nonStandardIdentifier	NonStandardIdentifier,


	data			OCTET STRING


}





H221NonStandard		::=SEQUENCE


{	t35CountryCode	INTEGER(0..255),	-- country, as per T.35


	t35Extension		INTEGER(0..255),	-- assigned nationally


	manufacturerCode	INTEGER(0..65535),	-- assigned nationally


	...


}





NonStandardIdentifier		::=CHOICE


{


	object			OBJECT IDENTIFIER,


	h221NonStandard	H221NonStandard,


	...


}











The root structure for RAS messages is:





RasMessage	::= CHOICE   


{


	gatekeeperRequest		GatekeeperRequest,


	gatekeeperConfirm		GatekeeperConfirm,


	gatekeeperReject		GatekeeperReject,


	registrationRequest		RegistrationRequest,


	registrationConfirm		RegistrationConfirm,


	registrationReject		RegistrationReject,


	unregistrationRequest		UnregistrationRequest,


	unregistrationConfirm		UnregistrationConfirm,


	unregistrationReject		UnregistrationReject,


	admissionRequest		AdmissionRequest,


	admissionConfirm		AdmissionConfirm,


	admissionReject		AdmissionReject,


	bandwidthRequest		BandwidthRequest,


	bandwidthConfirm		BandwidthConfirm,


	bandwidthReject		BandwidthReject,


	disengageRequest		DisengageRequest,


	disengageConfirm		DisengageConfirm,


	disengageReject		DisengageReject,


	locationRequest		LocationRequest,


	locationConfirm			LocationConfirm,


	locationReject			LocationReject, 


	infoRequest			InfoRequest,


	infoRequestResponse		InfoRequestResponse,


	nonStandardMessage		NonStandardMessage,		


	unknownMessageResponse	UnknownMessageResponse,


	...


}














GatekeeperRequest		::=SEQUENCE --(GRQ)


{


	requestSeqNum	RequestSeqNum,			


	protocolIdentifier	ProtocolIdentifier,			


	nonStandardData	NonStandardParameter OPTIONAL,	


rasAddress		TransportAddress,			


endpointType		EndpointType,				


	gatekeeperIdentifier	GatekeeperIdentifier OPTIONAL,


	callServices		QseriesOptions  OPTIONAL, 


	endpointAlias		SEQUENCE OF AliasAddress OPTIONAL,


	...


}





GatekeeperConfirm	::=SEQUENCE --(GCF)


{


	


	requestSeqNum	RequestSeqNum,


	protocolIdentifier	ProtocolIdentifier,			


	nonStandardData	NonStandardParameter OPTIONAL,


	gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,


	rasAddress		TransportAddress,


	...


}





GatekeeperReject		::=SEQUENCE --(GRJ)


{


	


	requestSeqNum	RequestSeqNum,


	protocolIdentifier	ProtocolIdentifier,			


	nonStandardData	NonStandardParameter OPTIONAL,


	gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,


	rejectReason		GatekeeperRejectReason,


	...


}





GatekeeperRejectReason		::=CHOICE


{


	resourceUnavailable		NULL,


	terminalExcluded		NULL,	-- permission failure, not a resource failure


	invalidRevision			NULL,


	undefinedReason		NULL,


	...


}








RegistrationRequest		::=SEQUENCE --(RRQ)


{


	requestSeqNum	RequestSeqNum,				


	protocolIdentifier	ProtocolIdentifier,						nonStandardData	NonStandardParameter OPTIONAL,	


	discoveryComplete	BOOLEAN,					


callSignalAddress	SEQUENCE OF TransportAddress,		


	rasAddress		SEQUENCE OF TransportAddress,		


terminalType		EndpointType,					


	terminalAlias		SEQUENCE OF AliasAddress OPTIONAL,		


	gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,


	endpointVendor		VendorIdentifier,


	...


}








RegistrationConfirm	::=SEQUENCE --(RCF)


{


	requestSeqNum	RequestSeqNum,				


	protocolIdentifier	ProtocolIdentifier,				


	nonStandardData	NonStandardParameter OPTIONAL,


	callSignalAddress	SEQUENCE OF TransportAddress,		


	terminalAlias		SEQUENCE OF AliasAddress OPTIONAL,


	gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,


	endpointIdentifier	EndpointIdentifier,				


	...	


}








RegistrationReject		::=SEQUENCE --(RRJ)


{


	requestSeqNum	RequestSeqNum,				


	protocolIdentifier	ProtocolIdentifier,				


	nonStandardData	NonStandardParameter OPTIONAL,


	rejectReason		RegistrationRejectReason,					


	gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,


	...


}





RegistrationRejectReason		::=CHOICE


{


	discoveryRequired		NULL,	-- registration permission has aged


	invalidRevision			NULL,


	invalidCallSignalAddress	NULL,


	invalidRASAddress		NULL,	-- supplied address is invalid


	duplicateAlias			SEQUENCE OF AliasAddress,	-- alias registered to another endpoint


	invalidTerminalType		NULL,


	undefinedReason		NULL,


	transportNotSupported		NULL,	-- one or more of the transports


	...


}











UnregistrationRequest		::=SEQUENCE --(URQ)


{


	requestSeqNum	RequestSeqNum,			


	callSignalAddress	SEQUENCE OF TransportAddress,	


	endpointAlias		SEQUENCE OF AliasAddress OPTIONAL,		


	nonStandardData	NonStandardParameter OPTIONAL,


	endpointIdentifier	EndpointIdentifier OPTIONAL,


	...


}


UnregistrationConfirm	::=SEQUENCE --(UCF)


{


	requestSeqNum	RequestSeqNum,				


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}





UnregistrationReject		::=SEQUENCE --(URJ)


{


	


	requestSeqNum	RequestSeqNum,				


	rejectReason		UnregRejectReason,				


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}





UnregRejectReason		::=CHOICE


{


	notCurrentlyRegistered	NULL,	


	callInProgress		NULL,	


	undefinedReason	NULL,


		...


}








AdmissionRequest		::=SEQUENCE --(ARQ)


{


	requestSeqNum	RequestSeqNum,		


	callType	CallType,		


	callModel	CallModel OPTIONAL,		


	endpointIdentifier	EndpointIdentifier,		


	destinationInfo	SEQUENCE OF AliasAddress OPTIONAL, --Note 1


	destCallSignalAddress	TransportAddress OPTIONAL,	-- Note 1 


	destExtraCallInfo	SEQUENCE OF AliasAddress OPTIONAL,


	srcInfo	SEQUENCE OF AliasAddress,	


	srcCallSignalAddress	TransportAddress OPTIONAL, 


	bandWidth	BandWidth,   				callReferenceValue	CallReferenceValue,			


	nonStandardData	NonStandardParameter OPTIONAL,


	callServices	QseriesOptions  OPTIONAL,


	conferenceID	ConferenceIdentifier,


	activeMC	BOOLEAN,


	answerCall	BOOLEAN,	-- answering a call		


	...


}








CallType			::=CHOICE


{	


	pointToPoint		NULL,		-- Point to point


	oneToN			NULL,		-- no interaction (FFS)


	nToOne		NULL,		-- no interaction (FFS)


	nToN			NULL,		-- interactive (multipoint)


	...


}


	





CallModel	::=CHOICE


{


	direct			NULL,


	gatekeeperRouted	NULL,


	...


}








AdmissionConfirm	::=SEQUENCE --(ACF)


{


	requestSeqNum	RequestSeqNum,			


	bandWidth		BandWidth,				


	callModel		CallModel,				


	destCallSignalAddress	TransportAddress,			


	irrFrequency		INTEGER (1..65535) OPTIONAL,			


	nonStandardData	NonStandardParameter OPTIONAL,


	...	


}





AdmissionReject		::=SEQUENCE --(ARJ)


{


	requestSeqNum	RequestSeqNum,		


	rejectReason		AdmissionRejectReason,	


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}








AdmissionRejectReason		::= CHOICE


{


	calledPartyNotRegistered	NULL,	-- can’t translate address	


	invalidPermission		NULL,	-- permission has expired


	requestDenied			NULL,	-- no bandwidth available


	undefinedReason		NULL,


	callerNotRegistered		NULL,


	routeCallToGatekeeper		NULL,


	invalidEndpointIdentifier	NULL,


	resourceUnavailable		NULL,


		...


}





.





BandwidthRequest		::=SEQUENCE --(BRQ)


{


	requestSeqNum	RequestSeqNum,			


	endpointIdentifier	EndpointIdentifier,			


	conferenceID		ConferenceIdentifier,			


	callReferenceValue	CallReferenceValue,			


	callType		CallType OPTIONAL,


 	bandWidth		BandWidth,   				


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}














BandwidthConfirm	::=SEQUENCE --(BCF)


{


	requestSeqNum	RequestSeqNum,			


	bandWidth		BandWidth,   					


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}





BandwidthReject		::=SEQUENCE --(BRJ)


{


	requestSeqNum	RequestSeqNum,		


	rejectReason		BandRejectReason,		


	allowedBandWidth	BandWidth,   			


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}








BandRejectReason		::= CHOICE


{


	notBound			NULL,	-- discovery permission has aged


	invalidConferenceID		NULL,	-- possible revision


	invalidPermission		NULL,	-- true permission violation


	insufficientResources		NULL,	


		invalidRevision			NULL,


	undefinedReason		NULL,


		...


}





LocationRequest		::=SEQUENCE --(LRQ)


{


	requestSeqNum		RequestSeqNum,		


	endpointIdentifier		EndpointIdentifier OPTIONAL,		


	destinationInfo			SEQUENCE OF AliasAddress,  		nonStandardData		NonStandardParameter OPTIONAL,


	replyAddress			TransportAddress,


	...


}











LocationConfirm	::=SEQUENCE --(LCF)


{


	requestSeqNum	RequestSeqNum,			


	callSignalAddress	TransportAddress,			


	rasAddress		TransportAddress,			 	nonStandardData	NonStandardParameter OPTIONAL,


	...	


}





LocationReject		::=SEQUENCE --(LRJ)


{


	requestSeqNum	RequestSeqNum,		


	rejectReason		LocationRejectReason,	


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}








LocationRejectReason		::=CHOICE


{


	notRegistered			NULL,


	invalidPermission		NULL,	-- exclusion by administrator or feature


	requestDenied			NULL,	-- can't find location


	undefinedReason		NULL,


		...


}








DisengageRequest		::=SEQUENCE --(DRQ)


{


	requestSeqNum	RequestSeqNum,			


	endpointIdentifier	EndpointIdentifier,			


	conferenceID		ConferenceIdentifier,		


	callReferenceValue	CallReferenceValue,			


	disengageReason	DisengageReason,				


	nonStandardData	NonStandardParameter OPTIONAL,


	...	


}





DisengageReason		::=CHOICE


{


	forcedDrop			NULL,	-- gatekeeper is forcing the drop


	normalDrop			NULL,	-- associated with normal drop


	undefinedReason		NULL,


		...


}





DisengageConfirm		::=SEQUENCE --(DCF)


{


	requestSeqNum	RequestSeqNum,		


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}








DisengageReject	::= SEQUENCE --(DRJ)


{


	requestSeqNum	RequestSeqNum,		


	rejectReason		DisengageRejectReason,		


	nonStandardData	NonStandardParameter OPTIONAL,


	...


}


DisengageRejectReason		::=CHOICE


{


	notRegistered			NULL,	-- not registered with gatekeeper


	requestToDropOther		NULL, 	-- can’t request drop for others


		...


}








InfoRequest	::=SEQUENCE --(IRQ)


{


	requestSeqNum	RequestSeqNum,				


	callReferenceValue	CallReferenceValue,			


	nonStandardData	NonStandardParameter  OPTIONAL,


	replyAddress	TransportAddress OPTIONAL,


	...


}








InfoRequestResponse	::=SEQUENCE --(IRR)


{


	nonStandardData	NonStandardParameter OPTIONAL,


	requestSeqNum	RequestSeqNum,			


	endpointType		EndpointType,


	endpointIdentifier	EndpointIdentifier,�	rasAddress		TransportAddress,			


	callSignalAddress	SEQUENCE OF TransportAddress,


	endpointAlias		SEQUENCE OF AliasAddress OPTIONAL,


	perCallInfo		SEQUENCE OF SEQUENCE�	{				


		nonStandardData	NonStandardParameter OPTIONAL,


		callReferenceValue	CallReferenceValue,			


		conferenceID		ConferenceIdentifier,			


		originator		BOOLEAN OPTIONAL,				


		audio			SEQUENCE OF RTPSession OPTIONAL,


		video			SEQUENCE OF RTPSession OPTIONAL,


		data			SEQUENCE OF TransportChannelInfo OPTIONAL,


		h245			TransportChannelInfo,


		callSignaling		TransportChannelInfo,


		callType		CallType,				


		bandWidth		BandWidth,				


		callModel		CallModel,


		...


	} OPTIONAL,


	...


}





TransportChannelInfo ::= SEQUENCE


{


	sendAddress		TransportAddress OPTIONAL,


	recvAddress		TransportAddress OPTIONAL,


	...


}





RTPSession ::= SEQUENCE


{


	rtpAddress		TransportChannelInfo,


	rtcpAddress		TransportChannelInfo,


	cname			PrintableString,


	ssrc			INTEGER (1..4294967295),


	sessionId		INTEGER (1..255),�	associatedSessionIds	SEQUENCE OF INTEGER (1..255),


	...


}











NonStandardMessage		::=SEQUENCE


{


	requestSeqNum	RequestSeqNum,


	nonStandardData	NonStandardParameter,


	...


}














UnknownMessageResponse		::=	SEQUENCE		-- XRS


{


	requestSeqNum	RequestSeqNum,


...


}





END		-- of ASN.1

















	Decided H.225.0 	Page � PAGE �16�  � DATE  \l �6/19/96� � TIME �9:48 AM�





Contact:  Dale Skran	+1 908 957 5988


              2G432 AT&T	+1 908 957 5627


              200 Laurel Ave.


              Middletown, NJ 07748











