1
71

ITU - Telecommunication Standardization Sector
Temporary Document 28
STUDY GROUP 16
Dublin, 29 Oct – 2 Nov 2001

Question(s):
3

SOURCE*:
Editor

TITLE:
H.248 Implementors Guide draft

__________________

Abstract:

This document is a DRAFT version of Implementors Guide and is subject to change.  It includes changes suggested by consensus on the megaco mailing list and those given preliminary approval at rapporteur’s meetings.  These changes are not final until approved at a Working Party or Study Group meeting.  Implementors using these corrections must understand that they may be forced to change their implementations if the approved version differs from this draft.

Change Log:

V8 (Dublin, Oct 2001):


New:


6.88 
PkgdName Comments

6.89 
TimeStamp Optional in Notify ObservedEventsDescriptor

6.90 
NotifyCompletion Reasons for Signals that Play-to-completion

6.91 
Allowed Char Set for Identifier

6.92 
Clarified use of ServiceChangeMethod for various cases of MG losing connection with MGC

6.93 
“Gateway” should read “MGC”

6.94 
References for Continuity Test Tone

6.95 
ServiceChangeMethod and ServiceChangeReason are required

6.96 
E.1.2 typo

6.97 
V4hex range typo in ABNF Specification (IPV4address)

6.98 
Only One Media Description Allowed in SDP

6.99 
Specify error code when Topology not supported

6.100 
State in Annex A and B that syntax does not enforce all restrictions

6.101 
When does Subtract Return Statistics

6.102 
Clarify the definition of Signal

6.103 Specification of Root Termination and associated Error

6.104 Update to IG 6.46 Encoding of ASN.1 Octet String

6.105 Annex C encoding

6.106 Encoding of VPCI/VCI in Annex C.4

6.107
Missing extension marker in ASN.1 for TopologyRequest 

6.108
ASN.1 Profile Name Amendment

6.109 
Clarify property types

8.0
Added new section 8 for Technical and Editorial Corrections for H.248 Annex G, renumbered all items in higher sections

8.1  
Correct Binary PropertyIDs in Soft Key Package

8.2 Correct Binary DigitMap Completion EventID in Keypad Package

10.2
Announcement Playing Clarification

Modified:

6.8 
corrected Description to refer to Annex E.13.1 rather than Appendix E which does not exist.

6.16 
corrected title to indicate Annex A & B rather than Appendix A and showed correction to apply to A.3 as well as B.2

6.27
was edited to remove a spurious extra paragraph and missing deletion, to agree with (item 6.30) from version approved in Geneva Nov 2000.

6.46 
was corrected to restore original change. “8.7 X.690” had spuriously become “A.7 X.690”

6.66 
removed spurious change bar on mId in ABNF.

6.69 
was edited to restore a deleted space after 0xffff.

6.75 
differences in ABNF were not against version with 6.50 changes.  Edited to fix.

6.86 corrected the comment delimiter in the change from ‘;’ to ‘--‘

7.7
modified to include other minor error of spurious dot in value name

8.1 
and higher were renumbered to add 1 to section number

12.3 
(formerly 11.3) was edited replacing the “Audit” command (which does not exist) with “AuditValue”.

12.4
was edited to fix the first sentence by adding “or the base package name”

	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	H.248 Series Implementors' Guide

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(02/xx/2002)   

	
	SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS

Infrastructure of audiovisual services – Communication procedures



	
	Media Gateway Control Protocol

Implementors' Guide




Contact Information
	ITU-T Study Group 16 / Question 3 Rapporteur
	Glen Freundlich

Avaya Communications

1300 W. 120th Avenue

Westminster, Colorado 80234

USA
	Tel:
+1 303 538 2899

Fax:
+1 303 538 3907

E-mail:
ggf@avaya.com

	Implementors' Guide ITU-T Recommendation H.248 Editor
	Terry Anderson

Lucent Technologies, Rm 2B-121

600 Mountain Ave

Murray Hill, NJ 07974-2006

USA
	Tel:        +1 908 582 7013

Fax:
+1 908 582 6729

E-mail:
 tla@lucent.com



51
Introduction

2
Scope
5
3
Defect Resolution Procedure
5
4
References
5
5
Nomenclature
5
6
Technical and Editorial Corrections to ITU-T Recommendation H.248 (2000)
6
7
Technical and Editorial Corrections to H.248 Annex F(2000)
71
8
Technical and Editorial Corrections to H.248 Annex G
76
9
Technical and Editorial Corrections to H.248 Annex H
78
10
Technical and Editorial Corrections to H.248 Annex K
78
11
Technical and Editorial Corrections to RFC-3015
81
12
Implementation Clarifications for H.248
82
13
Implementation Clarifications for H.248 annex H
84
14
H.248 Recommendation Series Defect Report Form
85



1
Introduction

This document is a compilation of reported defects identified as at 09/14/2001 with the following recommendations:

· 2000 decided edition of ITU-T Recommendation H.248,

· 2000 decided edition of ITU-T Recommendation H.248 Annex F

· 2000 decided edition of ITU-T Recommendation H.248 Annex G

· 2000 decided edition of ITU-T Recommendation H.248 Annex H

· 2000 decided edition of ITU-T Recommendation H.248 Annex K

· RFC3015

It must be read in conjunction with the Recommendation to serve as an additional authoritative source of information for implementors'. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected H.248 Recommendations.

2
Scope

This guide resolves defects in the following categories:

· editorial errors

· technical errors, such as omissions and inconsistencies

· ambiguities

In addition, the Implementors' Guide may include explanatory text found necessary as a result of interpretation difficulties apparent from the defect reports.

This Guide will not address proposed additions, deletions, or modifications to the Recommendations that are not strictly related to implementation difficulties in the above categories.  Proposals for new features should be made in through contributions to the ITU-T.

3
Defect Resolution Procedure

Upon discovering technical defects with any components of the H.248 Recommendation, please provide a written description directly to the editors of the affected Recommendations with a copy to the Q.3/16 Rapporteur.  The template for a defect report is located at the end of the Guide.  Contact information for these parties is included at the front of the document.  Return contact information should also be supplied so a dialogue can be established to resolve the matter and an appropriate reply to the defect report can be conveyed.  This defect resolution process is open to anyone interested in H.248 Recommendation.  Formal membership in the ITU is not required to participate in this process.

4
References

ITU-T Recommendation H.248 (2000), Media Gateway Control Protocol

ITU-T Recommendation H.248 (2000) Annex F, Facsimile, Text Conversation and Call Discrimination packages
ITU-T Recommendation H.248 (2000) Annex H, Transport over Stream Control Transmission Protocol (SCTP)

ITU-T Recommendation H.248 (2000) Annex K, Generic Announcement Package

5
Nomenclature

In addition to traditional revision marks, the following marks and symbols are used to indicate to the reader how changes to the text of a Recommendation should be applied:

	Symbol
	Description

	[Begin Correction]
	Identifies the start of revision marked text based on extractions from the published Recommendations affected by the correction being described.

	[End Correction]
	Identifies the end of revision marked text based on extractions from the published Recommendations affected by the correction being described.

	...
	Indicates that the portion of the Recommendation between the text appearing before and after this symbol has remained unaffected by the correction being described and has been omitted for brevity.

	--- SPECIAL INSTRUCTIONS --- {instructions}
	Indicates a set of special editing instructions to be followed.


6
Technical and Editorial Corrections to ITU-T Recommendation H.248 (2000)

6.1 Correction in bibliographic reference

	Description:
	Section 2.1/H.248 contains a bibliographic reference to Q.765.  The Q.765 series consists of a number of recommendations.  The correct reference is Q.765.5, Application transport mechanism – Bearer independent call control (BICC), instead of the entire series.


 [Begin Addition]

2.1 Normative references

…


ITU-T Recommendation Q.765.5, "Application transport mechanism – Bearer independent call control (BICC)".

…

[End Addition]

The reference to Q.765 in section C.1/H.248 should be corrected too:

[Begin Correction]

	ACodec
	1006
	Octet String
	Audio Codec Type:

Reference: ITU-T Rec. Q.765.5 -

Non-ITU codecs are defined with the appropriate standards organisation under a defined Organizational identifier


[End Correction]

6.2 Valid parameters to Add, Modify and Move commands

	Description:
	The EventBufferDescriptor parameter was inadvertently omitted as a valid parameter to Add, Modify and Move commands in sections 7.2.1, 7.2.2, 7.2.4/H.248.


 [Begin Addition]

7.2.1 Add

The Add command adds a Termination to a Context.

TerminationID

[, MediaDescriptor]

[, ModemDescriptor]

[, MuxDescriptor]

[, EventsDescriptor]

[, SignalsDescriptor]

[, DigitMapDescriptor}

[, ObservedEventsDescriptor]

[, EventBufferDescriptor]

[, StatisticsDescriptor]

[, PackagesDescriptor]

Add(
TerminationID

[, MediaDescriptor]

[, ModemDescriptor]

[, MuxDescriptor]

[, EventsDescriptor]

[, EventBufferDescriptor]

[, SignalsDescriptor]

[, DigitMapDescriptor]

[, AuditDescriptor]

)

[End Addition]

[Begin Addition]

…

The EventsDescriptor parameter is optional.  If present, it provides the list of events that should be detected on the Termination.

The EventBufferDescriptor parameter is optional.  If present, it provides the list of events that the MG is requested to detect and buffer when EventBufferControl equals LockStep.

…

 [End Addition]

[Begin Addition]

7.2.2 Modify

The Modify command modifies the properties of a Termination.

TerminationID

[, MediaDescriptor]

[, ModemDescriptor]

[, MuxDescriptor]

[, EventsDescriptor]

[, SignalsDescriptor]

[, DigitMapDescriptor}

[, ObservedEventsDescriptor]

[, EventBufferDescriptor]

[, StatisticsDescriptor]

[, PackagesDescriptor]

Modify(
TerminationID

[, MediaDescriptor]

[, ModemDescriptor]

[, MuxDescriptor]

[, EventsDescriptor]

[, EventBufferDescriptor]

[, SignalsDescriptor]

[, DigitMapDescriptor]

[, AuditDescriptor]

)

[End Addition]

[Begin Addition]

7.2.4 Move

…

TerminationID

[, MediaDescriptor]

[, ModemDescriptor]

[, MuxDescriptor]

[, EventsDescriptor]

[, SignalsDescriptor]

[, DigitMapDescriptor}

[, ObservedEventsDescriptor]

[, EventBufferDescriptor]

[, StatisticsDescriptor]

[, PackagesDescriptor]

Move(
TerminationID

[, MediaDescriptor]

[, ModemDescriptor]

[, MuxDescriptor]

[, EventsDescriptor]

[, EventBufferDescriptor]

[, SignalsDescriptor]

[, DigitMapDescriptor]

[, AuditDescriptor]

)

[End Addition]

6.3 Cold Start

	Description:
	Section 11.2/H.248 contains a leftover from old terminology: Transaction Accept instead of Transaction Reply.


 [Begin Correction]

11.2 Cold Start

A MG is pre-provisioned by a management mechanism outside the scope of this protocol with a Primary and (optionally) an ordered list of Secondary MGCs.  Upon a cold start of the MG, it will issue a ServiceChange command with a "Restart" method, on the Root Termination to its primary MGC.  If the MGC accepts the MG, it will send a Transaction  Reply, with the ServiceChangeMgcId set to itself.

…

[End Correction]

6.4 Digit map syntax

	Description:
	Annex A.3/H.248 provides a copy of the syntax of digit maps, stating that the definition given in Annex B/H.248 takes precedence in case of discrepancies.  A discrepancy occurs in the production rule for digitStringList, and it is proposed to correct this discrepancy: the quoted forward slash should be replaced by a quoted vertical bar.


 [Begin Correction]

A.3 Digit maps and path names

…
digitStringList = digitString *(LWSP "|" LWSP digitString)

[End Correction]

6.5 Omission in specification of text encoding

	Description:
	The specification of the text encoding of H.248 messages currently allows multiple occurrences of the same servicechange parameter, while the intention is that every such parameter should occur only once.  The proposed resolution is to add a comment to the ABNF indicating this restriction.


 [Begin Correction]


B.2 ABNF specification

…

; each parameter at-most-once

serviceChangeParm    = (serviceChangeMethod / serviceChangeReason /

serviceChangeDelay / serviceChangeAddress /  

serviceChangeProfile / extension / TimeStamp /

serviceChangeMgcId / serviceChangeVersion )

[End Correction]

6.6 Ambiguity in text encoding

	Description:
	The text encoding as specified in Annex B/H.248 contains an ambiguity because the token "EB" was inadvertently used twice, in the production rules for EmbedToken and EventBufferToken.  It is proposed to change the short tokens in the production rules for EmbedToken and EmergencyToken, and leave the production rule for EventBufferToken unchanged.


 [Begin Correction]


B.2 ABNF specification

…
EmbedToken

= ("Embed"

/ "EM")

EmergencyToken
= ("Emergency"
/ "EG")  

[End Correction]

6.7 Use of ServiceChange for MG registration

	Description:
	There is an inconsistency between section 7.2.8 and section 11.2 on when the ServiceChangeMgcID is used.


 [Begin Correction]

11.2 Cold Start

…
A MG is pre-provisioned by a management mechanism outside the scope of this protocol with a Primary and (optionally) an ordered list of Secondary MGCs.  Upon a cold start of the MG, it will issue a ServiceChange command with a "Restart" method, on the Root Termination to its primary MGC.  If the MGC accepts the MG's registration, it sends a Transaction Reply not including a ServiceChangeMgcId parameter.  If the MGC does not accept the MG's registration, it sends a Transaction Reply, providing the address of an alternate MGC to be contacted by including a ServiceChangeMgcId parameter.
If the MG receives a Transaction Reply that includes a ServiceChangeMgcId parameter, it sends a ServiceChange to the MGC specified in the ServiceChangeMgcId. It continues this process until it gets a controlling MGC to accept its registration, or it fails to get a reply.  Upon failure to obtain a reply, either from the Primary MGC, or a designated successor, the MG tries its pre-provisioned Secondary MGCs, in order.  If the MG is unable to establish a control relationship with any MGC, it shall wait a random amount of time as described in section 9.2 and then start contacting its primary, and if necessary, its secondary MGCs again.

It is possible that the reply to a ServiceChange with Restart will be lost, and a command will be received by the MG prior to the receipt of the ServiceChange response.  The MG shall issue error 505 – Command Received before Restart Response.

[End Correction]

6.8 Echo cancellation parameters

	Description:
	Annex E.13.1/H.248 contains a property to turn echo cancellation off or on.  In addition, C.1 and C.9 contain codepoints dealing with echo cancellation.  The codepoints are 

· Echocanc (100B), with allowed values Off, G.165 and G.168;

· ECHOCI (9021), with allowed values Off, incoming echo canceler on, outgoing echo canceler on, and incoming and outgoing echo canceler on.

The codepoints in Annex C/H.248 are for use with binary encoding only, while packages define properties for use with both text and binary encodings.  In addition it is expected that SG 11 will complete work on their SPNE package, allowing more advanced control of echo cancellers than the basic control offered by the TDM circuit package of Annex E.13.1.  Therefore it is proposed that the codepoints of Annex C dealing with echo cancellation be deprecated, and that the entries in the tables in C.1 and C.9 be listed as Reserved.


[Begin Correction]

C.1 General Media Attributes

…

	Echocanc
	100B
	
	
Not used. See H.248 E.13 for an example of possible Echo Control properties.


 [End Correction]

 [Begin Correction]

C.9 Bearer Capabilities

…

	ECHOCI
	9021
	
	

Not used. See H.248 E.13 for an example of possible Echo Control properties.


 [End Correction]

6.9 Topology Triples in ABNF

	Description:
	In the ABNF (Annex B), the term TopologyDescriptor allows the specification of only one triple.  The ASN.1 permits a sequence of such triples.


 [Begin Correction]

B.2 ABNF Specification

…
topologyDescriptor   = TopologyToken LBRKT topologyTriple 

*(COMMA topologyTriple) RBRKT
topologyTriple = terminationA COMMA terminationB COMMA topologyDirection 
[End Correction]

6.10 Local Control for Annex C and optionality

	Description:
	Currently the introduction of Annex C specifies that native tags are applicable to Local and Remote descriptors. This introduction should also say that native tags are applicable to the Local Control descriptor as the ASN1 encoding makes use of native tags in the Local Control descriptor.

	Reference:
	Subject: Re: MEGACO: More Annex C Questions

Date: Wed, 06 Dec 2000 11:36:38 -0500

From: Troy Cauble <troy@lucent.com>

To: Christian Groves <Christian.Groves@ericsson.com>


 [Begin Correction]

ANNEX C Tags for media stream properties (normative)

Parameters for Local , Remote and Local Control descriptors are specified as tag-value pairs if binary encoding is used for the protocol.  This annex contains the property names (PropertyID), the tags (Property Tag), type of the property (Type) and the values (Value).Values presented in the Value field when the field contains references shall be regarded as "information". The reference contains the normative values.  If a value field does not contain a reference then the values in that field can be considered as "normative".

Tags are given as hexadecimal numbers in this annex. When setting the value of a property, a MGC may underspecify the value according to one of the mechanisms specified in section 7.1.1.

It is optional to support the properties in this Annex or any of its sub-sections. For example 3 properties from C.3 and five properties from C.8 may be implemented only.
[End Correction]

6.11 Echo Canceller Default

	Description:
	As the Echo Cancellation properties in Annex C have been deprecated in 6.8 of this implementors' guide the default of the Echo Canceller property should be provisioned to allow for a wider change of applications.


 [Begin Correction]

E.13.1 Properties

Echo Cancellation

PropertyID: ec (0x0008)


Type: boolean

Possible Values:

"on" (when the echo cancellation is requested) and

"off" (when it is turned off.)

The default is provisioned.


Defined In: LocalControlDescriptor


Characteristics: read/write

[End Correction]

6.12 Error in text on interim AH header

	Description:
	The UDP destination port should be encoded as 20 hex digits, representing 10 bytes (4 source, 4 dest, 2 port).

The Interim AH scheme should apply to all transactions in a message. The current text in H.248 section 10.2 indicates that the ICV calculation should be performed on one transaction. This is incorrect.

	Reference:
	Subject: RE: Interim AH Scheme

Date: Wed, 28 Mar 2001 09:53:25 -0500

From: "Tom-PT Taylor" <taylor@nortelnetworks.com>

To: "'Christian Groves'" <Christian.Groves@ericsson.com>, girirs@netlab.hcltech.com

CC: megaco@fore.com


 [Begin Correction]

10.2 Interim AH Scheme

…

As an interim solution, an optional AH header is defined within the H.248 protocol header. The header fields are exactly those of the SPI, SEQUENCE NUMBER and DATA fields as defined in [RFC2402]. The semantics of the header fields are the same as the "transport mode" of [RFC2402], except for the calculation of the Integrity Check value (ICV). In IPsec, the ICV is calculated over the entire IP packet including the IP header. This prevents spoofing of the IP addresses.  To retain the same functionality, the ICV calculation should be performed across all the transactions (concatenated) in the message prepended by a synthesized IP header consisting of a 32 bit source IP address, a 32 bit destination address and a 16 bit UDP destination port encoded as 20 hex digits. When the interim AH mechanism is employed when TCP is the transport Layer, the UDP Port above becomes the TCP port, and all other operations are the same. 

[End Correction]

6.13 Termination Subtract from NULL context

	Description:
	A subtraction of a termination from a NULL context is not allowed however this is not clear in the recommendation. This should be stated.


 [Begin Correction]

7.2.3 Subtract 

…

ALL may be used as the ContextID as well as the TerminationId in a Subtract, which would have the effect of deleting all contexts, deleting all ephemeral terminations, and returning all physical terminations to Null context. Subtract of termination from the NULL context is not allowed.
[End Correction]

6.14 Missing M= Line in Annex SDP

	Description:
	Section C.11 SDP Equivalents lists various SDP encoding lines. However the Media Line (m=) is missing from this table. The Media line should occur in this table.


 [Begin Addition]

C.11 SDP Equivalents

…
	SDP_M
	B00F
	STRING
	Media name and transport address

Reference: IETF RFC2327


 [End Correction]

6.15 Missing Optional on Keepactive Flag

	Description:
	In section 7.1.9 EventsDescriptor is states " Each event in the descriptor contains the Event name, an optional streamID, an optional KeepActive flag, and optional parameters." Clearly the KeepActive flag is meant to be optional however in the ASN.1 this flag is mandatory. The ASN.1 should be updated indicating OPTIONAL. 


 [Begin Correction]

A.2 ASN.1 syntax specification

…

RequestedActions ::= SEQUENCE

{


keepActive


BOOLEAN OPTIONAL,


eventDM


EventDM OPTIONAL,


secondEvent         
SecondEventsDescriptor OPTIONAL,


signalsDescriptor   
SignalsDescriptor OPTIONAL,


...

}

…

SecondRequestedActions ::= SEQUENCE

{


keepActive


BOOLEAN OPTIONAL,


eventDM


EventDM OPTIONAL,


signalsDescriptor 
SignalsDescriptor OPTIONAL,


...

} [End Correction]

6.16 Syntax Problem in Annex A & B

	Description:
	According to the definition of digitMapRange :

digitMapRange = ("x" / LWSP "[" LWSP digitLetter LWSP "]" LWSP)

 "x" must be followed by "[" .


 [Begin Correction]

B.2 ABNF syntax specification

...

digitMapRange        = ("x" / (LWSP "[" LWSP digitLetter LWSP "]" LWSP))
[End Correction]

[Begin Correction]

A.3 Digit maps and path names

...

digitMapRange        = ("x" / (LWSP "[" LWSP digitLetter LWSP "]" LWSP))
[End Correction]

6.17 Retaining Descriptors on MOVE

	Description:
	When a MOVE command is issued on a termination the descriptors currently residing on that termination are retained. This is current ambiguous in the recommendation text.


 [Begin Correction]

7.2.4 Move

The Move command does not affect the properties of the Termination on which it operates, except those properties explicitly modified by descriptors included in the Move command. The AuditDescriptor with the Statistics option, for example, would return statistics on the Termination just prior to the Move.  Possible descriptors returned from Move are the same as for Add.

 [End Correction]

6.18 Clarification on extending packages

	Description:
	An extended package shall not redefine or overload an identifier defined in the original package. For example: if package "aa" has a signal "ab", then if package "bb" extends aa it cannot define signal "ab".  This is also valid for not redefining an id in "earlier" packages, when multiple levels of extension occur.

Several packages in H.248 Annex E have made this error. Corrections are below.


 [Begin Correction]

12.1.1 Package

A package may extend an existing package. The version of the original package must be specified. When a package extends another package it shall only add additional Properties, Events, Signals, Statistics and new possible values for an existing parameter described in the original package. An extended package shall not redefine or overload an identifier defined in the original package andpackages it may have extended (multiple levels of extension).
Hence, if package B version 1 extends package A version 1, version 2 of B will not be able to extend the A version 2 if A version 2 defines a name already in B version 1.

[End Correction]

[Begin Correction]

Section E.6.2 Events

…

DigitMap Completion Event

EventID: ce, 0x0004
Generated when a digit map completes as described in section 7.1.14.

[End Correction]

[Begin Correction]

Section E.7.3

	Signal Name
	Signal ID/tone id

	Dial Tone
	dt (0x0030)

	Ringing Tone
	rt (0x0031)

	Busy Tone
	bt (0x0032)

	Congestion Tone
	ct (0x0033)

	Special Information Tone
	sit(0x0034)

	Warning Tone
	wt (0x0035)

	Payphone Recognition Tone
	prt (0x0036)

	Call Waiting Tone
	cw (0x0037)

	Caller Waiting Tone
	cr (0x0038)


[End Correction]

6.19 Context = ALL in Transaction Reply

	Description:
	The ASN1 and ABNF allows for the return of Context ID = ALL. This is used in the Wildcard response case. However the text in 8.2.2 Transaction Reply states that only a Specific or NULL is valid for the Context ID. This should be updated to allow ALL.


 [Begin Correction]

8.2.2 TransactionReply

…

The ContextID parameter must specify a value to pertain to all Responses for the action.  The ContextID may be specific, all or null. 

…

[End Correction]

6.20 Number of Events on a Termination

	Description:
	The text in H.248 Sect 9.1 says that "On a given Termination, there should normally be at most one outstanding Notify command at any time." This is only a consideration when using UDP. A termination typically can realise multiple events on a terminations.


 [Begin Correction]

9.1 Ordering of Commands

…

3.
For transport that do not guarantee in sequence delivery of messages (ie. UDP), on a given Termination, there should normally be at most one outstanding Notify command at any time.

…

[End Correction]

6.21 Error Code for Number of Terminations in a Context exceeded

	Description:
	H.248 provides a means of setting the maximum number of terminations in a context. However no mechanism is provided to allow an error when the maximum number of terminations is requested to be exceeded.


 [Begin Correction]

Command error code "434 - Max number of Terminations in a Context exceeded." has been added to H.248 Annex L – Error Codes.
[End Correction]

6.22 Optional Statistics Parameter Value

	Description:
	In section 7.2.6 for Audit capability its stated that "StatisticsDescriptor returns the names of the statistics being kept on the termination."

But the ABNF grammar states the descriptor as "statisticsDescriptor = StatsToken LBRKT statisticsParameter *(COMMA statisticsParameter ) RBRKT

;at-most-once per item

statisticsParameter  = pkgdName EQUAL VALUE"

which doesn't allow specification of stats parameter always with value. If only names needs to be sent from MG, the value field needs to be made optional.


 [Begin Correction]

B.2 ABNF Syntax Specification

…

statisticsParameter = pkgdName [EQUAL VALUE]
…

 [End Correction]

[Begin Correction]

A.2 – ASN.1 Syntax Specification

…

StatisticsParameter ::= SEQUENCE

{


statName 

PkgdName,


statValue 

Value OPTIONAL
}

…

 [End Correction]

6.23 Event Permanancy 

	Description:
	The current Event Descriptor text is unclear on whether or not a event continues to trigger notifications after the first event is detected. The intention is that the event shall do this.


 [Begin Correction]

7.1.9 Event Descriptor

…

When an event is processed against the contents of an active Events descriptor and found to be present in that descriptor ("recognized"), the default action of the MG is to send a Notify command to the MG. Notification may be deferred if the event is absorbed into the current dial string of an active digit map (see section 7.1.14).  Any other action is for further study.  Moreover, event recognition may cause currently active signals to stop, or may cause the current Events and/or Signals descriptor to be replaced, as described at the end of this section. Unless the events descriptor is replaced by another events descriptor, it remains active after an event has been recognized.
…
[End Correction]

6.24 Wildcard Response Alignment between ASN1 and ABNF

	Description:
	Chapter 6.2.2 allows wildcard response on all commands. It mentions that it is useful for audit. The ASN1 allows you to request a wildcard response on all commands [w-].

The ABNF only has it at:

auditRequest         = ["W-"] (AuditValueToken / AuditCapToken ) EQUAL

                                 TerminationID LBRKT auditDescriptor RBRKT

Although they might be deduced, the meanings of the "O-" and "W-" command prefixes are not explained.

	Reference:
	Subject:      Command prefixes

Date:         Sun, 31 Dec 2000 13:18:04 -0600

Reply-To: plong@ipdialog.com
From: Paul Long <plong@packetizer.com>

Organization: ipDialog, Inc.

To: MEGACO@STANDARDS.NORTELNETWORKS.COM


 [Begin Correction]

Section B.2 ABNF Specification

…

; "O-" indicates an optional command

; "W-" indicates an wildcarded response to a command
commandRequestList= ["O-"]["W-"] commandRequest *

(COMMA ["O-"]["W-"]commandRequest)

…

subtractRequest      =  SubtractToken EQUAL TerminationID 

                        [ LBRKT auditDescriptor RBRKT]

…

auditRequest         = (AuditValueToken / AuditCapToken ) EQUAL

                        TerminationID LBRKT auditDescriptor RBRKT

…

 [End Correction]

6.25 MTP addressing for non ITU variants

	Description:
	The MTP MID needed to be updated to allow for American and Japanese variants as it only currently allows ITU defined MTP addresses to be used.


 [Begin Correction]

A.2 ASN.1 Syntax Specification

MId ::= CHOICE

{


ip4Address 



IP4Address,


ip6Address



IP6Address,


domainName   


DomainName,


deviceName   


PathName,

mtpAddress



OCTET STRING(SIZE(2..4)),

 -- Addressing structure of mtpAddress:

 --                        0

 --        |  PC        | NI |

 --        24 -  bits    2 bits

-- Note: 14 bits is defined for international use.

 -- Two national options exist where the point code is 16 or 24 bits.

 -- To octet align the mtpAddress the MSBs shall be encoded as 0s.

...

}

B.2 ABNF Syntax Specification

…

; Addressing structure of mtpAddress:

;                      0

;    |  PC        | NI |

;    24 - 14 bits    2 bits

; Note: 14 bits is defined for international use.

; Two national options exist where the point code is 16 or 24 bits.

; To octet align the mtpAddress the MSBs shall be encoded as 0s. 

; An octet shall be represented by 2 hex digits.


mtpAddress           = MTPToken LBRKT 4*8(HEXDIG) RBRKT

[End Correction]

6.26 Audit Descriptor and Subtract and Statistics

	Description:
	The protocol document mentions:

7.1.1 Specifying Parameters

...

"A missing Audit descriptor is equivalent to an empty Audit Descriptor."

and also

7.1.15 Statistics Descriptor

...

"By default, statistics are reported when the Termination is Subtracted from the Context.  This behavior can be overridden by including an empty AuditDescriptor in the Subtract command."

 According to this text, if Subtract command does not have an AuditDescriptor it would mean that there is an empty audit descriptor and no statistics would be reported. And, if the MGC needs termination statistics, it must send AuditDescriptor with Statistics token in the Subtract command. But this seems to change the definition of "By default".


 [Begin Correction]

7.1.1 Specifying Parameters

…

If a required descriptor other than the Audit descriptor is unspecified (i.e., entirely absent) from a command, the previous values set in that descriptor for that termination, if any, are retained. In commands other than Subtract, a missing Audit descriptor is equivalent to an empty Audit Descriptor.  The behavior of the MG with respect to unspecified parameters within a descriptor varies with the descriptor concerened, as indicated in succeeding sections.  Whenever a parameter is underspecified or overspecified, the descriptor containing the value chosen by the responder is included as output from the command.

[End Correction]

6.27 Signal Lists

	Description:
	There are several inconsistency in the way the signal has been documented they are:

Section 7.1.11 states that Signal Lists have type. This is incorrect.

Section E.1.2 Doesn't not allow for the specification of which list instance of a signal contained in several lists should generate a notify completion.


 [Begin Correction]

7.1.11 Signal Descriptor

A sequential signal list consists of a signal list identifier and a sequence of signals to be played sequentially .  Only the trailing element of the sequence of signals in a sequential signal list may be an on/off signal.  The duration of a sequential signal list  is the sum of the durations of the signals it contains.  
 [End Correction]

[Begin Correction]

Section E.1.2 Events

…

Termination Method

ParameterID:  Meth (0x0002)

Indicates the means by which the signal terminated.
Type: enumeration

Possible values:

"TO" (0x0001) Duration expired

"EV" (0x0002) Interrupted by event

"SD" (0x0003) Halted by new Signals Descriptor

"NC" (0x0004) Not completed, other cause

Signal List ID

ParameterID:  SLID (0x0003)

Indicates to which signal list a signal belongs. The SignalList ID is only returned in cases where the signal resides in a signal list.

Type: integer
Possible values: Any integer
 [End Correction]

6.28 Topology

	Description:
	Topology specifications are cumulative over the life of the context. This is ambiguous in the text.


 [Begin Correction]

7.1.18 Topology Descriptor

…

The CHOOSE wildcard in a topology descriptor matches the TerminationID that the MG assigns in the first Add command that uses a CHOOSE wildcard in the same action.  An existing Termination that matches T1 or T2 in the Context to which a Termination is added, is connected to the newly added Termination as specified by the topology descriptor. If a termination is not mentioned within a topology descriptor, any topology associated with it remains unchanged. If, however, a new termination is added into a context its association with the other terminations within the context defaults to bothway, unless a topology descriptor is given to change this (eg. if T3 is added to a context with T1 and T2 with topology (T3,T1,oneway) it will be connected bothway to T2).
 [End Correction]

6.29 Value optionality in Packages

	Description:
	When supporting packages you must support all properties, signals, events and statistics. It is unclear in the specification on whether or not you must support all values of properties and parameter. The intention is that a subset of values may be supported. 


 [Begin Correction]

6.2.3 Packages

…

Properties, Events, Signals and Statistics defined in Packages, as well as parameters to them, are referenced by identifiers (Ids).  Identifiers are scoped. For each package, PropertyIds, EventIds, SignalIds, StatisticsIds and ParameterIds have unique name spaces and the same identifier may be used in each of them.  Two PropertyIds in different packages may also have the same identifier, etc.

To support a particular package the MG must support all Properties, Signals, Events and Statistics defined in a package. It must also support all Signal and Event parameters. The MG may support a subset of the values listed in a package for a particular Property or Parameter.

…

 [End Correction]

6.30 RequestID in AuditCapReply

	Description:
	Section 7.2.6  says "... The EventsDescriptor returns the list of possible events on the Termination together with the list of all possible values for the EventsDescriptor Parameters. ..."

What is the value of requestId sent in the events Descriptor from MG to MGC for a AuditCap reply ? ALL should be returned in this case.


 [Begin Correction]

A.2 ASN.1 Syntax Specification

…
-- For an AuditCapReply with all events, the RequestID SHALL be ALL.

-- ALL is represented by 0xffffffff.

RequestID ::= INTEGER(0..4294967295)   -- 32 bit unsigned integer

…

[End Correction]

[Begin Correction]

B.2 ABNF Syntax Specification

…

; For an AuditCapReply with all events, the RequestID should be ALL.

RequestID            = ( UINT32 / "*" )
…
 [End Correction]

6.31
Context ID Audit

	Description:
	H.248 allows you the audit the Context ID of where a termination currently belongs. This is not represented in the table in section 7.2.5 Audit Value. It should be added.


 [Begin Correction]

Section 7.2.5 Audit Value

…

	All
	wildcard
	Audit of all matching Terminations and the Context to which they are associated

	All
	Root
	List of all ContextIds

	All
	Specific
	(Non-null) Context Id in which the Termination currently exists


[End Correction]

6.32
Context Priorities

	Description:
	It is unclear in the recommendation on what the values of the priorities represent. 


[Begin Clarification]

6.1.1 Context Attributes and Descriptors

…

· The priority is used for a context in order to provide the MG with information about a certain precedence handling for a context. The MGC can also use the priority to control autonomously the traffic precedence in the MG in a smooth way in certain situations (e.g. restart), when a lot of contexts must be handled simultaneously. Priority 0 is the lowest priority and a priority of 15 is the highest priority.
…

[End Clarification]

6.33 Error and topology descriptors

	Description:
	Section 7.3/H.248 correctly states that when a MG reports an error to a MGC, it does so in an error descriptor.  However, the preceding sections listing (supposedly) all descriptors and detailing their contents do not contain any reference to error descriptors.  The command descriptions in Section 7.2 also seems to disallow error descriptors to be returned by MGs in reponse to requests from MGCs.  We propose to add clarifying text as shown below.

The topology descriptor was also accidentally omitted from the table in Section 6.2.4; its use is, however, explained in Section 7.1.18

	 Reference:
	 Subject: RE: Wherefore out thou Error Descriptor?

Date: Tue, 24 Apr 2001 15:50:40 -0500

From: Paul Long <plong@ipdialog.com>

To: <megaco@fore.com>


 [Begin Correction]

6.2.4  Termination Properties and Descriptors
…

	Statistics
	In Subtract and Audit, Report of Statistics kept on a Termination

	Topology
	Specifies flow directions between Terminations in a Context

	Error
	Contains and error code and optionally error text; it may occur in command replies and in Notify requests


…

[End Correction]

[Begin Correction]

7.1.19  Error descriptor

If a command responder encounters an error when processing a transaction request, it must include an error descriptor in its response.  A Notify request may contain an error descriptor as well.

An error descriptor consists of an error code, optionally accompanied by an error text.  Section 7.3 contains a list of valid error codes.
An error descriptor shall be specified at the "deepest level" that is semantically appropriate for the error being described and that is possible given any parsing problems with the original request. An error descriptor may refer to a syntactical construct other than where it appears. For example, error descriptor, 422 – Syntax Error in Action, could appear within a command even though it refers to the larger construct--the action--and not the particular command within which it appears.
 [End Correction]

[Begin Correction]

7.2 Command Application Programming Interface
Following is an Application Programming Interface (API) describing the Commands of the protocol.  This API is shown to illustrate the Commands and their parameters and is not intended to specify implementation (e.g. via use of blocking function calls).  It describes the input parameters in parentheses after the command name and the return values in front of the Command. This is only for descriptive purposes; the actual Command syntax and encoding are specified in later subsections.  The order of parameters to commands is not fixed.  Descriptors may appear as parameters to commands in any order.  The descriptors SHALL be processed in the order in which they appear.

An error descriptor is a possible reply to any command, the API does not specifically show this.

All parameters enclosed by square brackets ([. . . ]) are considered optional.

[End Correction]

6.34 Error in digit map activation

	Description:
	The current text in section 7.1.14.6 specifies that a digitmap is activated by means of an (possibly embedded) events descriptor that includes a digit map completion event, which itself contains a digit map parameter.  

A digit map completion event, however, cannot contain a digit map parameter.  Section E.6.2 also specifies that a digit map parameter has to be present.  

It is more accurate to say that the events descriptor that requests detection of the digitmap completion event must contain an eventDM parameter.  


 [Begin Correction]

7.1.14.6 DigitMap Activation

A digit map is activated whenever a new event descriptor is applied to the termination or embedded event descriptor is activated, that event descriptor contains a digit map completion event and the digit map completion event contains an eventDM field in the requested actions field. Each new activation of a digit map begins at step 1 of the above procedure, with a clear current dial string.  Any previous contents of the current dial string from an earlier activation are lost.

A digit map completion event that does not contain an eventDM field in its requested actions field, is considered an error.  Upon receipt of such an event in an EventsDescriptor, a MG shall respond with an error reponse, including error 457 – Missing parameter in signal or event.
 [End Correction]

[Begin Correction]

Error code 457 – Missing parameter in signal or event has been added to H.248 Annex L Error Codes and Service Change reasons.
 [End Correction]

[Begin Correction]

E.6.2 Events

…

EventsDescriptor parameters: None
…

E.6.5 Procedures
 Digit map processing is activated only if an events descriptor is activated that contains a digit map completion event as defined in Section E.6.2 and that digit map completion event contains an eventDM field in the requested actions as defined in Section 7.1.9.  Other parameters such as KeepActive or embedded events of signals descriptors may also be present in the events descriptor and do not affect the activation of digit map processing.
[End Correction]

6.35 Use of wildcarded TerminationIDs in Add command

	Description:
	The text in Section 7.2.1/H.248 implies that a CHOOSE wildcard is used only in Add commands that create ephemeral terminations, and cannot be used to allow a MG to choose a particular physical Termination.  Moreover, the text implies CHOOSE must be used to create ephemeral Terminations.  Neither restriction is valid. 


 [Begin Correction]

7.2.1 Add

…
The TerminationID specifies the termination to be added to the Context.  The Termination is either created, or taken from the null Context. If a CHOOSE wildcard is used in the TermintationID, the  selected TerminationID will be returned.  Wildcards may be used in an Add, but such usage would be unusual.  If the wildcard matches more than one TerminationID, all possible matches are attempted, with results reported for each one.  The order of attempts when multiple TerminationIDs match is not specified.

…

 [End Correction]

6.36 Meaning of Transaction replies

	Description:
	It is unclear when Transaction replies are sent, in particular in the presence of commands that activate signals.  Is the reply sent when

· the signals have finished,

· the signals have been activated, or

· when the signals have been queued for activation?

The intention is that the reply is sent when the signals have been queued for activation, implying that the signals descriptor was syntactically correct and only supported signals were requested.


 [Begin Correction]

Section 8.2.2 TransactionReply

The TransactionReply is invoked by the receiver.  There is one reply invocation per transaction.  A reply contains one or more Actions, each of which must specify its target Context and one or more Responses per Context. The TransactionReply is invoked by the command responder when it has processed the TransactionRequest.

A TransactionRequest has been processed

· when all commands in that TransactionRequest have been processed, or

· when an error is encountered in processing a non-optional command in that TransactionRequest.

A command has been processed when all descriptors in that command have been processed.

A SignalsDescriptor is considered to have been processed when it has been established that the descriptor is syntactically valid, the requested signals are supported and they have been queued to be played out.

An EventsDescriptor or EventBufferDescriptor is considered to have been processed when it has been established that the descriptor is syntactically valid, the requested events can be observed, any embedded signals can be generated, any embedded events can be detected, and the MG has been brought in a state in which the events will be detected.
…

[End Correction]

6.37 Empty Action requests

	Description:
	The syntax specification in Annex B/H.248 forbids actions that are completely empty.  In particular, an Action has to contain at least a command, a context modification request or a context audit request.  The text in Section 8 does not reflect this.  


 [Begin Correction]

8. Transactions

Commands between the Media Gateway Controller and the Media Gateway are grouped into Transactions, each of which is identified by a TransactionID.  Transactions consist of one or more Actions.  An Action consists of a non-empty series of Commands, Context property modifications, or Context property audits that are limited to operating within a single Context.

…

[End Correction]

6.38 Auditing list of TerminationIDs

	Description:
	The protocol contains syntax to allow a MGC to audit which Terminations are in a Context.  The relevant clauses in the binary and text encodings are contextAuditResult and contextTerminationAudit.

The intention that the binary and text versions have the same functionality is not met in this case. 


 [Begin Correction]

A.2 ASN.1 Syntax Specification

…

AuditReply ::= CHOICE
{


contextAuditResult
TerminationIDList,

error



ErrorDescriptor,


auditResult


AuditResult,


...

}

AuditResult ::= SEQUENCE
{


terminationID

TerminationID,


terminationAuditResult
TerminationAudit

}

TerminationAudit ::= SEQUENCE OF AuditReturnParameter

…
[End Correction]

6.39 Handoff in case of MGC failure

	Description:
	Section 11.5/H.248 contains procedures to be followed by MGs in case of MGC failure.  The scenario addressed in the second paragraph of this section states that a MG that does not receive an Audit request after having established a control relationship with a backup MGC, acts as if this backup MGC failed.  This imposes restrictions on MGC behavior that are unnecessary.  For instance, the backup MGC could be a hot standby that does not need to audit the MG when it takes over control.  Therefore we propose striking the clause stating this.

Furthermore, the text states that the MG should follow its controlling MGC’s Handoff request.  


 [Begin Correction]

11.5 Failure of an MG

…

In partial failure, or manual maintenance reasons, an MGC may wish to direct its controlled MGs to use a different MGC.  To do so, it sends a ServiceChange method to the MG with a "HandOff" method, and its designated replacement in ServiceChangeMgcId. If "Handoff" is supported the MG shall send a ServiceChange message with a "Handoff" method and a "MGC directed change" reason to the designated MGC.  If it fails to get a reply from the designated MGC,  the MG shall behave as if its MGC failed, and start contacting secondary MGCs as specified in the previous paragraph.  If the MG is unable to establish a control relationship with any MGC, it shall wait a random amount of time as described in section 9.2 and then start contacting its primary, and if necessary, its secondary MGCs again.

[End Correction]

6.40 Syntax for signal and event parameters in Annex A.2

	Description:
	Section A.2 contains a clause defining EventParameters as a SEQUENCE consisting of an eventParameterName followed by a value.  The type of the value is defined as OCTET STRING.  In order to support lists of values mentioned in Section 12.2/H.248, the type of the value field has to be changed to SEQUENCE OF OCTET STRING.


 [Begin Correction]

A.2 ASN.1 Syntax Specification

…

TimeNotation ::= SEQUENCE

{


date 

IA5String(SIZE(8)),  -- yyyymmdd format


time 

IA5String(SIZE(8))   -- hhmmssss format

}

Value ::= SEQUENCE OF OCTET STRING
END
[End Correction]

6.41 Definition of PathName in Annex A.3

	Description:
	Annex A states that it reproduces the definition of  PathName of Annex B.  However, the definition presented there is not the same as that provided in Annex B. The appropriate text from Annex B should be copied to Annex A, replacing the current definition given in A.3.


 [Begin Correction]

A.3 Digit maps and path names

…

A path name is also a string with syntactic restrictions imposed upon it.  The ABNF production defining it is copied from Annex B.


; Total length of pathNAME must not exceed 64 chars.

pathNAME             = ["*"] NAME *("/" / "*"/ ALPHA / DIGIT /"_" / "$" ) 

                       ["@" pathDomainName ]

; ABNF allows two or more consecutive "." although it is meaningless 

; in a path domain name. 

pathDomainName       = (ALPHA / DIGIT / "*" ) 

                       *63(ALPHA / DIGIT / "-" / "*" / ".")

NAME                 = ALPHA *63(ALPHA / DIGIT / "_" )

[End Correction]

6.42 Packaged Name in Modem Descriptor in ABNF

	Description:
	The ASN.1 Modem Descriptor contains a sequence of Modem parameters of format Packaged Name. The ABNF only contains NAME which does not allow for package definition. Package definition should be allowed.


 [Begin Correction]

B.2 ABNF Syntax Specification

…

modemDescriptor      = ModemToken (( EQUAL modemType) / 

                       (LSBRKT modemType *(COMMA modemType) RSBRKT))

                       [ LBRKT propertyParm 

                      *(COMMA propertyParm) RBRKT ]

[End Correction]

6.43 Error descriptor in Notify request

	Description:
	Recommendation H.248 allows Notify requests to contain error descriptors.  The recommendation does not specify under which circumstances error descriptors are to be included in Notify requests.  One case where this is useful is in a Notify request that contains the generic error event defined in Annex E.1.2.  This is used in the case when a general error event is triggered.


 [Begin Correction]

7.2.7 Notify

…

The RequestID returns the RequestID parameter of the EventsDescriptor that triggered the Notify Command.  It is used to correlate the notification with the request that triggered it.  The events in the list must have been requested via the triggering EventsDescriptor or embedded events descriptor unless the RequestID is 0 (which is for further study).

The ErrorDescriptor may be sent in the Notify as a result of Error 518 - Event buffer full.
 [End Correction]

6.44 Octet strings in text encoding

	Description:
	Sometimes it is desirable to transfer octet strings between MG and MGC.  The definition of octet string in Annex B is not general enough because it is essentially a text string.  Not all characters are allowed in text strings.  The null character (0x00) is an example of a character that is not allowed in a text string.

A solution to this problem is to use a standard way of encoding the octet string into a text string.  Prescribing one way to be used in all cases facilitates uniform encoding and decoding routines.

Another problem with the current definition of octetString in Annex B/H.248 is the fact that opening and closing braces must be escaped (i.e. preceded by a backslash).  This contradicts the provision in section 7.1.8 that SDP session descriptions conformant to RFC 2327 must be accepted.


 [Begin Correction]

Section Annex B Text encoding of the protocol (Normative)

…

B.3 Hexadecimal octet coding: 
Hexadecimal octet coding is a means for representing a string of octets as a string of hexadecimal digits, with two digits representing each octet. This octet encoding should be used when encoding octet strings in the text version of the protocol.

For each octet, the 8-bit sequence is encoded as two hexadecimal digits. Bit 0 is the first transmitted; bit 7 is the last.

Bits 7-4 are encoded as the first hexadecimal digit, with Bit 7 as MSB and Bit 4 as LSB. Bits 3-0 are encoded as the second hexadecimal digit, with Bit 3 as MSB and Bit 0 as LSB.

Examples:

	Octet bit pattern
	Hexadecimal coding

	00011011
	D8

	11100100
	27

	10000011 10100010 11001000 00001001
	C1451390


B.4 Hexadecimal octet sequence: 

A hexadecimal octet sequence is an even number of hexadecimal digits, terminated by a <CR> character.

[End Correction]

6.45 Annex C USI Correction

	Description:
	H.248 Annex C lists tag 9008 as being for the USI. However the values only represent the User Information Layer 1. The whole USI value should be supported and the layer 1 protocol.


 [Begin Correction]

Annex C.9 Bearer Capabilities

…

	layer1prot
	9008
	5 BITS
	User Information Layer 1 Protocol

Reference: ITU Recommendation Q.931

Bits 5 4 3 2 1

00001 – CCITT standardized rate adaption V.110 and X.30. 

00010 - Recommendation G.711  u-law

00011 - Recommendation G.711 A-law

00100 – Recommendation G.721 32 kbit/s ADPCM and Recommendation I.460.

00101 - Recommendations H.221 and H.242

00110 – Recommendations H.223 and H.245

00111 – Non-ITU-T standardized rate adaption.  

01000 – ITU-T standardized rate adaption V.120. 

01001 – CCITT standardized rate adaption X.31 HDLC flag stuffing.


All other values are reserved.


...

	USI
	9023
	OCTET STRING
	User Service Information

Reference ITU Recommendation Q.763 Section 3.57


 [End Correction]

6.46 Encoding of ASN.1 Octet String

	Description:
	It is ambigious how Integer, boolean etc is encoded in the ASN.1 when OCTET STRING is used to encode properties.

	Reference:
	Subject: Re: OCTET STRINGs in H.248

Date: Wed, 8 Nov 2000 14:25:29 –0500

From: Troy Cauble <troy@LUCENT.COM>

To:  MEGACO@STANDARDS.NORTELNETWORKS.COM


 [Begin Correction]

A.2 ASN.1 Syntax Specification

…

NOTE – The ASN.1 specification below contains a clause defining TerminationIDList as a sequence of TerminationIDs.  The length of this sequence SHALL be one, except possibly when used in contextAuditResult.

NOTE – The ASN.1 in this section uses OCTET STRING to encode values for property parameter, signal parameter and event parameter values and statistics. The actual types of these values vary and are specified in Annex C or the relevant package definition.

A value is first ASN.1 BER encoded based on it's type using the table below.  The result of this ASN.1 BER encoding is then encoded as an ASN.1 BER OCTET STRING, "double wrapping" the value. The format specified in Annex C or the package relates to ASN.1 BER encoding according to the following table:

	Type Specified in Package
	ASN.1 BER Type

	String (UTF-8)
	IA5String

	Integer (4 Octet)
	INTEGER

	Double (8 octet signed int)
	INTEGER (Note 3)

	Character (UTF-8, Note 1)
	IA5String

	Enumeration
	ENUM

	Boolean
	BOOLEAN

	Unsigned Integer (Note 2)
	INTEGER (Note 3)

	Note 1 : Can be more than one byte

	Note 2 : Unsigned integer is referenced in Annex C

	Note 3 : ASN.1 BER encoding of INTEGER does not imply the use of 4 bytes.


See 8.7 X.690 for definition of encoding of Octet String value.
MEDIA-GATEWAY-CONTROL DEFINITIONS AUTOMATIC TAGS::=

BEGIN

…

 [End Correction]

6.47 Property and descriptor values of subtracted physical Terminations

	Description:
	The text of H.248 states that only property values in TerminationState and LocalControl descriptors remain visible when a physical Termination is subtracted, unless provisioned otherwise.

More specifically, Section 7.2.3/H.248 states that the property values revert to provisioned values or, if no value is provisioned, the default value specified for that property.  Section 6.2.4 states that all descriptors except TerminationState and LocalControl revert to empty/"no value" when a physical Termination is returned to the NULL context or when it is first created, except when provisioned otherwise.

H.248 packages (should) include default values for properties they define, or specify that the default value is provisioned.  Hence, a MGC that supports a particular package has knowledge of the property values that result from subtracting a termination.  However, an MGC does not know what the provisioned values of properties in descriptors other than TerminationState and LocalControl are.  This may lead to interoperability problems, unless the MGC audits all physical Terminations after cold boot and finds out about the provisioned property values.

	Reference:
	Editor


 [Begin Correction]

6.2.4 Termination Properties and Descriptors

Terminations have properties.  The properties have unique PropertyIDs.  Most properties have default values, which are explicitly defined in this standard or in a package (see Section 12) or set by provisioning.  If not provisioned otherwise, the properties in all descriptors except TerminationState and LocalControl default to empty/"no value" when a Termination is first created or returned to the null Context.  The default contents of the two exceptions are described in sections 7.1.5 and 7.1.7.
The provisioning of a property value in the MG will override any default value, be it supplied in this standard or a package.  Therefore if it is essential for the MGC to have full control over the property values of a Termination, it should supply explicit values when ADDing the Termination to a Context.  Alternatively, for a physical Termination the MGC can determine any provisioned property values by auditing the Termination while it is in the NULL Context.
…
[End Correction]

6.48 Problem with syntax of auditOther

	Description:
	In the text of section 7.2.5 on AuditValue, the following can be found:

"Specifying an empty Audit Descriptor results in only the TerminationID being returned."

In the syntax in Annex B, this would be achieved by using the auditOther

construction. However, the syntax is:

 auditOther           = EQUAL TerminationID LBRKT  terminationAudit RBRKT

In order to return only the TerminationID, the syntax should be:

 auditOther           = EQUAL TerminationID [ LBRKT  terminationAudit

                                RBRKT ]

The optionality is shown correctly in the ASN.1 syntax.

	Reference:
	Subject:  Problem with syntax of auditOther
Date:  Fri, 17 Nov 2000 17:43:28 -0500

From: "Brown, Michael" <C.Michael.Brown@AMERICASM10.NT.COM>

Organization: Nortel Networks

To: MEGACO@STANDARDS.NORTELNETWORKS.COM


 [Begin Correction]

B.2 ABNF Syntax Specification

...

auditOther           = EQUAL TerminationID [LBRKT 

terminationAudit RBRKT]
...
 [End Correction]

6.49 Stream Mode default 

	Description:
	 When adding a termination to a context the stream mode should be set to inactive unless specified overwise. This is not mentioned in the text. It is also unclear 

	Reference:
	 Subject: Re: H.248 Mode Parameter

 Date: Fri, 8 Dec 2000 13:23:09 –0500

 From: "Taylor, Tom-PT [NORSE:B901:EXCH]" <taylor@AMERICASM01.NT.COM>

 To: MEGACO@STANDARDS.NORTELNETWORKS.COM


 [Begin Correction]

7.1.7 LocalControl Descriptor

The LocalControl Descriptor contains the Mode property, the ReserveGroup and ReserveValue properties and properties of a termination (defined in Packages) that are stream specific, and are of interest between the MG and the MGC.  Values of properties may be underspecified as in section 7.1.1.

The allowed values for the mode property are send-only, receive-only, send/receive, inactive and loop-back.  "Send" and "receive" are with respect to the exterior of the context, so that, for example, a stream set to mode=sendonly does not pass received media into the context. The default value for the mode property is "Inactive". Signals and Events are not affected by mode. 

…

 [End Correction]

6.50 Optional Stream in ASN.1

	Description:
	 The specification of a stream in the media descriptor is optional. The ASN.1 does not allow for this optionality and should be updated.

	Reference:
	Subject: MEGACO: MediaDescriptor question

Date: Thu, 14 Dec 2000 22:42:47 -0500

From: Troy Cauble <troy@LUCENT.COM>

To: MEGACO@STANDARDS.NORTELNETWORKS.COM


 [Begin Correction]

A.2 ASN.1 Syntax Specification

…

MediaDescriptor ::= SEQUENCE

{


termStateDescr
TerminationStateDescriptor OPTIONAL,


streams

CHOICE 





{






oneStream
StreamParms,






multiStream
SEQUENCE OF StreamDescriptor





} OPTIONAL,


...

}

…

 [End Correction]

[Begin Correction]

B.2 ABNF Syntax Specification

…

; at-most-once per item

; and may include either streamParm or streamDescriptor but not both

mediaParm            = (streamParm / streamDescriptor / 

                        terminationStateDescriptor)

…

 [End Correction]

6.51 embedSig/EmbedSig

	Description:
	Although correct, "EmbedSig" should be replaced with "embedSig" in order to be stylistically consistent and therefore avoid confusion.

	Reference:
	Subject: Re: ABNF semantics

Date: Fri, 12 Jan 2001 15:48:34 –0500

From: "Rosen, Brian" <Brian.Rosen@MARCONI.COM>

To: MEGACO@STANDARDS.NORTELNETWORKS.COM


 [Begin Correction]

B.2 ABNF Syntax Specification

…

secondEventParameter = ( embedSig / KeepActiveToken / eventDM / 

                         eventStream / eventOther )

…

 [End Correction]

6.52 ABNF syntax error of Local Control Descriptor, Modem and Termination State Descriptor
	Description:
	According to the ASN.1 syntax the property parameter is SEQUENCE, it can be only one as per the ABNF syntax. A comment "at-most-once per item, except for propertyParm" can be added to solve this issue.

	Reference:
	Subject:  RE: ABNF syntax error of LocalControlDescriptor and TerminationStateDescriptor

Date: Thu, 4 Jan 2001 21:13:25 -0500

From: "Tom-PT Taylor" <taylor@nortelnetworks.com>

To: Richie Wu <wuyongji@YAHOO.COM>, megaco@standards.nortelnetworks.com

 


 [Begin Correction]

B.2 ABNF Syntax Specification

…

localControlDescriptor = LocalControlToken LBRKT localParm 

                         *(COMMA localParm) RBRKT

; at-most-once per item except for propertyParm
localParm            = ( streamMode / propertyParm / reservedValueMode

     / reservedGroupMode )

…
terminationStateDescriptor = TerminationStateToken LBRKT 

            terminationStateParm *( COMMA terminationStateParm ) RBRKT

; at-most-once per item except for propertyParm
terminationStateParm =(propertyParm / serviceStates / eventBufferControl )

…
; at-most-once except for extensionParameter
modemType            = (V32bisToken / V22bisToken / V18Token / 

                        V22Token / V32Token / V34Token / V90Token / 

                        V91Token / SynchISDNToken / extensionParameter)

…
 [End Correction]

6.53 Wilcarding Context IDs

	Description:
	 Wildcarding ContextIDs is allowed however it is poorly specified in the current specification.

	Reference:
	Subject: Re: Wildcard in ContextId

Date: Fri, 12 Jan 2001 15:14:46 –0500

From: Michael Brown C.Michael.Brown@NORTELNETWORKS.COM
Organization: Nortel Networks

To: MEGACO@STANDARDS.NORTELNETWORKS.COM


 [Begin Correction]

6.1.1 Context Attributes and Descriptors

The attributes of Contexts are:

· ContextID. A wildcarding mechanism using two types of wildcards can be used with ContextIDs.  The two wildcards are ALL and CHOOSE.  The former is used to address ALL (except the NULL context) Contexts at once in a command request and/or reply, while the latter is used to indicate to a media gateway that it must create a Context. 
…

[End Correction]

6.54 ABNF is Context dependent

	Description:
	It is not clear whether the ABNF is context dependent.

	Reference:
	Subject:      context-free?

Date:         Wed, 13 Dec 2000 17:54:32 –0600

Reply-To: plong@ipdialog.com
From: Paul Long <plong@packetizer.com>

Organization: ipDIalog, Inc.

To: MEGACO@STANDARDS.NORTELNETWORKS.COM


 [Begin Correction]

B.2 ABNF Syntax Specification

The protocol syntax is presented in ABNF according to RFC2234. 
Note - The syntax is context-dependent. For example, "Add" can be the AddToken or a NAME depending on the context in which it occurs.
…

 [End Correction]

6.55 Double quote not allowed in quotedString and Empty Quote allowed.
	Description:
	Reiterate that the double-quote character is not allowed in a quotedString.

Empty quotedString is allowed as, section E.6.2 says under parameter "ds",

"string of digit map symbols (possibly empty) returned as

a quoted string".

	Reference:
	Subject:      Re: Double quote not allowed

Date:         Tue, 19 Dec 2000 16:54:21 -0600

Reply-To: plong@ipdialog.com

From: Paul Long <plong@packetizer.com>

Organization: ipDialog, Inc.

To: MEGACO@STANDARDS.NORTELNETWORKS.COM
Subject: RE: empty quotedString ?

Date: Thu, 10 May 2001 16:30:42 -0500

From: "Paul Long" plong@ipdialog.com
To:  "MEGACO list" <megaco@fore.com>


 [Begin Correction]

B.2 ABNF Syntax Specification

…

;Note – The double-quote character is not allowed in quotedString.
quotedString         = DQUOTE *(SafeChar / RestChar/ WSP) DQUOTE

…

 [End Correction]

6.56 Ranges and Multiple Values for Signal and Event Parameter Values

	Description:
	 The ABNF specification allows the specification of multiples values and ranges for a signal and event parameter value. The ASN.1 currently does not and needs to be aligned.

	Reference:
	Subject:  Re: ASN.1 - Values in Event and Signal Parameter

Date: Mon, 15 Jan 2001 12:42:25 +0530

From: Sandeep Gautam <sgautam@HSS.HNS.COM>

To: MEGACO@STANDARDS.NORTELNETWORKS.COM


 [Begin Correction]

A.2 ASN.1 Syntax Specification

…

EventParameter ::= SEQUENCE

{


eventParameterName
Name,


value            

Value,

-- For use of extraInfo see the comment related to PropertyParm


extraInfo CHOICE


{



relation
Relation,



range

BOOLEAN,



sublist
BOOLEAN


} OPTIONAL,


...
}

…

SigParameter ::= SEQUENCE

{


sigParameterName 

Name,


value            

Value,
-- For use of extraInfo see the comment related to PropertyParm

extraInfo CHOICE


{



relation
Relation,



range

BOOLEAN,



sublist
BOOLEAN


} OPTIONAL,

...
}

…

 [End Correction]

6.57 TimeStamp in registration replies

	Description:
	There is a mismatch between the text in section 7.2.8 which states " The TimeStamp parameter shall be sent with a registration command and its response." and the ASN.1 and ABNF syntax which only allows it in the request.

There is confusion regarding whether the timeStamps in registration requests and replies are relative or absolute.

	Reference:
	Subject: TimeStamp in registration replies

Date: Mon, 15 Jan 2001 13:33:19 –0500

From: Terry L Anderson <tla@LUCENT.COM>

Organization: Lucent Technologies

To: MEGACO@STANDARDS.NORTELNETWORKS.COM
Subject: RE: "Does anybody really know what time it is?" :-)

Date: Wed, 2 May 2001 13:30:37 –0500

From: "Paul Long" <plong@ipdialog.com>

To: "'Megaco \(E-mail\)" <megaco@fore.com>


[Begin Clarification]

7.2.8 Service Change

…

The optional TimeStamp parameter specifies the actual time as kept by the sender.  As such, it is not necessarily absolute time according to, for example, a local time zone—it merely establishes an arbitrary starting time against which all future timestamps transmitted by a sender during this association shall be compared.  It can be used by the responder to determine how its notion of time differs from that of its correspondent.  TimeStamp is sent with a precision of hundredths of a second, and is expressed in UTC.  

…

 [End Clarification]

[Begin Correction]

A.2 ASN.1 Syntax Specification

…

ServiceChangeResParm ::= SEQUENCE

{


serviceChangeMgcId 
MId OPTIONAL,


serviceChangeAddress
ServiceChangeAddress OPTIONAL,


serviceChangeVersion
INTEGER(0..99) OPTIONAL,

serviceChangeProfile
ServiceChangeProfile OPTIONAL,
timestamp


TimeNotation OPTIONAL,
...

}

…

[End Correction]

[Begin Correction]

B.2 ABNF Syntax Specification

…

servChgReplyParm     = (serviceChangeAddress / serviceChangeMgcId /

                       serviceChangeProfile / serviceChangeVersion /

                       TimeStamp)

…

[End Correction]

6.58 ABNF Token for Signals and Events overlap with packages

	Description:
	In the ABNF it is possible to define new tokens for Signal and Event information elements. Ie. 

sigParameter = sigStream / sigSignalType / sigDuration

              / sigOther / notifyCompletion / KeepActiveToken

The package identity can be contained in sigOther. The problem lies in the fact that in the future that the introduction of a new Token for a signal Parameter may cause overlap with an existing package identity. This would lead to an ambiguous interpretation. This problem relates to both signals and events.

The solution below limits the any new sigParameter or eventParameter Tokens to start with a certain prefix. An update is also made to the package definition rules saying that packages identiities cannot start with this prefix.


 [Begin Correction]

B.2 ABNF Syntax Specification

…

RestChar             = ";" / "[" / "]" / "{" / "}" / ":" / "," / "#" / 

                       "<" / ">" / "=" 

; New Tokens added to sigParameter must take the format of SPA*

; * may be of any form ie. SPAM

; New Tokens added to eventParameter must take the form of EPA*

; * may be of any form ie. EPAD

AddToken                   = ("Add"                   / "A")

[End Correction]

[Begin Correction]

12.2 Guidelines to defining Properties, Statistics and Parameters to Events and Signals

Parameter Name: only descriptive

ParameterID: Is an identifier. The textual ParameterID of parameters to Events and Signals shall not start with "EPA" and "SPA", respectively. The textual ParameterID shall also not be "ST", "Stream", "SY", "SignalType", "DR", "Duration", "NC", "NotifyCompletion", "KA", "Keepactive", "EB", "Embed", "DM" or "DigitMap".
[End Correction]

6.59 Aligning Error Code Description with Annex L

	Description:
	 In the development of H.248 Annex L Service Change Reason and Error Code definition it was identified that the definition of several of the codes in 7.3 were incomplete. These should be aligned.

	Reference:
	 Editor


 [Begin Correction]

7.3 Command Error Codes

Errors consist of an IANA registered error code and an explanatory string.  Sending the explanatory string is optional.  Implementations are encouraged to append diagnostic information to the end of the string.

When a MG reports an error to a MGC, it does so in an error descriptor.  An error descriptor consists of an error code and optionally the associated explanatory string.
H.248 Annex L contains the error codes supported by H.248.




















…
[End Correction]

6.60 Termination ID in Notify Reply

	Description:
	The API does not show a notify reply returning a termination ID. However the ASN.1 and ABNF return the terminationID. The API should be updated. The ASN.1 also shows that the return is optional however the ABNF shows the return as manadatory. The return should be mandatory in line with the other commands.

	Reference:
	Editor


[Begin Correction]

7.2.7 Notify

The Notify Command allows the Media Gateway to notify the Media Gateway Controller of events occurring within the Media Gateway.

TerminationID

Notify(TerminationID,


           ObservedEventsDescriptor,

[ErrorDescriptor]

…

[End Correction] 

[Begin Correction]

A.2 ASN.1 Syntax Specification 

NotifyReply ::= SEQUENCE

{


terminationID   


TerminationIDList,


errorDescriptor 


ErrorDescriptor OPTIONAL,


...

}

…

[End Correction]

6.61 missing Descriptor vs. missing properties in a Descriptor
	Description:
	The issue is that when a MGC sends a Command to the MG, it makes a big difference whether a Descriptor is completely omitted from the Command, or only some properties are omitted from an existing Descriptor.  At the moment this is not well explained in section 6.2.4/H.248, as it does not clearly distinguish between a “Descriptor” and a “Property”.  In addition, sections 6.2.4 and 7.1.x are contradicting each other, viz. where the sections 7.1.x state explicitly that a new setting of a Descriptor completely replaces the previous setting of that Descriptor – see e.g. the last para of section 7.1.6/H.248.   The matter can be resolved by changing the 4th paragraph of section 6.2.4/H.248 as follows:

	Reference:
	Editor


 [Begin Correction]

6.2.4 Termination Properties and Descriptors

…

When a Termination is Added to a Context, the value of its read/write properties can be set by including the appropriate descriptors as parameters to the Add command.  Similarly, a property of a Termination in a Context may have its value changed by the Modify command.  Properties may also have their values changed when a Termination is moved from one Context to another as a result of a Move command.  In some cases, descriptors are returned as output from a command.


In general, if a Descriptor is completely omitted from one of the aforementioned Commands, the properties in that Descriptor retain their prior values for the Termination(s) the Command acts on. On the other hand, if some properties are omitted from a Descriptor in a Command – i.e., the Descriptor is only partially specified – those properties will be removed/reset for the Termination(s) the Command acts on. For more details, see section 7.1 dealing with the individual Descriptors.
…

[End Correction]

6.62 Redundant MGs

	Description:
	

	Reference:
	Subject: Re: FW: Redundant MG's

Date: Wed, 31 Jan 2001 08:04:04 -0500

From: "Rosen, Brian" <Brian.Rosen@MARCONI.COM>


 [Begin Correction]

11.4 Failure of an MG

…

Allowing the MGC to send duplicate messages to both MGs accommodates pairs of MGs that are capable of redundant failover of one of the MGs.  Only the Working MG shall accept or reject transactions.  Upon failover, the Primary MG sends a ServiceChange command with a "Failover" method and a "MG Impending Failure" reason.  The MGC then uses the secondary MG as the active MG.  When the error condition is repaired, the Working MG can send a "ServiceChange" with a "Restart" method. Redundant failover MGs with the current protocol definition requires a reliable transport, and knowledge in the MGC of the redundancy at the MG.
 [End Correction]

6.63 Use of MID

	Description:
	There is some confusion on the usage of MID and how it relates to the control association between a MGC and MG.

	Reference:
	Subject: RE: Use of MID: Implementors Guide Issue

Date: Tue, 13 Feb 2001 16:09:38 -0800

From: "Kaul, Bharat" <bharat@trillium.com>

To: "'Christian Groves'" <Christian.Groves@ericsson.com>, "Kaul, Bharat" 


 [Begin Correction]

8.3 Messages

Multiple Transactions can be concatenated into a Message.  Messages have a header, which includes the identity of the sender. The Message Identifier (MID) of a message is set to a provisioned name (e.g. domain address/domain name/device name) of the entity transmitting the message.  Domain name is a suggested default. An H.248 entity (MG/MGC) must consistently use the same MID in all messages it originates for the duration of control association with the peer (MGC/MG).
…

[End Correction]

6.64 Typographical error in ASN.1 choose

	Description:
	There is a typographical error in the numerical value for CHOOSE in the ASN.1 description.

	Reference:
	Subject: Typo in H.248 standard.

Date: Wed, 21 Mar 2001 08:56:14 -0700 (MST)

From: Nattapong Mongkolnavin nm59@avaya.com
To: Christian.Groves@ericsson.com


 [Begin Correction]

A.2 ASN.1 Syntax Specification

...

-- Context NULL Value: 0

-- Context CHOOSE Value: 4294967294 (0xFFFFFFFE) 

-- Context ALL Value: 4294967295 (0xFFFFFFFF)

...

 [End Correction]

6.65 Events desciptor and eventDM 

	Description:
	H.248 Implementors' Guide 6.38 introduces an inconsistency in H.248.  Namely, section 7.1.14.8/H.248 implies it is possible to specify an EventsDescriptor comprising a Digit Map Completion event, and still omit the eventDM parameter.  Section 7.1.14.8/H.248 should be changed to align with the change in approved H.248 Implementors' Guide 6.38.

	Reference:
	Editor


 [Begin Correction]

7.1.14.8  Wildcards

Note that if a package contains a digit map completion event, then an event specification consisting of the package name with a wildcarded ItemID (Property Name) will activate a digit map– to that end the event specification must include an eventDM field according to section 7.1.14.6.If the package also contains the digit events themselves, this form of event specification will cause the individual events to be reported to the MGC as they are detected.

 [End Correction]

6.66 MTP Address and ServiceChange

	Description:
	The definition of the MTP address when used with a ServiceChange is different from the MTP address used for the MID. The usage should be consistent.

	Reference:
	Editor


 [Begin Correction]

A.2 ASN.1 Syntax Specification

...

ServiceChangeAddress ::= CHOICE

{


portNumber


INTEGER(0..65535), -- TCP/UDP port number


ip4Address 


IP4Address,


ip6Address


IP6Address,


domainName   

DomainName,


deviceName   

PathName,

mtpAddress


OCTET STRING(SIZE(2..4)),


...

}

...

[End Correction]

[Begin Correction]

B.2 ABNF Syntax Specification

...

mId                  = (( domainAddress / domainName )

        [":" portNumber]) / mtpAddress / deviceName

...

serviceChangeAddress = ServiceChangeAddressToken EQUAL ( mId / portNumber )
...

 [End Correction]

6.67 Optional Reserve in ASN.1

	Description:
	The procedural text and the ABNF specification state that reserve group and reserve property have a default and thus do not need to be set. The ASN.1 does not have this possibity. OPTIONAL should be added.

	Reference:
	Subject: RE: [Fwd: RE: Adding Streams]

Date: Thu, 15 Mar 2001 16:27:44 –0600 (CST)

From: David Dykhuizen <eusdbd@exu.ericsson.se>

To: Christian.Groves@ericsson.com, Brian.Rosen@marconi.com


 [Begin Correction]

A.2 ASN.1 Syntax Specification

...

LocalControlDescriptor ::= SEQUENCE

{


streamMode    
StreamMode OPTIONAL,


reserveValue
BOOLEAN OPTIONAL,


reserveGroup
BOOLEAN OPTIONAL,


propertyParms 
SEQUENCE OF PropertyParm,


...

}

...

 [End Correction]

6.68 Transaction Acknowledgement Lists

	Description:
	As per the ABNF definition we can acknowledge a list of unsequential transaction, for example:

TransactionResponseAck = { 1234, 123-456, 9, 78-100 }.

But, as per ASN.1 syntax, only one transaction or a sequential range of transaction can be acknowledged at one time, viz. Either

TransactionResponseAck = { 123} or

TransactionResponseAck = {123-456}.

These methods should be aligned.

	Reference:
	Subject: Confilict definitions of transaction response acknowledgement

Date: Tue, 13 Mar 2001 23:45:13 -0800 (PST)

From: Richie Wu wuyongji@yahoo.com
To: megaco@fore.com


 [Begin Correction]

A.2 ASN.1 syntax specification

...

TransactionResponseAck ::= SEQUENCE OF TransactionAck

TransactionAck
::= SEQUENCE

{


firstAck
TransactionId,


lastAck
TransactionId OPTIONAL

}

...

 [End Correction]

6.69 Private Package ID registration

	Description:
	The IANA resgistration at < <http://www.isi.edu/in-notes/iana/assignments/megaco-h248>

Differs from the registration procedure in the H.248 recommendation for package Ids for private packages. The recommendation should align with the IANA procedure.

	Reference:
	Editor


 [Begin Correction]

13.1 Packages

The following considerations SHALL be met to register a package with IANA:

1. A unique string name, unique serial number and version number is registered for each package.  The string name is used with text encoding.  The serial number shall be used with binary encoding.  Serial Numbers 0x8000 to 0xffff are reserved for private use. Serial number 0 is reserved.
…

[End Correction]

6.70 Gain Control property type

	Description:
	The gain control parameter in the "tdmc" package specifies the type to be "enumeration" when clearly it should be "integer".

	Reference:
	Subject: Gain control parameter

Date: Thu, 8 Mar 2001 16:05:34 –0800

From: "Kumar, Sanjay1" <sanjayk@trillium.com>

To: "'megaco@fore.com'" <megaco@fore.com>


 [Begin Correction]

E.13.1 Properties

Gain Control

PropertyID: gain (0x000a)

Gain control, or usage of of signal level adaptation and noise level reduction is used to adapt the level of the signal. However, it is necessary, for example for modem calls, to turn off this function.

Type: integer
Possible Values:

…

[End Correction]

6.71 Missing Characteristics in the base root package

	Description:
	The characteristics of properties in the base root package are missing. The characteristics should be present in a package.

	Reference:
	Editor


 [Begin Correction]

…

E.2.1 Properties

MaxNrOfContexts


PropertyID: maxNumberOfContexts (0x0001)

The value of this property gives the maximum number of contexts that can exist at any time.  The NULL context is not included in this number.

Type: Double

Possible values: 1 and up

Defined in: TerminationState
Characteristics: read only
MaxTerminationsPerContext

PropertyID: maxTerminationsPerContext (0x0002)

The maximum number of allowed terminations in a context, see section 6.1

Type: Integer

Possible Values: any integer

Defined In: TerminationState
Characteristics: read only
normalMGExecutionTime

PropertyId: normalMGExecutionTime (0x0003)

Settable by the MGC to indicate the interval within which the MGC expects a response 
to any transaction from the MG (exclusive of network delay)

Type: Integer

Possible Values: any integer, represents milliseconds

Defined in: TerminationState
Characteristics: read / write
normalMGCExecutionTime

PropertyId: normalMGCExecutionTime (0x0004)

Settable by the MGC to indicate the interval within which the MG should expects a 
response to any transaction from the MGC (exclusive of network delay)

Type: Integer

Possible Values: any integer, represents milliseconds

Defined in: TerminationState
Characteristics: read / write
…

[End Correction]

6.72 Service Change address / Service Change MGCid mutual exclusion

	Description:
	Section 7.2.8 mentions "ServiceChangeAddress and ServiceChangeMgcId Parameters must not both be present in the ServiceChangeDescriptor or the ServicechangeResultDescriptor"

If I understand this right,  a comment on this mutual exclusion should

be included in Annex B just like there is one in  the case of eventParameter for embedWithSig / embedNoSig

	Reference:
	Subject: ServiceChangeAddress / ServiceChangeMgcId

Date: Wed, 21 Feb 2001 16:35:39 -0500

From: Preeti Sharma <preetis@lucent.com>

To: megaco@fore.com


 [Begin Correction]

B.2 ABNF syntax specification

...

;at most one of either serviceChangeAddress or serviceChangeMgcId but not both

serviceChangeParm    = (serviceChangeMethod / serviceChangeReason /

                       serviceChangeDelay / serviceChangeAddress /  

                       serviceChangeProfile / extension / TimeStamp /

                       serviceChangeMgcId / serviceChangeVersion )

serviceChangeReplyDescriptor = ServicesToken LBRKT

                       servChgReplyParm *(COMMA servChgReplyParm) RBRKT

;at-most-once. Version is REQUIRED on first ServiceChange response

;at most one of either serviceChangeAddress or serviceChangeMgcId but not both

servChgReplyParm     = (serviceChangeAddress / serviceChangeMgcId /

                       serviceChangeProfile / serviceChangeVersion )

...

 [End Correction]

6.73 ROOT termination not in ALL

	Description:
	When wildcarding a termination with ALL this does not address the root termination. This is not documented clearly in the specification.

	Reference:
	Subject: Re: ROOT not in ALL

Date: Tue, 20 Feb 2001 15:49:07 -0500

From: "Kevin Boyle" <kboyle@nortelnetworks.com>

Organization: Nortel Networks

To: Madhu Babu Brahmanapally <madhubabu@kenetec.com>

CC: "'Rosen, Brian'" <Brian.Rosen@marconi.com>, megaco <megaco@fore.com>


 [Begin Correction]

6.2.2 TerminationIDs

…

When ALL is used in the TerminationID of a command, the effect is identical to repeating the command with each of the matching TerminationIDs.  The use of ALL does not address the ROOT termination. Since each of these commands may generate a response, the size of the entire response may be large.  If individual responses are not required, a wildcard response may be requested.  In such a case, a single response is generated, which contains the UNION of all of the individual responses which otherwise would have been generated, with duplicate values suppressed.    For instance, given a Termination Ta with properties p1=a, p2=b and Termination Tb with properties p2=c, p3=d, a UNION response would consist of a wildcarded TerminationId and the sequence of properties p1=a, p2=b,c and p3=d.  Wildcard response may be particularly useful in the Audit commands.

…

[End Correction]

6.74 Case sensitivity of ABNF and text encoding
	Description:
	It is unclear whether or the ABNF and its text encoding is case sensistive. The ABNF is not case sensistive.

	Reference:
	Subject: Case insensitivity

Date: Tue, 20 Feb 2001 15:07:11 -0500

From: "Rosen, Brian" <Brian.Rosen@marconi.com>

To: "'megaco@fore.com'" <megaco@fore.com>


 [Begin Correction]

B.2 ABNF Syntax Specification

The protocol syntax is presented in ABNF according to RFC2234.

EVERYTHING IN THE ABNF AND TEXT ENCODING IS CASE INSENSITIVE. This includes TerminationIDs, digitmap Ids etc. THE SDP IS CASE SENSITIVE AS PER RFC2327.

…

 [End Correction]

6.75 Multiple Media Descriptor parameters

	Description:
	The ABNF says the following:

   ; at-most-once per item 

   ; and either streamParm or streamDescriptor but not both 

   mediaParm            = (streamParm / streamDescriptor / 

Someone interpreted the two comments to imply that you could only have

ONE streamParm.  This is not the intention, you can have one each

(one per item).

So you can say

Media{Local{..},LocalControl{...})

but you cannot say

Media{Local{..},Stream=1{LocalControl{..

	Reference:
	Subject: Media Descriptor Parameters

Date: Mon, 19 Feb 2001 16:41:17 –0500

From: "Rosen, Brian" <Brian.Rosen@marconi.com>

To: "'megaco@fore.com'" <megaco@fore.com>


 [Begin Correction]

7.1.4 Media Descriptor

…

As a convenience a LocalControl, Local, or Remote descriptors may be included in the Media Descriptor without an enclosing Stream descriptor.  In this case, the StreamID is assumed to be 1. 

…

[End Correction]
[Begin Correction]

B.2 ABNF Syntax Specification

…

; at-most-once per item

; and may include either streamParms or streamDescriptors but not both

mediaParm            = (streamParm / streamDescriptor / 

                        terminationStateDescriptor)

…

 [End Correction]

6.76 Provisional Timer Response Value

	Description:
	There is an inconsistency in package H.248 E.2 in that it has two values for execution time ie. normalMGExecutionTime and normalMGCExecutionTime. It however only has one value to represent the ProvisionalResponseTimerValue. This provisionalResponseTimerValue could in fact be set to two different times based upon the execution timers, one for the MGC and one for the MG. A second timer should be introducted for the provisionalResponseTime so that the MGC and MG can send timers at a different time.

	Reference:
	Subject: Re: Provisional Timer Response

Date: Tue, 27 Mar 2001 02:48:27 -0800 (PST)

From: Richie Wu <wuyongji@yahoo.com>

To: Christian Groves <Christian.Groves@ericsson.com>, megaco ietf <megaco@fore.com>


 [Begin Correction]

E.2.1 Properties

MGProvisionalResponseTimerValue

PropertyId: MGProvisionalResponseTimerValue (0x0005)

Indicates the time within which the MGC should expect a Pending Response from the MG if a Transaction cannot be completed.  Initially set to normalMGExecutionTime, plus network delay, but may be lowered.

Type: Integer

Possible Values: any integer, represents milliseconds

Defined in: TerminationState
Characteristics: read / write
MGCProvisionalResponseTimerValue

PropertyId: MGCProvisionalResponseTimerValue (0x0006)

Indicates the time within which the MG should expect a Pending Response from the MGC if a Transaction cannot be completed.  Initially set to normalMGCExecutionTime, plus network delay, but may be lowered.

Type: Integer

Possible Values: any integer, represents milliseconds

Defined in: TerminationState
Characteristics: read / write
…

[End Correction]

6.77 Typographical Error in Event Descriptor

	Description:
	There is a typographical error in section 7.1.9 a Notify is sent to the MGC not the MG.

	Reference:
	Editor


 [Begin Correction]

7.1.9 Events Descriptor

…

When an event is processed against the contents of an active Events descriptor and found to be present in that descriptor ("recognized"), the default action of the MG is to send a Notify command to the MGC.  Notification may be deferred if the event is absorbed into the current dial string of an active digit map (see section 7.1.14).  Any other action is for further study.  Moreover, event recognition may cause currently active signals to stop, or may cause the current Events and/or Signals descriptor to be replaced, as described at the end of this section.

…

[End Correction]

6.78 Clearing Contexts in ServiceChange

	Description:
	There is confusion as to the handling on subtraction of terminations from a context when a ServiceChange with "forced" indication is used. When a ServiceChange "forced" is issued on a non-Root termination the MGC is responsible for subsequently subtracting the termination from the applicable context. When a ServiceChange "forced" is issued on the Root termination it is assumed that all connection are lost on the MG and thus the MGC can consider that all the terminations are subtracted. 



	Reference:
	Subject: Re: Context Id in Service Change

Date: Wed, 04 Apr 2001 19:31:17 -0400

From: "Kevin Boyle" <kboyle@nortelnetworks.com>

Organization: Nortel Networks

To: "Rosen, Brian" <Brian.Rosen@marconi.com>

CC: "'Paul Rheaume'" <paul.rheaume@alcatel.com>, "'megaco@fore.com'" <megaco@fore.com>


 [Begin Correction]

7.2.8 ServiceChange

…

1) Forced – indicates that the specified Terminations were taken abruptly out of service and any established connections associated with them may be lost. For non-Root terminations the MGC is responsible for cleaning up the context (if any) with which the failed termination is associated.  At a minimum the termination shall be subtracted from the context.  The termination serviceState should be "out of service". For the root termination the MGC can assume that all connections are lost on the MG and thus can consider that all the terminations have been subtracted.
…

[End Correction]

6.79 Cancelling Event Detection

	Description:
	The ASN.1 can say that there are no events to be monitored.  The ABNF needs changing to align.

The request ID should be omitted if there are no events. This has the advantage of reducing message size.  It also follows the precedent set in section 7.1, which uses the descriptor token alone to denote an empty descriptor in transaction replies.

It should be spelt out in section 7.1.9 what an empty Events or EventBuffer Descriptor means, and what happens to buffered events

collected in LockStep mode when the new Events Descriptor is empty.  

	Reference:
	Subject: RE: [Fwd: RE: Does the First Line of SDP need a newline]

Date: Fri, 30 Mar 2001 09:23:30 –0500

From: "Rosen, Brian" <Brian.Rosen@marconi.com>

To: "'David Stonehouse'" <stonehouse@nortelnetworks.com>, "'Christian Groves'" <Christian.Groves@ericsson.com>

CC: megaco ietf <megaco@fore.com>, Tom-PT Taylor <taylor@nortelnetworks.com>


 [Begin Correction]

A.2 ASN.1 Syntax Specification

EventsDescriptor ::= SEQUENCE

{


requestID  

RequestID OPTIONAL,





-- RequestID must be present if eventList

-- is non empty

eventList  

SEQUENCE OF RequestedEvent,


...

}

…

SecondEventsDescriptor ::= SEQUENCE

{


requestID 


RequestID OPTIONAL,


eventList 


SEQUENCE OF SecondRequestedEvent,


...

}

…

 [End Correction]

 [Begin Correction]

B.2 ABNF Syntax Specification

…

eventsDescriptor     = EventsToken [ EQUAL RequestID LBRKT 

                       requestedEvent *( COMMA requestedEvent ) RBRKT ]
…

; at-most-once of each

embedFirst      = EventsToken [ EQUAL RequestID LBRKT

               secondRequestedEvent *(COMMA secondRequestedEvent) RBRKT ]
…

eventBufferDescriptor= EventBufferToken [ LBRKT eventSpec

                       *( COMMA eventSpec) RBRKT ] 

…

 [End Correction]

[Begin Correction]

7.1.9 Events Descriptor

The EventsDescriptor parameter contains a RequestIdentifier and a list of events that the Media Gateway is requested to detect and report.  The RequestIdentifier is used to correlate the request with the notifications that it may trigger.  Requested events include, for example, fax tones, continuity test results, and on-hook and off-hook transitions. The RequestIdentifier is omitted if the EventsDescriptor is empty (i.e. no events are specified).
…

An EventsDescriptor received by a media gateway replaces any previous Events Descriptor.  Event notification in process shall complete, and events detected after the command containing the new EventsDescriptor executes, shall be processed according to the new EventsDescriptor.

An empty Events Descriptor disables all event recognition and reporting.  An empty EventBuffer Descriptor clears the EventBuffer and disables all event accumulation in LockStep mode: the only events reported will be those occurring while an Events Descriptor is active.  If an empty Events Descriptor is activated while the termination is operating in LockStep mode, the events buffer is immediately cleared.
…

[End Correction]

6.80 Encoding of ABNF Value construct

	Description:
	It is unclear in the ABNF what may be included in the different ABNF values constructs. This correction gives guidance on the allowable fields.

	Reference:
	Subject:  [FINAL;)] Re: encoding package values as ANBF VALUES

Date: Thu, 26 Apr 2001 18:32:56 -0400

From: Troy Cauble <troy@lucent.com>

Reply-To: Troy Cauble <troy@bell-labs.com>

To: MEGACO list <megaco@fore.com>, Christian Groves <Christian.Groves@ericsson.com>


 [Begin Correction]

B.2 ABNF Syntax Specification

The protocol syntax is presented in ABNF according to RFC2234.


; NOTE -- The ABNF in this section uses the VALUE construct (or lists of

; VALUE constructs) to encode various package element values (properties,

; signal parameters, etc.).  The types of these values vary and are

; specified the relevant package definition.  Several such types are

; described in section 12.2.

;

; The ABNF specification for VALUE allows a quotedString form or a

; collection of SafeChars.  The encoding of package element values into

; ABNF VALUES is specified below.  If a type's encoding allows characters

; other than SafeChars, the quotedString form MUST be used for all values

; of that type, even for specific values that consist only of SafeChars.

;

; String:  A string MUST use the quotedString form of VALUE and can

; contain anything allowable in the quotedString form.

;

; Integer, Double, and Unsigned Integer:  Decimal values can be encoded

; using characters 0-9.  Hexadecimal values must be prefixed with '0x'

; and can use characters 0-9,a-f,A-F.  An octal format is not supported.

; Negative integers start with '-' and MUST be Decimal.  The SafeChar

; form of VALUE MUST be used.

;

; Character:  A UTF-8 encoding of a single letter surrounded by double

; quotes.

;

; Enumeration:  An enumeration MUST use the SafeChar form of VALUE

; and can contain anything allowable in the SafeChar form.

;

; Boolean:  Boolean values are encoded as "on" and "off" and are

; case insensitive.  The SafeChar form of VALUE MUST be used.

;

; Future types:  It is expected that packages will define types

; beyond these initial types.  Any defined types MUST fit within

; the ANBF specification of VALUE.  Specifically, if a type's encoding

; allows characters other than SafeChars, the quotedString form MUST

; be used for all values of that type, even for specific values that

; consist only of SafeChars.

;

; Note that there is no way to use the double quote character within

; a value.
…

[End Correction]

6.81 Alignment of Failover text between 7.2.8 and 11.5

	Description:
	There is discrepancy between the text with regards to Failover between sections 7.2.8 Service Change and 11.5 Failure of an MGC.

	Reference:
	Subject: RE: Reset of an MGC
Date: Wed, 11 Apr 2001 15:02:31 –0400

From: "Tom-PT Taylor" <taylor@nortelnetworks.com>

To: "'Ian Leighton'" <ian.leighton@alcatel.com>, megaco@fore.com


 [Begin Correction]

11.5 Failure of an MGC

If the MG detects a failure of its controlling MGC, it attempts to contact the next MGC on its pre-provisioned list.  It starts its attempts at the beginning (Primary MGC), unless that was the MGC that failed, in which case it starts at its first Secondary MGC.  It sends a ServiceChange message with a "Failover" method and a " MGC Impending Failure" reason. If the MG is unable to establish a control relationship with any MGC, it shall wait a random amount of time as described in section 9.2 and then start contacting its primary, and if necessary, its secondary MGCs again.

…

[End Correction]

[Begin Correction]

7.2.8 Service Change

6)
Failover – sent from MG to MGC to indicate the primary MG is out of service and a secondary MG is taking over. This serviceChange method is also sent from the MG to the MGC when the MG detects that MGC has failed.
…

A ServiceChange Command specifying the "Root" for the TerminationID and ServiceChangeMethod equal to Restart is a registration command by which a Media Gateway announces its existence to the Media Gateway Controller. The Media Gateway may also announce a registration command by specifying the "Root" for the TerminationID and ServiceChangeMethod equal to Failover when the MG detects MGC failures. The Media Gateway is expected to be provisioned with the name of one primary and optionally some number of alternate Media Gateway Controllers....

…

 [End Correction]

6.82 ALF not defined
	Description:
	The ALF acronym needs to be defined.

	Reference:
	Subject: ALF?

Date: Mon, 7 May 2001 09:08:44 -0400

From: "Rosen, Brian" <Brian.Rosen@marconi.com>

To: megaco@fore.com


 [Begin Correction]

4. ABBREVIATIONS

This recommendation defines the following terms.

ALF

Application Layer Framing
...

[End Correction]

[Begin Correction]

D.1 Transport over IP/UDP using Application Level Framing

Protocol messages defined in this document may be transmitted over UDP.  When no port is provided by the peer (see section 7.2.8), commands should be sent to the default port number, 2944 for text-encoded operation or 2945 for binary-encoded operation.  Responses must be sent to the address and port from which the corresponding commands were sent.

ALF is a set of techniques that allow an application, as opposed to a stack, to affect how messages are sent to the other side. A typical ALF technique is to allow an application to change the order of messages sent when there is a queue AFTER it has queued them.  There is no formal specification for ALF. The procedures in Annex D.1 contain a minimum suggested set of ALF behaviors 
...

[End Correction]

6.83 Source address
	Description:
	We need to be explicit about what a source address is.

	Reference:
	Subject: RE: From whence they came?

Date: Thu, 10 May 2001 14:49:30 -0400

From: "Tom-PT Taylor" <taylor@nortelnetworks.com>

To: megaco <megaco@fore.com>


 [Begin Correction]

9. TRANSPORT

...

As described in section 7.2.8, either the MG or the MGC may supply an address in the ServiceChangeAddress parameter to which subsequent transaction requests must be addressed, but responses (including the response to the initial ServiceChange request) must always be sent back to the address which was the source of the corresponding request. For example: in IP networks, this is the source address in the IP header and the source port number in the TCP/UDP/STCP header.
...

[End Correction]

6.84 Command, not Action
	Description:
	Cut-and-paste mistake most likely resulted in typographical error where “Actions” is specified rather than “Commands.”

	Reference:
	Subject: RE: Question regarding formulating transaction replies.

Date: Fri, 27 Apr 2001 20:32:46 -0500

From: Paul Long <plong@ipdialog.com>

To: "Megaco Mailing List \(E-mail\)" <megaco@fore.com>


 [Begin Correction]

8.2.2 TransactionReply

...

If the receiver encounters an error such that it cannot determine a legal Action, it will return a TransactionReply consisting of the TransactionID and a single error descriptor, 422 Syntax Error in Action. If the end of an action cannot be reliably determined but one or more Commands can be parsed, it will process them and then send 422 Syntax Error in Action as the last action for the transaction. If the receiver encounters an error such that is cannot determine a legal Transaction, it will return a TransactionReply with a null TransactionID and a single error descriptor (403 Syntax Error in Transaction).
...

[End Correction]

6.85 Message is a transport mechanism
	Description:
	Transactions are transported independently within messages.

	Reference:
	Subject: RE: Correspondence between requests and replies in messages

Date: Mon, 30 Apr 2001 12:53:42 -0700

From: "Tulpule, Naren" <naren@trillium.com>

To: <megaco@fore.com>


 [Begin Clarification]

8.3 Messages

...

The transactions in a message are treated independently.  There is no order implied; there is no application or protocol acknowledgement of a message. A message is essentially a transport mechanism.  For example, message X containing transaction requests A, B, and C may be responded to with message Y containing replies to A and C and message Z containing the reply to B. Likewise, message L containing request D and message M containing request E may be responded to with message  N containing replies to both D and E.
...

[End Clarification]

6.86 Interpretation of Value

	Description:
	The encoding of the information element "Value" is unclear in the ASN.1 and ABNF syntax specifications. This should be clarified.

	Reference:
	Subject: Re: [Fwd: RE: Interpretation of Value in ASN.1]

Date: Wed, 09 May 2001 11:35:53 –0400

From: Troy Cauble <troy@lucent.com>

Reply-To: Troy Cauble <troy@bell-labs.com>

To: Christian Groves <Christian.Groves@ericsson.com>


 [Begin Correction]

A.2 ASN.1 Syntax Specification

…

ServiceChangeParm ::= SEQUENCE

{


serviceChangeMethod  
ServiceChangeMethod,


serviceChangeAddress
ServiceChangeAddress OPTIONAL,


serviceChangeVersion
INTEGER(0..99) OPTIONAL,


serviceChangeProfile 
ServiceChangeProfile OPTIONAL,


serviceChangeReason  
Value,

-- A serviceChangeReason consists of a numeric reason code

-- and an optional text description.

-- The serviceChangeReason SHALL be a string consisting of 

-- a decimal reason code, optionally followed by a single 

-- space character and a textual description string.

-- This string is first ASN.1 BER encoded as an IA5String.

-- The result of this ASN.1 BER encoding is then encoded as

-- an ASN.1 BER OCTET STRING, "double wrapping" the value

-- as was done for package elements.

serviceChangeDelay   
INTEGER(0..4294967295) OPTIONAL,


                            -- 32 bit unsigned integer


serviceChangeMgcId   
MId OPTIONAL,


timeStamp


TimeNotation OPTIONAL,


nonStandardData

NonStandardData OPTIONAL,

...

}

…

[End Correction]

[Begin Correction]

B.2 ABNF Syntax Specification

…

serviceChangeMethod  = MethodToken EQUAL (FailoverToken / 

                       ForcedToken / GracefulToken / RestartToken / 

                       DisconnectedToken / HandOffToken / 

                       extensionParameter)

; A serviceChangeReason consists of a numeric reason code

; and an optional text description.

; A serviceChangeReason MUST be encoded using the quotedString 

; form of VALUE.

; The quotedString SHALL contain a decimal reason code,

; optionally followed by a single space character and a 

; textual description string.

serviceChangeReason  = ReasonToken  EQUAL VALUE

serviceChangeDelay   = DelayToken   EQUAL UINT32

…

[End Correction]

6.87 Digit Map ABNF error

	Description:
	eventDM = DigitMapToken (( EQUAL digitMapName ) / (LBRKT digitMapValue  RBRKT)) 

and a digit map descriptor is defined as: 

digitMapDescriptor = DigitMapToken EQUAL (( LBRKT 

 digitMapValue RBRKT ) / ( digitMapName [LBRKT digitMapValue RBRKT] )) 

As These structures are alike, the EQUAL in eventDM should be right after DigitMapToken as is the case in digitMapDescriptor.

	Reference:
	Subject: RE: ABNF Question: digit maps embedded in events

Date: Wed, 30 May 2001 17:27:14 –0400

From: "Tom-PT Taylor" <taylor@nortelnetworks.com>

To: "'Rosen, Brian'" <Brian.Rosen@marconi.com>, "'Steven Weisz'" <sweisz@mediatrix.com>, "Megaco Mailing List (E-mail)" <megaco@fore.com>


 [Begin Correction]

B.2 ABNF Syntax Specification

…

eventDM              = DigitMapToken EQUAL(( digitMapName ) / 

                       (LBRKT digitMapValue RBRKT )) 

…

[End Correction]

6.88 PkgdName Comments 

	Description:
	Annex A comments in PkgdName refers to "Property Name"

which is too specific (PkgdName is used for lots

of things.)  

Annex B similarly refers only to event in its comments.

	Reference:
	Subject:           ASN.1 PkgdName wildcard comments.

     Date:           Wed, 06 Jun 2001 16:38:57 -0400

    From:           Troy Cauble <troy@lucent.com>

       To:           Christian Groves <Christian.Groves@ericsson.com>, MEGACO list <megaco@fore.com>


[Begin Correction]

A.2 ASN.1 Syntax Specification

PkgdName ::= OCTET STRING(SIZE(4))

-- represents Package Name (2 octets) plus Property, Event,
-- Signal Names or Statistics ID.(2 octets)

-- To wildcard a package use 0xFFFF for first two octets, choose

-- is not allowed. To reference native property tag specified in

-- Annex C, use 0x0000 as first two octets.

-- To wildcard a Property, Event, Signal, or Statistics ID, use 

-- 0xFFFF for last two octets, choose is not allowed. 

-- Wildcarding of Package Name is permitted only if Property,
-- Event, Signal, or Statistics ID is also wildcarded.

[End Correction]

[Begin Correction]

B.2 ABNF Syntax Specification

pkgdName             = (PackageName SLASH ItemID) ;specific item




    / (PackageName SLASH "*") ;all items in package



    / ("*" SLASH "*") ; all items supported by the MG

[End Correction]

6.89 TimeStamp Optional in Notify ObservedEventsDescriptor

	Description:
	I believe that TimeStamp is intended to be optional in ObservedEvents Descriptor.  Both ABNF and ASN.1 have the element optional, but no where is this suggested in the text.  Section 7.1.17 and 7.2.7 both mention time as one of the contents of ObservedEvents

with no suggestion that it is optional.  If the intention was to make it optional, seems like the text should mention that.

Section 7.2.7 includes:

     Each event in the list is accompanied by parameters associated with

the event and an indication of the time that the event was detected.

which doesn't sound very optional and the statement in 7.1.17 includes it in a list where all other items are required.  If we agree it should be optional, I suggest clarifying this through the Implementors Guide.

	Reference:
	Subject:  TimeStamp Optional in Notify ObservedEventsDescriptor

Date: Tue, 19 Jun 2001 10:18:21 -0400

From: Terry L Anderson <tla@lucent.com>

Organization: Lucent Technologies

To:  Megaco Mailing List <megaco@fore.com>


[Begin Correction]

7.2.7 Notify

The ObservedEventsDescriptor contains the RequestID and a list of events that the Media Gateway detected in the order that they were detected. Each event in the list is accompanied by parameters associated with the event and optionally an indication of the time that the event was detected.  Procedures for sending Notify commands with RequestID equal to 0 are for further study.  

[End Correction]

[Begin Correction]

7.1.17 ObservedEvents Descriptor

ObservedEvents is supplied with the Notify command to inform the MGC of which event(s) were detected.  Used with the AuditValue command, the ObservedEventsDescriptor returns events in the event buffer which have not been Notified. ObservedEvents contains the RequestIdentifier of the EventsDescriptor that triggered the notification, the event(s) detected, optionally the detection time(s) and any parameters of the event. Detection times are reported with a precision of hundredths of a second.  Time is expressed in UTC.

[End Correction]

6.90 NotifyCompletion Reasons for Signals that Play-to-completion

	Description:
	NotifyCompletion reasons and g/sc{Meth=XX} codes are obviously paired.

TimeOut           -- TO Duration expired

IntByEvent       -- EV Interrupted by event

IntBySigDescr -- SD Halted by new Signals Descriptor

OtherReason    -- NC Not completed, other cause

What is not so clear is which of these is appropriate for the case of a

signal that "plays to completion" for reasons other than time, e.g., a

voice announcement file.  The use of OtherReason/NC does not seem

appropriate, since this is usually used for errors and it controdicts

the "not completed" text.  Such signals could be of type "brief" or time

"timeout".  I believe that TimeOut/TO is the most appropriate, in spite

of time not being the real cause and in private discussions, others

suggest that this was the original intent.

The choice of words for the reason is unfortunate - NormalCompletion

might have been clearer.  The "Duration expired" is more applicable than

the "TimeOut".  I suggest we add language to make it clear that this is

the correct reason.

	Reference:
	Subject: NotifyCompletion and g/sc Meth

Date: Wed, 20 Jun 2001 10:30:07 -0400

From: Terry L Anderson <tla@lucent.com>

Organization: Lucent Technologies

To: Megaco Mailing List <megaco@fore.com>


[Begin Correction]

7.1.11 Signals Descriptor

(paragraph 3)

Finally, the optional parameter "notifyCompletion" allows a MGC to indicate that it wishes to be  notified when the signal finishes playout.  The possible cases are that the signal timed out (or otherwise completed on its own), that it was interrupted by an event, that it was halted when a Signals Descriptor was replaced, or that it stopped or never started for other reasons.  If the notifyCompletion parameter is not included in a Signals Descriptor, notification is generated only if the signal stopped or was never started for other reasons.  For reporting to occur, the signal completion event (see section E.1.2) must be enabled in the currently active Events Descriptor.

[End Correction]

[Begin Correction]

E.1.2 Events

(Signal Completion/Termination Method)

Termination Method

ParameterID:  Meth (0x0002)

Indicates the means by which the signal terminated.
Type: enumeration

Possible values:

"TO" (0x0001) Signal timed out or otherwise completed on its own
"EV" (0x0002) Interrupted by event

"SD" (0x0003) Halted by new Signals Descriptor

"NC" (0x0004) Not completed, other cause

[End Correction]

6.91 Allowed Char Set for Identifier

	Description:
	In Sec 12.4, it says the following: 

Identifiers in text encoding shall be strings of up to 64 characters, containing no spaces, starting with an alphanumeric character and consisting of alphanumeric characters and / or digits, and possibly including the special character underscore ("_").

The first “alphanumeric” should be “alphabetic”?

	Reference:
	Subject: Allowed char set for Identifier

Date: Thu, 21 Jun 2001 18:24:22 +0530

From: "Sarju Garg" <sarju@bhartitelesoft.com>

To: "'megaco'" <megaco@fore.com>




[Begin Correction]

12.4 Identifiers

Identifiers in text encoding shall be strings of up to 64 characters, containing no spaces, starting with an alphabetic character and consisting of alphanumeric characters and / or digits, and possibly including the special character underscore ("_").

[End Correction]

6.92 Clarified use of ServiceChangeMethod for various cases of MG losing connection with MGC

	Description:
	Sections 7.2.8, 11.5 and D.1.5 are inconsistent and ambiguous about proper ServiceChangeMethod for cases where MG loses connection with MGC.  Intent is that MG should use "Failover" in attempts to contact next MGC on pre-provisioned list (if any) but

"Disconnected" if (possibly after trying others) one re-tries the original.  All three sections need minor changes.

	Reference:
	Subject: [Megaco] Failure of MGC

Date: Tue, 24 Jul 2001 16:08:51 +0530

From: vibansal@hss.hns.com

To: megaco@ietf.org


[Begin Correction]

7.2.8 Service Change 

(list of ServiceChangeMethods)

4) Disconnected – always applied with the Root TerminationID, indicates that the MG lost communication with the MGC, but it was subsequently restored to the same MGC (possibly after trying other MGCs on pre-provisioned list).  Since MG state may have changed, the MGC may wish to use the Audit command to resynchronize its state with the MG's.

6) Failover – sent from MG to MGC to indicate the primary MG is out of service and a secondary MG is taking over. This serviceChange method is also sent from the MG to the MGC when the MG detects that MGC has failed.

[End Correction]

[Begin Correction]

11.5 Failure of an MGC

If the MG detects a failure of its controlling MGC, it attempts to contact the next MGC on its pre-provisioned list.  It starts its attempts at the beginning (Primary MGC), unless that was the MGC that failed, in which case it starts at its first Secondary MGC.  It sends a ServiceChange message with a "Failover" method and a " MGC Impending Failure" reason. If the MG is unable to establish a control relationship with any MGC, it shall wait a random amount of time as described in section 9.2 and then start contacting its primary, and if necessary, its secondary MGCs again. When contacting its previously controlling MGC the MG sends the ServiceChange message with “Disconnected” method.
 [End Correction]

[Begin Correction]

D.1.5 Repeating Requests, Responses and Acknowledgements

(last paragraph)

It is, however, important that the maximum delay of retransmissions be bounded.  Prior to any retransmission, it is checked that the time elapsed since the sending of the initial datagram is no greater than T-MAX. If more than T-MAX time has elapsed, the MG concludes that the MGC has failed, and it begins its recovery process as described in section 11.5. If the MG retries the current MGC it shall use a ServiceChange with ServiceChangeMethod set to Disconnected so that the new MGC will be aware that the MG lost one or more transactions.  The value T-MAX is related to the LONG-TIMER value: the LONG-TIMER value is obtained by adding to T‑MAX the maximum propagation delay in the network.

[End Correction]

6.93 “Gateway” should read “MGC”

	Description:
	The following is from H.248 section 7.2.8: 

"The Media Gateway Controller may return an ServiceChangeMgcId parameter that describes the Media Gateway Controller that should preferably be contacted for further service by the Media Gateway. In this case the Media Gateway shall reissue the ServiceChange command to the new Media Gateway Controller. The **Gateway** specified in an ServiceChangeMgcId, if provided, shall be contacted before any further alternate MGCs. On a HandOff message from MGC to MG, the ServiceChangeMgcId is the new MGC that will take over from the current MGC."

Now, I think the word **Gateway** should really say MGC.

	Reference:
	Subject: [Megaco] Error in H.248 section 7.2.8

Date: Tue, 31 Jul 2001 17:18:01 +0100

From: "David Barr" <David.Barr@nortelnetworks.com>

To: "'megaco@ietf.org'" <megaco@ietf.org>


[Begin Correction]

7.2.8 ServiceChange

The Media Gateway Controller may return an ServiceChangeMgcId parameter that describes the Media Gateway Controller that should preferably be contacted for further service by the Media Gateway.  In this case the Media Gateway shall reissue the ServiceChange command to the new Media Gateway Controller.   The MGC specified in an ServiceChangeMgcId, if provided, shall be contacted before any further alternate MGCs.  On a HandOff message from MGC to MG, the ServiceChangeMgcId is the new MGC that will take over from the current MGC.

[End Correction]

6.94 Reference for Continuity Test Tone

	Description:
	E.10 Basic Continuity Package has no reference to a source for detail of the ct tone, the "appropriate response tone" in rsp for 2-wire circuits and other details of the test procedures.  I believe the correct reference is Q.724 Section 7 & 8.  I propose adding:

    Tones and test procedure details are given in Q.724 sections 7 and 8.

to the description in sec E.10 and the formal reference to Sec 2.1 Normative references through the Implementors Guide. Can we make only sections 7 & 8 normative?

Any objections?

	Reference:
	Subject: Continuity Test Package reference

Date:  Tue, 19 Jun 2001 11:31:31 –0400

From: Terry L Anderson tla@lucent.com
Organization: Lucent Technologies

To: Megaco Mailing List <megaco@fore.com>


[Begin Correction]

E.10.5 Procedures

(new paragraph)

(Informative Note) Example tones and test procedure details are given in Q.724 sections 7 and 8, Q.764 section 2.1.8 and Q.902.4.

[End Correction]

[Begin Correction]

2.2 Informative References

(add)

ITU-T Recommentation Q.724 (1998): “Signalling procedures”.
ITU-T Recommendation Q.764 (1999): “Signalling system No. 7 – ISDN user part signalling procedures”.
ITU-T Recommendation Q.1902.4 (2001) “Bearer independent call control protocol - Basic call procedures”.
[End Correction]

6.95 ServiceChangeMethod and ServiceChangeReason are required

	Description:
	This is another difference between the text and binary versions.  The ASN.1

clearly requires serviceChangeMethod and serviceChangeReason in each

ServiceChange command with all other parameters optional, but the ABNF simply

states (in comments added in IG 6.5 and 6.72) that each item can appear at most

once and only one of ServiceChangeAddress and ServiceChangeMgcId - no element

is required.

I am not sure that this difference was intentional but that is the current

state of the spec.  Do we want the ABNF to also require ServiceChangeMethod and

ServiceChangeReason?  The registration in the text-based example in A.1.1

contains a Method but not a Reason.

IPEK SARAC wrote:

> Hello,

>

> RFC3015 doesn't mention if ServiceChangeReason for ServiceChange command is

> mandatory or optional (it doesn't say optional so i take it as required).

> Megaco Call Flow Examples draft doesn't include a ServiceChangeReason in

> cold start scenario. Which one is true?

	Reference:
	Subject: Re: [Megaco] ServiceChangeReason parameter

Date: Wed, 01 Aug 2001 11:08:33 –0400

From: Terry L Anderson <tla@lucent.com>

Organization: Lucent Technologies

To: ipek.sarac@o2.com.tr
CC: megaco@ietf.org


[Begin Correction]

B.2 ABNF specification

…

;each parameter at-most-once

;at most one of either serviceChangeAddress or serviceChangeMgcId but not both

;serviceChangeMethod and serviceChangeReason are REQUIRED
serviceChangeParm    = (serviceChangeMethod / serviceChangeReason /

serviceChangeDelay / serviceChangeAddress /  

serviceChangeProfile / extension / TimeStamp /

serviceChangeMgcId / serviceChangeVersion )

[End Correction]

6.96 E.1.2 typo

	Description:
	In H.248 E.1.2, under "EventID: sc", there is a reference: "For further procedural description, see 7.1.1, 7.1.17 and 7.2.7."

The 7.1.1 should say 7.1.11. 

Note, RFC3015 does not have this problem.

RFC 2885 and prepublication version of H.248 did not have this error either, it is only in the final published version of H.248.

	Reference:
	Subject: [Megaco] H.248 E.1.2 typo

Date: Mon, 20 Aug 2001 16:37:02 +0100

From: "David Barr" <David.Barr@nortelnetworks.com>

To: "'megaco@ietf.org'" <megaco@ietf.org>




[Begin Correction]

E.2.1

EventID: sc (0x0002)


Indicates termination of one or more signals for which the notifyCompletion parameter was set to "ON". For further procedural description, see 7.1.11, 7.1.17 and 7.2.7.

[End Correction]

6.97 V4hex range typo in ABNF Specification (IPV4address)

	Description:
	Can someone please identify whether the RFC3015 text encoding of the IPV4address

(format nnn.nnn.nnn.nnn   where the range of nnn is 0-225) is correct or is the

225  a typo error.

These nnn values are 8 bit address segments so shouldn't the range be 0-255.

	Reference:
	Subject: [Megaco] IPV4 Address range

Date: Tue, 4 Sep 2001 13:06:47 +0100

From: "Andrew Russell-laing" <Andrew.Russell-laing@marconi.com>

To: megaco@ietf.org




[Begin Correction]

B.2 ABNF specification

IPv4address          = V4hex DOT V4hex DOT V4hex DOT V4hex

V4hex                = 1*3(DIGIT) ; "0".."255"

[End Correction]

6.98 Only One Media Description Allowed in SDP

	Description:
	SDP (RFC 2327) supports multimedia sessions, that is 

Sessions containing multiple Media.  

…

I would like to see an IG clarification that within H.248, 

SDP Session descriptions MUST NOT contain multiple Media 

descriptions, since an H.248 Stream Descriptor represents 

a single media stream.

	Reference:
	Subject: [Megaco] Multi Media SDP

Date: Thu, 06 Sep 2001 18:28:28 -0400

From: Troy Cauble <troy@lucent.com>

To: MEGACO list <megaco@ietf.org>


[Begin Correction]

7.1.8 Local and Remote Descriptors

When text encoding the protocol, the descriptors consist of session descriptions as defined in SDP (RFC 2327). In session descriptions sent from the MGC to the MG, the following exceptions to the syntax of RFC 2327 are allowed:

•
the "s=", "t=" and "o=" lines are optional;

•
the use of CHOOSE is allowed in place of a single parameter value; and

•
the use of alternatives is allowed in place of a single parameter value.

A Stream Descriptor specifies a single bi-directional media stream and so a single session description MUST NOT include more than one media description ("m=" line).  Any additional media streams for a termination must appear in distinct Stream Descriptors.  When multiple session descriptions are provided in one descriptor, the "v=" lines are required as delimiters; otherwise they are optional in session descriptions sent to the MG. Implementations shall accept session descriptions that are fully conformant to RFC 2327. When binary encoding the protocol the descriptor consists of groups of properties (tag-value pairs) as specified in Annex C. Each such group may contain the parameters of a session description.

[End Correction]

6.99 Specify error code when Topology not supported

	Description:
	You should get an error if you send a topology descriptor

and the MG doesn't implement topology.  We should agree on

which error: I propose 444 Unsupported or Unknown Descriptor

It would be possible to define a package - TopologySupport,

which had a single property - TopologySupported.  You could

then audit Root to find out.  Maybe we could generalize that

a bit.

	Reference:
	From: Rosen, Brian [mailto:Brian.Rosen@marconi.com]

Sent: Monday, August 13, 2001 1:36 PM

To: 'Dan Elbert'

Subject: RE: [Megaco] Topology support and use


[Begin Correction]

7.1.18
Topology descriptor

A Topology descriptor is used to specify flow directions between Terminations in a Context. Contrary to the descriptors in previous subclauses, the Topology descriptor applies to a Context instead of a Termination. The default topology of a Context is that each Termination's transmission is received by all other Terminations. The Topology descriptor is optional to implement. An MG that does not support Topology descriptors, but receives a command containing one, returns error “444 Unsupported or unknown descriptor”and includes a string containing the name of the unsupported Descriptor (“Topology”) in the error text in the error descriptor.

[End Correction]

6.100 State in Annex A and B that syntax does not enforce all restrictions

	Description:
	Agreed, it can't be mentioned for each such case. But a generic statement

can be included at the start of Annex A and B, stating that " The grammar

allows many possible combination of descriptors/properties.  The protocol

text defines the valid descriptors/parameters for each command. In case of

ambiguity between the grammar and protocol text, the protocol text

prevails." Might be the intention is implicit, but making this explicit

helps.

	Reference:
	Subject:  RE: [Megaco] Difference in RFC and grammar

Date: Thu, 23 Aug 2001 12:11:12 -0400

From: "Madhu Babu Brahmanapally" <madhubabu@kenetec.com>

To: "'Terry L Anderson'" <tla@lucent.com>

CC: "'Rosen, Brian'" <Brian.Rosen@marconi.com>,

         "'Prasad Nayak'" <PrasadN@netbrahma.com>, <megaco@ietf.org>


[Begin Correction]

A.2
ASN.1 syntax specification
Note 3 – This syntax specification does not enforce all restrictions on element inclusions and values.  Some additional restrictions are stated in comments and other restrictions appear in the text of this recommendation.  These additional restrictions are part of the protocol even though not enforced by this specification.
[End Correction]

[Begin Correction]

B.2
ABNF specification
Note 1 – This syntax specification does not enforce all restrictions on element inclusions and values.  Some additional restrictions are stated in comments and other restrictions appear in the text of this recommendation.  These additional restrictions are part of the protocol even though not enforced by this specification.

[End Correction]

6.101 When does Subtract Return Statistics

	Description:
	I guess this is a little ambiguous.  I am not quite sure why Statistics was

made "default" but I would assume that if an Audit descriptor is included then

ONLY those items specified would be returned (e.g., Audit{Media} would return

only media descriptors and NOT statistics) and that the "default" would only

occur if there was no Audit descriptor.  So Audit{} would turn off all audit,

Audit{Media} would return Media and omitting Audit would return Statistics.

Does anyone disagree?  Do folks think this needs to be clarified in IG?

Sean Leahy wrote:

> Re: Your statement that:

>

> "...... to suppress the default one must send an empty Audit descriptor

> Audit{}.".

>

> Would it not be useful to be able to suppress statistics whilst still being

> able to ask for other information e.g. Media descriptors ?

>

> Sean Leahy

	Reference:
	Subject: Re: [Megaco] H248 Subtract

Date: Fri, 05 Oct 2001 12:17:40 -0400

From: Terry L Anderson <tla@lucent.com>

To: Sean Leahy <Sean.Leahy@marconi.com>

CC: megaco@ietf.org


[Begin Correction]

7.2.3 Subtract

The AuditDescriptor is optional. If present, the command will return only those descriptors  specified in the AuditDescriptor, which may be empty. If omitted, the Statistics descriptor is returned, by default.  Possible return values are the same as those to Add.
[End Correction]

6.102 Clarify the definition of Signal

	Description:
	… I think there is an inconsistency in the core protocol about the definition of a signal:

section 6.2: signals are "MG generated streams"

section 6.2.4: SignalsDescriptors "describes signals and/or actions to be

applied to the Terminations". 

section 7.1.11: "SignalsDescriptor is a parameter that contains the set of

signals that the MG is asked to apply to a Termination". What about actions?

section 7.1.9 (for instance..): "recognition of an event shall cause any

active signals to stop". Again, what about actions?

So:

- according to 6.2 and 6.2.4, signals are "MG generated streams" only

- according to 7.1.11 and 7.1.9, signals are what is contained in a

SignalDescriptor, i.e. "MG generated streams" + actions (according to

6.2.4).

I think this should be fixed in the IG. I propose:

- modifying section 6.2 the following way: "Signals are MG generated streams

(such as tones and announcements), line signals such as hookswitch, and/or

actions."

- modifying section 6.2.4: "Signals: describes signals (see 6.2) applied to

the Terminations."

[editor’s note: in further discussion “actions” was deemed too general.]

	Reference:
	Subject: [Megaco] Inconsistency in signals definition

Date: Thu, 6 Sep 2001 11:55:26 +0200

From: CHAPRON Jean-Emmanuel FTRD/DAC/ISS <jeanemmanuel.chapron@rd.francetelecom.com>

To: MEGACO@ietf.org


[Begin Correction]

6.2 Terminations

Terminations may have signals applied to them (see 7.1.11). Terminations may be programmed to detect Events, the occurrence of which can trigger notification messages to the MGC, or action by the MG. Statistics may be accumulated on a Termination. Statistics are reported to the MGC upon request (by means of the AuditValue command, see 7.2.5) and when the Termination is taken out of the call it is in.

[End Correction]

[Begin Correction]

6.2.4 Termination Properties and Descriptors

(table)

	Signals
	Describes signals (see 7.1.11) applied to Terminations


[End Correction]

[Begin Correction]

7.1.11
Signals descriptor

Signals are MG generated media streams such as tones and announcements as well as line signals such as hookswitch. More complex signals may include a sequence of such simple signals interspersed with and conditioned upon the receipt and analysis of received media data and bearer-related signals (such as echoing of received data).
A SignalsDescriptor is a parameter that contains the set of signals that the Media Gateway is asked to apply to a Termination. A SignalsDescriptor contains a number of signals and/or sequential signal lists. A SignalsDescriptor may contain zero signals and sequential signal lists. Support of sequential signal lists is optional.

[End Correction]

6.103  Specification of Root Termination and associated Error

	Description:
	Currently the text of section 6.2.5 indicates the cases where the use of the root termination is correct. It then states that any other use is an error. However the error to use is not specified. An appropriate error code to be returned is error 410 – Incorrect Identifier.

	Reference:
	AVD-2127 contribution to the Dublin 2001 Rapporteur’s Meeting.


 [Begin Correction]

6.2.5 Root Termination

…

· a ServiceChange - to declare the gateway in or out of service  

Any other use of the root TerminationID is an error. Error code 410 – Incorrect identifier shall be returned in these cases.
 [End Correction]

6.104 Update to IG 6.46 Encoding of ASN.1 Octet String

	Description:
	IG 6.46 specifies the how package types are double wrapped in octet strings. However it does not specify how Octet (Strings) are encoded. This should be added.

	Reference:
	AVD-2127 contribution to the Dublin 2001 Rapporteur’s Meeting.


 [Begin Correction]

A.2 ASN.1 Syntax Specification

…

	Unsigned Integer (Note 2)
	INTEGER (Note 3)

	Octet (String)
	OCTET STRING

	Note 1 : Can be more than one byte

	Note 2 : Unsigned integer is referenced in Annex C

	Note 3 : ASN.1 BER encoding of INTEGER does not imply the use of 4 bytes.


…

[End Correction]

6.105 Annex C encoding

	Description:
	Annex C presents a number of properties that indicate the encoding as N bits, M Octets or N bit integer. This is not a consistent type with the types indicated for double wrapping in the protocol.

	Reference:
	AVD-2127 contribution to the Dublin 2001 Rapporteur’s Meeting.


 [Begin Correction]

Annex C Tags for Media Stream Properties

…

For type "enumeration" the value is represented by the value in brackets, e.g., Send(0), Receive(1). Annex C properties with the types "N bits" or "M Octets" should be treated as Octet Strings when encoding the protocol. Properties with "N bit integer" shall be treated as an integers. "String" shall be treated as an IA5String when encoding the protocol.
…

[End Correction]

6.106 Encoding of VPCI/VCI in Annex C.4

	Description:
	H.248 Annex C.4 indicates that the VPVC value is 2 integers. This becomes problematic to encode using ASN.1 as two integer values would have to be double wrapped. It is consistent with the VPCI/VCI values in Q.2931 that these values are encoded as 4 octets. The 1st 2 OCTETS as the VPCI and the 2nd 2 OCTETS as the VCI.

	Reference:
	AVD-2127 contribution to the Dublin 2001 Rapporteur’s Meeting.


 [Begin Correction]

C.4 General ATM properties

	PropertyID
	Property

Tag 
	Type
	Value

	AESA
	4001
	20 OCTETS
	ATM End System Address

	VPVC
	4002
	4 OCTETS, 
1st Least Significant 2 OCTETS VPCI, 2nd 2 OCTETS VCI
	VPCI/VCI.  Ref. : ITU Rec. Q.2931


…

[End Correction]
6.107 Missing extension marker in ASN.1 for TopologyRequest

	Description:
	Note: The topology descriptor doesn’t currently includes “...”, which seems to be an omission as most of the other descriptors do. Therefore, adding the streamed field will affect backward compatibility. We propose that a note should be added to the implementor’s guide to the effect that the topology descriptor should include the “...” .

	Reference:
	AVD-2115 contribution to the Dublin 2001 Rapporteur’s Meeting.


[Begin Correction]

TopologyRequest ::= SEQUENCE

{


terminationFrom      
TerminationID,


terminationTo      

TerminationID,


topologyDirection 

ENUMERATED


{



bothway(0),



isolate(1),



oneway(2)


},


…
}

[End Correction]

6.108
ASN.1 Profile Name Amendment

	Description:
	Currently the ASN.1 syntax allows the Profile Name to be 2 octets, thus meaning a profile name must defined in a binary manner. The text version of H.248 allows a textual name to be specified. This is an inconsistency and in order to simplify IANA registration procedures it is proposed to update the ASN.1 profile name to be an IA5string of 67 characters (64 for name, 1 for / and 2 for version).

	Reference:
	AVD-2122 contribution to the Dublin 2001 Rapporteur’s Meeting.


 [Begin Correction]

…

ServiceChangeProfile ::= SEQUENCE

{


profileName 
IA5String(SIZE (1..67)),


-- 64 for name, 1 for /, 2 for version to match ABNF

}

…

[End Correction]

6.109 Clarify property types

	Description:
	The "Octet String" type (not the ASN.1 storage unit) is used

in Annex C (and addressed in AVC-2127), but not listed in

12.1.2 or 12.2.  It should be.

The sub-list type needs to specify the item type of the list.

The IG item 6.80 reference to “future types” is too vague.


[Begin Correction]

12.1.2
Properties

Properties defined by the package, specifying:


Property Name: only descriptive


PropertyID: is an identifier


Description:


Type: One of:
Boolean


String: UTF-8 string
Octet String: Octet String: A number of octets.  See Annex A and B.3 for encoding


Integer: 4 byte signed integer



Double: 8 byte signed integer


Character: unicode UTF-8 encoding of a single letter. Could be more than one octet.



Enumeration: one of a list of possible unique values (see 12.3)

Sub-list: a list of several values from a list. The type of sub-list should also be specified in the package chosen from the types specified in this section with the exception of sub-list, e.g., Type: sub –list of enumeration.  The encoding of sub-lists is specified in Annexes 
A and B.



[End Correction]

[Begin Correction]

12.2
Guidelines to defining Properties, Statistics and Parameters to Events and Signals

Parameter Name: only descriptive


ParameterID: is an identifier

Type: One of:

Boolean

String: UTF-8 octet string

Octet String: Octet String: A number of octets.  See Annex A and B.3 for encoding

Integer: 4-octet signed integer


Double: 8-octet signed integer

Character: unicode UTF-8 encoding of a single letter. Could be more than one octet.


Enumeration: one of a list of possible unique values (see 12.3)

Sub-list: a list of several values from a list (not supported for statistics). The type of sub-list should also be specified in the package chosen from the types specified in this section with the exception of sub-list, e.g., Type: sub –list of enumeration.  The encoding of sub-lists is specified in Annexes


[End Correction]

[Begin Correction]

Annex B (as modified by IG 6.80)


Any defined types MUST fit within

; the ANBF specification of VALUE.  Specifically, if a type's encoding

; allows characters other than SafeChars, the quotedString form MUST

; be used for all values of that type, even for specific values that

; consist only of SafeChars. 

[End Correction]

7 Technical and Editorial Corrections to H.248 Annex F(2000)

7.1 Package ID of Text Telephone Package in Annex F shall be 0x0010

	Description:
	The numeric ID of the Text Telephone package in Section 7 of H.248 Annex F shall be changed to 0x0010 to match the IANA registration.


 [Begin Correction]

F.7
Text Telephone package

PackageID: 
txp (0x0010)

…

[End Correction]

7.2 Value of NAK

	Description:
	The numeric value of NAK shall be 0x000D, in the V8bistype parameter of the dtone event in the Call Type Discrimination package.


 [Begin Correction]

F.8.2.1
Discriminating tone detected

EventID: 

dtone (0x0001)




...

ObservedEventDescriptor parameters:

....

DiscriminatingToneValue

ParameterID:
dtvalue (0x0002)




....

V8bistype 

ParameterID:
v8bist (0x0004)

Type:

enumeration

Possible values:




ESi
(0x0001)
V.8bis signal ESi




ESr
(0x0002)
V.8bis signal ESr


MRe
(0x0003)
V.8bis signal MRe


MRdi
(0x0004)
V.8bis signal MRd from initiator


MRdr
(0x0005)
V.8bis signal MRd from responder




CRe
(0x0006)
V.8bis signal CRe


CRdi
(0x0007)
V.8bis signal CRd from initiator


CRdr
(0x0008)
V.8bis signal CRd from responder


MS
(0x0009)
V.8bis message MS with contents in "dtvalue"


CL
(0x000A)
V.8bis message CL with contents in "dtvalue"


CLR
(0x000B)
V.8bis message CLR with contents in "dtvalue"


ACK
(0x000C)
V.8bis message ACK with contents in "dtvalue"


NAK
(0x000D)
V.8bis message NAK with contents in "dtvalue"

…

[End Correction]

7.3 Correction in parameter values in Call Type Discrimination package in Annex F

	Description:
	Correction of conflicting parameter values for MRdrh, MRdrl and CReh in the V8bsn parameter of the V8bisSignal signal in the Call Type Discrimination package.


 [Begin Correction]

F.8.3.4
V8bisSignal

SignalID:
v8bs (0x0004)

Signaltype:
BR

Parameters:


V8bisSigname 

ParameterID:
V8bsn (0x0001)

Type:

Enumeration

Possible values:




ESi
(0x0001)
V.8bis signal ESi




ESr
(0x0002)
V.8bis signal ESr




MRe
(0x0003)
V.8bis signal MRe




MRdi
(0x0004)
V.8bis signal MRd from initiator





MRdrl
(0x0005)
V.8bis signal MRd from responder on low power





CRel
(0x0006)
V.8bis signal CRe on low power




CRdi
(0x0007)
V.8bis signal CRd from initiator




CRdr
(0x0008)
V.8bis signal CRd from responder




MS
(0x0009)
V.8bis message MS with contents in signalvalue




CL
(0x000A)
V.8bis message CL with contents in signalvalue




CLR
(0x000B)
V.8bis message CLR with contents in signalvalue




ACK
(0x000C)
V.8bis message ACK with contents in signalvalue




NAK
(0x000D)
V.8bis message NAK with contents in signalvalue




MRdrh
(0x000E)
V.8bis signal MRd from responder on high power




CReh
(0x000F)
V.8bis signal CRe on high power





Default may be provisioned

…

[End Correction]

7.4 Correction in parameter values in Call Type Discrimination package in Annex F

	Description:
	Correction of conflicting parameter values for dtt parameter in  dtone event. in the Call Type Discrimination package.


 [Begin Correction]

F.8.2.1
Discriminating tone detected

EventID: 

dtone (0x0001)

Description:

This event indicates that a signal valid for detection and discrimination of mode was detected. The signal name is given as a parameter. Further logic is needed in some cases to discriminate the call type from this information. The V.8bis related parameters are returned only when V.8bis is supported [5]. 

Note that some textphones operate with DTMF tones. This package decodes initial DTMF signals according to the specification for text telephones in V.18 [6]. DTMF detection may be indicated also from the "dd" package if that is active.

EventsDescriptor parameters:


none

ObservedEventDescriptor parameters:

DiscriminatingToneType

ParameterID:
dtt (0x0001)

Type: 

Enumeration

Possible values:

For FAX


CNG

(0x0001)
a T.30 fax calling tone 

V21flag 

(0x0002)
V21 tone and flags for fax answering

For TEXT

XCI

(0x0003)
a V.18 XCI

V18txp1

(0x0004)
a V.18 txp signal in channel V.21(1)

V18txp2

(0x0005)
a V.18 txp signal in channel V.21(2)

BellHi

(0x0006)
a Bell 103 carrier on the high




channel

BellLo

(0x0007)
a Bell 103 low channel

Baudot45
(0x0008)
a Baudot45 initial carrier and 


characters 

Baudot50
(0x0009)
a Baudot50 initial carrier and




characters 

Edt 

(0x000A)
an EDT initial tone and characters

DTMF

(0x000B)
DTMF signals

For DATA

Sig

(0x000C)
Modulation signal from a mode




only used for data, i.e. not 




V.21, V.23 nor Bell 103 


Common to TEXT and DATA:

CT

(0x000D)
a V.25 calling tone

V21hi

(0x000E)
a V.21 carrier on the higher




frequency channel

V21lo

(0x000F)
a V.21 carrier on the low




frequency channel

V23hi

(0x0010)
a V.23 high carrier

V23lo

(0x0011)
a V.23 low carrier

CI

(0x0012)
a V.8 CI with contents in




"dtvalue"

Common to FAX, TEXT and DATA:

ANS

(0x0013)
V.25 ANS, equivalent to T.30




CED from answering terminal

ANSbar

(0x0014)
V.25 ANS with phase reversals

ANSAM
(0x0015)
V.8 ANSam

ANSAMbar
(0x0016)
V.8 ANSam with phase reversals

CM

(0x0017)
V.8 CM with contents in




"dtvalue"

CJ

(0x0018)
V.8 CJ

JM

(0x0019)
V.8 JM with contents in




"dtvalue"

ENDOFSIG
(0x001A)
End of reported signal detected




reported for continuous or repeated




signals

V8BIS

(0x001B)
V.8bis signal, with signal type in




parameter V8bistype and value in




"dtvalue"

…

[End Correction]

7.5 Missing Keywords in F.8.1.2

	Description:
	F.8.1.2 neglects to specify "Defined in:" or "Characteristics:"

	Reference:
	Subject: Re: Annex F typos

Date: Wed, 02 May 2001 16:06:27 +1000

From: Christian Groves <Christian.Groves@ericsson.com>

To: Troy Cauble <troy@bell-labs.com>

CC: gunnar.hellstrom@era.ericsson.se, gparsons@nortelnetworks.com, jraff@brooktrout.com, rspitzer@telogy.com,MEGACO list <megaco@fore.com>


 [Begin Correction]

F.8.1.2 Text Call Types


…

V18

(0x0008)

Description: 

This parameter indicates for what text telephone modes the termination is monitored, used in TEXT mode.


Defined in: Termination State


Characteristics: Read / Write

…

[End Correction]

7.6 Duplicated propertyID in F.8.1

	Description:
	> F.8.1.3 and F.8.1.6 have the same PropertyID string (v8bsup).

[CHG] Yes. The authors can specify an appropriate name.

	Reference:
	Subject: Re: Annex F typos

Date: Wed, 02 May 2001 16:06:27 +1000

From: Christian Groves <Christian.Groves@ericsson.com>

To: Troy Cauble <troy@bell-labs.com>

CC: gunnar.hellstrom@era.ericsson.se, gparsons@nortelnetworks.com, jraff@brooktrout.com, rspitzer@telogy.com,MEGACO list <megaco@fore.com>


 [Begin Correction]

F.8.1.6 PhasereversalDetect


PropertyID:

phrevdet (0x0006)


Type: 


Boolean
…

[End Correction]

7.7 Inconsistencies in Fax Transport property F.9.1

	Description:
	F.9.1.1 and F.9.1.2 have the same PropertyID number (0x01). F.9.1.2 updated.

There is also a spurious dot in one of the value names.

	Reference:
	Subject: Re: Annex F typos

Date: Wed, 02 May 2001 16:06:27 +1000

From: Christian Groves <Christian.Groves@ericsson.com>

To: Troy Cauble <troy@bell-labs.com>

CC: gunnar.hellstrom@era.ericsson.se, gparsons@nortelnetworks.com, jraff@brooktrout.com, rspitzer@telogy.com,MEGACO list <megaco@fore.com>


 [Begin Correction]

F.9.1.2 Fax Transport

PropertyID: 

ftrpt (0x0004)

Type: 


Enumeration

Possible values: 


T30

(0x0001) 
for T.30 PSTN sessions without ECM


T30ECM
(0x0002)
for T.30 PSTN sessions with ECM (non-V.34)


T30V34
(0x0003) 
for T.30 PSTN sessions with V.34 (half-duplex)

[End Correction]

7.8 Duplicated PropertyID in F.10.1

	Description:
	F.10.1.1 and F.10.1.2 have the same PropertyID number (0x01). F.10.1.2 to be updated.

	Reference:
	Subject: Re: Annex F typos

Date: Wed, 02 May 2001 16:06:27 +1000

From: Christian Groves <Christian.Groves@ericsson.com>

To: Troy Cauble <troy@bell-labs.com>

CC: gunnar.hellstrom@era.ericsson.se, gparsons@nortelnetworks.com, jraff@brooktrout.com, rspitzer@telogy.com,MEGACO list <megaco@fore.com>


 [Begin Correction]

F.10.1.2 IPFaxTransport

PropertyID: 

ipftrpt (0x0007)

Type: 


Enumeration
…

[End Correction]

8 Technical and Editorial Corrections to H.248 Annex G

8.1 Correct Binary PropertyIDs in Soft Key Package

	Description:
	In the Soft Key Package (ks), Annex G H.248,  the Property ID of Propery

- nskeys ( Number of softkeys ) is given as 1. This package extends the Label Key package

(labelkey) whose Property – keylist ( Key List ) also has the Property ID 1. Is the clash of the property IDs an oversight?

	Reference:
	Subject: [Megaco] Regarding Soft Key Package

Date: Thu, 26 Jul 2001 16:49:50 -0700

From: "Anand, Rashim" <r_anand@trillium.com>

To: "'megaco@ietf.org'" <megaco@ietf.org>


[Begin Correction]

Soft Key Package

PackageID: ks, 0x001a

Properties

Number of softkeys

PropertyID: nskeys (0x0002)

PropertyType: Integer

Characteristics: read only

Defined in: TerminationState 

Description: Maximum number of individual soft keys. 

Display size

PropertyID: sz (0x0003)

Type: Integer

Characteristics: read only

Defined in: TerminationState 

Description: Maximum number of characters that can be displayed in each softkey. 

Supported unicode code pages

PropertyID: cdpgs (0x0004)

Description: a list of supported unicode pages 

Type: list of enumerated type

Defined in: TerminationState. 

Characteristics: read only

[End Correction]

8.2 Correct Binary DigitMap Completion EventID in Keypad Package

	Description:
	The Event ID ce (0x0001) of Keypad Package, which is derived from Key

Package clashes with the Event ID kd (0x0001) of Key Package. What should be the

Correct ID for the ce Event?

	Reference:
	From: Anand, Rashim [mailto:r_anand@trillium.com]

Sent: August 30, 2001 18:41

To: 'megaco@ietf.org'

Subject: [Megaco] Regarding Annex G


[Begin Correction]

Keypad Package

Events

DigitMap Completion Event

EventID: ce (0x0003)

Generated when a digit map completes as described in Megaco/H.248 Protocol section 7.1.14.  Form of this event is identical to its definition in DTMF Detection Package (dd), Megaco/H.248 Protocol section E.6.2.  
[End Correction]

9 Technical and Editorial Corrections to H.248 Annex H

9.1 SCTP Streams

	Description:
	In chapter H.8 Stream Independence within Annex H it reads:

"SCTP can provide up to 65536 unidirectional streams ... "

this is correct there can be 65536 unique stream numbers (0-65535). Though the number of streams is limited to what is specified in the INIT / INITACK.  There, according to the SCTP RFC 2960 variables:

Number of Outbound Streams

Number of Inbound Streams

are represented by a 16 bit variables where the value of 0 (zero streams) is not allowed.  Hence the actual number of streams which may ever be requested and accepted is 0xFFFF (65535).

Therefore, the 65536 value in chapter H.8 in Annex H should be 65535.



	Reference:
	Editor


 [Begin Correction]

H.8 Stream Independence

SCTP can provide up to 65535 unidirectional streams in each direction of an MGC-MG association. SCTP transmits messages and processes received messages in one stream independent to the order or status of messages in any other streams. H.248 may avoid head-of-line blocking by transmitting unrelated transactions on different streams. Reliability is still provided. Ordering of messages is available per-stream.

[End Correction]

10 Technical and Editorial Corrections to H.248 Annex K

10.1 Superflous information

	Description:
	The Annoucement Package Annex K contains fields which are superflous and may lead to misinterpretation. The Notifycompletion indicator is a core H.248 element and does not need to be specified in a package. Also Signal type only needs one element  ie. TO. Several signal do not need to be specified as these may be overridden by the core protocol.

	Reference:
	Editor


 [Begin Correction]

K.3 Signals

…

SignalID: apf (0x0001)

Description: Initiates the play of a fixed announcement.

SignalType: TO (default)

SignalDuration: Provisioned.


…

SignalID: apv (0x0002)

Description: Initiates the play of a variable announcement.

SignalType: TO (default)

SignalDuration: Provisioned.

 
[End Correction]

10.2 Announcement Playing Clarification

	Description:
	H.248 Annex K defines a package for the support of Announcements. It defines that announcements should have a certain signal type. It also defines that signals have a duration and number of cycles. Signal Type, Duration and Number of Cycles all interact when determining how to play an announcement. It becomes even more complicated when using defaults for these values or the default values are overridden. The rules for determining how to play an announcement are quite confusing. Thus it is proposed to add a table to H.248 Annex K to show the possible combinations and how these relate to how the announcement is played.

H.248 Annex K also specifies Variable announcement playing. It mentions that additional data may be sent for a variable announcement however it does not say how this interacts with keepactive indication on a signal. It is proposed that this behaviour is clarified.

	Reference:
	AVD-2126 contribution to the Dublin 2001 Rapporteur’s Meeting.


 [Begin Correction]

K.5 Procedures

…

If the signal duration is 0, the signal is played specified by the noc parameter only or when not included in the signal by the announcements number of cycle default.  

If the parameter noc is 0, the signal is played specified by the signal duration only or when not included in the signal by the corresponding announcement default.

To provide additional information when an announcement is to be played, the MGC sends a play variable announcement signal to the MG.  H.248 doesn’t guarantee in sequence processing of transactions. To ensure sequential playing of an announcement, a transaction reply for a command that affects the announcement signal should be received by the Media Gateway Controller before it sends the additional variable announcement data. If the Media Gateway receives a signal with the keep active flag with additional variable announcement data for an already playing announcement it shall continue playing the announcement according to the additional data. 

…

If the signal duration is 0, the signal is played specified by the noc parameter only or when not included in the signal by the announcements number of cycle default.

If the parameter noc is 0, the signal is played specified by the signal duration only or when not included in the signal by the corresponding announcement default.

Table 1 shows the possible combinations of Signal type, Duration, Number of Cycles and the resultant effect.

Table 1 / H.248 Annex K – Announcement Playing Result

	Signal Type
	Signal Duration
	Number of Cycles

(iterations)
	 Result

	Brief
	Not included
	Not included
	Plays message a number of times up to default duration or a default number of times which ever one is shorter. The message may be stopped part way through a signal.

	
	
	0
	Plays the message n times up to the default duration.

	
	
	1
	Plays message once or for the default duration which ever one is shorter. The message may be stopped part way through a signal.

	
	
	n times
	Plays message a number of times up to default duration or n number of times which ever one is shorter. The message may be stopped part way through a signal.

	
	0
	Not Included
	Plays the message n times according to the default Number of Cycles.

	
	
	0
	Plays multiple iterations (endless play)

	
	
	n times
	Plays the message n times 

	 
	Duration > per-announcement Duration
	Not included
	Plays message a number of times for the specified duration or a default number of times which ever one is shorter. The message may be stopped part way through a signal.

	
	
	0
	Plays the message a number of  times up to the specified duration. The message may be stopped part way through a signal.

	
	
	1
	Plays message once 

	
	
	n times
	Plays the message n times up to the specified duration. The message may be stopped part way through a signal.

	 
	Duration <= per-announcement Duration
	Not included, 0, 1 or n times
	Plays message for the specified duration, message stops before being fully played.

	Timeout
	Not Included
	Not included
	Plays message a number of times for the default duration or a default number of times which ever one is shorter.

	
	
	0
	Plays the message n times up to the default duration.

	
	
	1
	Plays message once up to the default duration

	
	
	n times
	Plays the message n times up to the default duration.

	
	0
	Not Included
	Plays the message n times according to the default Number of Cycles.

	
	
	0
	Plays multiple iterations (endless play)

	
	
	n times
	Plays the message n times 

	 
	Duration > per-announcement Duration
	Not included
	Plays message a number of times for the specified duration or a default number of times which ever one is shorter. The message may be stopped part way through a signal.

	
	
	0
	Plays multiple iterations until the specified duration

	
	
	1
	Plays message once, as 1 iteration is shorter than duration

	
	
	n times
	Plays message a number of times for the specified duration or the specified number of times which ever one is shorter. The message may be stopped part way through a signal.

	 
	Duration <= per-announcement Duration
	Not included, 0, 1, n times
	Plays for the specified duration, message stops before the message fully plays.

	On/Off
	Ignored
	Not included or 0
	Plays multiple iterations (endless play)

	
	
	once
	Plays multiple iterations (endless play)

	
	
	n times
	Plays multiple iterations (endless play)

	Note: This table is only applicable to Annex K. The behavior of signal playing is modified by the use of the Number of Cycles parameter. 


…

[End Correction]

11 Technical and Editorial Corrections to RFC-3015

This section contains technical and editorial correction to RFC-3015 only, that the faults described in this section do not affect the published H.248 (2000) recommendation.

11.1 Typographical Errors in the ASN.1 in RFC3015

	Description:
	When producing RFC3015 from recommendation H.248 (2000) two lines were omitted. It missed out the line defining IP4Address, which should be before IP6Address and there is a missing "..." at the end of the ServiceChangeParm definition.

	Reference:
	Subject: FW: Typos in RFC 3015

Date: Tue, 9 Jan 2001 14:27:55 -0500

From: "Rosen, Brian" <Brian.Rosen@marconi.com>

To: "Tom Taylor (E-mail)" <taylor@nortelnetworks.com>, "Christian Groves (E-mail)" <Christian.Groves@ericsson.com>

CC: "'sob@harvard.edu'" <sob@harvard.edu>


 [Begin Correction]

A.2 ASN.1 Syntax Specification

…
   IP4Address ::= SEQUENCE
   {

        address         OCTET STRING (SIZE(4)),

        portNumber      INTEGER(0..65535) OPTIONAL

   }

   IP6Address ::= SEQUENCE

   {

        address         OCTET STRING (SIZE(16)),

…
   ServiceChangeParm ::= SEQUENCE

   {

        serviceChangeMethod     ServiceChangeMethod,

        serviceChangeAddress    ServiceChangeAddress OPTIONAL,

        serviceChangeVersion    INTEGER(0..99) OPTIONAL,

        serviceChangeProfile    ServiceChangeProfile OPTIONAL,

        serviceChangeReason     Value,

        serviceChangeDelay      INTEGER(0..4294967295) OPTIONAL,

                                    -- 32 bit unsigned integer

        serviceChangeMgcId      MId OPTIONAL,

        timeStamp               TimeNotation OPTIONAL,

        nonStandardData         NonStandardData OPTIONAL,

        ...

        }
[End Correction]

12 Implementation Clarifications for H.248

12.1
Returning a Context ID List

	Description:
	In section 7.2.5 AuditValue it has the following table: The following illustrates other information that can be obtained with the Audit Command: 

ContextID       TerminationID   Information Obtained 

:

All                    wildcard                Audit of all matching Terminations and 

                                                       The Context to which they are associated. 

All
            Root                      List of all ContextIds


[Begin Clarification]

The list of Context Ids should be returned by using multiple Action Replys, each containing a context from the list.

[End Clarification]

12.2
Service Change Address and Ports

	Description:
	There is some confusion on  when to use either ServiceChange Address or ServiceChange MGCID. The text below offers some advice on  


[Begin Clarification]

1) The use of ServiceChangeAddress is not encouraged

2) MGCs must be able to cope with ServiceChangeAddress with either a full address or just a port number in the case of TCP transport.

[End Clarification]

12.3
Audit Response with and without wildcard response (w-)

	Description:
	There is some confusion on what should be sent in the response to an Audit in the cases where you have wildcarded the context or termination and have specified wildcarded response.


[Begin Clarification]

Assume the gateway has 4 terminations : t1/1, t1/2, t2/1 and t2/2

Assume terminations t1/* has implemented packages aaa and bbb and terminations t2/* has implemented packages ccc and ddd.

Assume context 1 has t1/1 and t2/1 in it, and context 2 has t1/2 and t2/2 in it.

The command:

Context=1{AuditValue=t1/1{Audit{Packages}}

Returns: 

Context=1{AuditValue=t1/1{Packages{aaa,bbb}}}}

The command: 

Context=*{AuditValue=t2/*{Audit{Packages}} 

Returns:

Context=1{AuditValue=t2/1{Packages{ccc,ddd}}},

Context=2{AuditValue=t2/2{Packages{ccc,ddd}}}

The command:

Context=*{W-AuditValue=t1/*{Audit{Packages}} 

Returns:

Context=*{W-AuditValue=t1/*{Packages{aaa,bbb}}

Wildcard response may also be used for other commands such as Subtract.

[End Clarification]

12.4
Package Extension and Referencing

	Description:
	The current text in H.248 Section 12.1 is ambiguous about the usage of package names when referencing properties, events, signals and statistics in a base package and an extended package. 


[Begin Clarification]

12.1 Guidelines for defining packages

When packages are extended, the properties, events, signals and statistics defined in the base package can be referred to using either the extended package name or the base package name.  For example, if Package A defines event e1, and Package B extends Package A, then B/e1 is an event for a termination implementing Package B. By definition, the MG MUST also implement the base Package, but it optional to publish the base package as an allowed interface.  If it does publish A, then A would be reported on the Package Descriptor in AuditValue as well as B, and event A/e1 would be available on a termination.  If the MG does not publish A, then only B/e1 would be available.  If published through AuditValue, A/e1 and B/e1 are the same event. 

For the purpose of improved interoperability and backward compatibility, the an MG MAY publish all Packages supported by its Terminations, including base Packages from which extended Packages are derived.  An exception to this is in cases where the base packages are expressly designed to be extended by others, not directly controlled, and may not have any function on their own or be nonsensical on their own, in which case the MG SHOULD NOT publish the base Packages.

 [End Clarification]

12.5 Zero in octetString
	Description:
	It is unclear why 0 is not included in the nonEscapeChar.

	Reference:
	Subject:      Zero not allowed

Date:         Sun, 31 Dec 2000 11:30:07 -0600

Reply-To: plong@ipdialog.com

From: Paul Long <plong@packetizer.com>

Organization: ipDIalog, Inc.

To: MEGACO@STANDARDS.NORTELNETWORKS.COM


 [Begin Clarification]

The octet zero is not among the permitted characters in octet string, which is used in the ABNF for local and remote descriptors.  As the current definition of these descriptors is limited to SDP, and a zero octet would not be a legal character in SDP, this is not a concern.
[End Clarification]

12.6 First Line of SDP a newline

	Description:
	There is confusion in the ABNF encoding with regards to the use of a new line as the first line of the SDP. The clarification below seeks to clarify this.

	Reference:
	Subject: RE: [Fwd: RE: Does the First Line of SDP need a newline]

Date: Tue, 13 Mar 2001 09:13:57 -0500

From: "Rosen, Brian" <Brian.Rosen@marconi.com>

To: "'Christian Groves'" <Christian.Groves@ericsson.com>, megaco ietf <megaco@fore.com>


 [Begin Clarification]

Referring to section 7.1.8 and the ABNF for Local/Remote, SDP disallows whitespace at the beginning of a line, Megaco ABNF allows whitespace before the beginning of the SDP in the Local/Remote descriptor.  Parsers should accept whitespace between the LBRKT following the Local/Remote token and the beginning of the SDP.  

[End Clarification]

13 Implementation Clarifications for H.248 annex H

13.1
MTP3 Interworking

	Description:
	When studying some network scenarios for a certain networks, there is a need to evolve the signalling transport from SS7 MTP3B in an ATM environment to the use of SCTP in IP environments. To provide this M3UA on top of SCTP can be used. It is also seen that M3UA supports flexible implementation scenarios. Therefore some addition indicating the use of M3UA on top of SCTP needs to specified in Annex H in H.248.

	Reference:
	Subject: MTP 3 interworking

Date: Thu, 3 May 2001 16:51:14 +0200

From: "Alf Heidermark (UAB)" <Alf.Heidermark@uab.ericsson.se>

To: "'Megaco (E-mail)" <megaco@fore.com>


[Begin Clarification]

To provide interworking between MTP3B and SCTP and to allow for flexible implementations of gateways and controllers in order to offer efficient use of SCTP associations the M3UA layer may be added on top of SCTP.

 [End Clarification

14 H.248 Recommendation Series Defect Report Form

	DATE:
	

	CONTACT INFORMATION

NAME:

COMPANY:

ADDRESS:

TEL:

FAX:

EMAIL:
	

	AFFECTED RECOMMENDATIONS:
	

	DESCRIPTION OF PROBLEM:
	

	SUGGESTIONS FOR RESOLUTION:
	


NOTE - Attach additional pages if more space is required than is provided above.

*contact: Terry Anderson, Lucent Technologies, +1 908 582 7013, tla@lucent.com  



