
	Question(s):
	2
	Meeting, date:
	ITU-T SG16 Geneva, 20-30 May 2003

	Study Group:
	12
	Working Party:
	2
	Intended type of document (R-C-D-TD):
	TD

	Source:
	EDITOR

	Title: 
	Draft Recommendation H.460.5 -- H.225.0 transport of multiple Q.931 IE of the same type

	Contact:
	Alexander Ruditsky

RADVISION Inc

USA
	Tel: +1(201)6896343

Fax: +1(201)6896301

Email:sasha@radvision.com

	Contact:
	
	Tel:

Fax:

Email:

	lease don’t change the structure of this table, just insert the necessary information.


Summary

This Recommendation specifies the method of transferring duplicate Recommendation Q.931 information elements in the Recommendation H.225.0 messages. Such method is required to overcome limitation imposed by Recommendation H.225.0. The latest recommendation allows including of only single Recommendation Q.931 information element of particular type in the single Recommendation H.225.0 message. 

	[image: image1.wmf]
	INTERNATIONAL TELECOMMUNICATION UNION


ITU-T
Draft H.460.5
TELECOMMUNICATION
(May, 2003)
STANDARDIZATION SECTOR

OF ITU

SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS

Infrastructure of audiovisual services – Systems and terminal equipment for audiovisual services

H.225.0 transport of multiple Q.931 IE of the same type



ITU-T Draft Recommendation H.460.5

CONTENTS

31
Scope


32
Normative References


33
Terms and Definitions


34
The conversion procedure


45
Generic Data Parameter specification



Scope 

Recommendation H.225.0 forbids including multiple Q.931 information elements of the same type in the same message. 

This Recommendation specifies the method of transferring duplicate Q.931 IE in H.225.0. 

Normative References

The following ITU-T Recommendations and other references contain provisions, which through reference in this text constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–
ITU-T Recommendation H.225.0 (2000), Call signalling protocols and media stream packetization for packet-based multimedia communications systems.

–
ITU-T Recommendation H.323 (2000), Packet-based multimedia communications systems.

–
ITU-T Recommendation Q.931 (1998), ISDN user-network interface layer 3 specification for basic call control. 

Terms and Definitions

For the purposes of this Recommendation the definitions given in clause 3 of Recommendations H.323, H.225.0 and H.245 apply. 

The conversion procedure

To transfer a Q.931 message containing multiple IE of the same type over the H.225.0 protocol, the conversion specified in this section shall be performed.  

The conversion applies to all IEs except for the IEs that have the first appearance of the particular IE identifier. The binary representations (that includes IE identifier, length and content) of all such IEs shall be assembled into a single octet string. The resulting octet string shall be used as the value of the GEF parameter, as defined in "Generic Data Parameter specification" section. In accordance with H.225.0, IEs shall not be duplicated in the Q.931 part of the H.225.0 message. 

Generic Data Parameter specification

This section defines the DuplicateIEs feature and parameter

	Feature name:
	DuplicateIEs 

	Feature Description:
	The duplicate Q.931 IE support.  

	Feature identifier type:
	Standard

	Feature identifier value:
	5


	Parameter name:
	IEsString

	Parameter description:
	The binary representation of the 2 to last IEs of particular type.

	Parameter identifier type:
	Standard

	Parameter identifier value:
	1

	Parameter type:
	raw 

	Parameter cardinality:
	1


                                            

	* Contact: 
	Paul E. Jones

Cisco Systems, Inc.

7025 Kit Creek Road

RTP, NC 27709
	Tel:
+1 919 392 6948

Fax:
+1 919 392 2177

E-mail: 
paulej@packetizer.com



