| Rapporteur Meeting of Questions 2, 3, 4, 12, 13, 21, 22, 24, 25, 27, & 28/16 |  |  | AVD-3811 |
| --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | STUDY GROUP 16 |  |
|  |  | Original: English |  |
| Question(s): | Q2 | Geneva, 22 – 25 June 2009 |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |
| Source: | RADVISION |  |  |
| Title: | The conference list feature in H.323 |  |  |
| Purpose: | Proposal |  |  |

Summary

This contribution contains a proposal extending conference list functionality in H.323.

# 1.	Introduction

The current version of H.323 allows MCU which receives a call from an H.323 terminal to respond with a list of possible conferences and as a result to instruct an H.323 terminal to join one of them. However, there is no procedure which is designed to allow sending the conference list to an H.323 terminal without an action initiated by the terminal.

The need for such procedure was discussed by C-113 of the January 2009 SG16 meeting.

This contribution contains such procedure and proposes to add it to H.323.

# 2.	Proposal

## 2.1	H.323 Changes

Add new Clause to the ITU-T Recommendation H.323

8.4.3.9 Conference out of conference list.

Endpoint 2 is an MC(U) that hosts multiple conferences and wishes to provide Endpoint 1 with a choice of conferences to join. Later Endpoint 1 willing to join a conference from the conference list uses the procedures outlined in Claus 8.4.3 of this Recommendation to do so.

A1)	Endpoint 2 sends a Setup message to Endpoint 1 containing CID 0 (the value containing 16 binary zeroes) and conferenceGoal = callIndependentSupplementaryService. The bearer capability information element of the Setup message shall be set as defined for call-independent connection in 7.2.2.1.2/H.225.0.

A2)	Endpoint 1 sends a Call Proceeding message to Endpoint 2.

A3)	Endpoint 2 sends a Facility message indicating conferenceListChoice and a list of conferences that Endpoint 1 may choose from. The list of conferences is sent as part of the Facility-UUIE.

A4)	Endpoint 1 sends a Release Complete message to the MC(U).

A5)	Later Endpoint 1 may join a conference from the list of conferences by sending a new Setup message to the MC(U) on the Call Signalling Channel which contains the selected CID from the conferenceListChoice field of the Facility message received before, and which has conferenceGoal = join.

# 3.	Conclusion

It is proposed that the changes specified in this document are agreed and incorporated into H.323.

__________________