| Rapporteur Meeting of Questions 2, 3, 4, 12, 13, 21, 22, 24, 25, 27, & 28/16 |  |  | AVD-3812 |
| --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | STUDY GROUP 16 |  |
|  |  | Original: English |  |
| Question(s): | Q2 | Geneva, 22 – 25 June 2009 |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |
| Source: | RADVISION |  |  |
| Title: | 3DPresence project and H.323 |  |  |
| Purpose: | Discussion |  |  |

Summary

This document describes an ongoing “3DPresence” EU project. The implementation of the project by means of H.323 would require standardization of additional H.323 functionality. The document outlines such missing functionality.

# 1.	Introduction

Regular videoconference allows only establishment of 2D single view per participant communication. 3DPresence project is working on breaking this boundary to provide multi-view per participant multi-participant per site 3D video. This creates a challenge of transmission and synchronization of the video signals in a standard and interoperable way.

# 2.	Discussion

## 2.1	3DPresence project description

The 3DPresence project is working on the development of a next generation tele-presence system, by combining 3D display technologies and non-verbal communication delivery techniques to video conferencing. 3DPresence is focused on delivering of a natural communication system with 3D perception that takes into account the requirement of a natural reproduction of both face and body expression, i.e., eye contact, directional gaze and gesture awareness. A 3DPresence communication system utilizes displays that are able to reproduce the size, colour and depth of the transmitted three-dimensional scene, and include tracking and rendering technologies capable of mapping the real world from one end point to the specific perspective that the rest of the conferees expect.

Depending on the overall 3D geometry of the video-conferencing scene and the specifications of the 3D displays, the 3D representation of the local participants is rendered in a layered video-plus-depth format. Each display is designed to have wide angle 3D visual cone, presenting different perspectives of the remote user to the different local users. Specifically, for three sites in triangle geometry with two users at each site (see figure below), the first half of the 3D vision cone will be used to show a particular perspective of a remote user to a local user, and the second half of the cone would then show a different perspective of the remote user to the other local user. This would allow rendering two different views of the same remote user per display, showing one view for each local participant.

One of the project’s objectives is to enable interoperability between 3D video-conferencing systems, such as the one developed as part of 3DPresence project and the legacy 2D video-conferencing equipment. To achieve this goal, it is highly desirable that 3DPresence signalling will be based on a standard video-conference protocol. The standard protocol shall provide diverse signalling communication, to enable 3DPresence sites to connect call, hold call, inquire information, modify media, disconnect call, etc. Furthermore, the 3DPresence call signalling design shall be robust to temporal changes in site attendance and user attendance, as well as global changes in the geometry of the 3DPresence video-conferencing scene. The standard protocol shall additionally provide means for multimedia capability negotiation, to enable the negotiation of media transport information, media codec information, etc. In addition, the 3DPresence multimedia capability signalling design shall enable the negotiation of the aforementioned unique 3DPresence display characteristics, namely video or depth format indication, 3D display geometry indication and 3D vision angle.

## 2.2	What is missing in H.323 to implement 3DPresence

The 3D display technology designed by the 3DPresence project uses video-plus-depth format. Using the video-plus-depth technology, 3D video is composed of 2D video stream and depth-map video stream, integrated by the receiver into a single 3D video stream rendered by a display. The signalling protocol shall provide standard means for signalling/transmitting 2D/depth-map streams.

The 3DPresence divides capture single video scene into several portions and transmits each portion independently. Each 3D display screen in a 3DPresence site presents a different portion of the remote site environment, based on 3DPresence geometry. A standard mechanism of negotiation of the available capturing devices/screens geometry is required. The signalling protocol shall provide standard semantics for such geometry indication.

To be able to display a scene in specific perspectives expected by different viewers, the 3DPresence project designs its 3D displays to render several views in a single wide angle 3D visual cone. Such 3D displays are able to present different view for each local viewer. A standard mechanism of negotiation of the available perspectives is required. The signalling protocol shall provide standard semantics for such perspectives indication.

The aforementioned mechanisms of 3D display geometry and perspectives negotiation strongly dependent on 3DPresence geometry. The standard shall be prepared to handle future changes in geometry with backward compatibility.

To obtain natural reproduction of face and body expression, 3D display screen choices shall be mutually determined for all sites. To demonstrate this, the geometry used by 3DPresence project is presented below. The following picture represents a site where two remote conferees from one site are presented on screens ‘a’ and ‘b’. Two remote conferees from another site are presented on screens ‘c’ and ‘d’. The local conferees are ‘e’ and ‘f’.

The order in which the conferees are presented on the site where local conferees are ‘a’ and ‘b’ and on the site where local conferees are ‘c’ and ‘d’ is important for correct scene reproduction.

# 3.	Conclusion

It is proposed that the information provided in this document is analyzed by Q2 experts and the group considers the development of the discussed capabilities for H.323.

__________________