| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, 24, 25, and 27/16 |  |  |  | TD-41 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Research Triangle Park, NC, USA, 29 Nov. - 3 Dec. 2010 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.VSprot |  |  |  |
| Title: | H.VSprot “Signalling and Protocols for visual surveillance”: Updated draft |  |  |  |
| Purpose: | Discussion |  |  |  |

This TD contains the clean version of the latest draft of H.VSprot “Signalling and Protocols for visual surveillance” as output of the Rapporteur Meeting of WP2/16 which was held from 29 November to 3 December 2010.

Introduction

According to the candidate serial work items and possible roadmap discussed in the ITU-T WP2/16 Rapporteur meeting (Geneva, 26 June 2009) showed in the following, it’s time to start the work item of Recommendation H.VSprot since the Recommendation H.VSarch is close to be completed and is prepare to consent in the planned next ITU-T SG16 plenary meeting (Geneva, 19 July 2010). In the stage 1 and 2, we plan to complete the work of requirements and architecture. Then, we should come into stage 3 to start the protocol work item.

Recommendation H.VSprot will focus on the detailed description of reference points, signalling and and protocols based on the overall architecture defined in the Recommendation H.VSarch.

The purpose of this new recommendation is trying to define detailed technical characteristics for each reference points.

For the reference points between different functional entities, it need to identifies relevant protocol, such as protocols between PU and service platform, CU and service platform, PU and CU.

For the reference points in the same functional entities, identifies the internal signaling, for example defining signaling flow or information elements relevant for each reference point, such as the signaling for service control flow, media distribution flow in the service platform.

Furthermore, this recommendation will utilize and extend some existing protocols, such as SIP/RTP/RTSP/HTTP, etc. and it should give the message format as well as message text.

B.1.	Candidate serial work items

The candidate serial work items for deliverables could be:

| No. | Series | Work Item | Questions | Type | Documents | Stage |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Use cases and Scenarios | F.Vsreqs | Q22/16 | Guideline | Rec. | Stage 1 |
| 2 | Requirements and Service Descriptions | F.Vsreqs& H.Vsarch | Q22/16 | Standards | Rec. | Stage 1 |
| 3 | Architecture | H.Vsarch | Q21/16 | Standards | Rec. | Stage 2 |
| 4 | Protocol | H.Vsprot | Q21/16 | Standards | Rec. | Stage 3 |
| 5 | Customer Unit | H.Vscu | Q21/16 | Specifications | Rec. | Stage 3 |
| 6 | Premises Unit | H.Vspu | Q21/16 | Specifications | Rec. | Stage 3 |
| 7 | Service Platform | H.Vssp | Q21/16 | Specifications | Rec. | Stage 3 |
| 8 | Security | H.Vssec | TBD (SG16,17) | Standards | Rec. | Stage 4 |
| 9 | QoE | H.Vsqoe | TBD (SG12, 16) | Standards | Rec. | Stage 4 |
| 10 | QoS | H.Vsqos | TBD (SG12& 16) | Standards | Rec. | Stage 4 |
| 11 | Management | H.Vsman | TBD | Standards | Rec. | Stage 4 |
| 12 | Codecs | H.Vscodec | Q6/16 | Specifications | Rec. | Stage 5 |
| 13 | Interworking | H.Vsiwk | Q21/16 | Guideline | Supp. | Stage 5 |
| 14 | Convergence | H.Vsconv | Q21/16 | Guideline | Supp. | Stage 5 |
| … |  |  |  |  |  |  |

B.2.	Possible Roadmap:

| No. | Stage | Work Item | Work initiated | Target Consented | Status | Participator | Maturity | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Stage 1 | Use cases and Scenarios |  |  |  |  |  |  |
| 2 | Stage 1 | Requirements and Service Descriptions | 2007.06 | 2009-10 | Approved | CATR |  |  |
| 3 | Stage 2 | Architecture | 2007.06 | 2011-03 | Refining | ZTE/CATR | 95% |  |
| 4 | Stage 3 | Protocol | 2010.03 | 2012 |  |  |  |  |
| 5 | Stage 3 | Customer Unit |  |  |  |  |  |  |
| 6 | Stage 3 | Premises Unit |  |  |  |  |  |  |
| 7 | Stage 3 | Service Platform |  |  |  |  |  |  |
| 8 | Stage 4 | Security |  |  |  |  |  |  |
| 9 | Stage 4 | QoE |  |  |  |  |  |  |
| 10 | Stage 4 | QoS |  |  |  |  |  |  |
| 11 | Stage 4 | Management |  |  |  |  |  |  |
| 12 | Stage 5 | Codecs |  |  |  |  |  |  |
| 13 | Stage 5 | Interworking |  |  |  |  |  |  |
| 14 | Stage 5 | Convergence |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |

B. 3 – Document history

| No. | Date | Source | Action | Status | Cause |
| --- | --- | --- | --- | --- | --- |
| #01 | 2010-03-12 | Editors of H.VSprot | Draft baseline document framework from AVD-3907 of WP2/16. | Draft | Initial draft version |
| #02 | 2010-07-30 | Editors of H.VSprot | Update with some new material and do some modifications, such as: 1) Modifications for some editorial changes, references; 2) Adding the detailed structure of clause 6 which is the main part for signalling and protocols; 3) Adding materials for the signalling and protocols on CU/PU registration, media playback, Video/audio acquisition; | Update draft | Update draft version |
| #03 | 2010-12-03 | Editors of H.VSprot | Update with some new material concerning the signalling and protocols, such as: - PU list obtaining - Voice communication - Recording in PU | Update draft | Update draft version |

CONTENTS

1	Scope	5

2	References	5

3	Definitions	6

3.1	Terms defined elsewhere	6

3.2	Terms defined in this Recommendation	6

4	Abbreviations	6

5	Protocol Reference Points	6

6	Signalling and Protocols	7

6.1	Service management	7

6.1.1	PU registration	7

6.1.2	CU registration	8

6.1.3	Accounting and authorization	8

6.1.4	PU list obtaining	8

6.1.4	Configuration	9

6.2	Alarming and linkage action (ALA)	9

6.3	Video/audio acquisition (VAA)	9

6.3.1	Media Acquisition Transferred by service platform	10

6.3.2	Media Acquisition directly from PU	11

6.4	Instant Image snapshot (IIS)	12

6.5	Recording and playback (RnP)	12

6.5.1	Recording	12

6.5.2	Playback	13

6.6	Voice (audio) communication	17

6.6.1	Voice communication	17

6.6.2	Voice (audio) broadcast	18

6.7	Pan-Tilt-Zoom (PTZ) control	18

6.8	Others	18

Draft new ITU-T Rec. H.VSprot

Signalling and Protocols for visual surveillance

AAP Summary

[To be provided before Consent]

Summary

Recommendation H.VSprot will focus on the detailed description of reference points, signalling and and protocols based on the overall architecture defined in the Recommendation H.VSarch.

For the reference points between different functional entities, it need to identifies relevant protocol, such as protocols between PU and service platform, CU and service platform, PU and CU.

For the reference points in the same functional entities, identifies the internal signaling, for example defining signaling flow or information elements relevant for each reference point, such as the signaling for service control flow, media distribution flow in the service platform.

Furthermore, this recommendation will utilize and extend some existing protocols, such as SIP/RTP/RTSP/HTTP, etc. and it should give the message format as well as message text.

Keywords

Visual Surveillance, Signalling, Control Protocols

# 1	Scope

The Visual Surveillance service is a telecommunication service focusing on video (audio included also) application technology, which is used to capture remotely multimedia (such as audio, video, image and various alarm signal) and present them to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability ensured.

The objective of this document is to describe the detailed specification of reference points, signalling and control methods, overall protocols of visual surveillance system based on the functional architecture defined in the Recommendation H.VSarch.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[1]	Recommendation ITU-T F.743 (2009), Requirements and service description for visual surveillance

[2]	Recommendation ITU-T H.VSARCH (TBD), Architectural requirements for visual surveillance

[3]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP)

[4]	IETF RFC 2616 (1999), Hypertext Transfer Protocol -- HTTP/1.1

[5]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[6]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications

{NOTE TSB: use the referencing style as in the ITU-T Recommendation Author’s Guide}

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

# 4	Abbreviations

# 5	Protocol Reference Points

Recommendation [ITU-T H.VSARCH] describes overall architecture framework of Visual Surveillance as the following figure, and it also introduces reference points of VS architecture. This document gives more detailed description of those reference points as well as some newly defined reference points.

Figure 5-1 –Functional Architecture Framework of Visual Surveillance

# 6	Signalling and Protocols

[NOTE – This section will provide the identification of signaling or protocols.]

The following is some illustrative items:

The SIP message format should comply with the definition in RFC3261;

The RTSP message format should comply with the definition in RFC2326;

The HTTP message format should comply with HTTP1.1 defined in RFC2616;

RTP/RTCP signaling and message refers to RFC3550.

## 6.1	Service management

### 6.1.1	PU registration

The PU registration uses SIP protocol (reference to RFC3261). The request message-header uses standard REGISTER method, and CMU return a response with “401 unauthorized” or “200 OK” message. The Signalling flows are shown in the following figure.

# Figure 6-1 PU registration

| STEP 1: | PU uses SIP REGISTER method to implement registration to CMU, the message header (SIP header) contains “From, To, Contract” fields to show the addresses of PU and CMU; |
| --- | --- |
| STEP 2: | CMU returns a response with SIP 401 message and requests PU to send the authentication information; |
| STEP 3: | PU adds authentication information and re-sends SIP REGISTER message to the CMU; |
| STEP 4: | CMU returns SIP 200 OK message or other error code to finish the registration. |

### 6.1.2	CU registration

# Figure 6-2: CU registration

CU registration has the same procedural flow and uses the same protocols as PU registration.

### 6.1.3	Accounting and authorization

### 6.1.4	PU list obtaining

After CU and PU register to service platform, CU needs to request to the SCU in service platform to obtain the PU list information and show a list to end users. The information includes the user information, static list information and dynamic list information of PUs.

# Figure 6-1: Procedural flows for PU list obtaining

| STEP 1: | CU sends a HTTP GET message with the Request-URI “UserInfo” to SCU to request user information. |
| --- | --- |
| STEP 2,3: | SCU needs to check the authentication information from CMU. CMU returns a response to SCU. |
| STEP 4: | SCU returns a HTTP 200 OK message with the user information to CU. |
| STEP 5,6: | CU sends a HTTP GET message with the Request-URI “StaticListInfo” to SCU to request static PU list information. SCU returns a HTTP 200 OK message with the static PU list information to CU. |
| STEP 7,8: | CU sends a HTTP GET message with the Request-URI “DynamicListInfo” to SCU to request dynamic PU list information. SCU needs to get the latest status of PUs and returns a HTTP 200 OK message as a response to CU. |
| STEP 9,10: | SCU sends a SIP MESSAGE with the updated PU list information to CU when SCU gets the relevant status changes event. |

### 6.1.4	Configuration

## 6.2	Alarming and linkage action (ALA)

## 6.3	Video/audio acquisition (VAA)

The session control uses SIP protocol and the message body uses SDP protocol or XML text，followings are the principles:

The media transmission uses RTP protocol. The ports for media channels are not defined in this recommendation, but they usually are pairs, such as port 7000 is used for RTP and 7001 is then used for RTCP.

### 6.3.1	Media Acquisition Transferred by service platform

# Figure 6-3: Real-time media acquisition transferred by MDU

| STEP 1: | CU sends INVITE message which includes CU media format list and CU address information to SCU, SCU transfers this message to PU. In this step can also handle authentication issue. |
| --- | --- |
| STEP 2: | PU returns a response with 200 OK message which includes media format PU selected and PU address information |
| STEP 3: | SCU modifies SDP information and then forwards 200 OK message to CU. |
| STEP 4: | CU sends ACK message including SDP information to SCU |
| STEP 5: | SCU notifies MDU to start media stream and MDU notifies this event to MSU. |
| STEP 6: | SCU sends the ACK message to PU |
| STEP 7: | SCU modifies SDP information in ACK and forwards it to PU. |
| STEP 8: | Media channels between CU and MDU, MDU and PU are established. PU sends RTP packages of media stream to MDU, and MDU transferred the media stream to CU. RTCP packages are transported between CU and MDU, MDU and PU. |
| STEP 9: | CU sends BYE message to end the session. SCU notifies MDU to stop media stream and MDU notifies this event to MSU, and MSU stops sending media stream to CU. Then SCU transferred the BYE message to PU. |
| STEP 10: | PU stops sending media stream and return a response with 200 OK message to SCU. |
| STEP 11: | SCU transfers the 200 OK message to CU. |

### 6.3.2	Media Acquisition directly from PU

# Figure 6-4: Real-time media acquisition directly from PU

| STEP 1: | CU sends INVITE message to SCU. In this step can also handle authentication issue. |
| --- | --- |
| STEP 2: | SCU sends the INVITE message to PU. |
| STEP 3: | PU handles the INVITE message and returns 200 OK message including SDP information. |
| STEP 4: | After receiving 200 OK message, SCU selects codec type, modifes SDP and then forwards 200 OK to CU. |
| STEP 5: | CU returns a response with ACK message including SDP answer. |
| STEP 6: | SCU modifies SDP in ACK and sends it to PU. |
| STEP 7: | PU sends RTP packages of media stream to CU. RTCP packages are transported between PU and CU. |
| STEP 8: | CU sends BYE message to end the session and SCU transfers the message to PU. |
| STEP 9: | PU stops sending media stream and returns 200 OK to SCU. |
| STEP 10: | SCU returns the 200 OK message to CU and stops sending media stream. |

## 6.4	Instant Image snapshot (IIS)

## 6.5	Recording and playback (RnP)

### 6.5.1	Recording

The Recording function is used to record multimedia (such as audio and/or video) information from Surveilled Object and stored this multimedia information either in PUs or service platform.

The following figure shows the flows of start and stop recording in PU. It uses SIP MESSAGE method and uses XML format to specific parameters.

Figure 6-1: Procedural flows for recording in PU

| STEP 1,2: | CU sends a SIP MESSAGE to SCU to query the recording status of requested PU. After authentication, SCU transfers this message to requested PU. |
| --- | --- |
| STEP 3, 4: | PU returns a SIP 200 OK message to SCU with the recording status of requested PU. SCU handles this response and returns a SIP 200 OK message to CU. |
| STEP 5,6: | CU sends a SIP MESSAGE to SCU to request to start recording in PU with the parameters of recording conditions. SCU handles this request to start a new recording task or change the existed recording task according the new request conditions. Then SCU sends a SIP MESSAGE to PU to request PU to start a recording task. |
| STEP 7,8: | PU returns a SIP 200 OK message to SCU and SCU transfers SIP 200 OK message to CU. |
| STEP 9,10: | CU sends a SIP MESSAGE to SCU to request to stop recording in PU. SCU handles this request to stop the recording task. Then SCU sends a SIP MESSAGE to PU to stop recording. |
| STEP 11,12: | PU returns a SIP 200 OK message to SCU and SCU transfers SIP 200 OK message to CU. |

### 6.5.2	Playback

The Playback function is used to play media resource which had been recorded and stored before.

According to the place of recorded media is stored, there are three different means including storing in platform, storing in PU and storing in CU.

The following shows the procedural flows for playback from PU directly.

Figure 6-5: Procedural flows for Playback from PU directly

| STEP 1: | CU sends a HTTP GET request to SCU to get the URL for media resource to play. |
| --- | --- |
| STEP 2: | SCU in the platform return the URL to CU with HTTP response. |
| STEP 3: | CU sends a RTSP DESCRIBE request to PU to get the session description information of PU. |
| STEP 4: | PU returns a response with RTSP 200 OK message, includes SDP information. |
| STEP 5: | CU initiates a RTSP SETUP request to PU, and this request include the following information at least: URL of the requested media resource, Request sequence number, Transport parameters for the CU client. |
| STEP 6: | PU returns a response with RTSP 200 OK message, includes information： Status Line: RTSP/1.0 200OK, Sequence number, Session ID, Transport parameters for the CU client. |
| STEP 7: | CU sends a “RTSP PLAY" request to start getting the media packets, and this request includes the following information: Request Line, include URL of the requested media resource, Request sequence number, Session ID, Range, this parameter is optional. The RTSP server (here is PU) is recommended to support Fast Forward/Fast Rewind and Random access functions through RTSP PLAY request with standard parameters defined in RFC2326. To implement Forward/Fast Rewind function: use “Scale” and/or “Speed” parameters. A scale value of 1 indicates normal play or record at the normal forward viewing rate. If not 1, the value corresponds to the rate with respect to normal viewing rate. Example of playing in reverse at 4 times normal rate: Scale: -4 The parameter value is expressed as a decimal ratio, e.g., a value of 2.0 indicates that data is to be delivered twice as fast as normal. Example: Speed: 2.0 To implement Random access function: use “Range” parameter to indicate the starting point requested by CU. Example: Range: clock=19960213T143205Z-;time=19970123T143720Z |
| STEP 8: | PU returns a response with RTSP 200 OK message, includes information： Status Line: RTSP/1.0 200 OK, Sequence number, Session ID, Range, RTP-Info：URL of the media resource，first RTP package sequence number, RTP timestamp relative to the time in the header of range. |
| STEP 9: | Sends RTP packages of media data from PU to CU. |
| STEP 10: | RTCP packages between PU and CU, RTCP is optional. |
| STEP 11: | If need to pause the media stream, CU sends RTSP PAUSE request to PU, and this request include the following information: Request Line, include URL of the requested media resource, Request sequence number, Session ID, Range (Optional). |
| STEP 12: | PU returns a response with RTSP 200 OK message, includes information： Status Line: RTSP/1.0 200 OK, Sequence number, |
| STEP 13: | If need to stop this media stream, CU sends RTSP TEARDOWN request，and this request include the following information: Request Line, include URL of the requested media resource, Request sequence number, Session ID, |
| STEP 14: | PU returns a response with RTSP 200 OK message, includes information： Status Line: RTSP/1.0 200 OK, Sequence number. |

Figure 6-2 shows the procedural flows for playback from MSU in service platform.

# Figure 6-6: Procedural flows for Playback from service platform

| STEP 1: | CU sends a HTTP GET request to SCU to get the URL for media resource to play; |
| --- | --- |
| STEP 2: | SCU transfers this request to MDU; |
| STEP 3: | MDU in the platform return the URL to SCU with HTTP response; |
| STEP 4: | SCU transfers this response to CU; |
| STEP 5 to 16: | From STEP 5 to 16, the signaling and message are the same as the steps 3 to 14 in Procedural flows for Playback from PU directly; just the RTSP media server for CU is DSU and the media server is recommended to support Fast Forward/Fast Rewind and Random access functions. |

## 6.6	Voice (audio) communication

### 6.6.1	Voice communication

The Voice Communication function is one of the advanced capabilities of Visual Surveillance, and it is used to communicate in voice between End User and Surveilled Object (Human beings).

Figure 6-1: Procedural flows for Voice communication

| STEP 1: | CU sends a SIP INVITE message with SDP information of CU to SCU to request voice communication. |
| --- | --- |
| STEP 2,3: | SCU needs to apply resources from MDU for media stream ‘forwarding. This is an internal interface in service platform; it may be implemented based on TCP protocol. MDU returns a response to SCU with SDP information of MDU. So here don’t give recommended protocol. |
| STEP 4,5: | SCU sends a SIP INVITE message with SDP information of MDU to PU to request voice communication. PU returns SIP 200 OK message with SDP information of PU. |
| STEP 6: | SCU sends a SIP ACK message to PU to confirm this session. |
| STEP 7: | The bidirectional audio channels are created between PU and MDU. |
| STEP 8: | PU returns SIP 200 OK message with SDP information of MDU. |
| STEP 9: | CU sends a SIP ACK message to SCU to confirm this session. |
| STEP 10: | The bidirectional audio channels are created between CU and MDU. |
| STEP 11: | RTP packages of audio media stream are transferred by MDU between CU and PU. |
| STEP 12: | To end voice communication, CU sends a SIP BYE message to SCU. |
| STEP 13,14: | SCU notifies MDU to stop transferring media stream and release resources.MDU returns a response to SCU. These are also internal interfaces in service platform. So here don’t give recommended protocol. |
| STEP 15: | SCU returns a SIP 200 OK message to CU to end communication between CU and MDU. |
| STEP 16,17: | SCU sends a SIP BYE message to PU, PU returns a SIP 200 OK message to SCU to end communication between PU and MDU. |

### 6.6.2	Voice (audio) broadcast

## 6.7	Pan-Tilt-Zoom (PTZ) control

## 6.8	Others

__________________