| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4053 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Q.3 Rapporteur |  |  |  |
| Title: | H.248.48 (ex H.248.QHR) “Gate Control Protocol: RTCP XR Block Reporting Package” (New): Input draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the editor’s input draft for draft new H.248.48 (ex H.248.QHR) “Gate Control Protocol: RTCP XR Block Reporting Package”.

2.	Discussion

The Q3/16 Rapporteur has submitted this document in the absence of an editor. Volunteers are sought for the editorship of this draft Recommendation.

The Rapporteur has analysed the contents of draft H.248.48 against the latest progress of the IETF "xrblock" working group. Currently the IETF is working on draft-ietf-avtcore-monarch-01 that describes a monitoring architecture based on XR reports. This is consistent with the work done thus far in H.248.48. Once the above IETF draft is more mature the IETF will focus on producing a measure identity IETF draft. This is the critical draft for H.248.48 as it is normatively referenced. It is also used by all the other XR reporting drafts. H.248.48 makes informally references a number of different XR report types. The drafts defining these are all currently expired. Therefore at this time and given the status in the IETF the Editor does not see the need for any modification at this meeting.

This document is a copy of TD 522/WP2 from the Q3/16 meeting in Geneva, 14 - 25 March 2011. There are no content changes from TD 522/WP2.

3.	Proposal

It is proposed to accept this document as the baseline text for the meeting.

AAP Summary

[To be provided before Consent]

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.qhr Ed. 0.1 | Initial draft into Ottawa meeting 08/06, AVD-2846 |
| H.248.qhr Ed 0.2 | Output from Ottawa meeting 09/06, TD-43 |
| H.248.48 Ed. 0.3 | Output from Shenzhen meeting 03/07, TD-61, with input from AVD-3043 |
| H.248.48 Ed. 0.4 | Output from Geneva meeting 07/07, input from C210, C211, C213, C243 |
| H.248.48 Ed. 0.5 | Output from Seoul meeting 01/08, input from AVD-3216 to 3221, AVD-3330, AVD-3331, AVD-3337, AVD-3348 to 3350 |
| H.248.48 Ed. 0.6 | Input to Geneva meeting, 22-April-2008 |
| H.248.48 Ed. 0.7 | Completely re-written for new RTCP XR blocks based on AVD-3795, output from Geneva rapporteurs' meeting 22-25 June 2009 |
| H.248.48 Ed. 0.8 | Unchanged from TD-36 output (Ed. 0.7) |
| H.248.48 Ed. 0.9 | Output from Geneva meeting 10-11/09, input from C209 |
| H.248.48 Ed 0.10 | Input to Rapporteur meeting 29/11-3/12/10, unchanged from TD234r1 |
| H.248.48 Ed 0.11 | Input to SG16, Geneva, 14 - 25th March 2011meeting. |
| H.248.48 Ed 0.12 | Input to Rapporteur meeting 18 - 22 July 2011, unchanged from TD 522/WP2. |

CONTENTS

1	Scope	4

2	References	5

3	Definitions	6

3.1	Terms defined elsewhere	6

3.2	Terms defined in this Recommendation	6

4	Abbreviations	7

5	Conventions	7

6	RTCP XR Block Reporting Package	7

6.1	Properties	7

6.1.1	Select Remote Measurement Packets for Reporting by Origination	7

6.1.2	Select Forwarded Packets by Origination	8

6.1.3	Select Local Measurement Blocks for Reporting	9

6.2	Events	10

6.3	Signals	10

6.4	Statistics	10

6.4.1	RTCP XR packets	10

6.5	Error Codes	11

6.5.1	Unsupported measurement block type	11

6.5.2	Unsupported block parameter value	11

6.5.3	Inconsistent block type parameters	11

6.6	Procedures	12

6.6.1	Overview	12

6.6.2	Example RTP session and RTCP XR* metrics blocks	12

6.6.3	Controlling the measurements to be made at the MG	14

6.6.4	Specifying metrics blocks measured at the MG and reported to the MGC	14

6.6.5	Specifying metrics blocks measured at the MG and sent to peer RTP systems	15

6.6.6	Specifying the collection of received RTCP XR* packets for the MGC	15

6.6.7	Specifying the forwarding of RTCP XR* packets to other RTP systems	15

6.6.8	Audit and reset of Statistics	16

6.6.9	MGC Procedure to determine the type of an RTP system	17

6.6.10	Use of Cumulative and Interval reports	17

6.6.11	Coding example for a complete RTCP XR packet	17

Bibliography	18

Draft new ITU-T Recommendation H.248.48 (ex H.248.QHR)

Gateway Control Protocol: RTCP XR Block Reporting Package

Summary

[To be provided before Consent]

# 1	Scope

This Recommendation defines packages which allow MGs to report media transmission quality and call quality to MGCs, using metrics defined in a series of RTCP extended report (XR) [IETF RFC 3611] blocks standardised Ed: not yet by IETF. The Recommendation is applicable to MGs which have at least one interface which uses RTP [IETF RFC 3550].

Editor's note: references to Internet Drafts will be replaced by references to RFCs as these become available

For brevity, the metrics defined in the series of Internet Drafts [b-IETF xr-burst-gap-discard] [b-IETF xr-burst-gap-loss] [b-IETF xr-concsec] [b-IETF xr-delay] [b-IETF xr-discard] [b-IETF xr-jb] [b-IETF xr-loss-conceal] [b-IETF xr-pdv] [b-IETF xr-postrepair-loss] [b-IETF xr-qoe] [b-IETF xr-siglevel] will be referred to below as "RTCP XR*". "RTCP XR*" includes neither RTCP XR itself [IETF RFC 3611] which defines RTCP XR Block Types 1 to 7 inclusive, nor other documents defining RTCP XR Block Types 8 to 10 inclusive (see [b-IANA RTCP XR]). The defining common characteristic of the RTCP XR* family is the use, by blocks containing measurements, of a tag pointer to the Measurement Identity block [IETF xr-meas-identity] which contains shared information about the set of packets to which the measurement applies.

The RTCP XR* metrics family may be applied to unicast bidirectional RTP sessions including zero or more RTP translators as well as two RTP end systems (NOTE). For such RTP sessions, an MG may receive the RTCP XR* metrics family reports from one or more of the other RTP systems involved in the RTP session, up to a possible maximum of all the RTP systems other than itself.

This package uses a sub-list of Octet String to permit transfer of a variable number of the RTCP XR* metrics family reports in an H.248 Statistics descriptor. Each Octet String contains all the data from a single RTCP XR* packet, which contains measurements made at one RTP system of one or more streams originating at another RTP system (or systems). The RTCP XR* packet contains the SSRC [IETF RFC 3550] of the RTP system which made the measurement, and an SSRC for each RTP system which originated a measured stream. This enables the MGC (or other entity observing the report) to determine the section of the RTP session to which the report applies.

NOTE: 	These models (including the RTP mixer) are the high-level RTP system models according to [IETF RFC 3550]. [b-IETF RFC 5117] is an extension and provides RTP topology models at a more detailed level. All these models are applicable in principle for H.248.48 because an H.248 IP Stream or Termination point may be a node of such an RTP topology. The particular RTP topology may in principle affect the processing of the report.

For instance, the RTP translator model is divided into an "RTP transport translator" and an "RTP media translator" model. The first one is media-agnostic, the second one is media-aware. The individual modes have different impact on RTCP packet types and/or report information.

Through the use of a H.248 Properties, the package allows the MGC to control the following aspects of MG behaviour:

- the reporting of a specified set of Report Blocks, measured locally at the MG, towards the MGC as H.248 Statistics

- the collection of RTCP XR packets incoming to the MG for reporting towards the MGC as H.248 Statistics. No more than one RTCP XR packet is to be stored for each unique tuple of Originating Point and Measurement Point. An existing stored packet is overwritten by the next packet to arrive for the same tuple.

- the forwarding of RTCP XR packets by RTP translators, including whether RTP translators forward packets which were generated by other RTP translators

The RTCP XR* family is intended to be extensible and it is the intention that this package will not require modification if RTCP XR* is extended. Hence Property values which control the RTCP XR* blocks which are to be reported by the MG are defined by reference to SDP syntactical elements standardised additively in [IETF RFC 3611] and in RTCP XR*. As new RTCP XR* blocks are defined, allowable Property values are implicitly extended with the SDP defined for the new block.

In addition the MGC may control the sending of a specified set of RTCP XR* Report Blocks (alongside RTP media) by an MG which is an RTP System (End System, Mixer or Translator), by addition to the Remote Descriptor of the same standardised SDP, in "a=rtcp-xr:" lines.

Where SDP or SDP-like syntax is used to specify blocks from the RTCP XR* family, either in the Remote Descriptor or in the Property xrbr/srb, this implies that an appropriate Measurement Identity block [IETF xr-meas-identity] must also be provided to describe the packet stream reported by the measurement blocks. The Measurement Identity block is not requested explicitly in SDP text, but a Measurement Identity block must be provided to support any set of metrics blocks which requires it. If a set of metrics blocks report measurements on the same set of packets, all these RTCP XR* metrics blocks should refer to the same Measurement Identity block.

The new Statistic xrbr/xrpkt may be sent by the MG to the MGC in an H.248 response to an Audit or Subtract command, in a similar manner to other H.248 Statistics. It may also be sent by the MG in parameter scr/val in an Observed Event of the Statistic Conditional Reporting Package [ITU-T H.248.47]. No immediate H.248 action is initiated by the arrival of RTCP XR* packets at the MG, but (depending on the value of xrbr/scpo) the incoming packet may be stored for future reporting. If a packet is stored, it overwrites any previously-stored packet having the same tuple of Originating Point and Measurement Point.

The MGC may reset the values of locally-measured metrics within reports in the statistic descriptor. It is not possible to reset the values of metrics received in RTCP metrics family reports from remote systems.

Detailed descriptions of RTCP XR* metrics are not repeated here.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation

[ITU-T H.248.1]	ITU-T Recommendation H.248.1 Amd.2 (2009), Gateway control protocol: Version 3: Corrections and clarifications

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications

[IETF RFC 3611]	IETF RFC 3611 (2003), RTP Control Protocol Extended Reports (RTCP XR)

[IETF xr-meas-identity]	draft-ietf-avt-rtcp-xr-meas-identity-02 (Work In Progress)

[Editor's note. References to Internet Drafts will be updated as the current draft Recommendation is updated, to track changes in the Internet Drafts as they progress towards RFC status.]

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	CNAME [IETF RFC 3550]: Canonical Name, a persistent transport-level identifier for an RTP source

3.1.2	Filtering Point, FP: Point at which the RTCP traffic is policed. The conditions of the FP policy rules here are typically based on RTP/RTCP information elements. The MGC may set policy on the MG to control this filtering, via the property xrbr/sfpo. The actions of FP policy rules are described in clause 6.1.2 below.

3.1.3	Measurement Point, MP[b-ITU-T M.2301]: The physical or logical point at which measurements can be made and to which the data obtained is related. In the context of this Recommendation, the RTP system which receives one or more streams of RTP packets, makes measurements on the streams, and may generate RTCP XR* packets containing the resulting metrics for transmission towards other RTP systems. The MG may be a Measurement Point. The SSRC of the Measurement Point is present in the RTCP XR header of an RTCP XR packet, see section 2 of [IETF RFC 3611].

3.1.4	RTCP XR [IETF RFC 3611]: RTCP Extended Reports. RFC 3611 defines both an extensible scheme for definition of new "blocks" to contain metrics, and the first set of seven blocks which use this scheme.

3.1.5	SDES [IETF RFC 3550]: Source Description RTCP packet containing Source Description items including CNAME.

3.1.6	SSRC [IETF RFC 3550]: Synchronisation Source identifier.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Originating Point, OP: The RTP system which originates a stream of RTP packets. For this package the Originating Point is a Synchronisation Source for the RTP packets according to [IETF RFC 3550], and hence must be an RTP End System or RTP Mixer. The SSRC of the Originating Point is reported in the Measurement Identifier block associated with a group of RTCP XR* report blocks.

3.2.2	Reporting Point, RP: An MG which implements the current package, defined as the device which sends suitably coded RTCP XR* reports as an H.248 Statistic towards the MGC.

3.2.3	RTCP XR*: The family (subset) of RTCP XR metrics blocks [b-IETF xr-burst-gap-discard] [b-IETF xr-burst-gap-loss] [b-IETF xr-concsec] [b-IETF xr-delay] [b-IETF xr-discard] [b-IETF xr-jb] [b-IETF xr-loss-conceal] [b-IETF xr-pdv] [b-IETF xr-postrepair-loss] [b-IETF xr-qoe] [b-IETF xr-siglevel] which use the Measurement Identifier block [IETF xr-meas-identity] to supply information about the set of packets to which the metrics refer, together with the Measurement Identifier block itself.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

| FP | Filtering Point |
| --- | --- |
| MP | Measurement Point |
| OP | Originating Point |
| QoS | Quality of Service |
| RP | Reporting Point |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| SDP | Session Description Protocol |
| XR | (RTCP) Extended Report |

# 5	Conventions

None

# 6	RTCP XR Block Reporting Package

|  | Package Name: | RTCP XR Block Reporting Package |
| --- | --- | --- |
|  | Package ID: | xrbr (0x00af) |
|  | Description: | Package for collection of metrics data for RTCP XR* report blocks using the RTCP XR* metrics architecture, and for control of gateway behaviour with respect to generation, forwarding, and collection of these report blocks. |
|  | Version: | 1 |
|  | Extends: | None |

## 6.1	Properties

### 6.1.1	Select Remote Measurement Packets for Reporting by Origination

|  | Property Name: | Select Remote Measurement Packets for Reporting by Origination |
| --- | --- | --- |
|  | Property ID: | scpo (0x0001) |
|  | Description: | Selects which RTCP XR* packets arriving at the MG are collected in the Statistics Descriptor and sent to the MGC. Packets are selected based on the type of RTP system which originated them. The two possibilities which may be distinguished at the MG are packets from an RTP Translator (not a Synchronization Source for RTP), or packets which are from either an RTP End System or RTP Mixer (both of which are Synchronization Sources for RTP). An RTCP XR packet from an RTP End System may be recognised because it has a sender SSRC in the RTCP XR header which is identical to an SSRC which appears in RTP media packets received at the same ephemeral termination. An RTCP XR packet from an RTP Translator may be recognised because it has a sender SSRC in the RTCP XR header which is not identical to an SSRC which appears in any RTP media packet received at the same ephemeral termination. Local measurements to be sent as RTCP XR* metrics blocks to the MGC are specified by Property xrbr/srb (see 6.1.3 below) and are not affected by xrbr/scpo. This package does not provide any means to restrict the set of RTCP XR* block types reported to the MGC from RTCP XR packets incoming to the MG. Selection by this Property is at the granularity of a complete RTCP XR packet. |
|  | Type: | Sub-list of enumeration |
|  | Possible values: | The sub-list contains one or two of the following: N (0x0001):Select no packets for forwarding to remote RTP entities E (0x0002):Select packets from an RTP End System or RTP Mixer T (0x0003):Select packets from an RTP Translator If enumeration value N is selected, it must be the only value present. If N is not present, the list may hold E alone, T alone, or both E and T together. |
|  | Default: | N |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 6.1.2	Select Forwarded Packets by Origination

|  | Property Name: | Select Forwarded Packets by Origination |
| --- | --- | --- |
|  | Property ID: | sfpo (0x0002) |
|  | Description: | Selects which RTCP XR packets arriving at the MG are forwarded in RTCP towards other RTP systems involved in the RTP session. Packets are selected based on the type of RTP system which originated them. The two possibilities which may be distinguished at the MG are packets from an RTP Translator (not a Synchronization Source for RTP), or packets which are from either an RTP End System or RTP Mixer (both of which are Synchronization Sources for RTP). This Property is primarily applicable to RTP Translators, because it would not be usual for an RTP End System or RTP Mixer to forward a received RTCP Packet. An RTCP XR packet from an RTP End System or RTP Mixer may be recognised because it has a sender SSRC in the RTCP XR header which is identical to an SSRC which appears in RTP media packets received at the same ephemeral termination. An RTCP XR packet from an RTP Translator may be recognised because it has a sender SSRC in the RTCP XR header which is not identical to an SSRC which appears in any RTP media packet received at the same ephemeral termination. This package does not provide any means to restrict the set of RTCP XR* block types forwarded from RTCP XR packets incoming to the MG. Selection by this Property is at the granularity of a complete RTCP XR packet. If a packet is forwarded, the forwarded packet should comply with the guidance on RTCP processing in Translators, see section 7.2 of [IETF RFC 3550]. |
|  | Type: | Sub-list of enumeration |
|  | Possible values: | The sub-list contains one or two of the following: N (0x0001) Select no packets for forwarding to the MGC E (0x0002) Select packets from an RTP End System or RTP Mixer T (0x0003) Select packets from an RTP Translator If enumeration value N is selected, it must be the only value present. If N is not present, the list may hold E alone, T alone, or both E and T together. |
|  | Default: | E |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 6.1.3	Select Local Measurement Blocks for Reporting

|  | Property Name: | Select Local Measurement Blocks for Reporting |
| --- | --- | --- |
|  | Property ID: | srb (0x0003) |
|  | Description: | Selects which locally-measured RTCP XR* Block Types are reported by the MG to the MGC. If one or more RTCP XR* metrics blocks are to be generated, a Measurement Identity block [IETF xr-meas-identity] MUST be reported. Each string element of the sub-list relates to the reporting of a single metrics block type.Locally-measured RTCP XR* Block Types may also be generated by the MG towards other RTP systems alongside RTP media. The Block Types to be generated as RTCP towards other RTP systems may also be controlled via H.248. This control is implemented by including SDP "a=rtcp-xr:" lines in the Remote Media Descriptor for the Stream or Termination, as described in clause 6.6.5 below. |
|  | Type: | Sub-list of String (SDP coding) |
|  | Possible values: | The syntax of each string element of this Property is the SDP defined in the <Internet Draft> describing the block type. As an informative example, to specify generation of the Packet Delay Variation block and the Concealed Seconds block, the sub-list could be: ["pkt-dly-var,2,npc=100.0,ppc=100.0","conc-sec=50"] indicating that the packet delay variation and concealed seconds blocks are to be created and reported to the MGC. Editor's note:SDP in the present [b-IETF -rtcp-xr-*] does not allow control over whether the report is Cumulative or Interval. This control is likely to be added to the IETF drafts and will be inherited here. The sub-list may be empty to indicate that no blocks are to be reported to the MGC. |
|  | Default: | null |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 6.2	Events

None.

## 6.3	Signals

None.

## 6.4	Statistics

### 6.4.1	RTCP XR packets

|  | Statistic Name: | RTCP XR packets |
| --- | --- | --- |
|  | Statistic ID: | xrpkt (0x0001) |
|  | Description: | Each Octet String in the sub-list codes a complete RTCP XR packet, inclding the RTCP XR header defined in Section 2 of [IETF RFC 3611] and all the RTCP XR* report blocks (see below for behaviour in cases where other RTCP XR block types are also present). "Packet" is used here in the sense of Section 6.1 of [IETF RFC 3550]:there are usually multiple RTCP packets within a UDP datagram containing RTCP, including either SR or RR, SDES, and possibly others including the RTCP XR packet which is the subject of this Statistic. Each such packet type is defined by a unique 8-bit packet type field, which is 207 for RTCP XR. Not all RTCP XR* packets sent by, or received by, the MG are necessarily forwarded to the MGC in this Statistic. The selection of packets is controlled by the Properties xrbr/scpo and xrbr/srb described above and in clauses below describing Procedures. Cases are likely to occur in practice where the RTCP XR packet contains RTCP XR block types defined before standardisation of the Measurement Identifier block type, as well as RTCP XR block types belonging to the new family of metrics blocks which use the Measurement Identifier block type. An important example is the series of 7 RTCP XR block types defined in the "RTP Control Protocol Extended Reports (RTCP XR)" document itself [IETF RFC 3611] which may be reported using the packages defined in [b-ITU-T H.248.30]. If such a case occurs, the MG encodes the entire RTCP XR packet containing both old-style and new-style blocks for transmission to the MGC. The MGC may also request Statistics defined in packages of [b-ITU-T H.248.30]. If it does, the data from blocks defined in [IETF RFC3611] will be duplicated in the Audit response or Subtract response, though in different Statistics having different formats. If the MGC does not request Statistics of [b-ITU-T H.248.30], it may parse the data in Statistic xrbr/xrpkt to obtain data corresponding to blocks defined in [IETF RFC3611], and present in RTCP incoming to the MG. However it will not obtain any data corresponding to blocks defined in [IETF RFC3611] and measured locally at the MG, because these are not encoded in the Statistic xrbr/xrpkt. RTCP XR packets are binary data. They are coded for transmission as an Octet String type as described in Annex A.2 of [ITU-T H.248.1] for the binary encoding of the protocol, or using the Hexadecimal Octet Coding described in Annex B.3 of [ITU-T H.248.1] for the text encoding of the protocol. See clause 6.6.11 below for an example of coding a packet according to the method of Annex B.3. |
|  | Type: | Sub-list of Octet String |
|  | Possible values: | Sub-list of Octet String or Hexadecimal Octet Coding of binary data. |
|  | Level: | Stream. If a Termination has only one Stream then xrbr/xrpkt may be reported at the Termination level. |

## 6.5	Error Codes

### 6.5.1	Unsupported measurement block type

|  | Error Code #: | 485 |  |
| --- | --- | --- | --- |
|  | Name: | Unsupported measurement block type |  |
|  | Definition: | The MGC has requested the MG either to report to the MGC, or to send to another RTP system, an RTCP XR measurement block type which it does not support. |  |
|  | Error Text in the Error Descriptor: |  | The SDP text from the Property xrbr/srb or from the Remote Descriptor describing the unsupported block. |
|  | Comment: | None |  |

### 6.5.2	Unsupported block parameter value

|  | Error Code #: | 486 |  |
| --- | --- | --- | --- |
|  | Name: | Unsupported block parameter value |  |
|  | Definition: | The MGC has requested the MG either to report to the MGC, or to send to another RTP system, a parameterised RTCP XR measurement block type. The MG supports the block type only for certain values of the parameters. The MG does not support the block type for the parameter value specified. |  |
|  | Error Text in the Error Descriptor: |  | The SDP text from the Property xrbr/srb or from the Remote Descriptor describing the unsupported block. |
|  | Comment: | None |  |

### 6.5.3	Inconsistent block type parameters

|  | Error Code #: | 487 |  |
| --- | --- | --- | --- |
|  | Name: | Inconsistent block type parameters |  |
|  | Definition: | The MGC has requested the MG to report a parameterised block type to the MGC, and to send the same block type to another RTP system, but the parameters specified in the two cases are different and cannot be supported simultaneously. |  |
|  | Error Text in the Error Descriptor: |  | The SDP text from the Property xrbr/srb or from the Remote Descriptor describing the unsupported block (whichever is received later at the MG). |
|  | Comment: | None |  |

## 6.6	Procedures

### 6.6.1	Overview

The primary role of the MG in connection with this package is to act as a Reporting Point. In this role it may code transmitted and received RTCP XR* packets in the statistic xrbr/xrpkt of the package, and to send this statistic to the MGC. To support this primary role it will usually be required to make local measurements on incoming packet streams.

Here a transmitted RTCP XR* packet is one containing metrics measured at the MG on packet streams incoming to the MG. Packets of this type may be physically transmitted by the MG, as part of an RTCP packet sent towards other RTP systems participating in the RTP session [IETF RFC 3550]. However it is also possible for the MG to make measurements on the incoming packet stream and format them into RTCP XR* blocks in an RTCP XR packet which, though it is not transmitted in RTCP, is coded and sent towards the MGC as one of the Octet String elements of Statistic xrbr/xrpkt. These local measurements are sent to the MGC using the same format as that used when the MG forwards to the MGC reports received in RTCP from other RTP systems of the RTP session.

#### 6.6.1.1	Application of RTCP Source Description Package

It is assumed that, if the MG uses the current package to send RTCP XR* metrics blocks to the MGC, it will always use the RTCP Source Description (rtcpsdes) package [b-ITU-T H.248.71] to send information linking all the SSRC (Synchronization Source identifiers) to the corresponding Canonical Names (CNAMEs).

### 6.6.2	Example RTP session and RTCP XR* metrics blocks

Figure 1 shows an example RTP session with two RTP End Systems with CNAMEs gw1.t1.com and gw7.t2.com, and an RTP Translator with CNAME sbc3.t1.com. Both RTP End Systems generate RTCP XR* blocks for Packet Delay Variation, Delay, and Concealed Seconds, and send them towards other RTP systems participating in the RTP session. In addition the RTP Translator generates RTCP XR* blocks for Packet Delay Variation and Delay for both incoming streams (one from each RTP End System of the session), and sends them to other participants in the RTP session. All RTP Systems allocate Synchronization Source identifiers for themselves, unique within the scope of the RTP session. When this example is used below, we assume that the RTP End System gw1.t1.com is the MG. Other RTP systems may, or may not, be controlled by H.248.

Figure 1: An example RTP session showing RTCP packets at the RTP interface

(H.248 reporting MG = left-hand RTP End System gw1.t1.com)

Figure 2 provides an overview of the coding of the resulting RTCP XR packets (those either reported or collected at the MG) into the Statistic xrbr/xrpkt of the package. It also shows that the resulting CNAME-SSRC "database" is coded into Statistics of the RTCP Source Description package rtcpsdes [b-ITU-T H.248.71] so that the MGC may interpret the SSRCs present in the coded RTCP XR headers and Measurement Identifier blocks within xrbr/xrpkt.

Figure 2: H.248 Reporting point - Coding RTCP XR packets and the CNAME database

Coding of a complete RTCP XR packet is shown in detail in clause 6.6.11 below.

### 6.6.3	Controlling the measurements to be made at the MG

Where an MG makes measurements and creates an RTCP XR packet which may either be sent in RTCP to other RTP systems, or be sent as an element of a xrbr/xrpkt Statistic to the MG, or both, it is necessary to control the measurements to be made by the MG.

The Property xrbr/srb ("Select Local Measurement Blocks for Reporting") specifies the metrics blocks to be reported by the MG to the MGC, which must therefore be populated with measurements made locally on RTP streams at the MG. Procedures for the use of Property xrbr/srb are described in more detail in clause 6.6.4 below.

If the MG does not support a requested block type, it returns error 485, "Unsupported measurement block type". If the MG supports a parameterised block type for certain values of the parameters defining the block, but not for the parameter values specified in the request, it returns error 486 "Unsupported block parameter value".

The MG may also make measurements to populate metrics blocks which are sent as RTCP XR* packets (alongside the RTP media packets) to other RTP systems participating in the RTP session. This function is controlled by lines in the SDP within the Remote Descriptor for the appropriate ephemeral Termination, and is described in more detail in clause 6.6.5 below.

Because the MG may send RTCP XR* metrics blocks both towards its MGC in the xrbr/xrpkt Statistic of this package, and towards other RTP systems of the RTP session as RTCP XR* packets alongside RTP media, the MG must make sufficient measurements to populate both sets of metrics blocks. The set of metrics blocks which must be populated by the MG is the set union of those specified in Property xrbr/srb for reporting to the MGC, and those specified with SDP in the Remote Descriptor for reporting in RTCP XR to other RTP systems. The MG should make the necessary measurements to populate the specified metrics blocks in this set union.

Because metrics blocks for local measurement are specified independently in xrbr/srb and in the Remote Descriptor, it is possible to specify the same metrics block in the two places, but with different parameters in those cases where the metrics block has optional features controlled by SDP. Some syntactically permitted combinations may exceed the capability of the MG. The MGC should ensure that the union of requests for metrics blocks specified by Property xrbr/srb, and requests for metrics blocks specified by SDP in the Remote Descriptor, are within the capability of the MG.

If the MG does not support the combination of blocks specified jointly in Property xrbr/srb and the Remote Descriptor, it returns error 487 "Inconsistent block type parameters" in response to the request which reveals the inconsistency.

### 6.6.4	Specifying metrics blocks measured at the MG and reported to the MGC

The Property xrbr/srb (Select Local Measurement Blocks for Reporting) specifies the RTCP XR* metrics blocks to be populated at the MG based on its measurements on incoming RTP streams, and reported to the MGC.

If any metrics blocks defined in [b-IETF -rtcp-xr-*] are to be reported, it is also necessary to report the corresponding Measurement Identity report block [IETF xr-meas-identity]. The Measurement Identity block is not requested explicitly, but the need for it is implied by the presence of any metrics block which must refer to it.

The syntax of xrbr/srb is a sub-list of OctetString, where each OctetString is an instance of the block-specific SDP defined in the <Internet Draft> which describes the required block [b-IETF -rtcp-xr-*].

As an informative example, Property xrbr/srb would be set to:

["pkt-dly-var,2,npc=100.0,ppc=100.0","delay","conc-sec=50"]

in order to send blocks for packet delay variation, delay, and concealed seconds to the MGC. The numerical values are parameters used to control the collection of statistics by the MG. Their use is described in [b-IETF xr-pdv] and [b-IETF xr-concsec].

### 6.6.5	Specifying metrics blocks measured at the MG and sent to peer RTP systems

In the text encoding of the protocol, metrics blocks to be sent in RTCP XR packets alongside media towards other RTP systems are specified in an "a=rtcp-xr:" line of the SDP media description in the Remote Descriptor, where the text following the "a=rtcp-xr:" element is the syntax element xr-format defined additively in [IETF RFC 3611] and in [b-IETF -rtcp-xr-*].

If any metrics blocks defined in [b-IETF -rtcp-xr-*] are to be sent to peer RTP systems, it is also necessary to send the corresponding Measurement Identity report block [IETF xr-meas-identity]. The Measurement Identity block is not requested explicitly, but the need for it is implied by the presence of any metrics block which must refer to it.

If the MG does not support a requested block type, it returns error 485, "Unsupported measurement block type". If the MG supports a parameterised block type for certain values of the parameters defining the block, but not for the parameter values specified in the request, it returns error 486 "Unsupported block parameter value".

If the MG does not support the combination of blocks specified jointly in Property xrbr/srb and the Remote Descriptor, it returns error 487 "Inconsistent block type parameters" in response to the request which reveals the inconsistency.

### 6.6.6	Specifying the collection of received RTCP XR* packets for the MGC

An RTCP XR packet received at the MG from another RTP system participating in the RTP session, and containing RTCP XR* blocks, may be coded into the statistic xrbr/xrpkt and sent to the MGC. The Property xrbr/scpo (Select Remote Measurement Packets for Reporting by Origination) allows the MGC to select which RTCP XR packets are collected by the MG and sent to the MGC. Property xrbr/scpo specifies whether the MG collects RTCP XR packets which it receives from RTP End Systems and RTP Mixers, and whether the MG collects RTCP XR packets which it receives from RTCP Translators. The Property is a sub-list of enumeration, so it is possible to specify the collection of neither type of report (an empty sub-list), either type, or both types.

RTCP XR packets are recognised as from RTP End Systems because the SSRC in the RTCP XR header is identical to an SSRC in an RTP packet received at the same ephemeral Termination. RTCP XR packets are recognised as from RTP Translators because the SSRC in the RTCP XR header is not identical to an SSRC in any RTP packet received at the same ephemeral Termination.

In the example above, the MG collects packets from its peer RTP End System and from an RTP Translator, as well as reporting the RTCP XR packet containing its own measurements. Hence Property xrbr/scpo is set to

[E,T].

The transmission to the MGC of metrics blocks containing the MG's local measurements is controlled by Property xrbr/srb and not by Property xrbr/scpo.

### 6.6.7	Specifying the forwarding of RTCP XR* packets to other RTP systems

An RTCP XR packet received at the MG from another RTP system participating in the RTP session, and containing RTCP XR* blocks, may be forwarded by the MG in RTCP towards other RTP systems participating in the RTP session. The Property xrbr/sfpo (Select Forwarded Packets by Origination) allows the MGC to select which RTCP XR packets are forwarded by the MG. Property xrbr/sfpo specifies whether the MG forwards RTCP XR packets which it receives from RTP End Systems and RTP Mixers, and separately whether the MG forwards RTCP XR packets which it receives from RTCP Translators. The Property is a sub-list of enumeration. It is possible to specify the forwarding of neither type of report, either type, or both types.

In the example above, the MG is an RTP End System and does not forward RTCP packets. However the RTP Translator sbc3.t1.com forwards RTCP XR packets from RTP End Systems, but not from other translators. If sbc3.t1.com were controlled by H.248, then Property xrbr/sfpo would be set to

[E].

RTP Translator sbc3.t1.com also generates RTCP XR packets based on its own measurements. As an informative example, if controlled by H.248 its MGC would provide the following SDP line in Remote Descriptors to cause each termination to measure, and send in RTCP XR, the Packet Delay Variation and Delay blocks:

a=rtcp-xr:pkt-dly-var,2,npc=100.0,ppc=100.0 delay

An H.248-controlled RTP Translator may, of course, also report Statistics to its MGC, and procedures for this are identical to those discussed above for the case of the RTP End System.

### 6.6.8	Audit and reset of Statistics

An MGC may retrieve Statistics xrbr/xrpkt by standard H.248 methods using Subtract or AuditValue commands. For Statistic xrbr/xrpkt, an AuditValue or Subtract will cause the most recently received RTCP XR* packets from remote RTP systems to be returned, if selected by Property xrbr/scpo. These received packets may several seconds old even if RTCP is sent frequently by the peer RTP systems. However for the MG's locally-measured metrics blocks, an AuditValue or Subtract shall sample the most recent values of the local measurements. This requires the MG to create an RTCP XR packet with metrics blocks (according to xrbr/srb) containing these most recent measurements, triggered on demand by the AuditValue or Subtract request.

An MGC may suspend collection of locally-measured RTCP XR* metrics with block-level granularity by sending a modified Property xrbr/srb which does not include the block or blocks. The suspended block or blocks maintain their value until Subtract, unless a further modified value of xrbr/srb is sent which includes a previously-suspended block or blocks. If this occurs the statistic is reset and re-enabled. See also Practises on Statistics in Appendix IV of [ITU-T H.248.1].

A use case for auditing statistics values (via AuditValue) and reset arises as follows. An MGC may modify an ephemeral termination during its lifetime with changed (remote destination connection) endpoint information in the Remote Descriptor such that it receives RTP packets from a different source, and in this case the MGC may audit and reset statistics near the time of modification so that the MGC has independent statistics for the RTP data received from the two sources.

However it may be undesirable to reset values of metrics which are measured locally and sent to remote RTP systems, because the remote RTP system may be unaware of the reset action and may infer false information about the quality of the IP transport connection between the RTP systems. Because RTCP XR* tags metrics blocks with the Synchronisation Source (SSRC) identifier for the source of the measured packets, it is not normally necessary to reset statistics on a Termination when a Termination is modified to receive packets from another source. However this requires the MG to maintain (typically until Subtract) statistics for RTP sources which are no longer active.

There is a known use case where the MGC may not be aware that the remote RTP source has changed and therefore may not be able to use an audit-and-reset sequence to ensure separation of metrics related to the two sources. This arises when an MG receives RTP packets from a border device (RTP Translator) performing Network Address and Port Translation. An RTP session re-arrangement may take place on the far side of the border device, causing the MG to receive packets sourced by a different RTP system, without any MGC involvement or change in the MG's Remote Descriptor for the Termination. However, the SSRC in received RTP media packets will change (it is not modified by a border device which is an RTP Translator). In this case, MG awareness of SSRC and maintenance of per-SSRC metrics is the only reliable method of separating packet metrics relevant to the two (or more) remote RTP sources beyond the border device.

### 6.6.9	MGC Procedure to determine the type of an RTP system

It is assumed that the MGC and MG use the RTCP Source Description package rtcpsdes [b-ITU-T H.248.71] so that the MGC is able to translate dynamically-assigned SSRC identifiers in RTCP XR* report blocks into the stable Canonical Names of the RTP systems involved in the RTP session. SSRC and CNAME data items provided in package rtcpsdes are identified by the MG as belonging either to the MG, to a remote RTP End System, or to a remote RTP Translator.

### 6.6.10	Use of Cumulative and Interval reports

The RTCP XR* reports all have two variants, Interval (accumulated only since the last report, per source SSRC) and Cumulative (accumulated over the lifetime of the RTP session to date, per source SSRC).

Usually the Cumulative report is a better match to typical H.248 use cases where the MGC receives a report only on Subtract, and it is desirable that this report should contain information about the whole lifetime of the RTP session rather than only about the last few seconds before the Subtract.

### 6.6.11	Coding example for a complete RTCP XR packet

The following example shows the text coding of a complete RTCP XR packet containing:

- an RTCP XR header,

- a Measurement Identity block and

- a Concealed Seconds block,

according to the text encoding of Octet String described in Annex B.3 of [ITU-T H.248.1]. The structure of this packet is shown in Figure 3, together with the hexadecimal values assumed in this coding example.

Block type numbers are not yet assigned by IANA and are shown as XX in the uncoded form and YY in the coded form.

The packet shown in Figure 3 below is coded according to the text encoding rules as the following string of length 112 characters for transmission as one of the Octet String elements of Statistic xrbr/xrpkt across the H.248 interface:

"01F300B09EAE7B5DYY0000E0316F404D000000000000CE3D000059330000B924008079080000E80EYY3000C000000066000000200080004C"

0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦V=2¦P¦reserved ¦ PT=XR=207 ¦ length ¦ 80 CF 00 0D +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ SSRC ¦ 79 75 DE BA +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ BT=NMI ¦0¦ tag ¦ resv ¦ block length = 7 ¦ XX 00 00 07 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ SSRC of stream source ¦ 8C F6 02 B2 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ sub-identifier ¦ 00 00 00 00 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ reserved ¦ first seq num ¦ 00 00 73 BC +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ extended first sequence number of interval ¦ 00 00 9A CC +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ extended last sequence number ¦ 00 00 9D 24 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ Measurement Duration (Cumulative) (ms) ¦ 00 01 9E 10 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ Measurement Duration (Interval) (ms) ¦ 00 00 17 70 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ BT=NCS ¦I¦ tag ¦plc¦rsv¦ block length = 3 ¦ XX 0C 00 03 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ Unimpaired Seconds ¦ 00 00 00 66 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ Concealed Seconds ¦ 00 00 00 04 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ Severely Concealed Seconds ¦ RESERVED ¦ SCS Threshold ¦ 00 01 00 32 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 3: Structure of an RTCP XR packet at the RTP interface

Bibliography

[b-ITU-T H.248.30]	ITU-T Recommendation H.248.30 (2007), Gateway control protocol:RTCP extended performance metrics packages

[b-ITU-T H.248.47]	ITU-T Recommendation H.248.47 (2007), Gateway control protocol:Statistic conditional reporting package

[b-ITU-T H.248.71]	ITU-T Recommendation H.248.71 (200X), Gateway control protocol:RTCP Support Packages

[b-ITU-T M.2301]	ITU-T Recommendation M.2301 (2002), Performance objectives and procedures for provisioning and maintenance of IP-based networks

[b-IANA RTCP XR]	IANA Registry of RTCP XR Block Types at www.iana.org/assignments/rtcp-xr-block-types/rtcp-xr-block-types.xhtml

[b-IETF RFC 5117]	IETF RFC 5117 (2007), RTP Topologies

[b-IETF xr-burst-gap-discard]	draft-ietf-avt-rtcp-xr-burst-gap-discard-02 (Work In Progress)

[b-IETF xr- burst-gap-loss]	draft-ietf-avt-rtcp-xr-burst-gap-loss-02 (Work In Progress)

[b-IETF xr- concsec]	draft-ietf-avt-rtcp-xr-concsec-02 (Work In Progress)

[b-IETF xr- delay]	draft-ietf-avt-rtcp-xr-delay-02 (Work In Progress)

[b-IETF xr- discard]	draft-ietf-avt-rtcp-xr-discard-02 (Work In Progress)

[b-IETF xr-jb]	draft-ietf-avt-rtcp-xr-jb-02 (Work In Progress)

[b-IETF xr-loss-conceal]	draft-ietf-avt-rtcp-xr-loss-conceal-02 (Work In Progress)

[b-IETF xr-pdv]	draft-ietf-avt-rtcp-xr-pdv-03 (Work In Progress)

[b-IETF xr-postrepair-loss]	draft-ietf-avt-rtcp-xr-postrepair-loss-02 (Work In Progress)

[b-IETF xr-qoe]	draft-ietf-avt-rtcp-xr-qoe-00 (Work In Progress)

[b-IETF xr-siglevel]	draft-ietf-avt-rtcp-xr-siglevel-00 (Work In Progress)

__________________