| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4054 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Q.3 Rapporteur |  |  |  |
| Title: | H.248.79 (ex H.248.Packets) “Gate Control Protocol: Guidelines for Packet-Based Streams” (New): Input draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the editor’s input draft for draft new H.248.79 (ex H.248.Packets) “Gate Control Protocol: Guidelines for Packet-Based Streams”.

2.	Discussion

The Q3/16 Rapporteur has submitted this document in the absence of an editor. Volunteers are sought for the editorship of this draft Recommendation.

This document is a copy of TD 523R1/WP2 with change marks accepted from the Q3/16 meeting in Geneva, 14 - 25 March 2011. There are no content changes.

3.	Proposal

It is proposed to accept this document as the baseline text for the meeting.

Document History:

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.Packets Ed. 0.1 | Output (TD-87) from SG16, Geneva, 27 Jan - 6 Feb 2009 meeting. Based on contribution C.75 and meeting agreements. |
| H.248.Packets Ed. 0.2 | Input to WP2/16 Geneva Jun 2009 meeting. Text is identical to Ed. 0.1. |
| H.248.Packets Ed. 0.3 | Input to SG16, Geneva, 26 Oct-06 Nov 2009 meeting. Text is identical to Ed 0.2. |
| H.248.Packets Ed. 0.4 | Input to SG16, Shanghai, 08-12 Mar 2010 meeting. Text is identical to Ed 0.3. |
| H.248.Packets Ed. 0.5 | Output from SG16, Shanghai, 08-12 Mar 2010 meeting. Based on contributions and meeting agreements. |
| H.248.79 Ed. 0.6 | Input to SG16, Geneva, 19-30 Jul 2010 meeting. Updated with assigned Recommendation number (H.248.79). |
| H.248.79 Ed. 0.7 | Output from SG16, Geneva, 19-30 Jul 2010 meeting. Changes based on contributions C.403 and C.444. |
| H.248.79 Ed. 0.8 | Input to SG16/WP2, Research Triangle Park, 29 Nov-03 Dec 2010 meeting. Text is identical to Ed 0.7. |
| H.248.79 Ed. 0.9 | Input to SG16, Geneva, 14 - 25th March 2011meeting. |
| H.248.79 Ed. 0.10 | Output from SG16, Geneva, 14-25th March 2011 meeting. Changes based on contributions C.606, C.608, C.609, C.607 and C.610. |
| H.248.79 Ed. 0.11 | Input to SG16/WP2, Andover, 18 - 22 July 2011 meeting. Text is identical to Ed 0.10. |

Living List Items:

- Need to discuss the classification of packets when a Termination does not have any Stream.

- Examples should be provided for:

  - Lookup-keys based on the Remote Descriptor.

  - Lookup-keys based on “a=” SDP fields.

  - Lookup-keys based on non-SDP Local/Remote Descriptors.

  - Lookup-keys using non-SDP wildcards.

  - Lookup-keys based on Termination ID.

- Need to decide whether classification must be stable when packets can be delivered to more than one Stream.

- The procedures for the classification of ICMP errors should be made more accurate.

- Assuming that a package details operations spread across multiple OSI layers; need to decide whether all operations should be delayed to the highest layer, or each operation performed at its own layer. One example that was given is the gm package, which filters packets according to parameters at the network and transport layers.

- The OSI layer at which some statistics are collected is still unclear. For example some usage statistics of L3 can actually be collected at L2.

- clause 1: final update of Scope

- clause 9: addition of a sub-clause Introduction, describing the relation to H.248.76 and this Recommendation

Contents

1	Scope	6

2	References	6

3	Definitions	7

3.1	Terms defined elsewhere	7

3.2	Terms defined in this Recommendation	7

4	Abbreviations any acronyms	7

5	Conventions	8

6	Classification of packets into Streams	8

6.1	Overview	8

6.1.1	Regular terminations with one or multiple Streams	8

6.1.2	Stream-less terminations	8

6.2	Lookup-keys based on the Local and Remote Descriptors	9

6.2.1	Basic concept	9

6.2.2	Lookup-key based on RD only	9

6.2.3	Use of Wildcards in the Local and Remote Descriptors	10

6.2	Lookup-keys based on H.248 Properties	10

6.3	Lookup-keys based on the Termination Type and TerminationID	10

6.4	Overlapping Classification Rules	11

6.5	Post-classification filtering	11

6.6	Special consideration for ICMP packets	11

6.6.1	ICMP message types	11

6.6.2	Dependency with Context-level “IP topology” in H.248 MG	12

6.6.3	Constitution of ICMP message flows	12

6.6.4	ICMP message handling from Stream perspective	13

6.6.5	Blocking or rate limiting of ICMP traffic by H.248 MGs	13

7	Ordering of operations	13

7.1	Assigning operations to protocol layers	13

7.1.1	Using the OSI Basic Reference Model as example	13

7.1.2	Notes regarding the IETF (Basic) Reference Model for IP	15

7.2	Ordering of operations within a layer	16

7.3	Policy rule based description of packet operations	17

8.	Processing costs between primary and secondary operations on packets (like for measurements)	18

8.1	Principal areas of operations on packets	18

8.2	Processing costs	19

9	Filtering of TCP/IP traffic	20

9.1	Overview – TCP and IP filter types	20

9.1.1	IP filters	20

9.1.2	TCP filters	21

9.2	TCP/IP filter terms	21

9.2.1	Setting the term’s match criteria	21

Bibliography	21

Draft new ITU-T Recommendation H.248.79 (ex H.248.Packets)

Gate Control Protocol: Guidelines for Packet-Based Streams

AAP Summary

<To be provided before Consent>

Summary

<TBD>

# 1	Scope

This Recommendation defines primarily guidelines for handling H.248 Streams that use a packet-based network as their underlying bearer technology. Two significant points are covered by the guidelines:

1.	How packets incoming into the MG are classified to a specific combination of Stream, Termination and Context.

2.	In what order different operations (usually controlled by different packages) are applied to incoming and outgoing packets.

Furthermore, this Recommendation:

3.	Provides qualitative statements concerning generic processing costs on packet handling.

4.	Discusses in more detail the classification and filtering of TCP/IP traffic.

{Editor’s note: the scope section should be reviewed and updated again before sending this draft for consent/approval.}

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	ITU-T Recommendation H.248.1 (09/2005) Amendment 2 (12/09), Gateway control protocol: Version 3

[ITU-T H.248.39]	ITU-T Recommendation H.248.39 (05/2006), Gateway control protocol: H.248 SDP parameter identification and wildcarding

[ITU-T H.248.58]	ITU-T Recommendation H.248.58 (05/2006), Gateway control protocol: Packages for application level H.248 statistics

[ITU-T H.248.64]	ITU-T Recommendation H.248.64 (05/2006), Gateway control protocol: IP router packages

[ITU-T X.200]	ITU-T Recommendation X.200 (07/1994), Information technology - Open Systems Interconnection - Basic Reference Model: The basic model

[ITU-T Y.2011]	ITU-T Recommendation Y.2011 (10/2004), General principles and general reference model for Next Generation Networks

[IETF RFC 791]	IETF RFC 791 (1981), Internet Protocol

[IETF RFC 792]	IETF RFC 792 (1981), Internet Control Message Protocol

[IETF RFC 1122]	IETF RFC 1122 (1989), Requirements for Internet Hosts -- Communication Layers

[IETF RFC 1123]	IETF RFC 1123 (1989), Requirements for Internet Hosts - Application and Support

[IETF RFC 1812]	IETF RFC 1812 (1995), Requirements for IP Version 4 Routers

[IETF RFC 3198]	IETF RFC 3198 (2001), Terminology for Policy-Based Management

[IETF RFC 4443]	IETF RFC 4443 (2006), Internet Control Message Protocol (ICMPv6) for the Internet Protocol Version 6 (IPv6) Specification

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol

# [IETF RFC 4884]	IETF RFC 4884 (2007), Extended ICMP to Support Multi-Part Messages3	Definitions

## 3.1	Terms defined elsewhere

3.1.1	Filter [ITU-T H.248.43]: In general [IETF RFC 3198]: A set of terms and/or criteria used for the purpose of separating or categorizing. This is accomplished via single- or multi-field matching of traffic header and/or payload data. "Filters" are often manipulated and used in network operation and policy. A filter rule is a specific policy rule.

In an H.248 framework: Packet filters specify the criteria for matching a pattern to distinguish separable classes of traffic. Filters are only related to ephemeral terminations. Filter rules are defined on the basis of H.248 properties.

NOTE – This filter definition implies the concept of filter actions besides filter conditions.

## 3.2	Terms defined in this Recommendation

<TBD>

# 4	Abbreviations any acronyms

This Recommendation uses the following abbreviations and acronyms:

| AMR | Adaptive Multi-Rate |
| --- | --- |
| B2BIH | Back-to-back Internet protocol Host |
| BRM | Basic Reference Model |
| DA | Destination Address |
| ICMP | Internet Control Message Protocol |
| IP | Internet Protocol |
| IPv4 | IP Version 4 |
| IPv6 | IP Version 6 |
| L3 | Layer 3 Header Inspection |
| L3HI | Layer 3 |
| L4 | Layer 4 |
| L4HI | Layer 4 Header Inspection |
| LD | Local Descriptor |
| LSP | Label Switch Path |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MSRP | Message Session Relay Protocol |
| NAT | Network Address Translator |
| OSI | Open Systems Interconnection |
| PDU | Protocol Data Unit |
| QoS | Quality of Service |
| RD | Remote Descriptor |
| RFC | Request For Comment |
| RTP | Real-Time Transport Protocol |
| SA | Source Address |
| SDP | Session Description Protocol |
| TCAM | Ternary Content-Addressable Memory |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| ToS | Type of Service |
| UDP | User Datagram Protocol |
| UDPTL | UDP Transport Layer |
| VLAN | Virtual Local Area Network |

# 5	Conventions

None

# 6	Classification of packets into Streams

## 6.1	Overview

### 6.1.1	Regular terminations with one or multiple Streams

For the media-gateway to operate correctly, it must be able to classify and deliver each incoming packet into a single, specific H.248 Stream. Once this classification is made, the packet is also associated with the Termination hosting the Stream, and the Context holding the Termination.

The rule used to match packets with a specific Stream is referred as the Stream’s classification-rule. This rule is based on information that can appear in one or more of the following protocol elements:

- The contents of the Stream’s Local and Remote Descriptors.

- Properties appearing in the Stream’s LocalControl Descriptor.

- The type and TerminationID of the Termination hosting the Stream.

- Properties appearing in the TerminationState Descriptor of the Termination hosting the Stream.

- Properties appearing in the ContextAttributes descriptor of the Context holding the Stream’s Termination.

We refer to the set of protocol elements that are used in the creation of the classification-rule as lookup-keys.

Some H.248 packages mandate that certain elements must be used as lookup-keys, or alternatively must not be used as lookup-keys. A MG implementing such a package shall follow these requirements when creating classification-rules. For most protocol elements, however, there is no normative requirement concerning classification-rules. Whether each element is used as a lookup-key is therefore dependant on the particular use-case or the employed H.248 profile. For example, certain use-cases may call for the MG to consider a source-filter attribute (see [b-IETF RFC 4570] appearing in a SDP Local Descriptor as a lookup-key, while other use-cases may require that such an attribute be ignored.

Usually, a logical AND operation is assumed between all the lookup-keys of a classification-rule, i.e. a packet must match all the lookup-keys of a Stream for it to be delivered to that Stream. However specific packages and use-cases may call for other logical operations between the different lookup-keys.

### 6.1.2	Stream-less terminations

Terminations with one ore multiple Streams may be considered as the regular case when looking at existing H.248 profile specifications with ephemeral terminations. This is however not the only case possible. Some packages (e.g. those of [ITU-T H.248.64]) describe Terminations that do not have Streams at all. Packets are then classified into the complete Termination.

## 6.2	Lookup-keys based on the Local and Remote Descriptors

### 6.2.1	Basic concept

Lookup-keys can be derived from elements of the Local and Remote Descriptors. Note that, according to clause 7.1.8 of [ITU-T H.248.1], the Local Descriptor “refers to the media received by the MG”. Therefore the use of the Local Descriptor’s fields as lookup-keys is a natural and implicit assumption of most H.248 use-cases. On the other hand, the Remote Descriptor “refers to the media sent by the MG”. Therefore using it to derive lookup-keys (that inherently relate to incoming traffic) is justified only in specialized use-cases.

These lookup-keys apply to both the text and binary versions of the H.248.1 protocol.

For Local and Remote Descriptors that contain SDP session descriptions, the possible uses of the SDP fields as lookup-keys is tied-up to the fields’ meaning, as described in [IETF RFC 4566] and various extension RFCs. Following is a list of examples to such uses of SDP fields as lookup-keys. This list is by no means exhaustive and should not limit the possible use of SDP fields as lookup-keys.

- SDP fields of the “c=” line of the Local Descriptor can be used for specifying:

  - The incoming packet’s network-layer protocol (e.g. IP4 or IP6).

  - The incoming packet’s network-layer destination address.

- SDP fields of the “m=” line of the Local Descriptor can be used for specifying:

  - The incoming packet’s transport-layer protocol (e.g. UDP, TCP, UDPTL).

  - The incoming packet’s combination of transport-layer and higher-layer protocols (e.g. RTP over UDP, MSRP over TLS).

  - The incoming packet’s destination transport port.

  - The incoming packet’s application-layer format (e.g. AMR codec frames)

NOTE 1 – SDP fields of SDP attributes, the “a=” lines, may also contain lookup-key elements.

NOTE 2 – It should be noted that even in the text version of H.248 lookup-key elements may also originate from non-SDP elements in Local/Remote descriptors.

### 6.2.2	Lookup-key based on RD only

The case of lookup-keys based on Remote Descriptor information only, may be not excluded. It could be subject of a pure unidirectional end-to-end packet connection, or e.g. a bidirectional packet connection which lacks (initially, temporarily) LD lookup information. A use-case calling for a lookup-key based on the Remote Descriptor is not trivial and for further studies.

### 6.2.3	Use of Wildcards in the Local and Remote Descriptors

To prevent the use of an element of the Local or Remote Descriptors as a lookup-key, the MGC can either omit the element from the Descriptor or set the element’s value to an “All” wildcard. H.248.1 allows different wildcards (CHOOSE, ALL) for elements in the Local and Remote Descriptors.

For Local and Remote Descriptors that contain SDP session descriptions, any wildcards should follow the procedures of [ITU-T H.248.39]. In the context of packet classification, the “Not-Significant” (“-”) SDP wildcard is fully equivalent to the “All” (“*”) SDP wildcard. Both types of wildcards can be used interchangeably

Using SDP wildcards is especially useful when the MGC wishes that only a part of an SDP field will be used for packet classification, or when the MGC wishes to avoid classification according to a mandatory SDP field. For example, many profiles consider the “c=” and “m=” SDP fields as mandatory. An MGC may wish to classify packets based on them being IPv4, but avoid using the IP address or any transport-layer information as lookup-keys. Such an MGC may employ a Local Descriptor similar to the following one:

Local {

v=0

c=IN IP4 *

m=- - - –

}

## 6.2	Lookup-keys based on H.248 Properties

Lookup-keys can be derived from properties appearing in the LocalControl, TerminationState and ContextAttributes Descriptors. Following is a list of examples for such lookup-keys. This list is by no means exhaustive and should not limit the possible use of properties as lookup-keys.

- The ipdc/realm property (see [b-ITU-T H.248.41]) can be used for specifying the IP address realm that the packet should arrive on.

- The vlan/tags property (see [b-ITU-T H.248.56]) can be used to indicate the stack of Ethernet VLAN tags of the incoming packet.

- The mpls/stack property (see [b-ITU-T H.248.54]) can be used to indicate the "Label Switched Path" (LSP) of the incoming packet.

## 6.3	Lookup-keys based on the Termination Type and TerminationID

Some simple lookup-keys can be derived from the type and TerminationID of the Termination hosting the Stream. The TerminationID (of a correspondent H.248 profile) could contain in general information elements which may be directly related to lookup-key elements.

Some examples for such lookup-keys are:

- The TerminationID of ETSI H.248 ETSI_BGF profile [b-ETSI TS 183 018] IP terminations include a generic field element called "interface". The semantic of this field is apparent from the name. This information can be used in the construction of a Stream’s lookup-key. Packets will match such a Stream only if they arrive on the "interface" (e.g. if semantic of logical or physical layer 3 or layer 2 interface would be used) indicated in the Termination ID. Packets arriving on other interfaces will not be delivered to the Stream, even if they match all other lookup-keys.

## 6.4	Overlapping Classification Rules

It is possible that a single incoming packet will match the classification-rules of more than one Stream. In such a scenario, the MG must choose one of these Streams and deliver the packet to it. How this choice is done is dependent on the use-case and cannot be generalized.

{Editor’s note: Do we want to put a general requirement that the choice is stable (i.e. that two packets with identical attributes will be delivered to the same Stream)? It is possible to think of some rare cases that do not need such a requirement, for example an IP-to-IP MG used for simple load-balancing. Contributions are solicited.}

An important special-case of overlapping classification rule happens when one classification-rule is completely contained in another. I.e. when:

- Classification-rule1 uses all the lookup-keys of classification-rule2.

- Classification-rule1 uses at least one additional lookup-key to those of classification-rule2.

- Classification-rule1 has exactly the same values as classification-rule2 for the lookup-keys that they share.

Under such a condition, every packet that matches classification-rule1 must also match classification-rule2. A generic behaviour would therefore be to always choose the more exact rule (i.e. classification-rule1) when a packet matches both. Otherwise, no packets will ever be delivered to the Stream corresponding to classification-rule1.

## 6.5	Post-classification filtering

Not all protocol elements that filter incoming packets must also be considered as part of the Stream’s classification-rule. It is possible to have some packet filtering occurring after the packets have been delivered to the appropriate Stream. Such post-classification filtering has the advantage that filtered packets are already associated with a Stream, and therefore can be counted by that Stream’s statistics. On the other hand, post-classification filtering suffers from the following drawbacks:

- It forces packet filtering to be performed in stages: first an initial classification based on the Streams’ classification-rules, then an additional filtering within the Streams. This can reduce performance and increase resource consumption in systems that can perform all stages of filtering at once (e.g. TCAM assisted filtering).

- It prevents having two Streams that are differentiated only by the post-classification filtering (as the classification-rule of both Streams would be identical).

The source-based filtering according to the properties of the gm package (see b-ITU-T H.248.43), is an example to filtering happening after classification. The existence of the gm/dp statistic (which counts the number of packets dropped due to gm filtering), seems to indicate that the filtering happens after the packets have already been delivered to a Stream.

## 6.6	Special consideration for ICMP packets

### 6.6.1	ICMP message types

The Internet Control Message Protocol (ICMP) ([IETF RFC 792] for ICMPv4 and [IETF RFC 4443] for ICMPv6 and [IETF RFC 4884] for ICMP extensions) defines error and informational messages types, which are carried in self-contained IP packets. There are ICMP messages:

- with end-to-end significance concerning the entire IP connection, typically sent back from destination host to the source host; and

- with hop-to-hop significance (e.g. scope on a particular network route).

### 6.6.2	Dependency with Context-level “IP topology” in H.248 MG

The created “IP topology” (see also [ITU-T H.248.64]) resulting from the H.248 context has potential impacts on ICMP handling. For instance, a H.248 context with (IP,IP) connection model in

- IPR mode

    - ICMP message processing defined by [IETF RFC 1812]

    - local ICMP message processing if hop-to-hop significance;

    - forwarding of ICMP messages if end-to-end significance.

- B2BIH mode

    - ICMP message processing defined by [IETF RFC 1122]

    - local creation (as destination host) and sending of ICMP messages in backward direction (as source host);

    - local delivery of incoming ICMP messages (to destination host);

    - forwarding of ICMP messages with L3+ information, if L3+ layer is not terminated by the MG (e.g., some ICMP messages are related to the end-to-end IP transport connection and must be therefore forwarded when the transport protocol is not terminated by the MG in B2BIH mode).

### 6.6.3	Constitution of ICMP message flows

Any ICMP message flow may be defined by the 2-tuple of {IP SA, IP DA}.

NOTE – Some ICMP messages are related to the IP transport connection (e.g., “Port Unreachable” error message), but there are not dedicated ICMP subflows defined on transport level.

It should be understood that there might be a difference between the granularity of

- ICMP message flows and their associated

- IP packet flows as processed on H.248 Stream-, Termination or Context level.

Some examples include:

- There might be a single IP connection between a Physical-to-IP access and IP-to-Physical trunking media gateway. Both H.248 MGs acting as IP host entities. Each H.248 IP Stream endpoint may be unambiguously identified by an IP transport connection endpoint.
The end-to-end ICMP messages would be already processed by the H.248 MGs in the “Context-less” stage (see Fig. 8/[ITU-T H.248.64]).

- Let’s consider an IP-to-IP border gateway enforced for media-aware processing functions in the case of RTP traffic. Each H.248 Stream would carry an RTP media flow. The underlying ICMP message flow may then cover multiple H.248 Streams or even multiple H.248 Contexts.

The IP network environment and/or the configured “IP topology” at H.248 Context level may affect the ICMP lookup-key (from H.248 Stream perspective). See the next sub-clause.

### 6.6.4	ICMP message handling from Stream perspective

Any error ICMP message is not delivered to a Stream according to the attributes of the ICMP packet itself. Instead:

- The attributes of the IP packet that generated the error or notification are used for the classification.

- The source and destination of the packet that generated the error are swapped before classification is performed.

For example, when classifying an ICMP packet concerning an IP packet whose source was X, the address X is matched against any lookup-keys on the destination address.

{Editor’s note: The above requirement is not accurate. The correct behaviour is that the ICMP error is delivered to the Stream that sent the original packet. Stage (2) will result in this behaviour only if the Stream is symmetric. Contributions are solicited.}

### 6.6.5	Blocking or rate limiting of ICMP traffic by H.248 MGs

ICMP is a well-known means for attacking IP networks. ICMP messages could be also (intentionally or unintentionally) blocked by H.248 MGs under specific H.248 Context configurations (see above). There might be then operational or interoperability impacts. See e.g. the discussion in [b-IETF draft-ietf-opsec-icmp-filtering], relevant if the H.248 MG would block or rate limit ICMP traffic.

Example:

An ICMP application using informational ICMP message types, like e.g. the Ping application (based on ICMP echo and echo reply messages), has an end-to-end significance, should be thus not locally blocked or delayed, but may be also misused as security threat (“Ping of death”).

# 7	Ordering of operations

The order in which the MG applies operations to packets, once they were assigned to a Stream, is important. For example, the gm package (see [b-ITU-T H.248.43]) allows the MGC to define filtering operations on incoming packets, while the nt/or statistic (see [ITU-T H.248.1]) allows the MG to report the number of octets received on a Stream. Whether the statistic is collected before or after filtering can affect the value of the statistic. This clause gives guidelines concerning the order in which different operations should be performed.

## 7.1	Assigning operations to protocol layers

### 7.1.1	Using the OSI Basic Reference Model as example

We can think of the packet processing within a Stream as happening in layers, corresponding to the layers of the OSI Basic Reference Model (BRM) [ITU-T X.200] and also NGN-BRM [ITU-T Y.2011].

Incoming packets traverse the layers bottom-to-top, while outgoing packets traverse the layers top-to-bottom. This notion is presented in Figure 1.

Figure 1 – Protocol layers of processing within an H.248 Stream 
(as an example of the OSI-BRM)

Figure 1 highlights scenarios where the H.248 MG only processes a subset of the entire protocol stack, e.g. up to the transport layer (i.e., L4+ agnostic packet processing for media agnostic configurations in H.248 border gateways). Alternatively up to the network layer (i.e., transport protocol (L4) agnostic packet processing like the IP router mode according [ITU-T H.248.64]).

An operation within a Stream should take place at the lowest layer where all the information required for an operation is available. Below are some examples for choosing the correct layer for applying an operation:

- The filtering of packets applied by the properties of the gm package (see [b-ITU-T H.248.43]) is based on network-layer and transport-layer information. It should therefore take place at the transport-layer.

{Editor’s note: It is not clear whether all gm based filtering should be applied at the transport-layer, or whether the filtering be split between the network and transport layers. Splitting the filtering can make complex logical relations between the different filtering elements more complex. Contributions are solicited.}

- The nt/or statistic (see [ITU-T H.248.1]) counts the number of octets received, at the transport-lyaer (i.e. excluding any network-layer headers). It should therefore be collected at the transport-layer.

- The ipocs/ipor statistic (see [b-ITU-T H.248.61]) counts the number of octets received at the IP layer. It should therefore be collected at the network-layer.

{Editor’s note: It is unclear whether nt/or and ipcs/ipor should be collected at L4 and L3 (as described above), or at L3 and L2 respectively. Theoretically, L2 has all the information it needs to collect ipocs/ipor (as anything above the L2 headers is at the IP layer). Likewise, L3 has all the information it needs to collect nt/or (as anything above the network-layer is transport-layer). Contributions are solicited.}

- The vlan/pri property (see [b-ITU-T H.248.56]) controls the priority bits appearing in the Ethernet header. It should therefore be applied at the data-link layer.

### 7.1.2	Notes regarding the IETF (Basic) Reference Model for IP

The legacy IETF protocol reference model may be described by Figure 1 / [IETF RFC 791], which is described in further detail by the RFCs for IP host ([IETF RFC 1122], [IETF RFC 1123]) and IP router ([IETF RFC 1812]) specifications.

The model may be called IETF-BRM and has a 1:1-relation to the OSI-BRM, but lacking protocol layers 5 and 6 (see Figure 2).

The IP protocol stacks in use of H.248 IP Streams in existing H.248 profile specifications may consider a refined reference model, by e.g. additional protocol layers between the application and transport layer. For instance, the application level framing protocol RTP in case of RTP traffic.

Figure 2 – IETF (Basic) Reference Model for IP

## 7.2	Ordering of operations within a layer

The list below gives the recommended order in which operations belonging to a single layer should take place. Note that this order is based on generalized considerations only. Any specific ordering requirements appearing in the procedures of a package should take precedence over this list.

- Maintaining any usage statistics (e.g. ipcs/ipor, see [b-ITU-T H.248.61]).

Counting the statistics first allows the MG to report the network’s conditions as accurately as possible, before these measurements are affected by the Stream’s operations.

- Applying any filtering, either explicitly controlled (e.g. through the gm package, see [b-ITU-T H.248.43]) or happening implicitly due to other operations (e.g. through the ipnapt package, see [b-ITU-T H.248.37]).

Filtering the traffic as soon as possible prevents wasting resources on packets that will be immediately dropped (and might even be considered malicious). Note that the resources being conserved are both MG resources (e.g. CPU) and Stream resources (e.g. the bandwidth budget of the Stream).

- Performing any rate-limiting (e.g. through the tman package, see [b-ITU-T H.248.53]) or traffic-shaping on the packets.

Performing rate-limiting early prevents wasting resources on packets that will be dropped. Performing traffic-shaping early prevents spikes in the usage of MG resources due to bursts in the traffic.

- Performing any other operations within the layer.

{Editor’s note: Are there additional generic principles for ordering operations within a layer? Contributions are solicited.}

The above recommended ordering is presented in Figure 2.

Figure 2 – Order of operations within a layer

## 7.3	Policy rule based description of packet operations

{Editor’s note: this clause provides initial text, indicating the problem statement concerning the order of operations and a policy rule based model. The solution to this problem would then be the specification/indication of an “order of precedence of policy rule enforcement”. Such an order may be then either inherent to the structure and specification of H.248 Descriptors or/and new H.248 protocol elements for the explicit specification. This is FFS. }

The various packet processing functions (“packet operations”), which are enabled in the H.248 MG bearer path, may be abstracted as policy rules. Each packet operation may be e.g. described as (generic) policy rule. The H.248 Streams Descriptor defines a number of policy rules for a particular Stream endpoint. An entire H.248 Context specification relates then to a series of policy rules which are executed on packets, when “handled by the Context”.

A generic policy rule would be comprised by policy condition(s), which would specify “how and when” an incoming/outgoing packet (“Lx-PDU in general”) could be identified, and the associated policy action(s) would specify operation(s) executed on the PDU itself or/and other operations in control-/management-/user-plane.

Figure 3 illustrates the example of a policy rule based description of packet operations by correlating an H.248 Context model with a correspondent H.248 MG packet path model.

Figure 3 – Order of operations (within a layer) – Unidirectional packet bearer traffic models using the concept of policy rules

The order of operations relates to the order of precedence of policy rule enforcement in the policy rule based description model.

# 8.	Processing costs between primary and secondary operations on packets (like for measurements)

## 8.1	Principal areas of operations on packets

The MG applies operations to packets, - or in general to protocol data units at layer x (Lx-PDU) of packet-switched traffic -, which may be categorized as primary and secondary (complementary) operations.

Primary operations:

- user plane related operations, related either directly to the end application like media format processing (e.g. audio transcoding, [ITU-T V.153] interworking) or indirectly by e.g. NAT traversal or QoS support (e.g., ToS marking);

and (typically optional)

Secondary, complementary operations:

- security related operations, i.e. packet processing from perspective of potential security threats against network infrastructure, user equipments, etc.;

- performance monitoring and measurement related operations, e.g. the considered capabilities by H.248.48 for RTP-based IP applications; or/and

- charging related operations, i.e. the generation and collection of packet based charging metrics.

The primary operations are typically enforced on the MG by the MGC through the use of “media descriptions” (i.e. SDP or H.248 properties) in the H.248 Stream Descriptor. These define the basic mode of operation with respect to “media aware/agnostic”, “transport protocol aware/agnostic” or/and other user plane stack information.

NOTE – For example, the MG could be located between a TLS and non-TLS security domain for the relay of MSRP-based Instant Messaging traffic. It may be then enforced for TLS/TCP/IP to non-TLS/TCP/IP user plane interworking, i.e. a transport protocol aware, but media agnostic mode of operation (because the MG should not know the application type of the TCP payload data).

The secondary operations are typically controlled via other H.248 protocol elements.

## 8.2	Processing costs

The protocol layer of the primary packet operation(s) could be identical to all secondary operations (LPriOp. = LSecOp.), but there might be also scenarios. For example the following protocol layer relations:

- LPriOp. > LSecOp., e.g. an RTP translator as primary operation and [b-ITU-T H.248.61] network layer measurements as secondary operation;

- LPriOp. < LSecOp., e.g. an [ITU-T H.248.64] IP hop function as primary operation and [ITU-T H.248.58] application layer measurements as secondary operation. See e.g. also Annex J in [b-ETSI TR 183 068].

The MG processing effort to enforce packet operations may be abstracted by cost factors CPriOp. and CSecOp.. It is typically expected that the primary operations determine the overall processing costs (e.g. a ratio of CPriOp. >> CSecOp.). However, there might be also H.248 Context configurations of the opposite, i.e. a packet processing cost ratio of CPriOp. << CSecOp., e.g. for above scenario (ii).

NOTE –	Example, type (ii) may require the application of “deep packet inspection” related packet operations, where the “depth” of processing or/and the number of secondary functions may just significantly increase the amount of MG resources.

Editor’s note: above text describes an abstract, qualitative cost model on a very high level. The cost factor is here mainly based on the “identification” part of a packet for executing the assigned operation. This is recognized as a significant cost factor due to the performance challenges for “wirespeed”, realtime packet identification in the MG packet bearer path (e.g. IP fast path), which may e.g. demand a deep inspection of higher layer protocol information (e.g. in case ii).

When describing a particular “packet operation (primary or secondary)” as “policy rule”, then the identification would related to conditions and the finally executed function(s) to action(s). A more detailed cost model could also consider further processing costs in the area of the evaluation of conditions or/and the execution of actions. E.g., the H.248.47-controlled, conditional reporting of statistics (as a “secondary operation”) may lead to extended “policy conditions” by addition of event-, time-, threshold- or/and other types of conditions, which could again represent significant processing costs per packet.

The purpose of this clause is to point out that there are a) different operational categories (primary, secondary), and b) that there might be different processing costs in  principle, but not in further elaborating on sub-level cost factors or even (implementation specific) quantitative estimates.

# 9	Filtering of TCP/IP traffic

{Editors note: This clause provides guidelines for the use of additional packages in order to define filters for TCP/IP traffic. The external packages used are:

- Gate Management (gm); defined in 7/[H.248.43]

- Destination Address/Port Filtering (dapf); defined in 8/[H.248.43]

- Incoming Protocol Filtering (ipf); defined in 9/[H.248.43]

- Incoming Filtering Behavior (ifb); defined in 11/[H.248.43]

- IP Layer Octets Count Statistics (ipocs); defined in 6/[H.248.61]

- IP Layer Packets Count Statistics (ippcs); defined in 7/[H.248.61]

}

## 9.1	Overview – TCP and IP filter types

### 9.1.1	IP filters

#### 9.1.1.1	IP filter categories

IP filter conditions are based on IP header information, and optionally on IP (connection) state information, dependent on an IP stateful or stateless filter type. IP filters imply (at least) a Layer 3 Header Inspection (L3HI).

The set of IP filter term types could be categorized on high level in:

- Stateless, IP address only based filters (source or/and destination address);

- Stateless, IP protocol type specific filters;

- Stateless, IP “QoS codepoint” specific filters (e.g. on fields like Type of Service, Differentiated Services or Traffic Class in IPv4/IPv6);

- Stateless, IPv4 specific filters on protocol options;

- Stateless, IPv4 specific filters on fragments;

- Stateless, IPv6 specific filters on extension headers; and

- others, not listed in above categories.

#### 9.1.1.2	IP connection filter types

An end-to-end IP connection is defined by the 2-tuple of L3 source/destination address values. Any IP connection filter type would comprise such a filter condition (beside optionally other conditions). The IPv6 header provides in addition the flow label element, which allows to discriminate IPv6 traffic flows within an IPv6 connection. E.g., an IPv6 specific flow filter would be based on the 3-tuple of L3 source and destination address values plus flow label.

### 9.1.2	TCP filters

#### 9.1.2.1	TCP filter categories

TCP filter terms are manifold, e.g. due to long experiences with well-known TCP security threats. Common to all TCP filters is the fact that the filter conditions are based on TCP header information, and optionally on TCP (connection) state information, dependent on a TCP statefull or stateless filter type. TCP filters imply consequently (at least) a Layer 4 Header Inspection (L4HI). The plethora of TCP filter term types could be categorized on high level in:

- Stateless, TCP port only based filters;

- Stateless, TCP flag (bits) specific filters;

- Stateless, TCP header or/and payload size specific filters;

- Statefull, TCP sequence/acknowledgement number specific filters; and

- Statefull, TCP flag (bits) specific filters; and

- others, not listed in above categories.

NOTE – The very majority of TCP filters, related to security belong, to category (1) and (2).

#### 9.1.2.2	TCP connection filter types

Typically TCP filtering is applied on TCP connections, which is defined by the 4-tuple of L3 source/destination address values and L4 source/destination port values. TCP connection filter types provide thus also IP filter term information

## 9.2	TCP/IP filter terms

For a Stream to represent a TCP/IP filter term it shall have a Local Descriptor and a Remote Descriptor. A stream missing either of those Descriptors indicates a still incomplete term that shall be ignored. As the filter does not perform any transformation of the traffic (with the exception of possibly blocking it), the Local and Remote Descriptors shall be identical. Any attempt to set different Local and Remote Descriptors shall be rejected using error #473 (Conflicting property values).

### 9.2.1	Setting the term’s match criteria

<TBD>

Bibliography

[b-ITU-T H.248.37]	ITU-T Recommendation H.248.37 (06/2008), Gateway control protocol: IP NAPT traversal package

[b-ITU-T H.248.41]	ITU-T Recommendation H.248.41 (05/2006), Gateway control protocol: IP domain connection package

[b-ITU-T H.248.43]	ITU-T Recommendation H.248.43 (06/2008), Gateway control protocol: Packages for gate management and gate control

[b-ITU-T H.248.53]	ITU-T Recommendation H.248.53 (06/2008), Gateway control protocol: Traffic management packages

[b-ITU-T H.248.54]	ITU-T Recommendation H.248.54 (08/2007), Gateway control protocol: MPLS support package

[b-ITU-T H.248.56]	ITU-T Recommendation H.248.56 (08/2007), Gateway control protocol: Packages for virtual private network support

[b-ITU-T H.248.61]	ITU-T Recommendation H.248.61 (03/2009), Gateway control protocol: Packages for network level H.248 statistics

[b-ITU-T V.153]	ITU-T Recommendation V.153 (03/2009), Interworking between ITU-T T.38 and ITU-T V.152 using IP peering for real-time facsimile services

[b-ETSI TS 183 018]	ETSI TS 183 018 v3.5.2 (01/2010), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: H.248 Profile version 3 for controlling Border Gateway Functions (BGF) in the Resource and Admission Control Subsystem (RACS); Protocol specification

[b-ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (2009-08), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines

[b-IETF RFC 4570]	IETF RFC 4570, Session Description Protocol (SDP) Source Filters

[b-IETF draft-ietf-opsec-icmp-filtering]	IETF draft-ietf-opsec-icmp-filtering-01 (2009), Recommendations for filtering ICMP messages

__________________