| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4069 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.248.82: Editorial Updates |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document proposes editorial changes to draft ITU-T Recommendation H.248.82.

2.	Discussion

During a review of draft H.248.82 the Contributors identified a number of minor editorial issues. As such the Contributors propose to update the text to fix these issues. The text is identified below.

3.	Proposal

It is proposed to accept the following changes. Changes are marked up against TD 542R1/WP2 from the July, Geneva SG16 meeting.

Draft new ITU-T Recommendation H.248.82 (ex. H.248.ECN)

Gateway Control Protocol: Explicit Congestion Notification Support

…

## 6.1	Basic ECN control loop

[IETF RFC 3168] defines an addition to IP for ECN support. This support relates to a unidirectional connection model, i.e., IP traffic in IP source host to IP destination host direction. Figure 1 illustrates the basic ECN control principle. The control loop may be abstracted by four components in the user plane:

- the congestion notification information flow in source to destination host direction;

- the feedback information flow in reverse direction, back to the traffic source;

- the actuating elements in case of indicated congestion (typically located in the source host); and

- the sensor elements, responsible for indicating congestion (at a network route, in an IP hop) by ECN marking in the IP path (see (1)).

There might be a further component in the control plane for:

- the indication and negotiation of a congestion control.

NOTE – 	The thick arrow shows the unidirectional IP packet path (where hop entities may provide ECN-CE marking. The thin arrow in reverse direction indicates protocol X packets in feedback direction.

Figure 1 – ECN supporting IP network (H.248 bearer plane) – 
Basic model of the ECN control loop

In Figure 1 the control loop spans two ECN-capable IP host entities with information flows in both directions, requiring a bidirectional communication path. There are different protocols in use for ECN (Figure 1) in each direction:

- IPv4 or IPv6 in the source-to-destination host direction for carrying ECN indications;

- Higher layer protocols in the reverse direction. This is normally the applied IP transport protocol if extended (like TCP) or designed for ECN support (like DCCP, SCTP); or a L4+ protocol (like RTCP in case of UDP transport).

H.248 MG entities may be involved in ECN control loops in various ways. There is potential use cases for all four of the above indicated protocol groups to be involved in the control loop. Figure 1 outlines some possible MG roles.

…

## 6.5	ECN interworking scenarios

For traffic and network engineering of IP networks with regards to ECN domains, the H.248 MG may play the role of an ECN-capable IP host endpoint or be located within the ECN IP path, as introduced in clause 6.1. ECN defines multiple code points for ECN marking in case of ECN Capable Transport (ECT), termed as ECT(0), ECT(1) and ECT(random) code points.

For traffic and network engineering of IP networks with regards to ECN domains, the ECT code point options and the various possible MG behaviours may lead to different ECN interworking scenarios. Some basic network configurations are illustrated below.

Figure 7 provides the example of a peering scenario. For instance, there may be different ECT code points in use in each ECN domain. The H.248 MG may then be requested to remark ECN code points in case of the end-to-end ECN control loop approach (option I.2 below).

NOTE – The IP topologies are basically associated with the H.248 Stream level. Above figure illustrates an example on Context level (referred to the IP topology models according clause 3.2.1 (B2BIH) and clause 6.5.1 (IP router) of [ITU-T H.248.64]).

Figure 7 – Peering separate ECN domains by H.248 (IP,IP) MGs

Each ECN domain may be also completely separated by limiting the control loops on each domain (option I.1).

Figure 8 illustrates another configuration, the example of an H.248 (IP,IP) MG located within the IP path of a single ECN domain.

Figure 8 – H.248 (IP,IP) MGs located in the IP path of a single ECN domain

The H.248 MG may be required to enforce different ECN support behaviour, like transparent forwarding of ECN information in IP headers, or the active marking of ECN bits due to “local congestion information”, or the reading and collecting of ECN information for local H.248 statistics, etc.

## 7	ECN for RTP-over-UDP Support Package

|  | Package Name: | ECN for RTP-over-UDP Support Package |
| --- | --- | --- |
|  | Package ID: | ecnrous (0x0x010c) |
|  | Description: | This package allows a MGC to indicate to a MG that it shall support Explicit Congestion Notification (ECN) for RTP over UDP (as defined by [IETF ECN]) on a particular Termination / Stream. |
|  | Version: | 1 |
|  | Extends: | None. |

## 7.1	Properties

### 7.1.1	ECN Enabled

|  | Property Name: | ECN Enabled |
| --- | --- | --- |
|  | Property ID: | ecnen (0x0001) |
|  | Description: | This property allows the MGC to indicate whether or not ECN is enabled on a Stream. |
|  | Type: | Boolean |
|  | Possible values: | True ECN is enabled False ECN is not enabled |
|  | Default: | False. |
|  | Defined in: | Local/Remote |
|  | Characteristics: | Read/Write |

### 7.1.2	Congestion Response Method

|  | Property Name: | Congestion response method |
| --- | --- | --- |
|  | Property ID: | crm (0x0002) |
|  | Description: | When RTP packets are received with ECN-CE marks from the remote side or local congestion is experienced, the sender and/or receivers MUST react with congestion control as-if those packets had been lost. However there are different control methods which a MG could use. This property allows the MGC to request different types of control. These types are defined in clause 7.3.3 / [IETF ECN]. |
|  | Type: | Enumeration |
|  | Possible values: | "SDCC" (0x0001) Sender Driven Congestion Control "RDCC" (0x0002) Receiver Driven Congestion Control "Hybrid" (0x0003) A hybrid mechanism |
|  | Default: | Provisioned. |
|  | Defined in: | Local/Remote |
|  | Characteristics: | Read/Write |

### 7.1.3	Initiation Method

|  | Property Name: | Initiation Method |
| --- | --- | --- |
|  | Property ID: | initmethod (0x0003) |
|  | Description: | This property indicates which ECN initiation method is to be used. This property is equivalent to the "init-value" defined by the ABNF grammar defined by clause 6.1/ [IETF ECN]. If the MGC requires that the MG choose the any appropriate initiation value it shall provide the list of possible value as an over specified list. |
|  | Type: | String |
|  | Possible values: | As per "init-value" in clause 6.1/ [IETF ECN]. "rtp", "ice" or "leap". This package also defines an additional value "inactive" that may be used with this property or for the "init-value" parameter if SDP is used. "Inactive" indicates that no ECN initiation is performed (and thus the ECN procedures in clause 7.6.3.1 do not apply) for the particular RTP stream. |
|  | Default: | Provisioned. |
|  | Defined in: | Local/Remote |
|  | Characteristics: | Read/Write |

### 7.1.5	ECT Marking

|  | Property Name: | ECT Marking |
| --- | --- | --- |
|  | Property ID: | ectmark (0x0005) |
|  | Description: | This property indicates whether the MG should set or read ECN marks, and should therefore provide the role of an ECN-capable IP source or destination host (Note1). This property is equivalent to the "ect" element defined by the ABNF grammar defined by clause 6.1 / [IETF ECN]. NOTE 1 – There is also an ECN-CE marking function (besides ECT marking), which may be supported by H.248 MGs and interim IP devices. Such marking due to experienced local congestion is out of scope of this property. |
|  | Type: | Enumeration |
|  | Possible values: | As per "ect" clause 6.1/[IETF ECN]: "1" (0x0001) "0" (0x0002) "Random" (0x0003) |
|  | Default: | "0". |
|  | Defined in: | Local / Remote |
|  | Characteristics: | Read/Write |

### …

### 7.4.7	Extended Highest Sequence number

|  | Statistic Name: | Extended Highest Sequence number |
| --- | --- | --- |
|  | Statistic ID: | ehsn (0x0007) |
|  | Description: | This statistic provides the highest sequence number seen when sending this report, but with additional bits, to handle disambiguation when wrapping the RTP sequence number field for a particular SSRC within the Stream. Each position of the sub-list is associated with a SSRC at related position in the ssrc statistic. See clause 7.4.2 / [IETF ECN] for more details. |
|  | Type: | Sub-list of integer |
|  | Possible values: | 0 and upwards. |
|  | Level: | Stream |

## 7.5	Error Codes

None.

## 7.6	Procedures

### 7.6.1	Topology

ECN functionality may be used with different RTP session topologies. The relationship of these topologies with ECN procedures is defined in clause 3.2 / [IETF ECN].  Through the MGC setting the combination of Terminations/Streams in a Context, the properties in the "ECN for RTP-over-UDP Support" and other elements in the Local and Remote Descriptors the MG is able to determine the correct ECN behaviour.

### 7.6.2	Negotiation of the capability to use ECN with RTP/UDP/IP

As per [IETF ECN] a MG supporting ECN with RTP/UDP/IP requires different RTCP extensions:

- RTP/AVPF [RFC4585] transport layer feedback format for urgent ECN information,

- RTCP XR [RFC3611] ECN summary report block type for regular reporting of the ECN marking information.

Thus in addition to the methods for indicating/negotiating ECN described below the MGC shall send other information (i.e. SDP m and a lines) indicating the support of RTP/AVPF [RFC4585] and RTCP XR [RFC3611] as well as the appropriate address information for the Stream.

Note: In some cases (i.e. where timely feedback is not required and usage of the leap of faith initialization method) transport layer feedback for urgent ECN information may not be required. See clause 3.3 / [IETF ECN] for further information on the need for transport layer feedback.

In order to be notified of failures the MGC shall also set the "ECN failure" (ecnrous/fail) event.

To be able collect and Audit information regarding the use of ECN the MGC should set the "Source" (ecnrous/ssrc), "CE Counter" (ecnrous/cecount), "ECT 0 Counter" (ecnrous/zero), "ECT 1 Counter" (ecnrous/ectone), "Not-ECT Counter" (ecnrous/notetc), "Lost Packets Counter" (ecnrous/lost) and the "Extended Highest Sequence Number" (ecnrous/ehsn) Statistics.

If the MGC requires a more dynamic method of discovering the values of the above statistics it may utilise conditional statistic reporting [ITU-T H.248.47]. For example if the MGC requires notification of the reception of a CE marked packet then it should set the "Conditional Reporting" (scr/cr) event with the statistic identifier value set to "ecnrous/cecount" together with an appropriate condition parameter.

…

#### 7.6.1.2	ICE Parameter to Signal ECN Capability

As per clause 6.4 / [IETF ECN] an Internet Connectivity Exchange (ICE) [IETF RFC5245] flag may be used to indicate ECN support for ICE. However it is assumed that this attribute is used between MGCs when they have knowledge that the MG supports ECN functionality. This may be known through provisioning or an Audit of the Packages Descriptor indicating the support of the "ECN Package".

### 7.6.2	Initiation and initial verification of ECN capable transport

The use and initiation of ECN on a Termination / Stream is mainly determined via the setting of "ECN Enabled" equal to "True" on a Termination/Stream in combination with an "Initiation Method". [IETF ECN] defines three initiation methods "rtp", "ice" and "leap". This package defines an additional method "inactive" which indicates that ECN initiation procedures should not be started. Other properties may be examined in the Local and Remote Descriptors to determine media handling that may have an impact on ECN procedures and error handling. For example to determine:

- If there is a change in packetisation (segmentation/reassembly),

- If there is transcoding.

NOTE: As per [IETF ECN] a transcoding scenario requires the use of ECN properties to split the ECN connection in two parts, each with its own negotiation.

If "ECN Enabled" equal to "True" and "Initiation Method" equal to "inactive" (either by default or specification) is set on the same stream (and direction) on both Terminations of a Context with no other ECN related properties set then the MG shall not perform ECN initiation. It shall behave as an ECN aware MG and copy ECN bits (including CE information) from ingress packets to the egress one.

If "ECN Enabled" is equal to "True" and "Initiation Method" equal to "inactive" (either by default or specification) is set on the same stream (and direction) on both Terminations of a Context with no other ECN related properties set, but the MG performs packet segmentation or reassembly, without media transcoding, the MG derives ECT and ECN-CE marks in the egress stream from the corresponding ones in the ingress stream, as specified in [IETF ECN].

If "ECN Enabled" is equal to "True", "Initiation Method" is set a value other than "inactive" and other ECN related properties are set then the MG shall perform ECN initiation and processing. The Termination at which these properties are set shall behave as an ECN endpoint, i.e. it shall not pass ECT markings transparently to its egress even if the corresponding termination has identical stream properties.

NOTE -The MGC may determine that ECN processing is required as a result of the need for transcoding.  [IETF ECN] indicates that where transcoding occurs, a middlebox (i.e. the MG) shall process ECN as separate ECN Endpoints. ECN processing may also be required due to independent ECN negotiation at the call control layer, independent of transcoding.

ECN processing may also be triggered by setting "ECN Enabled" equal to "True" on a single Termination/Stream and "ECN Enable" equal to "False" on the opposite Termination/Stream. If behaviour other than the specified default behaviour is required the properties in this package related to that behaviour shall be signalled.

NOTE - the support of clause 7.2.2 / [IETF ECN] is FFS. [ITU-T H.248.50] supports STUN and ICE but does not support a ECN field.

As per clause 7.2.1 and 7.2.3 / [IETF ECN] the transport level negotiation of ECN may be unsuccessful and this should be logged. The ECN failure event (ecnrous/fail) with failure type "INIT" is used to indicate this to the MGC.

___________________