| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4078 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21, 22 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editors |  |  |  |
| Title: | Invitation of comments on draft Rec. ITU-T H.IRP ¦ ISO/IEC 29177 |  |  |  |
| Purpose: | Proposal / Discussion |  |  |  |

Summary

This contribution invite comments on draft Recommendation ITU-T H.IRP | ISO/IEC 29177 which is contained in TD 616 (WP2/16).

- Introduction

At the last meeting of Q22/16 (Geneva, 9 – 13 May 2011) during the first IoT-GSI event, Q22/16 produced revised common text of ITU-T H.IRP | ISO/IEC 29177 and sent to ISO/IEC JTC 1/SC 31 as 3rd CD ballot comments.  After the comment resolution meeting on this, ISO/IEC JTC 1/SC 31/WG 6 decided to initiate four month FCD ballot with the text in the TD 616 (WP2/16).

- Discussion

The latest text for ITU-T H.IRP | ISO/IEC 29177 on ITU-T side is contained in TD 12R1 (IoT-GSI) and changes made by ISO/IEC JTC 1/SC 31/WG 6 from this text is attached to this contribution with the revision marks.  Only editorial changes are made and one new definition was added.

- Conclusion

We invite comments from experts to produce FCD ballot comments on ITU-T H.IRP | ISO/IEC 29177 from ITU-T side by 30 September 2011.

ATTACHMENT

CONTENTS

Page

Introduction	1

Discussion	1

Conclusion	1

1	Scope	3

2	Normative references	3

2.1	Identical Recommendations | International Standards	4

2.2	Additional references	4

3	Definitions	4

3.1	Imported definitions	4

3.2	Additional definitions	4

4	Abbreviations and acronyms	4

5	Overview of the ID resolution protocol	5

6	Application-specific OID resolution process	6

6.2	OID transformation rules	6

6.3	ID Structure Information	6

6.4	Operation of a client	6

6.5	Format of NAPTR resource record	7

6.6	Operation of application-specific OID resolution servers	7

7	ID resolution process	8

7.1	Overview	8

7.2	Query to ID resolution server	8

7.3	Response from ID resolution server	8

7.4	Configuration of NAPTR resource record	8

7.5	Operation of a client	9

7.6	Operation of ID resolution servers	9

A.1	Assumption	10

A.2	Configuration of NAPTR resource records	10

A.3	Operation example	11

INTERNATIONAL STANDARD ISO/IEC 29177

RECOMMENDATION ITU-T H.IRP

Information technology – Automatic identification and data capture technique –
Identifier resolution protocol for multimedia information access triggered by tag-based identification

Summary

This Recommendation | International Standard specifies the identifier (ID) resolution protocol for multimedia information access triggered by tag-based identification.  This ID resolution protocol is used to retrieve the location and access method of the multimedia information associated with an ID.

Keywords

identifier, IRP, resolution, tag-based identification

# Introduction

This Recommendation | International Standard specifies the identifier (ID) resolution protocol (IRP).  This ID resolution protocol is used to retrieve the location and access method of the multimedia information associated with an ID.

Clause 6 provides an overview of the ID resolution protocol.

Clause 7 specifies the application-specific OID resolution process which will follow the general OID resolution process defined in [ITU-T X.672].

Clause 8 specifies the ID resolution process.

There are alternatives techniques to meet the use case addressed by this Recommendation | International Standard.  One of those techniques is the use of EPC and ONS. Those interested in this technique are encouraged to contact GS1 for further information.

1	Scope

This Recommendation | International Standard defines the identifier (ID) resolution protocol for multimedia information access triggered by tag-based identification which is described in ITU-T Recommendations F.771 and H.621.

The identifier resolution protocol (IRP) in this Recommendation | International Standard consists of two processes: an application-specific object identifier (OID) resolution process which is specified in [ITU-T X.672] and an identifier resolution process.

2	Normative references

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

- 2.1	Identical Recommendations | International Standards

–	Recommendation ITU-T X.668 (2009) | ISO/IEC 9834-9:2009, Information technology – Open Systems Interconnection – Procedures for the operation of OSI Registration Authorities: Registration of object identifier arcs for applications and services using tag-based identification.

–	Recommendation ITU-T X.672 ISO/IEC 29168-1, Information technology – Open Systems Interconnection – Object Identifier Resolution System (ORS)

- 2.2	Additional references

- ITU-T Recommendation F.771 (2008), Service description and requirements for multimedia information access triggered by tag-based identification

- ITU-T Recommendation H.621 (2009), Architecture of a system for multimedia information access triggered by tag-based identification

- IETF RFC 1035:1987, Domain names – Implementation and specification.

–	IETF RFC 3403:2002, Dynamic Delegation Discovery System (DDDS) Part Three: The Domain Name System (DNS) Database.

3	Definitions

For the purposes of this Recommendation | International Standard, the following definitions apply.

- 3.1	Imported definitions

3.1.1	This Recommendation | International Standard uses the following term defined in ITU-T Rec. X.672 | ISO/IEC 9834-9:

a) application-specific OID resolution process;

b) general OID resolution process;

3.1.2	This Recommendation | International Standard uses the following term defined in ITU-T Rec. F.771:

a) ID tag;

b) ID terminal

- 3.2	Additional definitions

3.2.1	ID Resolution Protocol (IRP): A protocol used to find Uniform Resource Locator (URL) of multimedia information associated with ID

3.2.2	ID Resolution Server (IRS): A server that contains location and access method information of multimedia information and resolves an ID into URL of associated information.

3.2.3	Mobile AIDC service: use of a Mobile AIDC application to provide a value to end-users.

4	Abbreviations and acronyms

For the purposes of this Recommendation | International Standard, the following abbreviations apply.

C2U		Code to URL

DNS	Domain Name System

FQDN	Fully Qualified Domain Name

HLC	High Level Code

ID		Identifier

IRP		ID Resolution Protocol

IRS		ID Resolution Server

MIDS	Multimedia Information Delivery Server

NAPTR	Naming Authority Pointer

RegExp	Regular Expression

TLC		Top Level Code

URL	Uniform Resource Locator

5	Overview of the ID resolution protocol

This clause defines an overall architecture of IRP for multimedia information access triggered by tag-based identification based on the architecture described in clause 6 of [ITU-T H.621], requirements described in clause 7.3 of [ITU-T F.771] and OID Resolution System in [ITU-T X.672 | ISO/IEC 29168-1].

Figure 1 shows the overall architecture of IRP.

Figure 1 – Overall architecture of ID resolution protocol

The overall ID resolution protocol is composed of two processes: application-specific OID resolution process and ID resolution process.  The general OID resolution process is out of scope of this Recommendation | International Standard.

In multimedia information access process triggered by tag-based identification, an ID terminal reads ID from an ID tag such as a barcode and RFID (Step 1).  OID may be available directly from the tag or may be added by the tag reader and its software before the ID is passed to the ID terminal.  Or application software on the ID terminal may add an OID based on other context information.

If the ID terminal does not know how to process the ID, it firstly sends the OID to an ORS server. The ORS server sends back to the ID terminal ID processing information which contains converting rule of ID into a specific format for ID resolution protocol (Step 2 and 3).  If the ID terminal knows how to process the ID read from the ID tag, it can skip both general and application-specific OID resolution process.

Then, the ID terminal sends the ID in a suitable format for ID resolution to an ID resolution server (Step 4). The ID resolution server gets the ID as an input, and returns a URL of multimedia information in contained the Multimedia Information Delivery Server (MIDS).

Lastly, the ID terminal accesses MIDS to obtain corresponding multimedia information.

This Recommendation | International Standard specifies the application-specific OID resolution process for multimedia information access triggered by tag-based identification and the ID resolution process for Step 3 and 4 in Figure 1.

The general OID resolution process is defined in [ITU-T X.672 | ISO/IEC 29168-1] and it will return some information which will be used in the application-specific OID resolution process for multimedia information access triggered by tag-based identification.  An ID resolution process will follow this application-specific OID resolution process.

6	Application-specific OID resolution process

6.1	Overview

The application-specific OID resolution process works based on the DNS protocol defined in IETF RFC 1035 and NAPTR resource records defined in IETF RFC 3403.

The application-specific OID resolution process is initiated when the general OID resolution process returns an ORS service type “ORS+TINF”.  The returned NAPTR resource record with this service type contains a location address of application-specific ORS resolution server. An ID terminal should send a DNS query to this server with an OID as an input.  The result of this application-specific OID resolution process is ID structure information in RegExp field of a responding NAPTR resource record.

- 6.2	OID transformation rules

OID should be converted in the Fully Qualified Domain Name (FQDN) form to be sent to the application-specific OID resolution server.  The transformation rule of OID into FQDN is defined as following three steps:

Step 1: separate each arc with period (“.”)

Step 2: reverse the order of the arcs

Step 3: add “.oid-res.org.” at the end of result from Step 2

For example, OID {2 27 1} will be converted as “1.27.2.oid-res.org.”.

- 6.3	ID Structure Information

The ID Structure Information is stored in NAPTR Resource Records at the application-specific ORS server.  The ID Structure Information is expressed in RegExp field of NAPTR resource record using POSIX Regular Expression. This rule is substitution syntax of ID into a certain format for ID resolution process.

For this purpose following new Service field values of NAPTR resource record shall be defined.

IRP+FFT: 	Fixed Form Type identifier scheme. It indicates that RegExp field contains a rule for converting an ID into a certain format for ID resolution process and it needs no more query to an application-specific ORS server

IRP+VFT: 	Variable Form Type identifier scheme. It indicates that RegExp field contains a rule for extracting some fields from an ID and it needs more query to an application-specific ORS server

For example, the following NAPTR resource record is for FFT identifier scheme.

0 100 “U” “IRP+FFT” “!^(.{3})(.{5}).{3}$!\\2.\\1.anydomain.example.com.!”.

Applying the above RegExp to an ID, of which total length is 11 bits, a text string is obtained in the form of “nnnnn.nnn.anydomain.example.com.” where n is either ‘0’ or ‘1’.

Here is another example.  The following NAPTR resource record is for VFT identifier scheme.

0 100 “U” “IRP+VFT” “!^(.{2}).{11}$!\\1!”.

Applying the above RegExp to an ID, the first 2 bits of the ID is obtained which total length is 13 bits.

- 6.4	Operation of a client

Figure 2 shows the operation of a client in an application-specific OID resolution process.

Figure 2 – Operation of application-specific OID resolution process

The client converts OID into an FQDN form as defined in 7.1 of this Recommendation | International Standard and sends it to the application-specific ORS server.  When the client receives a NAPTR resource record with the Service field value of “IRP+FFT”, it applies the rule in RegExp field on the ID and obtains a new text value suitable for ID resolution process. When the client receives a NAPTR resource record with the Service field value “IRP+VFT”, it applies the rule in RegExp field on the ID and obtains a new text value which is added to the front of OID in FQDN form.

For example, if OID in FQDN form is “99.27.2.oid-res.org.” and the substitution result of applying the RegExp field obtained by the query to application-specific ORS server to the ID value is “5”, then new FQDN form “5.99.27.2.oid-res.org.” for the subsequent query will be generated.

- 6.5	Format of NAPTR resource record

The contents of a NAPTR resource record are formatted as in the following example:

| Order | Preference | Flags | Service | RegExp | Replacement |
| --- | --- | --- | --- | --- | --- |
| 0 | 100 | u | IRP+FFT | !^(.{2}).{11}$!\\1! | . |

The application-specific OID resolution server operators should obey the following rules to configure the NAPTR resource record:

a)	The order field should be zero

b)	The preference field should be non-negative integer

c)	The flags field should be set to “u”, indicating that the RegExp field contains URL

d)	The service field should be set to “IRP+VFT” or “IRP+FFT” (see 7.2)

e)	The RegExp field specifies an ID structure information.  The value of this field should be the string !^ID structure information!

- 6.6	Operation of application-specific OID resolution servers

The application-specific OID resolution server should behave as a DNS server which is defined in IETF RFC 1035.

NOTE – An example configuration and operation of application-specific OID resolution process is given in Annex A.

7	ID resolution process

- 7.1	Overview

The ID resolution process uses DNS protocol as defined in IETF RFC 1035 and NAPTR resource records as defined in IETF RFC 3034.

The purpose of this process is to find location and access method information for the multimedia information associated with an ID by using DNS.  In order to use DNS, the ID should be converted into an FQDN format that DNS can understand.

DNS servers used for this ID resolution process may contain a delegation pointer to another DNS server in NS records or URL of multimedia information associated with ID in NAPTR Resource Record.

The ID terminal reads an ID from an ID tag.  Then it knows how to convert the ID into the FQDN form, it converts the ID into an FQDN form for sending it as part of a DNS query.  If the user terminal does not know how to convert the ID into an FQDN form, it should initiate an OID resolution process as specified in [ITU-T X.672 | ISO/IEC 29168-1].

The NAPTR type is used for the DNS query.  This DNS query is sent to the default DNS server (local DNS server).  If the default DNS server has a NAPTR resource record for that ID, it returns the NAPTR resource record which includes the URL of the multimedia information content.  If the default DNS server does not have any NAPTR resource record for that ID, it sends a query to the ROOT DNS server and the ROOT DNS server returns the IP address of another DNS server which has been delegated from the delegation hierarchy of the ID.  The default DNS server then sends another query to this new address. This procedure runs recursively until a NAPTR resource record can be obtained.  When the default DNS server successfully receives a NAPTR resource record, it sends the NAPTR resource record back to the ID terminal and the ID resolution process is completed.

In case of using EPC Tag Data in [b-TagData], ID resolution process shall follow the Object Name Service [b-ONS].

- 7.2	Query to ID resolution server

This clause specifies the DNS query format.  The ID in this query should be presented in an FQDN (Fully Qualified Domain Name) format and the query type should be set to “NAPTR”.

The rule to convert an ID into an FQDN format is defined as follows:

STEP 1:	Put dots (“.”) between each field, producing for example, 101010.101.11

STEP 2:		Convert binary digits into hexadecimal, producing for example, 2A.5.3

STEP 3:		Reverse the order, producing for example, 3.5.2A

STEP 4:		Append the string “.anydomain.example.com.”, producing for example, 3.5.2A.anydomain.example.com.

NOTE – The anydomain.example.com here means the domain name for a particular identifier scheme.  Each identifier scheme should have a unique domain name to use ID resolution protocol in this Recommendation | International Standard.  If OID resolution process is performed, this converting process automatically done by application-specific OID resolution process.

- 7.3	Response from ID resolution server

This clause specifies the DNS response format.

The URL of multimedia information associated with ID is contained in RegExp field.  The rule for setting each field of NATPR resource record is specified in 8.4.

- 7.4	Configuration of NAPTR resource record

The content of a NAPTR resource record is formatted as in the following example:

| Order | Preference | Flags | Service | RegExp | Replacement |
| --- | --- | --- | --- | --- | --- |
| 0 | 100 | u | C2U | !^.*$!http://anydomain.example.com/example1.html! | . |

ID resolution server operators should obey the following rules to configure the NAPTR resource record:

a)	The order field should be zero

b)	The preference field should be non-negative integer

c)	The flags field should be set to “u”, indicating that the RegExp field contains a URL

d)	The service field should be set to “C2U”

e)	The RegExp field specifies a URL of multimedia information.  The value of this field should be the string !^.*$!URL of multimedia content!

NOTE – NAPTR record with ‘C2U’ in Service field should contain a valid URL in the RegExp field.

- 7.5	Operation of a client

The client should use the following procedure to interpret the results returned by a DNS query:

STEP 1:	From the results of the query, select those NAPTR resource records which have flag field value “u” and service field value “C2U”.  If there is no such record, stop

STEP 2:	From the result from STEP 1, select those records having the lowest value in the preference field

STEP 3: 	From the result form STEP 2, select a record randomly

STEP 4: 	Extract the URL from the record from STEP 3, by extracting the substring between the “!^.*$!” and the “!”

STEP 5: 	Get multimedia contents using the URL form STEP 4

STEP 6: 	If STEP 5 is not successful, go back to STEP 3, using a different record from the STEP 2.  If all records form STEP 2 have been tried, go back to STEP 2 using record from STEP 1 having the next lowest value in the preference field.  If all records from STEP 1 have been tried, stop

- 7.6	Operation of ID resolution servers

The ID resolution server should behave as DNS server which is defined in IETF RFC 1035.

NOTE – An example configuration of ID resolution servers is available in Annex B

Annex A

Configuration and operation of application-specific OID resolution process

(This annex forms an integral part of this Recommendation | International Standard)

- A.1	Assumption

This example used the ID scheme defined in [Recommendation ITU-T H.IDscheme | ISO/IEC 29174-1] and it is assumed that OID for this ID scheme is {2 27 99} and domain name for application-specific OID resolution server for [Recommendation ITU-T H.IDscheme | ISO/IEC 29174-1] is “idscheme.example.com”.

NOTE – Therefore, the result of general OID resolution process will be “!^.*$!99.27.2.idscheme.example.com!

It is assumed also that “.code.example.com.” is the domain name for the particular ID scheme given as an example here.

- A.2	Configuration of NAPTR resource records

The application-specific OID resolution server should have following resource records.  These NAPTR resource records can be stored in the distributed servers.

NAPTR resource record 1:

99.27.2.idscheme.example.com. IN NAPTR 0 100 “U” “IRP+VFT” “!^ (.{4})(.{16}) (.{4}).{104}$!\\3!”.

NAPTR resource record 2:

09.99.27.2.idscheme.example.com. IN NAPTR 0 100 “U” “O2U+FFT” “!^(.{4})(.{16}) (.{4})(.{8})(.{96})$!\\5.\\4.\\3.\\2.\\1.code.example.com!”.

NAPTR resource record 3:

110.99.27.2.idscheme.example.com. IN NAPTR 0 100 “U” “O2U+FFT” “!^(.{4})(.{16}) (.{4})(.{24})(.{80})$!\\5.\\4.\\3.\\2.\\1.code.example.com!”.

NAPTR resource record 4:

11.99.27.2.idscheme.example.com. IN NAPTR 0 100 “U” “O2U+FFT” “!^(.{4})(.{16}) (.{4})(.{40})(.{64})$!\\5.\\4.\\3.\\2.\\1.code.example.com!”.

NAPTR resource record 5:

12.99.27.2.idscheme.example.com. IN NAPTR 0 100 “U” “O2U+FFT” “!^(.{4})(.{16}) (.{4})(.{56})(.{48})$!\\5.\\4.\\3.\\2.\\1.code.example.com!”.

NAPTR resource record 6:

13.99.27.2.idscheme.example.com. IN NAPTR 0 100 “U” “O2U+FFT” “!^(.{4})(.{16}) (.{4})(.{72})(.{32})$!\\5.\\4.\\3.\\2.\\1.code.example.com!”.

NAPTR resource record 7:

14.99.27.2.idscheme.example.com. IN NAPTR 0 100 “U” “O2U+FFT” “!^(.{4})(.{16}) (.{4})(.{88})(.{16})$!\\5.\\4.\\3.\\2.\\1.code.example.com!”.

The NAPTR resource record 1 contains the rule for extracting specific fields.  In the example, this rule will extract the 4 bits class field from the ID.  The NAPTR resource record 2, 3, 4, 5, 6, and 7 contains the rule for converting an ID into an FQDN form for class A, B, C, D, E and F respectively.

- A.3	Operation example

It is supposed that the client has obtained an OID as {2 27 99} and an ID as “00001901000000000000000259FD720EHEX” from an ID tag, its reader or the reader software.

NOTE – The first four bits, HLC is 0HEX, the next 16 bits, TLC is 0001HEX, the next four bits and Class is 9HEX.

STEP 1:		When the client converts this OID {2 27 99} into an FQDN form as specified in 7.1, produces “99.27.2..oid-res.org.” and sends it to the application-specific ORS server.

NOTE – OID arc is converted to “decimal” string as opposed to the case of converting the ID binary digits into “hexadecimal” string as specified in step 2 of 8.2

STEP 2:		The application-specific ORS server returns NAPTR resource record 1 (see A.2) and the client extracts service field and RegExp from the NAPTR resource record.

STEP 3:		The client applies regular expression from STEP 2 to the ID and obtains the binary string “10012” for the third bit field, “(.{4})” and converts it to decimal number “9”.

NOTE – The first bit field “(.{4})” is “00002”. The second bit field, “(.{16})” is “0000000000000012”.

STEP 4:		The client check service (= “IRP+VFT”) field and adds “9” from STEP 3 in front of “99.27.2.idscheme.example.com.” and obtains “9.99.27.2.idscheme.example.com.”.

STEP 5:		The client sends the new FQDN form from STEP 4 to the application-specific ORS server.

STEP 6:		The application-specific ORS server returns NAPTR resource record 2 (see A.2) and the client extracts service field and RegExp.  The server returns NAPTR resource record 2 because the string, “9”, attached in STEP 4 to the new FQDN used in the query indicates that the Class is “9”, and thus the resolution for class A type identifier shall be performed. In this manner, resolution based on NAPTR can handle resolution based on the value of certain bit fields.

STEP 7:		The client check service (= “IRP+FFT”) field and applies the regular expression from STEP 6 to ID and converts each field to hexadecimal. The first bit field, “(.{4})” is “0HEX”. The second bit field, “(.{16})” is “0001HEX”. The third bit field “(.{4})” is “9HEX”. The fourth bit field, “(.{8})” is “01HEX”. The fifth field, “(.{96})”  is “000000000000000259FD720EHEX”.  Using these field values as expressed in hexadecimal notation after removing leading zeros, the client produces “259FD720E.1.9.1.0.code.example.com”.

Annex B

Configuration and operation of ID resolution servers

(This annex forms an integral part of this Recommendation | International Standard)

Figure B.1 shows an example configuration of ID resolution servers.

Figure B.1 – An example of delegation hierarchy of ID resolution servers

ID resolution servers are part of a global DNS infrastructure.  An organization that manages and allocates a specific identifier scheme should operate an ID resolution server.  There are many other organizations that manage and allocate some sub blocks of this specific identifier scheme and these organizations should also operate ID resolution servers.  The operation of ID resolution servers can be delegated to other organization by contract.

The intermediate ID resolution servers have a delegation pointer to another name server in NS records.  The ID resolution server at the end of the DNS tree has NAPTR resource records which contains actual URL of multimedia contents.

The hierarchical structure in Figure B.1 is a logical view and multiple ID resolution servers can be implemented in one physical server.

Annex C

Bibliography

(This annex does not form an integral part of this Recommendation | International Standard)

[b-ONS]		EPCglobal Object Name Service Version 1.0.1 (2008), Object Name Service (ONS)

[b-TagData] 	EPCglobal Tag Data Standard Version 1.5 (2010), EPC Tag Data Standard

__________________