| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, 24, and 25/16 |  |  |  | AVD-4083 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | Proposed requirements for Telepresence system |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution extracts some functional requirements of telepresence systems based on the discussion at the last Q5/16 meeting and proposes to use it as an initial document for F.TPS-Reqs.

# Introduction

At the last ITU-T Q5/16 meeting (Geneva, Mar, 2011), several issues were discussed, in particular defining the scope and requirements of telepresence systems. COM16-573  proposed initial requirements for telepresence systems.

# Discussion

At the March 2011 Q.5/16 meeting a new work item for F.TPS-Reqs “Definitions, requirements, and use cases for Telepresence Systems” was formally created. However there was no output baseline text for this work item. In order to progress the work, we produce a document skeleton based on former contributions and discussion. This is in order to facilitate further contribution from interested parties in a structured manner.

# Proposal

It is proposed to use this document as the initial base line text for F.TPS-Reqs.

Draft new Recommendation F.TPS-Reqs

Definitions, requirements, and use cases for Telepresence Systems

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

{Note: Contributions are solicited on the scope of the requirement document. The suggested headings below assume that this document will include definitions and the scope of telepresence systems, use cases and requirements of telepresence systems. Q5 may decide to limit the scope of this document to any of the above.}

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[H/F.TPS-Arch]	ITU-T Recommendation X.yyy (date), Telepresence System Architecture

[H.TPS-AV]	ITU-T Recommendation X.yyy (date), Audio/Video Parameters for Telepresence systems

[ITU-T X.yyy]	ITU-T Recommendation X.yyy (date), Title

# 3	Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

Telepresence - An interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size. [NOTE – From Living List for Q5/16 ]

{Note: Other definitions such as in AVD-4082 can be added here.}

## 3.1	Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

<abbr>	<definition>

<Include all abbreviations and acronyms used in this Recommendation>

# 5	Conventions

<Describe any particular notation, style, presentation, etc. used within the Recommendation, if any>

# 6   Scenarios and Use cases

{Note: This section may give some typical scenarios and use cases.}

# 7   Highlighted user experience requirements of telepresence systems

User01: The telepresence system should support life-size high-definition (includes 1080p, 1080i, etc.) imaging of video conference participants at the remote end.

Note: Life-size – the figure in the display is the same size as if the person is in the room (this is an advanced feature, not a basic feature). This can’t be a mandatory requirement.

User02: The telepresence system should provide natural eye contact experience.

User03: The telepresence system should support locating the speaker in a remote meeting room so that local participants can readily identify the position of different remote speakers. Audio/video synchronization (“lip-sync”) is an event. Maintain the order of the events (event is a combined audio/video event)

User04: The telepresence system shall enable synchronization of audio and video from the remote location. Synchronization includes “lip-sync” and the sequencing of multiple media streams.

User05: The telepresence system should provide easy one-touch control function thus enabling convenient centralized management and one-touch smart control capabilities.

# Functional requirements

## 8.1	Call control functions

Fun01: The telepresence system shall support either all or partial of the screens are enabled in a conference for multiple screens telepresence terminal subsystem when the number of received media stream is less than the number of screens.

Fun02: The telepresence system shall support description, identification and negotiation (alternately this could be announcement) of the seat information, the position information of displays and loudspeakers if a telepresence system with multiple screens participates in a meeting.

Note: Each terminal subsystem will provide position information to the MCU. The MCU received all such information from the terminal subsystems and mixes the audio. Then the MCU negotiates with each terminal subsystem to determine where to present each of the remote terminal information (audio and video).

## 8.2	Conference control functions

Fun03: The telepresence system shall enable switching the video from the whole of the remote meeting rooms to another meeting room, or from partial seats to another seat.

Note: Either a complete terminal system can be switched, or one of the terminals in a terminal subsystem may be switched.

Fun04: The telepresence system shall enable the conference chair person to select and view video from a given meeting room or from partial seats of a meeting room.

## Other functions

Fun05: The telepresence system shall enable audio mixing, and maintaining the spatial position of audio sources and recovery of spatial position of the audio sources.

Fun06: The telepresence system should support multiple video sources and multiple displays in each meeting room. The overlap of the scene from neighbouring cameras should be minimized. The output from the neighbouring cameras should provide a seamless view of the conference participants.

# Interoperation requirements

## 9.1   Interoperation with traditional videoconference system

Int01: The telepresence system should support that both traditional videoconference terminals and telepresence terminals can participate in a same conference.

Int02: The telepresence system should support that a traditional videoconference system can connect to the telepresence system in multiple cascaded conference mode.

## Interoperation between telepresence systems based on different open standards such as H.323 and SIP.

Int03: The telepresence system shall enable terminals based on different open standards such as H.323 and SIP to participate in the same conference.

# Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation)

<Body of annex A>

# Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation)

<Body of appendix I>

Bibliography

[b-ITU-T X.yyy]	ITU-T Recommendation X.yyy (date), Title

__________________