| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4085 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation, China Telecom |  |  |  |
| Title: | H.VSprot: Proposal of protocols for Instant image snapshot |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to add Signalling and Protocols materials for instant image snapshot to draft new Recommendation H.VSprot “Signalling protocols for visual surveillance”.

# Introduction

The draft new Recommendation H.VSprot focus on the detailed description of reference points, signalling and protocols of visual surveillance system based on the functional architecture defined in the Recommendation H.626 (i.e. H.VSarch).

By the end of the last ITU-T SG16 plenary meeting (Geneva, March 2011), most of the signaling and protocols for the basic service flows are completed. This contribution will continue to address on the Signalling and Protocols for the service of instant image snapshot of which the content is still blank in current draft document.

# Discussion

According to the Service Control Flow in the Recommendation H.VSarch, this contribution will give the corresponding Signalling and Protocols details (see Attachment).

# Conclusion

It is proposed to add Signalling and Protocols materials for instant image snapshot to clause 6.4 of H.VSprot.

Attachment
Proposed text

[Begin proposed text]

## 6.4	Instant image snapshot (IIS)

The Instant Image Snapshot (IIS) function is used to get instant image information based on video from the Surveilled Object.

The following procedural flow involves implementing the snapshot action in the PU which then uploads the snapshot image to the service platform from which the CU gets access to these images.

Figure 6-7: Procedural flows for Instant Image Snapshot

Figure 6-7 shows the high level procedural flows for IIS:

| STEP 1, 2, 3, 4 | CU sends a SIP MESSAGE to SCU to request Instant image snapshot, SCU transfers this message to requested PU. PU handles this request and returns a SIP 200 OK message to SCU. SCU adds an IIS (Instant image snapshot) record in service platform and returns a SIP 200 OK message to CU. |
| --- | --- |
| STEP 5,6: | The PU sends a SIP MESSAGE to the MDU to request for the image upload address. The MDU handles this request and returns a SIP 200 OK message with the upload address to the PU. |
| STEP 7,8: | The PU uploads the image file to the MSU through HTTP POST method with the URL of the upload address. And the MSU returns a HTTP 200 OK message. |
| STEP 9,10: | The MSU sends a SIP MESSAGE to notify the SCU for the completion of image uploading action. The SCU then returns a SIP 200 OK message to the MSU. |
| STEP 11,12: | The SCU sends a SIP MESSAGE to notify the CU for the completion of image uploading action. The CU then returns a SIP 200 OK message to the SCU. |
| STEP 13,14: | The CU uses HTTP GET method to request the MSU to download the snapshot image. The MSU handles this request and returns a HTTP 200 OK message with the image file. |

[End of proposed text]

__________________