| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4086 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Proposed texts for differentiating Telepresence services from videoconference services |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

In this contribution, we propose some texts for describing differences between telepresence and videoconferencing.

# Introduction

Telepresence is not newly arising terminology any more, but many people ask us what is the difference from videoconferencing; especially, HD-videoconference. Even though we have made a working definition on telepresence in the previous meeting, we also need more texts for explaining what telepresence is, what it can provide and what it is different from previous similar services.

Regarding this, we have made an agreement in the previous November meeting that the definition is needed for differentiating telepresence from videoconferencing. [T09-SG16-110314-TD-WP2-0453; Chapter 5.4.2]

In the last meeting, Q5 has agreed on a working definition of telepresence. We think the working definition is describing telepresence quite well. Anyway, we think that some additional texts for differentiating telepresence from videoconference will help people to get a better understanding on telepresence.

# Discussion

In the previous March meeting, Q5 has made a working definition as follows:

Telepresence - An interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size.

This clearly defines telepresence well, but we think some additional descriptions of differentiating from videoconferencing is helpful, because many people, non-experts and even  experts, still ask us “what telepresence is?” and “Is it HD videoconferencing? What are differences? What are technical advances?” So, we think that more detailed descriptions of comparison with videoconference are helpful on understandings. On the other hand, we also think that it is unnecessary to list up every difference into definition of telepresence, but it will be helpful on better understandings if we describe them elsewhere.

Hence, we have studied to address such differences by comparing the definitions of videoconference that is defined in F.703 and some related articles.

F.703 defines videoconference as follows:

Videoconference - An audiovisual conversational teleconference service providing bidirectional real-time transfer of voice and moving colour pictures between groups of users in two or more separate locations. The minimum requirement is that under normal conditions the picture information transmitted is sufficient for adequate representation of fluid movements of two or more persons in a typical meeting situation displayed in head and shoulders view. Although the moving picture information is the essential part of the service, other types of information, such as high resolution still pictures, text or data may also be exchanged.

Throughout the comparison of two definitions, we may deduce that videoconference provides real-time bidirectional audio-visual communication; whereas telepresence provides realism as if remote participants seem to be located in front of user.

When it comes to referring to “ITU-T Technology Watch Briefing Report Series, No.2” that is published at Dec. 2007 by ITU-T, it defines telepresence as a high-performance video conferencing. It describes the history of videoconference briefly and how videoconference has been developed. According to the report, videotelephony market has been developed in two different directions; low end and high end. The high-end videotelephony has been evolving into telepresence. This is reason why many people do not differentiate telepresence from high-end videotelephony clearly.

We summarized each characteristic as shown in following table. Although the definition of telepresence is not yet determined and the contents of table are premature, we can use it as a baseline of discussion.

|  | VC(Low end) | VC(High end) | Telepresence |
| --- | --- | --- | --- |
| Recommendations | H.320 (Narrow-band visual telephone systems and terminal equipment) H.321(Adaptation of H.320 visual telephone terminals to B-ISDN environments) H.322(Visual telephone systems and terminal equipment for local area networks which provide a guaranteed quality of service) H.324(Terminal for low bit-rate multimedia communication) | H.310(Broadband Audiovisual Communication systems and architecture) | *1)New One? |
| Codec | narrow-band voice codec low resolution | narrow/wide-band codec MP@ML(720x480, 720x576) | narrow/wide-band codec spatial CD quality audio codec FHD resolution |
| Control Data Collaboration | T.120 | T.120 | Gaze control Camera Control Remote prese |
| Display | small size 1 display | medium size 1 display | Life size multiple display Multiple HD camera |
| Service | Low-bandwidth low-quality low fps |  | Eye Contact File sharing Presentation |
| Usage | Low quality visual communication | Visual communication Video Telephony | Remote Surgery/Medication, Education, Remote Collaboration, Remote presentation/interviewing |
| Environment | Not an issue | Not an issue | Unified Interior, Studio, light, wall colour, position of table, invisible camera |

*1) As for my understanding, Q5 does not yet decide whether extends existing recommendations or create new one. In my opinion, we need to create new one for telepresence.

As shown above table, ITU-T H.320, 321, 322,324 describe narrow-band videoconference and H.310 describes wide-band videoconference. According to Table 3/H.320 “Audiovisual capabilities of H.310 terminals”, it requires H.262 MP@ML(Main Profile at Main Level) as a mandatory capability. In case of MP@ML medium quality services, transfer rate is 6.144Mbits/s and 9.216mbits/s in high quality services. In case of MP@ML, video resolutions are 720x480 with 30 frames or 720x576 with 25 frames. This level of resolution provides quite good quality for medium-size display, but it is not enough to provide immersive experience to user. In that sense, telepresence should support more resolution than MP@HL(1920x1080 with 30 frames or 1280x720 with 60 frames). Of course, H320 support much lower resolution and higher resolution such as HP@HL as optional recommendations also. Besides of minimum required video resolution, telepresence differs from videoconference on the point using multiple HD displays and HD cameras.

In case of audio aspect, H310 requires ISO/IEC 11172-3 MPEG1 Audio Layer 2 Decoder and G.711 as a mandatory audio codec; it also supports another narrow/wide band voice codec as optional. Since telepresence uses multiple display for a conference session, it assign audio/video stream of specific remote user to specific display and speaker. This requires more advanced spatial audio techniques.

# Conclusion/Proposal

We have concluded that below three factors are main differences between telepresence and videoconferences.

- Video Aspect: Better video quality with higher resolutions and multiple video I/O devices

- Audio Aspect: Support of spatial audio

- Realism Experience Aspect: Involvements of emotional techniques such as eye line tracking, gaze control and environmental harmonization such as illumination, wall colour and light.

- Service Aspect: Support of additional features such as remote presentations, file sharing and online document editing. These functions may be optional.

- Usage Aspect: Telepresence is very good solution for Remote Surgery/Medication, Education, Remote Collaboration

In this contribution, we propose to include attached texts into appropriate section of F.TPS.Reqs.

Attachment
Proposed text

[Begin proposed text]

X.x Differences between videoconferencing and telepresence

Telepresence looks like high-quality videoconferencing, but it is composed of more functional and advanced emotional technologies such as gaze control, eye-line detection, and so on.

Videoconference, which is defined in F.703, provides real-time bidirectional transfer of voice and moving colour picture communications. The main point of difference between videoconference and telepresence is feeling of immersive and realism. Videoconference only satisfies that user can see remote user’s movement or face whereas telepresence aims at strong sense of realism and presence. Telepresence provides not only high-density video and CD quality spatial audio but also emotional realism functions such as eye contact, eye line, camera control, gaze control and life-size video as if remote participants seem to be located in front of user. It also supports multiple displays, microphones and speakers whereas videoconference is usually composed of one audio-visual device. In addition, in order to achieve realistic experiences on remote user’s presence, telepresence provides cooperation functions such as remote presentation and file transfer as well as high-quality audio-visual multimedia stream.

[End of proposed text]

_____________________