| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4096 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 12/16 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Waseda University |  |  |  |
| Title: | AMS information flow - Application handover case |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

This contribution updates the information flow for the Application handover case with consideration of “pre-invoke” procedure and use of Interface C for transferring a large volume of Application data between local Applications. It also raises some issues to be yet studied.

# Introduction

For the regular communication case, the AMS information flow is currently understood as in Annex A to this contribution. The latest discussion on the AMS information flow at the AMS e-meeting on 31 May 2011 agreed on the concepts of “pre-invoke” for reserving distributed Applications for a multimedia communication session and “cancel pre-invoke” for releasing the reserved Applications when they become unnecessary[1].

The discussion on Application handover at the AMS e-meeting on 31 May 2011 also agreed on the use of Interface C for transfer of a large volume of date between local Applications.

This contribution reflects these agreements to the AMS information flow for the Application handover case originally raised in C.543[2] in March 2011 and updated subsequently in AMS-0029[3] and AMS-0031[4]. It also indicates items to be studied in blue characters.

# Application handover scenario

Based on the regular communication in Annex A, the following case is considered here:

Assemblage A  Assemblage B

Initial communication:	Application 3 - Application 3

is handed over to:		Application 4 - Application 3

During the regular communication as shown in Annex A is ongoing, Application 3 (video) of Assemblage A is handed over to Application 4 (video). Application 3 of Assemblage B continues video communication even if the corresponding Application of Assemblage A is switched to Application 4. The information flow for Application handover parallels with that of the regular communication in Annex A as much as possible.

# Information flow

Information flow for the Application handover is illustrated in Figure 1.

Figure 1 – Information flow for AMS application handover

Each information flow is numbered as #1nn and detailed in the following:

#100	Application registration – In advance of the handover, Application 4 of Assemblage A (denoted "4/A" here) registers to the Container.

#101	Notification of Application availability – Application 4 notifies Container of its availability.

#102	Presentation of Application 4 – Based on the registration and status information of Application 4, Container provides the human user with additional information on available Applications and their characteristics.

#103	Handover request – Human user of Assemblage A requests an Application handover from 3/A to 4/A. Here the human user requests the Application handover. It is noted that “There might be instances where the user's Container is provisioned to automatically perform a handover (e.g switching from mobile voice to Telepresence when coming into contact with the user's Telepresence system).”[5]

#104	Pre-invocation of Application 4 – Based on the human user’s request, Container A reserves Application 4 with an indication of handover from Application 3.

#105	Application 4/A returns acknowledgment to Container A.

#106	Handover notification – Container A notifies Application 3 of handover to Application 4.

#107	Handover negotiation (local) – Application 3 negotiates with Application 4 about the Application handover possibility. The signalling for this purpose is carried via Container A.

#108	Handover negotiation (with the remote end) – Application 3/A negotiates with Application 3/B about the Application handover to Application 4/A at the local end and gets agreement from Application 3/B.

#109	Handover initiation – Application 3/A initiates the Application handover to Application 4/A. This signalling includes information on the existing communication instance between Applications 3/A and 3/B. The signalling for this purpose is carried via Container.

#110	Transfer of Application data from Application 3/A to 4/A if necessary. This transfer is carried out through Interface C[1]. It is assumed that there is no admission control or bandwidth management entity inside the Assemblage and the media flow is directly established toward the destination Application.

#111	Capability negotiation  – Application 4/A negotiates its “lower level” detailed capabilities (type of codecs, picture resolution, sound bandwidth, etc) with Application 3/B and decides operational parameters.

#112	Request of network resource for media and control flows – Application 4/A requests network resources necessary for the media and control flows to the network (Service Node) via Container A and establishes relevant channels. SN conveys this information to Container B for user friendly alerting (see #114 below).

#113	Media and control flows – They are carried directly between local Application 4/A and remote Application 3/B through the channels established by the #112 process.

#114	Alerting the remote user – Once the media flow for handed over Application has become ready, the remote end user is alerted. The notes in #14 in Annex A are also applicable here.

#115	Response – The human user at the remote end responds to the alerting. Then the video is connected between Applications 4/A and 3/B.

#116	Release of original Application 3/A – Container releases Application 3/A at some point of time. Exact timing is yet to be studied.

#117	Release of network resource – Application 3/A requests the network of releasing network resource established for its media and control flows. SN conveys this information to Container B.

#118	Closing the media and control channels – Based on the signalling in #117, all the channels relevant to Application 3/A are closed.

# Items to be studied

The items needing further study are listed here.

1)	Timing of application data transfer. Figure 1 assumes that the application data transfer takes place after the initiation of handover from 3/A to 4/A and before the capability negotiation between Applications 4/A and 3/B. Is this the right timing?

2)	This contribution assumes that there is no entity to manage the network resource inside the Assemblage or home network. Hence the media is sent directly to the destination. Is this a right assumption in the AMS architecture? In the case of media transfer between local and remote Applications, we assume that the request for network resource is involved. The Application sends a request to the Service Node with indication of a desired amount of bandwidth and a desired level of QoS. This is similar to the GK routed model of H.323.

3)	Design of user friendly alerting like H.460.11 that establishes media before alerting.

4)	Timing of releasing existing session, Application 3/A – 3/B in the current example, before or after Application handover,.

# Conclusion

This contribution has provided information flows for the Application handover in AMS. Though there remain some points for further study, it is hoped that the materials assist the common understanding of the AMS architecture and the work of designing necessary protocols over various interfaces.

# References

[1]	Q12/16 Rapporteur: 2011-05-31 Meeting Minutes, http://wftp3.itu.int/av-arch/avc-site/2009-2012/AMS_emeetings/Minutes/

[2]	Waseda University: Information flow of AMS, Study Group 16 Contribution 543, March 2011

[3]	Waseda University: An update of the AMS information flow, AMS-0029, 20 April 2011, http://wftp3.itu.int/av-arch/avc-site/2009-2012/AMS_emeetings/

[4]	Waseda University: Invocation of Applications, AMS-0031, 1 June 2011, http://wftp3.itu.int/av-arch/avc-site/2009-2012/AMS_emeetings/

[5]	Q12/16 Rapporteur: Minutes of the electronic meeting on 2 November 2010,  TD 486/WP2, §5.3, March 2011

__________________

Annex A

Information flow for regular communication

The “regular communication” is meant to be basic communication between two Assemblages. Its information flow is exemplified in Figure A.1 where three Applications (1: audio, 2: whiteboard and 3: video) are involved and Assemblage A initiates the multimedia communication. Relevant entities of human user, local Application, local Container (A), Network represented by Service Node (SN), remote Container (B) and remote Application as well as information exchanged among them are indicated in Figure A.1 together with the relevant interface.

Figure A.1– Information flow for AMS regular multimedia communication

Each information flow is numbered as #nn and detailed in the following:

#01	Application registration – Each Application registers to Container in advance of a multimedia communication session. Application description is of “higher level” such as Application type. It is assumed that the Container is always beside the human user. Container may reside in a handheld device or an Application device.

#02	Container registration – Container registers to Service Node for the use of network resource. Though Figure A.1 indicates SN (Service Node) as a single device, SN may also be a federation of multiple distributed devices.

#03	Notification of Application availability – Each Application notifies Container of its availability. In case it is involved in a certain communication, it can not be used for any other communication.

#04	Presentation of Applications – Based on the registration and status information of each Application, Container provides the human user with information on available Applications and their characteristics. It may be an Application by Application indication or pre-defined Application set (e.g. videoconferencing) indication.

#05	Communication request – The human user requests a multimedia session of his/her choice to the Container.

#06	The calling Container A pre-invokes Applications, which the human user has designated, at both local and remote Assemblages. The remote Container B extends this pre-invoke message from the local Container A to remote Applications. Some Container to Container messages like this signalling may go through SN to allow policy control as in the H.323 GK routed model. This aspect is yet to be studied.

#07	Each Application returns an ACK or a NACK to the pre-invoke message. The remote Container B collects the responses from remote Applications and conveys them to the local Container A.

#08	The local Container A decides a set of Applications to be invoked for the communication session.

#09	The Container A invokes that set of Applications at both local and remote Assemblages. The remote Assemblage B extends this invocation message to the remote Applications.

#10	If some of the pre-invoked Applications become unnecessary due to unavailability of matching Applications, the local Container A sends “cancel pre-invoke” to those Applications for releasing reserved resources.

#11	Capability negotiation – Each Application negotiates its “lower level” detailed capabilities (type of codecs, picture resolution, sound bandwidth, etc) with the remote peer Application and decides operational parameters. It is noted that Container basically relays messages between local and remote Applications.

#12	Request of network resource for media and control flows – Each Application requests network resources necessary for the media and control flows to the network (Service Node) via Container A and establishes relevant channels. SN conveys this information to Container B for user friendly alerting (see #14 below). In case NAT/firewall traversal is involved, more signalling information exchange would be required among AMS entities. This aspect is yet to be studied.

#13	Media and control flows – They are carried directly between local and remote Applications through the channels established by the #12 process.

#14	Alerting the remote user – Once all the media flows for the multimedia communication have become ready, the remote end user is alerted[A.1]. This could be by the use of timer after the network resource signalling in #11, or by an explicit signalling from remote end Applications to Container B as a result of successful media flow establishment in #12. This aspect is yet to be studied. It is also noted that for some Applications like “flashing lamp” to know that another Application is in an alerting state we may need “subscribe/notify” functionality, but this aspect is yet to be studied.

#15	Response – The human user at the remote end responds to the alerting.

#16	Multimedia communication – The human users communicate each other via the network by using a desired combination of multiple media.

References

[A.1]	Recommendation ITU-T H.460.11 (03/2004) “Delayed call establishment within H.323 systems”