| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4097 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 12/16 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Waseda University |  |  |  |
| Title: | Possible content for draft Recommendation H.325 |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

This contribution tries to indicate some materials for possible inclusion in the draft H.325 by reviewing the discussion results to the contributions submitted by our organization. It also adds some notes to indicate the areas needing more study.

# 1	Introduction

The current draft H.325 is contained in [AMS-0004] that was last updated on 2 December 2009. There have been some discussions on the system architecture and protocols since then. We should include some points of common understanding in draft H.325 to progress the work.

# 2	Discussion

This contribution provides some content that may be added in the new draft H.325 in form of annotating on AMS-0004 as attached. Comments for discussion are indicated in blue characters and with { }. Obvious corrections are indicated with revision marks.

The following items are for discussion:

1)	AMS design philosophy in “1 Scope”

2)	Replacement of “Figure 1 AMS entities and interfaces”

3)	Addition of sub-clauses for “System architecture”, “Information flow” and “System profiles” in “6 System description”

4)	Addition of a sub-clause “8.7 Application control and indication”

5)	Addition of a clause “9 Media procedures”

# 3	Conclusion

Other contributors are encouraged to do similar reviews to improve the draft H.325 and identify points needing active study.

# References

[AMS-0004]	Editor: Updated draft of new H.AMS “AMS: Advanced multimedia systems for next generation and other packet-switched networks”, 2 December 2009

[AMS-0016]	Waseda University: Use of distributed input/output devices, 16 March 2010

[AVD-3995b]	Waseda University: Proposal of additional Interface E for AMS, November 2010

[AVD-4096]	Waseda University: AMS information flow - Application handover case -, July 2011

[TD 301/WP2]	Q12/16 Rapporteur: Agenda and documentation for Q12/16, §1.4 Issues under Discussion, March 2011

| Electronic Meetings of Questions 12/16 |  | AMS-0004 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | STUDY GROUP 16 |
|  |  | Original: English |
| Question(s): | 12 | Electronic, 2 December 2009 |
| RAPPORTEUR MEETING DOCUMENT |  |  |
| Source: | Editor |  |
| Title: | Updated draft of new H.AMS “AMS: Advanced multimedia systems for next generation and other packet-switched networks” |  |
| Purpose: | Information |  |

Summary

The text in this document is a starting point for H.AMS and is revised according to the outcomes of the previous meeting and resubmitted to each meeting.

CONTENTS

Page

1	Introduction	1

2	Discussion	1

3	Conclusion	1

References	2

1	Scope	5

2	Normative references	5

3	Definitions	6

4	Symbols and abbreviations	8

5	Conventions	8

6	System description	8

7	System components	9

7.1	Container	9

7.2	Applications	9

7.3	Service Node	9

7.4	Application Servers	9

8	Signalling procedures	9

8.1	Container registration	9

8.2	Application registration	9

8.3	Session establishment	9

8.4	Application invocation	9

8.5	Application handover	9

8.6	Session termination	9

8.7	Application control and indication	9

9	Media procedures	9

10	Interoperation with non-AMS devices	9

11	Optional enhancements	9

12	Maintenance	10

ITU-T Recommendation H.AMS

AMS: Advanced multimedia systems for next generation and other packet-switched networks

AAP Summary

[TO BE PROVIDED BEFORE CONSENT]

Summary

This Recommendation describes …

# Scope

This Recommendation covers the technical requirements for an Advanced Multimedia System for NGN and other packet-switched networks.

H.AMS is designed to be an open application platform, specifying the underlying communications methods and protocols, but leaving the exact implementation of the services to the end-equipment manufacturer.

{The AMS design philosophy can be added by stating something like [AMS-0016]:

The Advanced Multimedia System (AMS) is characterized by the following features among others:

- Flexible combination of various media for a multimedia communication session

- Decomposed architecture, namely separation of control and media processing (or separation of "Container" and "Applications")

- Flexible use of distributed input/output devices (through "Applications handover")}

In this way, H.AMS intends to enable rich-media communication between a vast array of devices, allowing users to take advantage of enabled end-points on an ad-hoc basis.  Additionally, through interworking functions, it is intended that access to certain functions of the H.AMS infrastructure can be enabled from non-H.AMS devices and end-points.

H.AMS represents the user to the network as a logical entity where several facets of the communication could be split over a number of physical devices; for example, in a full audio and video conference, the video portion could be delivered on a conventional television set, whilst the audio is delivered through a mobile cellular telephone.

This Recommendation describes the components of an H.AMS system. This includes the AMS Assemblage, Devices, Application Servers, Service Nodes and the Container comprising three elements; the Application Registry, the Orchestration Manager and Transport Agent. Control messages and procedures within this Recommendation define how these components communicate. Detailed descriptions of these components are contained in clause 6.

# Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[1]	TBD

# Definitions

Editor’s Note: The relationship of the service node, network services and facilities nodes and assemblages were agreed to be updated in H.AMS and ideally the relationship be captured in a figure, in that document. Further contributions are solicited in this area.

TSB Note: split definitions between “defined elsewhere” (§3.1) and “defined in this Recommendation” (§3.2).

The following definitions shall apply.

3.1	Application Servers: Application servers are elements on the network that provide common services, for example IPTV, interactive gaming, multipoint conferencing.

3.2	Application: The Application is the element in the AMS environment that represents one aspect of communication, e.g. Voice, Video.

3.3	Application Set: An Application Set is a group of Applications required to create a complete communication experience.  For example, in a user interaction requiring voice, video and text, the three discrete applications would comprise a single Application Set.

3.4	Assemblage: The AMS Assemblage is the set of AMS elements that represent the logical association between the elements required for the user interaction.  For example, a videoconferencing terminal consisting of distributed voice and video elements would be referred to as an AMS Assemblage.

3.5	Container: The container is the entity that represents the user to the network.  It comprises three elements:

- Application Registry

The Application Registry is the entity where individual Applications register to declare their availability to a particular container.

- Orchestration Manager

The Orchestration Manager coordinates events between the Applications in order to create an AMS session.

- Transport Agent

The Transport Agent manages the signaling between AMS Assemblages

{Changes to the rest of §3 come mostly from [AVD-3995b].}

3.6	Interface “A”: This interface refers to the communication control between two AMS Assemblages.

3.7	Interface “B”: This interface refers to the communication control between the Applications within the AMS Assemblage and the Container function.

3.8	Interface “C”: This interface refers to the media stream between two AMS Assemblages.

3.9	Interface “E”: This interface refers to the network access signalling between the Assemblage and Service Node for use of the network resources.

3.10	Interface “K”: This interface refers to the Application control between two Applications.

3.11	Session: A session refers to communication between two AMS Assemblages, which may involve one or more communicating applications.

3.12	Service Node: The Service Node is a network entity that enables the container to establish communication with a remote entity, facilitate NAT/FW traversal, and provide other network-based services.

Figure 1/AMS: Definitions and Interfaces

Editor’s note: Figure 1 should be further elaborated indicating what has been agreed.

{Figure 1/AMS is to be replaced by the following [AVD-3995b]:

Figure 1/AMS: AMS Entities and Interfaces

Contributor’s note: Home network and NAT/FW traversal aspects are yet to be studied.

}

# Symbols and abbreviations

This Recommendation uses the following abbreviations:

| AMS | Advanced Multimedia System |
| --- | --- |

# Conventions

In this Recommendation, the following conventions are used:

"Shall" indicates a mandatory requirement.

"Should" indicates a suggested but optional course of action.

"May" indicates an optional course of action rather than a recommendation that something take place.

References to clauses, sub-clauses, annexes and appendices refer to those items within this Recommendation unless another text is explicitly listed.

# System description

Editor’s note: Contributions are solicited in this area.

{

6.1	System architecture

AMS system entities and interfaces are shown in Figure 1. (Contributor’s note: Figure 1 is more appropriate here than in §3)

6.x	Information flow

The figures and descriptions in [AVD-4096] for a regular communication case and an Application handover case can be here if so agreed.

6.y	System profiles

A set of  system profiles may be defined according to the system capabilities; assemblage functionalities and/or available network resources.

}

# System components

Editor’s note: Contributions are solicited in this area.

## 7.1	Container

## 7.2	Applications

## 7.3	Service Node

## 7.4	Application Servers

# Signalling procedures

Editor’s note: Contributions are solicited in this area.

## 8.1	Container registration

## 8.2	Application registration

## 8.3	Session establishment

## 8.4	Application invocation

## 8.5	Application handover

## 8.6	Session termination

{

## 8.7	Application control and indication

Descriptions of various type of C&I signals and their transport methods. (Contributor’s note: the transport can be generically through Interface K (e.g. TCP), through RTCP, … as summarized in [§1.4 of TD 301/WP2])

# Media procedures

Method(s) of media transfer through Interface C and other media handling.

}

# Interoperation with non-AMS devices

Editor’s note: Contributions are solicited in this area.

# Optional enhancements

Editor’s note: Contributions are solicited in this area.

# Maintenance

Editor’s note: Contributions are solicited in this area.

__________________