| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4106 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q5 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Polycom |  |  |  |
| Title: | Standard Colour Gamut for Telepresence Systems |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

It is noted that there is no standardized colour gamut specified for videoconferencing systems.  Since it is impossible to accurately render the source picture correctly without knowing this information, it is proposed that appropriate recommendations be developed and incorporated into H.TPS-AV.

# Introduction

In order to provide a “being there” telepresence experience, it is important to render the received images so that they match the colour and contrast of the original source as closely as possible.

However, there is no standardized colour gamut for videoconferencing systems, and no method for negotiation of colour gamut between unlike systems.  Polycom proposes that recommendations on this topic be developed and incorporated into H.TPS-AV.

# Discussion

ITU-T Rec. H.264 Annex E provides a facility for signaling “Video Usability Information”  in an H.264 bitstream through a VUI message.  There are ten standard choices for various opto-electonic transfer characteristics of the source picture.  In addition, there are five choices of colour primaries, and six sets of matrix coefficients that can be used in deriving luma and chroma signals from those primaries.  There is also the ability to signal the use of a “wide gamut” picture (through the video_full_range_flag parameter) – 600 distinct combinations in all (not counting duplicates and “unspecified” choices).

The VUI message is only an optional component of the H.264 bitstream, and even if it is present the transfer characteristics, the colour primaries, and the matrix coefficients can each be signaled as “unspecified”.  However, without knowledge of these parameters the video stream cannot be rendered correctly.

ITU-R Rec. BT.709-5 specifies how video luminance and colour information is digitized for broadcast HDTV.  Polycom proposes that BT.709 also be used in High Definition telepresence systems.

Polycom also notes that the conventional colour gamut used in HDTV broadcasting does not permit the rendering of all colours that can be seen by the human eye.  Being able to render more colours might be valuable in a high-quality video application like Telepresence.  Although ITU-R Rec. BT.1361 standardizes a wide-gamut mode, it does not appear to be widely used.

# Conclusion

We propose that in the absence of VUI signaling (or if the VUI signaling indicates “unspecified”), that receivers shall process all received High Definition video signals per BT.709.

We also propose that senders should always signal the black level and range of the luma and chroma signals, transfer characteristics, the colour primaries, and the matrix coefficients in their video bitstreams.  Receivers may use this information to optimally render the received pictures.

We also suggest that it might be useful to seek guidance from SG 12 and/or ITU-R on the value of “wide gamut” picture formats in telepresence applications, and to enquire about any ongoing work on such formats.

__________________