| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4124 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.82: Definitions |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

The current draft of H.248.82 introduces as new concepts the actuating element and the sensor element in an ECN loop. However the behaviour of these elements correspond to entities defined in [IETF ECN].

# Discussion

The following definitions have been included in the H.248.82 draft:

- the actuating elements in case of indicated congestion (typically located in the source host);

- the sensor elements, responsible for indicating congestion (at a network route, in an IP hop) by ECN marking in the IP path

The following definitions are available in [IETF ECN]:

- Sender:  A sender of RTP packets carrying an encoded media stream. The sender has the possibility to effect how this transmission is performed.  It is one end-point of the ECN control loop.

- Receiver:  A receiver of RTP packets with the intention to consume the media stream.  It sends RTCP feedback on the received stream. It is the other end-point of the ECN control loop.

The behaviour of the actuating element matches the behaviour of the sender in the ECN control loop. The behaviour of the sensor element matches the behaviour of the receiver in the ECN control loop.

It is preferential to describe the ECN concepts in the terms already defined by IETF rather than introducing new terms, therefore it is proposed to replace the actuating and sensor elements concepts by the sender and receiver concepts in the sense of [IETF ECN]

[Begin Correction]

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Sender [IETF ECN]: A sender of RTP packets carrying an encoded media stream. The sender has the possibility to effect how this transmission is performed.  It is one end-point of the ECN control loop.

NOTE: the sender in this sense may or may not be the same as the RTP endsystem.

3.1.2	Receiver [IETF EDN]: A receiver of RTP packets with the intention to consume the media stream.  It sends RTCP feedback on the received stream. It is the other end-point of the ECN control loop.

NOTE: the receiver in this sense may or may not be the same as the RTP endsystem.

[End Correction]

[Begin Correction]

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	AuditValue.req: A H.248.1 AuditValue command request.

[End Correction]

[Begin Correction]

## 6.1	Basic ECN control loop

[IETF RFC 3168] defines an  addition to IP for ECN support. This support relates to a unidirectional connection model, i.e., IP traffic in IP source host to IP destination host direction. Figure 1 illustrates the basic ECN control principle. The control loop may be abstracted by four components in the user plane:

- the congestion notification information flow in source to destination host direction;

- the feedback information flow in reverse direction, back to the traffic source;

- the sender, capable of adapting the transmission bit rate in case of indicated congestion (typically located in the source host); and

- the receiver, responsible for indicating congestion (at a network route, in an IP hop) by ECN marking in the IP path (see (1)). The receiver may also be capable of performing actions to adapt to the detected congestion.

There might be a further component in the control plane:

- for the indication and negotiation of a congestion control.

[End Correction]

Figures 3, 4, 5 and 6 shown the ECN control loop between two ECN-capable hosts. These hosts are also shown as being the RTP end systems. However the ECN end points do not need to be the same as the RTP end points. The following change is proposed.

[Begin Correction]

It is proposed to change “RTP endsystems” by “ECN end point” (or alternatively by sender and receiver in the sense of [IETF ECN]) in figures 3, 4, 5 and 6.

[End Correction]

# Conclusion

It is proposed to accept above changes in the H.248.82 draft.

__________________