| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4125 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.82: Applicability of descriptors to H.248 properties |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

Most of the properties introduced in this Recommendation are defined for the local and remote descriptors. The motivation for that is to allow for asymmetric usage of ECN, i.e. different usage in different directions.

However, some of the properties by nature determine the behaviour of the ECN end-point for both directions at the same time, they characterize the complete bi-directional stream. Thus the definition of those properties in the local and remote descriptors seems confusing.

Furthermore, the description of the ECN end-point behaviour as determined by other properties defined in local and remote descriptors, would benefit from further clarification.

# Discussion

2.1	Congestion Response Method

The value “Hybrid” defined for this property implies, according to [IETF ECN], that “the congestion control algorithm needs to use the signaling to indicate which features of ECN for RTP are required”. Thus the use of this value must be accompanied by further information which is currently outside this Recommendation.

In addition, since the ECN feedback is sent in opposite direction to the received media, there is room for misinterpretation of the implied behaviour of each values. Some clarification is proposed.

[Begin Correction]

### 7.1.2	Congestion Response Method

|  | Property Name: | Congestion response method |
| --- | --- | --- |
|  | Property ID: | crm (0x0002) |
|  | Description: | When RTP packets are received with ECN-CE marks from the remote side or local congestion is experienced, the sender and/or receivers MUST react with congestion control as-if those packets had been lost. However there are different control methods which a MG could use. This property allows the MGC to request different types of control. These types are defined in clause 7.3.3 / [IETF ECN]. |
|  | Type: | Enumeration |
|  | Possible values: | "SDCC" (0x0001) Sender Driven Congestion Control "RDCC" (0x0002) Receiver Driven Congestion Control "Hybrid" (0x0003) A hybrid mechanism The value “Hybrid” requires additional MG behaviour description which is outside the scope of this Recommendation. This could be specified within an application profile. |
|  | Default: | Provisioned. |
|  | Defined in: | Local/Remote "SDCC" in Local Descriptor implies the MG, when receiving ECN-CE marked media, generates ECN feedback towards the sender of the media. "RDCC" in Local Descriptor implies the MG reacts to the received ECN-CE marks without contacting the sender. NOTE: as a result of the action taken by the MG, the sender is eventually notified, for example of a change of subscription to a less bandwidth consuming layer or through bit adaptation requests. "SDCC" in Remote Descriptor implies ECN feedback may be received from the remote receiver and the MG shall react to this feedback for example with bit adaptation requests. "RDCC" in Remote Descriptor requires no action from the MG. |
|  | Characteristics: | Read/Write |

[End Correction]

2.1.2	Congestion Response Method

This property is defined in local and remote descriptors while it determines the ECN behaviour of the MG in relation to both directions of the media at the same time. Thus some values are meaningless depending on in which descriptor they are present, for example, "setonly" when defined in a Local Descriptor. The nature of the property is that of a typical Local Control property as it describes the complete bi-directional stream behaviour.

[Begin Correction]

### 7.1.4	ECN Mode

|  | Property Name: | ECN Mode |
| --- | --- | --- |
|  | Property ID: | mode (0x0004) |
|  | Description: | This property indicates whether the MG should set and/or read ECN marks. This does not imply that the MG must "set" or mark any particular ECN code point it refers to the mode that the MG shall operate in order to determine the correct ECN procedures to apply. This property is equivalent to "mode" defined by the ABNF grammar defined by clause 6.1 [IETF ECN]. The MG specific semantic is detailed below. |
|  | Type: | Enumeration |
|  | Possible values: | As per "mode" clause 6.1/[IETF ECN]. "setonly" (0x0001); There is agreement to use ECN at the IP connection in the LS(A)-to-RD(A) direction (see naming convention in clause 5.2.3.1 [ITU-T H.284.1]).. "setread" (0x0002); There is agreement to use ECN at the IP connection in both directions, i.e. the LS(A)-to-RD(A) and RS(A)-to-LD(A) direction (see naming convention in clause 5.2.3.1 [ITU-T H.284.1]).. "readonly" (0x0003); There is agreement to use ECN at the IP connection in RS(A)-to-LD(A) direction (see naming convention in clause 5.2.3.1 [ITU-T H.284.1]).. |
|  | Default: | "setread" |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

[End Correction]

2.1.3	ECT Marking

The statement included in the definition of this property “indicates whether the MG should set or read ECN marks” is inaccurate, as it determines not whether, but how it should set, and what is to be expected from the peer. Some modifications are proposed.

[Begin Correction]

### 7.1.5	ECT Marking

|  | Property Name: | ECT Marking |
| --- | --- | --- |
|  | Property ID: | ectmark (0x0005) |
|  | Description: | When set in the Remote Descriptor this property indicates how the MG should set ECN marks. When set in the local descriptor this property indicates which ECN mark is expected from the remote sender (NOTE1). The expected value however does not usually influence the MG behaviour, even in the case that it doesn’t correspond to the value actually received. This property is equivalent to the "ect" element defined by the ABNF grammar defined by clause 6.1 [IETF ECN]. NOTE 1 – There is also an ECN-CE marking function (besides ECT marking), which may be supported by H.248 MGs and interim IP devices. Such marking due to experienced local congestion is out of scope of this property. |
|  | Type: | Enumeration |
|  | Possible values: | As per "ect" clause 6.1/[IETF ECN]: "1" (0x0001) "0" (0x0002) "Random" (0x0003) |
|  | Default: | "0". |
|  | Defined in: | Local / Remote |
|  | Characteristics: | Read/Write |

[End Correction]

### ECN Congestion Marking

This property determines whether the MG may mark outgoing datagrams with an ECN-CE mark when it experiences itself congestion. Thus the value is only relevant for the Remote Descriptor.

[Begin Correction]

### 7.1.6	ECN Congestion Marking

|  | Property Name: | ECN Congestion Marking |
| --- | --- | --- |
|  | Property ID: | congestmark (0x0006) |
|  | Description: | This property allows the MGC to indicate whether or not RTP packets may be ECN-CE marked by the MG (or not) if it the MG itself experiences congestion. |
|  | Type: | Enumeration |
|  | Possible values: | "mgdet" (0x0001): The MG determines if ECN Congestion Marking is enabled. "cemark" (0x0002): ECN Congestion marking is enabled. "nomark" (0x0003): ECN Congestion marking is not enabled. |
|  | Default: | Provisioned. |
|  | Defined in: | Local / Remote NOTE: the property is irrelevant when included in the Local Descriptor. |
|  | Characteristics: | Read Only |

[End Correction]

# Conclusion

It is proposed to accept above changes in the H.248.82 draft.

__________________