| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4127 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.82: Updates |  |  |  |
| Purpose: | Proposal |  |  |  |

# Introduction

This document contain a number of update proposals on the H.248.82 draft from Ericsson.

# Discussion

2.1	Redundant figure

The value of Figure 5 is unclear. It can be entirely derived from Figure 4. It is proposed to remove this figure.

[Begin Correction]

## 6.4	Capability declaration and negotiation of ECN in the control plane

There are multiple options of control plane protocols (e.g., H.323, SIP, SIP-I) for the establishment and release of RTP sessions. Figures 3 and 4 indicate the example of SIP with its embedded SDP Offer/Answer protocol, used for end-to-end capability declaration and negotiation of ECN usage.

[IETF ECN] assumes the use of a SIP based control plane with additional SDP information elements.

Figure 5 outlines an example scenario in scope of this Recommendation, e.g., SIP as session control signalling protocol at MGC level, and the mapping of SIP-level SDP information to H.248 control elements, and an H.248 MG, which may provide various ECN behaviour (as outlined in clauses 6.1 and 6.5).

Figure 5 – Control plane example with H.248 network elements at the edge of an ECN domain

[End Correction]

## 2.2 ECN behaviour vs. RTP topology

## Rewording of the text is proposed. Rather than defining the relationship between ECN and RTP topologies, the referred text in [IETF ECN] describes ECN behaviour of nodes that adopt a certain RTP topology.

[Begin Correction]

### 7.6.1	Topology

## ECN functionality may be used with different RTP session topologies. The ECN behaviour in conjunction with certain RTP topologies is described in clause 3.2 / [IETF ECN].  Through the MGC setting the combination of Terminations/Streams in a Context, the properties in the "ECN for RTP-over-UDP Support" and other elements in the Local and Remote Descriptors the MG is able to determine the correct ECN behaviour.

[End Correction]

## 2.3 RTCP control information in H.248

## Support of RTP/AVPF is highly recommended in ECN scenarios. However it is not mandatory. In addition, MG may support RTCP XR without specific information from MGC. Thus the following change is proposed.

[Begin Correction]

### 7.6.2	Negotiation of the capability to use ECN with RTP/UDP/IP

As per [IETF ECN] a MG supporting ECN with RTP/UDP/IP requires different RTCP extensions:

- RTP/AVPF [RFC4585] transport layer feedback format for urgent ECN information,

- RTCP XR [RFC3611] ECN summary report block type for regular reporting of the ECN marking information.

Thus in addition to the methods for indicating/negotiating ECN described below the MGC may need to send other information (i.e. SDP m and a lines) indicating the support of RTP/AVPF [RFC4585] and RTCP XR [RFC3611] as well as the appropriate address information for the Stream.

Note: In some cases (i.e. where timely feedback is not required and usage of the leap of faith initialization method)  transport layer feedback for urgent ECN information may not be required. See clause 3.3 / [IETF ECN] for further information.

In order to be notified of failures the MGC shall also set the "ECN failure" (ecnrous/fail) event.

To be able collect and Audit information regarding the use of ECN the MGC should set the "Source" (ecnrous/ssrc), "CE Counter" (ecnrous/cecount), "ECT 0 Counter" (ecnrous/zero), "ECT 1 Counter" (ecnrous/ectone), "Not-ECT Counter" (ecnrous/notetc), "Lost Packets Counter" (ecnrous/lost) and the "Extended Highest Sequence Number" (ecnrous/ehsn) Statistics.

If the MGC requires a more dynamic method of discovering the values of the above statistics it may utilise conditional statistic reporting [ITU-T H.248.47]. For example if the MGC requires notification of the reception of a CE marked packet then it should set the "Conditional Reporting" (scr/cr) event with the statistic identifier value set to "ecnrous/cecount" together with an appropriate condition parameter.

[End Correction]

2.4 Clause incorrect numbering

[Begin Correction]

Numbering of subclauses 7.6.2, 7.6.1.1, 7.6.1.2, 7.6.2 in clause 7 must be corrected.

[End Correction]

2.5 Editorial correction

[Begin Correction]

#### 7.6.1.2	ICE Parameter to Signal ECN Capability

As per clause 6.4 / [IETF ECN] an Internet Connectivity Exchange (ICE) [IETF RFC5245] flag may be used to indicate ECN support for ICE. However it is assumed that this attribute is used between MGCs when they have knowledge that the MG supports ECN functionality. This may be known through provisioning or an Audit of the Packages Descriptor indicating the support of the "ECN Package".

[End Correction]

2.6 ICE based ECN initialization method

Chapter 7.6.2 states that the support of 7.2.2 / [IETF ECN], i.e. the ICE based ECN initialization method, is for further study. The reason mentioned is that there is no support for the ECN field in the STUN package. If the ICE based ECN initialization method is for further study this should probably be highlighted also for the “ice” value of the initmethod property. However it is not clear for the contributor whether the ECN field needs to be transmitted between MGC and MG in order to ensure the proper operation of the method. Further analysis is needed. The item is brought up for discussion. No proposal is provided at this stage.

2.7 ECN behaviour in lack of initialization

A more precise reference to 7.6.2 is proposed in relation to the ECN behavioiur when no ECN initialization is performed.

[Begin Correction]

#### 7.6.3.2	No ECN Initiated

If no ECN initiation was performed the MG shall continue to operate according to clause 7.6.2 as per cases where "Initiation Method" equals "inactive".

[End Correction]

2.8 Detection of ECN failure

The text refers to clause 7.4 in [IETF ECN] for the description of the node behaviour when detecting ECN failures. However, subclause 7.4.1 is the one having a more normative description of how the node should react upon detection of ECN failures. Therefore it is proposed to also explicitly list the subclauses in the reference.

Furthermore, the ECN failure condition may be characterized by different error situations: high packet loss, mismatches in the number of ECN marks received and those indicated in the RTCP reports, etc. This Recommendation does not define precisely upon which conditions the ECN failure event with type set to “USE” should be triggered.

[Begin Correction]

### 7.6.4	Handling of dynamic groups through failure detection, verification and fallback

When detecting failures the MG should operate according to clauses 7.4, 7.4.1 and 7.4.2 / [IETF ECN]. This allows the MG to act autonomously to congestion, high packet loss rate and other failures without the need for interaction with a MGC. However if the MG determines that the use of ECN should be disabled it should indicate this to the MGC via the ECN failure event with type set to "USE"; the criteria for the MG to determine this is outside this Recommendation.

Associated with ECN are summary reports that may be used to detect various types of ECN path issues. These reports are per source SSRC and contain the following information: CE counter, ECT (0/1) counters, not-ECT Counter, Lost Packets counter, Extended Highest Sequence number. For more information see clause 7.4.2 / [IETF ECN]. In order to make this information available to a MGC this information is recorded in H.248 statistics. Thus the MGC should ensure that the statistics are set during the capability negotiation phase (see clause 6.1). The MGC may then perform an AuditValue.req on the statistics. In order to support multiple SSRCs per Stream the values of the Statistics are in a sub-list form. The values in a certain sub-list position are relative to each other across all the statistics in the ECN package.

[End Correction]

# Conclusion

It is proposed to accept above changes in the H.248.82 draft.

__________________