| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | TD-35 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.82 (ex H.248.ECN) |  |  |  |
| Title: | H.248.82 (ex H.248.ECN) “Gateway control protocol: Explicit Congestion Notification Support” (New): output text |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the editor’s output draft for draft new H.248.82 (ex H.248.ECN) “Explicit Congestion Notification Support” from the Q3/16 meeting in Andover, MA, USA, 18 - 22 July 2011.

AAP Summary

[To be provided before Consent]

Document History:

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.ECN Ed. 0.1 | Output from SG16, Geneva, 14-25th March 2011 meeting. Initial baseline based on contribution C.617, C.616, C.615, C.603, C.591R1 and TD 375/GEN. |
| H.248.82 Ed. 0.2 | Input to SG16/WP2, Andover, 18 - 22 July 2011 meeting. Text is identical to Ed 0.1 with the addition of the designation H.248.82. |
| H.248.82 Ed. 0.3 | Output from SG16/WP2, Andover, 18 - 22 July 2011 meeting. Changes are based on AVD-4048, AVD-4063, AVD-4068, AVD-4069, AVD-4124, AVD-4125, AVD-4126 and AVD-4127. |

Draft ITU-T Recommendation H.248.82

Gateway Control Protocol: Explicit Congestion Notification Support

AAP Summary

[To be added before Consent]

Summary

[To be added]

1	Scope

[To be added]

2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (09/05) + Amendment 1 (05/08) + Amendment 2 (12/09), Gateway Control Protocol: Version 3.

[ITU-T H.248.47]	Recommendation ITU-T H.248.47 (07/08), Gateway Control Protocol: Statistic conditional reporting package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/10), Gateway Control Protocol: NAT traversal toolkit packages.

[ITU-T H.248.64]	Recommendation ITU-T H.248.64 (12/09), Gateway Control Protocol: IP router packages.

[IETF RFC 3168]	IETF RFC 3168 (2001), The Addition of Explicit Congestion Notification (ECN) to IP.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 3611]	IETF RFC 3611 (2003), RTP Control Protocol Extended Reports (RTCP XR).

[IETF RFC 4585]	IETF RFC 4585 (2006), Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF).

[IETF RFC 5245]	IETF RFC 5245 (2010), Interactive Connectivity Establishment (ICE):A Protocol for Network Address Translator (NAT) Traversal for  Offer/Answer Protocols

[IETF RFC 5506]	IETF RFC 5506 (2009), Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences.

[IETF ECN]	IETF draft-ietf-avtcore-ecn-for-rtp-04, Explicit Congestion Notification (ECN) for RTP over UDP.

{Editor's Note: Further investigation to check the dependency of IETF RFC 2198 is solicited.}

3	Definitions

3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Sender [IETF ECN]: A sender of RTP packets carrying an encoded media stream. The sender has the possibility to effect how this transmission is performed.  It is one end-point of the ECN control loop.

NOTE: the sender in this sense may or may not be the same as the RTP End System.

3.1.2	Receiver [IETF ECN]: A receiver of RTP packets with the intention to consume the media stream.  It sends RTCP feedback on the received stream. It is the other end-point of the ECN control loop.

NOTE: the receiver in this sense may or may not be the same as the RTP End System.

3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	AuditValue.req: A H.248.1 AuditValue command request.

4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ABNF | Augmented Backus Naur Format |
| --- | --- |
| B2BIH | Back-to-Back Internet protocol Host |
| B2BRE | Back-to-Back RTP End system. |
| CE | Congestion Experienced |
| DCCP | Datagram Congestion Control Protocol |
| ECN | Explicit Congestion Notification |
| ECT | ECN Capable Transport |
| IP | Internet Protocol |
| IPR | Internet Protocol Router |
| LD | Local Destination |
| LS | Local Source |
| Lx | Layer number |
| MCU | Media Control Unit |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MSRP | Message Sending Relay Protocol |
| RD | Remote Destination |
| RS | Remote Source |
| RTP | Real Time Protocol |
| RTCP | RTP Control Protocol |
| SCTP | Stream Control Transport Protocol |
| SDP | Session Description Protocol |
| SIP | Session Initiation Protocol |
| SSRC | Synchronization Source |
| UDP | User Datagram Protocol |

5	Conventions

Elements of the H.248 protocol model, e.g. Context, Termination, Stream, Event are represented using the first letter capitalised. H.248 Property, Event, Signal and Parameter identities are given in italics. When used in the main text SDP syntax is highlighted in bold i.e. “m=”.

6	Use of ECN with H.248

H.248 entities (MG, MGC) may be involved with ECN in multiple ways. Clause 6.1 introduces the basic control principle. Subsequent clauses illustrate example IP transport (and possibly application) protocol specific use of ECN with H.248.

6.1	Basic ECN control loop

[IETF RFC 3168] defines an addition to IP for ECN support. This support relates to a unidirectional connection model, i.e., IP traffic in IP source host to IP destination host direction. Figure 1 illustrates the basic ECN control principle. The control loop may be abstracted by four components in the user plane:

- the congestion notification information flow in source to destination host direction;

- the feedback information flow in reverse direction, back to the traffic source;

- the actuating elements in case of indicated congestion (typically located in the source host); and

- the sensor elements, responsible for indicating congestion (at a network route, in an IP hop) by ECN marking in the IP path (see (1)).

There might be a further component in the control plane for:

- the indication and negotiation of a congestion control.

NOTE – 	The thick arrow shows the unidirectional IP packet path (where hop entities may provide ECN-CE marking. The thin arrow in reverse direction indicates protocol X packets in feedback direction.

Figure 1 – ECN supporting IP network (H.248 bearer plane) – 
Basic model of the ECN control loop

In Figure 1 the control loop spans two ECN-capable IP host entities with information flows in both directions, requiring a bidirectional communication path. There are different protocols in use for ECN (Figure 1) in each direction:

- IPv4 or IPv6 in the source-to-destination host direction for carrying ECN indications;

- Higher layer protocols in the reverse direction. This is normally the applied IP transport protocol if extended (like TCP) or designed for ECN support (like DCCP, SCTP); or a L4+ protocol (like RTCP in case of UDP transport).

H.248 MG entities may be involved in ECN control loops in various ways. There may be potential use cases for all four of the above indicated protocol groups to be involved in the control loop. Figure 1 outlines some possible MG roles.

6.2	Use of ECN with TCP traffic

TCP relates to a bidirectional transport connection, allowing the use of TCP control elements for the ECN feedback path. The use of ECN with TCP is defined in clause 6 of [IETF RFC 3168]. It is achieved primarily by the use of two additional TCP header flags. Figure 2 illustrates the control model of the use of ECN with TCP traffic.

Figure 2 – Use of ECN with TCP traffic

Use of ECN with TCP is relevant for H.248 MGs with TCP-based streams (e.g. used for MSRP-over-TCP based transport of Instant Messaging traffic).

6.3	Use of ECN with RTP-over-UDP traffic

UDP is a connectionless transport protocol, lacking control elements for carrying ECN feedback in the UDP header. The ECN feedback information path is thus delegated to higher layer protocols, e.g. RTCP in case of RTP-over-UDP sessions. Figure 3 summarizes the ECN solution framework as defined by [IETF ECN].

Figure 3 – Use of ECN with RTP-over-UDP traffic

There are multiple deployment scenarios, given interim NAT devices, NAT-traversal support functions or “middlebox” behaviour in the ECN IP path, as well as control plane options for RTP session control (see next clause). Figure 4 summarizes the procedures defined by [IETF ECN].

Figure 4 – Procedures for use of ECN with RTP-over-UDP traffic

6.4	Capability declaration and negotiation of ECN in the control plane

There are multiple options of control plane protocols (e.g., H.323, SIP, SIP-I) for the establishment and release of RTP sessions. Figures 3 and 4 indicate the example of SIP with its embedded SDP Offer/Answer protocol, used for end-to-end capability declaration and negotiation of ECN usage.

[IETF ECN] assumes the use of a SIP based control plane with additional SDP information elements.

Figure 5 outlines an example scenario in scope of this Recommendation, e.g., SIP as session control signalling protocol at MGC level, and the mapping of SIP-level SDP information to H.248 control elements, and an H.248 MG, which may provide various ECN behaviour (as outlined in clauses 6.1 and 6.5).

Figure 5 – Control plane example with H.248 network elements at the edge of an ECN domain

6.5	ECN interworking scenarios

For traffic and network engineering of IP networks with regards to ECN domains, the H.248 MG may play the role of an ECN-capable IP host endpoint or be located within the ECN IP path, as introduced in clause 6.1. ECN defines multiple code points for ECN marking in case of ECN Capable Transport (ECT), termed as ECT(0), ECT(1) and ECT(random) code points.

For traffic and network engineering of IP networks with regards to ECN domains, the ECT code point options and the various possible MG behaviours may lead to different ECN interworking scenarios. Some basic network configurations are illustrated below.

Figure 7 provides the example of a peering scenario. For instance, there may be different ECT code points in use in each ECN domain. The H.248 MG may then be requested to remark ECN code points in case of the end-to-end ECN control loop approach (option I.2 below).

NOTE – The IP topologies are basically associated with the H.248 Stream level. Above figure illustrates an example on Context level (referred to the IP topology models according clause 3.2.1 (B2BIH) and clause 6.5.1 (IP router) of [ITU-T H.248.64]).

Figure 6 – Peering separate ECN domains by H.248 (IP,IP) MGs

Each ECN domain may be also completely separated by limiting the control loops on each domain (option I.1).

Figure 8 illustrates another configuration, the example of an H.248 (IP,IP) MG located within the IP path of a single ECN domain.

Figure 7 – H.248 (IP,IP) MGs located in the IP path of a single ECN domain

The H.248 MG may be required to enforce different ECN support behaviour, like transparent forwarding of ECN information in IP headers, or the active marking of ECN bits due to “local congestion information”, or the reading and collecting of ECN information for local H.248 statistics, etc.

7	ECN for RTP-over-UDP Support Package

|  | Package Name: | ECN for RTP-over-UDP Support Package |
| --- | --- | --- |
|  | Package ID: | ecnrous (0x010b) |
|  | Description: | This package allows a MGC to indicate to a MG that it shall support Explicit Congestion Notification (ECN) for RTP over UDP (as defined by [IETF ECN]) on a particular Termination / Stream. |
|  | Version: | 1 |
|  | Extends: | None. |

7.1	Properties

7.1.1	ECN Enabled

|  | Property Name: | ECN Enabled |
| --- | --- | --- |
|  | Property ID: | ecnen (0x0001) |
|  | Description: | This property allows the MGC to indicate whether or not ECN is enabled on a Stream. |
|  | Type: | Boolean |
|  | Possible values: | True ECN is enabled False ECN is not enabled |
|  | Default: | False. |
|  | Defined in: | Local/Remote |
|  | Characteristics: | Read/Write |

7.1.2	Congestion Response Method

|  | Property Name: | Congestion response method |
| --- | --- | --- |
|  | Property ID: | crm (0x0002) |
|  | Description: | When RTP packets are received with ECN-CE marks from the remote side or local congestion is experienced, the sender and/or receivers MUST react with congestion control as-if those packets had been lost. However there are different control methods which a MG could use. This property allows the MGC to request different types of control. These types are defined in clause 7.3.3 / [IETF ECN]. |
|  | Type: | Enumeration |
|  | Possible values: | "SDCC" (0x0001) Sender Driven Congestion Control "RDCC" (0x0002) Receiver Driven Congestion Control "Hybrid" (0x0003) A hybrid mechanism The value “Hybrid” requires additional MG behaviour description which is outside the scope of this Recommendation. This could be specified within an application profile. |
|  | Default: | Provisioned. |
|  | Defined in: | Local/Remote "SDCC" in Local Descriptor implies the MG, when receiving ECN-CE marked media, generates ECN feedback towards the sender of the media. "RDCC" in Local Descriptor implies the MG reacts to the received ECN-CE marks without contacting the sender. NOTE: as a result of the action taken by the MG, the sender is eventually notified, for example of a change of subscription to a less bandwidth consuming layer or through bit adaptation requests. "SDCC" in Remote Descriptor implies ECN feedback may be received from the remote receiver and the MG shall react to this feedback for example with bit adaptation requests. "RDCC" in Remote Descriptor requires no action from the MG. |
|  | Characteristics: | Read/Write |

7.1.3	Initiation Method

|  | Property Name: | Initiation Method |
| --- | --- | --- |
|  | Property ID: | initmethod (0x0003) |
|  | Description: | This property indicates which ECN initiation method is to be used. This property is equivalent to the "init-value" defined by the ABNF grammar defined by clause 6.1/ [IETF ECN]. If the MGC requires that the MG choose the any appropriate initiation value it shall provide the list of possible value as an over specified list. |
|  | Type: | String |
|  | Possible values: | As per "init-value" in clause 6.1/ [IETF ECN]. "rtp", "ice" or "leap". This package also defines an additional value "inactive" that may be used with this property or for the "init-value" parameter if SDP is used. "Inactive" indicates that no ECN initiation is performed (and thus the ECN procedures in clause 7.6.3.1 do not apply) for the particular RTP stream. |
|  | Default: | Provisioned. |
|  | Defined in: | Local/Remote |
|  | Characteristics: | Read/Write |

7.1.4	ECN Mode

|  | Property Name: | ECN Mode |
| --- | --- | --- |
|  | Property ID: | mode (0x0004) |
|  | Description: | The ECN Mode parameter as defined in [IETF ECN] describes an endpoint’s ability to set and read ECN marks in UDP packets. Thus it describes a bi-directional flow. TheECN mode property in this package indicates whether the MG should set and/or read ECN marks. This property is equivalent to "mode" defined by the ABNF grammar defined by clause 6.1 [IETF ECN]. However the mode information is set in both the Local and Remote Descriptors. The property refers to the mode that the MG shall operate in order to determine the correct ECN procedures to apply. The MG specific semantic is detailed below. |
|  | Type: | Enumeration |
|  | Possible values: | As per "mode" clause 6.1/[IETF ECN]. "setonly" (0x0001); There is agreement to use ECN at the IP connection in the LS(A)-to-RD(A) direction.. "setread" (0x0002); There is agreement to use ECN at the IP connection in both directions, i.e. the LS(A)-to-RD(A) and RS(A)-to-LD(A) direction.. "readonly" (0x0003); There is agreement to use ECN at the IP connection in RS(A)-to-LD(A) direction.. Note: For directions see naming conventions in clause 5.2.3.1/[ITU-T H.248.1]. |
|  | Default: | "setread" |
|  | Defined in: | Local / Remote |
|  | Characteristics: | Read/Write |

7.1.5	ECT Marking

|  | Property Name: | ECT Marking |
| --- | --- | --- |
|  | Property ID: | ectmark (0x0005) |
|  | Description: | When set in the Remote Descriptor this property indicates how the MG should set ECN marks. When set in the local descriptor this property indicates which ECN mark is expected from the remote sender (NOTE1). The expected value however does not usually influence the MG behaviour, even in the case that it doesn’t correspond to the value actually received. This property is equivalent to the "ect" element defined by the ABNF grammar defined by clause 6.1 [IETF ECN]. NOTE 1 – There is also an ECN-CE marking function (besides ECT marking), which may be supported by H.248 MGs and interim IP devices. Such marking due to experienced local congestion is out of scope of this property. |
|  | Type: | Enumeration |
|  | Possible values: | As per "ect" clause 6.1/[IETF ECN]: "1" (0x0001) "0" (0x0002) "Random" (0x0003) |
|  | Default: | "0". |
|  | Defined in: | Local / Remote |
|  | Characteristics: | Read/Write |

7.1.6	ECN Congestion Marking

|  | Property Name: | ECN Congestion Marking |
| --- | --- | --- |
|  | Property ID: | congestmark (0x0006) |
|  | Description: | This property allows the MGC to indicate whether or not RTP packets may be ECN-CE marked by the MG (or not) if it the MG itself experiences congestion. |
|  | Type: | Enumeration |
|  | Possible values: | "mgdet" (0x0001): The MG determines if ECN Congestion Marking is enabled. "cemark" (0x0002): ECN Congestion marking is enabled. "nomark" (0x0003): ECN Congestion marking is not enabled. |
|  | Default: | Provisioned. |
|  | Defined in: | Local / Remote NOTE: the property is irrelevant when included in the Local Descriptor. |
|  | Characteristics: | Read Only |

7.1.7	ECN SDP Usage

|  | Property Name: | ECN SDP Usage |
| --- | --- | --- |
|  | Property ID: | ecnsdp (0x0007) |
|  | Description: | This property allows the MGC to determine which method for signalling the ECN "initiation method", "mode" and "ect marking" the MG supports. The MGC is may then use any method that the MG supports. |
|  | Type: | Enumeration |
|  | Possible values: | "P" (0x0001) The properties in clauses 7.1.1, 7.1.3, 7.1.4 and 7.1.5. "S" (0x0002) The SDP "a=ecn-capable-rtp" attribute. "B" (0x0003) Either of the above two methods. |
|  | Default: | Provisioned. |
|  | Defined in: | Termination State (Root Termination Only) |
|  | Characteristics: | Read Only |

7.2	Events

7.2.1	ECN Failure

|  | Event Name: | ECN Failure |
| --- | --- | --- |
|  | Event ID: | fail (0x0001) |
|  | Description: | This event indicates that ECN has failed on the indicated StreamID. The event may be set at a Termination or Stream level. The ObservedEvent shall return the StreamID of where the ObservedEvent was detected. The MG shall return the failure type. The MG may optionally return the path that has failed. |

7.2.1.1	EventsDescriptor parameters

None.

7.2.1.2	ObservedEventsDescriptor parameters

7.2.1.2.1	Failure Type

|  | Parameter Name: | Failure Type |
| --- | --- | --- |
|  | Parameter ID: | type (0x0001) |
|  | Description: | This parameter indicates at what stage ECN has failed. A failure may occur during the transport level initiation phase or during the ongoing use phase. A failure during the initiation phase indicates a problem with the signalling used for the initiation of ECN. A failure during the ongoing use phase indicates that whilst ECN was successfully negotiated, the continued use of ECN is causing problems. The detection of failures during ongoing use is described in clause 7.4 / [IETF ECN]. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | INIT (0x0001): Failure during initiation Phase USE (0x0002): Failure during ongoing use phase |
|  | Default: | None. |

7.2.1.2.2	Media Sender SSRC

|  | Parameter Name: | Media Sender SSRC |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter contains the SSRC of the media sender with whom ECN has failed. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | SSRC as per [IETF RFC 3550]. |
|  | Default: | None. If not sent it indicates all media senders associated with the StreamID have failed. |

7.3	Signals

None.

7.4	Statistics

7.4.1	Source (SSRC)

|  | Statistic Name: | Source |
| --- | --- | --- |
|  | Statistic ID: | ssrc (0x0001) |
|  | Description: | This statistic provides a list of RTP sources associated with the Stream. |
|  | Type: | Sub-list of integer |
|  | Possible values: | SSRC as per [IETF RFC 3550]. |
|  | Level: | Stream |

7.4.2	CE Counter

|  | Statistic Name: | CE Counter |
| --- | --- | --- |
|  | Statistic ID: | cecount (0x0002) |
|  | Description: | This statistic provides the number of RTP packets received so far in the session with an ECN field set to CE for a particular SSRC within the Stream. Each position of the sub-list is associated with a SSRC at related position in the ssrc statistic. See clause 7.4.2 / [IETF ECN] for more details. |
|  | Type: | Sub-list of double |
|  | Possible values: | 0 and upwards. |
|  | Level: | Stream |

7.4.3	ECT 0 Counter

|  | Statistic Name: | ECT0 Counter |
| --- | --- | --- |
|  | Statistic ID: | ectzero (0x0003) |
|  | Description: | This statistic provides the number of RTP packets received so far in the session with an ECN field set to ECT (0) for a particular SSRC within the Stream. Each position of the sub-list is associated with a SSRC at related position in the ssrc statistic. See clause 7.4.2 / [IETF ECN] for more details. |
|  | Type: | Sub-list of double |
|  | Possible values: | 0 and upwards. |
|  | Level: | Stream |

7.4.4	ECT 1 Counter

|  | Statistic Name: | ECT1 Counter |
| --- | --- | --- |
|  | Statistic ID: | ectone (0x0004) |
|  | Description: | This statistic provides the number of RTP packets received so far in the session with an ECN field set to ECT (1) for a particular SSRC within the Stream. Each position of the sub-list is associated with a SSRC at related position in the ssrc statistic. See clause 7.4.2 / [IETF ECN] for more details. |
|  | Type: | Sub-list of double |
|  | Possible values: | 0 and upwards. |
|  | Level: | Stream |

7.4.5	Not-ECT Counter

|  | Statistic Name: | Not-ECT Counter |
| --- | --- | --- |
|  | Statistic ID: | notetc (0x0005) |
|  | Description: | This statistic provides the number of RTP packets received so far in the session with an ECN field set to not-ECT for a particular SSRC within the Stream. Each position of the sub-list is associated with a SSRC at related position in the ssrc statistic. See clause 7.4.2 / [IETF ECN] for more details. |
|  | Type: | Sub-list of double |
|  | Possible values: | 0 and upwards. |
|  | Level: | Stream |

7.4.6	Lost Packets Counter

|  | Statistic Name: | Lost Packets Counter |
| --- | --- | --- |
|  | Statistic ID: | lost (0x0006) |
|  | Description: | This statistic provides the number of RTP packets that are expected minus the number received for a particular SSRC within the Stream. Each position of the sub-list is associated with a SSRC at related position in the ssrc statistic. See clause 7.4.2 / [IETF ECN] for more details. |
|  | Type: | Sub-list of double |
|  | Possible values: | 0 and upwards. |
|  | Level: | Stream |

7.4.7	Extended Highest Sequence number

|  | Statistic Name: | Extended Highest Sequence number |
| --- | --- | --- |
|  | Statistic ID: | ehsn (0x0007) |
|  | Description: | This statistic provides the highest sequence number seen when sending this report, but with additional bits, to handle disambiguation when wrapping the RTP sequence number field for a particular SSRC within the Stream. Each position of the sub-list is associated with a SSRC at related position in the ssrc statistic. See clause 7.4.2 / [IETF ECN] for more details. |
|  | Type: | Sub-list of integer |
|  | Possible values: | 0 and upwards. |
|  | Level: | Stream |

7.4.8	Duplication Counter

|  | Statistic Name: | Duplication Counter |
| --- | --- | --- |
|  | Statistic ID: | dup (0x0008) |
|  | Description: | This statistic provides the cumulative number of RTP packets received that are a duplicate of an already received packet from this SSRC since the receiver joined the RTP session. Each position of the sub-list is associated with a SSRC at related position in the ssrc statistic. See clause 7.4.2 / [IETF ECN] for more details. |
|  | Type: | Sub-list of integer |
|  | Possible values: | 0 and upwards. |
|  | Level: | Stream |

7.5	Error Codes

None.

7.6	Procedures

7.6.1	RTP topology

ECN functionality may be used with different RTP session topologies. The ECN behaviour in conjunction with certain RTP topologies is described in clause 3.2 / [IETF ECN].  Through the MGC setting the combination of Terminations/Streams in a Context, the properties in the "ECN for RTP-over-UDP Support" and other elements in the Local and Remote Descriptors the MG is able to determine the correct ECN behaviour.

7.6.2	Negotiation of the capability to use ECN with RTP/UDP/IP

As per [IETF ECN] a MG supporting ECN with RTP/UDP/IP requires different RTCP extensions:

- RTP/AVPF [IETF RFC 4585] transport layer feedback format for urgent ECN information,

- RTCP XR [IETF RFC 3611] ECN summary report block type for regular reporting of the ECN marking information.

Thus in addition to the methods for indicating/negotiating ECN described below the MGC may need to send other information (i.e. SDP m and a lines) indicating the support of RTP/AVPF [IETF RFC 4585] and RTCP XR [IETF RFC 3611] as well as the appropriate address information for the Stream.

Note: In some cases (i.e. where timely feedback is not required and usage of the leap of faith initialization method) transport layer feedback for urgent ECN information may not be required. See clause 3.3 / [IETF ECN] for further information on the need for transport layer feedback.

In order to be notified of failures the MGC shall also set the "ECN failure" (ecnrous/fail) event.

If the MGC requires information regarding the use of ECN the MGC should set the "Source" (ecnrous/ssrc), "CE Counter" (ecnrous/cecount), "ECT 0 Counter" (ecnrous/zero), "ECT 1 Counter" (ecnrous/ectone), "Not-ECT Counter" (ecnrous/notetc), "Lost Packets Counter" (ecnrous/lost) and the "Extended Highest Sequence Number" (ecnrous/ehsn) Statistics. Alternatively, for packet loss counters, the statistics defined in the RTP package [ITU-T H.248.1] can be used.

[ITU-T H.248.47] provides a means to dynamically discover the values of statistics. However [ITU-T H.248.47] is only applicable where a statistic has a single value. It does not support statistics with type "list of". Thus if [ITU-T H.248.47] is used with the statistics in this Recommendation it shall be used in the case that a statistic relates to a single SSRC.

7.6.2.1	Signalling ECN Capability

In order to use ECN, the use of ECN needs to be enabled and information regarding the "initiation method", "ecn mode" and "ect marking" needs to be determined. These may be communicated to the MG either utilising the "ECN Enable" (ecnrous/en), "Initiation Method" (ecnrous/initmethod), "ECN Mode" (ecnrous/mode) and "ECT Marking" (ecnrous/ectmark) properties or via the use of the "a=ecn-capable-rtp" SDP attribute as defined in clause 6.1 / [IETF ECN]. The MGC can determine which method is supported via an audit of the "ECN SDP Usage" (ecnrous/ecnsdp) property. Alternatively, the supported method may be provisioned. For the property based signalling method any property that has a provisioned value does not need to be sent for the relevant Stream. For the SDP based method the MGC shall include a CHOOSE wildcard for the particular information element in question.

In order to utilise the RTCP ECN feedback and the RTCP XR block for ECN summary the use of these should be indicated as per clauses 6.2 and 6.3 / [IETF ECN].

If the MGC requires that the MG apply a particular congestion control method as a result of the reception of ECN-CE marked RTP packets it shall send the "Congestion Response Method" (ecnrous/crm) property to the MG with the required value.

Furthermore the MGC may also control through the use of the "ECN Congestion Marking" (ecnrous/congestmark) property whether or not RTP packets may be ECN-CE marked by the MG (or not) if it the MG itself experiences congestion.

The above properties are defined for use in the Local and Remote Descriptors as these are related to stream resources which need to be reserved on a per stream basis. H.248 Streams are bi-directional in nature with one direction described by the Local Descriptor and the other via the Remote Descriptor. This allows an asymmetric usage of ECN. However for a typically bi-directional use case, symmetric usage with the parameters placed in both the Local and Remote Descriptors may be assumed.

7.6.2.2	ICE Parameter to Signal ECN Capability

As per clause 6.4 / [IETF ECN] an Internet Connectivity Exchange (ICE) [IETF RFC5245] flag may be used to indicate ECN support for ICE. However it is assumed that this attribute is used between MGCs when they have knowledge that the MG supports ECN functionality. This may be known through provisioning or an Audit of the Packages Descriptor indicating the support of the "ECN Package".

7.6.3	Initiation and initial verification of ECN capable transport

The use and initiation of ECN on a Termination / Stream is mainly determined via the setting of "ECN Enabled" equal to "True" on a Termination/Stream in combination with an "Initiation Method". [IETF ECN] defines three initiation methods "rtp", "ice" and "leap". This package defines an additional method "inactive" which indicates that ECN initiation procedures should not be started. Other properties may be examined in the Local and Remote Descriptors to determine media handling that may have an impact on ECN procedures and error handling. For example to determine:

- If there is a change in packetisation (segmentation/reassembly),

- If there is transcoding.

NOTE: As per [IETF ECN] a transcoding scenario requires the use of ECN properties to split the ECN connection in two parts, each with its own negotiation.

If "ECN Enabled" equal to "True" and "Initiation Method" equal to "inactive" (either by default or specification) is set on the same stream (and direction) on both Terminations of a Context with no other ECN related properties set then the MG shall not perform ECN initiation. It shall behave as an ECN aware MG and copy ECN bits (including CE information) from ingress packets to the egress one.

If "ECN Enabled" equal to "True" and "Initiation Method" equal to "inactive" (either by default or specification) is set on the same stream (and direction) on both Terminations of a Context with no other ECN related properties set, but the MG performs packet segmentation or reassembly, without media transcoding, the MG derives ECT and ECN-CE marks in the egress stream from the corresponding ones in the ingress stream, as specified in [IETF ECN].

If "ECN Enabled" is equal to "True", "Initiation Method" is set a value other than "inactive" and other ECN related properties are set then the MG shall perform ECN initiation and processing. The Termination at which these properties are set shall behave as an ECN endpoint, i.e. it shall not pass ECT markings transparently to its egress even if the corresponding termination has identical stream properties.

NOTE -The MGC may determine that ECN processing is required as a result of the need for transcoding.  [IETF ECN] indicates that where transcoding occurs, a middlebox (i.e. the MG) shall process ECN as separate ECN Endpoints. ECN processing may also be required due to independent ECN negotiation at the call control layer, independent of transcoding.

ECN processing may also be triggered by setting "ECN Enabled" equal to "True" on a single Termination/Stream and "ECN Enable" equal to "False" on the opposite Termination/Stream. If behaviour other than the specified default behaviour is required the properties in this package related to that behaviour shall be signalled.

NOTE - The support of clause 7.2.2 / [IETF ECN] is FFS. [ITU-T H.248.50] supports STUN and ICE but does not support an ECN field.

As per clause 7.2.1 / [IETF ECN] the transport level negotiation of ECN may be unsuccessful and this should be logged. The ECN failure event (ecnrous/fail) with failure type "INIT" is used to indicate this to the MGC.

7.6.4	Ongoing use of ECN within an RTP session

7.6.4.1	ECN Initiated

Once the MG has initiated ECN it shall operate according to clause 7.3 / [IETF ECN]. If reduced size RTCP [IETF RFC5506] is to be used this should be indicated (via "a=rtcp-rsize") this should be signalled as part of clause 6.1.

When RTP packets are received with ECN-CE marks, senders and/or receivers shall react with an appropriate congestion control. One of several different types of congestion control may be used (see clause 7.3.3 / [IETF ECN]). When the MG receives RTP packets with ECN-CE marks it should act according to the type specified in the "Congestion Response Method" (ecnrous/crm) property.

On receipt of the RTP packets with ECN-CE marks if the MGC has previously requested notification of this (via [ITU-T H.248.47], see clause 7.6.2 above) and the relevant criteria is met then the MG shall notify the MGC of this ObservedEvent.

7.6.4.2	No ECN Initiated

If no ECN initiation was performed the MG shall continue to operate according to clause 7.6.2 as per cases where "Initiation Method" equals "inactive".

7.6.5	Handling of dynamic groups through failure detection, verification and fallback

When detecting failures the MG should operate according to clause 7.4, 7.4.1 and 7.4.2 / [IETF ECN]. This allows the MG to act autonomously to congestion, high packet loss rate and other failures without the need for interaction with a MGC. However if the MG determines that the use of ECN should be disabled it should indicate this to the MGC via the ECN failure event (ecnrous/fail) with type set to "USE"; the criteria for the MG to determine this is outside this Recommendation.

Associated with ECN are summary reports that may be used to detect various types of ECN path issues. These reports are per source SSRC and contain the following information: CE counter, ECT (0/1) counters, not-ECT Counter, Lost Packets counter, Extended Highest Sequence number and Duplication Counter. For more information see clause 7.4.2 / [IETF ECN]. The MG sends these reports to its peer using RTCP. The MGC may also request this information for its own purposes, which are outside this Recommendation. In order to make this information available to a MGC this information is recorded in H.248 statistics. Thus if the MGC requires this information, it should set the statistics during the capability negotiation phase (see clause 6.1). The MGC may then perform an AuditValue.req on the statistics. In order to support multiple SSRCs per Stream the values of the Statistics are in a sub-list form. The values in a certain sub-list position are relative to each other across all the statistics in the ECN package.

Appendix I

Living list of open issues

(This appendix does not form an integral part of this Recommendation)

I.1	ECN capability for TCP's SYN/ACK Packets

RFC 5562 “Adding Explicit Congestion Notification (ECN) Capability to TCP's SYN/ACK Packets” indicates the coexistence of different semantics related to the IP header ECN field. This item needs to be considered from interoperability (ECN interworking) perspective.

This RFC is related to the ECN for TCP scenario, and defines an extension to the basic ECN TCP control loop from RFC 3168.

This RFC describes an optional, experimental modification to RFC 3168 to allow TCP SYN/ACK packets to be ECN-Capable.  For TCP, RFC 3168 specifies setting an ECN-Capable codepoint on data packets, but not on SYN and SYN/ACK packets.  However, because of the high cost to the TCP transfer of having a SYN/ACK packet dropped, with the resulting retransmission timeout, this document describes the use of ECN for the SYN/ACK packet itself, when sent in response to a SYN packet with the two ECN flags set in the TCP header, indicating a willingness to use ECN.  Setting the initial TCP SYN/ACK packet as ECN-Capable can be of great benefit to the TCP connection, avoiding the severe penalty of a retransmission timeout for a connection that has not yet started placing a load on the network.  The TCP responder (the sender of the SYN/ACK packet) must reply to a report of an ECN-marked SYN/ACK packet by resending a SYN/ACK packet that is not ECN-Capable.  If the resent SYN/ACK packet is acknowledged, then the TCP responder reduces its initial congestion window from two, three, or four segments to one segment, thereby reducing the subsequent load from that connection on the network.  If instead the SYN/ACK packet is dropped, or for some other reason the TCP responder does not receive an acknowledgement in the specified time, the TCP responder follows TCP standards for a dropped SYN/ACK packet (setting the retransmission timer).

It may be noted that there might be a potential relation to Draft H.248.NATTP2P because NAT traversal support for TCP bearers is related to the TCP connection establishment phase (i.e., TCP SYN/ACK packets). (Note: the network functions of congestion control and NAT traversal support itself are decoupled.)

I.2	Alternate semantics

RFC 4774 “Specifying Alternate Semantics for the Explicit Congestion Notification (ECN) Field” indicates the coexistence of different semantics related to the IP header ECN field. This item needs to be considered from interoperability (ECN interworking) perspective.

For example, see Figure 1/RFC 4774:

|--------|

Alternate-ECN traffic ---->  |        | ---> CE-marked packet

|  Old   |

Non-ECN traffic ---------->  | Router | ---> dropped packet

|        |

RFC-3168 ECN traffic ----->  |        | ---> CE-marked packet

|--------|

Figure 1: Alternate-ECN traffic, an old router, using RFC-3168 ECN,

that is congested and ready to drop or mark the arriving packet.

Draft H.248.ECN should indicate potential interoperability issues, e.g. when an H.248 MG would be connected to “an old router” or just peering an “ECN domain” with a different semantic. Such information could be provided e.g. in the scope section, or an Appendix.

Bibliography

TBD

_____________________