| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 426 (PLEN/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 21 November – 02 December 2011 |
| TEMPORARY DOCUMENT |  |  |
| Source: | Editor H.248.12 |  |
| Title: | Consent: H.248.12 “Gateway control protocol: H.248.1 packages for H.323 and H.324 interworking” (Rev) |  |

1.	Introduction

This document contains the draft of revised H.248.12 “Gate Control Protocol: H.248.1 packages for H.323 and H.324 interworking” for Consent.

AAP Summary

This Recommendation defines several H.248.1 packages that are intended for Media Gateways designed to support interworking between H.323 terminals and H.324 terminals.

Amendment 1 defined packages, which extend the existing H.324, H.245 command and H.245 indication packages by adding properties.

Amendment 2 defines a package which enables sending H.245 messages between a Media Gateway Controller (MGC) and a Media Gateway (MG) when the interworking function is handled by the MGC.

This revision allows the MGC to request the MG to report when the h223Skewindication parameter exceeds a certain amount.

(Full text available only in the electronic version)

CONTENTS

Page

1	Introduction	5

2	Scope	5

3	References	6

4	H.324 package	8

4.1	Properties	9

4.1.1	Communication mode	9

4.1.2	Highest Multiplexing Level	9

4.1.3	Demultiplex	10

4.1.4	Remote H.223 capability	10

4.1.5	Incoming Multiplex Table	11

4.1.6	Outgoing Multiplex Table	11

4.2	Events	12

4.3	Signals	12

4.4	Statistics	12

4.4.1	MUXPDU sent	12

4.4.2	MUXPDU received	12

4.4.3	MUXPDU error	12

4.5	Procedures	12

5	H.245 Package	13

5.1	Properties	13

5.1.1	H.245 channel state	13

5.1.2	Terminal Type	13

5.2	Events	14

5.2.1	H.245 Message	14

5.2.2	H.245 Channel Closed	17

5.3	Signals	17

5.4	Statistics	17

5.5	Procedures	17

6	H.323 Bearer Control Package	17

6.1	Properties	18

6.1.1	Fast Connect	18

6.1.2	H.245 Message Encapsulation	18

6.2	Events	19

6.3	Signals	19

6.4	Statistics	19

6.5	Procedures	19

7	H.245 Command Package	20

7.1	Properties	20

7.1.1	Incoming MiscellaneousCommand	20

7.1.2	Outgoing Miscellaneous Command	20

7.1.3	Incoming H223MultiplexReconfiguration Command	21

7.1.4	Outgoing H223MultiplexReconfiguration Command	21

7.2	Events	21

7.3	Signals	21

7.4	Statistics	22

7.5	Procedures	22

8	H.245 Indication package	22

8.1	Properties	22

8.1.1	Incoming MiscellaneousIndication	22

8.1.2	Outgoing MiscellaneousIndication	22

8.2	Events	23

8.3	Signals	23

8.4	Statistics	23

8.5	Procedures	23

9	Call flows	23

9.1	H.323-Annex C/H.324 interworking with H.245 in MG	23

9.1.1	Calls with origination by the Annex C/H.324 side	23

9.1.2	Calls with origination by the H.323 side	25

9.2	H.323-Annex C/H.324 interworking with H.245 in MGC	27

9.3	Tunnelling of Annex C/H.324 bitstream	28

Annex A  Extended H.324, H.245 command and H.245 indication packages	30

A.1	Scope	30

A.2	References	30

A.3	Definitions	30

A.4	Abbreviations and acronyms	30

A.5	Extended H.324 package	31

A.5.1	Properties	31

A.5.2	Events	32

A.5.3	Signals	32

A.5.4	Statistics	32

A.5.5	Procedures	32

A.6	Extended H.245 command package	35

A.6.1	Properties	35

A.6.2	Events	36

A.6.3	Signals	36

A.6.4	Statistics	36

A.6.5	Procedures	36

A.7	Extended H.245 indication package	37

A.7.1	Properties	37

A.7.2	Events	38

A.7.2.1	Skew Update	38

A.7.3	Signals	39

A.7.4	Statistics	39

A.7.5	Procedures	39

A.8	H.245 transport package	40

A.8.1	Properties	41

A.8.2	Events	41

A.8.3	Signals	41

A.8.4	Statistics	42

A.8.5	Procedures	42

Bibliography	42

Draft revised ITU-T Recommendation H.248.12

Gateway control protocol: 
H.248.1 packages for H.323 and H.324 interworking

Summary

This Recommendation defines several H.248.1 packages that are intended for Media Gateways designed to support interworking between H.323 terminals and H.324 terminals. More specifically, this Recommendation contains: package "h324" for creating terminations supporting H.324, package "h245" for creating terminations supporting H.245 channels for H.324 calls, package "h323bc" for creating terminations supporting H.245 channels for H.323 calls, package "h245com" for the communication of H.245 commands between a MGC and MG, and package "h245ind" for the communication of H.245 indications.

NOTE  This Recommendation has been renumbered. It was formerly known as ITU-T Rec. H.248, Annex M4.

Amendment 1 defined packages, which extend the existing H.324, H.245 command and H.245 indication packages by adding properties.

Amendment 2 defines a package which enables sending H.245 messages between a Media Gateway Controller (MGC) and a Media Gateway (MG) when the interworking function is handled by the MGC.

This revision allows the MGC to request the MG to report when the h223Skewindication parameter exceeds a certain amount.

# 1	Introduction

This Recommendation gathers packages for H.245, H.245 parameters specific to H-series audiovisual terminals, and Annex C/H.324 terminals for use with the H.248.1 gateway control protocol. The packages in this Recommendation are in conformance with clause 12/H.248.1 package definition guidelines.

Amendment 1 introduced extended H.324, H.245 command and H.245 indication packages for interworking with H.324 terminals having different H.324 capabilities, when the interworking function is handled by the MGC.

Amendment 2 introduces a H.245 transport package that allows H.245 signalling to be sent between a MGC and MG across H.248.

# 2	Scope

This Recommendation describes packages for the H.248.1 gateway control protocol related to interworking of H.323 and H.324 terminals, as follows:

Package "h324" for termination of H.324 bitstream on MGs.

Package "h245" for termination of H.245 messages on MGs.

Package "h323bc" for H.245 parameters specific to H.323.

Package "h245com" for providing properties that allow the MGC to indicate to the MG that the MGC has sent or received an H.245 command.

Package "h245ind" for providing properties that allow the MGC to indicate to the MG that the MGC has sent or received an H.245 indication.

With the terminations implementing "h324", "h245com" and "h245ind" packages, the decomposed gateway may support H.324 communication with the H.245 control function in the MGC. In the H.324 and H.323 interworking scenario with this decomposed gateway, H.245 control messages are terminated in the MGC (a backhaul is used between the MG and the MGC to transport H.245 messages between the MG and the MGC, on the H.324 side). The terminations for media on both H.324 and H.323 sides are managed by the MGC.

With the terminations implementing "h324" and "h245" packages, the decomposed gateway may support H.324 communication with H.245 control in the MG. In the H.324 and H.323 interworking scenario with this decomposed gateway, H.245 control messages from the H.324 side and the H.323 side are terminated in the "h245" termination and in the "h323bc" termination respectively, and are processed in the MG. The MG manages the logical channels for media.

Annex A describes extended H.324, H.245 command, H.245 indication and H.245 transport packages for interworking with H.324 terminals having different H.324 capabilities, when the interworking function is handled by the MGC.

# 3	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.223]	Recommendation ITU-T H.223 (07/2001), Multiplexing protocol for low bit rate multimedia communication

[ITU-T H.245]	Recommendation ITU-T H.245 (05/2006), Control protocol for multimedia communication

[ITU-T H.323]	Recommendation ITU-T H.323 (11/2000), Packet-based multimedia communications systems

[ITU-T H.324]	Recommendation ITU-T H.324 (09/2005), Terminal for low bit-rate multimedia communication

[ITU-T V.8]	Recommendation ITU-T V.8 (11/2000), Procedures for starting sessions of data transmission over the public switched telephone network

[ITU-T V.8bis]	Recommendation ITU-T V.8bis (11/2000), Procedures for the identification and selection of common modes of operation between data circuit-terminating equipments (DCEs) and between data terminal equipments (DTEs) over the public switched telephone network and on leased point-to-point telephone-type circuits

[ITU-T V.34]	Recommendation ITU-T V.34 (02/1998), A modem operating at data signalling rates of up to 33 600 bit/s for use on the general switched telephone network and on leased point-to-point 2-wire telephone-type circuits

[ITU-T V.42]	Recommendation ITU-T V.42 (10/1996), Error-correcting procedures for DCEs using asynchronous-to-synchronous conversion

[ITU-T V.42bis]	Recommendation ITU-T V.42bis (01/1990), Data compression procedures for data circuit-terminating equipment (DCE) using error correction procedures

[ITU-T V.140]	Recommendation ITU-T V.140 (02/1998), Procedures for establishing communication between two multiprotocol audiovisual terminals using digital channels at a multiple of 64 or 56 kbit/s

[ITU-T X.691]	Recommendation ITU-T X.691 (07/2002), Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)

-

--

# 4	H.324 package

|  | Package Name: | H.324 |
| --- | --- | --- |
|  | Package ID: | h324 (0x002c) |
|  | Version: | 1 |
|  | Extends: | None |
|  | Description: | This package is defined to support terminations for calls using ITU-T Rec. H.324. The transport mechanism or bearer channel will be different for each environment where the package is used. Figure 1 shows the functional view of a H.324 multimedia call and the MGC control points defined in the H.324 package. In ITU-T Rec. H.324, there are three communication modes defined according to the underlying network: a PSTN mode defined in ITU-T Rec. H.324 main body (H.324P), a mobile network mode defined in Annex C/H.324 (H.324M), and an ISDN mode defined in Annex D/H.324 (H.324I). One of these communication modes shall be selected when the termination implementing the H.324 package is invoked by the MGC. In version 1 of the H.324 package, only the functionalities required of the H.324M mode are defined. The functionalities for H.324P and H.324I are FFS, and will be defined in version 2, or later. |

Figure 1/H.248.12  Control point defined in H.324 package

## 4.1	Properties

### 4.1.1	Communication mode

|  | Property Name: | Communication mode |
| --- | --- | --- |
|  | Property ID: | cmod (0x0001) |
|  | Type: | Enumeration |
|  | Possible values: | H324P (0x0001): H324 operating in the base PSTN mode using a V-series modem H324M (0x0002): Annex C/H.324 (Mobile) H324I (0x0003): Annex D/H.324 (ISDN) |
|  | Default: | Provisioned |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |
|  | Description: | This property indicates the communication mode to be followed by the termination. |

### 4.1.2	Highest Multiplexing Level

|  | Property Name: | Highest Multiplexing level |
| --- | --- | --- |
|  | Property ID: | muxlv (0x0002) |
|  | Type: | Enumeration |
|  | Possible values: | Level0 (0x0001): H.223 level 0 defined in ITU-T Rec. H.223 Level1 (0x0002): H.223 level 1 defined in Annex A/H.223 Level2 (0x0003): H.223 level 2 defined in Annex B/H.223 Level3 (0x0004): H.223 level 3 defined in Annex C/H.223 |
|  | Default: | Level0 |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |
|  | Description: | This property indicates the highest level of H.223 multiplexing which can be used by the termination. The indicated level is used in the multiplex level initialization phase as defined in C.6/H.324. If the communication mode property is set to 'H324P' or 'H324I', the value of the highest multiplexing level property shall be set to 'Level0'. |

### 4.1.3	Demultiplex

|  | Property Name: | Demultiplex |
| --- | --- | --- |
|  | Property ID: | demux (0x0003) |
|  | Type: | Boolean |
|  | Possible values: | TRUE (0x0001): H.223 multiplexed stream is demultiplexed FALSE (0x0000): H.223 multiplexed stream is not demultiplexed |
|  | Default: | TRUE |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | This property specifies the operation on the H.223 multiplexed stream received by the termination. If set to 'FALSE', the H.223 multiplexed stream is relayed to the other termination without being demultiplexed. If set to 'TRUE', each logical channel data is extracted from the multiplexed stream, and is given to the appropriate termination. |

### 4.1.4	Remote H.223 capability

|  | Property Name: | Remote H.223 capability |
| --- | --- | --- |
|  | Property ID: | h223capr (0x0004) |
|  | Type: | Octet String |
|  | Possible values: | This property indicates the value of the H.245 H223Capability structure encoded by applying the Packed Encoding Rules (PER) specified in ITU-T Rec. X.691. |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | This property indicates the remote endpoint's capabilities specific to the H.223 multiplex capability. The MGC shall set the value of the 'h223capr' property to the value of the H223Capability in the received H.245 message. |

### 4.1.5	Incoming Multiplex Table

|  | Property Name: | Incoming Multiplex Table |
| --- | --- | --- |
|  | Property ID: | muxtbl_in (0x0005) |
|  | Type: | Octet String |
|  | Possible values: | This property indicates the value of H.245 MultiplexEntrySend message received by the MGC (see description for details). The MultiplexEntrySend structure is encoded by applying PER. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | This property indicates the H.223 multiplex table which associates each octet within an H.223 MUX-PDU with a particular logical channel number. Upon receipt of the MultiplexEntrySend message from the remote endpoint, the MGC shall decide whether it confirms or rejects each multiplex table entry. The MGC shall create an equivalent MultiplexEntrySend message that contains only the entries that it confirmed to the remote H.324 terminal and the MGC shall send the created message as the content of the muxtbl_in property so that the MG can perform the demultiplexing of the incoming H.223 MUX-PDUs. |

### 4.1.6	Outgoing Multiplex Table

|  | Property Name: | Outgoing Multiplex Table |
| --- | --- | --- |
|  | Property ID: | muxtbl_out (0x0006) |
|  | Type: | Octet String |
|  | Possible values: | This property indicates the value of H.245 MultiplexEntrySend message sent by the MGC (see description for details). The MultiplexEntrySend structure is encoded by applying PER. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | This property indicates the H.223 multiplex table which associates each octet within an H.223 MUX-PDU with a particular logical channel number. Upon receipt of the MultiplexEntrySendAck message from the remote endpoint, the MGC shall send multiplex table entries which are confirmed by the remote endpoint as the content of the muxtbl_out property. Thus the MG can perform the multiplexing of the outgoing H.223 MUX-PDUs. |

## 4.2	Events

None.

## 4.3	Signals

None.

## 4.4	Statistics

### 4.4.1	MUXPDU sent

|  | Statistics ID: | muxsent (0x0001) |
| --- | --- | --- |
|  | Type: | Integer |
|  | Units: | number of MUX-PDUs |
|  | Description: | Number of MUX-PDUs sent from the termination. |

### 4.4.2	MUXPDU received

|  | Statistics ID: | muxrec (0x0002) |
| --- | --- | --- |
|  | Type: | integer |
|  | Units: | number of MUX-PDUs |
|  | Description: | Number of MUX-PDUs received by the termination. |

### 4.4.3	MUXPDU error

|  | Statistics ID: | muxerr (0x0003) |
| --- | --- | --- |
|  | Units: | number of MUX-PDUs |
|  | Description: | Number of the received MUX-PDU in which MUX-PDU header error is detected. |

## 4.5	Procedures

MGC shall create an H.324 termination with the appropriate value of the "cmod" property. MG shall establish the connection according to procedures defined in ITU-T Rec. H.324 for the communication mode indicated by the 'cmod' property.

For the version 1 of the package, only H324M mode is supported.

After the connection is established with the communication mode of 'H324M', the MG shall immediately start the H.223 level setup procedure as defined in Annex C/H.324 according to the value of the 'muxlv' property. After agreement of H.223 multiplexing level, the MG shall start to receive and transmit H.223 MUX-PDUs.

If the 'demux' property is set to 'FALSE', the received MUX-PDUs should be passed to the termination connected to the H.324 termination without being demultiplexed. If the 'demux' property is set to 'TRUE', the MG shall demultiplex the received MUX-PDUs to logical channels. The first logical channel opened after the connection is established, is logical channel 0 for H.245 control messages. The MGC may choose whether H.245 control is located in the MGC or in the MG.

If the MGC decides to allow the MG to manage H.245 control, the termination which implements the H.245 package shall be invoked and connected to the H.324 termination. The H.245 messages are exchanged between the remote endpoint and the H.245 termination via the H.324 termination, and the media logical channels may be opened or closed without indication from the MGC.

If the MGC decides to execute H.245 control on itself, the MGC shall not invoke the H.245 termination connected to the H.324 termination. The H.245 messages received/transmitted by the H.324 termination are forwarded to/from the MGC. The MG shall control the H.223 operation according to the 'h223capr' property indicated by the MGC. The 'muxtbl' property shall be used to inform the MG of the incoming/outgoing multiplex table entry.

# 5	H.245 Package

|  | Package Name: | H.245 |
| --- | --- | --- |
|  | Package ID: | h245 (0x002a) |
|  | Version: | 1 |
|  | Extends: | None |
|  | Description: | This package is defined to support MGC-MG configurations where H.245 messages are received on the MG device. This package shall be implemented only if the MG supports the scenario where the H.245 control function is in the MG. This package shall not be used for H.324 communication where the H.245 control is in the MGC. |

## 5.1	Properties

### 5.1.1	H.245 channel state

|  | Property ID: | cs (0x0001) |
| --- | --- | --- |
|  | Type: | Boolean |
|  | Possible values: | TRUE (0x0001): The H.245 channel is open FALSE (0x0000): Close the H.245 channel |
|  | Default: | TRUE |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |
|  | Description: | This property specifies whether the H.245 termination is open or closed. |

### 5.1.2	Terminal Type

|  | Property ID: | termtype (0x0002) |
| --- | --- | --- |
|  | Type: | Enumeration |
|  | Possible values: | H.324M (0x0001): H.245 termination is associated with an H.324M terminal H.323 (0x0002): H.245 termination is associated with an H.323 terminal/endpoint |
|  | Default: | Provisioned |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |
|  | Description: | This property defines the type of terminal that is associated with this termination. In this version of the package only two terminal types are defined, but the possible values may be extended in future versions of the package. |

## 5.2	Events

### 5.2.1	H.245 Message

|  | Event Name: |  | Incoming H.245 Message |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Event ID: |  | h245msg (0x0001) |  |  |
|  | Description: |  | This event occurs when the MG detects an incoming H.245 message on the termination realizing this package. |  |  |
|  | EventsDescriptor Parameters: |  |  |  |  |
|  | Parameter Name: |  | Type of H.245 Message to be detected |  |  |
|  | Parameter ID: |  | h245mt (0x0001) |  |  |
|  | Type: |  | Sub-list |  |  |
|  | Possible values: |  |  |  |  |
|  |  | Value for binary encoding; Value for text encoding; Description. 0xffff; "none"; No H.245 Messages. 0x0001; "all"; All H.245 Messages. 0x0002; "Req"; All H.245 Request Messages. 0x0003; "Res"; All H.245 Response Messages. 0x0004; "Com"; All H.245 Command Messages. 0x0005; "Ind"; All H.245 Indication Messages. 0x0006; "NSreq"; Non-Standard Request. 0x0007; "MSD"; Master Slave Determination. 0x0008; "TCS"; Terminal Capability Set Request. 0x0009; "OLC"; Open Logical Channel Request. 0x000a; "CLC"; Close Logical Channel Request. 0x000b; "RCC"; Request Channel Close Request. 0x000c; "MES"; Multiplex Entry Send Request. 0x000d; "RME"; Request Multiplex Entry Request. 0x000e; "RM"; Request Mode Request. 0x000f; "RTD"; Roundtrip Delay Request. 0x0010; "ML"; Maintenance Loop Request. 0x0011; "CM"; Communication Mode Request. 0x0012; "CR"; Conference Request. 0x0013; "MR"; Multilink Request. 0x0014; "LCR"; Logical Channel Rate Request. 0x0015; "NSres"; Non-Standard Response. 0x0016; "MSDack"; Master Slave Determination Ack. 0x0017; "MSDrej"; Master Slave Determination Reject. 0x0018; "TCSack"; Terminal Capability Set Ack. 0x0019; "TCSrej"; Terminal Capability Set Reject. 0x001a; "OLCack"; Open Logical Channel Ack. 0x001b; "OLCrej"; Open Logical Channel Reject. 0x001c; "CLCack"; Close Logical Channel Ack. 0x001d; "RCCack"; Request Channel Close Ack. 0x001e; "RCCrej"; Request Channel Close Reject. 0x001f; "MESack"; Multiplex Entry Send Ack. 0x0020; "MESrej"; Multiplex Entry Send Reject. 0x0021; "RMEack"; Request Multiplex Entry Ack. 0x0022; "RMErej"; Request Multiplex Entry Reject. 0x0023; "RMack"; Request Mode Ack. 0x0024; "RMrej"; Request Mode Reject. 0x0025; "RTDres"; Roundtrip Delay Response. 0x0026; "MLack"; Maintenance Loop Ack. 0x0027; "MLrej"; Maintenance Loop Reject. 0x0028; "CMres"; Communication Mode Response. 0x0029; "Cres"; Conference Response. 0x002a; "MLres"; Multilink Response. 0x002b; "LCRack"; Logical Channel Rate Ack. 0x002c; "LCRrej"; Logical Channel Rate Reject. 0x002d; "NScom"; Non-Standard Command. 0x002e; "MLO"; Maintenance Loop Off Command. 0x002f; "STCS"; Send Terminal Capability Set Command. 0x0030; "ENC"; Encryption Command. 0x0031; "FC"; Flow Control Command. 0x0032; "ES"; End Session Command. 0x0033; "MC"; Miscellaneous Command. 0x0034; "CMcom"; Communication Mode Command. 0x0035; "Ccom"; Conference Command. 0x0036; "h223MR"; H223 Multiplex Reconfiguration Command. 0x0037; "NAVcom"; New ATM VC Command. 0x0038; "MMRcom"; Mobile Multilink Reconfiguration Command. 0x0039; "NSind"; Non-Standard Indication. 0x003a; "FNU"; Function Not Understood. 0x003b; "MSDrel"; Master Slave Determination Release. 0x003c; "TCSrel"; Terminal Capability Set Release. 0x003d; "OLCcon"; Open Logical Channel Confirm. 0x003e; "RCCrel"; Request Channel Close Release. 0x003f; "MESrel"; Multiplex Entry Send Release. 0x0040; "RMErel"; Request Multiplex Entry Release. 0x0041; "RMrel"; Request Mode Release. 0x0042; "MI"; Miscellaneous Indication. 0x0043; "JI"; Jitter Indication. 0x0044; "h223SI"; H223 Skew Indication. 0x0045; "NAVind"; New ATM VC Indication. 0x0046; "UII"; User Input Indication. 0x0047; "H2250MSI"; H2250 Maximum Skew Indication. 0x0048; "MCL"; MC Location Indication. 0x0049; "CI"; Conference Indication. 0x004a; "VI"; Vendor Identification. 0x004b; "FNS"; Function Not Supported. 0x004c; "MLI"; Multilink Indication. 0x004d; "LCRrel"; Logical Channel Rate Release. 0x004e; "FCind"; Flow Control Indication. 0x004f; "MMRind"; Mobile Multilink Reconfiguration Indication. |
|  | Default: |  | ES |  |  |
|  | Description: |  | This parameter defines the type of message(s) that the MGC wants the MG to detect. The MGC may specify "all" if it wants the MG to detect and notify for all the H.245 messages received on the H.245 termination. MGC may specify "Req", "Res", "Com" or "Ind" if it wants the MG to detect all the H.245 Request messages, all the H.245 Response messages, all the H.245 Command messages or all the H.245 Indication messages received on the H.245 termination. Alternatively the MGC may specify a more specific list of messages for which it wants to be notified by the MG. In principle MGC should not specify individual messages if it uses "all" or "none" in the sub-list. "none" or "all", if present, should be the only element in the sub-list. There may be situations where the MGC wants all the H.245 Requests to be detected along with some other H.245 messages; this is allowed in the sub-list. The MG should ignore all the other elements in the sub-list if "none" is present. If "none" is not present, but "all" is present, the MG should ignore all other elements in the sub-list. The MG should also ignore all elements that specify individual H.245 Request messages, if the element "Req" is present in the sub-list. The same is true with the presence of "Res", "Com" or "Ind", wherein the MG should ignore elements specifying individual H.245 Responses, Commands or Indications. Support for the event notification of commands is optional. Support for the event notification of the EndSession command is recommended in order to ensure smooth call release. |  |  |
|  | ObservedEvents‌Descriptor Parameters: |  |  |  |
|  | Parameter Name: |  | Contents of H.245 message detected |  |  |
|  | Parameter ID: |  | h245mc (0x0002) |  |  |
|  | Type: |  | Octet String |  |  |
|  | Description: |  | Specifies the actual contents of the H.245 message detected by the MG. The octet string is the actual X.691 encoding received by the MG. |  |  |

### 5.2.2	H.245 Channel Closed

|  | Event Name: | H.245 Channel Closed |  |
| --- | --- | --- | --- |
|  | Event ID: | h245ChC (0x0002) |  |
|  | Description: | This event occurs when the MG completes the closing of a H.245 channel. |  |
|  | EventsDescriptor Parameters: |  | None |
|  | ObservedEventsDescriptor Parameters: |  | None |

## 5.3	Signals

None.

## 5.4	Statistics

None.

## 5.5	Procedures

This package may be used on terminations that carry H.245 information, if the MGC wants to process this information on the MG. The MGC may ask the MG for notification on detecting a particular type of H.245 message on the h245 termination.

The MGC shall set the "cs" (Channel State) property to true initially. When the MGC wants MG to close the H.245 channel it shall modify the value of the "cs" property to "FALSE". The MG shall follow the procedures defined in Annex F/H.246 to close the H.245 channel.

# 6	H.323 Bearer Control Package

|  | Package Name: | H.323 Bearer Control |
| --- | --- | --- |
|  | Package ID: | h323bc (0x002b) |
|  | Version: | 1 |
|  | Extends: | None |
|  | Description: | This package specifies an extension to the h245 Package to support H.323 fast start and the H.323 use of H.245 tunnelling. This package shall be implemented only if the MG supports H.323 communication where the H.245 control function is in the MG. This package shall not be used for H.323 communication where the H.245 control is in the MGC. |

## 6.1	Properties

### 6.1.1	Fast Connect

|  | Property ID: | fastconnect (0x0001) |
| --- | --- | --- |
|  | Type: | Enumeration |
|  | Possible values: | noFastStart (0x0001): MGC using separate H.245 channel or using H.245 tunnelling fastStart (0x0002): Only fastStart procedures are being used, no parallel H.245control parallelH245 (0x0003) Parallel H.245 control is supported |
|  | Default: | noFastStart |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | This property specifies if the fast connect procedure is used. If the MGC rejects or does not initiate the fast connect procedure, this field shall contain "noFastStart". This property indicates to the MG that the MGC has decided to use H.245 tunnelling or a separate channel for H.245 to the H.323 endpoint. If the MGC accepts or initiates a request for the fast connect procedure, this field shall be set to "fastStart". If however the MGC also decides to support the parallel H.245 procedures along with fast connect, this field shall be set to "parallelH245". |

### 6.1.2	H.245 Message Encapsulation

|  | Property ID: | h245encapstatus (0x0002) |
| --- | --- | --- |
|  | Type: | Boolean |
|  | Possible values: | FALSE (0x0000): H.245 message encapsulation is disabled TRUE (0x0001): H.245 message encapsulation is enabled |
|  | Default: | FALSE |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | This property specifies whether H.245 message encapsulation is enabled or not enabled on the connection to the H.323 endpoint. Practically this property reflects the value of h245Tunneling element in the H.225.0 messages. |

## 6.2	Events

|  | Event Name: | Switch to separate H.245 channel |
| --- | --- | --- |
|  | Event ID: | sepH245 (0x0001) |
|  | Description: | The MG shall notify the MGC with this event when the MG requires the opening of a separate H.245 channel to the H.323 endpoint. This typically happens in a case where the call was set-up using fastStart procedures and the H.245 message tunnelling was disabled. The MGC shall ignore this event if a separate H.245 channel is already open to the H.323 endpoint. |

## 6.3	Signals

None.

## 6.4	Statistics

None.

## 6.5	Procedures

This package supports various ways to start H.245 procedures as specified in 8.2/H.323. The termination that realizes this package shall be made if and only if the processing of H.245 messages from the H.323 side of the gateway is to be performed in the MG. In the following description, it is premised that the MGC requests the creation of the termination that terminates H.245 control.

The MGC that decides to utilize H.245 tunnelling or initiates a separate H.245 channel with the H.323 side, shall generate the termination with the 'fastconnect' property set to the value "noFastStart".

The MGC that decides to use the fast connect procedure shall request the creation of a termination that has the value of the 'fastconnect' property set to other than "noFastStart" according to the procedure to be taken.

If the MGC decides to use H.245 tunnelling or fast connect procedures (with or without parallelH245), H.245 messages shall terminate at the MGC and the MGC shall be responsible for redirecting H.245 messages to the H323bc termination at the MG. The MGC can implement this H.245 message redirection by using one of its own addresses as the remote address of the h323bc termination when this is created.

The MGC shall indicate a switch to a separate H.245 channel by changing the value of the fastconnect property from "fastStart" or "parallelH245" to "noFastStart", and the value of h245encapstatus property to FALSE (using a MODIFY command). The MGC should not MODIFY the value of the fastconnect property to "fastStart" or "parallelH245" from "noFastStart" if the MG has already received a message on that termination. In the event of a switch from fastStart (or parallel H.245) procedures to a separate H.245 channel the MGC may need to subtract and request the creation of a new H.245 termination. This termination should be a direct H.245 channel between the MG and the remote H.323 endpoint.

NOTE 1 – A state where value h245encapstatus property is "FALSE", and the value of the fastconnect property is other than "noFastStart", indicates that a switch to a separate H.245 channel would be required if any further H.245 messages are generated.

NOTE 2 – The intent of this package is that it be used by a MG supporting H.323 calls when a H.245 termination is needed on the MG. As an optional capability, the MG may also use this package (and the other procedures of this Recommendation) in support of H.323 calls that employ Fast Connect or H.245 tunnelling. The value for the inherited property "termtype" shall always be H.323.

# 7	H.245 Command Package

|  | Package Name: | H.245 Command |
| --- | --- | --- |
|  | Package ID: | h245com (0x002d) |
|  | Version: | 1 |
|  | Extends: | None |
|  | Description: | This package defines properties that may be used to indicate that the MGC has sent or received an H.245 command message and the MG should take appropriate action. |

## 7.1	Properties

### 7.1.1	Incoming MiscellaneousCommand

|  | Property Name: | Incoming H.245 MiscellaneousCommand |
| --- | --- | --- |
|  | Property ID: | misc_in (0x0001) |
|  | Type: | Octet String |
|  | Possible values: | This property indicates the value of the H.245 MiscellaneousCommand received by the MGC. The MiscellaneousCommand structure is encoded by applying PER. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | MG shall take appropriate action as described in B.13.5/H.245. |

### 7.1.2	Outgoing Miscellaneous Command

|  | Property Name: | Outgoing H.245 MiscellaneousCommand |
| --- | --- | --- |
|  | Property ID: | misc_out (0x0002) |
|  | Type: | Octet String |
|  | Possible values: | This property indicates the value of the H.245 MiscellaneousCommand sent by the MGC. The MiscellaneousCommand structure is encoded by applying PER. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | MG shall take appropriate action as described in B.13.5/H.245. |

### 7.1.3	Incoming H223MultiplexReconfiguration Command

|  | Property Name: | Incoming H.223MultiplexReconfiguration Command |
| --- | --- | --- |
|  | Property ID: | h223mr_in (0x0003) |
|  | Type: | Octet String |
|  | Possible values: | This property indicates the value of the H223MultiplexReconfiguration command received by the MGC. The H223MultiplexReconfiguration structure is encoded by applying PER. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | A MG which supports the H.223 level change procedure shall start the dynamic H.223 level change procedure as defined in C.7/H.324. |

### 7.1.4	Outgoing H223MultiplexReconfiguration Command

|  | Property Name: | Outgoing H.223MultiplexReconfiguration Command |
| --- | --- | --- |
|  | Property ID: | h223mr_out (0x0004) |
|  | Type: | Octet String |
|  | Possible values: | This property indicates the value of the H223MultiplexReconfiguration command sent by the MGC. The H223MultiplexReconfiguration structure is encoded by applying PER. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | The MG which supports H.223 level change procedure shall start the dynamic H.223 level change procedure as defined in C.7/H.324. |

## 7.2	Events

None.

## 7.3	Signals

None.

## 7.4	Statistics

None.

## 7.5	Procedures

The H.245 Command package defines the properties that may be used by the MGC to pass on the H.245 commands that it sends or receives to/from the remote terminal. The presence of the property in the LocalDescriptor or the RemoteDescriptor indicates whether the command was sent or received by the MGC (see the descriptions with individual properties for more details).

# 8	H.245 Indication package

|  | Package Name: | H.245 Indication |
| --- | --- | --- |
|  | Package ID: | h245ind (0x002e) |
|  | Version: | 1 |
|  | Extends: | None |
|  | Description: | This package defines properties that may be used to indicate that the MGC has sent or received an H.245 indication message and the MG should take appropriate action. |

## 8.1	Properties

### 8.1.1	Incoming MiscellaneousIndication

|  | Property Name: | Incoming H.245 MiscellaneousIndication |
| --- | --- | --- |
|  | Property ID: | misc_in (0x0001) |
|  | Type: | Octet String |
|  | Possible values: | This property indicates the value of the H.245 MiscellaneousIndication received by the MGC. The MiscellaneousIndication structure is encoded by applying PER. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | The MG shall take appropriate action as described in B.14.2/H.245. |

### 8.1.2	Outgoing MiscellaneousIndication

|  | Property Name: | Outgoing H.245 MiscellaneousIndication |
| --- | --- | --- |
|  | Property ID: | misc_out (0x0002) |
|  | Type: | Octet String |
|  | Possible values: | This property indicates the value of the H.245 MiscellaneousIndication sent by the MGC. The MiscellaneousIndication structure is encoded by applying PER. |
|  | Default: | Provisioned |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |
|  | Description: | The MG shall take appropriate action as described in B.14.2/H.245. |

## 8.2	Events

None.

## 8.3	Signals

None.

## 8.4	Statistics

None.

## 8.5	Procedures

This package defines the properties that may be used by the MGC to pass on the H.245 indications that it sends or receives to/from the remote terminal. The presence of the property in the LocalDescriptor or the RemoteDescriptor indicates whether the command was sent or received by the MGC (see the descriptions with individual properties for more details).

# 9	Call flows

This clause describes possible configurations of decomposed gateway and its communication procedures.

## 9.1	H.323-Annex C/H.324 interworking with H.245 in MG

### 9.1.1	Calls with origination by the Annex C/H.324 side

The MG which supports interworking between an H.323 and Annex C/H.324 endpoints may support H.245 signalling, H.245 messages translation as specified in Annex F/H.246, and resource control (for example, audio/video streams, transcoding units and so on) without direct commands from the MGC. The MG may support event notification to the MGC such as the result of opening/closing audio/video logical channels. If the MGC decides to allow the MG to control resource for an H.323-Annex C/H.324 call and uses a separate H.245 channel on the H.323 side, the decomposed gateway configuration looks like as shown in Figure 2. In this case, the MG shall manage audio and video streams without commands from the MGC.

Figure 2/H.248.12  Resource control at the MG (without fastStart/H.245 tunnelling)

The following steps are involved in a call originated by the H.324M side.

1)	MGC detects an incoming call from a H.324M terminal.

2)	MGC shall establish a call using H.225.0 with the H.323 side without using fastStart or H.245 message encapsulation.

3)	MGC shall create a TDM termination, specify an appropriate value for h324/Muxlv and set h324/demux property to TRUE. H.324/cmod shall be set to "H324M" (0x0002).

4)	MGC shall create a termination that realizes h245 package and initialize properties to appropriate values. The h245/termtype property shall be set to "H324M". MGC shall add this termination as the first in the MuxDescriptor of the h324 termination created in step 2).

5)	MGC shall create a termination that realizes h323bc package and set the fastconnect property to "noFastStart", and h245encapstatus property to FALSE. The MGC assigns appropriate addressing information in both local and remote descriptors according to the values exchanged in the H.225.0 channel.

6)	MGC shall associate the H.245 termination created in step 4) and h323bc termination created in step 5) in a context.

7)	MG shall perform H.245 mapping between the two terminations as defined in Annex F/H.246.

If the MGC decides to use H.245 tunnelling but no fastStart, it shall set the h245encapstatus to TRUE, and specify the transport address (which may be an IPv4 address) such that a channel is formed between the MG and the MGC. The MGC shall then relay the messages received in the h245Control element of the H.225.0 messages to the MG on this channel. The MG can safely treat them as messages originating directly at the remote H.323 endpoint. MGC shall encapsulate any H.245 messages received on this channel from the MG in the h245Control field of the H.225.0 messages for transport to the H.323 endpoint.

### 9.1.2	Calls with origination by the H.323 side

Described in this clause are the four different methods of H.323 call setup: conventional H.323 call setup using a separate connection for the H.245 channel, call setup with H.225.0 tunnelling of H.245 messages, Fast Connect, and Fast Connect with the use of H.245 tunnelling (i.e., parallel H.245).

The following steps are involved for calls originating from the H.323 side if no H.245 message encapsulation or fast connect procedures are used (i.e., conventional H.323 call setup):

1)	MGC detects an incoming call from H.323 endpoint.

2)	MGC shall establish a call to the H.324M terminal.

3)	MGC shall create an h324 termination with MediaTx property as TDM Circuit, and specify the appropriate Multiplex level, and Demultiplex property set to TRUE. H.324/cmod shall be set to "H324M" (0x0002).

4)	MGC shall create a termination that realizes the h245 package and initialize properties to appropriate values. The h245/termtype property shall be set to "H324M". MGC shall add this termination as the first in the MuxDescriptor of the h324 termination created in step 3) above.

5)	MGC shall create a termination that realizes h323bc package and set the fastconnect property to "noFastStart", and h245encapstatus property to FALSE. The MGC assigns appropriate addressing information in both local and remote descriptors according to the values exchanged in the H.225.0 channel.

6)	MGC shall associate the H.245 termination created in step 4) and h323bc termination created in step 5) in a context.

7)	MG shall perform H.245 mapping between the two terminations as defined in Annex F/H.246.

8)	The MG shall be responsible for the creation of any logical channels that are negotiated via H.245 channel. Each of these new logical channels shall be a locally controlled media stream.

9)	When an EndSession command is encountered, the MG shall follow the procedures of Annex F/H.246 and close any logical channels that are open. MG shall notify the MGC when the H.245 logical channel is closed from its side.

If the MGC decides to use fastStart or parallel H.245, it shall do the following in step 5).

If the MGC decides to use H.245 tunnelling but no fastStart, it shall set the h245encapstatus to TRUE, and specify the transport address (which may be an IPv4 address) such that a channel is formed between the MG and the MGC. The MGC shall then relay the messages received in the h245Control element of the H.225.0 messages to the MG on this channel (see Figure 3). The MG can safely treat them as messages originating directly at the remote H.323 endpoint. The MGC shall encapsulate any H.245 messages received on this channel from the MG in the h245Control field of the H.225.0 messages for transport to the H.323 endpoint.

Figure 3/H.248.12  Resource control at the MG (with fastStart/H.245 tunnelling)

If the MGC decides to use fastStart or parallel H.245, it shall do the following in step 5).

It shall set the fastconnect property to "fastStart" or parallelH245 accordingly. h245encapstatus property shall be set to reflect the h245Tunneling flag in the H.225.0 messages exchanged with the H.323 endpoint. MGC shall specify the transport address (which may be an IPv4 address) such that a channel is formed between the MG and the MGC. The MGC shall then relay the messages received in the fastStart or parallelH245 element of the H.225.0 messages to the MG on this channel. The MG can safely treat them as messages originating directly at the remote H.323 endpoint. The MGC shall encapsulate any Logical Channel messages received on this channel from the MG in the fastStart field of the H.225.0 messages for transport to the H.323 endpoint. Capability Exchange and Master Slave determination messages shall be sent in the h245Control/parallelH245Control field.

The MGC shall set the fastconnect property to "noFastStart" as soon as the fast connect procedures are completed, regardless of whether these procedures were successful or not.

## 9.2	H.323-Annex C/H.324 interworking with H.245 in MGC

Figure 4 shows the configuration of the decomposed gateway where the H.245 control is in the MGC. In this configuration, the MGC shall know the H.324 related capabilities of the MG before starting the H.245 capability negotiation with the remote endpoint. The retrieval of the capability set of the MG is out of the scope of this Recommendation.

Figure 4/H.248.12  Resource control concentrated at the MGC

The following is an example call flow for the call originated by the Annex C/H.324 terminal.

1)	MGC detects an incoming call from the remote endpoint. The MGC may identify the incoming call type as H.324 call using the information contained in the call signalling messages.

2)	MGC establishes a call with the remote H.324 endpoint.

3)	MGC creates an h324 termination with MediaTx property as TDM Circuit, and specifies the appropriate Multiplex level, and Demultiplex property set to TRUE.

4)	After the MG establishes the digital communication with the remote endpoint, the MG, which wants to start Annex C/H.324 communication, shall initiate H.223 level setup procedure according to the Multiplex level indicated in the previous step. After the level setup, the H.245 control channel shall be opened according to the procedure given in C.8/H.324.

5)	MG and MGC shall transport the H.245 messages between each other. One way is a SCTP connection between the MGC and the MG as defined in [b-IETF RFC 4960] to reliably transport the H.245 message, and associate the SCTP connection to the h324 termination. The exact nature of interfaces with SCTP for transport of H.245 messages between the MG and the MGC is out of the scope of this Recommendation. As a result the MG will transmit any h245 messages detected on the h324 termination to the MGC via the SCTP connection. Also the MG will transmit any messages received via the SCTP connection to the H.324M terminal.

Another method is to use the h245transport/h245msgout to send messages over the H.248 connection and to use h245transport/h245msgin event to receive messages over the H.248 connection.

6)	MGC shall map the H.245 control information between the H.324 and the H.323 endpoints as defined in Annex F/H.246.

7)	MGC shall create and associate appropriate media terminations on the MG to realize the creation of Logical channel.

8)	When the end session procedure is initiated by either the remote endpoint or the MGC, the MGC shall transmit the H.245 EndSessionCommand message, and then stop all H.245 message transmissions. After the completion of the end session procedure, the MGC shall send Subtract message to the MG to disconnect the H.324 termination from its Context.

## 9.3	Tunnelling of Annex C/H.324 bitstream

The scenario described in this clause enables the H.324 bitstream transmission over an IP-based network between the gateways. The configuration of the decomposed gateway that supports this scenario is shown in Figure 5. The MGC, which decides to create IP tunnelling for H.324 call between MGs, creates a context that contains H.324 terminations on the TDM and IP sides. It is optional for the decomposed gateway to support this function.

Figure 5/H.248.12  Configuration for IP tunnelling of Annex C/H.324 bitstream

The following is the call flow example for this scenario.

1)	If the MGC decides to create IP tunnelling for H.324 upon receipt of the audiovisual call from the H.324 endpoint, the MGC shall request the MG to create a new context and create terminations, which support H.324 package, in the context. On the H.324 side, the 'h324' termination is created with MediaTx property of TDM, 'muxlv' property of appropriate value, and 'demux' property of 'FALSE'. On the IP side, the 'h324' termination is created with MediaTx property of RTP, 'muxlv' property of 'Level0', and 'demux' property of 'FALSE'.

2)	The MG shall perform the mapping of H.324 bitstream (i.e., H.223 multiplexed bitstream) from TDM to RTP, and vice versa, according to procedures stated in F.10/H.225.0.

3)	Upon receipt a message to release the call (e.g., Q.931 Release Complete message), the MGC shall send Subtract message/s to the MG to disconnect the corresponding H.324 termination/s from the Context.

Annex A

Extended H.324, H.245 command and H.245 indication packages

(This annex forms an integral part of this Recommendation)

## A.1	Scope

This annex defines properties for H.324 and H.323 interworking, which are needed together with the properties in the H.324 package, H.245 command package, H.245 indication package and H.245 transport package, for interworking with H.324 terminals having different H.324 capabilities, when the interworking function is handled by the MGC.

## A.2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2002), Gateway control protocol: Version 2

[ITU-T H.248.15]	Recommendation ITU-T H.248.15 (2002), Gateway control protocol: SDP H.248 package attribute

[IETF RFC 2327]	IETF RFC 2327 (1998), SDP: Session Description Protocol

## A.3	Definitions

N/A.

## A.4	Abbreviations and acronyms

This annex uses the following abbreviations and acronyms:

| LSB | Least Significant Bit |
| --- | --- |
| MGC | Media Gateway Controller |
| MSB | Most Significant Bit |
| MUX | MUltipleX |
| PDU | Protocol Data Unit |
| PER | Packed Encoding Rules |
| TCS | Terminal Capability Set |

## A.5	Extended H.324 package

|  | Package ID: | h324ext (0x0063) |
| --- | --- | --- |
|  | Description: | The extended H.324 package extends the H.324 package defined in the main body of this Recommendation. The package defines new properties for H.324 and H.323 interworking, which are needed when the interworking function is handled by the MGC. |
|  | Version: | 1 |
|  | Designed to be extended only: | No |
|  | Extends: | h324 (0x002d) version 1 |

### A.5.1	Properties

#### A.5.1.1	Maximum H.223 MUX PDU size

|  | Property ID: | maxH223MUXPDUSize (0x0007) |
| --- | --- | --- |
|  | Description: | This property indicates the maximum size for H.223 MUX PDU in number of octets. |
|  | Type: | Integer |
|  | Characteristics: | Read/Write |
|  | Defined in: | LocalControl |
|  | Possible values: | Any positive integer |
|  | Default: | Provisioned |

#### A.5.1.2	Local H.223 capability

|  | Property ID: | h223capl (0x0008) |
| --- | --- | --- |
|  | Description: | This property indicates the local endpoint's capabilities specific to the H.223 multiplex capability. |
|  | Type: | Octet String |
|  | Characteristics: | Read/Write |
|  | Defined in: | LocalControl |
|  | Possible values: | This property indicates the H.245 H223Capability structure encoded by applying the PER specified in [ITU-T X.691]. |
|  | Default: | Provisioned |
|  | Binary encoding: |  |

Structure of the octet string

| 8 |  |  |  | 7 |  | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSB |  |  |  |  |  |  |  |  |  |  | LSB | Octet 1 |  |
|  | · · · |  |  |  |  |  |  |  |  |  |  | · · · |  |  |
| MSB |  |  |  |  |  |  |  |  |  |  | LSB | Octet n |  |
| NOTE Octet 1 contains the most significant octet of data. |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | Text encoding: | Shall be encoded using the mechanism defined in B.3/[ITU-T H.248.1]. |
| --- | --- | --- |

#### A.5.1.3	H.223 logical channel parameters

|  | Property ID: | h223lcparm (0x0009) |
| --- | --- | --- |
|  | Description: | This property is used to indicate parameters specific to using H.223. It indicates both adaptation layer and multiplex layer properties used for the logical channel. |
|  | Type: | Octet String |
|  | Characteristics: | Read/Write |
|  | Defined in: | Local and remote descriptors |
|  | Possible values: | This property indicates the H.245 H223LogicalChannelsParameters structure encoded by applying the PER specified in [ITU-T X.691]. Value encoded as per clause A.5.1.2. For text encoding the mechanism defined in [ITU-T H.248.15] is used. |
|  | Default: | Provisioned |

### A.5.2	Events

None.

### A.5.3	Signals

None.

### A.5.4	Statistics

None.

### A.5.5	Procedures

These procedures are related to the scenario in which MGC executes H.245 control itself as shown in Figures A.1 to A.3.

Figure A.1/H.248.12  Sequence diagram terminal capability
exchange at H.324 and H.323 interworking

The MGC will participate in H.245 terminal capability exchange procedures after it has created an H.324 termination in the MG as per Figure A.1.

In order to know the H.223 multiplex capabilities to be provided to H.324 terminal, the MGC audits the MG for the "Local H.223 capability" and "Maximum H.223 MUX PDU size".

Figure A.2/H.248.12  Sequence diagram for opening the logical channel

When logical channels are opened between terminals by the H.245 protocol, the MGC adds the streams between the multiplex and RTP terminations in the MG as per Figure A.2. The streams are described by the usual manner as described in 7.1/[ITU-T H.248.1]. The H.223 logical channel parameters define the H.223 multiplexing properties used for the stream towards the H.324 terminal, e.g., the adaptation layer to be used. When textual encoding is used, the property is coded within SDP by using [ITU-T H.248.15].

Figure A.3/H.248.12  Sequence diagram for maximum PDU specification

At any time when a remote H.324 terminal commands to restrict the size of the H.223 MUX-PDUs, the MGC shall order the MG to restrict the H.223 MUX PDU size that it transmits to a maximum of the specified number of octets as per Figure A.3.

## A.6	Extended H.245 command package

|  | Package ID: | h245comext (0x0064) |
| --- | --- | --- |
|  | Description: | The extended H.245 command package extends the H.245 command package defined in the main body of this Recommendation. The package defines new properties for H.324 and H.323 interworking, which are needed when the interworking function is handled by the MGC. |
|  | Version: | 1 |
|  | Designed to be extended only: | No |
|  | Extends: | h245com (0x002e) version 1 |

### A.6.1	Properties

#### A.6.1.1	H.245 version

|  | Property ID: | h245version (0x0005) |
| --- | --- | --- |
|  | Description: | This property indicates the highest version of the H.245 protocol used. |
|  | Type: | Integer |
|  | Characteristics: | Read Only |
|  | Defined in: | LocalControl descriptor |
|  | Possible values: | Any positive integer as per the version from the H.245 protocolIdentifier structure. |
|  | Default: | Provisioned |

#### A.6.1.2	Flow control command

|  | PropertyID: | flowControlCom (0x0006) |
| --- | --- | --- |
|  | Description: | This property indicates the FlowControl to be used. |
|  | Type: | Octet String |
|  | Characteristics: | Read/Write |
|  | Defined in: | LocalControl descriptor |
|  | Possible values: | This property indicates the H.245 FlowControlCommand structure encoded by applying the PER specified in [ITU-T X.691]. Value encoded as per clause A.5.1.2. |
|  | Default: | Provisioned |

### A.6.2	Events

None.

### A.6.3	Signals

None.

### A.6.4	Statistics

None.

### A.6.5	Procedures

These procedures are related to the scenario in which MGC executes H.245 control itself as shown in Figure A.4 below.

The MGC may audit the H.245 version, which the MG supports, after a service change.

Figure A.4/H.248.12  Sequence diagram for flow control modification

When a remote H.324 or H.323 terminal wants to restrict the bit rate of a channel, or whole multiplex, and the MGC accepts the request, the MGC orders the MG to restrict the bit rate of the stream(s) as per Figure A.4.

## A.7	Extended H.245 indication package

|  | Package ID: | h245indext (0x0065) |
| --- | --- | --- |
|  | Description: | The extended H.245 indication package extends the H.245 indication package defined in the main body of this Recommendation. The package defines new properties for H.324 and H.323 interworking, which are needed when the interworking function is handled by the MGC. |
|  | Version: | 2 |
|  | Designed to be extended only: | No |
|  | Extends: | h245ind (0x002e) version 1 |

### A.7.1	Properties

#### A.7.1.1	H.223 skew indication

|  | Property ID: | h223SkewInd (0x0003) |
| --- | --- | --- |
|  | Description: | This property indicates the average skew between two transmitted media streams represented by two logical channels (i.e. the amount the video signal trails the audio signal). |
|  | Type: | Octet String |
|  | Characteristics: | Read/Write |
|  | Defined in: | LocalControl |
|  | Possible values: | This property indicates the H.245 H223SkewIndication structure encoded by applying the PER specified in [ITU-T X.691]. Value encoded as per clause A.5.1.2. NOTE - The above structure contains two logical channel numbers. To indicate which media stream/logical channel the skew is measured against the "Stream H.223 skew measured against" property should be used. If not used a default of StreamID of "1" is assumed. |
|  | Default: | Provisioned |

#### A.7.1.2	Jitter indication

|  | Property ID: | jitterIndication (0x0004) |
| --- | --- | --- |
|  | Description: | This property indicates the jitter. |
|  | Type: | Octet String |
|  | Characteristics: | Read/Write |
|  | Defined in: | LocalControl descriptor |
|  | Possible values: | This property indicates the H.245 JitterIndication structure encoded by applying the PER specified in [ITU-T X.691]. Value encoded as per clause A.5.1.2. |
|  | Default: | Provisioned |

#### A.7.1.3	Stream H.223 skew measured against

|  | Property ID: | skewstr (0x0005) |
| --- | --- | --- |
|  | Description: | This property indicates the media stream/logical channel the "H.223 skew indication" is measured against. |
|  | Type: | Integer |
|  | Characteristics: | Read/Write |
|  | Defined in: | LocalControl |
|  | Possible values: | 0-65535 |
|  | Default: | 1 |

### A.7.2	Events

### A.7.2.1	Skew Update

|  | Event Name: | Skew Update |
| --- | --- | --- |
|  | Event ID: | skewup (0x0001) |
|  | Description: | This event allows the MG to indicate to the MGC when the synchronization time (skew) between two received streams has changed. |

#### A.7.2.1.1	EventsDescriptor parameters

##### A.7.2.1.1.1	Threshold

|  | Parameter Name: | Threshold |
| --- | --- | --- |
|  | Parameter ID: | thres (0x0001) |
|  | Description: | This parameter indicates the threshold for the generation of a Skew Update event. If the MG detects that the skew between the stream defined by the "Skew Stream" property and the stream that the Event is set on differs by more than the threshold value since the last report then the "Skew Update" ObservedEvent is reported. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 1 - 4095, in milliseconds |
|  | Default: | 50ms |

#### A.7.2.1.2	ObservedEventsDescriptor parameters

##### A.7.2.1.2.1	H223 Skew Value

|  | Parameter Name: | Skew Value |
| --- | --- | --- |
|  | Parameter ID: | skewval (0x0001) |
|  | Description: | This parameter indicates the difference in time synchronization between the Stream represented by the ObservedEvent and the Stream indicated by the "Stream H.223 skew measured against" (skewstr) property. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 1 - 4095 in milliseconds |
|  | Default: | None. |

### A.7.3	Signals

None.

### A.7.4	Statistics

None.

### A.7.5	Procedures

As Figure A.2 above shows, when establishing a multiplex Termination to support H.324 the MGC may establish individual H.248 streams for each of the H.245 logical channels used in the H.223 multiplex. The MGC can then assign a logical channel number (i.e. via the "LCN" property clause C.12/[ITU-T H.248.1]) and other related H.223 logical channel properties to a stream. The MGC and MG are then both unambiguously aware of the mapping between the "Logical Channel Number" and the "H.248 StreamID".

These procedures are related to the scenario in which MGC executes H.245 control itself, as shown in Figure A.5 below.

Figure A.5/H.248.12  Sequence diagram for skew and jitter indication

When a remote H.324 terminal indicates the average amount of skew between two logical channels, the MGC forwards the skew to the MG in order for the MG to take the skew into consideration at synchronization of the channels, or at packetization of the channels, into RTP as per Figure A.5. This is based on an allocation of an audio stream to StreamID = 1 and that the stream where the h223SkewIndication property is set represents a video stream. If the MGC requires another mapping the "Stream H.223 skew measured against" (skewstr) is used to indicate the media stream/logical channel the skew is measured against.

Where the MGC/MG measures the skew and originates a H.245 H223SkewIndication message the MGC may set the "SkewUpdate" (skewup) event. Upon setting of the event, once the skew can be determined the MG shall return an ObservedEvent with the initial skew value. Subsequent reporting of the event will occur whenever the value of the "Threshold" (thres) event parameter is exceeded. On reception of the ObservedEvent the MGC may send a H.245 H223SkewIndication based on the skew value received. The logical channel numbers used may be derived from the associated media stream where the event was set and the media stream specified by the skewstr property.

At any time when the remote H.323 terminal indicates the amount of jitter in the received media, the MGC forwards the information to the MG in order for the MG to choose the bit rate and buffer control in video channels as per Figure A.5.

## A.8	H.245 transport package

|  | Package ID: | h245tp (0x00b4) |
| --- | --- | --- |
|  | Description: | This package defines a new signal for sending H.245 messages from a MGC to an MG and a new event for sending H.245 messages from the MG to the MGC which are needed when the interworking function is handled by the MGC. |
|  | Version: | 1 |
|  | Designed to be extended only: | No |
|  | Extends: | None |

### A.8.1	Properties

None.

### A.8.2	Events

#### A.8.2.1	Incoming H.245 message

|  | Event Name: | Incoming H.245 message |
| --- | --- | --- |
|  | Event ID: | h245msgin (0x0001) |
|  | Description: | This event occurs when the MG detects an incoming H.245 message on the termination realizing this package. |

##### A.8.2.1.1	EventsDescriptor Parameters

None.

##### A.8.2.1.2	ObservedEventsDescriptor Parameters

###### A.8.2.1.2.1	Contents of H.245 message

|  | Parameter Name: | Contents of H.245 message |
| --- | --- | --- |
|  | Parameter ID: | h245mc (0x0001) |
|  | Description: | Specifies the actual contents of a H.245 message. |
|  | Type: | Octet String |
|  | Optional: | No |
|  | Possible values: | The octet string is the actual [ITU-T X.691] encoding of the H.245 message. |
|  | Default: | None |

### A.8.3	Signals

#### A.8.3.1	Outgoing H.245 message

|  | Signal Name: | Outgoing H.245 message |
| --- | --- | --- |
|  | Signal ID: | h245msgout (0x0001) |
|  | Description: | Sends a H.245 message. |
|  | Signal Type: | Brief |
|  | Duration: | Provisioned |

##### A.8.3.1.1	Additional parameters

###### A.8.3.1.1.1	Contents of H.245 message

|  | Parameter Name: | Contents of H.245 message |
| --- | --- | --- |
|  | Parameter ID: | h245mc (0x0001) |
|  | Description: | Specifies the actual contents of a H.245 message. |
|  | Type: | Octet String |
|  | Optional: | No |
|  | Possible values: | The octet string is the actual [ITU-T X.691] encoding of the H.245 message. |
|  | Default: | None |

### A.8.4	Statistics

None.

### A.8.5	Procedures

The MGC shall indicate the MG for notification on detecting any H.245 message. When the MG receives a H.245 message from the remote end, it shall notify the contents of the message to the MGC through the event "h245transport/h245msgin". When the MGC sends a H.245 message to the remote end, it indicates the content of the message to the MG through the signal "h245transport/h245msgout". In both cases, the MG shall relay the H.245 message between the MGC and the remote end without modifying it.

Bibliography

[b-IETF RFC 4960]	IETF RFC 4960 (2000), Stream Control Transmission Protocol.

_____________________