| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4181 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Rapporteur Q3/16 |  |  |  |
| Title: | H.248.1v4: Input draft of Living List |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the Rapporteur’s input draft for the H.248.1v4 Living List.

2.	Discussion

It is a copy of TD 634/WP2 from the Geneva, Switzerland meeting 21 November – 02 December 2011 meeting. There are no changes.

3.	Proposal

It is proposed to accept this document as the baseline text for the meeting.

Appendix I

H.248.1 Living List

# I.1	Introduction

This Appendix contains functionality that has been identified to have impacts on H.248.1 that would require a new version of the protocol. Items are listed in this Appendix so that when it is decided that a new version of H.248.1 is required they may be incorporated into the new version.

Item 3 has been added to record the discussion on the package registration process. Like packages it is independent of protocol version.

# I.2	Items

| Item Number | Description | Date Added | Reference |
| --- | --- | --- | --- |
| 1 | Addition for floating type number support | 13 Apr 2006 | See Appendix IV (see also C.230 / D.227 / D.228) |
| 2 | ASN-ABNF differences correction | 1 Sep 2006 | AVD-2908 |
| 3 | Timestamp Notification Flag | 16 Mar 2007 | AVD-3042 |
| 4 | Reserve Value and Group Enhancements | 27 June 2007 | See Appendix II (see also C.228) |
| 5 | H.248.1: Service Change extension parameter ABNF | 27 June 2007 | See Appendix V (see also C.277 Item 1) |
| 6 | H.248.1: Service Unavailability Time | 23 January 2008 | AVD-3329 Option 2 in Appendix II |
| 7 | New Appendix V – H.248 Streams “Examples for valid media format suites” | 23 January 2008 | AVD-3346 |
| 8 | Inconsistency between Audit Value and Request ABNF | 25 June 2009 | AVD-3706 |

Appendix II

Reserve Value and Group Enhancements

| Description: | Clause 7.1.8 was still subject for questions since publication of H.248.1 Version 3. There is apparently still demand for semantic clarifications/enhancement thus there may be a need updates to H.248.1v4. |
| --- | --- |
| Reference: | COM16 C-228 |

[Begin Proposed Correction]

### 7.1.8	Local and Remote Descriptors

Editor’s note: 
Following items should be also considered in H.248.1 V4 text:

- symmetrical vs asymmetrical media formats of H.248 Streams (= bidirectional media flows)

- different terminology: H.248.1 main body “Reserve…” vs Annex B syntax “Reserved…” (Note: Annex A is consistent)

- different terminology for Boolean values: {True, False} vs {On, Off}

[End Proposed Correction]

Appendix III

Floating point number support

| Description: | Floating Point numbers are not supported in H.248.1v3. This type should be added to H.248.1v4. |
| --- | --- |
| Reference: | C.230 / D.227 / D.228 |

[Begin Proposal]

H.248.1

…

### 12.1.2	Properties

Properties defined by the package, specifying:

Property Name: only descriptive

PropertyID: is an identifier.

Description: is a description of the function of the property

Type: One of:

Boolean

String: UTF-8 string

Octet String: A number of octets. See Annex A and B.3 for encoding

Integer: 4 byte signed integer

Double: 8 byte signed integer

Float32: 4 byte floating point number according the single-precision 32-bit IEEE 754 format

Float64: 8 byte floating point number according the double-precision 64-bit IEEE 754 format

Character: Unicode UTF-8 encoding of a single letter. Could be more than one octet.

Enumeration: one of a list of possible unique values.

Sub-list: a list of several values from a list. The type of sub-list shall also be specified. The type shall be chosen from the types specified in this section (with the exception of sub-list). For example, Type: sub-list of enumeration. The encoding of sub-lists is specified in Annexes A and B.2.

Possible values:

A package must specify either a specific set of values or a description of how values are determined. A package must also specify a default value or the default behaviour when the value is omitted from its descriptor. For example, a package may specify that procedures related to the property are suspended when its value is omitted.

Default:

A default value (but not procedures) may be specified as provisionable.

Defined in:

The H.248.1 descriptor in which the property is defined. LocalControl is for stream-dependent properties. TerminationState is for stream-independent properties. ContextAttribute is for properties that affect the context as a whole, i.e., mixing properties. These are expected to be the most common cases, but it is possible for properties to be defined in other descriptors. Context properties must be defined in the ContextAttribute Descriptor.

Characteristics: Read-Only or Read/Write, and (optionally), global:

Indicates whether a property is read-only, or read/write, and if it is global. If global is omitted, the property is not global. If a property is declared as global, the value of the property is shared by all terminations realizing the package.  If a context property is declared as global, the property is shared by all contexts realizing the package.

### 12.1.3	Events

Events defined by the package, specifying:

Event name: only descriptive

EventID: is an identifier

Description: a description of the function of the event

EventsDescriptor Parameters:

Parameters used by the MGC to configure the event, and found in the Events Descriptor. See clause 12.2. If there are no parameters for the Events Descriptor, then “none” shall be specified.

ObservedEventsDescriptor Parameters:

Parameters returned to the MGC in Notify requests and in replies to command requests from the MGC that audit ObservedEvents Descriptor, and found in the ObservedEvents Descriptor. See clause 12.2. If there are no parameters for the ObservedEvents Descriptor, then “none” shall be specified.

### 12.1.4	Signals

Signals defined by the package, specifying:

|  | Signal Name: | Only descriptive |  |
| --- | --- | --- | --- |
|  | SignalID: | SignalID is an identifier and it is used in a Signals Descriptor |  |
|  | Description: | A description of the function of the signal |  |
|  | SignalType: | One of: - OO (OnOff) - TO (TimeOut) - BR (Brief) NOTE SignalType may be defined such that it is dependent on the value of one or more parameters. The package must specify a default signal type. If the default type is TO, the package must specify a default duration which may be provisioned. A default duration is meaningless for BR. |  |
|  | Duration: | in hundredths of seconds |  |
|  | Additional Parameters: |  | see clause 12.2 |

### 12.1.5	Statistics

Statistics defined by the package, specifying:

|  | Statistic name: | Only descriptive |
| --- | --- | --- |
|  | StatisticID: | StatisticID is an identifier and is used in a Statistics Descriptor. |
|  | Description: | A description of the statistic |
|  | Type: | One of: - Boolean - String: UTF-8 string - Octet String: A number of octets. See Annex A and Annex B.3 for encoding - Integer: 4 byte signed integer - Double: 8 byte signed integer - Float32: 4 byte floating point number according the single-precision 32-bit IEEE 754 format - Float64: 8 byte floating point number according the double-precision 64-bit IEEE 754 format - Character: Unicode UTF-8 encoding of a single letter. Could be more than one octet. - Enumeration: One of a list of possible unique values. - Sub-list: A list of several values from a list. The type of sub-list shall also be specified. The type shall be chosen from the types specified in this section (with the exception of sub-list). For example, Type: sub–list of enumeration. The encoding of sub-lists is specified in Annexes A and B.2. |
|  | Possible Values: | A package must indicate the unit of measure, e.g. milliseconds, packets, either here or along with the type above, as well as indicating any restriction on the range. |
|  | Level: | Specify if the statistic can be kept at the termination level, Stream level or either. |

[End Proposal]

[Begin Proposal]

## A.2	ASN.1 syntax specification

This clause contains the ASN.1 specification of the H.248.1 protocol syntax.

NOTE 1  In case a transport mechanism is used that employs application level framing, the definition of Transaction below changes. Refer to the annex or to the Recommendation of the H.248.x Subseries defining the transport mechanism for the definition that applies in that case.

NOTE 2 – This syntax specification does not enforce all restrictions on element inclusions and values. Some additional restrictions are stated in comments and other restrictions appear in the text of this Recommendation. These additional restrictions are part of the protocol even though not enforced by this Recommendation.

NOTE 3 – The ASN.1 module in this annex uses octet string types to encode values for property parameter, signal parameter and event parameter values and statistics. The actual types of these values vary and are specified in Annex C or the relevant package definition.

A value is first BER-encoded based on its type using the table below. The result of this BER-encoding is then encoded as an ASN.1 octet string, "double wrapping" the value. The format specified in Annex C or the package relates to BER encoding according to the following table:

| Type Specified in Package | ASN.1 BER Type |
| --- | --- |
| String | IA5String or UTF8String (Note 4) |
| Integer (4 Octet) | INTEGER |
| Double (8 octet signed int) | INTEGER (Note 3) |
| Character (UTF-8) (Note 1) | IA5String |
| Enumeration | ENUMERATED |
| Boolean | BOOLEAN |
| Unsigned Integer (Note 2) | INTEGER (Note 3) |
| Octet (String) | OCTET STRING |
| Float32 | REAL (Note 5) |
| Float64 | REAL (Note 5) |
| NOTE 1 – Can be more than one byte. |  |
| NOTE 2 – Unsigned integer is referenced in Annex C. |  |
| NOTE 3 – The BER encoding of INTEGER does not imply the use of 4 bytes. |  |
| NOTE 4 – String should be encoded as IA5String when the contents are all ASCII characters, but as UTF8String if it contains any non-ASCII characters. |  |
| NOTE 5 – The BER encoding of REAL does not imply the use of 4 or 8 bytes. |  |

See clause 8.5/X.690, for the definition of the encoding of a real value and Annex C/X.690 for the "illustration of real value encoding". See clause 8.7/X.690, for the definition of the encoding of an octet string value.

…

[End Proposal]

[Begin Proposal]

## B.2	ABNF specification

The protocol syntax is presented in ABNF according to RFC 2234.

NOTE 1 – This syntax specification does not enforce all restrictions on element inclusions and values.  Some additional restrictions are stated in comments and other restrictions appear in the text of this Recommendation. These additional restrictions are part of the protocol even though not enforced by this Recommendation.

NOTE 2 – The syntax is context-dependent. For example, "Add" can be the AddToken or a NAME depending on the context in which it occurs.

Everything in the ABNF and text encoding is case insensitive. This includes TerminationIDs, DigitMapIDs, etc. SDP is case sensitive as per RFC 2327.

; NOTE – The ABNF in this section uses the VALUE construct (or lists of

; VALUE constructs) to encode various package element values (properties,

; signal parameters, etc.).  The types of these values vary and are

; specified in the relevant package definition.  Several such types are

; described in 12.2.

;

; The ABNF specification for VALUE allows a quotedString form or a

; collection of SafeChars.  The encoding of package element values into

; ABNF VALUES is specified below.  If a type's encoding allows characters

; other than SafeChars, the quotedString form must be used for all values

; of that type, even for specific values that consist only of SafeChars.

;

; String:  A string must use the quotedString form of VALUE and can

; contain anything allowable in the quotedString form.

;

; Integer, Double, and Unsigned Integer:  Decimal values can be encoded

; using characters 0-9.  Hexadecimal values must be prefixed with '0x'

; and can use characters 0-9,a-f,A-F.  An octal format is not supported.

; Negative integers start with '-' and must be Decimal.  The SafeChar

; form of VALUE must be used.

;

; Character:  A UTF-8 encoding of a single letter surrounded by double

; quotes.

;

; Enumeration:  An enumeration must use the SafeChar form of VALUE

; and can contain anything allowable in the SafeChar form.

;

; Boolean:  Boolean values are encoded as "on" and "off" and are

; case insensitive.  The SafeChar form of VALUE must be used.

;

; Float32:

;

; Float64:

;

{Editor’s note:

### General layout

The three fields in an IEEE 754 float

Binary floating-point numbers are stored in a sign-magnitude form where the most significant bit is the sign bit, exponent is the biased exponent, and "fraction" is the significand minus the most significant bit.

The text encoded representation needs further definition. Ie. It could be +123.456 or hex encoded 4 or 8 octets. Futhermore an exponential notation such a notation could be as explicit as: +2097*2-103could be used. Contribution is invited.}

;

; Future types:  Any defined types must fit within

; the ABNF specification of VALUE.  Specifically, if a type's encoding

; allows characters other than SafeChars, the quotedString form must

; be used for all values of that type, even for specific values that

; consist only of SafeChars.

;

; Note that there is no way to use the double quote character within

; a value.

;

[End Proposal]

Appendix V

H.248.1: Service Change extension parameter ABNF

| Description: | The ABNF grammar defined for the ServiceChange extension parameter allows the COMMA to be optional when adding subsequent parameters. However the COMMA should always be used because the grammar for parmValue allows for lists. Therefore it is recommended to add a note for H.248.1v3 to advise that the COMMA should always be included. Furthermore the use of “paramValue” is incorrect the correct production is “parmValue”. |
| --- | --- |
| Reference: | C.277 Item 1 |

[Begin Proposal]

7.2.8.1.8 ExtensionParameter

The optional Extension parameter may contain any value whose meaning is mutually understood by

the MG and MGC. The value "X-SC" is reserved for the use of signalling ServiceChange

parameters that have been added as a result of H.248.1 version 3 (and subsequent versions). It is

only used in the initial H.248.1 version 1 coded ServiceChange Command from the MG to MGC

when the ServiceChangeVersion is greater than or equal to 3. The structure of the value is defined

by the following ABNF:

X-SC = NAME EQUAL parmValue * (COMMA NAME EQUAL parmValue)

…

[End Proposal]

_________________