| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4190 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Proposed modification on H.IDscheme |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution contains proposed modification on draft Recommendation H.IDscheme.

# Introduction

At the last meeting (Geneva, 21 November – 2 December 2011), it was agreed to stop developing common text with ISO/IEC JTC 1/SC 31/WG 6 on H.IDscheme and the text format was changed into ITU-T only Recommendation format.

# Discussion

Three Editor’s NOTEs are removed and related comments are reflected in the main body of this draft Recommendation.

One Editor’s NOTE is also removed since we do not need a name for the identification scheme in this draft Recommendation.

# Conclusion

We propose to Consent attached text for H.IDscheme at the next meeting.

CONTENTS

Page

Introduction	1

Discussion	1

Conclusion	1

1	Scope	3

2	References	4

3	Definitions	4

3.1	Terms defined elsewhere	4

3.2	Terms defined in this Recommendation	4

4	Abbreviations and acronyms	5

5	Conventions	5

6	Concept of identification scheme for multimedia information access triggered by tag-based identification	5

6.1	Overview	5

6.2	Uniqueness of ID value	5

7	Identification scheme for multimedia information access triggered by tag-based identification	6

7.1	Structure	6

7.2	High Level Code (HLC)	8

7.3	Top Level Code (TLC)	8

7.4	Class	8

7.5	Second Level Organization Code (SLOC)	8

7.6	Serial Code (SC)	8

Appendix I  Survey on ID schemes	9

I.1	Introduction	9

I.2	Need for generic ID scheme for multimedia information access triggered by tag-based identification	9

I.3	Requirements on identifier for multimedia information access triggered by tag-based identification	9

I.4	Survey on existing ID schemes	11

Draft new Recommendation ITU-T H.IDscheme

Identification scheme for multimedia information access triggered by tag-based identification – Part 1: Identification scheme

AAP Summary

To be provided before Consent.

Summary

This Recommendation defines identification scheme for the multimedia information access triggered by tag-based identification. Multimedia information associated with such an identifier can be retrieved using a resolution process.

Keywords

Identifier, identification scheme

Introduction

The identification scheme defined in this Recommendation provides a new method to access multimedia content without typing its address on a keyboard or inputting the name of objects and/or places of relevant information.  This is a major communication service that uses an identifier in data carriers such as radio frequency identifications (RFIDs), smart cards and barcodes.

The purpose of this Recommendation is to specify a new identification scheme to be used in applications and services where the existing identification schemes and their combinations cannot be used (See appendix I for some examples).

From the standpoint of compatibility and reusability of existing identifier schemes, this Recommendation adopts the approach of multiple identification schemes. In this approach, possibly multiple existing identification schemes together with the proposed new identification scheme can be used for identification where appropriate.  Each identification scheme is identified by object identifier (OID).

Clause 6 introduces the generic concept of multiple identification schemes for the multimedia information access triggered by tag-based identification.

Clause 7 specifies a new generic identification scheme for multimedia information access triggered by tag-based identification.

Appendix I provides additional information to explain why a new generic identification scheme is needed.

# 1	Scope

This Recommendation defines an identification scheme for multimedia information access triggered by tag-based identification.  This identification scheme is mainly used in the multimedia information system architecture defined in [ITU-T H.621].  It also satisfies the requirements defined in [ITU-T F.771].

This Recommendation does not define encoding rules to store the identifier value into data carriers such as barcode tags and RFID tags.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.668]	Recommendation ITU-T X.668 (2008), Information technology – Open Systems Interconnection – Procedures for the operation of OSI Registration Authorities: Registration of object identifier arcs for applications and services using tag-based identification.

[ITU-T X.680]	Recommendation ITU-T X.680 (2008), Information technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation.

[ITU-T F.771]	Recommendation ITU-T F.771 (2008), Service description and requirements for multimedia information access triggered by tag-based identification.

[ITU-T H.621]	Recommendation ITU-T H.621 (2008), Architecture of a system for multimedia information access triggered by tag-based identification.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	ID resolution [ITU-T F.771]: A function to resolve an identifier into associated information.

3.1.2	Multimedia information [ITU-T F.771]: Digital information that uses multiple forms of information content and information processing, such as text, pictures, audio, video, three-dimensional panoramic pictures and digital maps, which informs or entertains users.

3.1.3	Object Identifier [ITU-T X.680]: A globally unique value associated with an object to unambiguously identify it.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Top Level Code: Part of identifier assigned to countries and international standardization organizations.

3.2.2	Identifier value: A string of characters that represents the value of the identifier.

3.2.3	Identification scheme: Definition and description of the structure of identifiers

3.2.4	Second Level Organization Code: Part of identifier assigned to lower level Registration Authority or to an entity that uses the subspace directly.

3.2.5	Serial Code: Part of identifier to serialize individual multimedia information and service.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| GIAI | Global Individual Asset Identifier |
| --- | --- |
| GRAI | Global Returnable Asset Identifier |
| HLC | High Level Code |
| ID | Identifier |
| IP | Internet Protocol |
| ISBN | International Standard Book Number |
| ISSN | International Standard Serial Number |
| LRA | Local registration authority |
| OID | Object identifier |
| RA | Registration authority |
| RFID | Radio frequency identification |
| SC | Serial Code |
| SGLN | Serialized Global Location Number |
| SGTIN | Serialized Global Trade Item Number |
| SLOC | Second Level Organization Code |
| SSCC | Serial Shipping Container Code |
| TLC | Top Level Code |
| URL | Uniform resource locator |
| URN | Uniform resource name |

# 5	Conventions

In this Recommendation, xx2 denotes that number xx is expressed as a binary (base-2) number.

# 6	Concept of identification scheme for multimedia information access triggered by tag-based identification

## 6.1	Overview

An identification scheme in this Recommendation is designed to discriminate multimedia information and serviceassociated with "objects" or "places" of the real world in the architecture defined by [ITU-T H.621].

The data carrier such as RFID tag that triggers the access can be attached on anywhere and the whereabouts is out of scope of this Recommendation.

## 6.2	Uniqueness of ID value

Figure 1 gives an overview of the structure of multiple identification schemes.  Many different identification schemes can be used for multimedia information access triggered by tag-based identification. Each identification scheme should be identified by object identifier (OID) under the arc {joint-iso-itu-t(2) tag-based (27)} as specified in [ITU-T X.668]. For example, in the figure, the top-most row indicates that identification scheme 1 is identified by OID arch {2 27 i1}.

Identifier schemes which are widely used and already have an OID allocated to them (not under arc {joint-iso-itu-t(2) tag-based (27)}) can continue to use that OID. In Figure 1, “Existing identification scheme 1”, that has OID value not under {2 27} OID arc is identified by OID arc {x1 y1 z1}.

For a given identifier scheme (identified by an OID), the uniqueness of identifier values is required to be managed by the organization who is allocating those values. That is, the uniqueness of identifier values should be managed by the Registration Authority (RA) for that identifier scheme with the cooperation of the users of the identifiers.

| OID:2.27.i1 | + | Identification scheme 1 |
| --- | --- | --- |
| OID:2.27.im | + | Identification scheme m |
| OID:x1.y1.z1 | + | Existing Identification scheme 1 |
| OID:xN.yN.zN | + | Existing Identification scheme N |

Figure 1 – Multiple identification schemes

# 7	Identification scheme for multimedia information access triggered by tag-based identification

## 7.1	Structure

An identifier defined in this Recommendation consists of the fields: HLC, TLC and Class, followed by some of the elements (SC, and SLOC) as shown in Figure 2.

| HLC | TLC | CLASS | SLOC / SC |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 bits | 16 bits | 4 bits | 8bits | 16 bits |  | 16 |  |  | 16 | 16 | 16 |  | 16 |
| 00012 | 16 bits | 00001 | reserved |  |  |  |  |  |  |  |  |  |  |
|  |  | 00011 | reserved |  |  |  |  |  |  |  |  |  |  |
|  |  | 00101 | reserved |  |  |  |  |  |  |  |  |  |  |
|  |  | 00111 | reserved |  |  |  |  |  |  |  |  |  |  |
|  |  | 01001 | reserved |  |  |  |  |  |  |  |  |  |  |
|  |  | 01011 | reserved |  |  |  |  |  |  |  |  |  |  |
|  |  | 01101 | reserved |  |  |  |  |  |  |  |  |  |  |
|  |  | 01111 | reserved |  |  |  |  |  |  |  |  |  |  |
|  |  | 10001 | reserved |  |  |  |  |  |  |  |  |  |  |
|  |  | 10011 | SLOC |  | SC |  |  |  |  |  |  |  |  |
|  |  | 10101 | SLOC |  |  |  | SC |  |  |  |  |  |  |
|  |  | 10111 | SLOC |  |  |  |  | SC |  |  |  |  |  |
|  |  | 11001 | SLOC |  |  |  |  |  |  | SC |  |  |  |
|  |  | 11011 | SLOC |  |  |  |  |  |  |  |  | SC |  |
|  |  | 11101 | SLOC |  |  |  |  |  |  |  |  |  | SC |
|  |  | 11111 | reserved |  |  |  |  |  |  |  |  |  |  |
| 00002 | reserved for backward compatibility (see 7.2) |  |  |  |  |  |  |  |  |  |  |  |  |
| 00102 ~ 11112 | reserved for future use |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 2 – Structure of identifier

Table 1 – Field name and its length

| Field Name | Length |
| --- | --- |
| HLC | 4 bits |
| TLC | 16 bits |
| CLASS | 4 bits |
| SLOC, SC | Variable length, but the sum of these is 104 bits Value of Class; length of SLOC; length of SC. 9; 8 bits; 96 bits. 10; 24 bits; 80 bits. 11; 40 bits; 64 bits. 12; 56 bits; 48 bits. 13; 72 bits; 32 bits. 14; 88 bits; 16 bits. |

## 7.2	High Level Code (HLC)

HLC is 4 bits long and represents the identifier sub-blocks. For example, the identification scheme defined in this Recommendation has an HLC of 00012. It is composed of TLC and Class, and the other elements shown in Figure 2.

HLC from 00102 to 11112 are reserved for the future use.  If a new identifier sub-block is needed, this Recommendation shall be changed to allocate an HLC to that new sub-block.

Part of the identifier sub-block with HLC of 00002 has already been allocated to already existing ‘ucode’ [b-uID].  For this historical reason, the new TLC assignment shall be done only in identifier sub-block with HLC 00012 only.

## 7.3	Top Level Code (TLC)

TLC is 16 bits long and is assigned to countries and international standardization organizations as specified in [ITU-T H.ID-RA]. RA may assign TLC to other organizations. These organizations that are assigned TLC act as the RA of the second level organization code assignment below.

## 7.4	Class

Class indicates the layout of a particular identifier format. There are different formats that correspond to Class 9, 10, …, 14 as shown on Figure 2 (Class 0, 1, 2, …, 8, 15 are reserved for future use).

## 7.5	Second Level Organization Code (SLOC)

An SLOC is allocated to a lower level RA that handles the allocation of the subspace to other organizations or to an entity that uses the subspace directly in the case of  Class from 9 to14. The SLOC assignment is done by an entity that is assigned the TLC of that subspace.  The subspace controlled by the lower level RA is called the SLOC space. It has six different sizes from 16 bits to 96 bits by steps of 16 bits.

SLOC spaces are assumed to be managed by entities smaller than the entities that are assigned TLC, such as companies, associations, universities, schools, ministries of governments, local governments, etc.

## 7.6	Serial Code (SC)

This code is a serialization number to identify individual multimedia information and service.

Appendix I

Survey on identification schemes

(This appendix does not form an integral part of this Recommendation)

## I.1	Introduction

The purpose of this Appendix is to investigate whether currently existing identification schemes (most of them being internationally standardized) satisfy the requirements presented in F.771 or not.

## I.2	Need for generic ID scheme for multimedia information access triggered by tag-based identification

Generic identification scheme in this Recommendation is applicable to multimedia information and service associated with any kind of objects and places.  Any systems for multimedia information access triggered by tag-based identification should implement the generic identification scheme defined in this Recommendation.

## I.3	Requirements on identifier for multimedia information access triggered by tag-based identification

[ITU-T F.771] defines five requirements for identifiers (clause 7.3, Identifier requirements) for multimedia information access triggered by tag-based identification.  In the following, we give clarification of each identifier requirement to stress that identification scheme defined in this Recommendation satisfies these identifier requirements.

- ID-001: Identifier is recommended to be used by different applications.

Multimedia information access triggered by tag-based identification specifies a mechanism of information processing and communications, but no applications.  The mechanism is proposed and recommended to be used by any applications.  In other words, it is "general purpose".  Also, identifiers should be general purpose too.  This means, it is recommended to be used by different applications.  Most identification schemes can be used for many applications, so almost every identification scheme satisfies this requirement.

- ID-002: Identifier is required to be assigned for real-world entities such as physical/logical objects, persons, and places.

“Tag-based identification” deals with every type of real-world entities such as tangible objects (items), persons and places because all of these can be targets of tag-based identification. (Although “people” is mentioned here, since defining method for identifying people is not in the scope of SG16 and it is studied by SG17, this Recommendation is not attempting to define identifier for people). In some applications, information service is triggered by some tangible objects (items), persons, or places.  So identifiers must be assignable to tangible objects, logical objects, people, and places.  It is important to note that many application target areas do not have an applicable international standard identification scheme.  For example, locations with meanings vegetables, fishes, meats, houses, roads, buildings, bridges, boats, art items, posters, and so on do not have proper identification schemes based on international standard. This Recommendation is meant for such wider area which lacks previous standard ID scheme.

- ID-003: Identifier is required to be issuable by any organizations such as business companies, non-profit organizations, governments and individual users.

ID-003 is a requirement for governance of identification scheme.  To satisfy this requirement, for example, even an individual without any corporate affiliation should be able to obtain identifiers, and identifiers must be allocated at low enough cost.  For example, if an identification scheme has a short company code bit field, and it is too short to be used by all the companies in the country or in the world, it does not satisfy this requirement.  If the company code cannot be assigned to non-company organizations such as governments, NPOs, tiny shops, schools, and individuals, it does not satisfy this requirement.  Governance of this Recommendation should satisfy this requirement.

- ID-004: Identifier is required to be globally unique so that the multimedia information access triggered by the identifier is globally available.

Essentially, identification scheme proposed must be a globally unique numbering system.  When the same identifier is assigned to two different entities, confusion arises.  However, some identification schemes do not follow this principle quite completely.  For example, IP address system has private address subspace in its numbering space, such as "192.168.1.1".  Everyone can assign the private address to the network interfaces of his/her own networked machines.  Some SCM identification schemes have "in-house ID code area", which can be used by any shops or factories freely, thus the code area is not globally unique.  In [b-ITU-T X.667], identifiers are generated automatically by standard algorithm and procedure, so each identifier is assured to be globally unique almost.  However, there is a small possibility of collisions theoretically when random number generators are used.  Additionally, there may be bugs in the identifier generating software, and some evil people may issue duplicate identifiers intentionally.  So, X. 667 do not satisfy this requirement quite well.  Governance of this Recommendation is meant to satisfy this requirement.

- ID-005: Multiple identifier schemes are required to be supported.

This is an important requirement to accommodate both the existing and the yet-to-appear future identification schemes.

Among the reuirements outlined ablove, ID-005 is not a requirement for identification scheme itself, but for the system that utilizes identifiers. Therefore, we excluded this requirement from the use case analysis.  ID-001 is also excluded from the use case analysis because all the target identification schemes listed below satisfy this requirement.

## I.4	Survey on existing identification schemes

### I.4.1	Existing identification schemes

To make this use case analysis comprehensive, we investigate widely used existing identification schemes from wide range of application areas.

#### I.4.1.1	EPC

EPC is a family of identification schemes proposed by EPCglobal. It is currently one of the most widely used identification schemes in the field of supply chain management, even though it is not a de jure international standard. It is designed to be used in a low-cost method of tracking goods using RFID technology, to meet the needs of various industries, to identify each item manufactured (unlike barcode systems), and to guarantee uniqueness for all EPC-compliant tags. This implies that the initial intension of EPC is to identify products manufactured by using RFID-based tags.

It contains Serialized Global Trade Item Number (SGTIN), Serial Shipping Container Code (SSCC), Serialized Global Location Number (SGLN), Global Returnable Asset Identifier (GRAI) and Global Individual Asset Identifier (GIAI).

#### I.4.1.2	ISO/IEC 15459-x

ISO/IEC 15459-x is an identification scheme designed for supply chain management obviously because the original intended application of this identification scheme is supply chain management system.

#### I.4.1.3	ISO/IEC 6709

ISO/IEC 6709 is an international standard representation of latitude, longitude and altitude for (fixed) geographic point locations. It is used by many GIS applications, but other kinds of ID-based applications such as supply chain management systems, whose intention is identification of moving objects, cannot use this identification scheme.

#### I.4.1.4	ISO 2108 International Standard Book Number

International Standard Book Number (ISBN) is a numbering scheme for identification of books.

#### I.4.1.5	ISO 3297 International Standard Serial Number

International Standard Serial Number (ISSN) is a numbering scheme for identification of serial publications such as newspapers, magazines, journals and annually published books globally used in the world.

#### I.4.1.6	Universally Unique Identifier

Universally Unique Identifier (UUID) in [b-ITU-T X.667] is an identification scheme designed for identifying information or components in the distributed systems without assumption of any management systems of identifiers. One of the most famous implementation of UUID is Microsoft’s GUID.

#### I.4.1.7	ISO/IEC 15963

[b-ISO 15963] is an identification scheme designed for RFID tags and mainly used for supply chain management. Since this identification scheme is designed only for RFID tags, it cannot be used for applications that require other kinds of ID tags such as printing code.

#### I.4.1.8	ISO/IEC 11784

[b-ISO/IEC 11784] is an identification scheme designed for identifying animals.

Note that all of the above identification schemes other than EPC are de jure international standards, and EPC is used widely.

### I.4.2	Analysis

Surveys on existing identification schemes are in Table I.1. For comparison, we also include identification scheme defined in this Recommendation in this analysis.

Table I.1 – Analysis on existing identification schemes

| ID schemes | ID-002 | ID-003 | ID-004 |
| --- | --- | --- | --- |
| EPC | Most of identifiers are mainly assigned for objects, but not for the logical objects. SSCC can be assigned for the shipping container and SGLN can be assigned only for the location. | Registered issuing agencies may issue identifiers and hierarchical structure of issuing agencies is provided. | Each identifier is assured to be globally unique. |
| ISO/IEC 15459-x | Identifiers are mainly assigned for objects, but not places. Since this identification scheme is designed for ID tags, it is impossible to assign the identifier to any logical objects. | Registered issuing agencies may issue identifiers and hierarchical structure of issuing agencies is provided. | Each identifier is assured to be globally unique. |
| ISO 6709 | Identifiers are assigned for geographic point locations. This identifier cannot be assigned to any objects that move around. | It is a representation for identifying geographic point locations, thus no one can issue the identifiers. | Each identifier is assured to be globally unique. |
| ISBN | ISBN numbers are assigned only for books and physical publications. ISBN numbers are not assignable for any logical objects and other physical objects. | Only affiliated organizations (sometimes national organizations) that manage books in the region or country may issue the ISBN numbers. | Identifiers are assured to be globally unique. |
| ISSN | ISSN numbers are assigned only for serial publications. ISSN numbers are not assignable for any other logical or physical objects. | Only affiliated organizations (sometimes national organizations) that manage serial publications in the region or country may issue the ISSN numbers. | Identifiers are assured to be globally unique. |
| UUID | Identifiers can be assigned for everything that can be identified. | Individual users or programs may issue identifiers. Since central coordination is difficult in this identification scheme, it is not suitable for organizations which want to control issuing of identifiers from the fixed numbering space that can freely be managed by the organization to hierarchically manage products. | Each identifier is almost assured to be globally unique, but there are some possibilities of collisions (except if the UUID is registered as an OID). |
| ISO/IEC 15963 | Identifiers are mainly assigned for objects, but not places. Since this identification scheme is designed for RFID tags, it is impossible to assign the identifier to any logical objects. | This identifier is always written into RFID tag when tag chip manufacturer produce RFID tag and only tag chip manufacturer can issues identifier. | Each identifier is assured to be globally unique. |
| ISO/IEC 11784 | Identifiers are assigned only for animals. | Registered issuing agencies may issue identifiers and hierarchical structure of issuing agencies is provided. | Each identifier is assured to be globally unique. |
| Identification scheme defined in this Recommendation | Identification scheme defined in this Recommendation can be assigned for multimedia information and service associated with any object or place. | Registered issuing agencies may issue identifiers and hierarchical structure of issuing agencies is provided. | Each identifier is assured to be globally unique. |

### I.4.3	Summary

Table I.2 summarizes the information in the preceding clauses. According to this investigation, it is considered that no existing identification scheme other than identification scheme defined in this Recommendation satisfies all the requirements described in ITU-T F.771.

Table I.2 – Summary of analysis

|  | ID-002 | ID-003 | ID-004 |
| --- | --- | --- | --- |
| EPC | U | S | S |
| ISO/IEC 15459-x | U | S | S |
| ISO 6709 | U | U | S |
| ISBN | U | S | S |
| ISSN | U | S | S |
| UUID | S | U | P |
| ISO/IEC 15963 | U | P | S |
| ISO/IEC 11784 | U | S | S |
| Identification scheme defined in this Recommendation | S | S | S |

Legend: S: satisfied, U: unsatisfied, P: Not strictly satisfied

Bibliography

[b-ITU-T X.667]		Recommendation ITU-T X.667 | ISO/IEC 9834-8 (2008), Information technology – Open Systems Interconnection – Procedures for the operation of OSI Registration Authorities: Generation and registration of Universally Unique Identifiers (UUIDs) and their use as ASN.1 object identifier components

[ITU-T H.IRP]			Draft Recommendation ITU-T H.IRP, ID resolution protocols for multimedia information access triggered by tag-based identification.

[b-ISO/IEC 15459-x]	ISO/IEC 15459-x (2006), Information technology – Unique identifiers

[b-ISO/IEC 15963]		ISO/IEC 15963 (2009), Information technology -- Radio frequency identification for item management -- Unique identification for RF tags

[b-ISO/IEC 11784]		ISO/IEC 11784 (1996), Radio frequency identification of animals – Code structure

[b-ISO 2108]			ISO 2108 (2005), Information and documentation – International standard book number (ISBN)

[b-ISO 3297]			ISO 3297 (2007), Information and documentation – International standard serial number (ISSN)

[b-ISO 6709]			ISO 6709 (2008), Radio frequency identification of animals – Code structure

[b-uID] 					uID center “Ubiquitous Code: ucode” Version 1.A0.10 (2009)

__________________