| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4194 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Change request to clause 3 and 6 of ITU.F.USN-SM |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes changes to clause 3 and 6 of ITU-T F.USN-SM.

----------- Start of changes -----------

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	sensor [ITU-T Y.2221]: An electronic device that senses a physical condition or chemical compound and delivers an electronic signal proportional to the observed characteristic.

3.1.2	sensor network [ITU-T Y.2221]: A network comprised of inter-connected sensor nodes exchanging sensed data by wired or wireless communication.

3.1.3	sensor node [ITU-T Y.2221]: A device consisting of sensor(s) and optional actuator(s) with  capabilities of sensed data processing and networking.

3.1.4	smart grid [ITU-T FG-Smart]: An electricity network that can intelligently integrate the actions of all users connected to it - generators, consumers and those that do both - in order to efficiently deliver sustainable, economic and secure electricity supplies.

3.1.5	ubiquitous sensor network (USN) [ITU-T Y.2221]: A conceptual network built over existing physical networks which makes use of sensed data and provides knowledge services to anyone, anywhere and at any time, and where the information is generated by using context awareness.

3.1.6	USN gateway [ITU-T Y.2221]: A node which interconnects sensor networks with other networks.

## Terms defined in this Recommendation

This Recommendation defines the following terms:

[Contributor’s Note] Clause 3.2 is rearranged as an alphabetical order.

3.2.1	demand response: A smart metering feature that allows consumers to reduce or change their use patterns of electricity, gas and water during peak demand usually in exchange for a financial incentive.

3.2.2	sensor network gateway: Sensor network element that connects a sensor network to another network with different architectures or protocols, permitting information exchange between them. See also USN gateway.

NOTE: Sensor network gateway functionalities may include either address or protocol translation or both.

3.2.3	smart meter: A premise device to monitor and control electrical power, gas and water usage of home appliances based on demand response information from home appliances.

3.2.4	smart metering: An operation to provide information to customers and smart metering operator about energy consumption. The information includes how much energy the customers are consuming or generating and how much it costs, etc.

3.2.5	smart metering gateway: See USN gateway.

# Overview of smart metering

Smart meters are utility meters, for example electricity, gas, water and heat meters, which may bring about the end of estimated bills and meter readings, and provide customers and energy distributors and suppliers with accurate information on the amount of utility being used.

Smart metering provides:

- Customers with the information they require to become energy savvy and make smarter decisions about their energy usage,

- Energy suppliers with the means to better understand and service their customers,

- Distributors with an effective tool to better monitor and manage their networks.

In addition, smart metering enables those customers who choose to generate their own electricity (micro generators) to be financially rewarded for their contribution to the national grid and distributors to better manage this contribution [b-ESTI TR 102.691].

Smart metering may be regarded as one of the key technologies for smart grid system.

## Smart grids and smart metering

Smart grid is a type of electrical grid that attempts to predict and intelligently respond to the behavior and actions of all electric power users connected to it - suppliers, consumers and those that do both – in order to efficiently deliver reliable, economic, and sustainable electricity services including:

- Enhancement of reliability

- Reducing peak demand

- Shifting usage to off-peak hours

- Lower total energy consumption

- Actively managing electricity charging

- Actively managing other usage to respond to other renewable resources

To enable smart grid, information regarding the generation, flow and utilization of electricity should be collected from the grid, shared with all players in the system via various communication channels and processed appropriately.

[Contributor’s Note] The following paragraphs do not seem to be relevant to the purpose of clause 6.1, therefore this contribution proposes to remove the following paragraphs.

[Contributor’s Note] The following paragraphs are moved to the bottom of the first paragraph.

Smart grids technologies have already used in other applications, such as manufacturing and telecommunications. In general, smart grid technology can be divided into seven areas, such as integrated communications, sensing and measurement, smart metering, advanced components and control, improved interfaces and decision support, and smart power generation. In other words, smart meters are part of smart grid as key components, and also smart metering is one of crucial features for smart grids.

Figure 1: Smart Grid Architecture [b-NIST]

## Technologies of smart metering

Not only governments and utility companies, such as electricity, gas, and water suppliers, but also researchers have been interested in automatic meter reading based on communication system, known as smart metering because smart metering can bring various benefits to customers and governments as well as utility companies as follows:

- Lower metering cost

- Energy savings for residential consumers

- Reliability of supply

- Various pricing schemes to attract new consumers

- Easier detection of fraud and outage

- Automated billing

Smart metering performs metering and exchanging metering information between smart meters and utility companies. Various technologies for metering and exchanging metering information have been implemented. Power line communication has been used as means for delivering the electricity, gas and water measurements to the utility providers. In addition, messaging over GSM network has been utilized for automatic meter reading system.

Sensor network technologies can be used for metering and collecting information, and communication networks can be used for exchanging the information. Figure 1 depicts an overall diagram of smart metering systems. The metering information, obtained from home appliances by sensor nodes including communication interfaces in smart home, are collected and delivered to operators of utility companies.

The operators manage the collected information and inform consumers of variable pricing information or enforced load control messages. Such operators’ action leads to consumers’ responses to reduce energy consumption.

Figure 2: Overview of smart metering

----------- End of changes -----------

________________________