| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4195 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Change request to clause 8, 9 and 10 of ITU.F.USN-SM |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes changes to clause 8, 9 and 10 of ITU-T F.USN-SM.

----------- Start of changes -----------

# Requirements of smart metering services

This clause specifies the requirements of smart metering services. However, all the requirements of smart metering services are not specified here but the requirements which may be related with sensor networks or USN are specified.

## Time synchronization

In smart metering services, exchanged data uses time stamp between smart meters and the systems of smart metering operators. Therefore, accurate and secure time synchronization should be supported in smart metering systems.

## Reliable information delivery

In the smart metering services, meter reading data can be frequently transferred from distributed meters, and also operators can send many control messages to smart meters. When delivery is failed, the operators cannot collect all of meter reading data from smart meters. As a result, missing meter reading data would lead to unwanted results. Therefore, the smart metering services are required to provide reliable information delivery, such as meter reading data and messages described in the scenario.

## Minimal time delay

In case of on-demand remote meter reading, meter reading request and response should be delivered to the peer with minimal time delay.

## Real-time delivery of meter reading data

Demands of smart metering operators require real-time responses of consumers. Therefore, meter reading data are required to arrive at the operators within a given amount of time. In the scenario I-3 and II-3, if the meter reading data are not delivered within the given time, the operators cannot obtain real-time results of smart metering. Therefore, smart metering services are required to guarantee real-time delivery.

## Bi-directional communication between meters and operators

Meter reading data are delivered from smart meters to smart metering operators as shown in the scenario I-3, II-3, and III-2, and the messages are also sent from the operators to the smart meters vice versa (in scenario III-3 and IV-3). For doing this, the smart metering services are required to support bi-directional communication between meters and operators.

## Change of parameters for regularly scheduled reporting including tariff configuration

During operation of smart metering service, smart metering applications should be able to specify a regularly scheduled reporting (scenario I) of specific parameters for specific metring devices. Also, smart metering application should be able to modify the value of the requested time period. When tariff configuration is necessary to be changed, smart metering applications should be able to change the tariff configuration of smart meters.

## Security support including authorization of operator and data confidentiality

Operators need to control meters in order that the meters take actions according to the indication of operators. In this case, the operators should be authorized by smart meters for taking secure actions (scenario II-2).

Because metering information may include privacy information, it should not be revealed to unauthenticated third party. Therefore, the metering information is required to be secured to unauthenticated third party as well as support of integrity check of the data exchanged.

## Authentication of smart meters

Smart meters should be authenticated by smart metering applications. Authentication of smart metering devices can be achieved by smart metring applications directly or the authenticated smart metering gateways.

## Meter reading data processing

Either operators or smart meters should be able to perform data processing on meter reading such as filtering, validation and aggregation. In some case, data mining processing is necessary, for example, analyzing patterns and predicting some events.

## Monitoring and management of smart meters

Smart meters or smart metering gateways should be monitored proactively in order to attempt to prevent and correct errors. In addition, the following management capabilities should be supported from the network side:

- Secure software and firmware provisioning

- Configuration management

- Auto-configuration functions for smart meters area networks

# USN-based smart metering services

The smart metering services shown in Figure 7 require technological facilities to support metering and exchanging metering information between smart meters and metering operators in utility control center. Basic characteristic of USN applications and services is gathering data by sensor networks and transmitting data to a remote server through communication networks. Also, control information is transmitted to sensor networks from the remote server. Furthermore, if USN middleware is deployed, the middleware can provide USN applications or sensor networks with various functions such as data query, data mining, event processing, sensor network metadata directory service, data filtering, context-aware rule processing and sensor network management. These features of USN can be applied to the smart metering services. Sensor nodes in sensor networks have capabilities of metering and delivering metering information to a server of utility control center. USN middleware can provide functions which are required by on-demand remote meter reading scenario and demand response meter reading scenario.

A smart metering system can be implemented by sensor nodes and USN middleware as follows.

- Smart metering sensor nodes: Each sensor node may have capabilities of smart metering, therefore sensor nodes can acts as smart meters supporting of processing smart metering information for the smart metering procedures, such as meter reading collection and tariff setting.

- Smart metering gateway: A sensor network gateway or a USN gateway is capable of connecting a sensor network to another network with different architectures or protocols, permitting information exchange between them. Therefore a sensor network gateway or a USN gateway can be used as a smart metering gateway. Smart metering gateway delivers information between the sensor nodes (smart meters) and metering operators. In case a gateway has capabilities of smart metering, it collects meter reading data and then delivers to the metering operator(s).

- USN middleware: USN middleware provides data filtering, data query, data mining and context-aware rule processing, etc. These features of USN middleware satisfy meter reading data processing requirement and can be applied to on-demand remote meter reading scenario and demand response meter reading scenario.

Figure 7: Smart metering service based on USN

# USN capabilities for smart metering services

## Time synchronization

USN is required to support accurate and secure time synchronization among sensor nodes, sensor network gateways and the smart metering applications.

## Reliable transmission

Smart metering requires reliability on metering information delivery to get correct results. Therefore, USN is recommended to guarantee reliable transmission for data and message delivery.

## Scalability

New smart meters (sensor nodes) can be added to, or one of the existing meters (sensor nodes) can be removed from an existing smart metering group (sensor networks). Such a change should not degrade the performance of the smart metering.  Therefore, sensor networks in USN are required to support scalability.

## Mobility support

In some cases, a consumer can change the positions of home appliances. This change may involve the changes meters’ position in intra-sensor network or inter-sensor networks. Therefore, sensor networks in USN are recommended to support mobility of smart meters (sensor nodes).

## Delivery latency

Meter reading data should be delivered with minimal time delay or within a given time. For this requirement, USN is recommended to guarantee data delivery with minimal time delay or within a given time.

## Remote configuration setting and re-setting of sensor nodes

Smart metering applications require change of parameters in smart meter for regularly scheduled reporting as well as change of tariff configuration. For satisfying this requirement, USN is recommended to support remote configuration setting and re-setting of sensor nodes acting as smart meters.

## Fault detection and recovery

There is a possibility of link failure between nodes due to the characteristics of wireless transmission when sensor nodes use wireless communications,. Such a failure can give negative effects on reliability and delivery latency of the smart metering services. USN is recommended to detect faults and recover from the faults.

## Security supporting confidentiality, integrity check, authorization and authentication

Meter reading data and messages may be transferred to/from operators and meters by hop-by-hop transmission among sensor nodes. Therefore, hop-by-hop security among sensor nodes is recommended to be implemented and USN is recommended to provide integrity check of the data exchanged. In addition, USN is recommended to provide smart meters with authorization capabilities for access to smart meters of smart metering operators.

Smart meters should be authenticated by smart metering applications directly or through smart metering gateway. Therefore USN is recommended to provide authentication capability to smart metering applications for authentication of sensor nodes (smart meters).

## Connectivity

Meter reading data, obtained from each sensor node, are collected by a gateway, and then the collected meter reading data are delivered to smart metering operators through transport networks.  In addition, control messages of the operators are also delivered to each sensor node. For doing this, connectivity between sensor nodes, connectivity between sensor nodes and the gateway, and connectivity between the gateway and outer networks are recommended to be guaranteed.

## Unicasting & Multicasting

A unicast networking service is required to be supported among sensor nodes in order that meter reading data are delivered to smart metering operators. Furthermore, control messages of the metering operators should be delivered to all of sensor nodes. Therefore, multicast networking support is also required.

## Data aggregation

Meter reading data from smart meters is collected and aggregated at smart meter or smart metering gateway. And then they are transferred to the smart metering operator(s).

## Monitoring and management of sensor nodes

Sensor nodes (smart meters) or sensor networks gateways (smart metering gateways) are recommended to be monitored and managed proactively in order to attempt to prevent and correct errors including secure software and firmware provisioning and configuration management as well as auto-configuration functions for sensor networks.

----------- End of changes -----------

_________________________