| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4200 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Advanced Telecommunications Research Institute International (ATR) |  |  |  |
| Title: | F.USN-NRP: Proposed modifications to the draft Recommendation (TD 674R1, Geneva, 21 November - 2 December 2011) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes modification of Draft Recommendation ITU-T F.USN-NRP.

Contents

1	Scope	4

2	References	4

3	Definitions	4

3.1	Terms defined elsewhere:	4

3.2	Terms defined in this Recommendation	5

4	Abbreviations and acronyms	5

5	Conventions	6

6	Overview of UNR-PF in terms of USN applications and services	6

6.1	General overview of UNR-PF	6

6.2	Requirements for UNR-PF applications and services	7

6.2.1	Abstraction of functionality	7

6.2.2	Inter-service collaboration	8

6.2.3	Service among multiple areas	9

6.2.4	Service execution based on customer attributes	11

6.3	Relationships of USN and UNR-PF	12

7	Use cases of UNR-PF	13

7.1	Health support service	13

7.2	Shopping support service	14

8	Reference architecture of UNR-PF	15

8.1.1	Robot registry	16

8.1.2	Operator registry	17

8.1.3	User registry	17

8.1.4	Map registry	17

8.1.5	Service queue	17

8.1.6	State manager	17

8.1.7	Resource manager	17

8.1.8	Message manager	17

9	USN capability requirements for UNR-PF applications and services	18

9.1	Formal device profile description	18

9.2	Dynamic management of devices in spatiotemporal domain	18

9.3	Common abstracted interface	18

Bibliography	18

Appendix I Gap analysis on network robot platform	20

Draft new Recommendation F.USN-NRP

Requirements for Ubiquitous Network Robot Platform (UNR-PF) applications and services in USN environment

(Editor’s note: The word “robot” may remain but title may be changed under the future consideration.)

AAP Summary

To be provided before Consent.

Summary

This Recommendation describes concept and use cases of UNR-PF as one of key USN applications and services.

Keywords

Network Robot System (NRS), Ubiquitous Network Robot Platform (UNR-PF), USN, middleware

# Scope

This Recommendation covers the followings:

- Overview of UNR-PF in terms of USN applications and services;

- Use cases of UNR-PF;

- USN service requirements for UNR-PF;

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.2221]		ITU-T Recommendation Y.2221 (2009), Requirements for support of ubiquitous sensor network (USN) applications and services in the NGN environment.

[ITU-T F.744] 		ITU-T Recommendation F.744 (2009), Service description and requirements for ubiquitous sensor network middleware

# Definitions

## Terms defined elsewhere:

This Recommendation uses the following terms defined elsewhere:

sensor [ITU-T Y.2221]: An electronic device that senses a physical condition or chemical compound and delivers an electronic signal proportional to the observed characteristic.

- sensor network [ITU-T Y.2221]: A network comprised of inter-connected sensor nodes exchanging sensed data by wired or wireless communication.

- sensor node [ITU-T Y.2221]: A device consisting of sensor(s) and optional actuator(s) with capabilities of sensed data processing and networking.

service [ITU-T Z.100]: A set of functions and facilities offered to a user by a provider.

ubiquitous sensor network (USN) [ITU-T Y.2221]: A conceptual network built over existing physical networks which makes use of sensed data and provides knowledge services to anyone, anywhere and at anytime, and where the information is generated by using context awareness.

## Terms defined in this Recommendation

This Recommendation defines the following terms:

robot:

The

A device with a processing unit often accompanied with sensors and actuators that can work with real world phenomena and entities. Robots can be roughly classified into the following three types:

visible robot, virtual

- robot and unconscious robot.

ubiquitous network robot platform: A middleware that enables applications to perform services continuously by combining  multiple robotic devices effectively across multiple areas.

unconscious robot: A robot that mainly senses real world phenomena and process measurement results into high-level estimations, such as cameras equipped with a processing unit to detect people. Unconscious robots are often embedded in environments such as roads, town, room, or are hidden in clothes or accessories.

visible robot: A robot that can be seen and it can take various forms of body such as humanoid, pet, stuffed animal and so on. Visible robots interact with people in combination of verbal and nonverbal ways such as using gestures.

virtual robot: A robot that mainly processes and utilizes information via network such as smartphones. Virtual robots typically interact with people through audio and visual modalities.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| GPF | Global PlatForm |
| --- | --- |
| ID | IDentifier |
| LPF | Local PlatForm |
| NGN | Next Generation Network |
| NRS | Network Robot System |
| PF | PlatForm |
| QoS | Quality of Service |
| SOA | Service Oriented Architecture |
| UNR | Ubiquitous Network Robot |
| UNR-PF | Ubiquitous Network Robot PlatForm |

# Conventions

None.

# Overview of UNR-PF in terms of USN applications and services

## General overview of UNR-PF

Demands for assistive robots are quickly emerging these days with the approach of aged society. In recent decades, researchers had been focusing on constructing robots that can support human daily activities by interactive means. As the structure and functionality of robots grow large and complicated, constructing robots now requires greater time, cost and effort. Similar to traditional software engineering, people started to seek modularity and reusability of basic functional components, which has led to research and development of common libraries and middlewares. Such allows developers to utilize existing functional modules in combination with their own software and to rapidly develop working robots. This modularized development process has accelerated the development of stand-alone robots as well as individual functional components. At the same time, however, variations in both hardware and software have decreased the reusability of robotic applications.

Another approach to enhance capabilities of robots is the concept named Network Robot System (NRS) or Networked Robots [b-Sanfeliu2010]. The main goal of NRS was to integrate various types of devices such as robots, sensor networks and smartphones, so that the whole system could act as an integrated system with enhanced capabilities that cannot be attained by a single robot or multiple uniform robots. Since its proposal in 2002, many research projects have been carried out accompanied with real-world field experiments, and they have successfully shown the concept to be effective [b-Jung2007, b-Nakamura2008, b-Tezuka2006, b-Shiomi2011, b-Salvini2011, b-Sanfeliu2011].

However, people need robots to support us in a much wider range of daily activities. Robots for individual household tasks such as cleaning floors or folding laundry are not sufficient. Support for the elderly and disabled in a variety of daily activities is greatly demanded. Such support requires robots to accompany people to many different places (e.g. homes, shopping malls, hospitals) and to assist people for various activities (e.g. checking health, showing routes, carrying luggage) in ways appropriate for each place and for physical demands of each person (e.g. using wheelchairs). At the same time, these sequences of activities shall be well integrated to provide comfortable and seamless support throughout our daily lives.

Existing robotic systems cannot yet provide such continuous support in various aspects of our daily lives. Although they have had success in enriching capabilities of individual robotic applications, a general framework is missing for adapting behaviors of robots and composing them to form an integrated sequence of applications. Similar concepts can be seen in computer systems, such as Service Oriented Architecture (SOA). SOA provides design principles for constructing large-scale information systems has been adopted in numbers of middlewares, especially in web service systems. In combination with the concepts of grid- or cloud-computing systems, many commercial services with high dependability are available today. However, these concepts are not sufficient for robotic systems.

One major element that makes information systems different from robotic systems is where the applications are performed. Information system applications are performed in cyberspace with information terminals such as personal computers or cellphones. This is especially clear for web-based systems where every application run through the web browsers. Based on the information shown on the screen, people perform actions such as business procedures, travel and cooking. Thus, the actual work in the physical world is left to the customers. And this is where the robotic systems with sensor networks start. Robotic systems need to consider various factors in the real-world environments including physical abilities and limitations of both customers and devices. Moreover, as stated previously, a variety of different devices shall be integrated to realize continuous robotic support in our daily lives.

As such, different kinds of “ubiquity,” not just for locations but also various applications and ways of providing each service on the basis of customer attributes, shall be focused on, as well as inter-application and inter-location continuity. As the complexities of robotic devices are much larger than traditional information systems, at the same time, we shall provide abstracted, common access methods for application programmers and service providers to make the development process easy and highly reusable. A new framework is now required that can bridge the gap between that improves and extends traditional robot systems that satisfy such demands. Based on such consideration, an extended infrastructure based on NRS has been proposed, the Ubiquitous Network Robot Platform (UNR-PF) [b-Sato2011b]. UNR-PF combines multiple robots in multiple areas with the support of NGN, USN and their middlewares that provide access to sensor networks and portable devices. This concept offers the possibility of providing innovative services promptly and at low lost.

Requirements for UNR-PF

## Relationships of USN and

## applications and services

### Abstraction of functionality

One of the most simple use cases of UNR-PF is collaboration among sensor networks and robots. A sensor network measures phenomena in the world, notifies them to the application, and then the application commands actuator devices such as robots and smartphones to interact with people. Such collaboration may be performed by USN, without UNR-PF, when treating robots as one of “sensor nodes” with actuator facility. However, by the abstraction of functionality that UNR-PF provides, this can be realized in much simple way.

Consider the example shown in Figure 6.1. Here, based on the sensing result that a person is approaching a dangerous construction area, a robot makes warning to the person. In order to warn the person a) a mobile robot can go close to her and warn her or b) a speaker nearby can output voice to warn her. From the viewpoint of the effect, to let her know that she is approaching a dangerous area, these two devices can perform equivalent services. Dangerous spot changes day by day. If the application was coded to use a navigation robot and if there were no robot available nearby, the customer cannot receive warning. However, if the application was programmed to utilize certain functionality, then the system can search for device that can provide such functionality and choose it based on resource availability. This is also useful from the point that applications are not bound to a certain device at the time when it was coded; even though the devices has changed, if they provided the same kind of functionality, then the application can continue to provide service without change.

Figure 6.1: Service execution by abstracted functionalities

Although in the above we focused on the actuation part, the same concept can be applied for the sensing part. When the sensing functionality is also abstracted, applications can request abstracted sensing functionalities, such as detection of dangerous area, to the infrastructure, while the actual sensing equipment or sensor network in use may change.

### Inter-service collaboration

Consider the photo-printing service shown in Figure 6.2(a). This service has two tasks: take a picture and then print it out. This represents a simple functional dependency. Robot-A possesses the photo-taking function while Robot-B possesses the photo-print function; they are spatially separated, but linked via a network. To realize this service, the photo-printing task can be performed only after the photo-taking task. Therefore, the robots need to interact with the customer based on the customer’s history.

Another example is a

service that guides exhibits in a museum, exhibition hall etc., as shown in Figure 6.2(b). The tasks are to explain the exhibit and to guide the visitor to the next exhibit. Given that the visitor is free to visit the exhibits in any order, each robot needs to modify its responses according to the visitor’s service history.

(a) Photo-printing service

(b) Guiding service in museum

Figure 6.2: Inter-service collaboration

As can be seen from these examples, there are many cases where multiple tasks or applications need to collaborate with each other to form a consistent service to customers. In some cases, applications from different service providers may need to collaborate. In the photo-printing service example, the photo shooting and printing may be performed by different companies. In both situations, the important thing here is that the applications share certain aspects of customer history. In the photo printing service case, only the photograph data may be exchanged. However, in the museum guidance service, robots may share much detailed information such as in which point the customer was interested in, what kind of questions did the customer asked, or how long the customer spent time in front of Gogh’s painting. In some cases, such information sharing can be performed within frameworks of traditional information system. However, for certain information that is required to affect human-robot interaction, a common infrastructure for sharing information and reflecting it to robot behavior is necessary.

### Service among multiple areas

One of the key aspects in multiple device collaboration is resource allocation in spatiotemporal domain. Robots, as well as sensor network, can only be effective in limited areas due to their physical nature. Sensors can only sense phenomena happening in a limited area, and robots can only serve within a certain area. For example, robots nowadays have only a limited capability for navigation and are interfered by obstacles, floor materials and bumps. Battery is another concern; after a while, robots need to go for charging. Due to these limits, in order to serve people this has much wider area of activity, multiple robots and sensor networks shall collaborate with each other while applications continue. For example, consider a navigation service where a robot guides around a customer in a shopping mall. Depending on its ability, the robot may not be able to navigate in certain areas such as outdoor corridor between different mall buildings. In such case, however, the service must continue. Therefore, UNR-PF will perform hand-over of the navigation act that is necessary for the guidance service (Figure 6.3). Such hand-over may occur in different patterns based on the ability of the robot and the resource availability. Similar case may likely to happen for sensing equipment such as for localizing or identifying people throughout the mall. When a customer reaches end of sensing area of one device, UNR-PF searches for other devices that can continue to sense the customer so that the application can continue to receive and utilize sensing results.

Figure 6.3: Example of service hand-over between robots

In order to realize a smooth hand-over between various devices, UNR-PF shall provide rich support so that applications do not need to concern about the actual resource availability or the device ability. UNR-PF shall manage various devices in not only through spatial but also in temporal space in order to effectively allocate them for application requirements. At the same time, in order to perform effective hand-overs among devices of various natures and in varieties of areas, UNR-PF needs to manage spatial information as well as specifications of devices. As shown in Figure 6.4, navigation service is one typical example that clearly shows necessity for such information. In many indoor areas as well as outdoor areas, there are places not suitable for robots to move around. This limitation depends on specifications of robots and what kind of service the robot needs to perform. Moreover, the spatial nature may change by time. For example, think of a case where a guides around in a station. In mornings and evenings on weekdays, in the rush-hours, there are huge numbers of people passing through and thus the corridors becomes not suitable for robots to move around. But in the daytime, the station is much sparse and the robot can easily perform its service. As there are many cases where such dynamic changes occur, spatiotemporal nature of areas shall be managed and considered by UNR-PF.

Figure 6.4: Spatial information for robot and customer activities

### Service execution based on customer attributes

Another important requirement for UNR-PF is selection of devices based on customer attributes. Similar to selecting combination of robots based on spatiotemporal availability and robot ability, customer attribute is one element that UNR-PF needs to consider for its robot allocation planning. As for general customer tendencies, many data mining algorithms and applications has been built and are now actively in use, such as online stores. In such sites, the application servers ‘remember’ what a certain customer has chosen in the past and would recommend products that are likely to suite the customers. However, for real-world applications using robots, this is much serious issue. One case that UNR-PF handles is about physical disabilities. If a customer can only walk slowly, the robots need also need to slow down. If a customer feels burden walking on roads settled over hills, the route to navigate shall be chosen appropriately. When a customer requires a wheelchair to move around, a wheelchair robot or a robot that can push wheelchairs shall be used for application execution. As such, walking ability is one clear example to show why customer attribute is important in planning device allocation for service execution. There are, however, number of such factors including visual disabilities or impairment in audition. Based on descriptions for each customer, UNR-PF needs to carefully select appropriate devices to make customer experience as better as possible.

Figure 6.5: Service execution based on customer ability

## Relationships of USN and UNR-PF

UNR-PF need to utilize USN and USN middlewares in two ways: a) as an infrastructure with advanced functionalities such as intelligent routing of messages, and b) as a representative of various sensor networks / nodes. USN may provide connectivity not only to sensors, or unconscious robots, but also to visual and virtual types of robots. From the viewpoint of USN, UNR-PF is one application. However, this application, UNR-PF, may require much detailed information and control over USN, such as detailed device profiles, common representation of measurements and abstracted command interface. We can also think of UNR-PF as higher layer router over USN. UNR-PF selects and allocates appropriate devices to applications. This allocation may change dynamically over time, while the applications do not care or notice the actual changes.

# Use cases of UNR-PF

## Health support service

Figure 7.1: Health support service

A health support service using two different platforms is shown in Figure 7.4. One is an electronic tag attestation platform. It discloses customer’s information according to her demand. A user is registered in Entrance area at first. Also, the disclosure level of user information is set in here. Next, a user sets up the alarm clock of get-up time in alarm clock support area. And then the user can exercise in Physical exercise area for checking the user’s health condition. A teacher robot instructs the exercise and the assistant robot talks to the user according to the action of user. The users’ health condition is checked by the action recognition unit embedded in robots. Finally, the user gets the result of his/her health condition in the health check area.

## Shopping support service

Figure 7.2: Guide service in a shopping mall

The

second use case of UNR-PF is shopping support. A typical service scenario is performed across three areas: customer’s home, shopping mall, and operator center. Figure 7.2 shows the overview of the scenario. When a customer feels like going for a shopping, she first makes reservation of the service using her smartphone. Here, with a help from a remote operator, she makes rough plan of what to buy today. The operator or the smartphone app may provide her some recommendations on things to buy (“fishes are on sale today”)  or on where to go for shopping (“Many people marked store A to be grade A”). After arriving at the mall, she finds that a robot is waiting for her at the entrance on time. In this example, as the customer had disabilities in walking, a wheelchair robot was prepared. After riding the wheelchair robot, the robot guides her around in the mall so that she can purchase her planned items.

Figure 7.3 shows the structure and interaction of the service. At first, the customer makes a reservation via the virtual-type robot on the mobile device at his/her home. The virtual-type robot is connected to the local platform (LPF) settled in the user’s home. LPF notifies the global platform (GPF) of the reservation information. The shopping support service application is connected to the GPF and receives the reservation. When receiving the reservation, the service application registers its service ID and the start condition, i.e., the user’s arrival at the shopping mall, to the service queue on the GPF. The GPF refers to its map register to confirm the LPF of the shopping mall and then registers the service and its start condition to the LPF’s service queue. When the user approaches the shopping mall, the virtual-type robot on the user’s mobile device connects to the LPF of the shopping mall and notifies it of the user arrival. The LPF of the shopping mall determines that the state meets the start condition for the service and notifies the service application of the start via the GPF. Then, the service application requests the resource manager to reserve the robot in the shopping mall and the operator in the operator center via the GPF. To reserve the robot, the resource manager refers to the user registry and selects the robot suitable for the user. In the shopping support service, the manager selects the wheelchair robot if the user has difficulty walking. Otherwise, it selects another movable robot. After the allocation of the resources, the service is executed in accordance with the service flow defined in the service application. The service application refers to the map registry in the LPF and instructs the robot to move around the shopping mall.

In the case of a large shopping mall, the shopping support service is provided across several LPFs. When the robot comes close to the boundary between the two areas, the robot notifies GPF’s state manager of the alert via LPF’s state manager of the current area. Then, GPF’s state manager refers to its map register to find the LPF of the next area and then registers the service and its start condition, i.e., the customer’s arrival at the next area or the service queue of the LPF. When the user arrives at the next area, the robot disconnects from the LPF of the first area and connects to the next LPF if the robot can work in the next area. Otherwise, another robot in the next area comes to pick the customer up. Thus, the service is executed continuously. In this way, the customer can receive the service seamlessly in a wide area regardless of the area segmentation for the robotic system.

Figure 7.3: Overview of guide service sequence

# Reference architecture of UNR-PF

UNR-PF consists of three layers: 1) the functional module layer, 2) the service application layer and 3) the common platform layer. The first layer consists of reusable functional modules that belong to individual robots, and the second layer manages logic for service contents. This service application layer is typically held and maintained by service providers, using the common interface that UNR-PF provides.

The common platform layer should be independent of and serve common functions to the other two layers. Once such a common platform is developed and provided stably, it will separate robotic functions and services so that the whole robotic system will be able to be developed at lower cost and become more dependable in terms of reusability, scalability, and availability. Reference architecture of UNR-PF is shown in the dotted part of Figure 8.1. Service applications, which define flow of the service contents, and robots in each environment connect and register to UNR-PF at first. Then, the interaction between the service applications and the robots is established via UNR-PF.

Figure 8.1: Reference architecture of UNR-PF

ces and is composed of two layers of platform: a local platform (LPF) and a global platform (GPF). The LPF is a platform for configuring the robotic system in a single area. The GPF is a platform for configuring the robotic system in a wider range of areas that includes a number of LPFs These platforms serve as a middle-layer between the service application layer and robotic function layer. The platform has five database functions and three management functions. The database functions consist of robot registry, operator registry, user registry, map registry, and service queue. The management functions consist of a state manager, resource manager, and message manager. Each function is detailed below.ces and is composed of two layers of platform: a local platform (LPF) and a global platform (GPF). The LPF is a platform for configuring the robotic system in a single area. The GPF is a platform for configuring the robotic system in a wider range of areas that includes a number of LPFs These platforms serve as a middle-layer between the service application layer and robotic function layer. The platform has five database functions and three management functions. The database functions consist of robot registry, operator registry, user registry, map registry, and service queue. The management functions consist of a state manager, resource manager, and message manager. Each function is detailed below.

As such, UNR-PF serves as middleware between services and robotic devices and is composed of two layers of platform: a local platform (LPF) and a global platform (GPF). LPF is a platform for configuring the robotic system in a single area. GPF is a platform for configuring the robotic system in a wider range of areas that includes a number of LPFs. These platforms serve as a middle-layer between the service application layer and robotic function layer. The platform has five database functions and three management functions. The database functions consist of robot registry, operator registry, user registry, map registry, and service queue. The management functions consist of a state manager, resource manager, and message manager. Each function is detailed below.

### Robot registry

The system contains a registry (database) for managing attributes for robots, operators and robotic service users. These are used for selecting appropriate robots and/or operators on the basis of each user’s demands. The robot registry contains information about the robots available in each area, such as their shapes and capabilities as well as their status. In addition, functionalities that each robot provides, such as navigation, people detection or face recognition, are stored in the registry as functional profiles. These pieces of information shows what kind of service a certain robot can perform, what kind of route the robot can navigate, and whether the robot is available now or in the near future. From this information, UNR-PF can decide what device to allocate for a certain application.

### Operator registry

Operators for tele-operation of robots are managed in the operator registry. Such operators are often necessary to compensate for the limitations of artificial intelligence in robot AIs. In UNR-PF, operators are treated as one kind of robot. The same selection mechanism works for the operators on service application demands based on the information stored in the operator registry.

### User registry

The user registry holds attributes on each customer who wishes to receive robotic support, as well as a history of services that customer has used. From these pieces of information, UNR-PF can allocate the robots and operators necessary to provide certain services.

### Map registry

The map registries in both LPF and GPF are used to improve the linkage between different areas. The map registry of the LPF contains the map information of the service execution environment, such as the properties of the floor and information about movable zones, and no-go zones. The map registry of the GPF contains the positional relationship of each single area.

### Service queue

The service queue is a function implemented in both LPF and GPF. This function is used to manage the start of the service. This database contains IDs of the service and their initiation conditions. At first, the service application registers its ID and initiation condition to the service queue in the GPF and the state manager in the GPF registers the ID and condition to the service queue in the appropriate LPF in accordance with the state notification from the LPFs.

### State manager

The state manager is implemented in both LPF and GPF. This function subscribe to the message manager for the state notifications that are registered in the service queue. When receiving the state notification, the manager determines if the state complies with the start conditions in the service queue. If the state complies with the start condition, the manager sends a message to the service application to start the service.

### Resource manager

The resource manager is implemented in LPF. When receiving the command message for executing the service application, the manager refers the robot registry, the user registry and operator registry and reserves the robot suitable for the customer and the operator who can operate the service depending on the situation.

### Message manager

The message manager is implemented in both LPF and GPF. This function manages the messages exchanged between the service applications and the robotic functions through the common interface. The robotic functions provide the profile of available messages to the manager. When receiving the message from the service applications, the manager refers to the profiles and selects the robotic functions that fit the service application requirements. In LPF, the manager requires the resource manager to reserve the necessary resources. When receiving the message, i.e., the state notification from the robotic function, the manager checks the delivery addresses, which are given at the time of the state notification subscription, and forwards the message to the appropriate state manager and/or the service applications.

# USN capability requirements for UNR-PF applications and services

## Formal device profile description

Device profiles are essential for UNR-PF in order to dynamically allocate and configure devices for applications and services. While USN optionally provides device profiles for sensor networks and/or sensor nodes [ITU-T Y.2221], this is not sufficient. Every devices including sensor networks and sensor nodes, robots and portable information devices that are managed by UNR-PF shall provide formal description of their abilities and physical natures as profiles. Moreover, profiles for abstracted functionalities shall be prepared by UNR-PF so that applications and services can utilize them to organize their process flow. An effort to standardize functional profiles of robotic devices as well as sensing systems has begun [b-Sato2011a] and it is recommended that USN activity be in collaboration and harmonized with such effort.

## Dynamic management of devices in spatiotemporal domain

UNR-PF, as well as USN, can be viewed as middlewares that bind applications and services that are controlled in the cyberspace to devices and people in the real environments, through resource allocation and message routing. As such, information on dynamically changing nature of space and time shall be treated effectively and formally in every aspect of management, especially for applications and services that span across multiple areas and relatively long term. One of the most important is to manage the availability and validity of devices in space and time, such as effective range of sensors and motion limit of actuators. Such information shall be handled promptly in USN, in both static and dynamic way. Efforts to standardize effective range of sensors have begun [b-Kolbe2011], as well as spatial descriptions suitable for robots [b-OGC2011], and it is recommended that USN activity be in collaboration and harmonized with such efforts.

## Common abstracted interface

For effective development and maintenance of applications and services, USN shall provide common and abstracted interface for controlling devices and for representing measurement results. The command sets for controlling and configuring sensor networks / nodes as well as other devices including robots and information devices shall be defined in common and semantic way. Measurement or estimation results from sensors as well as status notification from various devices shall be represented based on formal basis and exchanged in standardized steps. Efforts to standardize common abstracted commands and measurement representation have begun [b-OMG2010, b-Nishio2010, b-Sato2011a] and it is recommended that USN activity be in collaboration and harmonized with such effort.

Bibliography

[b-Jung2007]	Jung, H.-S. et al., Unified Ubiquitous Middleware for U-City, in Proc. ICCIT2007, pp. 2374-2379, 2007.

[b-Kolbe2011]	Kolbe, T.H., Indoor Localization and Tracking supported by 3D Models, OGC Standards for the Internet of Things - a Workshop, 2011.

[b-Nakamura2008]	Y. Nakamura, et. al Network Robot Systems, Robotics and Autonomous Systems, Volume 56, pp.

, Framework and service allocation for network robot platform and execution of interdependent services, Robotics and Autonomous Systems, Volume 56, pp.831-843, 2008.

[b-Nishio2010]	Nishio, S. et al., Robotic Localization Service Standard for Ubiquitous Network Robots, in Abdellatif, H. ed., Robotics 2010 Current and Future Challenges, pp. 381-400, In-Teh:Vukovar, Croatia, 2010.

[b-OGC2011]	Open Geospatial Consortium, OpenGIS City Geography Markup Language (CityGML) Encoding Standard, version 1.1.0, 2011.

[b-OMG2010] Object Management Group, Robotic Localization Service, version 1.0, formal/2010-02-03, http://www.omg.org/spec/RLS/, 2010.

[b-Shiomi2011]	Shiomi, M. et al., Field Trial of a Networked Robot at a Train Station, Int. J. Social Robotics, Vol. 3, No. 1, pp. 27-40, 2011.

[b-Salvini2011]	Salvini, P. et al., Do Service Robots Need a Driving Licence? , IEEE Robotics and Automation Magazine, Vol. 18, No. 2, pp. 12-13, 2011.

[b-Sanfeliu2010]	Sanfeliu, A., et al., Network Robot Systems, Robotics and Autonomous Systems, Volume 56, pp. 	793-797, 2008.

[b-Sanfeliu2011]	Sanfeliu, A. et al., Decentralized Sensor Fusion for Ubiquitous Networking Robotics in Urban Areas, Proc. Sensors, pp. 2274-2314, 2010.

[b-Sato2011a]	Sato, M., et. al, Standardizing Framework for Robotic Services and Functions, Proc. ICRA Workshop on Robotics Modular Architecture Design and Standardization, 2011.

[b-Sato2011b]	Sato, M. et al., The Ubiquitous Network Robot Platform: Common Platform for Continuous Daily Robotic Services, in Proc. 2011 IEEE/SICE International Symposium on System Integration, pp. 318-323, 2011.

[b-Tezuka2006]	Tezuka, H. et al., Robot platform architecture for information sharing and collaboration among multiple networked robots, Journal of Robotics and Mechatronics, Vol. 18 No. 3, pp. 325–332, 2006.

Appendix I
Gap analysis on network robot platform

(This appendix does not form an integral part of this Recommendation)

Editor’s note: In this clause, it will be introduced what issues of network robot platform exist and how network robot platform is exploited in the other SDOs, and then what issues of them can be standardized in ITU-T will be analyzed.

- [TBD]

____________________