| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4203 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q5 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | F.TPS-Reqs: Proposed updates on requirement of FECC function |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to add a requirement item on support of FECC function in telepresence system and to delete the living item in the draft Rec. F.TPS-Reqs.

# Introduction

During the last ITU-T SG16 plenary meeting (Geneva, Nov 2011), a contribution on support of FECC function has been reviewed, after discussion, this item has been added to the living list with an editor’s note that “suggest that the proponent amplify this (with the use cases) and resubmit”.

So this contribution gives some clarification.

# Discussion

As described in the architecture of telepresence system (F/H.TPS-Arch), a telepresence endpoint may have multiple main cameras and document cameras. It’s necessary for end user to operate PTZ (Pan Tilt Zoom) cameras to physically adjust one or multiple of these cameras separately.

For example, one endpoint with 3 cameras, the middle camera can be used as main video zoomed in on the middle region or it could be used in zoomed out mode and capture the whole region.

Another example, a document camera hung from the ceiling is used to capture images of objects in a conference room, it needs to support far-end camera control by end user to observe this objects from different viewing angles.

Since a telepresence endpoint may have multiple cameras, to implement far-end camera control of each camera, it needs to identify the camera related with its location and transmit this information. Hence it’s different from FECC function from traditional videoconferencing endpoint with just one camera.

# Conclusion

It’s proposed to pick up FECC function as a requirement item and delete the related living list item.

Attachment
Proposed text

[Begin proposed text]

Living List Items:

## 8.3	Conference control functions

Con03:  The telepresence system should support far-end camera control (FECC) of a certain camera separately with its identification information related with location.

[End proposed text]

___________________