| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4205 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q21 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation, China Telecom |  |  |  |
| Title: | H.VSprot: Proposed text for camera pan-tilt-zoom control |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to add text on signalling and protocols for far-end camera control in clause 6 of draft new Recommendation H.VSprot “Signalling protocols for visual surveillance”.

# 1.	Introduction

The draft new Recommendation H.VSprot focus on the detailed description of reference points, signalling and protocols of visual surveillance system based on the functional architecture defined in the Recommendation H.626 (i.e. H.VSarch).

By the end of the last ITU-T SG16 plenary meeting (Geneva, Nov 2011), most of the sections are completed. This contribution will continue to address on the signalling and protocols for the service of far-end camera control (FECC) which the content is still blank in current draft document.

# 2.	Discussion & Proposal

We notice that in conversational video applications, far-end camera control protocol is used by participants to control the remote camera.  The protocol that is commonly used is ITU H.281 over H.224.

And IETF RFC4573 registers the H.224 media type.  It defines the syntax and the semantics of the session Description Protocol (SDP) parameters needed to support far-end camera control protocol using H.224. This media type depends on RTP framing and thus is only defined for transfer via RTP [RFC3550].  Transport within other framing protocols is not defined in this RFC.

For the scenario of far-end camera control in Visual surveillance system, it’s not the always-on communication application; there is not always a session, nor such a RTP/UDP channel between CU and PU, so it’s not suitable to use RTP/UDP to transport far-end camera control command in such a visual surveillance system defined in ITU-T Rec. H.626. On the other hand, there is additional remote control to auxiliary peripherals (such as flashlight, fan and rain brush) more than PTZ (pan-tilt-zoom control of camera) control defined in H.281.

When end users need to do FECC operation though CU, the CU sends a SIP MESSGE to create a session and transmit control command to the requested PU, and the session will be terminated after the completion of the remote operation without any living channel. The detailed structure of control command will be studied in future PU terminal related recommendations; it can also refer message structure defined in ITU-T Rec. H.281.

# 3.	Conclusion

It is proposed to add to clause 6 of H.VSprot the text on Signalling and Protocols for far-end camera control as described in the Attachment herein.

Attachment
Proposed text for H.VSprot §6 on PTZ control

[Begin proposed change]

# 6. Signalling and Protocols

## 6.7	Far-end Camera Control (FECC)

The far-end control function is used to pan, tilt and zoom cameras, switch auxiliary peripherals (such as flashlight, fan and rain brush), preset position of related physical devices.

Figure 6-1 shows the procedural flows.

# Figure 6-1 Procedural flows for FECC function

| STEP 1: | CU sends a SIP MESSAGE with information of far-end camera control command to SCU to request remote control of requested camera or auxiliary peripherals. |
| --- | --- |
| STEP 2: | SCU handles authentication with some information from CMU. |
| STEP 3: | After authentication, SCU transfers the SIP MESSAGE to the requested PU. |
| STEP 4: | PU parses this control commands and sends to related physical devices to implement the far-end camera control operations. |
| STEP 5: | PU returns a response to SCU with SIP 200 OK message. |
| STEP 6: | SCU transfers this SIP 200 OK message to CU to complete this operation. |

[End of proposed change]

__________________