| Rapporteur Meeting of Questions 3, 5, 21, 22, and 25/16 |  |  |  | AVD-4212a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 25 | Geneva, 20 - 24 February 2012 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | ITU.F.USN-SM: proposed modification of draft new Recommendation |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution contains revised texts of draft new Recommendation ITU-T F.USN-SM, whose base document is an output text from the last ITU-T SG16 meeting on November 2011.

----------- Start of changes -----------

Justification of this Recommendation:

(Editor’s note)  This is not a part of the main body of F.USN-SM and was initially prepared for information. It is because it could keep used for information until its final state.

Gap analysis on smart metering of ITU and other SDOs

There are two related activities for smart metering issues in the ITU and other SDOs.

- ITU-T

In ITU-T, there is an activity to deal with a topic about smart metering. The title is “Smart metering using connected objects in ubiquitous home networking environments,” and deals with the concept, characteristics, scenario and functional architecture for smart metering in telecommunication perspectives.

- ETSI

ETSI TC M2M (Machine-to-Machine) published Smart Metering Use Cases. This introduces M2M application for smart metering and examples, and it describes the use cases for the smart metering M2M applications.

However, these two activities do not deal with USN capabilities for supporting smart metering services.

Contents

Pages

1	Scope	3

2	References	3

3	Definitions	3

3.1	Terms defined elsewhere	3

3.2	Terms defined in this Recommendation	4

4	Abbreviations and acronyms	4

5	Conventions	4

6	Overview of smart metering	4

6.1	Smart grids and smart metering	4

6.2	Technologies of smart metering	6

7	Smart metering scenarios	7

7.1	Scenario I: regularly scheduled remote meter reading	7

7.2	Scenario II: on-demand remote meter reading	7

7.3	Scenario III: demand response	8

7.4	Scenario IV: tariff configuration	9

7.5	Scenario V: meter reading data aggregation	10

8	Requirements of smart metering services	10

8.1	Reliable information delivery	10

8.2	Real-time delivery of meter reading data	11

8.3	Bi-directional communication between meters and operators	11

8.4	Security support	11

8.5	Meter reading data processing	11

9	USN-based smart metering services	11

10	USN capabilities for smart metering services	14

10.1	Reliable transmission	14

10.2	Scalability	14

10.3	Mobility support	14

10.4	Delivery latency	14

10.5	Fault detection and recovery	14

10.6	Security	14

10.7	Connectivity	14

10.8	Unicasting & Multicasting	15

10.9	Data aggregation	15

Bibliography	15

Draft new Recommendation ITU-T F.USN-SM

Capabilities of ubiquitous sensor network (USN) for supporting requirements of smart metering services

AAP Summary

To be provided before Consent.

Summary

This Recommendation identifies capabilities of ubiquitous sensor network for supporting requirements of smart metering services.

Keywords

Smart grid, smart metering, USN

# Scope

The main purpose of this Recommendation is to identify capabilities of ubiquitous sensor network which supports the requirements of smart metering services. The scope of this Recommendation covers the following:

- Overview of smart metering;

- Smart metering scenarios;

- Requirements of  smart metering services;

- USN capabilities for supporting requirements of  smart metering services;

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T Y.2221]	ITU-T Recommendation Y.2221 (2010), Requirements for support of ubiquitous sensor network (USN) applications and services in the NGN environment

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	sensor [ITU-T Y.2221]: An electronic device that senses a physical condition or chemical compound and delivers an electronic signal proportional to the observed characteristic.

3.1.2	sensor network [ITU-T Y.2221]: A network comprised of inter-connected sensor nodes exchanging sensed data by wired or wireless communication.

3.1.3	sensor node [ITU-T Y.2221]: A device consisting of sensor(s) and optional actuator(s) with  capabilities of sensed data processing and networking.

3.1.4	smart grid [ITU-T FG-Smart]: An electricity network that can intelligently integrate the actions of all users connected to it - generators, consumers and those that do both - in order to efficiently deliver sustainable, economic and secure electricity supplies.

3.1.5	ubiquitous sensor network (USN) [ITU-T Y.2221]: A conceptual network built over existing physical networks which makes use of sensed data and provides knowledge services to anyone, anywhere and at any time, and where the information is generated by using context awareness.

## Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	smart meter: A premise device to monitor and control electrical power, gas and water usage of home appliances based on demand response information from home appliances.

3.2.3	smart metering: An operation to provide information to customers and smart metering operator about energy consumption. The information includes how much energy the customers are consuming or generating and how much it costs, etc.

3.2.4	demand response: A smart metering feature that allows consumers to reduce or change their use patterns of electricity, gas and water during peak demand usually in exchange for a financial incentive.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ETSI | European Telecommunications Standards Institute |
| --- | --- |
| M2M | Machine-to-Machine |

(Editor’ Note) The Abbreviations and acronyms will be added.

# Conventions

None.

# Overview of smart metering

Smart metering is an operation to provide consumers with useful information about consumption of energy, such as electricity, gas, and water. It has more advanced features than simple automated meter reading. Also, smart metering can be part of a smart grid, but only smart meters do not constitute a smart grid. Therefore, this section introduces smart grids, relationships between smart grids and smart meters, technology of smart metering as follows.

## Smart grids and smart metering

Smart grid is a type of electrical grids, which attempt to efficiently deliver reliable, economic, and sustainable electricity services through prediction and response to the behavior and actions of all electric power consumers (see Figure 1). It is because existing systems of power grids cannot keep pace with modern challenges, such as:

- Security threats from cyber attack

- National goals to employ alternative power generation sources

- Conservation goals that seek to lesson peak demand surges so that less energy is wasted

- High demand for an electricity supply

- Digital controlled devices that can alter the nature of electrical load and result in electricity demand

Therefore, the smart grids which can overcome the challenges provide a lot of benefits as follows:

- Enhancement of reliability

- Reducing peak demand

- Shifting usage to off-peak hours

- Lower total energy consumption

- Actively managing electricity charging

- Actively managing other usage to respond to other renewable resources

Smart grids technologies have already used in other applications, such as manufacturing and telecommunications. In general, smart grid technology can be divided into seven areas, such as integrated communications, sensing and measurement, smart metering, advanced components and control, improved interfaces and decision support, and smart power generation. In other words, smart meters are part of smart grid as key components, and also smart metering is one of crucial features for smart grids.

Figure 1: Smart Grid Architecture [b-NIST]

## Technologies of smart metering

Not only governments and utility companies, such as electricity, gas, and water suppliers, but also researchers have been interested in automatic meter reading based on communication system, known as smart metering because smart metering can bring various benefits to customers and governments as well as utility companies as follows:

- Lower metering cost

- Energy savings for residential consumers

- Reliability of supply

- Various pricing schemes to attract new consumers

- Easier detection of fraud and outage

- Automated billing

Smart metering performs metering and exchanging metering information between smart meters and utility companies. Various technologies for metering and exchanging metering information have been implemented. Power line communication has been used as means for delivering the electricity, gas and water measurements to the utility providers. In addition, messaging over GSM network has been utilized for automatic meter reading system.

Sensor network technologies can be used for metering and collecting information, and communication networks can be used for exchanging the information. Figure 1 depicts an overall diagram of smart metering systems. The metering information, obtained from home appliances by sensor nodes including communication interfaces in a smart home, are collected and delivered to operators of utility companies.

The operators manage the collected information and inform consumers of variable pricing information or enforced load control messages. Such operators’ action may lead to consumers’ reducing energy consumption.

Figure 2: Overview of smart metering

# Smart metering scenarios

## Scenario I: regularly scheduled remote meter reading

Scenario I describes procedures that meter reading data are delivered to smart metering operators at regularly scheduled intervals.

- Smart metering operator sends a message including schedule information to smart meters.

- Smart meters are set up as the schedule for measurement and data transfer.

- According to the schedule, smart meters measure energy consumption from home appliances and obtain measurement data.

- The smart meters deliver the data to the smart metering operator.

Figure 3: Regularly scheduled remote metering

## Scenario II: on-demand remote meter reading

Scenario II describes procedures that smart meters measure energy consumption and delivered the results to smart metering operator on demand.

- When a smart metering operator is required to collect measurement data from the smart meters, the operator sends a task message to smart meters in order to collect measurement data.

- The smart meters verify the operator’s message and then measure energy consumption if the message is valid.

- The result data are delivered to the smart metering operator.

Figure 4:  On-demand remote meter reading

## Scenario III: demand response

Scenario III describes how customers respond to metering operator’s demands.

- Smart metering operator sends a message including schedule information to smart meters.

- Smart meters are set upas the schedule for measurement and data transfer.

- When the number of home appliances turned on simultaneously is on the increase, the total amount of energy consumption also steeply increases.

- Information about the increasing amount of energy consumption is being reported to the smart metering operator at scheduled intervals, and price of the energy may also be increased by tariff policies depending on supply and demand.

- (a) The smart metering operator sends a message which may inform consumers of the increased price.

(b) The smart metering operator sends load control messages to enforce to reduce energy consumption or shut off by force due to management policy.

- (a) The smart meters display the message to the consumers, and the consumers may decide to reduce energy consumption, or the home appliances may be switched off by a consumer.

(b) When receiving the load control messages, the smart meter display the message and shut off. The connected home appliance is automatically switched off.

Figure 5: Demand response

## Scenario IV: tariff configuration

Scenario IV describes how price of energy consumption is decided and configured into the meters.

- Smart metering operator sends a message including schedule information to smart meters.

- Smart meters are set up as the schedule for measurement and data transfer.

- The smart meters regularly deliver meter reading data to the metering operator according to regularly scheduled intervals.

- The price of the electricity may be dynamically changed by pricing policies depending on supply and demand.

- When the price is changed, a message including changed price information is delivered to all of the smart meters.

- The changed cost information may be shown on their displays, and the changed cost is applied.

Figure 6: Tariff configuration

## Scenario V: meter reading data aggregation

This scenario describes procedures that meter reading data, obtained at regularly scheduled intervals by smart meter, are collected at aggregating smart meter (or at aggregating smart metering gateway) and then delivered to metering operators.

- Smart metering operator sends smart meters message to request aggregated metering data.

- Smart meters may be scheduled to obtain meter reading data.

- According to the schedule, smart meters measure energy consumption and obtain meter reading data from home appliances.

- The obtained meter reading data are collected in an aggregating smart meter (or aggregating smart metering gateway).

- Finally, meter reading data are transferred to a smart metering operator.

Figure 7: Meter reading aggregation

(Contributor’s Note) The data aggregation scenario is updated with a new figure for clarification. Therefore, the following editor’s notes may be removed.

(Editor’s Note) The data aggregation for batch process can give negative effects to smart metering real-time services. This fact need to be took into account.

(Editor’s Note) The scenario V is not enough to explain metering data aggregation so required to be updated with a figure in the further contributions.

# Requirements of smart metering services

## Reliable information delivery

In the smart metering services, meter reading data can be frequently transferred from distributed meters, and also operators can send many control messages to smart meters. If delivery has failed, the operators cannot collect all of meter reading data from smart meters. As a result, missing meter reading data would lead to undesirable results. Therefore, the smart metering services are required to provide reliable information delivery, such as meter reading data and messages described in the scenario.

## Real-time delivery of meter reading data

Demands of smart metering operators require real-time responses of consumers. Therefore, meter reading data are required to arrive at the operators within a given amount of time. In the scenario I-3 and II-3, if the meter reading data are not delivered within the given time, the operators cannot obtain real-time results of smart metering. Therefore, smart metering services are required to guarantee real-time delivery.

## Bi-directional communication between meters and operators

Meter reading data are delivered from smart meters to smart metering operators as shown in the scenario I-3, II-3, and III-2, and the messages are also sent from the operators to the smart meters vice versa (in scenario III-3 and IV-3). For doing this, the smart metering services are required to support bi-directional communication between meters and operators.

## Security support

Operators need to control meters in order that the meters take actions according to the indication of operators. In this case, the operators should be authorized by smart meters to take actions to secure against unauthorized access (scenario II-2).

Because metering information may include personal information, it should not be revealed to unauthenticated third parties. Therefore, the metering information is required to be secured to unauthenticated third parties.

## Meter reading data processing

Either operators or smart meters should be able to perform data processing on meter reading such as filtering, validation and aggregation. In some case, data mining processing is necessary, for example, analysing patterns and predicting some events.

# USN-based smart metering services

The smart metering services shown in Figure 8 require technological facilities to support metering and exchanging metering information between smart meters and metering operators in utility control centers. Basic characteristic of USN applications and services is gathering data by sensor networks and transmitting data to a remote server through communication networks. Also, control information is transmitted to sensor networks from the remote server. Furthermore, if USN middleware is deployed, the middleware can provide USN applications or sensor networks with various functions such as data query, data mining, event processing, sensor network metadata directory service, data filtering, context-aware rule processing and sensor network management. This feature of USN can be applied to the smart metering services. Sensor nodes in sensor networks have capabilities of metering and delivering metered information to a server at a utility control center. USN middleware can provide functions which are required by on-demand remote meter reading scenario and demand response meter reading scenario.

A smart metering system can be implemented by sensor nodes and USN middleware as follows.

- Smart metering sensor nodes: each sensor node has capabilities of smart metering, so sensor nodes process smart metering information for the smart metering procedures, such as meter reading collection and tariff setting.

- Smart metering gateway: A gateway is only responsible to deliver information between the sensor nodes and metering operators. In case a gateway has capabilities of smart metering, it collects meter reading data and then delivers to the metering operator(s).

- USN middleware: USN middleware provides data filtering, data query, data mining and context-aware rule processing, etc. These features of USN middleware satisfy meter reading data processing requirement and can be applied to on-demand remote meter reading scenario and demand response meter reading scenario.

Figure 8: Smart metering service based on USN

# USN capabilities for smart metering services

## Reliable transmission

Smart metering requires reliability of metering information delivery to ensure correct results. Therefore, USN is recommended to guarantee reliable transmission for data and message delivery.

## Scalability

New smart meters (sensor nodes) can be added to, or one of the existing meters (sensor nodes) can be removed from an existing smart metering group (sensor networks). Such a change should not degrade the performance of the smart metering.  Therefore, sensor networks in USN are required to support scalability.

## Mobility support

In some cases, a consumer can change the positions of home appliances. This change may require changes a meters’ position in the intra-sensor network or inter-sensor networks. Therefore, sensor networks in USN are recommended to support mobility of smart meters (sensor nodes).

## Delivery latency

Meter reading data should be delivered within a given time. For this requirement, USN is recommended to guarantee data delivery within a given time.

## Fault detection and recovery

There is a possibility of link failure between nodes due to the characteristics of wireless transmission when sensor nodes use wireless communications,. Such a failure can give negative effects on reliability and delivery latency of the smart metering services. USN is recommended to detect faults and recover from such faults.

## Security

Meter reading data and messages may be transferred to/from operators and meters by hop-by-hop transmission among sensor nodes. Therefore, hop-by-hop security among sensor nodes is recommended to be implemented.

## Connectivity

Meter reading data, obtained from each sensor node, are collected by a gateway, and then the collected meter reading data are delivered to smart metering operators through transport networks.  In addition, control messages of the operators are also delivered to each sensor node. In order to achieve this, connectivity between sensor nodes, connectivity between sensor nodes and the gateway, and connectivity between the gateway and outer networks are recommended to be guaranteed.

## Unicasting & Multicasting

A unicast networking service is required to be supported among sensor nodes so that meter reading data are delivered to smart metering operators. Furthermore, control messages of the metering operators should be delivered to all sensor nodes. Therefore, multicast networking support is also required.

## Data aggregation

Meter reading data from smart meters is collected and aggregated at smart meter or smart metering gateways. And then they are transferred to the smart metering operator(s). Therefore, USN sensor nodes and gateways are required to support data aggregation.

## Distributed processing

If there are a number of sensor nodes (smart meters) in smart home or smart building, burst meter reading data can be created and transferred to smart metering operator simultaneously. To prevent node and service failure because of burst data centralisation, distributed processing is required.

Bibliography

[b-ESTI TR 102.691]	ESTI TR 102.691 v1.1.1, “Machine-to-Machine communications (M2M); Smart Metering Use Cases,” May 2010.

[b-ITU-T Smart-I-6]		ITU-T FG-Smart Grid, Smart-I-6, “Smart metering using connected objects in ubiquitous home networking environments,” June 2010.

[b-Khalifa] 			T. Khalifa et. al., “A Survey of Communication Protocols for Automatic Meter Reading Applications,” IEEE Communications surveys and tutorials, Vol. PP, Issue 99, January 2010.

[b-NIST] 				NIST Special Publication 1108, NIST Framework and Roadmap for Smart Grid Interoperability Standards, Release 1.0, January, 2010

[TBD]

----------- End of changes -----------